ADD	CHN/78/12

5.FWA1	Additional allocation:  In [countries ...], the band 5 250-5 350 MHz is also allocated to the fixed service for fixed wireless access systems subject to compliance with Recommendation ITU-R F.[Doc.9/BL/46, FWA5GHz-EESS]. In addition, fixed wireless access systems shall not claim protection from the radiodetermination, Earth exploration-satellite (active) and space research (active) services, although the Earth exploration-satellite (active) and space research (active) services are not subject to No. 5.43A.

Reasons:	To enable operation of FWA systems in specific Region 3 countries while protecting the existing services in the band 5 250-5 350 MHz.