ADD	CHN/78/9

5.WAS1	Use of the 5 150-5 250 MHz band by the mobile service is limited to those applications described in Recommendation ITU-R M.1450 for wireless access systems, including radio local area networks, which shall be operated in accordance with the following conditions (see also Resolution [RLAN 2.3-1](WRC-03)):

–	deployment of wireless access systems, including radio local area networks, shall be restricted to indoor use only;

–	the mean equivalent isotropically radiated power density limit of any wireless access system station, including radio local area network stations, shall not exceed –20 dBW in any 1 MHz band (or equivalent –44 dBW in any 4 kHz band).

In the band 5 150-5 250 MHz, wireless access systems, including radio local area networks, must neither cause harmful interference to, nor claim protection from, nor otherwise impose constraints on operation or development of the existing services.

Reasons:	In order to protect feeder links (Earth-to-space) for non-GSO MSS and other existing services in this band, the above technical and operational restrictions should be applied to WAS including RLANs.