MOD	CHN/79/4

ANNEX  4     (WRC-03)

Need for coordination of a transmitting space station in the fixed-satellite service or in the broadcasting-satellite service where this service is not
subject to a Plan: in Region 2 (11.7-12.2 GHz) with respect to the
Plan, the List or proposed new or modified assignments 
in the List for Regions 1 and 3, in Region 1 (12.5-12.7 GHz) and in
Region 3 (12.2-12.7 GHz) with respect to the Plan or proposed modifications to the Plan for Region 2, in Region 3 (12.2-12.5 GHz) with 
respect to the Plan, the List or proposed new or modified assignments 
in the List for Region 1

(See Article 7)

With respect to § 7.1 and 7.2 of Article 7, coordination of a space station in the fixed-satellite service (space-to-Earth) or in the broadcasting-satellite service where this service is not subject to a Plan is required when, under assumed free-space propagation conditions, the power flux-density over any portion of the service area of the overlapping frequency assignments in the broadcasting-satellite service of an administration (for Region 3, broadcasting-satellite service which is limited to the List) exceeds the value derived from the expressions given below.

–147 		dB(W/(m2  27 MHz))		for  0° 		 θ < 0.23°

–135.7 + 17.74 log θ  		dB(W/(m2  27 MHz))	for  0.23° 	 θ < 2.0°

–136.7 + 1.66 θ2 	dB(W/(m2  27 MHz))		for  2.0° 	 θ < 3.59°

–129.2 + 25 log θ 		dB(W/(m2  27 MHz))	for  3.59° 	 θ < 10.57°

–103.6 		dB(W/(m2  27 MHz))		for  10.57°  θ

where  is the minimum geocentric orbital separation between the wanted and interfering space stations taking into account the respective East-West station-keeping accuracies.

...