| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Document 81-E |
|  |  |  | 27 May 2003 |
|  |  |  | Original: English |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | PLENARY MEETING |
| --- | --- |
| China (People’s Republic of) |  |
| PROPOSALS FOR THE WORK OF THE CONFERENCE |  |
| AGENDA ITEM 1.30 |  |

1.30	to consider possible changes to the procedures for the advance publication, coordination and notification of satellite networks in response to Resolution 86 (Minneapolis, 1998)

Introduction

WRC-2000 limited the modification of the orbital location of a GSO FSS satellite network/system at the stage of coordination within 12 from the orbital location published in the corresponding API.

The request for coordination is not receivable until at least six months after the submission of the advance publication information. At the same time, the date of receipt of the request for coordination determines the priority of a satellite network with respect to other satellite networks.

An administration having an API within 12 of a new API of another administration can therefore in the six-month period before the request from the latter administration is receivable submit a request for coordination in this orbital location, receive a higher priority and thus block the network of this administration. This despite the fact that the administration submitted its advance publication information for a different orbital location.

To avoid the above situation, it is proposed that WRC-03 limit the allowable orbital location of the request for coordination in No. 9.2 to within 1 of that of the corresponding API.

Proposals

ARTICLE  9

Procedure for effecting coordination with or 
obtaining agreement of other administrations1, 2, 3, 4, 5, 6

Section I  –  Advance publication of information on satellite
networks or satellite systems

MOD	CHN/81/1

9.2		Amendments to the information sent in accordance with the provisions of No. 9.1 shall also be sent to the Bureau as soon as they become available. The use of an additional frequency band or modification of the orbital location by more than  1º for a space station using the geostationary-satellite orbit will require the application of the advance publication procedure for this band or orbital location, as appropriate.     (WRC-03)

Reasons:	To avoid administrations being in a position to unduly hinder other administrations’ access to resources.

MOD	CHN/81/2

RESOLUTION  56  (Rev.WRC-03)

Modification of the procedures and requirements
for advance publication

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	Resolution 86 (Marrakesh, 2002) of the Plenipotentiary Conference;

b)	that there is concern among a number of administrations that some of the current procedures and requirements for advance publication may give rise to inequalities in the satellite filing and coordination process,

resolves

1	that, as of [end of WRC-03], the Radiocommunication Bureau and administrations shall apply the provisions of No. 9.2, as revised by this Conference;

2	that any request for coordination or modifications to a previously submitted API received by the Bureau after [end of WRC-03] shall be examined in accordance with the provisions of No. 9.2 as revised by this Conference.

Reasons:	To specify the introduction and application of revised Radio Regulation 9.2.

_______________