MOD	CHN/83/1

China proposes the modified Method B be adopted in the WRC-03 Conference. With respect to the unidentified value of X and Y, China prefer the values as follows:

An administration planning to use FSS earth stations with an antenna diameter less than 4.5 m in a geostationary network in this band shall ensure that the single entry power flux-density produced by any earth station operating within its territory does not exceed:

–111.5 dB(W/(m2 · 10 MHz)) not to be exceeded for more than 0.5% of the time produced at 36 m above sea level at the [normal baseline as defined] in UN Convention of the Law Of the Sea 1982.

–111.5 dB(W/(m2 · 10 MHz)) not to be exceeded for more than 0.5% of the time produced 3 m above ground at the border of a neighbouring country deploying land mobile radar in this band unless prior agreement has been obtained.

Reasons:	This method would offer the following advantages:

The constraint that only an earth station antenna size larger than 4.5 m can be used for FSS would be relaxed. In practice, more flexible operation will be realized for the FSS service whilst the radiolocation service is also achieved. In particular, the efficient use of spectrum is further improved.