| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Document 85-E |
|  |  |  | 27 May 2003 |
|  |  |  | Original: English |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | PLENARY MEETING |
| --- | --- |
| China (People’s Republic of) |  |
| proposals for the work of the conference |  |
| agenda item 1.34 |  |

1.34	to review the results of studies in response to Resolution 539 (WRC-2000) concerning threshold values for non-GSO BSS (sound) in the band 2 630-2 655 MHz, and to take actions as required

# 1	Background

The band 2 630-2 655 MHz is allocated to the fixed and mobile services on a primary basis in all three Regions. The band 2 535-2 655 MHz is additionally allocated to the BSS (sound) on a primary basis in nine countries in accordance with No. 5.418 (some countries may delete their names from this footnotes at this Conference).

The band 2 500-2 690 MHz was identified at WRC-2000 as an additional band for IMT-2000 and there is considerable interest and activity under way in China to make this band available for use by IMT-2000 at an appropriate time in the future. It is therefore necessary to ensure that the future use of the band for IMT-2000 is safeguarded in terms of protection from potential interference from other services, including non-GSO BSS (sound) systems.

In order to satisfy this agenda item, the CPM adopted four possible methods called Methods 1, 2, 3 and 4 as specified in the text of Sections 3.5.3 and 3.5.4 of the CPM text. Each method provides a different degree of flexibility to non-GSO BSS (sound) systems and a different degree of protection to terrestrial networks, which takes into account the expected development of terrestrial and space services in the band when this regulatory approach was adopted for this particular case. The advantages and disadvantages of each method are clearly mentioned in the relevant texts of the CPM Report.

# 2	Proposals

ARTICLE  5

Frequency allocations

MOD	CHN/85/1

5.418	Additional allocation:  in Bangladesh, Belarus, Korea (Rep. of), India, Japan, Pakistan, Singapore, Sri Lanka and Thailand, the band 2 535-2 655 MHz is also allocated to the broadcasting-satellite service (sound) and complementary terrestrial broadcasting service on a primary basis. Such use is limited to digital audio broadcasting and is subject to the provisions of Resolution 528 (WARC-92). The provisions of No. 9.21 do not apply to this additional allocation and the provisions of Table 21-4 of Article 21 do not apply to GSO networks using this additional allocation. The use of this allocation by non-geostationary-satellite broadcasting-satellite service (sound) systems is limited to national systems unless agreement has been reached to include the territories of other administrations in the service area, and operation with a minimum elevation angle over the service area of not less than 40°.     (WRC-03)

ARTICLE  9

Procedure for effecting coordination with or 
obtaining agreement of other administrations1, 2, 3, 4, 5, 6

Section II  –  Procedure for effecting coordination10, 11

Sub-Section IIA  –  Requirement and request for coordination

MOD	CHN/85/2

9.35	a)	examine that information with respect to its conformity with No. 11.3116, 16A; (WRC-03)

ADD	CHN/85/3

_______________

16A 9.35.2	When examining under No. 9.35  an assignment subject to No. 21.16.3B with respect to its conformity with No. 11.31, the Bureau shall identify and publish, under No. 9.38, the names of the administrations on the territory of which the pfd limits in Table 21-4 are exceeded. Under this examination, any excess will be considered as in conformity with No. 11.31 until the assignment is examined by the Bureau under Article 11.

ARTICLE  21

Terrestrial and space services sharing frequency bands above 1 GHz

Section V  –  Limits of power flux-density from space stations

MOD	CHN/85/4

TABLE  21-4     (WRC-03)

| NOC CHN/85/5 Frequency band | Service* | Limit in dB(W/m2) for angle of arrival () above the horizontal plane |  |  | Reference bandwidth |
| --- | --- | --- | --- | --- | --- |
|  |  | 0°-5° | 5°-25° | 25°-90° |  |
| ... |  |  |  |  |  |
| 2 500-2 690 MHz 2 520-2 670 MHz 2 500-2 516.5 MHz (No. 5.404) | Fixed-satellite Broadcasting-satellite9A Radiodetermination-satellite | –152 9 | –152 + 0.75( – 5) 9 | –137 9 | 4 kHz |
| 2 630-2 655 MHz (No. 5.418) | Broadcasting-satellite (sound) (non-geostationary-satellite orbit) | –128 9B, 9C | –128 + 0.75 ( – 5) 9B, 9C | –113 9B, 9C | 1 MHz |
| 2 630-2 655 MHz (No. 5.418) | Broadcasting-satellite (sound) (non-geostationary-satellite orbit) | –132 9B, 9D | –132 + 0.5 ( – 5) 9B, 9D | –122 9B, 9D | 1 MHz |

21.17		2)	The limits given in Table 21-4 may be exceeded on the territory of any country whose administration has so agreed.

ADD	CHN/85/6

_______________

9A	21.16.3A	These limits do not apply to the broadcasting-satellite service (sound) allocation under No. 5.418.

ADD	CHN/85/7

_______________

9B	21.16.3B	In the band 2 630-2 655 MHz, an assignment to a non-geostationary space station in the broadcasting-satellite service (sound) under No. 5.418 shall be considered in conformity with No. 11.31 if all the agreements required under No. 21.17 have been received.

ADD	CHN/85/8

_______________

9C	21.16.3C	These values apply to assignments to non-geostationary-satellite systems in the broadcasting-satellite service (sound) for which complete Appendix 4 coordination or notification information, as appropriate, has been received by the Bureau after 2 June 2000 and the due diligence information has been received by the Bureau before 9 June 2003.

ADD	CHN/85/9

_______________

9D	21.16.3D	These values apply to assignments to non-geostationary-satellite systems in the broadcasting-satellite service (sound) for which complete Appendix 4 coordination or notification information, as appropriate, has been received by the Bureau after 2 June 2000 and the due diligence information has been received by the Bureau after 9 June 2003.

APPENDIX  5  (WRC-03)

Identification of administrations with which coordination is to be effected or agreement sought under the provisions of Article 9

NOC

1	For the purpose of effecting coordination under Article 9, except in the case under No. 9.21, and for identifying the administrations with which coordination is to be effected, the frequency assignments to be taken into account are those in the same frequency band as the planned assignment, pertaining to the same service or to another service to which the band is allocated with equal rights or a higher category1 of allocation, which might affect or be affected, as appropriate, and which are:

MOD	CHN/85/10

a)	in conformity with No. 11.31MOD 2; and

_______________

2	For the purpose of effecting coordination, an assignment for which the process of obtaining agreement under No. 9.21 has been initiated or which is subject to No. 21.16.3B, as appropriate, is considered to be in conformity with No. 11.31 with respect to No. 9.21 or No. 21.16.3B, as appropriate.

MOD	CHN/85/11

TABLE  5-1 (continued)

| Reference of Article 9 | Case | Frequency bands (and Region) of the service for which coordination is sought | Threshold/condition | Calculation method | Remarks |
| --- | --- | --- | --- | --- | --- |
| No. 9.11 GSO, non-GSO/ terrestrial | A space station in the broadcasting-satellite service (BSS) in any band shared on an equal primary basis with terrestrial services and where the BSS is not subject to a Plan, in respect of terrestrial services | 620-790 MHz 1 452-1 492 MHz 2 310-2 360 MHz 2 535-2 655 MHz (GSO BSS (sound) in the countries mentioned in No. 5.418) 12.5-12.75 GHz (Region 3) 17.3-17.8 GHz (Region 2) 21.4-22 GHz (Region 1 and 3) 74-76 GHz | Bandwidths overlap- | Check by using the assigned frequencies and bandwidths |  |
| ... |  |  |  |  |  |

SUP	CHN/85/12

RESOLUTION  539  (WRC-2000)

Use of the band 2 630-2 655 MHz in certain Region 3 countries 
by non-geostationary satellite systems in the 
broadcasting-satellite service (sound)

________________