MOD	CHN/87/2

14.25-15.63 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 14.25-14.3 FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 RADIONAVIGATION 5.504 Mobile-satellite (Earth-to-space) ADD 5.YYY ADD 5.ZZ Space research 5.505 5.508 5.509 |  |  |
| 14.3-14.4 FIXED FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) ADD 5.YYY Radionavigation-satellite | 14.3-14.4 FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 Mobile-satellite (Earth-to-space) ADD 5.YYY Radionavigation-satellite | 14.3-14.4 FIXED FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) ADD 5.YYY ADD 5.ZZ Radionavigation-satellite |
| 14.4-14.47 FIXED FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) ADD 5.YYY ADD 5.ZZ Space research (space-to-Earth) |  |  |
| 14.47-14.5 FIXED FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) ADD 5.YYY ADD 5.ZZ Radio astronomy 5.149 |  |  |

Reasons:	Adding the footnote 5.ZZ is to indicate the protection requirement of the FS on the territory of China. The Administrations operating or planning to operate AMSS aircraft earth stations (AES) are informed of this protection requirement in order to avoid unacceptable interference caused by AMSS aircraft earth stations.