ADD	CTI/103/2

5.447E	The use of the band 5 100-5 250 MHz by the mobile service is limited to the applications described in Recommendation ITU-R M.1450-1 for wireless access systems, including RLANs, used indoors with mean equivalent isotropically radiated power (e.i.r.p.) not exceeding 200 mW and mean e.i.r.p. density not exceeding 10 mW in any 1 MHz band.