ADD	CTI/103/3

5.447F	The use of the band 5 250-5 350 MHz by the mobile service is limited to the applications described in Recommendation ITU-R M.1450-1 for wireless access systems, including RLANs, the operation of which shall be governed by the following conditions:

–	in the band 5 250-5 350 MHz, the use of wireless access systems, including RLANs, is limited to indoor use with mean e.i.r.p. not exceeding 200 mW and mean e.i.r.p. density not exceeding 10 mW in any 1 MHz band;

–	in the band 5 250-5 350 MHz, wireless access systems, including RLANs, must employ power control to reduce output power by at least 3 dB. In the absence of power control, the power limits given above must be reduced by 3 dB;

–	in the bands 5 250-5 350 MHz and 5 470-5 725 MHz, wireless access systems, including RLANs, must make use of interference reduction techniques (such as dynamic frequency selection, DFS) to avoid co-channel operation with other terrestrial systems, in particular radars. Arrangements must be designed so as to use the available spectrum in a broadly uniform manner so as to improve sharing with satellite services.