ADD	CTI/103/4

5.451A	The use of the band 5 470-5 725 MHz by the mobile service is limited to the applications described in Recommendation ITU-R M.1450-1 for wireless access systems, including RLANs, the operation of which shall be governed by the following conditions:

–	in the band 5 470-5 725 MHz, the use of wireless access systems, including RLANs, indoors or outdoors, must not exceed a maximum value of 1 W for the mean e.i.r.p. and the mean e.i.r.p. density must be less than 50 mW in any 1 MHz band;

–	in the band 5 470-5 725 MHz, wireless access systems, including RLANs, must employ power control to reduce output power by at least 3 dB. In the absence of power control, the power limits given above must be reduced by 3 dB;

–	in the bands 5 250-5 350 MHz and 5 470-5 725 MHz, wireless access systems, including RLANs, must make use of interference reduction techniques (such as dynamic frequency selection, DFS) to avoid co-channel operation with other terrestrial systems, in particular radars. Arrangements must be designed so as to use the available spectrum in a broadly uniform manner so as to improve sharing with satellite services.