| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Addendum 1 to Document 104-E |
|  |  |  | 3 June 2003 |
|  |  |  | Original: English |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | PLENARY MEETING |
| --- | --- |
| Korea (Republic of), Japan |  |
| proposals for the work of the conference |  |
| agenda item 1.23 |  |

1.23	to consider realignment of the allocations to the amateur, amateur-satellite and broadcasting services around 7 MHz on a worldwide basis, taking into account Recommendation 718 (WARC-92)

Introduction

This agenda item deals with a realignment of the bands around 7 MHz with a view to harmonizing the global frequency allocations.

The Republic of Korea and Japan support the principle of harmonization of frequency allocations around 7 MHz; however, we are of the view that impacts of the realignment on existing services involved in the realignment should be minimized.

Sharing of frequency bands by the amateur service and the broadcasting service should be avoided even in a transition period, because of incompatibility between the two services. Thus, the amateur service and the broadcasting service need to be allocated different frequency bands. In addition, the amount of spectrum allocated to the broadcasting service should not be reduced in order not to impair coordination of the HF broadcasting. Consequently, frequency bands exclusively allocated to the fixed or land mobile services will be reduced if an additional spectrum is allocated to the amateur service.

A reduction of the amount of spectrum exclusively allocated to the fixed and land mobile services may be compensated partly by sharing between the fixed and land mobile services and other services. Sharing between the fixed and land mobile services and the broadcasting service should be avoided because of the uncertainty of sharing technologies such as dynamic frequency sharing. Sharing between the fixed and land mobile services and the amateur service could ease the congestion problem to some extent, but operations in the fixed and land mobile services may be restricted due to unpredicted interference from the amateur services. It would be difficult to fully accommodate existing stations of the fixed and land mobile services in the narrower bands due to spectrum congestion in the bands. Therefore, a reduction of the amount spectrum exclusively allocated to the fixed or land mobile services should be no more than 100 kHz and an upgrade of land mobile service to mobile (except aeronautical mobile (R)) service would give the flexibility in operation.

The frequency realignment will inevitably impose technical, operational and economic burdens on the existing operators of the broadcasting, fixed and land mobile services, and a large audience of the HF broadcasting. In order to lighten the burdens, a corresponding time-frame for the realignment needs to be as long as 12 years so that any necessary changes can be accommodated within the normal time-frame of equipment renewal. In addition, the realignment should be implemented in a single stage because frequency changes in more than one stage will impose an extra burden on the existing operators.

Proposals

Republic of Korea and Japan propose modifications of Article 5 of Radio Regulations to provide:

As from 1 April 2015

6 765-7 000 kHz	Fixed and mobile (except aeronautical mobile (R)) co-primary

7 000-7 100 kHz	Amateur and amateur-satellite co-primary (NOC)

7 100-7 200 kHz	Amateur, fixed and mobile (except aeronautical mobile (R)) co-primary in Regions 1 and 3, amateur primary in Region 2

7 200-7 300 kHz	Broadcasting primary in Regions 1 and 3, amateur primary in Region 2 (NOC)

7 300-7 450 kHz	Broadcasting primary

7 450-8 100 kHz	Fixed  and mobile (except aeronautical mobile (R)) co-primary

In order to minimize the impacts of changes on existing operations in the broadcasting, fixed and land mobile services, it is proposed that the changes be introduced from 1 April 2015.

MOD	KOR/J/104A1/1

5 003-7 350 kHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 6 765-7 000 FIXED MOBILE (except aeronautical mobile (R)) 5.138 |  |  |
| 7 000-7 100 AMATEUR AMATEUR-SATELLITE 5.140 5.141 |  |  |

| 7 100-7 200 AMATEUR FIXED MOBILE (except aeronautical mobile (R)) ADD 5.142B | 7 100-7 300 AMATEUR | 7 100-7 200 AMATEURFIXED MOBILE (except aeronautical mobile (R)) ADD 5.142B |
| --- | --- | --- |
| 7 200-7 300 BROADCASTING | MOD 5.142 ADD 5.142A | 7 200-7 300 BROADCASTING |

MOD	KOR/J/104A1/3

7 300-7 450 BROADCASTING 5.143 ADD 5.143A

MOD	KOR/J/104A1/4

7 350-13 360 kHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 7 450-8 100 FIXED MOBILE (except aeronautical mobile (R)) 5.144 |  |  |

MOD	KOR/J/104A1/5

5.142	The use of the band 7 200-7 300 kHz in Region 2 by the amateur service shall not impose constraints on the broadcasting service intended for use within Region 1 and Region 3.

Reasons:	Modification to footnote is proposed in order to reflect the change of frequency allocation in the band 7 100-7 200 kHz in Regions 1 and 3.

ADD	KOR/J/104A1/6

5.142A	The use of the band 7 100-7 200 kHz in Region 2 by the amateur service shall not impose constraints on the broadcasting service intended for use within Region 1 and Region 3 until 31 March 2015.

Reasons:	Modification to footnote is proposed in order to reflect the change of frequency allocation in the band 7 100-7 200 kHz in Regions 1 and 3.

ADD	KOR/J/104A1/7

5.142B	The band 7 100-7 200 kHz is allocated to the broadcasting services on a primary basis until 31 March 2015.

Reasons:	Changes to the Table of Frequency Allocations are proposed in order to provide a worldwide allocation to the amateur service in the band 7 000-7 200 kHz and to provide regional co-primary allocations to the fixed and mobile (except aeronautical mobile (R)) services in Region 1 and Region 3 as from 1 April 2015.

ADD	KOR/J/104A1/8

5.143A	The band 7 350-7 450 kHz is allocated to the fixed service on a primary basis and to the mobile (except aeronautical mobile (R)) service on a secondary basis until 31 March 2015.

Reasons:	Changes to the Table of Frequency Allocations are proposed in order to provide a worldwide allocation to the broadcasting service in the band 7 300-7 450 kHz as from 1 April 2015.

______________