| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Addendum 2 to Document 104-E |
|  |  |  | 3 June 2003 |
|  |  |  | Original: English |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | PLENARY MEETING |
| --- | --- |
| Korea (Republic of), Japan |  |
| proposals for the work of the conference |  |
| agenda item 1.27 |  |

1.27	to review, in accordance with Resolutions 540 (WRC-2000) and 735 (WRC-2000), the ITU-R studies requested in those resolutions, and modify, as appropriate, the relevant regulatory procedures and associated sharing criteria contained in Appendices 30 and 30A and in the associated provisions

Introduction

In response to Resolution 540 (WRC-2000), the Administrations of the Asia Pacific Telecommunity (APT) agreed to propose several modifications to AP30 of the Radio Regulations under agenda item 1.27 regarding the pfd mask for the protection of the BSS Plan and the protection of the FSS with the BSS Plan and List. The Republic of Korea and Japan support the APT common proposals on agenda item 1.27. This document intends to complement the APT common proposals, which has undetermined elements for the modification of Sections 1 and 6 of Annex 1, Annex 4 and Annex 6 of AP30. For example, in the case of Sections 1 and 6 of Annex 1, the following texts appear:

In Section 1 of Annex 1 to AP30:

An administration in Region 3, of which the BSS Plan has national coverage only and has been brought into service before [DD.MM.YY], is considered as not being affected if either of the following two conditions are met:

In Section 6 of Annex 1 to AP30:

In the case an administration in Region 3 intends to protect its FSS assignments, which have been notified to the Bureau, brought into use and the relevant information as described in the Annex to Resolution 49 (Rev.WRC-2000) have been received by the Bureau before [DD.MM.YYYY], the administration is considered as not being affected if the proposed new or modified assignment in the Region 1 List, or if a proposed modification to the Region 2 Plan, gives a power flux-density anywhere over any portion of its own territory of its overlapping frequency assignments in its FSS networks less than:

The Republic of Korea and Japan propose [DD.MM.YY] and/or [DD.MM.YYYY] to be “5 July 2003” for all cases.

Proposals

APPENDIX  30*  (WRC-2000)

ANNEX  1     (WRC-2000)

Limits for determining whether a service of an administration is affected
by a proposed modification to the Region 2 Plan or by a proposed
new or modified assignment in the Regions 1 and 3 List
or when it is necessary under this Appendix to seek
the agreement of any other administration14

(See Article 4)

MOD	KOR/J/104A2/1

# 1	Limits for the interference into frequency assignments in conformity with the Regions 1 and 3 Plan or with the Regions 1 and 3 List or into new or modified assignments in the Regions 1 and 3 List

Under assumed free-space propagation conditions, the power flux-density of a proposed new or modified assignment in the List shall not exceed the value of –103.6 dB(W/(m2 · 27 MHz)).

With respect to § 4.1.1 a) or b) of Article 4, an administration in Region 1 or 3 is considered by the Bureau as being affected if the minimum orbital spacing between the wanted and interfering space stations, under worst-case station-keeping conditions, is less than 9°.

However, an administration in Region 1 or 3 is considered as not being affected if either of the following two conditions are met:

a)	under assumed free-space propagation conditions, the power flux-density at any test point within the service area associated with any of its frequency assignments in the Plan or in the List or for which the procedure of Article 4 has been initiated, does not exceed the following values:15

–147     dB(W/(m2 · 27 MHz))	for  0°	 θ < 0.23°

–135.7 + 17.74 log θ  dB(W/(m2 · 27 MHz))	for  0.23°	 θ < 2.0°

–136.7 + 1.66 θ2   dB(W/(m2 · 27 MHz))	for  2.0°	 θ < 3.59°

–129.2 + 25 log θ      dB(W/(m2 · 27 MHz))	for  3.59°	 θ < 9°

where  is the minimum geocentric orbital separation between the wanted and interfering space stations taking into account the respective East-West station keeping accuracies;

b)	the effect of the proposed new or modified assignments in the List is that the equivalent downlink protection margin16 corresponding to a test point of its assignment in the Regions 1 and 3 Plan or List, or for which the procedure of Article 4 has been initiated, including cumulative effect of any previous modification to the List or any previous agreement, does not fall more than 0.45 dB below 0 dB or, if already negative, more than 0.45 dB below the value resulting from:

–	the Regions 1 and 3 Plan and List as established by WRC-2000; or

–	a proposed new or modified assignment to the List in accordance with this Appendix; or

–	a new entry in the Regions 1 and 3 List as a result of successful application of Article 4 procedures.

An administration in Region 3, of which the broadcasting-satellite service Plan has national coverage only and has been brought into service before 5 July 2003, is considered as not being affected if either of the following two conditions are met:

a)	under assumed free-space propagation conditions, the power flux-density at any test point within the service area associated with any of its frequency assignments in the Plan or in the List or for which the procedure of Article 4 has been initiated, does not exceed the following values15:

–147   dB(W/(m2  27 MHz))	for  0°	 θ < 0.23°

–135.7 + 17.74 log θ   dB(W/(m2  27 MHz))	for  0.23°	 θ < 1.8°

–134.0 + 0.89  θ2  dB(W/(m2  27 MHz))	for  1.8°	 θ < 5.0°

–129.2 + 25 log θ   dB(W/(m2  27 MHz))	for  5.0°	 θ < 9°

where  is the minimum geocentric orbital separation between the wanted and interfering space stations taking into account the respective East-West station keeping accuracies;

b)	the effect of the proposed new or modified assignments in the List is that the equivalent downlink protection margin16 corresponding to a test point of its assignment in the Regions 1 and 3 Plan or List, or for which the procedure of Article 4 has been initiated, including cumulative effect of any previous modification to the List or any previous agreement, does not fall more than 0.45 dB below 0 dB or, if already negative, more than 0.45 dB below the value resulting from:

–	the Regions 1 and 3 Plan and List as established by WRC-2000; or

–	a proposed new or modified assignment to the List in accordance with this Appendix; or

–	a new entry in the Regions 1 and 3 List as a result of successful application of Article 4 procedures.

NOTE – In performing the calculation, the effect at the receiver input of all the co-channel and adjacent-channel signals is expressed in terms of one equivalent co-channel interfering signal. This value is usually expressed in decibels.

MOD	KOR/J/104A2/2

# 6	Limits to the change in the power flux-density of assignments in the Regions 1 and 3 Plan or List to protect the fixed-satellite service (space-to-Earth) in the band 11.7-12.2 GHz in Region 2 or in the band 12.2-12.5 GHz in Region 3, and of assignments in the Region 2 Plan to protect the fixed-satellite service (space-to-Earth) in the band 12.5-12.7 GHz in Region 1 and in the band 12.2-12.7 GHz in Region 3

With respect to § 4.1.1 e) of Article 4, an administration is considered as being affected if the proposed new or modified assignment in the Regions 1 and 3 List would result in an increase in the power flux-density over any portion of the service area of its overlapping frequency assignments in the fixed-satellite service in Region 2 or Region 3 of 0.25 dB or more above that resulting from the frequency assignments in the Plan or List for Regions 1 and 3 as established by WRC-2000.

With respect to § 4.2.3 e), an administration is considered as being affected if the proposed modification to the Region 2 Plan would result in an increase in the power flux-density over any portion of the service area of its overlapping frequency assignments in the fixed-satellite service in Region 1 or 3 of 0.25 dB or more above that resulting from the frequency assignments in the Region 2 Plan at the time of entry into force of the Final Acts of the 1985 Conference.

With respect to § 4.1.1 e) or § 4.2.3 e) of Article 4, an administration is considered as not being affected if the proposed new or modified assignment in the Regions 1 and 3 List, or if a proposed modification to the Region 2 Plan, gives a power flux-density anywhere over any portion of the service area of its overlapping frequency assignments in the fixed-satellite service in Region 1, 2 or 3 of less than:

–186.5    dB(W/(m2  40 kHz)) 	for  0° 	 θ < 0.054°

–164.0 + 17.74 log θ    dB(W/(m2  40 kHz))	for  0.054°	 θ < 2.0°

–165.0 + 1.66 θ2  dB(W/(m2  40 kHz))	for  2.0° 	 θ < 3.59°

–157.5 + 25 log θ   dB(W/(m2  40 kHz))	for  3.59° 	 θ < 10.57°

–131.9   dB(W/(m2  40 kHz))	for  10.57°	 θ

where θ is the minimum geocentric orbital separation between the wanted and interfering space stations taking into account the respective East-West station keeping accuracies.

In the case an administration in Region 3 intends to protect its fixed-satellite service assignments, which have been notified to the Bureau, brought into use and the relevant information as described in the Annex to Resolution 49 (Rev.WRC-2000) have been received by the Bureau before 5 July 2003, the administration is considered as not being affected if the proposed new or modified assignment in the Region 1 List, or if a proposed modification to the Region 2 Plan, gives a power flux-density anywhere over any portion of its own territory of its overlapping frequency assignments in its fixed-satellite service networks less than:

–186.5    dB(W/(m2  40 kHz)) 	for  0° 	 θ < 0.054°

–164.0 + 17.74 log θ    dB(W/(m2  40 kHz))	for  0.054°	 θ < 1.8°

–162.3 + 0.89 θ2     dB(W/(m2  40 kHz))	for  1.8° 	 θ < 5.0°

–157.5 + 25 log θ    dB(W/(m2  40 kHz))	for  5.0° 	 θ < 10.57°

–131.9     dB(W/(m2  40 kHz))	for  10.57°	 θ

where θ is the minimum geocentric orbital separation between the wanted and interfering space stations taking into account the respective East-West station keeping accuracies.

MOD	KOR/J/104A2/3

ANNEX  4     (WRC-03)

Need for coordination of a transmitting space station in the fixed-satellite service or in the broadcasting-satellite service where this service is not
subject to a Plan: in Region 2 (11.7-12.2 GHz) with respect to the
 Plan, the List or proposed new or modified assignments 
in the List for Regions 1 and 3, in Region 1 (12.5-12.7 GHz) and in
Region 3 (12.2-12.7 GHz) with respect to the Plan or proposed modifications to the Plan for Region 2, in Region 3 (12.2-12.5 GHz) with 
respect to the Plan, the List or proposed new or modified assignments 
in the List for Region 1

(See Article 7)

With respect to § 7.1 and 7.2 of Article 7, coordination of a space station in the fixed-satellite service (space-to-Earth) or in the broadcasting-satellite service where this service is not subject to a Plan is required when, under assumed free-space propagation conditions, the power flux-density over any portion of the service area of the overlapping frequency assignments in the broadcasting-satellite service of an administration (for Region 3, broadcasting-satellite service which is limited to the List) exceeds the value derived from the expressions given below:

–147    dB(W/(m2  27 MHz))	for  0° 	 θ < 0.23°

–135.7 + 17.74 log θ     dB(W/(m2  27 MHz))	for  0.23° 	 θ < 2.0°

–136.7 + 1.66 θ2    dB(W/(m2  27 MHz))	for  2.0° 	 θ < 3.59°

–129.2 + 25 log θ    dB(W/(m2  27 MHz))	for  3.59° 	 θ < 10.57°

–103.6    dB(W/(m2  27 MHz))	for  10.57°	 θ

where  is the minimum geocentric orbital separation between the wanted and interfering space stations taking into account the respective East-West station keeping accuracies.

With respect to § 7.1 and 7.2 of Article 7, coordination of a space station in the fixed-satellite service (space-to-Earth) of Region 2 is required when, under assumed free-space propagation conditions, the power flux-density on the territory over any portion of the service area of the overlapping frequency assignments in the broadcasting-satellite service of an administration in Region 3, of which the broadcasting-satellite service Plan has national coverage only and has been brought into service before 5 July 2003, exceeds the value derived from the expressions given below:

–147    dB(W/(m2  27 MHz)) 	for  0° 	 θ < 0.23°

–135.7 + 17.74 log θ     dB(W/(m2  27 MHz))	for  0.23° 	 θ < 1.8°

–134.0 + 0.89 θ2    dB(W/(m2  27 MHz))	for  1.8° 	 θ < 5.0°

–129.2 + 25 log θ    dB(W/(m2  27 MHz))	for  5.0° 	 θ < 10.57°

–103.6    dB(W/(m2  27 MHz))	for  10.57°	 θ

where  is the minimum geocentric orbital separation between the wanted and interfering space stations taking into account the respective East-West station keeping accuracies.

MOD	KOR/J/104A2/4

ANNEX  6 MOD 39

Criteria for sharing between services

ADD	KOR/J/104A2/5

part a – assumptions used in deriving sharing criteria adopted by warc-77

NOC

Sections 1 to 3 of Annex 6.

ADD	KOR/J/104A2/6

Part B – Assumptions used in deriving sharing criteria adopted by WRC-03

The establishment of new sharing criteria between the fixed-satellite service and the broadcasting-satellite service has been based on the following assumptions:

# 1	Reference antenna patterns

1.1	For earth station antennas in the fixed-satellite service or in the broadcasting-satellite service with diameters between 60 cm and 240 cm, the gain of the side lobes is given by Recommendation ITU-R BO.1213.

1.2	For earth station antennas in the fixed-satellite service with diameters greater than 240 cm, the gain of the side lobes is given by Recommendation ITU-R S.580-5, with 29 – 25log  side-lobe envelope, complemented in the main lobe by Annex III to Appendix 8, which is equivalent to Section 3 of Annex 3 to Appendix 7 (WRC-2000).

# 2	Antenna sizes41 and total noise temperatures

The range of antenna sizes and total noise temperatures considered for the protection of the fixed-satellite service and the broadcasting-satellite service are given in the following Table:

| Receive earth station antenna diameter (m) | 0.45 | 0.60 | 0.80 | 1.20 | 2.4 | 5.0 | 8.0 | 11.0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Receive earth station noise temperature (K) | 110 | 110 | 125 | 150 | 150 | 200 | 250 | 250 |
| Total link noise temperature (K) | 174 | 174 | 198 | 238 | 238 | 317 | 396 | 396 |

The total link noise temperature was calculated from the receive earth station noise temperature (which includes the antenna temperature, the receive amplifier temperature and the noise increase resulting from feeder losses), and adding 2 dB for all other sources of noise (uplink noise, GSO interference, cross polarization isolation and frequency reuse interference).

# 3	Protection criteria

The pfd masks developed in Sections 1, 3 and 6 of Annex 1 and in Annex 4 to Appendix 30 to protect the fixed-satellite service and the broadcasting-satellite service have been determined by specifying to 6% the allowable relative noise increase (T/T) into the range of earth station antennas given in the above Table.

The allowable interfering pfd was calculated by the following expression:

pfdall(θ) = 10 log(T/T) + 10 log(kT brf) + Gm – Ga(φ)

where:

pfdall(θ):	allowable level of interfering pfd for an orbital separation of θ degrees

T/T:	allowable relative increase in receiver link noise = 6%

k:	Boltzmann's constant (1.38 × 1023 J/K)

T:	total link noise temperature (K; see Table in Section 2 above)

brf:	reference bandwidth (27 MHz in Regions 1 and 3; 24 MHz in Region 2)

Gm:	gain of a 1 m2 effective aperture (dBi/m2)

Ga(φ):	receive antenna gain for topocentric angle of φ (dBi)

φ:	topocentric angle between interfering and wanted satellites (see Annex 1 of Appendix 8 of the Radio Regulations) (degrees). It was assumed that φ = 1.1 θ.

# 4	Power flux-density to protect the fixed-satellite service and the broadcasting-satellite service with specific antenna diameters42

The Table below contains required power flux-density levels for the protection of fixed-satellite service and broadcasting-satellite service with the characteristics in Section 2 above using the criteria specified in Section 3 above.

| Required power flux-density (pfd) (dB(W/(m2 · 27 MHz))) corresponding to different antenna diameters |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Orbital separation between wanted and interfering space stations | 45 cm | 60 cm | 80 cm | 120 cm | 240 cm | 500 cm | 800 cm | 1 100 cm |
| 0° | 134.2 | 136.7 | 138.7 | –141.4 | 147.4 | 152.5 | 155.6 | –158.4 |
|  > 0 | For any value of the orbital separation  between the wanted and interfering space stations, the applicable pfd should be relaxed from the value corresponding to 0° orbital separation by adding the off-axis antenna discrimination, as calculated under the assumptions in Section 1 above. |  |  |  |  |  |  |  |

Reasons: For the protection of the BSS Plan:

The modified pfd mask should be in force just after this Conference. Therefore, administrations in Region 3, of which BSS Plan has national coverage only and has been brought into service before the end of WRC-03, are considered as not being affected under the specific conditions.

The comparison of various pfd masks is given in Fig. 1. The candidate pfd masks, which corresponds to 45 cm and 60 cm antenna as a minimum size, only differ in the range 2.0-5.0° of orbital separations, with a maximum difference of 7.2 dB for an orbital separation of 3.6° between the interfering and wanted space stations.

Figure 1

Comparison of the pfd masks for the protection of BSS and FSS

The 45 cm antenna has widely been used for the Plan assignments in some Region 3 countries. The pfd mask for the protection of the BSS Plan in Region 3, which has national coverage only and has been brought into service, should be derived by using the 45 cm antenna as a minimum antenna size. As described in Section 3.2.2.3 of the CPM Report, two kinds of masks are presented with minimum antenna size of 45 cm and 60 cm. The two masks are identical in the range of the orbital separation larger than 5°. The Region 1 and 3 Plans are based on an orbital separation of 6° with few exceptions. The pfd mask with a minimum antenna size of 45 cm therefore provides no difficulties for practical implementation of the BSS Plan assignment.

From the viewpoint of proliferation of BSS receivers to the subscribers, the 45 cm antenna provides a great benefit to viewers in some Region 3 countries as follows:

–	The 45 cm antenna is easier to set up than the 60 cm antenna. The 45 cm antenna occupies smaller space than the 60 cm antenna. The weight of the 45 cm antenna is about a half of the 60 cm antenna.

–	The 45 cm antenna is easier to purchase than the 60 cm antenna. The price of the 45 cm antenna is considerably lower than the 60 cm antenna in most case.

–	The 45 cm antenna has less influence than the 60 cm antenna from strong wind like typhoon or cyclone.

Considering the above reasons, the 45 cm antenna should be taken into account in the development of the pfd mask for Region 3 BSS Plan, which has national coverage only and has been brought into service.

For the protection of the FSS with the BSS Plan and List:

We believe that the modified pfd mask should be in force just after this Conference. Therefore, an administration in Region 3 intends to protect its FSS assignments, which have been notified to the Bureau, brought into use and the relevant information as described in the Annex to Resolution 49 (Rev.WRC-2000) have been received by the Bureau before the end of WRC-03, is considered as not being affected under the specific conditions in Section 6 above.

The following reasons should be considered:

–	Since less than 60 cm FSS receiving antennas, such as 45 cm, are widely filed to ITU and a large number of such antennas are already used in certain Region 3 countries for its domestic service, it is essential to protect these 45 cm FSS receiving antennas in these countries.

–	The current pfd masks prescribed in the Annex to Resolution 540 (WRC-2000) to protect the FSS in the 11.7-12.7 GHz band does include 45 cm FSS antenna in its scope of protection. Any significant modification (e.g. ignoring the protection of 45 cm FSS antenna) of the current pfd masks applied to FSS protection in the Annex to Resolution 540 would result in a considerably serious difficulty for the current FSS protection in certain Region 3 countries.

–	In general, any change of regulations should enter into force after the Conference which decides such change.

Therefore, taking account that the current regulation (Resolution 540, see above) does include the protection of 45 cm FSS antennas, the FSS assignments using 45 cm antennas, which have been notified to the Bureau, brought into use and the relevant information as described in the Annex to Resolution 49 (Rev.WRC-2000) have been received by the Bureau before the end of WRC-03, should be protected.

_____________

15 	For the protection of analogue assignments brought into service before 17 October 1997, the following values shall be used until 1 January 2015:

–147     dB(W/(m2  27 MHz))			for  0°              0.44°

–138 + 25 log      dB(W/(m2  27 MHz))	for  0.44°        9°.

16 	For the definition of the equivalent protection margin, see § 3.4 of Annex 5.

21 	In place of these values, the values given in the Annex to Resolution 540 (WRC-2000) shall be applied by administrations and the Bureau until this section is revised by a subsequent conference.

39 	Sections 1 and 2 of Part A of this Annex are applicable when the services of Regions 1 or 3 are involved. Section 3 of Part A is applicable to all Regions.

41 	The 45 cm diameter in the range of antennas to be protected is applied to Region 3 broadcasting-satellite service Plan which has been brought into service before 5 July 2003, and Region 3 fixed-satellite service which has been notified to the Bureau, brought into use and the relevant information as described in the Annex to Resolution 49 (Rev.WRC-2000) have been received by the Bureau before 5 July 2003.

42 	The 45 cm diameter in the range of antennas to be protected is applied to Region 3 broadcasting-satellite service Plan which has been brought into service before 5 July 2003, and Region 3 fixed-satellite service which has been notified to the Bureau, brought into use and the relevant information as described in the Annex to Resolution 49 (Rev.WRC-2000) have been received by the Bureau before 5 July 2003.