ADD	IND/108/5

5.EESS	To protect the amateur, radiolocation, fixed and mobile services, synthetic aperture radars within the Earth exploration-satellite service (active) shall operate in accordance with the technical and operational constraints contained in Recommendation ITU-R SA.1260-1. Also, administrations are urged to take all practicable steps to protect the radio astronomy service in the bands 406.1-410 MHz from harmful interference due to unwanted emissions. The threshold levels of interference detrimental to the radio astronomy service are shown in Table 1 of Recommendation ITU-R RA.769.

Reasons:	To identify the mechanism for protecting the amateur, radiolocation, fixed, mobile and radio astronomy services from interference by synthetic aperture radars in the Earth exploration-satellite service.