MOD	KOR/109A12/3

5.418A	In certain Region 3 countries listed in No. 5.418, use of the band 2 630-2 655 MHz by non-geostationary-satellite systems in the broadcasting-satellite service (sound) for which complete Appendix 4 coordination information, or notification information, has been received after 2 June 2000, is subject to the application of the provisions of No. 9.12A, in respect of geostationary-satellite networks for which complete Appendix 4 coordination information, or notification information, is considered to have been received after 2 June 2000, and No. 22.2 does not apply. No. 22.2 shall continue to apply with respect to geostationary-satellite networks for which complete Appendix 4 coordination information, or notification information, is considered to have been received before 3 June 2000. Use of the band by non-geostationary-satellite systems in the broadcasting-satellite service (sound) shall be in accordance with Resolution 528 (WARC-92).     (WRC-03)

Reasons:	Resolution 539 (WRC-539) could be suppressed at WRC-03.