ADD	KOR/109A3/11

5.WAS2	The use of the 5 250-5 350 MHz, 5 470-5 725 MHz band by the mobile service is limited to those applications described in Recommendation ITU-R M.1450 for wireless access systems, including radio local area networks, which shall be operated in accordance with the conditions described below:

–	the indoor and outdoor use of wireless access systems, including radio local area networks, shall be restricted to a mean e.i.r.p. limit of 1 W (30 dBm) and a mean e.i.r.p. density limit of 50 mW (17 dBm) in any 1 MHz;

–	wireless access system stations, including radio local area network stations, shall implement transmitter power control to ensure a mitigation factor of at least 3 dB on the average output power. If transmitter power control is not implemented, the e.i.r.p. limits given above shall be reduced by 3 dB;

–	wireless access system stations, including radio local area network stations, shall implement dynamic frequency selection techniques described in [draft new] Recommendation ITU-R M.[8A-9B.RLAN.DFS] to avoid co-channel operation with other terrestrial systems, notably radar systems. The dynamic frequency selection devices shall also be designed to ensure a near uniform spread of the loading of the devices across the available spectrum to improve the sharing with satellite services.

Reasons:	As for the use of the band 5 250-5 350 MHz, 5 470-5 725 MHz for the mobile service, the Republic of Korea considers outdoor use. Footnote 5.WAS2 can support outdoor usage and also consider frequency sharing with the existing services.