ADD	KOR/109A5/9

5.DDD 	The power flux-density produced by any geostationary space station in the fixed-satellite service (space-to-Earth) operating in the band 10.7-10.95 GHz shall not exceed at a radio astronomy station operating only in the very long baseline interferometry mode in the band 10.6-10.7 GHz:

–	–145 dB(W/m2) in any 50 kHz.

These values shall apply at any radio astronomy station that has been notified to the ITU either before the end of WRC-03 or before the date of receipt of the advance publication information of the space station to which the limits are to apply. Other radio astronomy stations, notified after these dates, may seek an agreement with administrations authorizing the space stations.

Reasons:	ITU-R studies have resulted in that in the band 10.6-10.7 GHz allocated to the RAS on a primary basis the unwanted emissions produced by space stations in the FSS (space-to-Earth) operating in the 10.7-10.95 GHz can meet the protection criteria for the VLBI observations.