INTERNATIONAL TELECOMMUNICATION UNION

Radiocommunication Bureau (Direct Fax N°. +41 22 730 57 85)

| Circular Letter CCRR/8 | 30 November 2000 |
| --- | --- |

To Administrations of Member States of the ITU

Subject:	Draft modifications to the Rules of Procedure

To the Director General

Dear Madam/Sir,

Following the conclusions of WRC-2000 (Istanbul, 2000), the Radiocommunication Bureau has identified a number of cases, which require consideration of new Rules of Procedure or, modifications to, or suppression of, existing Rules of Procedure. At its 20th Meeting 
(11 – 15 September 2000), the Radio Regulations Board (RRB) agreed to deal with these cases in subsequent meetings taking into account the workload of the RRB and the Bureau. The first set of draft Rules of Procedure, which was intended for submission to the 21st Meeting of the RRB 
(20 - 24 November 2000), was distributed for comment in Circular Letter CCRR/7 of 22 September 2000.

In this Circular Letter (CCRR/8), please find enclosed the second set of draft Rules of Procedure containing new Rules, and modifications to, and suppression of, some existing Rules. These draft Rules of Procedure are intended for consideration by the RRB at its 22nd Meeting 
(5 – 9 February 2001). The proposals relate to the following provisions:

-	Articles Nos. S5.2.1, S5.415, S5.416, and S5.491 (Annex 1).

-	Article No. S9.1 and Resolution 51 (Annex 2).

-	Articles Nos. S9.19, S9.27, S9.36, S9.58, S11.32 S11.43A and Section 1 of Appendix S5 (Annex 3).

-	Article S13 (Annex 4).

-	§3.9 of Annex 5 to Appendix S30 and §3 of Annex 3 to Appendix S30 (Annex 5).

-	§2.2 of Article 2 of Appendices S30/S30A (Annex 6).

-	Section 1 of Annex 1 to Appendix S30 and Section 4 of Annex 1 to Appendix S30A (Annex 7).

-	Annex 4 to Appendix S30 (Annex 8).

-	Annex 7 to Appendix S30 (Annex 9).

-	Section 1 of Annex 4 to Appendix S30A (Annex 10).

-	Section 2 of Annex 4 to Appendix S30A (Annex 11).

-	Notification of assignments contained in the Regions 1 and 3 Lists (Annex 12).

-	Appendix S4 (Annex 13).

In accordance with Article S13.17 of the Radio Regulations, these proposals are made available for comment by administrations before being submitted to the RRB pursuant to Article S13.14.

To enable the Radiocommunication Bureau to prepare, translate and post on the ITU website the consolidated submission to the RRB in time for the 22nd Meeting, which commences on 5 February 2001, any comments that you may wish to submit should reach the Bureau not later than 4 January 2001.

Yours faithfully,

Robert W. Jones
					Director, Radiocommunication Bureau

Annexes: 13

Distribution:

-	Administrations of Member States of the ITU
-	Members of the Radio Regulations Board
- 	Director and Heads of Department of the Radiocommunication Bureau

ANNEX 1

Rules concerning Nos. S5.2.1, S5.415, S5.416, and S5.491 of the RR

MOD S5.2.1

Several footnotes of the Table of Frequency Allocations contain a reference to “regional” or “sub-regional” uses (for example No. S5.415) with a small “r”. The Board considered such footnotes on the basis of this provision together with No. S5.22 and reached the following conclusions:

–	Where the allocation is made to one Region only, the term “sub-Regional” is interpreted in the meaning of No. S5.22, i.e. sub-Regional applies only within a Region.

–	When the allocation is made to more than one Region, the term “sub-regional” or “regional” may cover territories in different Regions.

–	A “sub-region” or “sub-Region” is not necessarily limited to bordering countries.

Reason: Following the changes performed by WRC-2000 to S5.488, the quotation of No. S5.488 does not represent a good illustrative example. Consequently, another provision is quoted for illustrative purposes.

MOD S5.415

1	In this footnote, the allocation “is limited to national and regional systems”. The Board concluded that a national system is a system having a service area limited to the territory of the notifying administration. As a consequence of this, the regional system to which reference is made shall be considered to be an aggregate of two or more national systems; they shall be limited to the territories of the administrations concerned and they shall be notified by one of these administrations on behalf of all the administrations concerned. The Board reached this conclusion keeping in mind No. S5.2.1, relating to the interpretation of the word “regional” without a capital “R”.

2	In accordance with this provision, the fixed-satellite service is limited for use by national or regional systems in the band 2 500-2 690 MHz in Region 2 and in the bands 2 500-2 535 MHz and 2 655-2 690 MHz in Region 3. Only those assignments which satisfy the following conditions shall be considered to be in conformity with the Table of Frequency Allocations:

a)	The service area for a regional system is within the Region concerned, i.e. in Region 2 only in the band 2 535-2 655 MHz or in Regions 2 and 3 in the other bands between 2 500 and 2 690 MHz.

b)	In the case of a national system, the service area is limited to the territory under the jurisdiction of the notifying administration.

c)	If the satellite network is operated within the framework of an international system to which other countries pertain, the notice must indicate that the use is limited to the Region(s) concerned.

Reason: Editorial correction.

ADD S5.416

See comments under the Rules of Procedure concerning S5.415.

Reason: To introduce a Rule for a footnote similar to S5.415.

SUP S5.491

Reason: WRC-2000 removed the limitation to “national and sub-regional systems”.

ANNEX 2

Rules concerning No. S9.1 and Resolution 51 of the RR

MOD S9.1

# 1	Postponement of the date of bringing into use

1.1	The Board understands from the reference to Nos. S11.44 and S11.44B to S11.44I in No. S9.1 in conjunction with in No. S11.48 that, for a space station of a satellite network not yet brought into use, the maximum overall period of validity of an advance publication can be five years under No. S9.1 plus a maximum of two years extension if granted. Consequently, any postponement of the date of bringing into use (beyond the original five years), at any stage of the procedure, is acceptable only if the date of bringing into use (2C date) remains within five years plus any agreed extension of up to two years following the date of receipt by the Bureau of the relevant advance information referred to in Nos. S9.1 and S9.2. The granting of the extensions within this overall seven year period is, however, subject to several other conditions which are described in the appropriate parts of the Rules of Procedure. (See comments under the Rules of Procedure concerning Nos. S9.5D, S11.44 and Resolution 51 (Rev.WRC-2000)).

1.2	The last paragraph of this provision relates to the establishment of the date of receipt for coordination and/or notification, as the case may be. The Conference decided to treat the cases of satellite networks for which the coordination procedure of Section II of Article S.9 is applicable differently from those satellite networks for which the above-mentioned coordination is not applicable. For the former the earliest possible date of receipt of a coordination request is six months after the date of receipt of the information for advance publication and for the latter, the information date of receipt of a notification is six months after the date of publication of the advance information.

The Bureau shall periodically provide the information concerning the date of receipt of the advance publication information for both types of the space network in order to apply this provision as well as Nos. S11.44 and S11.48.

# 2	Cancellation of advance publication

On the basis of the above (in particular Nos. S11.44 and S11.48) and irrespective of the regulatory status of the network (under advance publication, coordination or already recorded in the Master Register), the Bureau will, after having informed the administration concerned, cancel from the Master Register or its advance publication or coordination files those networks which are not notified as being brought into use within the above mentioned period. Administrations intending to bring these networks into use at a later date will have to re-start the procedures from the advance publication stage. In accordance to No. S11.48, the Bureau shall inform the administration responsible for the space station not later than 3 months before the expiry date of the five-year period, and 3 months before the expiry date of seven-year period, if extension is granted. (See comments under the Rules of Procedure concerning Nos. S9.5D, S11.44 and Resolution 51 (Rev.WRC-2000)).

3	The seven-year period (five years plus up to maximum two years of extension, if granted) mentioned under §1 above is not taken into account in case for the addition, at any time, of an earth station, even if it was not foreseen in the advance publication.

Reason: Editorial, to change the references to Resolution 51 (Rev.WRC-2000).

## MOD RESOLUTION 51 (Rev.WRC-2000)

Transitional arrangements relating to the advance publication
and coordination of satellite networks

Resolves

1	Resolution 51 (Rev.WRC-2000) concerns the transitional arrangements relating to the advance publication and coordination of satellite networks. In its resolves 3, Resolution 51 (Rev.WRC-2000) indicates that: “for satellite networks for which the API has been received by the Bureau prior to 22 November 1997, the maximum allowed time period from the date of publication of the API to bring the relevant frequency assignments into use shall be six years plus the extension pursuant to No. 1550 (see also Resolution 49 (Rev.WRC-2000))”.

2	In view of the above, the Board decided that:

2.1	For satellite networks for which Advance Publication Information has been received by the Bureau prior to 22 November 1997, the maximum allowed regulatory time is 9 years from the date of publication of Advance Publication.

2.2	If the characteristics of a satellite network are modified after assignments are brought into use and if new coordination agreements are required without a need to publish a new advanced publication then a total allowed regulatory time for bringing into use of any assignment for the modified characteristics should be:

2.2.1	8 1/2 years from the date of receipt of the request for publication of the modified characteristics for the subject satellite networks if the request for modification is received by the Bureau prior to 22 November 1997,

2.2.2	5 years from the date of receipt of the request for publication of the modified characteristics for the subject satellite networks if the request for modification is received by the Bureau after 22 November 1997 (see No. S11.43A).

Reason: WRC-2000 modified Resolution 51 so that, for networks for which the API was received by the Bureau before 22 November 1997, the counting of the subject period now starts from the date of publication of the API. Resolution 51 now has only one resolves.

ANNEX 3

Rules concerning Nos. S9.19, S9.27, S9.36, S9.58, S11.32 S11.43A and Section 1 of Appendix S5 of the RR

MOD S9.19

This provision relates to the requirements of coordination of transmitting terrestrial stations and transmitting earth stations in the fixed satellite service (Earth-to-space) with respect to typical BSS earth stations in the plan bands and non-planned bands. To date, there is no ITU-R Recommendation defining the power flux-density level produced by the terrestrial stations at the edge of the service area of non-planned BSS to be used for triggering the coordination. Until such time that a calculation method and technical criteria are included in the relevant ITU-R Recommendations, in applying this provision, for the identification of affected administration, the Bureau, in addition to the frequency overlap examination, also uses, on a provisional basis, the power flux density limits in the nearest frequency band(s), where available.

Reason:	Consequential to the adoption by WRC-2000 of modification to No. S9.19.

MOD S9.27

# 1	Frequency assignments to be taken into account in the coordination procedure

Frequency assignments to be taken into account in the coordination procedure are mentioned in §1 to 5 of Appendix S5 (see also Rules of Procedure concerning S9.36 and Appendix S5).

1.1 	As stated under the Rules of Procedure concerning No. S9.1 the period between the date of receipt by the Bureau of relevant information under Nos. S9.1 and S9.2 for a satellite network and the date of bringing into use of the assignments of the satellite network in question shall in no circumstance exceed five years as referred to in No. S9.1 plus any extension up to two years granted according to the procedure of Nos. S11.44B to S11.44I. Consequently, frequency assignments not complying with these time limits will no longer be taken into account under the provisions of No. S9.27 and Appendix S5. (See also Nos. S9.1, S9.2, S11.43A, S11.44, S11.48 and Resolution 49 (WRC-97/WRC-2000, as applicable))

Reason:	Consequential to the adoption by WRC-2000 of modification to Resolution 49.

Reason:	WRC-2000 adopted a new regulatory regime for Regions 1 and 3 BSS and Feederlink Plan and the List of additional uses now annexed to the MIFR. The conference also modified No. S5.43 and added S5.43A. The relations between various categories of services become relatively more clearer than before. It is, however, more difficult to reflect these relations in a tabular form with confidence that no confusion would be caused at the time of its application. In view of the above, it is suggested to delete this Table. Clarification on the status of the frequency assignment under S11.41 is covered by modifying the Rule of procedure relating to §1 of Appendix S5.

# 2	Modification of characteristics of a satellite network during coordination

2.1	After an administration informs the Bureau of a modification of characteristics of its network, it is essential to establish its proper coordination requirements with respect to other administrations, i.e. with which administration(s), and for which of their network(s), the modified part of the network needs to effect coordination before it can be notified for recording.

2.2	The guiding principles for dealing with modifications are:

–	general obligation to effect coordination before notification (No. S9.6), and

–	the fact that coordination is not required when the nature of the change is such as not to increase the interference to or from, as the case may be, the assignments of other administration, as specified in Appendix S5.

2.3	Based on these principles, and provided that the coordination trigger limit is exceeded, the modified part of the network will need to effect coordination with respect to space networks that are to be taken into account for coordination (see Table above):

a)	with dates of receipt (DR) before the original date of submission (D1) of the subject network; and

b)	with date of receipt (DR) after the original date of submission (D1) of the modified network but before the date of the modification (D2), where the nature of the change is such as to increase the interference to or from, as the case may be, the assignments of those networks which were received in the period between D1 and D2. In case of GSO networks referred to in No. S9.7 including those to which coordination arc approach (frequency bands 1), 2) and 3) of No. 9.7 of Table S5-1 of Appendix S5) have been applied, the increase of interference will be measured in terms of T/T.

2.3.1	Where the coordination requirements of the modification involve any network under b) above, the modified assignments will have as their date of receipt (DR) the date of submission of the modification (i.e. DR = D2). Otherwise, they will retain their original date of receipt (DR) (i.e. DR = D1).

2.3.2	In case of successive modifications of the same part of the network, if the next modification (compared with the previous modification) does not increase the interference to or from a particular network not included in the coordination requirements under b) above, that particular network will not be included in the coordination requirements of that next modification.

2.3.3	If it is not possible to verify that there is no increase of interference (e.g. in absence of appropriate criteria or calculation methods), the date of receipt (DR) date of the modified assignments will be D2.

2.4	After having examined the modified network as described in § 2.3 above, the Bureau shall publish the modification, including its coordination requirements, in the appropriate Special Section for comments by administrations within the usual 4-month period. Initial characteristics are thus replaced by the published modified characteristics, and only the latter will be taken into account in subsequent applications of No. S9.36.

Reason:	Consequential to the adoption by WRC-2000 of suppression of the provisions Nos. S9.8 and S9.9. As there is no standard methodology to check an increase of interference in the cases of examination under S9.7A and S9.7B (these provisions were adopted by WRC-2000), §2.3.3 will apply to those cases.

# 3	Modification to characteristics of an earth station

3.1	The use of another associated space station may be one of the modifications of characteristics to an earth station. In the case of examination under S9.15, S9.17 and S9.17A, a new coordination contour is drawn and compared with the previous one. Coordination is then required with any administration on the territory of which a coordination distance is increased. In the case of examination under S9.19, the pfd of the transmitting earth station with modified characteristics is calculated at the edge of the BSS service area. Coordination is then required with any administration on the territory of which the pfd at the edge of the BSS service area is increased as the result of modification of characteristics of the transmitting earth station in fixed satellite service and is above the permissible level. However, if the initial associated space station has been cancelled or if the coordinated frequency assignments of the earth station do not cover the newly notified assignments, this notification of the assignments of the earth station will be considered as a new notice (first notification).

3.2	Generally, the Bureau uses the same approach, i.e. an increase of the coordination distance or an increase of the pfd at the edge of the BSS service area, according to the case, in order to decide if there is an increase of interference.

Reason:	Consequential to the adoption by WRC-2000 of modification to No. S9.19 which extends the scope of application to a transmitting earth station in the fixed-satellite service (Earth-to-space) in a frequency band shared on equal primary basis with the broadcasting-satellite service. (See also MOD ROP S11.32 and S11.43A)

MOD S9.36

1	Under this provision, the Bureau “shall identify any administrations with which coordination may need to be effected”. In applying Appendix S5 with respect to No. S9.21, the Bureau uses the following calculation methods and criteria1:

–	space network vs. space network: Appendix S8;

–	earth station vs. terrestrial stations (and vice versa): Rules of Procedure B1, B2 (derived from Appendix S7);

–	transmitting terrestrial stations vs. receiving space stations: criteria of Article S21;

–	transmitting space stations vs. terrestrial services: pfd limits defined in Article S21;

–	between stations of terrestrial services in some specific frequency bands: Rules of Procedure B4, B5 and B6 as appropriate.

Reason:	Consequential to the adoption by WRC-2000 of the revised footnote S5.488.

2	For coordination requests under Nos. S9.11 to S9.14 and S9.21, it is to be noted that irrespective of the identification by the Bureau under No. S9.36 (see footnote S9.36.1), any administration, even one which was not identified, may object to the published assignment under No. S9.52 and any administration, including one identified by the Bureau, that has not commented on the proposed use within the regulatory time limit is considered to have no objection to that use in accordance with No. S9.52C.

3.	See also Rule of procedure concerning Appendix S5.

Reason:	Consequential to the draft revision of the Rules of Procedure relating to No. S9.27.

MOD S9.58

This provision refers to changes in the characteristics which have been decided during the coordination procedure of the assignment of the network. For processing of the modification, the Bureau will apply §2 of the Rules concerning No. S9.27. When publishing the modified characteristics in a modification to the Special Section containing the original coordination request, the Bureau will indicate the nature of the modification as specified in No. S9.58.

Reason:	Consequential to the draft revision of the Rules of Procedure relating to No. S9.27

MOD S11.32

# 1	Examination of a frequency assignment to a space station

The literal application of this provision would lead to the examination of the notified assignment with any station identified in application of No. S9.27 while this examination or a major part of it was already done during the application of the coordination procedure. The Board adopted a practical approach which consists of the following:

a)	Calculations with respect to networks of an administration indicated in the notice as having given its agreement to the coordination Nos. S9.7 and S9.7B are not carried out, assuming that any difference that may exist between the notified characteristics and those published in the relevant Special Section under Nos. S9.7 and S9.7B is coordinated with and accepted by this administration.

b)	If all administrations identified in the relevant Special Sections mentioned above are not included in Boxes A5/A6 without any reference to § 6 of Appendix S5 or No. S11.32A, the notice shall be sent back to the administration with an unfavourable finding with respect to No. S11.32. For practical reasons, when an unfavourable finding with respect to No. S11.32 is given at this stage, the examination under No. S11.31 shall not be performed.

(See Circular Letter No. 104 of 10 August 1998 and Rules of Procedure under No. S9.52C).

c)	In order to identify other administrations that may be affected, the notified characteristics are compared with those published in the Special Section mentioned above and, if they are identical or covered by those published in these Special Sections, the result of calculations/examination already made for these Special Sections is used.

- If the notified characteristics are different from those published, calculations are made on the basis of Appendix S5 and, if additional administrations (other than those listed in corresponding Special Sections in column A5/A6) which either receive more interference or cause more interference due to the modified characteristics than that previously received or caused are identified, an unfavourable finding shall be given and the notice form shall be returned to the notifying administration. The notifying administration would be requested to publish a modification to the Special Section in question and initiate coordination with administrations identified in that modified Special Section. If there is no additional administration which would receive more interference or cause more interference due to the modified characteristics than that previously received or caused is identified, a favourable finding shall be given. See also Rules of Procedure relating to No. S9.27.

Reason:	Consequential to the adoption by WRC-2000 of No. S9.7B and the suppression of Nos. S9.8 and S9.9.

# 2	Examination of a frequency assignment to an earth station with respect to the application of Nos. S9.7,  S9.12, S9.12A and S9.13

a)	This examination would normally involve the application of Table S5-1 of Appendix S5 to each frequency assignment of each earth station, the comparison of the results so obtained with the values corresponding to the already published or notified earth stations, and the identification of the administrations affected.

b)	It was noted that in practice, when coordinating their satellite networks, administrations usually take account of the earth stations whether their characteristics were published or not. WARC Orb-88 considered the complexity of the procedures of former Articles 11 (now S9) and 13 (now S11), mainly with respect to their application to earth stations and decided to adopt a network coordination approach. In view of the above, the Board decided that the following simplified procedure should be applied.

## 2.1	Examination of an assignment to an earth station received for the first time

The examination of frequency assignments to earth stations with respect to the application of Nos. S9.7, S9.12, S9.12A and S9.13 shall be carried out by verifying the status of the corresponding assignments to the associated space station (i.e. the satellite network).

### 2.1.1	Case where the space station’s assignments are recorded in the MIFR

a)	In the case of a space station recorded with a Favourable No. S11.32 Finding (successfully coordinated or not requiring coordination), the assignment to the associated earth station shall be assumed to have been coordinated and shall be given a Favourable No. S11.32 Finding with the following indication in Column A5/A6 of Part II-S of the BRIFIC:

–	Z/S9.7, S9.12, S9.12A and S9.13 as the case may be/--- (see Preface) followed by the names of administrations appearing in Column A5/A6 under the symbol S9.7, S9.12, S9.12A and S9.13 as the case may be/--- of the associated space station (if no administration is listed because of application of §6 of Appendix S5, only Z/S9.7, S9.12, S9.12A and S9.13 as the case may be will be indicated); and

–	numbers S9.7, S9.12, S9.12A and S9.13 as the case may be/--- followed by the names of administrations indicated in the form of Notice of the earth station, if appropriate.

b)	If, after the publication of such an assignment to an earth station in Part II of the BRIFIC, any administration objects to the Bureau’s action described in § a) above the Bureau shall examine the already recorded assignment to the earth station with respect to Nos. S9.7, S9.12, S9.12A and S9.13 by applying criteria and method prescribed in Appendix S5. As a result of this examination, the Bureau will either review or retain the Finding initially reached on the assignment in question and, in either case, will communicate its conclusions to the administration which had objected to the recording.

c)	The approach of § a) and b) above was extended to the case of a space station recorded with a Favourable No. S11.32A Finding (examination of the probability of harmful interference). The assignment of the associated earth station shall be given a Favourable No. S11.32 Finding with respect to the application of Nos. S9.7, S9.12, S9.12A and S9.13 with the appropriate indications, in Column A5/A6, as described in § a) above.

d)	In the case of an associated space station recorded with an Unfavourable Finding under No. S11.36 (operating in accordance with No. S4.4), the earth station will be given a regulatory (No. S11.31) Finding and, if applicable, a coordination conformity Finding, independent from the unfavourable regulatory Finding of the space station. The coordination conformity Finding shall nevertheless only concern its conformity with the coordination procedure under Nos. S9.15, S9.17, S9.17A, and S9.19. When recorded, a symbol describing the situation will also be added to the assignment to mean that the earth station has this status only with respect to terrestrial services and has no recognized status in the space network coordination context (Nos. S9.7, S9.12, S9.12A and S9.13.)

### 2.1.2	Case where the space station’s assignments are not recorded in the MIFR

This category may include the following cases:

a)	a space station for which the procedure of Section II of Article S9 is applicable and not yet communicated to the Bureau under Nos. S9.30 and S9.32;

b)	a space station in the process of coordination (the coordination procedure not yet completed and the space station not yet notified under No. S11.15);

c)	a space station successfully completed the procedure of Article S9 but not yet notified to the Bureau under NoS11.15;

d)	a space station notified (No. S11.15) but returned to the administration with an unfavourable finding Nos. S11.31 or S11.32 and S11.32A; and

e)	a space station already notified (No. S11.15) but not yet recorded (being processed by the Bureau).

2.1.2.1	Starting from the principle that the leading element of a space network is the space station and that it would be misleading to record in the Master Register earth stations for which a space station (network) is not recorded, the Board decided that an earth station cannot be recorded in the Master Register before its associated space station. Consequently the earth stations of categories §2.1.2 a) to d) above will be given an Unfavourable No. S11.32 Finding.

2.1.2.2	The earth station notices of category §2.1.2 e) above shall be processed by the Bureau together with the associated space station and the No. S11.32 Finding with respect to the application of Nos. S9.7, S9.12, S9.12A and S9.13 will be given in accordance with the Finding of the space station either in application of §2.1.1 a) (Favourable Finding) or 2.1.2.1 (Unfavourable Finding).

### 2.1.3	Earth stations outside the service area of the associated space station

Earth stations outside the service area of the associated space station shall be given an unfavourable No. S11.32 Finding with respect to the application of Nos. S9.7, S9.12, S9.12A and S9.13 as appropriate, assuming that the coordination of the associated space station could not have taken account of earth stations outside the service area.

## 2.2	Examination of a modification of a recorded assignment to an earth station

The modification of an assignment to an earth station may concern:

–	the modification of the orbital position of the associated space station; or

–	the replacement of the associated space station with another one; or

–	the modification of any other characteristic(s).

### 2.2.1	Modification of the orbital position of the associated space station

The modification of the orbital position of the associated space station may affect other satellite networks and may have led the administration responsible for the space station to re-apply the coordination procedure. The Bureau assumes that the concerned earth stations were taken into account in the coordination of the modification of the associated space station and consequently will apply the rules indicated in §2.1 above.

### 2.2.2	Replacement of the associated space station

The Board considers that the replacement of the associated space station results in the earth station participating in a different network. Consequently, the notification of the modification will be considered as a first notification, the notice will be modified, and the administration will be informed accordingly. The examination under No. S11.32 with respect to application of Nos. S9.7, S9.12, S9.12A and S9.13, as appropriate will be carried out as indicated in §2.1 above.

### 2.2.3	Modification of other characteristics

The Board assumes that the modified characteristics of concerned earth stations were also taken into account in the coordination of the associated space station and consequently will apply the Rules indicated in §2.1 above.

## 2.3	Cancellation of the space station’s assignment

If the space station’s assignment is cancelled by the notifying administration, the Bureau shall review the earth station(s) associated with that space station and in accordance with No. S13.13 suggest to the notifying administration to either cancel or suitably modify the basic characteristics of the entry.

Reason:	Consequential to the adoption by WRC-2000 of No. S9.7B, S9.12A and the suppression of Nos. S9.8 and S9.9. The examination of frequency assignments to earth stations with respect to the application of No. S9.7A should be carried out separately. Editorial changes due to the name and format change of the former weekly circular are included.

# 3	Examination of a frequency assignment to an earth station with respect to the application of Nos. S9.15, S9.17, S9.17A and S9.19

See comments under the Rules of Procedure concerning No. S9.27 (§3.1 and 3.2).

# 4	Examination of frequency assignment notices to stations in terrestrial services in the bands shared with equal rights with space service

4.1	In its examinations of a frequency assignment notice for a station of a terrestrial service, in the bands shared with equal rights with space services, from the point of view of its conformity with the procedures relating to coordination with respect to earth stations of other administrations, the Bureau takes into account those earth stations which are recorded in the Master Register.

If, within a period of three years following the date of notification of the terrestrial station, the Bureau receives a comment from another administration, indicating that the concerned assignment was included in a coordination procedure initiated by this later administration pursuant to No. S9.29 in respect to its earth station(s) coordination under No. S9.15 or S9.17, and was not agreed to, or was agreed with different technical characteristics, the Bureau will review the situation in accordance with the relevant provisions of Article S14 and will proceed accordingly.

# 5	Examination of frequency assignment notices to earth stations operating in opposite direction of transmission

5.1	In its examinations of a frequency assignment notice to an earth operating in opposite direction of transmission, from the point of view of its conformity with the procedures relating to coordination with respect to earth stations of other administrations, the Bureau takes into account those earth stations which are recorded in the Master Register.

If, within a period of three years following the date of notification2 of the earth station operating in the opposite direction of transmission, the Bureau receives a comment from another administration, indicating that the concerned assignment was included in a coordination procedure initiated by this later administration pursuant to No. S9.29 in respect to its earth station(s) coordination under No. S9.17A, and was not agreed to, or was agreed with different technical characteristics, the Bureau will review the situation in accordance with the relevant provisions of Article S14 and will proceed accordingly.

Reason:	Consequential to the adoption by WRC-2000 of modification to No. S9.19. Now relevant parts of No. S9.19 relate to the coordination of earth station in the fixed satellite service.

Due to the backlog of treatment of notifications of earth and terrestrial stations, three years period for comments should be counted from the date of publication of notification data.

# 6	Examination of frequency assignments to an inter-satellite link of a geostationary space station communicating with a non-geostationary space station

6.1	The Board has noted the requirement to consider a calculation method for the Radiocommunication Bureau to examine an inter-satellite link of a geostationary space station communicating with a non-geostationary space station in frequency bands allocated to the inter-satellite service with respect to No. S11.32.

6.2	Taking into account the discussion during WRC-2000 and the current lack of criteria, calculation method and associated tools to proceed with such examination, until such time as the relevant ITU-R Study Groups provide/establish necessary criteria and a required calculation method which could be adopted by the Board, the Bureau when examining the above-mentioned cases in frequency bands allocated to the inter-satellite service with respect to their conformity to No. S11.32, shall proceed as follows:

6.2.1	To give a favourable regulatory finding with respect to S11.32 (symbol "A" in Column 13A2).

6.2.2	To insert a symbol "K" in Column 13B2 with the following text:

"K"	: this frequency assignment to an inter-satellite link of a geostationary space station communicating with a non-geostationary space station is not taken into account by the Bureau in its examination under S11.32.

6.3	See also the Rules of Procedure concerning S5.392.

Reason:	WRC-2000 considered the Bureau’s report (§2.3.1 of Doc. 16) and a contribution from administration on the issue of examination of inter-satellite link under S11.32 and accepted the current practice (no establishment of coordination requirement until completion of the studies by relevant ITU-R Study Groups) for a link of a geostationary station communicating with non-geostationay stations (see Minutes of Committee 4 (Doc. 532)) . Study Group 4 is currently undertaking the studies.

MOD S11.43A

1	Modification of a space network may take place during the coordination process; this case is covered in the comments under the Rules of Procedure concerning Nos. S9.27 (§3), S9.58, S11.28 and S11.32.

2	With respect to applicable procedures for cases of modifications to assignments to satellite networks which are recorded in the Master Register, WARC Orb-88 decided that, in the case of geostationary satellite networks, any modification to the basic characteristics of an assignment, in the application of No. S11.43A (former RR1548), should be subject only to the coordination procedure (Section II of Article S9). On the basis of this decision, the Bureau does not require an administration to recommence the advance publication procedure, for a modification of a frequency assignment recorded in the Master Register unless the modification concerns the addition of a new frequency band which was not included in the advance publication of the network or a change of orbital location by more than 12 (see also the Rule under S9.2).

The purpose of the examination under No. S11.43A is to determine whether the coordination requirements remained unchanged or, where appropriate, whether the probability of harmful interference has not increased (See also the Rules of Procedure concerning Nos. S11.28 and S11.32). In these cases, the provisions of No. S11.43B apply with the effect of maintaining unchanged the status (Findings) and the date of receipt of the assignment. If, due to the modifications, new coordination requirements are identified by comparing the level of interference (such as T/T) resulted from consideration of the initial characteristics and that of modified characteristics, then unfavourable finding shall be given and the form of notice shall be returned to the notifying administration. The notifying administration should be requested to apply Section II of Article S9. Findings with respect to No. S11.32 are determined on the basis of the coordination agreements effected to meet the new coordination requirements. In the case, where the provisions of Nos. S11.32A and S11.33 are applicable and the examinations show an increase in the probability of harmful interference compared with that which resulted from the initial examination, then the finding is unfavourable and the notice shall be returned in accordance with provision No. S11.38. See also Rules of Procedure under No. S11.43B.

3	Modification of an earth station by changing the associated space station or the associated beam so far as No. S11.32 is concerned is covered in the comments under the Rules of Procedure concerning No. S11.32 in §2.2.2 and 2.2.3.

4	When the modification of a frequency assignment to an earth station is examined in application of Nos. S9.15, S9.17 and S9.17A, the coordination distance is calculated in each azimuth and the coordination under Nos. S9.15, S9.17 and S9.17A is required only with those countries on whose territory the coordination distance is increased owing to the modification. (See comments under the Rules of Procedure concerning No. S9.27 (§ 3.1 and 3.2.)

5	When the modification of a frequency assignment is examined in application of No. S9.19, the pfd of the transmitting station (terrestrial station or FSS earth station) with modified characteristics is calculated at the edge of the BSS service area and the coordination under No. S9.19 is required only with those countries where the pfd limit at the edge of the BSS service area is increased as the result of modification of characteristics of the transmitting station and is above the permissible level. (See comments under the Rules of Procedure concerning No. S9.27 (§3.1 and 3.2.).

Reason:	Consequential to the adoption by WRC-2000 of modification to Nos. S9.2 and S9.19. (See also MOD ROP S9.2 which was adopted in the 21st meeting of the RRB and MOD ROP S11.27 and S11.32)

MOD Rules concerning Appendix S5 to the RR

1

In addition to the frequency assignment listed in a) to g), the frequency assignment recorded in Master Register with an unfavourable Finding under No. S11.32 and an unfavourable Finding under No. S11.32A without having caused harmful interference, in application of No. S11.41, shall be taken into account.

See also Rules of Procedure relating to Nos. S9.27, S9.29, S9.31 and S11.32.

Reason:	Consequential to the proposed deletion of the table in Rule of Procedure relating to S9.27.

ANNEX 4

Rules concerning Article S13 of the RR

MOD Rules concerning Article S13 of the RR

In reviewing Sections III and IV of Article S13, the Radio Regulations Board noted that modifications were introduced by WRC-97 particularly in relation to the process of considering proposed changes or additions to Rules of Procedure and the opportunity available to administrations to comment on such proposals.

Nos. S13.14 and S13.15 in Section III establish procedures for changes to the Rules of Procedure and a sequence for Board consideration, publication, comment by administrations and possible further review or special study. On the other hand, No. S13.17 in Section IV also refers to preparation of draft modifications or additions to Rules of Procedure.

The Board has concluded that there is a lack of clarity in the procedures to be followed for modifications or additions to Rules of Procedure. It also had regard to the desirability for transparency in considering such proposed modifications or additions.

Accordingly, the Board decided that the following procedures should be followed with respect to the application of Nos. S13.14, S13.15 and S13.17:

a)	Proposals for changes or additions to the Rules of Procedure can emerge from administrations, from the Radiocommunication Bureau, or from the Board itself. Irrespective of the source of proposals, the Board regards No. S13.17 as requiring that the Bureau should prepare draft modifications or additions to the Rules of Procedure arising from such proposals. In the interests of transparency, the Board considers that such drafts should then be made available for a period of at least four weeks for comment by administrations.

b)	The Bureau, in accordance with No. S13.14, shall submit to the Board the final drafts of all proposed changes to the Rules of Procedure, as well as the comments received in response to the procedure in § a) above.

c)	Any need pursuant to No. S13.15, for a special study in relation to the Rules of Procedure submitted by an administration or identified by the Board or the Bureau, or the need for any new Rules or modification or addition to the existing Rules of Procedure shall be handled in accordance with the procedure in § a) and b) above.

Reason:	Rules of Procedure relating to Article S13 was brought to the attention of WRC-2000 and no objection was made to it during the Conference. WRC-2000 decided to maintain the current provisions S13.14, S13.15 and S13.17.

ANNEX 5

Deletion of current Rules of Procedure relating to §3.9 of Annex 5 to Appendix S30 and §3 of Annex 3 to Appendix S30A

1.	Current Rules of Procedure relating to §3.9 of Annex 5 to Appendix S30 and §3 of Annex 3 to Appendix S30 stipulate that:

Quote:

“Frequency assignments in the guardbands of Plans are only subject to publication in Special Sections APS30/A/… and APS30A/A/… (Advance publication, emphasis is added) and no other publication or technical examination shall be effected by the Bureau.”

Unquote.

2.	WRC-2000 added §2.2 to Article 2 of Appendices S30/S30A which stipulate that:

For APS30

Quote:

“The use of the guardbands of the Plans in this Appendix, as defined in §3.9 of Annex 3 to this Appendix, to provide space operations functions in accordance with No. S1.23 in support of the operation of GSO BSS networks, shall be coordinated with the assignments subject to these Plans using provisions of Article 7 of this Appendix. Coordination among assignments intended to provide these functions and services not subject to a Plan shall be effected using the provisions of No. S9.7 and the associated provisions of Articles S9 and S11. Coordination of modifications to the Region 2 Plan or assignments to be included in the Regions 1 and 3 List with assignments intended to provide these functions shall be effected using § 4.1.1 e), § 4.2.3 e) or § 4.2.3 f) as appropriate, of Article 4 of this Appendix.”

Unquote.

For APS30A

Quote:

“The use of the guardbands of the Plans in this Appendix, as defined in §3.1 and §4.1 of Annex 3 to this Appendix, to provide space operations functions in accordance with No. S1.23 in support of the operation of GSO BSS networks, shall be coordinated with the assignments subject to these Plans using provisions of Article 7 of this Appendix. Coordination among assignments intended to provide these functions and services not subject to a Plan shall be effected using the provisions of No. S9.7 and the associated provisions of Articles S9 and S11. Coordination of modifications to the Region 2 Plan or assignments to be included in the Regions 1 and 3 List with assignments intended to provide these functions shall be effected using §4.1.1 d) of Article 4 of this Appendix.”

Unquote.

3.	This means that, the use of the guardbands of Appendices S30/S30A for space operations functions in support of GSO BSS networks is now subject to various coordination procedures as prescribed in the above paragraphs.

4.	In light of the above and considering that the current Rules were adopted because at that time no coordination procedures were established for the space operation functions in support of GSO BSS networks, it is proposed to delete the current Rules of Procedure relating to §3.9 of Annex 5 to Appendix S30 and §3 of Annex 3 to Appendix S30A.

5.	A draft proposal for deletion of the above mentioned Rules of Procedure is presented in Appendix 1 to this Annex 5.

Appendix:	1

APPENDIX 1 TO ANNEX 5

Deletion of current Rules of Procedure relating to §3.9 of Annex 5 to Appendix S30 and §3 of Annex 3 to Appendix S30A

The Board, in reviewing the subject matter, decided to:

Delete:

3.9

The provisions of §3.9 of Annex 5 allow the use of the guard bands at the lower and upper edges of the planned bands for transmissions in the space operation service. No procedure is however defined for these transmissions. Consequently, the Board decided that frequency assignments in the guard bands of the Plans are only subject to publications in a Special Section APS30/A/... and no other publication or technical examination shall be effected by the Bureau.

Delete:

3

Procedures in the guardbands

The provisions of §3.1 (Regions 1 and 3) and 4.1 (Region 2) of Annex 3 make it possible to use the guard bands at the lower and upper edges of the planned bands for transmissions in the space operation service. No coordination procedure is, however, established by Appendix S30A for protection of these transmissions.

Consequently, the Board decided that frequency assignments in the guard bands of the Plans are only subject to publications in a Special Section APS30A/A/... and no other publication or technical examination shall be effected by the Bureau.

Reasons:

1.	In accordance with WRC-2000 decisions, the use of the guardbands of Appendices S30 and S30A to provide space operations functions in support of GSO BSS networks, according to §2.2 of Article 2 of these Appendices is now subject to 3 types of coordination.

a.	With other assignments intended to provide space operations functions in support of GSO BSS and/or associated Feeder link networks of Appendices S30 and S30A, as well as with the services not subject to a Plan, using the provisions of S9.7.

b.	With the assignments subject to Appendices S30/S30A as contained in the Plan(s), the List(s), those for which there is a request for modification to or inclusion of new assignments in the Regions 1 and 3 List, and requests for modification of the Region 2 Plan, using the procedure of Article 7 of the Appendices.

c.	With respect to modifications to the Region 2 Plan or modification to and/or inclusion of new assignments to be included in the Regions 1 and 3 List, using §4.1.1.e, §4.2.3.e and §4.2.3.f of Article 4 of Appendix S30 and §4.1.1.d of Article 4 of Appendix S30A.

2.	This means that, the use of the guardbands of Appendices S30/S30A for space operations functions in support of GSO BSS networks is now subject to various coordination procedures as prescribed in the above paragraphs.

3.	The current Rules, which were adopted in the period where there were no established coordination procedures for the space operation functions, need to be deleted as such procedures are now established in Article 2 of both Appendices.

ANNEX 6

Protection Criteria and Calculation Methods to be used for the implementation of §2.2 of Article 2 to Appendices S30/S30A

1.	WRC-2000 included in the new §2.2 of Article 2 to both Appendices S30 and S30A the regulatory provisions to coordinate assignments intended to provide space operation functions in the guardbands of the Appendices S30/S30A frequency bands with other services using the same bands. However, WRC-2000 did not refer explicitly to the protection criteria and calculation methods, which shall be applied to implement these new provisions.

2.	At its last meeting (Geneva, 13-26 September 2000) ITU-R Working Party 6S considered the applicable protection criteria and calculation methods for the coordination of assignments intended to provide space operation functions in the guardbands of the Appendices S30/S30A frequency bands, and advised that, in the absence of further study and until Study Group 6 concludes on the issue, the protection criteria and calculation methods can be implicitly found in the various coordination procedures referred to in §2.2 of Article 2 to Appendices S30 and S30A, as revised by WRC-2000.

3.	Consequently, in order to avoid any future misunderstanding and to allow the Bureau to practically implement §2.2 of Article 2 to Appendices S30/S30A (WRC-2000), a draft new Rule of Procedure relating to this paragraph is presented in Appendix 1 to this Annex 6.

Appendix:	1

APPENDIX 1 TO ANNEX 6

Protection Criteria and Calculation Methods to be used for the implementation of §2.2 to Article 2 of Appendices S30/S30A

1.	WRC-2000 included in the new §2.2 of Article 2 to both Appendices S30 and S30A the regulatory provisions to coordinate assignments intended to provide space operation functions in the guardbands of the Appendices S30/S30A frequency bands with other services using the same bands. However, WRC-2000 did not refer explicitly to the protection criteria and calculation methods, which shall be applied to implement these new provisions.

2.	At its meeting in Geneva (13-26 September 2000) ITU-R Working Party 6S considered the applicable protection criteria and calculation methods for the coordination of assignments intended to provide space operation functions in the guardbands of the Appendices S30/S30A frequency bands, and advised that, in the absence of further study and until Study Group 6 concludes on the issue, the protection criteria and calculation methods can be implicitly found in the various coordination procedures referred to in §2.2 of Article 2 to Appendices S30 and S30A, as revised by WRC-2000.

3.	Consequently, in order to avoid any future misunderstanding and to allow the Bureau to practically implement §2.2 of Article 2 to Appendices S30/S30A (WRC-2000), the Board, in reviewing the subject matter in conjunction with the practices currently used by the Bureau, decided to instruct the Bureau to apply the following course of action:

In the absence of further study and until Study Group 6 concludes on the issue, the protection criteria and calculation methods associated with the provisions referred to in §2.2 of Article 2 to Appendices S30 and S30A, as adopted by WRC-2000, shall be used by the Bureau.

ANNEX 7

Review of the Rules of Procedure relating to Section 1 of Annex 1 to Appendix S30 and to Section 4 of Annex 1 to Appendix S30A

1.	WRC-2000 revised Section 1 of Annex 1 to Appendix S30 and Section 4 of Annex 1 to Appendix S30A and adopted new protection criteria and methods associated with §4.1.1 a) and §4.1.1 b) of Article 4 of Appendices S30/S30A (WRC-2000) for the coordination of proposed new or modified assignments to the Regions 1 & 3 Lists with those already contained in the Regions 1 & 3 Plans or in the Regions 1 & 3 Lists, as well as with other proposed new or modified assignments to the Regions 1 & 3 Lists.

2.	In doing so, WRC-2000 introduced new protection criteria and methods, which are defined by a combination of PFD limits and an Equivalent Protection Margin (EPM) degradation criterion. In the former Appendices S30 and S30A, only EPM degradation criterion was used.

3.	In preparing the implementation of these new protection criteria and methods, the Bureau identified that further clarifications or complementary information were required and, to that aim, presented a document to the last ITU-R Working Party 6S meeting (Geneva, 13-26 September 2000).

4.	Taking into account the views expressed by Working Party 6S, the Bureau presented a revised version of this document for information to the 21st RRB meeting (Geneva, 20-24 November 2000).

5.	In light of the above and considering that the current Rules of Procedure relating to former Section 1 of Annex 1 to Appendix S30 and former Section 4 of Annex 1 to Appendix S30A were drafted under different circumstances, and that these Sections were revised by WRC-2000, draft revised Rules of Procedure relating to these Sections are proposed in Appendix 1 to this Annex 7.

6.	Current Rules of Procedure relating to Section 1 of Annex 1 to Appendix S30 and to Section 4 of Annex 1 to Appendix S30A are also provided in Appendix 2 to this Annex 7 for easy reference.

7.	The draft revised rules proposed in Appendix 1 to this Annex 7 should be brought to the attention of the Special Committee on Regulatory and Procedural Matter (SC), CPM-02 and WRC-03 for a possible revision of Section 1 of Annex 1 to Appendix S30 and the former Section 4 of Annex 1 to Appendix S30A at WRC-03, as appropriate.

Appendices:	2

APPENDIX 1 TO ANNEX 7

Draft revised Rules of Procedure relating to Section 1 of Annex 1 to Appendix S30 (WRC-2000) and Section 4 of Annex 1 to Appendix S30A (WRC-2000)

The Board, in reviewing the subject matter, decided to revise the current Rules of Procedure relating to former Section 1 of Annex 1 to Appendix S30 and former Section 4 of Annex 1 to Appendix S30A, as follows.

# 1.	Rules concerning Appendix S30 (WRC-2000) to the RR

An. 1

## Limits for determining whether a service of an administration is
affected by proposed modifications to the Plan

1

a)	Test points

1.	In examining a proposed modification, all test points communicated to the Bureau by administrations are used. These test points are periodically published by the Bureau together with the updated reference situation of the Plan(s) and List(s).

b)	Implementation of the PFD Limit referred to in the first paragraph of Section 1 of Annex 1 to Appendix S30

1.	The PFD limit of –103.6 dB(W/(m2/27 MHz)) which is indicated in the first paragraph of Section 1 of Annex 1 to Appendix S30 was established in order to protect BSS assignments from interference that may be caused by BSS networks located outside an arc of +/- 9° around a wanted BSS network, under worst-case station-keeping conditions. Therefore, it appears that this PFD limit was intended to be considered as a hard-limit that shall not be exceeded.

2.	In order for the Bureau to practically implement this provision in a reasonable time period, i.e. without having to capture and process the relevant Appendix S4 data, which is currently done several months after the data submission, the Board concluded that the PFD limit of -103.6 dB(W/(m2/27 MHz)) could be converted into two e.i.r.p. limits as follows:

2.1	“First e.i.r.p. limit”:

An e.i.r.p. value of 58.4 dBW, which corresponds to the maximum e.i.r.p. level below which the PFD limit is never exceeded, i.e. this e.i.r.p. value corresponds to a PFD value of
-103.6 dB(W/(m2/27 MHz)) produced by a satellite pointing at its sub-satellite point (the shortest distance from the GSO to the Earth).

2.2	“Second e.i.r.p. limit”:

An e.i.r.p. value of 59.8 dBW, which corresponds to the minimum e.i.r.p. level above which the PFD limit is always exceeded, i.e. this e.i.r.p. value corresponds to a PFD value of -103.6 dB(W/(m2/27 MHz)) produced by a satellite pointing at the edge of the visible part of the Earth (the longest distance from the GSO to the earth).

3.	The Board therefore decided that the PFD limit of –103.6 dB(W/(m2/27 MHz)) shall be implemented by the Bureau by checking the e.i.r.p. value of each assignment of a given network against the e.i.r.p. limits defined in §2 above.

4.	To this aim, the Board instructed the Bureau to apply the following course of action:

4.1	If the “first e.i.r.p. limit” of 58.4 dBW is not exceeded by any assignment of a given network, the PFD limit of -103.6 dB(W/(m2/27 MHz)) would be considered to be met.

4.2	If the e.i.r.p. value of at least one assignment of a given network exceeds the “second e.i.r.p. limit” of 59.8 dBW, the Bureau shall then consult with the Administration responsible for this network in order for it to reduce this e.i.r.p. value at least below 59.8 dBW and preferably below 58.4 dBW. This consultation would have to be carried out according to the Rules of Procedure on the Receivability of forms of notice, i.e. within the 30+15 days referred to in §3.2 of these Rules.

Should the responsible Administration insist on keeping the original e.i.r.p. value(s) of the assignment(s) in question for this network, the assignment(s) would then be considered as exceeding the PFD limit referred to in the first paragraph of Section 1 of Annex 1 to Appendix S30 (i.e. -103.6 dB(W/(m2/27 MHz))), and thus not being in conformity with Article 4 of Appendix S30. The assignment(s) would then be deleted from the network and the responsible Administration would be informed accordingly.

4.3	Otherwise, if the e.i.r.p. value of at least one assignment of a given network is in the range between both above-mentioned e.i.r.p. limits (i.e. 58.4 dBW and 59.8 dBW), the Bureau should proceed further with this network and study more deeply the conformity with the PFD limit of -103.6 dB(W/(m2/27 MHz)) at the time of the other regulatory and technical examinations.

Should it be found at that time that the assignment(s) in question exceed(s) the above-mentioned PFD limit, a Note would be included in the corresponding Special Section drawing the attention of the responsible administration to the need to take necessary action at the stage of Part B publication (application of §4.1.12 of Appendix S30) to ensure that the e.i.r.p. level of the assignment(s) satisfies the PFD limit of -103.6 dB(W/(m2/27 MHz)), otherwise the assignment(s) shall be considered not to be in conformity with Article 4 of Appendix S30 (WRC-2000) and shall not be thus included in the List even if all other paragraphs of Article 4 were successfully applied.

5.	The Board noted that considering the level of e.i.r.p. of current BSS satellite networks, this PFD limit was unlikely to be exceeded and therefore the Bureau might face a limited number of cases of this nature.

c)	Implementation of the PFD masks and EPM criterion referred to in sub-paragraphs a) and b) of Section 1 of Annex 1 to Appendix S30

1.	In accordance sub-paragraphs a) and b) of Section 1 of Annex 1 to Appendix S30 (WRC-2000), an Administration, which has assignment(s) in the Plan, in the List or assignment(s) for which the procedure of Article 4 of Appendix S30 has already been initiated, is considered as affected by a proposed new or modified assignment in the List if all the following conditions are met:

–	the orbital spacing between both assignments is less than 9 degrees, under worst-case station-keeping conditions; and

–	there is a frequency overlap between the bandwidths assigned to each assignment; and

–	under assumed free-space propagation conditions, the PFD value derived from the appropriate PFD mask given in § a) of Section 1 of Annex 1 to Appendix S30 is exceeded at least at one of the test-points1 of the wanted assignment; and

–	the reference EPM of at least one of the test-points1 of that wanted assignment falls more than 0.45 dB below 0 dB, or if already negative, more than 0.45 dB below that reference EPM value.

d)	Reference protection margin 2

1.	The reference equivalent protection margin (EPM) values of:

–	the assignments in the downlink or feeder-link Plans;

–	the assignments in the downlink or feeder-link Lists;

–	the assignments for which the procedure of Article 4 of Appendices S30 or S30A has been initiated,

include the potential interference effects of the other assignments of the corresponding Plan and List, as established at WRC-2000, and those of the other assignments entered in the corresponding List after a successful application of the Article 4 procedure.

2.	The reference EPM used as the basis for comparing the effect of a proposed new or modified assignment is that periodically published by the Bureau and updated once a new or modified assignment is entered in the corresponding List after a successful application of the Article 4 procedure.

# 2.	Rules concerning Appendix S30A (WRC-2000) to the RR

An. 1

## Limits for determining whether a service of an administration is
affected by proposed modifications to the Plan

4

a)	Test points

See comments made under the Rules of Procedure concerning § a) of Section 1 of Annex 1 to Appendix S30.

b)	Implementation of the PFD Limit referred to in the first paragraph of Section 4 of Annex 1 to Appendix S30A

1.	The PFD limit of –76 dB(W/(m2/27 MHz)) which is indicated in the first paragraph of Section 4 of Annex 1 to Appendix S30A was established in order to protect BSS feeder-link assignments from interference which may be caused by BSS feeder-link networks located outside an arc of +/- 9° around the wanted BSS feeder-link network, under worst-case station-keeping conditions. Therefore, it appears that this PFD limit was intended to be considered as a hard-limit that shall not be exceeded.

2.	In order for the Bureau to practically implement this provision in a reasonable time period, i.e. without having to capture and process the relevant Appendix S4 data which is currently done several months after the data submission, the Board concluded that the PFD limit of -76 dB(W/(m2/27 MHz)) could be converted into two e.i.r.p. limits as follows:

2.1	“First e.i.r.p. limit”:

An e.i.r.p. value of 86 dBW which corresponds to the maximum e.i.r.p. level below which the PFD limit is never exceeded, i.e. this e.i.r.p. value corresponds to a PFD value of -76 dB(W/(m2/27 MHz)) produced by a transmitting earth station located at the sub-satellite point (the shortest distance from the Earth to the GSO).

2.2	“Second e.i.r.p. limit”:

An e.i.r.p. value of 87.4 dBW which corresponds to the minimum e.i.r.p. level above which the PFD limit is always exceeded, i.e. this e.i.r.p. value corresponds to a PFD value of -76 dB(W/(m2/27 MHz)) produced by a transmitting earth station located at the edge of the visible part of the Earth (the longest distance from the Earth to GSO the).

3.	The Board therefore decided that the PFD limit of –76 dB(W/(m2/27 MHz)) shall be implemented by the Bureau by checking the e.i.r.p. value of each assignment of a given network against the e.i.r.p. limits defined in §2 above, together with the compliance of the relative off-axis e.i.r.p of the associated feeder-link antenna with Figure A (WRC-97 curves) of Annex 3 to Appendix S30A.

4.	To this aim, the Board further instructed the Bureau to apply the following course of action:

4.1	If the “first e.i.r.p. limit” of 86 dBW is not exceeded by any assignment of a given network and if the relative off-axis e.i.r.p of the associated feeder-link antenna is in compliance with Figure A (WRC-97 curves) of Annex 3 to Appendix S30A, the PFD limit of -76 dB(W/(m2/27 MHz)) would be considered to be met.

4.2	If the e.i.r.p. value of at least one assignment of a given network exceeds the “second e.i.r.p. limit” of 87.4 dBW or if the relative off-axis e.i.r.p of the associated feeder-link antenna is not in compliance with Figure A (WRC-97 curves) of Annex 3 to Appendix S30A, the Bureau shall then consult with the Administration responsible for this network in order for it to reduce this e.i.r.p. value at least below 87.4 dBW and preferably below 86 dBW, and/or to ensure that the relative off-axis e.i.r.p. of the associated feeder-link antenna is in conformity with Figure A (WRC-97 curves) of Annex 3 to Appendix S30A. This consultation would have to be carried out according to the Rules of Procedure on the Receivability of forms of notice, i.e. within the 30 + 15 days referred to in §3.2 of these Rules.

Should the responsible Administration insist on keeping the original characteristics of the assignment(s) in question for this network, the assignment(s) would then be considered as being not in conformity the first paragraph of Section 4 of Annex 1 to Appendix S30A, and thus not in conformity with Article 4 of Appendix S30A. The assignment(s) would then be deleted from the network and the responsible Administration would be informed accordingly.

4.3	Otherwise, if the e.i.r.p. value of at least one assignment of a given network is in the range between both above-mentioned e.i.r.p. limits (i.e. 86 dBW and 87.4 dBW) and if the relative off-axis e.i.r.p. of the associated feeder-link antenna is in conformity with Figure A (WRC-97 curves) of Annex 3 to Appendix S30A, the Bureau should proceed further with this network and study more deeply the conformity with the PFD limit of -76 dB(W/(m2/27 MHz)) at the time of the other regulatory and technical examinations.

Should it be found at that time that the assignment(s) in question exceeds the above-mentioned PFD limit, a Note would be included in the corresponding Special Section drawing the attention of the responsible administration to the need to take necessary action at the stage of Part B publication (application of §4.1.12 of Appendix S30A) to ensure that the e.i.r.p. level of the assignment(s) satisfies the PFD limit of -76 dB(W/(m2/27 MHz)) and that the relative off-axis e.i.r.p of the associated feeder-link antenna is in compliance with Figure A (WRC-97 curves) of Annex 3 to Appendix S30A, otherwise the assignment(s) shall be considered not to be in conformity with of Article 4 of Appendix S30A (WRC-2000) and shall not be thus included in the List even if all other paragraphs of Article 4 were successfully applied.

5.	The Board noted that considering the level of feeder-link e.i.r.p. of current BSS satellite networks, this PFD limit was unlikely to be exceeded and therefore the Bureau might face a limited number of cases of this nature.

c)	Implementation of the EPM degradation criterion referred to in the third paragraph of Section 4 of Annex 1 to Appendix S30A

1.	In accordance the third paragraph of Section 4 of Annex 1 to Appendix S30A (WRC-2000), an Administration, which has assignment(s) in the 14 or 17 GHz Plan, in the 14 or 17 GHz List or assignment(s) for which the procedure of Article 4 of Appendix S30A has already been initiated, is considered as affected by a proposed new or modified assignment in the 14 or 17 GHz List if all the following conditions are met:

–	the orbital spacing between both assignments is less than 9 degrees, under worst-case station-keeping conditions; and

–	there is a frequency overlap between the bandwidths assigned to each assignment; and

–	the reference EPM of at least one of the test-points3 of that wanted assignment falls more than 0.45 dB below 0 dB, or if already negative, more than 0.45 dB below that reference EPM value.

d)	Reference protection margin

See comments made under the Rules of Procedure concerning § d) of Section 1 of Annex 1 to Appendix S30.

APPENDIX 2 TO ANNEX 7

Current Rules of Procedure relating to former Section 1 of Annex 1 to Appendix S30 and former Section 4 of Annex 1 to Appendix S30A

# 1.	Rules concerning former Appendix S30 to the RR

An. 1

## Limits for determining whether a service of an administration is
affected by proposed modifications to the Plan

1

a)	Test points

In examining a proposed modification all test points communicated to the Bureau by administrations are used. These test points are periodically published by the Bureau together with the reference situation of the updated Plan.

b)	Reference protection margin 4

The reference equivalent protection margin (EPM) used as the basis for comparing the effect of a proposed modification is that periodically published by the Bureau and updated once a modification to the Plan is successfully applied. In making these examinations to determine which administrations might be affected, the assignments considered are:

–	all assignments in the Plan at the time of the Conference;

–	all modifications that successfully applied the procedure of Article 4 or those which are in process under Article 4, i.e. for which the complete Annex 2 data have been received.

If the calculations for a proposed modification show that the equivalent protection margin (EPM) of any assignment which is currently 0 dB or negative, decreases by more than 0.25 dB5 then that administration is identified as possibly affected.

In the application of this section the cumulative effect of the proposed modification of all assignments in the Plan is calculated and compared as follows:

i)	For an assignment entered in the Plan by the Conference:

The cumulative effect is compared with the reference situation on the date of entry into force of the Plan, such as modified after the administration responsible for this assignment gave its coordination agreement to another assignment which was, as a consequence of this acceptance, modified or entered in the Plan.

ii)	For an assignment entered in the Plan by the Conference and modified by the administration:

The cumulative effect is compared with the reference situation on the date of modification, such as modified after the administration responsible for this assignment gave its coordination agreement to another assignment which was, as a consequence of this acceptance, modified or entered in the Plan.

iii)	For a new assignment entered in the Plan in application of Article 4:

The cumulative effect is compared with the reference situation on the date of entry of the new assignment in the Plan, such as modified after the administration responsible for this assignment gave its coordination agreement to another assignment which was, as a consequence of this acceptance, modified or entered in the Plan.

# 2.	Rules concerning former Appendix S30A to the RR

An. 1

## Limits for determining whether a service of an administration is
affected by proposed modifications to the Plan

4

See comments made under the Rules of Procedure concerning §1 of Annex 1 to Appendix S30.

ANNEX 8

Draft new Rule of Procedure relating to Annex 4 to Appendix S30 to align its title with the provisions contained in the new Article 7 and to clarify its application

1.	WRC-2000 adopted new Regions 1 and 3 downlink and feeder-link Plans and Lists, as defined in Articles 11 and 9A of Appendices S30 and S30A respectively and in Resolution 542 (WRC-2000).

2.	This Conference also revised the regulatory procedures associated with these Regions 1 and 3 Plans and Lists and established new protection criteria and methods for sharing between services in the frequency bands of Appendices S30 and S30A.

3.	Alignment of the title of Annex 4 to Appendix S30

3.1	In reviewing the implementation of the regulatory procedures of these Appendices, the Bureau has identified that the title of Annex 4 to Appendix S30 (WRC-2000) requires an alignment with the provisions contained in the new Article 7 to Appendix S30 (WRC-2000).

3.2	According to the new Article 7 to Appendix S30 (WRC-2000), frequency assignments to transmitting space stations in the FSS and in the BSS not subject to a Plan or the List, for which the corresponding request for coordination was received by the Bureau as from 3 June 2000, shall protect, using the criteria given in Annex 4 to Appendix S30, frequency assignments in conformity with the appropriate Regional Plan, frequency assignments included in the Regions 1 and 3 List and also those for which the procedure of Article 4 of Appendix S30 has been initiated as from 3 June 2000, in accordance with the date of receipt of the complete Appendix S4 information.

3.3	The current title of Annex 4 to Appendix S30 (WRC-2000) only refers to assignments in the Regional Plans. Therefore, in order to avoid any future misunderstanding in its application, it is proposed to include in the title of Annex 4 to Appendix S30 (WRC-2000) a reference to assignments in the Regions 1 and 3 List as well as to Article 4 assignments in all 3 Regions, thus aligning it with the provisions contained in the new Article 7 to Appendix S30 (WRC-2000).

3.4	In addition, the Bureau noted that this Annex does not contain a reference to the protection of Region 1 BSS assignments from Region 3 FSS assignments in the band 12.2-12.5 GHz. Therefore it is proposed to include in the title of Annex 4 to Appendix S30 (WRC-2000) a reference to this missing sharing situation.

4.	Clarification on the implementation of Annex 4 to Appendix S30

4.1	In reviewing the implementation of the protection criteria and methods of Appendices S30 and S30A, the Bureau has identified that the text of Annex 4 to Appendix S30 (WRC-2000) requires further clarification or complementary information with regard to the implementation of the corresponding protection criteria and method for sharing between the services involved.

4.2	Annex 4 to Appendix S30 contains the PFD mask applicable to protect BSS receiving earth stations subject to a Plan or the List from FSS or BSS transmitting space stations not subject to a Plan nor the List. This PFD mask was not discussed thus not reviewed at WRC-2000.

4.3	The method described in this Annex refers to the calculation of “the power flux-density on the territory of an administration”. However, a BSS assignment in a Plan, in the List or for which the procedure of Article 4 of Appendices S30 has been initiated needs to be protected on the basis of its service area.

4.4	In addition to that and in order to allow the Bureau to practically verify the compliance with this PFD mask over the service area of the BSS assignments, the following course of action is proposed:

    - When the service area of the BSS assignment is defined by a contour, the Bureau will apply the same methodology as the one used for FSS systems (see the Rule of Procedure AP30 / former §4.3.1.5, §2 b) and §3 b));

    - In the absence of available service area contour, the PFD will be checked at each of the BSS test points associated with the service area.

5.	It should be noted that this document includes the views expressed by ITU-R Working Party 6S, which considered the applicable protection criteria and methods for sharing between services at its last meeting (Geneva, 13-26 September 2000), and agreed with this proposed course of actions.

6.	As mentioned in 3.4 above, the Bureau noted that this Annex does not contain a reference to the protection of Region 1 BSS assignments from Region 3 FSS assignments in the band 12.2-12.5 GHz. Therefore it is proposed to protect Region 1 BSS assignments from Region 3 FSS assignments in the band 12.2-12.5 GHz by applying the same limits already contained in this Annex.

7.	A draft new Rule of Procedure covering the proposals presented above is contained in Appendix 1 to this Annex 8.

8.	The above-mentioned changes should be brought to the attention of Special Committee on Regulatory and Procedural Matter (SCRPM), CPM-02 and WRC-03 for a possible revision of Annex 4 to Appendix S30 at WRC-03, as appropriate.

Appendix:	1

APPENDIX 1 TO ANNEX 8

Draft new Rule of Procedure relating to Annex 4 to Appendix S30 to align its title with the provisions contained in the new Article 7 and to clarify its application

An. 4

Need for coordination of a transmitting space station in the fixed-satellite service or in the broadcasting-satellite service where this service is not subject to a Plan: 
in Region 2 (11.7-12.2 GHz) with respect to the Regions 1 and 3 Plan, 
 in Region 1 (12.5-12.7 GHz) and in Region 3 (12.2-12.7 GHz) with 
respect to the Region 2 Plan

(See Article 7)

Alignment of the title of Annex 4 to Appendix S30

1.	The Board, in reviewing the implementation of the regulatory procedures of Appendix S30, has identified that the title of Annex 4 to Appendix S30 (WRC-2000) requires an alignment with the provisions contained in the new Article 7 to Appendix S30 (WRC-2000).

2.	According to the new Article 7 to Appendix S30 (WRC-2000), frequency assignments to transmitting space stations in the FSS and in the BSS not subject to a Plan or the List, for which the corresponding request for coordination was received by the Bureau as from 3 June 2000, shall protect, using the criteria given in Annex 4 to Appendix S30, frequency assignments in conformity with the appropriate Regional Plan, frequency assignments included in the Regions 1 and 3 List and also those for which the procedure of Article 4 of Appendix S30 has been initiated as from 3 June 2000, in accordance with the date of receipt of the complete Appendix S4 information.

3.	The current title of Annex 4 to Appendix S30 only refers to assignments in the Regional Plans. In addition, the Board noted that this Annex does not contain a reference to the protection of Region 1 BSS assignments from Region 3 FSS assignments in the band 12.2-12.5 GHz.

4.	In view of the above and in order to avoid any future misunderstanding in its application, the Board decided on a provisional basis and until the time when this Annex is further reviewed by WRC-2003, to align the title of Annex 4 to Appendix S30 (WRC-2000) to read as follows1:

Need for coordination of a transmitting space station in the fixed-satellite service or in the broadcasting-satellite service where this service is not subject to a Plan: 
in Region 2 (11.7-12.2 GHz) with respect to the Plan, the List or proposed new or modified assignments in the List for Regions 1 and 3, 
in Region 1 (12.5-12.7 GHz) and in Region 3 (12.2-12.7 GHz) with respect to the Plan or proposed modifications to the Plan for Region 2, 
in Region 3 (12.2-12.5 GHz) with respect to the Plan, the List or proposed new or modified assignments in the List for Region 1

(See Article 7)

Clarification on the implementation of Annex 4 to Appendix S30

1.	The Board, in reviewing the implementation of the protection criteria and methods of Appendix S30, has identified that the text of Annex 4 to Appendix S30 (WRC-2000) requires further clarification or complementary information with regard to the implementation of the corresponding protection criteria and method for sharing between the services involved.

2.	Annex 4 to Appendix S30 contains the PFD mask applicable to protect BSS receiving earth stations subject to a Plan or the List from FSS or BSS transmitting space stations not subject to a Plan nor the List. This PFD mask was not discussed thus not reviewed at WRC-2000.

3.	The method described in this Annex refers to the calculation of “the power flux-density on the territory of an administration”. However, the Board is of the understanding that a BSS assignment in a Plan, in the List or for which the procedure of Article 4 of Appendices S30 has been initiated needs to be protected on the basis of its service area.

4.	The Board also noted that Annex 4 to Appendix S30 (WRC-2000) does not contain a reference to the protection of Region 1 BSS assignments from Region 3 FSS assignments in the band 12.2-12.5 GHz. Therefore the Board decided that, to protect Region 1 BSS assignments from Region 3 FSS assignments in the band 12.2-12.5 GHz, the same limits already contained in this Annex shall be applied.

5.	In addition to that and in order to allow the Bureau to practically verify the compliance with this PFD mask over the service area of the BSS assignments, the Board, in reviewing the subject matter in conjunction with the practices currently used by the Bureau, decided to instruct the Bureau to apply the following course of action:

5.1	When the service area of the BSS assignment is defined by a contour, the Bureau will apply the same methodology as the one used for the protection of FSS systems, which is described in the Rule of Procedure AP30 / former § 4.3.1.5, 2 b) and 3 b), i.e.:

An administration in Region 1 or Region 3 is identified among those whose agreement is required when, under assumed free-space propagation conditions, the power flux-density over any portion of the service area of the corresponding Region 1 or Region 3 BSS assignment resulting from the proposed Region 2 FSS assignment exceeds the limits prescribed in Annex 4 to Appendix S30.

An administration in Region 2 is identified among those whose agreement is required when, under assumed free-space propagation conditions, the power flux-density over any portion of the service area of the corresponding Region 2 BSS assignment resulting from the proposed Region 1 or Region 3 FSS assignment or proposed Region 3 BSS assignment not subject to a Plan or the List exceeds the limits prescribed in Annex 4 to Appendix S30.

An administration in Region 1 is identified among those whose agreement is required when, under assumed free-space propagation conditions, the power flux-density over any portion of the service area of the corresponding Region 1 BSS assignment resulting from the proposed Region 3 FSS assignment exceeds the limits prescribed in Annex 4 to Appendix S30.

5.2	In the absence of available service area contour of the BSS assignment, the methodology described in 5.1 above will be applied but instead of verifying the power flux-density compliance over any portion of the service area, it will be verified at each of the BSS test-points associated with the service area of the corresponding BSS assignment.

ANNEX 9

Deletion of the current Rule of Procedure and its replacement by a draft new Rule of Procedure relating to Annex 7 to Appendix S30 to improve the text in it

1.	Deletion of the current Rule of Procedure relating to Annex 7 to Appendix S30

1.1	WRC-2000 adopted new Regions 1 and 3 downlink and feeder-link Plans and Lists, as defined in Articles 11 and 9A of Appendices S30 and S30A respectively and in Resolution 542 (WRC-2000).

1.2	This Conference also revised the regulatory procedures associated with these Regions 1 and 3 Plans and Lists and established new protection criteria and methods for sharing between services in the frequency bands of Appendices S30 and S30A.

1.3	In reviewing the implementation of the regulatory procedures of these Appendices, the Bureau noted that the current Rule of Procedure relating to Annex 7 to Appendix S30 is no longer required due to the adoption of the new Annex 7 to Appendix S30 by WRC-2000.

1.4	In light of the above it is proposed to delete the current Rule of Procedure relating to Annex 7 to Appendix S30.

1.5	A draft proposal to delete the current Rule of Procedure relating to Annex 7 to Appendix S30 is contained in Appendix 1 to this Annex 9.

2.	Draft new Rule of Procedure relating to Annex 7 to Appendix S30

2.1	In reviewing the new Annex 7 to Appendix S30 (WRC-2000), the Bureau also noted that parts of the text in it requires further improvements in order to align it with current texts contained in other Articles and Annexes of Appendix S30 (WRC-2000), in particular with regard to required references to the Regions 1 and 3 Lists.

2.2	Consequently, in order to avoid any future misunderstanding in its application, it is proposed to amend the text of Annex 7 to Appendix S30 (WRC-2000), thus aligning it with other provisions contained in Appendix S30 (WRC-2000).

2.3	A draft new Rule of Procedure relating to Annex 7 of Appendix S30 (WRC-2000) is presented in Appendix 2 to this Annex 9.

3.	The above-mentioned changes should be brought to the attention of Special Committee on Regulatory and Procedural Matter (SCRPM), CPM-02 and WRC-03 for a possible revision of Annex 7 to Appendix S30 at WRC-03, as appropriate.

Appendices:	2

APPENDIX 1 TO ANNEX 9

Deletion of the current Rule of Procedure and its replacement by a draft new Rule of Procedure relating to Annex 7 to Appendix S30 to improve the text in it

In reviewing the implementation of the regulatory procedures of Appendices S30 and S30A, the Board noted that the current Rule of Procedure relating to former Annex 7 to Appendix S30 (WRC-97) is no longer required due to the adoption of the new Annex 7 to Appendix S30 by WRC-2000.

Therefore, the Board decided to delete the current Rule of Procedure relating to former Annex 7 to Appendix S30 (WRC-97)

APPENDIX 2 TO ANNEX 9

Draft new Rule of Procedure relating to Annex 7 to Appendix S30 (WRC-2000)

(to replace the deleted Rule on this Annex)

An. 7

## Orbital position limitations

1.	The Board, in reviewing the new Annex 7 to Appendix S30 (WRC-2000), noted that parts of the text in it requires further improvements in order to align it with current texts contained in other Articles and Annexes of Appendix S30 (WRC-2000), in particular with regard to required references to the Regions 1 and 3 Lists.

2.	Consequently, in order to avoid any future misunderstanding in its application, the Board decided on a provisional basis and until the time when this Annex is further reviewed by WRC-2003, that the following alignments be made in the textual part of Annex 7 to Appendix S30 (WRC-2000), as indicated below.

2.1	The Board decided to align Section A of Annex 7 to Appendix S30 (WRC-2000) to read as follows:

“In applying the procedure of Article 4 for proposed modifications to the Region 2 Plan or for proposed new or modified assignments in the Regions 1 and 3 List, administrations should observe the following criteria:”

2.2	The Board decided to align Section A3) of Annex 7 to Appendix S30 (WRC-2000) to read as follows:

“The purpose of the following orbital position and e.i.r.p. limitations is to preserve access to the GSO by the Region 2 fixed-satellite service in the band 11.7-12.2 GHz. Within the orbital arc of the GSO between 37.2 W and 10° E, the orbital position associated with any proposed new or modified assignment in the Regions 1 and 3 List of additional uses shall lie within one of the portions of the orbital arc listed in Table 1. The e.i.r.p. of such assignments shall not exceed 56 dBW, except at the positions listed in Table 2.”

2.3	The Board decided to align the note below Table 2 of Annex 7 to Appendix S30 (WRC-2000) to read as follows:

“* Proposed new or modified assignments in the List which involve this orbital position shall not exceed the pfd limit –138 dB (W/(m2  27 MHz)) at any point in Region 2.”

ANNEX 10

Draft new Rule of Procedure relating to Section 1 of Annex 4 to Appendix S30A to align its title and text with the provisions contained in the new Article 7 and to clarify its application with regard to Plan assignments at 17 GHz

1.	WRC-2000 adopted new Regions 1 and 3 downlink and feeder-link Plans and Lists, as defined in Articles 11 and 9A of Appendices S30 and S30A respectively and in Resolution 542 (WRC-2000).

2.	This Conference also revised the regulatory procedures associated with these Regions 1 and 3 Plans and Lists and established new protection criteria and methods for sharing between services in the frequency bands of Appendices S30 and S30A.

3.	Alignment of the title and text in Section 1 of Annex 4 to Appendix S30A

3.1	In reviewing the implementation of the regulatory procedures of these Appendices, the Bureau has identified that the title and text in Section 1 of Annex 4 to Appendix S30A (WRC-2000) require an alignment with the provisions contained in the new Article 7 to Appendix S30A (WRC-2000).

3.2	According to the new Article 7 to Appendix S30A (WRC-2000), frequency assignments to transmitting space stations in the FSS (all 3 regions) and in the BSS (Region 2) not subject to a Plan or a List, for which the corresponding request for coordination was received by the Bureau as from 3 June 2000, shall protect, using the criteria given in Section 1 of Annex 4 to Appendix S30A, frequency assignments in conformity with the appropriate Regional Plan, frequency assignments included in the Regions 1 and 3 List and also those for which the procedure of Article 4 of Appendix S30A has been initiated as from 3 June 2000, in accordance with the date of receipt of the complete Appendix S4 information.

3.3	The current title and text in Section 1 of Annex 4 to Appendix S30A only refers to assignments in the Regional Plans. Therefore, in order to avoid any future misunderstanding in its application, it is proposed to include in the title and text in Section 1 of Annex 4 to Appendix S30A a reference to assignments in the Regions 1 and 3 List as well as to Article 4 assignments in all 3 Regions, thus aligning it with the provisions contained in the new Article 7 to Appendix S30A (WRC-2000).

4.	Clarification on the implementation of Section 1 of Annex 4 to Appendix S30A with regard to Plan assignments at 17 GHz

4.1	In reviewing the implementation of the protection criteria and methods of Appendices S30 and S30A, the Bureau has identified that the text in Section 1 of Annex 4 to Appendix S30A (WRC-2000) requires further clarification or complementary information with regard to the implementation of the corresponding protection criteria and method for sharing between the services involved.

4.2	Section 1 of Annex 4 to Appendix S30A contains the criterion and the method applicable to protect receiving space stations subject to a feeder-link Plan or a feeder-link List from FSS (all 3 regions) or BSS (Region 2) transmitting space stations not subject to a Plan or a List. Case II of the method given in Appendix S8 is used together with a Ts / Ts criterion of 4%. WRC-2000 did not change these elements.

4.3	However, as mentioned in Note 7 of Section 9A.2 to Article 9A of Appendix S30A, the system noise temperature of the receiving space station of most of the assignments in the WRC-2000 Regions 1 and 3 feeder-link Plan at 17 GHz was changed from 900 K to 600 K, and the Ts / Ts criterion used to carry out before WRC-2000 the feasibility studies of the draft Plan with respect to the other services was changed from 4% to 6%. These changes were recommended by WP6S via the IRG Report to WRC-2000 (Document CMR-2000/34) as well as in § 5.3.1.6 of Chapter 5 of the CPM Report to that Conference.

4.4	In view of the above and in order to avoid different levels of protection WP6S, at its last meeting (Geneva, 13-26 September 2000), considered the matter and agreed that Section 1 of Annex 4 to Appendix S30A would need to be urgently revised to take into account the changes adopted by WRC-2000, i.e. to protect the Regions 1 and 3 Feeder-link Plan assignments at 17 GHz, for which the system noise temperature at the receiving space stations was changed from 900 K to 600 K, on the basis of a Ts / Ts criterion of 6%. It is thus proposed to further amend Section 1 of Annex 4 to Appendix S30A accordingly.

5.	A draft new Rule of Procedure covering the proposals presented above is contained in Appendix 1 to this Annex 10.

6.	The above-mentioned changes should be brought to the attention of Special Committee on Regulatory and Procedural Matter (SCRPM), CPM-02 and WRC-03 for a possible revision of Section 1 of Annex 4 to Appendix S30A at WRC-03, as appropriate.

Appendix:	1

APPENDIX 1 TO ANNEX 10

Draft new Rule of Procedure relating to Section 1 of Annex 4 to Appendix S30A to align its title and text with the provisions contained in the new Article 7 and to clarify its application with regard to Plan assignments at 17 GHz

An. 4

Criteria for sharing between services

1

Alignment of the title and text in Section 1 of Annex 4 to Appendix S30A

1.	The Board, in reviewing the implementation of the regulatory procedures of Appendix S30A, has identified that the title and text in Section 1 of Annex 4 to Appendix S30A (WRC-2000) requires an alignment with the provisions contained in the new Article 7 to Appendix S30A (WRC-2000).

2.	According to the new Article 7 to Appendix S30A (WRC-2000), frequency assignments to transmitting space stations in the FSS (all 3 regions) and in the BSS (Region 2) not subject to a Plan or a List, for which the corresponding request for coordination was received by the Bureau as from 3 June 2000, shall protect, using the criteria given in Section 1 of Annex 4 to Appendix S30A, frequency assignments in conformity with the appropriate Regional Plan, frequency assignments included in the Regions 1 and 3 List and also those for which the procedure of Article 4 of Appendix S30A has been initiated as from 3 June 2000, in accordance with the date of receipt of the complete Appendix S4 information.

3.	The current title and text in Section 1 of Annex 4 to Appendix S30A only refers to assignments in the Regional Plans. In view of the above and in order to avoid any future misunderstanding in its application, the Board decided on a provisional basis and until the time when this Annex is further reviewed by WRC-2003, to align the title and text in Section 1 of Annex 4 to Appendix S30A (WRC-2000), to read as follows1:

# 1	Threshold values for determining when coordination is required between transmitting space stations in the fixed-satellite service or the broadcasting-satellite service and a receiving space station in the feeder-link Plan or in the List, or a proposed new or modified receiving space station in the List in the frequency band 17.3-18.1 GHz (Regions 1 and 3) and in the feeder-link Plan or proposed modification to the Plan in the frequency band 17.3-17.8 GHz (Region 2)

With respect to §7.1, Article 7 of this Appendix, coordination of a transmitting space station in the fixed-satellite service or in the broadcasting-satellite service with a receiving space station in a broadcasting-satellite feeder link in the Region 1 and 3 Plan or in the List, or a proposed new or modified receiving space station in the List or in the Region 2 Plan or proposed modification to the Plan is required when the power flux-density arriving at the receiving space station of a broadcasting-satellite feeder link of another administration would cause an increase in the noise temperature of the feeder-link space station which exceeds a threshold value of Ts / Ts corresponding to 4%. Ts / Ts is calculated in accordance with Case II of the method given in Appendix S8.

Clarification on the implementation of Section 1 of Annex 4 to Appendix S30A with regard to Plan assignments at 17 GHz

1.	The Board, in reviewing the implementation of the protection criteria and methods of Appendix S30A, has identified that the text in Section 1 of Annex 4 to Appendix S30A (WRC-2000) requires further clarification or complementary information with regard to the implementation of the corresponding protection criteria and method for sharing between the services involved.

2.	Section 1 of Annex 4 to Appendix S30A contains the criterion and the method applicable to protect receiving space stations subject to a feeder-link Plan or a feeder-link List from FSS (all 3 regions) or BSS (Region 2) transmitting space stations not subject to a Plan or a List. Case II of the method given in Appendix S8 is used together with a Ts / Ts criterion of 4%. WRC-2000 did not change these elements.

3.	However, as mentioned in Note 7 of Section 9A.2 to Article 9A of Appendix S30A, the system noise temperature of the receiving space station of most of the assignments in the WRC-2000 Regions 1 and 3 feeder-link Plan at 17 GHz was changed from 900 K to 600 K, and the Ts / Ts criterion used to carry out before WRC-2000 the feasibility studies of the draft Plan with respect to the other services was changed from 4% to 6%. These changes were recommended by WP6S via the IRG Report to WRC-2000 (Document CMR-2000/34) as well as in § 5.3.1.6 of Chapter 5 of the CPM Report to that Conference.

4.	In view of the above and in order to avoid different levels of protection WP6S, at its last meeting (Geneva, 13-26 September 2000), considered the matter and agreed that Section 1 of Annex 4 to Appendix S30A would need to be urgently revised to take into account the changes adopted by WRC-2000, i.e. to protect the Regions 1 and 3 Feeder-link Plan assignments at 17 GHz, for which the system noise temperature at the receiving space stations was changed from 900 K to 600 K, on the basis of a Ts / Ts criterion of 6%.

5.	The Board, in reviewing the subject matter, decided to further amend the text in Section 1 of Annex 4 to Appendix S30A by adding the sentence indicated below.

In the case of Regions 1 and 3 Plan assignments at 17 GHz for which the system noise temperature at the receiving space stations was changed from 900 K to 600 K, a threshold value of ΔTS/TS corresponding to 6% shall be used.

ANNEX 11

Draft new Rule of Procedure relating to Section 2 of Annex 4 to Appendix S30A to align its title and text with the provisions contained in the new Article 7

1.	WRC-2000 adopted new Regions 1 and 3 downlink and feeder-link Plans and Lists, as defined in Articles 11 and 9A of Appendices S30 and S30A respectively and in Resolution 542 (WRC-2000).

2.	This Conference also revised the regulatory procedures associated with these Regions 1 and 3 Plans and Lists and established new protection criteria and methods for sharing between services in the frequency bands of Appendices S30 and S30A.

3.	Alignment of the title and text in Section 2 of Annex 4 to Appendix S30A

3.1	In reviewing the implementation of the regulatory procedures of these Appendices, the Bureau has identified that the title and text in Section 2 of Annex 4 to Appendix S30A (WRC-2000) require an alignment with the provisions contained in the new Article 7 to Appendix S30A (WRC-2000).

3.2	According to the new Article 7 to Appendix S30A (WRC-2000), frequency assignments to transmitting feeder-link earth stations in the FSS in Region 2, for which the corresponding request for coordination was received by the Bureau as from 3 June 2000, shall protect, using the criteria given in Section 2 of Annex 4 to Appendix S30A, frequency assignments in conformity with the Regions 1 and 3 Plan, frequency assignments included in the Regions 1 and 3 List and also those for which the procedure of Article 4 of Appendix S30A has been initiated as from 3 June 2000, in accordance with the date of receipt of the complete Appendix S4 information.

3.3	The current title and text in Section 2 of Annex 4 to Appendix S30A only refers to assignments in the Regions 1 and 3 Plan and List. Therefore, in order to avoid any future misunderstanding in its application, it is proposed to include in the title and text in Section 2 of Annex 4 to Appendix S30A a reference to Article 4 assignments in Regions 1 and 3, thus aligning it with the provisions contained in the new Article 7 to Appendix S30A (WRC-2000).

4.	A draft new Rule of Procedure covering the proposal presented above is contained in Appendix 1 to this Annex 11.

5.	The above-mentioned changes should be brought to the attention of Special Committee on Regulatory and Procedural Matter (SCRPM), CPM-02 and WRC-03 for a possible revision of Section 2 of Annex 4 to Appendix S30A at WRC-03, as appropriate.

Appendix:	1

APPENDIX 1 TO ANNEX 11

Draft new Rule of Procedure relating to Section 2 of Annex 4 to Appendix S30A to align its title and text with the provisions contained in the new Article 7

An. 4

Criteria for sharing between services

2

Alignment of the title and text in Section 2 of Annex 4 to Appendix S30A

1.	The Board, in reviewing the implementation of the regulatory procedures of Appendix S30A, has identified that the title and text in Section 2 of Annex 4 to Appendix S30A (WRC-2000) requires an alignment with the provisions contained in the new Article 7 to Appendix S30A (WRC-2000).

2.	According to the new Article 7 to Appendix S30A (WRC-2000), frequency assignments to transmitting feeder-link earth stations in the FSS in Region 2, for which the corresponding request for coordination was received by the Bureau as from 3 June 2000, shall protect, using the criteria given in Section 2 of Annex 4 to Appendix S30A, frequency assignments in conformity with the Regions 1 and 3 Plan, frequency assignments included in the Regions 1 and 3 List and also those for which the procedure of Article 4 of Appendix S30A has been initiated as from 3 June 2000, in accordance with the date of receipt of the complete Appendix S4 information.

3.	The current title and text in Section 2 of Annex 4 to Appendix S30A only refers to assignments in the Regions 1 and 3 Plan and List. In view of the above and in order to avoid any future misunderstanding in its application, the Board decided on a provisional basis and until the time when this Annex is further reviewed by WRC-03, to align the title and text in Section 2 of Annex 4 to Appendix S30A (WRC-2000), to read as follows1:

# 2	Threshold values for determining when coordination is required between transmitting feeder-link earth stations in the fixed-satellite service in Region 2 and a receiving space station in the feeder-link Plan or List, or a proposed new or modified receiving space station in the List for Regions 1 and 3 in the frequency band 17.8-18.1 GHz

With respect to §7.1, Article 7 of this Appendix, coordination of a transmitting feeder-link earth station in the fixed-satellite service with a receiving space station in a broadcasting-satellite feeder link in the Regions 1 and 3 Plan or List or a proposed new or modified receiving space station in the List is required when the power flux-density arriving at the receiving space station of a broadcasting-satellite feeder link of another administration would cause an increase in the noise temperature of the feeder-link space station which exceeds a threshold value of T/T corresponding to 3%, where T/T is calculated in accordance with the method given in Appendix S8, except that the maximum power densities per hertz averaged over the worst 1 MHz are replaced by power densities per hertz averaged over the total RF bandwidth of the feeder-link carriers.

ANNEX 12

Notification of assignments contained in the Regions 1 & 3 Lists

1.	WRC-2000, in its Resolution 542, resolved that assignments contained in the Regions 1 and 3 Lists of additional uses shall be annexed to the MIFR. However, there is no clear and specific reference to how the assignments contained in the Lists (which is already annexed to the MIFR) will be notified. Do they need to be notified or not?

2.	On the other hand, § 4.1.3 of Appendices S30/S30A (WRC-2000) refer to the date of bringing into use of assignments in the Lists, which implies that these assignments need to be notified before being brought into use.

3.	The title of Article 5 of both Appendices S30 & S30A (WRC-2000) “Notification, examination and recording in the Master International Frequency Register of frequency assignments to space stations in the broadcasting-satellite service” is general and implies that it covers BSS and Feeder-link Plans of all Regions as well as BSS and Feeder-link Lists of Regions 1 and 3.

4.	Moreover, as a general principle, the objective is that the MIFR should include all assignments notified with their operational characteristics.

5.	For that reason, assignments in the Lists (annexed to the MIFR), once notified and having successfully completed the procedure of Article 5 of both Appendices S30 and S30A, according to the case, need to be recorded in the Master Register. To this effect, a draft Rule of Procedure is submitted in Appendix 1 to this Annex 12 for consideration by the Board.

Appendix:	1

APPENDIX 1 TO ANNEX 12

Draft Rule of Procedure relating to
Notification of assignments contained in the Regions 1 & 3 Lists

WRC-2000, in its Resolution 542 resolved that assignments contained in the Regions 1 and 3 Lists of additional uses shall be annexed to the MIFR. However, there is no clear reference how these assignments should be notified.

On the other hand, § 4.1.3 of Appendices S30/S30A (WRC-2000) refer to the date of bringing into use of assignments in the List, which implies that these assignments before being brought into use should apply the notification procedure:

In view of the above, the Board concludes that:

Assignments contained in the Regions 1 and 3 Lists, once notified under Article 5 of Appendices S30/S30A and having successfully completed the procedure of that Article, will be included/recorded in the Master Register like other notified assignments of Regions 1 and 3, and Region 2 Plans. These assignments should also provide the due diligence information, in application of Res. 49 (Rev.WRC-2000), where appropriate.

The Board also noted that WRC-2000 decided to include in the Regions 1 & 3 List(s), assignments which were notified and brought into use before 3 June 2000 and which were not recorded in the MIFR. These assignments therefore need not to follow the formal application of Article 5 of Appendices S30/S30A. They will therefore be simply copied to/included in the Master Register, as appropriate.

The Board also decided that the above conclusion shall be brought to the attention of the Special Committee on Regulatory and Procedural Matter (SC), CPM-02 and WRC-03 for the required actions by the Bureau, as appropriate.

ANNEX 13

Draft new Rules of Procedure on Annexes 2A and 2B of Appendix S4 (WRC-2000)
to clarify their use in application of Appendices S30, S30A and S30B of the Radio Regulations

1.	In adopting the revised version of Annex 2 of Appendices S30 and S30A (WRC-2000), WRC-2000 decided to definitively include in Appendix S4 (WRC-2000) the data elements which were previously contained in Annex 2. These data elements are related to the basic characteristics to be furnished in notices relating to space stations in the Broadcasting-Satellite Service and to the associated feeder-link stations in the Fixed Satellite Service.

2.	In response to WRC-2000 instructions, the Radio Regulation Board (RRB) adopted at its 20th meeting, Geneva, 11 – 15 September 2000, a Rule of Procedure which requires administrations to use Appendix S4 (WRC-2000) when furnishing the basic data relating to stations in the fixed-satellite service subject to Appendix S30B.

3.	In reviewing Appendix S4 in the context of its use as of 3 June 2000 for the submission of notices in application of Appendices S30, S30A and S30B, the Bureau has identified some discrepancies between Annexes 2A and 2B of Appendix S4 (WRC-2000 Final Acts) on the one hand, and the former Annexes 2 to Appendices S30 and S30A and Annex 2 to Appendix S30B on the other hand. These discrepancies were not considered at WRC-2000 and thus do not result from explicit decisions taken at this Conference.

4.	Appendix 2 of this Annex 13 contains proposed alignments to those data items in Annexes 2A and 2B to Appendix S4 (WRC-2000), which were not discussed at WRC-2000, with those of former Annex 2 to Appendices S30 and S30A and those of Annex 2 to Appendices S30B.

5.	It is worth mentioning that the refinements to Appendix S4, which are proposed in Attachments 1 and 2 of Appendix 2 to this Annex 13, have no impact on the use of Appendix S4 for submissions of notices in application of other parts of the Radio Regulations.

6.	Moreover, Attachment 1 of Appendix 2 to this Annex 13 contains a cross-reference Table between data items of Annex 2A to Appendix S4 (WRC-2000) on the one hand, and those of the former Annex 2 to Appendices S30 and S30A and Annex 2 to Appendix S30B on the other hand, with a view to assist administrations in using Appendix S4 as of 3 June 2000 instead of the above-mentioned Annexes 2.

7.	It should be noted that this document includes the views expressed by ITU-R Working Parties 1A, 4A and 6S at their last meetings, Geneva, 23 - 31 October 2000, 25 September - 4 October 2000 and 13 - 26 September 2000 respectively.

8.	In view of the above, draft Rules of Procedure are proposed in Appendix 1 to this Annex 13 in order to indicate that the changes proposed to Annexes 2A and 2B to Appendix S4 (WRC-2000) would have to be taken into account by administrations for the submission of new notices in application of Appendices S30, S30A and S30B.

9.	The above-mentioned changes should be brought to the attention of the Special Committee on Regulatory and Procedural Matter (SC), CPM-02 and WRC-03 for a possible revision of Appendix S4 at WRC-03, as appropriate.

Appendices:	2

APPENDIX 1 TO ANNEX 13

Draft new Rules of Procedure concerning the use of Appendix S4 (WRC-2000)
in application of Appendices S30, S30A and S30B to the Radio Regulations

## APPENDIX  S4 to the RR

Use of Appendix S4 in application of Appendices S30, S30A and S30B to the RR

1.	The Board, in reviewing Appendix S4 in the context of its use as of 3 June 2000 for the submission of notices in application of Appendices S30, S30A and S30B, has identified some discrepancies between Annexes 2A and 2B of Appendix S4 (WRC-2000 Final Acts) on the one hand, and the former Annexes 2 to Appendices S30 and S30A and Annex 2 to Appendix S30B on the other hand. These discrepancies were not considered at WRC-2000 and thus do not result from explicit decisions taken by this Conference.

2.	In view of the above, the Board decided, as a temporary measure until WRC-03 reviews and revises Appendix S4, where appropriate, to instruct the Bureau to publish a Circular Letter containing the required amendments and refinements to Annexes 2A and 2B of Appendix S4 (WRC-2000). This Circular Letter shall also describe the purpose and objective of the changes with a view to assist administrations in using Appendix S4 (WRC-2000) as of 3 June 2000 instead of former Annexes 2 to Appendices S30 and S30A and Annex 2 to Appendix S30B.

APPENDIX 2 TO ANNEX 13

Draft refinements to Appendix S4 (WRC-2000) with respect to its use in application of Appendices S30, S30A and S30B to the Radio Regulations

Attachment 1 to this Appendix contains a cross-reference Table between relevant data items of Annex 2A to Appendix S4 (WRC-2000) on the one hand, and those of the former Annex 2 to Appendices S30 and S30A and Annex 2 to Appendix S30B on the other hand.

Attachment 1 also includes the proposed draft refinements to some of the data items of Appendix S4 (WRC-2000), which are either:

-	Proposed additions of data items previously in Annexes 2 to Appendices S30, S30A or S30B but missing in Appendix S4 (WRC-2000) (see items A.4.a.2, A.4.a.4, A.18, C.3.a, C.8.h, C.10.c.2, C.10.c.3, C.10.c.4, C.10.c.6, D.1); or

-	Proposed deletions of data items which were not in Annexes 2 to Appendices S30, S30A or S30B but which were forgotten to be deleted from Appendix S4 at WRC-2000 (see items A.8, B.3.f, B.3.g.1, B.3.g.2, B.3.g.3, C.9.d*, C.10.c.1); or

-	Proposed modifications of some data items in order to suppress other redundant items (see items A.1.b, A.1.c, A.7.c.1, C.11.b, C.11.c); or

-	Proposed editorial changes (see items A.11, C.6, C.8.j).

Attachment 2 to this Appendix contains an extract of Annex 2B to Appendix S4 (WRC-2000) only with the relevant data items which are proposed to be amended as mentioned above.

The draft refinements contained in this Appendix are proposed with revision marks (i.e.: new text underlined or deleted text strike through) to the version of Annexes 2A and 2B to Appendix S4 which will appear in the Radio Regulations after inclusion of the amendments contained in the WRC-2000 provisional Final Acts (2nd Edition).

The proposed course of action for each data item of Appendix S4 related to Appendices S30, S30A or S30B is indicated in column No.3 of the Table provided in Attachment 1. An indication of the use of these data items for submissions of notices in application of other parts of the Radio Regulations is provided in column No.7 of that Table.

attachment 1 to Appendix 2 of Annex 13

Correspondence between Annex 2A to Appendix S4 and former Annexes 2 to Appendices S30 and S30A and Annex 2 to Appendix S30B, and proposed draft refinements to Annex 2A to Appendix S4 (WRC-2000)

Notes/Abbreviations used in the table below:

1.	NOC:	No change required.

2.	MOD:	Modification required to Annex 2A to Appendix S4 and/or, if mentioned, to Annex 2B to Appendix S4.

3.	ADD:	Additional data item to Appendix S4.

4.	SUP:	Data item not to be used.

5.	NA:	Data item Not Applicable.

6.	Shaded data items correspond to draft additional items.

| 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- |
| Annex 2A to Appendix S4 Data Item Nb | Extract of Annex 2A to Appendix S4 Data Item Label (draft modifications to WRC-2000 version are proposed with revision marks) | Proposed course of action to Appendix S4 | Former Annex 2 to Appendix S30 Data Item Nb. | Former Annex 2 to Appendix S30A Data Item Nb. | Annex 2 to Appendix S30B Data Item Nb. | Applicable to other cases |
| A | General characteristics to be provided for the satellite network or the earth or radio astronomy station |  |  |  |  |  |
| A.1 | Identity of the satellite network or the earth or radio astronomy station |  |  |  |  |  |
| A.1.a | Identity of a satellite network | NOC | 6 | 2.4 & 3.2 | 1.1 & 1.5 & 2.1 | Yes |
| A.1.b | Not used Country and ITU number (Regions 1 and 3); country and beam identification (Region 2) Reason: To align similar item of Aps30 and APS30A. Moreover, there is no definition for “ITU number” | SUP | NA1 | NA | NA | No |
| A.1.c | Country and beam identification Reason: consequence of the deletion of item A.1.b | MOD (See Annex 2B) | 1NA | 1.1 | NA | No |
| A.1.d | Country and identification of the allotment; for a network not derived from the Allotment Plan, the identity of the network | NOC | NA | NA | 1.1 | No |
| A.1.f | Country symbol of the notifying administration. In the case of advance information, give the symbol of the administration or the symbols of the administrations in the group submitting the advance information on the satellite network. | NOC | 1 | 1.1 | 1.1 | Yes |
| A.2 | Date of bringing into use |  |  |  |  |  |
| A.2.a | The date (actual or foreseen, as appropriate) of bringing the frequency assignment (new or modified) into use. The date of bringing into use denotes the date at which the frequency assignment is brought into regular operation2a to provide the published radiocommunication service with the technical parameters within the technical characteristics notified to the Bureau. Whenever the assignment is changed in any of its basic characteristics (except in the case of a change in § A.1 a)), the date to be given shall be that of the latest change (actual or foreseen, as appropriate). | NOC | 5 | 1.4 | 1.4 | Yes |
| A.3 | Operating administration or agency | NOC | 21 | 2.11 & 3.11 | NA | Yes |
| A.4 | Orbital information |  |  |  |  |  |
| A.4.a | For the case of a space station onboard a GSO satellite: |  |  |  |  |  |
| A.4.a.1 | The nominal geographical longitude on the geostationary-satellite orbit | NOC | 2 | 3.1 | 1.2 | Yes |
| A.4.a.2 | The planned longitudinal tolerance and inclination excursion | MOD (See Annex 2B) | 13 | 3.6 | 1.10NA | Yes |
| A.4.a.4 | the service arc (the arc of the geostationary-satellite orbit within which the space station could provide the required service to its associated earth stations or service areas), also in the case of Appendix S30B for satellite network not derived from the Allotment Plan. | MOD (See Annex 2B) | NA | NA | 1.2NA | Yes |
| A.5 | Coordination | NOC | 18 | 2.8 & 3.8 | 3 | Yes |
| A.6 | Agreements | NOC | 19 | 2.9 & 3.9 | 3 | Yes |
| A.7 | Earth station site characteristics |  |  |  |  |  |
|  | For a specific earth station: |  |  |  |  |  |
| A.7.a | The horizon elevation angle in degrees for each azimuth around the earth station | NOC | NA | 2.6.g | NA | Yes |
| A.7.c | That is operating to an associated GSO space station, and having due regard to possible inclined-orbit operation of the associated space station: |  |  |  |  |  |
| A.7.c.1 | The planned minimum angle of elevation of the antenna in the direction of maximum radiation in degrees from the horizontal plane Reason: this information will be derived by the Bureau directly from the location of the satellite and the associated earth station | MOD (See Annex 2B) | NA | NA2.6.i | NA | Yes |
| A.7.e | The altitude (metres) of the antenna above mean sea level | NOC | NA | 2.6.h | NA | Yes |
| A.8 | Not used The rain climatic zone(s) Reason :this item of former Annex 2 to Appendix S30 was deleted at WRC-97. It is calculated by the Bureau. | SUP | NAMissing item | NA | NA | No |
| A.11 | Regular hours of operation (UTC) Reason: As it was in former Annexes 2 to Appendices S30 and S30A | MOD | 17 | 2.7 & 3.7 | NA | No |
| A.12 | Range of automatic gain control | NOC | NA | 3.12 | NA | No |
| A.18 | Subregional systems In the case of a space network submitted in accordance with Appendix S30B, indicate the type of system and participating administrations. If applicable, indicate the part of the national allotment proposed to be used to form the subregional system, and the notifying administration. | ADD | NA | NA | 5 | No |
| B. | Characteristics to be provided for each satellite antenna beam or each earth or radio astronomy station antenna |  |  |  |  |  |
| B.1 | The designation of the satellite antenna beam and, if appropriate, an indication as to whether it is a steerable or reconfigurable antenna beam. The designation shall be a character code, and the last character shall be an “R” for steerable or reconfigurable beams. | NOC | Missing item | Missing item3 | Missing item3 | Yes |
| B.2 | Transmission/reception indicator | NOC | NA | NA | Missing item | Yes |
| B.3 | Geostationary space station antenna characteristics |  |  |  |  |  |
| B.3.d | The pointing accuracy of the antenna | NOC | 12.b | 3.4.b | 1.8.c | Yes |
| B.3.f | The gain of the antenna in the direction of those parts of the geostationary-satellite orbit which are not obstructed by the Earth, in the case of operation in a band allocated in the Earth-to-space direction and in the space-to-Earth direction Reason: this information was included in item B.3.g.5 at WRC-2000 | MOD (See Annex 2B) | NA | NA3.4.g | NA | Yes |
| B.3.g | For the case of a space station submitted in accordance with Appendix S30, Appendix S30A or Appendix S30B: |  |  |  |  |  |
| B.3.g.1 | Co-polar gain of the antenna in the direction of maximum radiation referred to an isotropic radiator (dBi) and cross-polar gain of the antenna in the case of a beam of other than elliptical shape Reason: Cross-polar gain is not used in Appendix S30B | MOD (See Annex 2B) | 12.a | 3.4.a | 1.8.a | No |
| B.3.g.2 | Not used Shape of the beam (elliptical, circular, or other) Reason: This item of Annexes 2 to Appendices S30 and S30A was deleted at WRC-97, and is not necessary in the case of Appendix S30B | SUP | NAMissing item | NAMissing item | NA1.8.d | No |
| B.3.g.3 | Not used for circular beams: - half-power beamwidth in degrees; - co-polar and cross-polar radiation patterns;nominal intersection of the - antenna beam axis with the Earth (boresight longitude and latitude); Reason: This item of Annexes 2 to Appendices S30 and S30A was deleted at WRC-97, and is included in item B.3.g.4 in the case of Appendix S30B | SUP | NAMissing item | NAMissing item | NA1.8.e | No |
| B.3.g.4 | For elliptical* beams: - co-polar and cross-polar radiation patterns; - rotational accuracy in degrees; - major axis orientation in degrees anticlockwise from the Equator; - major axis (degrees) at the half-power beamwidth; - minor axis (degrees) at the half-power beamwidth; - nominal intersection of the antenna beam axis with the Earth (boresight longitude and latitude); * A circular beam is considered as a particular elliptical beam where the major and minor axes are equal and where the major axis orientation and rotational accuracy are equal to 0. Reason: This footnote were included in former Annexes 2 to Appendices S30 and S30A at WRC-97 and are also valid in the case of Annex 2 to Appendix S30B | NOC | 12.e & 8 | 3.4.e | 1.8.f & 1.8.b | No |
| B.3.g.5 | for beams of other than circular or elliptical shape: - co-polar and cross-polar gain contours plotted on a map of the Earth’s surface, preferably in a radial projection from the satellite on to a plane perpendicular to the line from the centre of the Earth to the satellite. The isotropic or absolute gain shall be indicated at each contour which corresponds to a decrease in gain of 2, 4, 6, 10 or 20 dB and thereafter at 10 dB intervals down to a value of 0 dB relative to an isotropic radiator. Whenever practicable, a numerical equation or table providing the necessary information to allow the gain contours to be plotted should be provided; - beam aim point longitude and latitude; - where a steerable beam (see No. S1.191) is used, the maximum antenna gain and the effective antenna gain contours (see No. S1.176); these contours shall be provided as defined above; - for an assignment in the bands 14.5-14.8 GHz or 17.7-18.1 GHz, the isotropic gain in the direction of those parts of the geostationary-satellite orbit which are not obstructed by the Earth. Use a diagram to show estimated isotropic gain relative to orbit longitude; Reason: consequence of the deletion of item B.3.g.3, noting that the last item is no longer required for beams with elliptical shape, as decided WRC-2000 | MOD | 12.f & 8 | 3.4.f & 3.4.g | 1.8.g & 1.8.b | No |
| C | Characteristics to be provided for each group of frequency assignments for a satellite antenna beam or an earth or radio astronomy station antenna |  |  |  |  |  |
| C.1 | Frequency range The frequency range within which the carriers will be located for each Earth-to-space or space-to-Earth service area, or for each space-to-space relay. | NOC | NA | NA | 1.3 | Yes |
| C.2 | Assigned frequency (frequencies) |  |  |  |  |  |
| C.2.a | The assigned frequency (frequencies), as defined in No. S1.148, in kHz up to 28 000 kHz inclusive, in MHz above 28 000 kHz to 10 500 MHz inclusive and in GHz above 10 500 MHz. Alternatively, in the case of a space station submitted in accordance with Appendix S30, the channel number. If the basic characteristics are identical, with the exception of the assigned frequency, a list of frequency assignments may be provided. | NOC | 3 | 1.2 | NA | Yes |
| C.3 | Assigned frequency band |  |  |  |  |  |
| C.3.a | The bandwidth of the assigned frequency band in kHz (see No. S1.147). | MOD (See Annex 2B) | 4NA | 1.3 | NA | Yes |
| C.4 | Class of station(s) and nature of service The class of station and nature of service performed, using the symbols shown in the Preface to the International Frequency List. | NOC | 9 | 3.3 | NA | Yes |
| C.5 | Receiving system noise temperature |  |  |  |  |  |
| C.5.a | In the case of a space station, the lowest total receiving system noise temperature, in kelvins, referred to the output of the receiving antenna of the space station. | NOC | NA | 3.5 | 1.9 | Yes |
| C.6 | Polarization The type of polarization and, if appropriate, sense of polarization of the antenna. In the case of circular polarization, indicate the direction of polarization (see Nos. S1.154 and S1.155). In the case of linear polarization, indicate the angle (degrees) measured counter-clockwise in a plane normal to the beam axis from the equatorial plane to the electric vector of the waves as seen from the satellite. In the case of a space station submitted in accordance with Appendix S30 or S30A, this indication is to be in the direction of the boresight or the aim point or as defined in § B.3 g) 3), B.3 g) 4) and § B.3 g) 5), respectively Reason: To align with the modification of item B.3.g.3 above | MOD | 12.c & 12.d | 2.6.e & 2.6.f & 3.4.c & 3.4.d | NA | Yes |
| C.7 | Class of emission, necessary bandwidth and description of the transmission In accordance with Article S2 and Appendix S1: |  |  |  |  |  |
| C.7.a | The class of emission and the necessary bandwidth | NOC | 10 | 1.5 | NA | Yes |
| C.8 | Power characteristics of the transmission |  |  |  |  |  |
| C.8.h | In the case of a space station submitted in accordance with Appendix S30, the power supplied to the antenna (dBW) and the maximum power density per Hz supplied to the antenna (dB(W/Hz)), averaged over the worst 5 MHz, 4 kHz and 27 MHz, and averaged over the worst 40 kHz in the case of Region 2. Reason: The maximum power density per Hz supplied to the antenna (dB(W/Hz)), averaged over the worst 4 kHz is still required to check the PFD limits of Section 4 of Annex 1 to Appendix S30 (WRC-2000) | MOD | 11 | NA | NA | No |
| C.8.i | In the case of an earth station submitted in accordance with Appendix S30A: - total transmitting power (dBW) in the assigned frequency band supplied to the input of the antenna; - for the band 17.3-18.1 GHz, the maximum power density per Hz (dB(W/Hz)) supplied to the input of the antenna averaged over the worst 1 MHz band; - for the band 14.5-14.8 GHz, the maximum power density per Hz (dB(W/Hz)) supplied to the input of the antenna averaged over the worst 4 kHz band; - for the band 17.3-17.8 GHz, the maximum power density per Hz (dB(W/Hz)) supplied to the input of the antenna averaged over the total RF bandwidth (24 MHz for Region 2 or 27 MHz for Regions 1 and 3); - range of power control, expressed in dB, above the transmitting power indicated above (if power control is used). | NOC | NA | 2.5.a & 2.5.b | NA | No |
| C.8.j | In the case of a space station or an earth station submitted in accordance with Appendix S30B: - the maximum value of power density, in dB(W/Hz), averaged over the necessary bandwidth of the modulated carrier, supplied to the input of the antenna (this value will be used for calculation of the C and D parameters in the case of a transmitting space station or the A and B parameters in the case of a transmitting earth station, see Appendix S30B, Annex 1, Section B); - the frequency below which signals whose peak-to-average ratio is less than 5 dB will be located; - maximum carrier power density, in dB(W/Hz), averaged over the worst 4 kHz band, supplied to the antenna input. Reason: Align with text of Annex 2 to Appendix S30B | MOD | NA | NA | 1.7.a & 1.7.c & 1.7.b & 2.2.a & 2.2.c & 2.2.b | No |
| C.9 | Information on modulation characteristics |  |  |  |  |  |
| C.9.b | In the case of a space station submitted in accordance with Appendix S30 or the case of a space station submitted in accordance with Appendix S30A: |  |  |  |  |  |
| C.9.b.1 | Type of modulation | NOC | 14.a | 1.6.a | NA | No |
| C.9.b.2 | Pre-emphasis characteristics | NOC | 14.b | 1.6.b | NA | No |
| C.9.b.3 | TV standard | NOC | 14.c | 1.6.c | NA | No |
| C.9.b.4 | Sound-broadcasting characteristics | NOC | 14.d | 1.6.d | NA | No |
| C.9.b.5 | Frequency deviation | NOC | 14.e | 1.6.e | NA | No |
| C.9.b.6 | Composition of the baseband | NOC | 14.f | 1.6.f | NA | No |
| C.9.b.7 | Type of multiplexing of the video and sound signals | NOC | 14.g | 1.6.g | NA | No |
| C.9.b.8 | Energy dispersal characteristics | NOC | 14.h | 1.6.h | NA | No |
| C.9.b.9 | In the case of a digital modulation, the effective and transmitted bit rate (Mbits/s) and symbol rate (Msymbols/s) | NOC | 14.i | 1.6.i | NA | No |
| C.9.b.10 | Roll-off factor of the filter of the receiver | NOC | Missing item | Missing item6 | NA | No |
| C.9.d | For stations operating in a frequency band subject to Nos. S22.5C, S22.5D or S22.5F, provide: - the type of mask; - the mask identification code. | MOD (See Annex 2B) | NAMissing item | NAMissing item7 | NA | Yes |
| C.10 | Type and identity of the associated station(s) The associated station may be another space station, a typical earth station of the network or a specific earth station. |  |  |  |  |  |
| C.10.b | For a specific associated earth station, the identity of the earth station and the geographical coordinates of the antenna site. | NOC | NA | 2.2 | NA | Yes |
| C.10.c | For an associated earth station (whether specific or typical): |  |  |  |  |  |
| C.10.c.1 | The class of station and nature of service performed, using the symbols shown in the Preface to the International Frequency List | MOD (See Annex 2B) | NA | NAMissing item | NAMissing item8 | Yes |
| C.10.c.2 | The isotropic gain (dBi) of the antenna in the direction of maximum radiation (see No. S1.160) | MOD (See Annex 2B) | 16.aNA | 2.6.b | 2.3.a | Yes |
| C.10.c.3 | The beamwidth in degrees between the half power points (describe in detail if not symmetrical) | MOD (See Annex 2B) | 16.bNA | 2.6.c | 2.3.b | Yes |
| C.10.c.4 | Either the measured radiation pattern of the antenna or the reference radiation pattern | MOD (See Annex 2B) | 16.cNA | 2.6.d | 2.3.c | Yes |
| C.10.c.5 | The lowest total receiving system noise temperature, in kelvins, referred to the output of the receiving antenna of the earth station under clear-sky conditions, when the associated station is a receiving earth station | NOC | NA | NA | 2.4 | Yes |
| C.10.c.6 | The equivalent antenna diameter (metres) Reason: Align with text of former Annex 2 to Appendix S30 | MOD (See Annex 2B) | 16.dNA | 2.6.a | NA | No |
| C.11 | Service area |  |  |  |  |  |
| C.11.b | In the case of a space station submitted in accordance with Appendix Appendices S30, S30A or S30B (for both transmitting and receiving space stations): - a set of a maximum of twenty feeder-link test points; and - a service area contour on the surface of the Earth or a service area defined by a minimum elevation angle in degrees. Reason: C.11.b and C.11.c have now the same meaning | MOD (See Annex 2B) | 7.a & 7.bNA | 2.3.a & 2.3.b | 1.6NA | No |
| C.11.c | Not usedIn the case of a space station submitted in accordance with Appendix S30 or Appendix S30B, the service area identified by a set of a maximum of twenty test points and by a service area contour on the surface of the Earth or a service area defined by a minimum elevation angle in degrees. Reason: consequence of the modification of item C.11.b | SUP | NA7.a & 7.b | NA | NA1.6 | No |
| C.12 | Required protection ratio The minimum acceptable aggregate carrier-to-interference ratio, if less than 26 dB. The carrier-to-interference ratio is to be expressed in terms of the power averaged over the necessary bandwidth of the modulated wanted and interfering signals, assuming both the desired carrier and interfering signals have equivalent bandwidths and modulation types. | NOC | NA | NA | 6 | No |
| C.15 | Description of the group(s) required in the case of non-simultaneous emissions | NOC | 23 | 5 | NA | No |
| D | Overall link characteristics To be provided only when simple frequency-changing transponders are used on the space station onboard a geostationary satellite. In the case of FSS networks using the frequency bands specified in No. S9.7 (GSO/GSO) of Appendix S5, Table S5-1 (items 1, 2 and 3 of the frequency band column), the data specified in this section of the Appendix is not mandatory and should not be submitted to the Bureau. |  |  |  |  |  |
| D.1 | Connection between Earth-to-space and space-to-Earth frequencies in the network The connection between uplink and downlink frequency assignments in each transponder for each intended combination of receiving and transmitting beams. Reason: Required in the case of Region 2 Appendices S30 and S30A | MOD (See Annex 2B) | 22NA | 4NA | NA | Yes |

attachment 2 to Appendix 2 of Annex 13

Proposed revision to relevant parts of Annex 2B to Appendix S4 (WRC-2000)
(Extract of the Table of characteristics to be submitted for space and radio astronomy services)

A  –  General characteristics of the satellite network or the earth station

| Items in Appendix | Advance publication of a geostationary-satellite network | Advance publication of a non-geostationary-satellite network subject to coordination under Section II of Article S9 | Advance publication of a non-geostationary-satellite network not subject to coordination under Section II of Article S9 | Notification or coordination of a geostationary-satellite network (including Appendix S30B) | Notification or coordination of a non-geostationary-satellite network | Notification or coordination of an earth station | Notice for space stations in the broadcasting-satellite service under Appendix S30 | Notice for feeder-link stations under Appendix S30A | Notice for stations in the fixed- satellite service under Appendix S30B | Items in Appendix | Radio astronomy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A.1.b |  |  |  |  |  |  | X |  |  | A.1.b |  |
| A.1.c |  |  |  |  |  |  | X | X |  | A.1.c |  |
| A.4.a.2 |  |  |  | X |  |  | X | X | X | A.4.a.2 |  |
| A.4.a.4 |  |  |  | X |  |  |  |  | X | A.4.a.4 |  |
| A.7.c.1 |  |  |  |  |  | X11 |  | X |  | A.7.c.1 |  |
| A.8 |  |  |  |  |  |  | X |  |  | A.8 |  |
| A.18 |  |  |  |  |  |  |  |  | X | A.18 |  |

X  Mandatory information	O  Optional information	C  This information need only be furnished when it has been used as a basis to effect coordination with another administration

11	Not required for coordination under No. S9.7A or S9.7B.

B  –  Characteristics to be provided for each satellite antenna beam and for each earth station antenna

| Items in Appendix | Advance publication of a geostationary-satellite network | Advance publication of a non-geostationary-satellite network subject to coordination under Section II of Article S9 | Advance publication of a non-geostationary-satellite network not subject to coordination under Section II of Article S9 | Notification or coordination of a geostationary-satellite network (including Appendix S30B) | Notification or coordination of a non-geostationary-satellite network | Notification or coordination of an earth station | Notice for space stations in the broadcasting-satellite service under Appendix S30 | Notice for feeder-link stations under Appendix S30A | Notice for stations in the fixed- satellite service under Appendix S30B | Items in Appendix | Radio astronomy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B.3.f |  |  |  | X |  |  |  | X |  | B.3.f |  |
| B.3.g.1 |  |  |  |  |  |  | X | X | X9 | B.3.g.1 |  |
| B.3.g.2 |  |  |  |  |  |  | X | X | X | B.3.g.2 |  |
| B.3.g.3 |  |  |  |  |  |  | X | X | X | B.3.g.3 |  |

X  Mandatory information	O  Optional information	C  This information need only be furnished when it has been used as a basis to effect coordination with another administration

9	Only information on co-polar antenna characteristics is required.

C  –  Characteristics to be provided for each group of frequency assignments for a satellite antenna beam or an earth station antenna

| Items in Appendix | Advance publication of a geostationary-satellite network | Advance publication of a non-geostationary-satellite network subject to coordination under Section II of Article S9 | Advance publication of a non-geostationary-satellite network not subject to coordination under Section II of Article S9 | Notification or coordination of a geostationary-satellite network (including Appendix S30B) | Notification or coordination of a non-geostationary-satellite network | Notification or coordination of an earth station | Notice for space stations in the broadcasting-satellite service under Appendix S30 | Notice for feeder-link stations under Appendix S30A | Notice for stations in the fixed- satellite service under Appendix S30B | Items in Appendix | Radio astronomy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C.3.a |  |  |  | X | X | X | X | X |  | C.3.a |  |
| C.9.d |  |  |  |  | X |  | X | X |  | C.9.d |  |
| C.10.c.1 |  |  | X | X | X |  |  | X | X | C.10.c.1 |  |
| C.10.c.2 |  |  | X | X | X |  | X | X | X | C.10.c.2 |  |
| C.10.c.3 |  |  | O | X | X |  | X | X | X | C.10.c.3 |  |
| C.10.c.4 |  |  | X | X | X |  | X | X | X9 | C.10.c.4 |  |
| C.10.c.5 |  |  | X | X | X |  |  |  | X | C.10.c.5 |  |
| C.10.c.6 |  |  |  |  |  |  | X | X |  | C.10.c.6 |  |
| C.11.b |  |  |  |  |  |  | X | X | X | C.11.b |  |
| C.11.c |  |  |  |  |  |  | X |  | X | C.11.c |  |

X  Mandatory information	O  Optional information	C  This information need only be furnished when it has been used as a basis to effect coordination with another administration

9	Only information on co-polar antenna characteristics is required.

D  –  Overall link characteristics

| Items in Appendix | Advance publication of a geostationary-satellite network | Advance publication of a non-geostationary-satellite network subject to coordination under Section II of Article S9 | Advance publication of a non-geostationary-satellite network not subject to coordination under Section II of Article S9 | Notification or coordination of a geostationary-satellite network (including Appendix S30B) | Notification or coordination of a non-geostationary-satellite network | Notification or coordination of an earth station | Notice for space stations in the broadcasting-satellite service under Appendix S30 * | Notice for feeder-link stations under Appendix S30A * | Notice for stations in the fixed- satellite service under Appendix S30B | Items in Appendix | Radio astronomy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| D.1 |  |  |  | X |  |  | X14 | X14 |  | D.1 |  |

X  Mandatory information	O  Optional information	C  This information need only be furnished when it has been used as a basis to effect coordination with another administration

*	The application of this column is suspended pending the decision of WRC-99.

14	Information is required only in the case of Region 2 notices.

The reference to “sub-regional” uses, that appeared in several footnotes prior to WRC-2000, does not appear in any provision of the current Article S5

1	 	For cases not covered under this paragraph, the Bureau, in collaboration with the appropriate ITU-R Study Groups, continue to develop applicable calculation methods and criteria in the form of Rules of Procedure to be submitted to the RRB for approval.

In cases where the Bureau is not in a position to publish notification data under No. S11.28 within three years following the date of notification, a comment received three months after the date of publication shall be taken into account by the Bureau.

Frequency bands allocated to the inter-satellite service to which par. 6 applies:

22.55 - 23.55 GHz24.45 - 24.75 GHz25.25 - 27.5 GHz32 - 33 GHz59.3 - 71 GHz122.25 - 123 GHz130 - 134 GHz167 – 174.8 GHz191.8 - 200 GHz

1	 	In the case of a wanted assignment in the Plan, the test-points referred to in this paragraph are those defined in that Plan. In the case of a wanted assignment in the List or for which the procedure of Article 4 of Appendices S30/S30A has already been initiated, the test-points referred to in this paragraph are those provided under former Annex 2 to Appendices S30/S30A or under Appendix S4.

2	 	An analysis carried out by the Radiocommunication Bureau has shown that the sensitivity to interference, in terms of being identified as affected, by networks received by the Bureau under Article 4 of Appendices S30 and S30A, caused by subsequent proposed modifications to the Plan, decreases when those networks have a very low equivalent protection margin (EPM). In those cases where, because of the above phenomenon they are not identified as affected (the equivalent protection margin reduces by at least 0.45 dB) it is up to the administrations concerned to take necessary action, as appropriate.

3	 	In the case of a wanted assignment in the Plan, the test-points referred to in this paragraph are those defined in that Plan. In the case of a wanted assignment in the List or for which the procedure of Article 4 of Appendices S30/S30A has already been initiated, the test-points referred to in this paragraph are those provided under former Annex 2 to Appendices S30/S30A or under Appendix S4.

4	 	An analysis carried out by the Radiocommunication Bureau has shown that the sensitivity to interference, in terms of being identified as affected, by networks received by the Bureau under Article 4 of Appendices S30 and S30A, caused by subsequent proposed modifications to the Plan, decreases when those networks have a very low equivalent protection margin (EPM). In those cases where, because of the above phenomenon they are not identified as affected (the equivalent protection margin reduces by at least 0.25 dB) it is up to the administrations concerned to take necessary action, as appropriate.

5	 	For reasons of uniformity, the Board decided that the Bureau shall use the same margin of 0.25 dB as indicated for the same purpose in Section 2 of Annex 1 of Appendix S30.

1	 	Proposed changes are printed in italic character.

1	 	Proposed changes are printed in italic character.

1	 	Proposed changes are printed in italic character.

*	 	The final version of the WRC-2000 Final Acts includes the modification proposed for this data item in the column titled “Notification or coordination of a non-geostationary-satellite network” of the table provided in Attachment 2 to Appendix 2 of this Annex.

Other cases than those of Appendices S30, S30A or S30B, as per Annex 2B of Appendix S4 (WRC-2000).

2	a 	Pending further studies by ITU-R on the applicability of the term “regular operation” to non-GSO satellite networks, the condition of regular operation shall be limited to GSO satellite networks.

The antenna gain in the direction of the horizon for each azimuth around the earth station is no longer required as decided by WRC-2000. This information will be generated by the Bureau directly from the description of the antenna and the horizon elevation angle in degrees for each azimuth around the earth station.

This information was generally provided implicitly using another Annex 2 data item.

Item newly required as a consequence of the application of Appendix S4 instead of Annex 2 to Appendix S30B so decided by WRC-2000. This item is indeed necessary.

Althought this information was kept in the WRC-2000 source document CMR2000/485, the maximum power density per Hz supplied to the antenna (dB(W/Hz)) averaged over the worst 4 kHz is no longer required according to the WRC-2000 provisional Final Acts (2nd edition.

This information is required for the implementation of Recommendation ITU-R BO.1293-1, as decided by WRC-2000.

Information newly required as a consequence of the application of Appendix S4 instead of Annexes 2 to Appendices S30 and S30A, as decided by WRC-2000. However, this information was not in former Annexes 2 to Appendices S30 and S30A and thus is not required. It is proposed to assign it to the “notification or coordination of a non-geostationary-satellite network”, according to the provisions referred to in the definition of this data item.

Information newly required as a consequence of the application of Appendix S4 instead of Annexes 2 to Appendices S30A and S30B, as decided by WRC-2000. However, this information was not in former Annex 2 to Appendix S30A nor in Annex 2 to Appendix S30B. It is thus not required.

The name of other administration(s) included in the service area is no longer required, as decided by WRC-2000. This information will be generated by the Bureau directly from the Service Area contours.