| Received: 4 April 2007 TECHNOLOGY | INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- | --- |
|  | RADIOCOMMUNICATION STUDY GROUPS | Document 8F/1189-E |
|  |  | 7 May 2007 |
|  |  | English only |

Subject:	Question ITU-R 229-1/8

Canada

CANADIAN EVALUATION GROUP (CEG)

EVALUATION REPORT of IP-OFDMA

# 1	Introduction

The IMT-2000 Canadian Evaluation Group (CEG) was formed in October 1996 under the auspices of the ITU Canadian National Organization (CNO) in response to the ITU-R request for evaluations of IMT-2000 Candidate RTT submissions (BR Circular Letter 8/LCCE/47). The CEG is open to all Canadian industry.

In 1998 the CEG evaluated the terrestrial radio transmission technology submissions for IMT-2000 and the report is in Document 8-1/119 (Study Period 1998-2000).

It should be noted that the original IMT-2000 submissions that were included in Recommendation ITU-R M.1457 have been enhanced over the years in subsequent revisions and there has been no need to formally evaluate these enhancements.

In view of the submission of a new terrestrial radio interface for IMT-2000, namely IP-OFDMA in Document 8F/1065, the current CEG report provides an evaluation of the proposal, considering also the information in Documents 8F/1075, 8F/1079(Rev.1) and 8F/1177.

# 2	CEG approach

In 1998, the CEG, bound by time and resource constraints, considered only the highest priority attributes (as indicated by the Category G1) in Recommendation ITU-R M.1225 and performed  “horizontal” evaluations by comparing the characteristics of the various RTT proposals across the board for each evaluation criterion/attribute.

For the 2006-2007 evaluation, the objective of the CEG was to evaluate the G1 attributes and as many of the G2, G3 and G4 attributes as possible.  To distribute the work load among the CEG members, a co-ordinator was appointed to lead the evaluation for each evaluation criterion, as shown in Annex 1, and to produce the evaluation summary for that criterion. The name and e-mail address of the co-ordinators and the CEG chairman were originally included to facilitate the clarification of any questions that arose during the process. All CEG members were asked to provide input to the co-ordinators.

For the evaluation of the IP-OFDMA proposal, the CEG employed the submission (Document 8F/1065) and other available documents (8F/1075, 8F/1079(Rev.1) and 8F/1177). Annex 3 of this report contains the CEG comments, based on Annex 3 of Recommendation ITU-R M.1225 and the self-evaluation report in Section 3 of Document 8F/1079(Rev.1).

Some CEG members participated in the evaluation group coordination meeting held in Orlando, FL, USA, 13-14 March 2007 (Ref.: Doc. 8F/1176) and followed up with additional questions for clarification that were distributed directly to the proponents and the other registered evaluation groups <http://www.itu.int/ITU-R/go/ip-ofdma> as well as through the evaluation group coordination forum that was set up for this purpose <http://ip-ofdma-forum.wirelessman.org/>.  The CEG questions and answers are recorded in Annex 2, included as embedded files, and the relevant information pertaining to the evaluation report has also been incorporated in the corresponding attribute in the table in Annex 3.

3	Summary of evaluation results

A summary of the conclusions and recommendations follows.

## 3.1	Criterion A3.1: spectrum efficiency

The RTT is a TDD system, supports scalable bandwidth and advanced antenna technologies such as MIMO and beam forming. The voice capacity and information capacity varies with the TDD UL/DL ratio, the bandwidth used and the antenna configuration.

The RTT is a pure packet base system, it supports voice via VoIP. The voice capacity between 80-90 Erlangs/MHz/cell for reuse 3, SIMO, appears to be equivalent or better than other IMT-2000 RTTs.

In terms of information capacity, the numbers provided are in line with what can be expected for a full buffer traffic data model for the 2x2 MIMO (DL: 5.52 Mbit/s/MHz/cell, UL: 2.1 Mbit/s/MHz/cell ) and 1x2 SIMO (DL: 3.57 Mbit/s/MHz/cell, UL: 1.59 Mbit/s/MHz/cell) configurations for a three sector cell.  A particular implementation with more antennas can offer higher efficiency. These numbers are equivalent or better than the number provided by other IMT-2000 RTTs.

## 3.2	Criterion A3.2: technology complexity-effect on cost of installation and operation

It is understood that the peak transmitter power is not limited by the RTT being proposed, which is the usual practice.  Broadband power amplifier is required by the RTT.

## 3.3	Criterion A3.3: quality

The RTT supports voice via voice over IP. The voice quality is controlled by the QoS mechanism, link adaptation and HARQ. The RTT maintains a reasonable round trip delay (140 ms including Vocoder) and handover latency (50ms -80ms) which are important for VoIP application.

The RTT supports scalable bandwidth, variable DL/UL ratio and advanced antenna technologies. Therefore, the maximum user bit rate for data depends on system configurations. The maximum bit rates are 20160 kbit/s for the case of DL/UL ratio = 2:1, 10 MHz channel single transmit antenna. For two transmit antennas, the maximum bit rates are doubled. The RTT appears capable of providing a higher peak data rate than other IMT-2000 RTTs.

There is no need for RF channel aggregation since the support of variable bandwidths is intrinsic to this RTT.

The RTT supports various QoS services and flexible resource allocation, system overload causes graceful degradation as data transmission bandwidth can be traded off for lower quality connections.

## 3.4	Criterion A3.4: flexibility of radio technologies

In terms of user bit-rates and variation of these rates as a function of radio propagation conditions, the RTT appears flexible. Bit-rates from 9.6 kbps to 23040 kbps on the D/L (with ratio DL:UL=35:12) and from 9.6 kbps to 6048 kbps on the U/L (with ratio DL:UL=26:21) are possible. Voice is treated as a packet service and appropriate QoS mechanisms are defined to provide adequate packet-technology-based services. The specific parameters requiring clarification are delay spread, receiver sensitivity, and vocoder parameters optimized to work with this RTT.

In terms of spectrum and spectrum-related matters, it is noted that this is a TDD technology with no duplexers or duplex separation required. Either 5 MHz or 10 MHz deployment is possible. The system being OFDM-based, sub-carriers can be grouped to increase data rates.  Due to the TDD nature, BS synchronisation and transmit-receive gaps (TTG, RTG) are necessary. Frequency re-use of 1 and 3 are possible.  The self-evaluation indicates that a hierarchical cell-structure is supported, though the meaning of “proper segmentation” of channels across the hierarchy should be clarified.

The RTT is also suited to the deployment of fixed wireless access.

Existing systems, such as those in ITU-R M.1033 and M.1073 cannot migrate to this technology.

## 3.5	Criterion A3.5: implication on network interfaces

The self-evaluation is reasonable and base station synchronization through GPS is the same method used by other RTTs.

## 3.6	Criterion A3.6: handportable performance optimization capability

The self-evaluation does not sufficiently address the mobile station average power output, receiver linearity requirements, and DSP requirements.  Mobile station power levels for 64 QAM are not discussed.

## 3.7	Criterion A3.7: coverage/power efficiency

The base site coverage efficiency can be quantitatively determined by addressing coverage limitation and/or by calculating the maximum coverage range for the lowest traffic loading.

On the Evaluation of Base Site Coverage Efficiency (A3.7.1.1), coverage efficiency of lightly loaded or unloaded system varies by environment and service: The self-evaluation, based on link budget calculations, considers speech and packet data services in vehicular, pedestrian and indoor environments (Reference Tables 20, 21, and 22 in Doc. 1079(Rev.1)).  The self-evaluation does provide the required information as per M.1225. Based on the information available from the self-evaluation, it can be concluded that IP-OFDMA meets this evaluation criterion.

On the Evaluation of Coverage Efficiency Enhancement Techniques (A3.7.1.2): With reference to the self-evaluation, various techniques are briefly described, most involving antenna technologies such as remote antenna, distributed antenna, adaptive switching and MIMO.  Many of the enhancement techniques, e.g. MIMO, are not unique to IP-OFDMA, and some of them are already employed or demonstrated with existing wireless systems.  It is therefore concluded that this evaluation criterion requiring only qualitative input is also met.

# 4	Conclusion

The CEG has evaluated the proposed sixth terrestrial RTT for IMT-2000 based on Rec. ITU-R M.1225.

A number of questions remain to be answered as indicated in Annex 3.

# List of Annexes

Annex 1	Canadian Evaluation Group (CEG) Evaluation criteria Co-ordinators

Annex 2	Reference documents

Annex 3	CEG comments

Annex 1

Canadian Evaluation Group (CEG) 
Evaluation criteria Co-ordinators and Members

| Number | Criteria | Co-ordinators |
| --- | --- | --- |
| A3.1 | Spectrum efficiency | Peiying Zhu (Nortel) Alternate: |
| A3.2 | Technology complexity-effect on cost of installation and operation | Priscilla Santos (Bell) Alternate: P.F. Ng (Rogers) |
| A3.3 | Quality | Peiying Zhu (Nortel) Alternate: Rémi Chayer (Wavesat) |
| A3.4 | Flexibility of radio technologies | Venkatesh Sampath (Ericsson) Alternate: Rémi Chayer (Wavesat) |
| A3.5 | Implication on network interfaces | Serge Bertuzzo (TELUS) Alternate: Priscilla Santos (Bell) |
| A3.6 | Handportable performance optimisation capability | Paul Frew (Motorola) Alternates: Vino Vinodrai (RIM) Rémi Chayer (Wavesat) |
| A3.7 | Coverage/power efficiency | P.F. Ng (Rogers) Alternate: Priscilla Santos (Bell) |

Chairman of the CEG:  	José Costa (Nortel)
Tel.: 613 763-7574

CEG Members

| Name | Company |
| --- | --- |
| P.F. Ng | Rogers |
| Peiying Zhu | Nortel |
| Vino Vinodrai | RIM |
| Paul Frew | Motorola |
| Serge Bertuzo | TELUS |
| José Costa | Nortel |
| Venkatesh Sampath | Ericsson |
| Dawood Khan | KAZAM Technologies |
| Priscilla Santos | Bell Canada |
| Rémi Chayer | Wavesat |
| Phuong Vu | Industry Canada |
| Viet Nguyen | Ericsson |
| Cindy Cook | Industry Canada |
| John Visser | Nortel |
| Ed O'Leary | Rogers |
| Kelly Hisaki | Bell Canada |
| Jorge del Rio | TELUS |
| Daniel Badiere | RIM |
| Peter Nurse | Sigma Delta Communications, Inc. |
| Wen Tong | Nortel |
| Jane C Brownley | Alcatel-Lucent Canada |
| Andy McGregor | Nortel |
| Jianglei Ma | Nortel |

Annex 2

Reference documents 
(also posted at http://www.imt-2000.ca)

CEG-QA-1-Responses	Comments/Questions/Answers on the IP-OFDMA submission to ITU-R; distributed by CEG on 2 April 2007 and further responses received from Jose Puthenkulam and Shailender Timiri (Intel Corporation), posted on <http://ip-ofdma-forum.wirelessman.org/> on 17 April 2007, as identified with change marks in the electronically-attached file:

CEG-QA-2	Comments/Questions/Answers on the IP-OFDMA submission to ITU-R (Second Set); distributed by CEG on 18 April 2007.

CEG-QA-2-Answers-02	Comments/Questions/Answers on the IP-OFDMA submission to ITU-R (Second Set); distributed by CEG on 18 April 2007 and further responses received from Jose Puthenkulam and Apostolos Papathanassiou (Intel Corporation) on 24 April 2007, posted on <http://ip-ofdma-forum.wirelessman.org/> in the electronically-attached file (Note: the answers to the second set of questions were received after the CEG completed its work and were incorporated editorially in the report without review by the CEG):

Annex 3

CEG comments

The CEG comments are provided in the electronically-attached file:

________________

Reuse 3 refers to frequency reuse = 3, this is slightly different from traditional reuse, it is in fact a frequency reuse by transmitting using a subset of orthogonal sub carrier for each sector.  From a cell perspective, it is still frequency reuse 1.

SIMO refers to single input and multiple output.  For example, in the case of DL, single transmit antenna on base station, multiple receive antenna on terminal (in this case = 2).