| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Document 7-E |
|  |  |  | 7 February 2003 |
|  |  |  | Original: Russian |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | PLENARY MEETING |
| --- | --- |
| Uzbekistan (Republic of) |  |
| PROPOSALS FOR THE WORK OF THE CONFERENCE |  |

The Administration of the Republic of Uzbekistan, taking into account the ITU-R documents prepared by the study groups and the CPM Report, hereby submits the following proposals in relation to the items on the WRC-03 agenda in Resolution 1156, for consideration by WRC-03.

Agenda item 1.1

1.1	requests from administrations to delete their country footnotes or to have their country name deleted from footnotes, if no longer required, in accordance with Resolution 26 
(Rev.WRC-97)

The Republic of Uzbekistan requests that the name "Uzbekistan" be deleted from footnotes 5.56 and 5.482 in Article 5 of the Radio Regulations.

MOD	UZB/7/1

5.56	The stations of services to which the bands 14-19.95 kHz and 20.05-70 kHz and in Region 1 also the bands 72-84 kHz and 86-90 kHz are allocated may transmit standard frequency and time signals. Such stations shall be afforded protection from harmful interference. In Armenia, Azerbaijan, Belarus, Bulgaria, Georgia, Kazakstan, Mongolia, Kyrgyzstan, Slovakia, the Czech Rep., Russian Federation, Tajikistan, Turkmenistan and Ukraine, the frequencies 25 kHz and 50 kHz will be used for this purpose under the same conditions.     (WRC-03)

Reasons:	The standard frequency and time signal service frequencies 25 and 50 kHz are not used in Uzbekistan.

MOD	UZB/7/2

5.482	In the band 10.6-10.68 GHz, stations of the fixed and mobile, except aeronautical mobile, services shall be limited to a maximum equivalent isotropically radiated power of 40 dBW and the power delivered to the antenna shall not exceed –3 dBW. These limits may be exceeded subject to agreement obtained under No. 9.21. However, in Saudi Arabia, Armenia, Azerbaijan, Bahrain, Bangladesh, Belarus, China, the United Arab Emirates, Georgia, India, Indonesia, Iran (Islamic Republic of), Iraq, Japan, Kazakstan, Kuwait, Latvia, Lebanon, Moldova, Nigeria,

Pakistan, the Philippines, Qatar, Syria, Kyrgyzstan, Russian Federation, Tajikistan, Turkmenistan and Ukraine, the restrictions on the fixed and mobile, except aeronautical mobile, services are not applicable.

Reasons:	No longer required.

The Republic of Uzbekistan requests that the name "Uzbekistan" be inserted in footnotes 5.152, 5.203A and 5.237 in Article 5 of the Radio Regulations.

MOD	UZB/7/3

5.152	Additional allocation:  in Armenia, Azerbaijan, China, Côte d'Ivoire, Georgia, Iran (Islamic Republic of), Kazakstan, Moldova, Uzbekistan, Kyrgyzstan, the Russian Federation, Tajikistan, Turkmenistan and Ukraine, the band 14 250-14 350 kHz is also allocated to the fixed service on a primary basis. Stations of the fixed service shall not use a radiated power exceeding 24 dBW.      (WRC-03)

MOD	UZB/7/4

5.203A	Additional allocation:  in Israel, Mauritania, Uzbekistan, Qatar and Zimbabwe, the band 136-137 MHz is also allocated to the fixed and mobile, except aeronautical mobile (R), services on a secondary basis until 1 January 2005.      (WRC-03)

MOD	UZB/7/5

5.237	Additional allocation:  in the Congo, Eritrea, Ethiopia, Gambia, Guinea, Libya, Malawi, Mali, Uzbekistan, Senegal, Sierra Leone, Somalia, Tanzania and Zimbabwe, the band 174-223 MHz is also allocated to the fixed and mobile services on a secondary basis.     (WRC-03)

The Republic of Uzbekistan requests that the following footnotes be added to the Table of Frequency Allocations in Article 5 of the Radio Regulations:

ADD	UZB/7/6

5.132A	Additional allocation: in Uzbekistan, the band 5 005-5 060 kHz is also allocated to the land mobile service on a secondary basis.

ADD	UZB/7/7

5.133A	Additional allocation: in Uzbekistan, the bands 5 480-5 680 kHz and 6 685-6 765 kHz are also allocated to the fixed service on a primary basis.

ADD	UZB/7/8

5.135	Additional allocation: in Uzbekistan, the band 5 950-6 200 kHz is also allocated to the fixed and mobile, except aeronautical mobile, services on a secondary basis.

ADD	UZB/7/9

5.145A	Additional allocation: in Uzbekistan, the band 9 040-9 400 kHz is also allocated to the land mobile service on a secondary basis.

ADD	UZB/7/10

5.146A	Additional allocation: in Uzbekistan, the bands 10 005-10 100 kHz, 11 275-11 400 kHz, 13 200-13 360 kHz, 17 900-17 970 kHz and 25 670-26 100 kHz are also allocated to the fixed service on a secondary basis.

ADD	UZB/7/11

5.151A	Additional allocation: in Uzbekistan, the bands 14 000-14 250 kHz and 21 000-21 450 kHz are also allocated to the fixed and mobile, except aeronautical mobile, services on a secondary basis.

ADD	UZB/7/12

5.155C	Additional allocation: in Uzbekistan, the bands 21 870-21 924 kHz and 23 200-23 350 kHz are also allocated to the land mobile service on a secondary basis.

ADD	UZB/7/13

5.158	Additional allocation: in Uzbekistan, the band 25 550-25 670 kHz is also allocated to the fixed and mobile, except aeronautical mobile, services on a primary basis.

ADD	UZB/7/14

5.159	Additional allocation: in Uzbekistan, the band 24 890-24 990 kHz is also allocated to the fixed and land mobile services on a secondary basis.

ADD	UZB/7/15

5.313	Additional allocation: in Uzbekistan, the band 790-890 MHz is also allocated to the radiolocation service on a secondary basis.

Agenda item 1.3

1.3	to consider identification of globally/regionally harmonized bands, to the extent practicable, for the implementation of future advanced solutions to meet the needs of public protection agencies, including those dealing with emergency situations and disaster relief, and to make regulatory provisions, as necessary, taking into account Resolution 645 (WRC-2000)

Uzbekistan considers that the frequencies and bands identified may be used globally/regionally by public protection agencies, including those dealing with emergency situations. However, Uzbekistan cannot support the use for Region 1 of the frequency band 380-400 MHz already harmonized in Europe (CPM Report Method C), since this band is already overloaded in our country. It is difficult to select globally/regionally one single band for public protection agencies, including those dealing with emergency situations and disaster relief, from the 16 frequency bands put forward in the CPM Report. For this reason, Uzbekistan supports CPM Report Method D, namely the adoption of a new resolution at WRC-03 for ITU to conduct the relevant studies with a view to identifying specific frequency bands for public safety and disaster relief in the event of major catastrophes and taking a decision on their global/regional use at the next conference (WRC-07).

Agenda item 1.5

1.5	to consider, in accordance with Resolution 736 (WRC-2000), regulatory provisions and spectrum requirements for new and additional allocations to the mobile, fixed, Earth exploration-satellite and space research services, and to review the status of the radiolocation service in the frequency range 5 150-5 725 MHz, with a view to upgrading it, taking into account the results of ITU-R studies

Having considered the results of the ITU-R studies presented in the CPM Report, Uzbekistan's position is as follows:

With respect to Resolution 736 (WRC-2000) resolves 1 on the allocation of frequencies to the mobile service in the bands 5 150-5 350 MHz and 5 470-5 725 MHz for the implementation of wireless access systems, including RLANs, Uzbekistan supports CPM Report Method B, namely a secondary allocation to the mobile service in the bands 5 150-5 350 MHz and 5 470-5 725 MHz for the implementation of wireless access systems, including RLANs, with appropriate footnotes and/or resolutions to ensure protection of existing services.

With respect to Resolution 736 resolves 3 on an additional primary allocation for the Earth exploration-satellite service and the space research service in the frequency range 5 460-5 570 MHz, Uzbekistan supports Method B, namely no allocation for EESS in the band 5 460-5 570 MHz, since in Uzbekistan the radionavigation service operates in this band.

With respect to Resolution 736 resolves 4 on possible upgrading of the status of frequency allocations to the radiolocation service in the frequency range 5 350-5 650 MHz, Uzbekistan supports CPM Report Method A, namely upgrading the status of the radiolocation service to primary in the band 5 350-5 650 MHz on condition that the conference considers a footnote ensuring the protection of existing and future systems in the radionavigation service from the radiolocation service in that band.

Agenda item 1.7

1.7	to consider issues concerning the amateur and amateur-satellite services:

1.7.1	possible revision of Article 25;

1.7.2	review of the provisions of Article 19 concerning the formation of call signs in the amateur services in order to provide flexibility for administrations;

1.7.3	review of the terms and definitions of Article 1 to the extent required as a consequence of changes made in Article 25

With respect to possible revision of Article 25 of the Radio Regulations, Uzbekistan's position is as follows:

•	We object to the deletion of No. 25.1 forbidding international communications if one of the administrations concerned objects.

•	We support the modification of No. 25.2 proposed in the CPM Report.

•	We do not support the modification (CPM Report Method A) or deletion (CPM Report Method B) of No. 25.3 and insist that the provision be retained with the following wording:

NOC	UZB/7/16

25.3		2)	It is absolutely forbidden for amateur stations to be used for transmitting international communications on behalf of third parties.

•	We do not object to the deletion of No. 25.4.

•	We support the modifications to No. 25.5 (CPM Report Method B).

•	We support the modifications to No. 25.6 as per CPM Report Method A.

•	We support the deletion of Nos. 25.7, 25.8 and 25.9, since these provisions duplicate the contents of Nos. 15.2, 3.6 and 3.7, and 19.4 and 19.5, respectively.

•	We support the modification of No. 25.11 as per the CPM Report.

•	We support the addition of new Nos. 25.X and 25.XX in Article 25 relating to the amateur service.

With respect to possible revision of the provisions of Article 19 of the Radio Regulations, the Uzbekistan Administration's position is as follows:

•	We support the deletion of No. 19.49 removing restrictions on some call signs, thereby providing more flexibility for administrations, particularly those which do not yet possess call signs for amateur stations.

•	We support the modification to No. 19.68 proposed in the CPM Report giving administrations more flexibility in determining call signs and introducing the new Article 19Y.

With respect to possible revision of terms and definitions in Article 1 consequential on the amendments to Article 25, the Administration of the Republic of Uzbekistan considers that no modifications to Article 1 are necessary.

Agenda item 1.11

1.11	to consider possible extension of the allocation to the mobile-satellite service (Earth-to-space) on a secondary basis in the band 14-14.5 GHz to permit operation of the aeronautical mobile-satellite service as stipulated in Resolution 216 (Rev.WRC-2000)

Uzbekistan supports broadening the allocation to include the aeronautical mobile-satellite service, as indicated in Resolution 216 (Rev.WRC-2000), for the purpose of communications with aircraft not related to flight safety which would not be part of the aeronautical mobile-satellite (R) service. This may enhance the overall efficiency of the airline business. The Administration of the Republic of Uzbekistan supports CPM Report Method A, namely deleting "except aeronautical mobile-satellite" from the Table of Frequency Allocations in Article 5 of the Radio Regulations for the band 14-14.5 GHz.

Agenda item 1.12

1.12	to consider allocations and regulatory issues related to the space science services in accordance with Resolution 723 (Rev.WRC-2000) and to review all Earth exploration-satellite service and space research service allocations between 35 and 38 GHz, taking into account Resolution 730 (WRC-2000)

Having considered the studies presented in the CPM Report, the Uzbekistan Administration's position is as follows:

With respect to Resolution 723 resolves 1 on the provision of up to 3 MHz of frequency spectrum in the frequency range 100 MHz to 1 GHz for the implementation of telecommand links in the space research and space operations services, the CPM Report considers a procedure for allocation of

spectrum from 100 MHz to 1 GHz. Detailed studies were carried out in the band 235-328.6 MHz. Since the band 257-262 MHz has already been used successfully for many years now in some countries by telecommand systems, the possibility of allocating 3 MHz within the range 257-262 MHz is considered. Services operating in accordance with the Radio Regulations within that range might experience occasional interference from such systems if they are located within the coordination distance of around 400 kilometres. Given that in Uzbekistan this band is used by the aeronautical mobile (OR) service and an additional allocation in that band may have an adverse impact on flight safety, the Administration of the Republic of Uzbekistan supports CPM Report Method A, namely not to allocate 3 MHz in the range 257-262 MHz for telecommand links in the space research and space operations services.

With respect to Resolution 723 resolves 2 on possible incorporation in the Table of Frequency Allocations of the existing primary allocation to the space research service in the band 7 145-7 235 MHz under No. 5.460, we support the method proposed in the CPM Report, namely: add in Article 5 of the Radio Regulations in the Table of Frequency Allocations a primary allocation to the space research service (Earth-to-space) in the bands 7 145-7 190 MHz and 7 190-7 235 MHz; modify No. 5.460 to delete the reference to No. 9.21 in the band 7 145-7 235 MHz; retain the restriction that no emissions to deep space are permitted in the subband 7 190-7 235 MHz; and add to No. 5.460 the provision that GSO satellites in the space research service shall not claim protection from the fixed service and the mobile service, and that No. 5.43A shall not apply to them.

With respect to Resolution 723 resolves 3 on reviewing the allocations to the space research service (deep space) (space-to-Earth) and the inter-satellite service, taking into account the coexistence of these two services in the frequency range 32-32.3 GHz, with a view to facilitating the satisfactory operation of these services, we support the CPM Report proposal to delete the inter-satellite service allocation in the range 32-32.3 GHz.

With respect to Resolution 723 resolves 4 on reviewing existing allocations to space science services near 15 GHz and 26 GHz, with a view to accommodating wideband space-to-Earth space research applications, we support CPM Report Method C, namely no change to the Radio Regulations in respect of the space science services allocation near 15 GHz and 26 GHz.

Having considered the results of ITU-R studies presented in the CPM Report on the allocations to the Earth exploration-satellite service and the space research service between 35 and 38 GHz, having regard to Resolution 730 of the use of the frequency band 35.5-35.6 GHz by spaceborne precipitation radars, the Uzbekistan Administration's position is as follows:

With respect to the frequency band 35.5-36 GHz, we support CPM Report Method B, namely replacing footnote 5.551A with a footnote limiting the mean power flux-density at the Earth's surface from active sensors operating in the SRS and EESS in the band 35.5-36 GHz.

With respect to the frequency band 36-37 GHz, we support CPM Report Method A, namely no change to the Radio Regulations and continuation of studies under normal ITU-R activities.

With respect to the frequency band 37-38 GHz, we support the method proposed in the CPM Report, namely no change to the existing SRS (space-to-Earth) allocation in the band 37-38 GHz and continuation of studies under normal ITU-R activities.

Agenda item 1.16

1.16	to consider allocations on a worldwide basis for feeder links in bands around 1.4 GHz to the non-GSO MSS with service links operating below 1 GHz, taking into account the results of ITU-R studies conducted in response to Resolution 127 (Rev.WRC-2000), provided that due recognition is given to the passive services, taking into account No. 5.340

Having considered the studies proposed in the CPM Report in relation to a worldwide allocation in the frequency bands 1 390-1 393 MHz and 1 429-1 432 MHz for feeder links in the non-GSO mobile-satellite service, Uzbekistan's position is as follows:

With respect to the band 1 390-1 393 MHz, we support CPM Report Method A2, namely no allocation to the mobile-satellite service in this band, and with respect to the band 1 429-1 432 MHz, we support CPM Report Method B2, namely no allocation to the mobile-satellite service in this band, since the bands in question are used in Uzbekistan for aeronautical services.

Agenda item 1.18

1.18	to consider a primary allocation to the fixed service in the band 17.3-17.7 GHz for Region 1, taking into account the primary allocations to various services in all three Regions

Having considered the ITU-R studies presented in the CPM Report, the Administration of the Republic of Uzbekistan does not support the CPM Report method, namely no allocation to the fixed service in Region 1 in the band 17.3-17.7 GHz, and requests that an additional footnote be added in Article 5 of the Radio Regulations, as follows:

ADD	UZB/7/17

5.514A	Additional allocation: in Uzbekistan, the band 17.3-17.7 GHz is also allocated to the fixed service on a primary basis.

Agenda item 1.20

1.20	to consider additional allocations on a worldwide basis for the non-GSO MSS with service links operating below 1 GHz, in accordance with Resolution 214 (Rev.WRC-2000)

In Uzbekistan there are a large number of frequency assignments on frequencies below 1 GHz relating to aeronautical services. An additional allocation may have an adverse impact on flight safety and Uzbekistan proposes not to consider an additional spectrum allocation and supports the CPM Report proposal to delete Resolution 214 on sharing studies relating to consideration of the allocation of bands below 1 GHz to the non-GSO mobile-satellite service, and no change to Article 5 of the Radio Regulations.

Agenda item 1.23

1.23	to consider realignment of the allocations to the amateur, amateur-satellite and broadcasting services around 7 MHz on a worldwide basis, taking into account Recommendation 718 (WARC-92)

Under the Radio Regulations, the frequency band 7 000-7 100 kHz is allocated worldwide to the amateur service and the amateur-satellite service on a primary basis, and the band 7 100-7 300 kHz in Region 1 and 7 300-7 350 kHz worldwide are allocated to the broadcasting service, with the restrictions laid down in footnotes 5.134 and 5.143 and subject to footnotes 5.140 and 5.141.

In Uzbekistan, those bands are also allocated to the fixed service and the land mobile service on a secondary basis, and the Administration of Uzbekistan therefore supports CPM Report Method B, providing for the following realignment of allocations in the band 6 765-8 100 kHz:

6 765-7 000 kHz	Fixed and mobile (except aeronautical mobile (R)), co-primary

7 000-7 100 kHz	Amateur and amateur-satellite, co-primary

7 100-7 200 kHz	Amateur, primary

7 200-7 300 kHz	Amateur, fixed and mobile (except aeronautical mobile (R)), co-primary Regions 1 and 3; Amateur, primary Region 2

7 300-7 550 kHz	Broadcasting, primary

7 550-8 100 kHz	Fixed and mobile (except aeronautical mobile (R)), co-primary

and requests that the following footnote be included in the Table of Frequency Allocations:

ADD	UZB/7/18

5.141A	Additional allocation: in Uzbekistan, the bands 7 000-7 100 kHz and 7 100-7 200 kHz are also allocated to the fixed and land mobile services on a secondary basis.

Agenda item 1.28

1.28	to permit the use of the band 108-117.975 MHz for the transmission of radionavigation satellite differential correction signals by ICAO standard ground-based systems

In Uzbekistan, the frequency band 108-117.975 MHz is allocated to the aeronautical mobile (OR) service on a primary basis. Worldwide allocation of this band for the transmission of differential correction systems may have an adverse impact on flight safety. Our Administration cannot support any of the methods proposed in the CPM Report and requests that the following footnote be added in Article 5 of the Radio Regulations:

ADD	UZB/7/19

5.195	Additional allocation: in Uzbekistan, the band 108-117.975 MHz is also allocated to the aeronautical mobile (OR) service on a primary basis.

Agenda item 1.31

1.31	to consider the additional allocations to the mobile-satellite service in the 1-3 GHz band, in accordance with Resolutions 226 (WRC-2000) and 227 (WRC-2000)

Having considered the ITU-R studies presented in the CPM Report, the Uzbekistan Administration's position is as follows:

With respect to Resolution 226 on sharing studies for and possible allocations to the mobile-satellite service (space-to-Earth) in the 1-3 GHz range, including consideration of the band 1 518-1 525 MHz, in Uzbekistan the band 1 429-1 535 MHz is allocated under footnote 5.342 to the aeronautical mobile service on a primary basis exclusively for the purposes of aeronautical telemetry, and the Administration of Uzbekistan thus supports CPM Report Method C, namely no allocation to the mobile-satellite service in the band 1 518-1 525 MHz for Regions 1 and 3.

With respect to Resolution 227 on sharing studies for and possible additional allocations to the mobile-satellite service (Earth-to-space) in the 1-3 GHz range, including consideration of the band 1 683-1 690 MHz, Uzbekistan supports CPM Report Method D, namely no additional allocations within the range 1 670-1 710 MHz to accommodate the mobile-satellite service, since future meteorological systems (MetAids and MetSat) operated as previously will require the whole of the band 1 675-1 690 MHz in order to provide vitally important services in a wide range of applications, including safety of life.

The Administration of the Republic of Uzbekistan requests that the following footnote be included in the Table of Frequency Allocations:

ADD	UZB/7/20

5.375A	Additional allocation: in Uzbekistan, the band 1 645.5-1 660 MHz is also allocated to the fixed service on a primary basis.

Agenda item 1.38

1.38	to consider provision of up to 6 MHz of frequency spectrum to the Earth exploration-satellite service (active) in the frequency band 420-470 MHz, in accordance with Resolution 727 (Rev.WRC-2000)

In Uzbekistan, there are a large number of radio stations operating in this band, and the band is also allocated for the development of mobile communications using the CDMA standard; also, under footnote 5.277, the band 430-440 MHz is allocated in Uzbekistan to the fixed service on a primary basis. Uzbekistan fears that harmful interference may be caused if frequencies are allocated to EESS (active) in the band 420-470 MHz. Therefore, the Administration of the Republic of Uzbekistan supports CPM Report Method B, namely no allocation to the Earth exploration-satellite service in the band 420-470 MHz.

Agenda item 4

4	in accordance with Resolution 95 (Rev.WRC-2000), to review the resolutions and recommendations of previous conferences with a view to their possible revision, replacement or abrogation

Having reviewed the resolutions and recommendations of previous conferences presented in the CPM Report, the Administration of the Republic of Uzbekistan objects to the deletion of:

•	Resolution 63 (Protection of radiocommunication services against interference caused by radiation from industrial, scientific and medical (ISM) equipment), since regulatory provisions are necessary to protect the different services from harmful effects of ISM equipment.

•	Resolution 536 (WRC-97) (Operation of broadcasting satellites serving other countries), since we believe that before satellite broadcasting services are provided to other administrations, the administrations planning to begin providing such services should secure the agreement of the other administrations concerned.

The Administration of the Republic of Uzbekistan also objects to the modification of Resolution 77 (WRC-2000) (Protection of terrestrial services in all regions from geostationary-satellite networks in the fixed-satellite service in Region 2 using the frequency band 11.7-12.2 GHz), since it considers that terrestrial services must be protected from GSO FSS services in the band 11.7-12.2 GHz.

Agenda item 7.2

7.2	to recommend to the Council items for inclusion in the agenda for the next WRC, and to give its views on the preliminary agenda for the subsequent conference and on possible agenda items for future conferences, taking into account Resolution 801 (WRC-2000)

In the light of the issues considered in the agenda of WRC-03, the Administration of the Republic of Uzbekistan proposes that the following item be placed on the agenda of WRC-07:

–	to consider global/regional spectrum to meet the needs of public protection agencies, including those dealing with emergency situations and public safety.