| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Addendum 3 to Document 12-E |
|  |  |  | 29 May 2003 |
|  |  |  | Original: English |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | PLENARY MEETING |
| --- | --- |
| United States of America |  |
| Proposals for the work of the Conference |  |
| Agenda item 1.27 |  |

1.27	to review, in accordance with Resolutions 540 (WRC-2000) and 735 (WRC-2000), the ITU-R studies requested in those resolutions, and modify, as appropriate, the relevant regulatory procedures and associated sharing criteria contained in Appendices 30 and 30A and in the associated provisions

Background Information

The United States in reviewing the proposals submitted to WRC-03 notes that the technical criteria between services in the 11.7-12.7 GHz band are a significant concern to countries in all three Regions. This is evident by the proposals submitted by the APT, the Arab Group, CEPT, CITEL and other proposals submitted individually by other administrations. The United States also recognizes the importance of resolving outstanding issues regarding § 4.1.18 to 4.1.20 of Appendices 30 and 30A.

The United States supports the establishment of a fair, balanced and equitable regulatory approach that addresses the needs of all three Regions. We also expect the regulatory solutions to take into account the specific requirements and conditions of each Region. The United States, therefore, believes that the concept of interregional reciprocity, must not be applied in isolation and in an arbitrary and rigid fashion, but instead it must be carefully considered along with specific regional requirements and conditions.

With regard to Region 2 in particular, the United States notes the need to address the following two issues where differences exists between the proposals submitted to WRC-03:

a)	the sharing of Region 2 FSS and terrestrial services in Regions 1, 2 and 3 in the band 11.7-12.2 GHz; and

b)	protection to Region 2 BSS to 45 cm dishes.

The proposals from some administrations link a) above with resolution of issues concerning the § 4.1.18 to 4.1.20 provisions. In our view, however, the second issue, b), must be considered in the context of the § 4.1.18 to 4.1.20 regulatory provisions. Recognizing the complexity of these issues, we propose two alternative regulatory solutions. The first solution is based on technical, regulatory and geographical characteristics that are relevant to Region 2. The second solution emphasizes the concept of interregional reciprocity. Both solutions are presented below. It is noted that both solutions resolve the concerns raised in the proposals linking a) above to § 4.1.18 to 4.1.20.

Proposed Regulatory Solution 1: A recognition and agreement that Region 2 BSS should be protected to 45 cm receive antennas and that this will not overly constrain the services in other Regions. As is recognized in Resolution 540 (WRC-03) “there are differing geographic situations between the ITU Regions and that this may have an impact on the sharing criteria and therefore should be taken into account in any revision to the sharing criteria in the relevant Annexes of Appendices 30 and 30A”. The situation of protection of Region 2 BSS is unique when one considers the large geographic separation between Region 2 and Region 1 or Region 3, which should result in increased satellite antenna gain isolation into Region 2 from a satellite serving Regions 1 or 3. Furthermore, Region 2 BSS is unique in that it is the only service that is constrained by restrictions on the eastern and westernmost orbital positions that may be used to provide Region 2 BSS, across the entire Region 2 BSS allocation. These constraints, taken together, minimize any potential impact that protection of Region 2 BSS to a smaller minimum diameter could have on the satellite services in the other Regions.

Moreover, Resolution 540 also recognizes the need to provide appropriate protection to all services and notes that BSS networks are in operation. In this regard, Region 2 is again in a somewhat unique circumstance in that is has over ten operational BSS satellites serving over 20 million consumers with 45 cm receive antennas. In general, smaller dishes are used for reception of BSS service than in the FSS service. Given the large number of consumers it is a priority for the United States to ensure protection of these receivers from intra- and interregional interference.

If protection to Region 2 BSS to 45 cm in Annex 4 and § 3 of Annex 1 is agreed then the United States would propose that the § 4.1.18-4.1.20 provisions be applied by Regions 1 and 3 BSS with respect to all Region 2 services. Additionally, the United States would propose that provisions similar to § 4.1.18-4.1.20 be included in the appropriate sections of Appendices 30/30A so that Region 2 BSS could apply these provisions with respect to Regions 1 and 3 services and unplanned services within Region 2.

It is noted that the United States does not agree to the application of provisions similar to § 4.1.18-4.1.20 from Region 2 BSS modifications to the Region 2 BSS Plan or modifications. It is the view of the United States that these types of provisions applied intraregionally between Region 2 BSS would severely constrain Region 2 administrations from developing BSS systems in the future. This would especially affect those administrations that have not yet deployed BSS satellite networks. Unlike the Regions 1 and 3 Plans, the technical parameters of the Region 2 Plans have not been updated recently; as a result, Region 2 administrations implementing their Plan assignments would generally modify their Plan assignments to bring the characteristics up to date before bringing it into use. Also, while WRC-2000 modified Appendices 30 and 30A so that changes cannot be made to Regions 1 and 3 Plan assignments, Section 4.2 of Appendix 30 specifically envisions that Region 2 Plan assignments will be modified as needed. An example of the very detrimental effect that such provision could have if applied intraregionally between Region 2 BSS and its modifications is as follows:

A modification by Administration A to the Plan affects the assignment of Administration B, and Administration B does not give its agreement. Administration A applies § 4.1.18 type provisions. When Administration B wants to deploy a BSS network with different parameters than those appearing in the Plan it would submit a modification to ITU. That modification from Administration B would have no rights vis-à-vis the modification from Administration A and would have to protect and possibly seek the agreement of Administration A to deploy its system.

Proposed Regulatory Solution 2: Alternatively, the United States believes that applicability of § 4.1.18 to 4.1.20 type provisions to BSS services in Regions 1 and 3 should be suppressed and these provisions should apply only with respect to terrestrial services in all Regions. This would lead to similar provisions being applied between satellite services in the 11.7-12.2 GHz band vis-à-vis terrestrial services in this band. This approach would resolve the concerns raised in the ECP proposal for Resolution 77 to this Conference.

Proposals

Add similar provisions to § 4.1.18-4.1.20 to Sections 4.2 of Appendices 30 and 30A with respect to all interregional services in Regions 1 and 3 and with respect to unplanned services in Region 2 operating in the 12.2-12.7 GHz band. Adopt ECP proposal on Appendix 30 Section 4.1 and for Appendix 30A Section 4.1.

APPENDIX  30*  (WRC-2000)

Provisions for all services and associated Plans and List for
the broadcasting-satellite service in the frequency bands
11.7-12.2 GHz (in Region 3), 11.7-12.5 GHz (in Region 1)
             and 12.2-12.7 GHz (in Region 2)     (WRC-2000)

ARTICLE  4     (WRC-2000)

Procedures for modifications to the Region 2 Plan or 
for additional uses in Regions 1 and 32

...

## 4.2	Provisions applicable to Region 2

4.2.1	When an administration intends to make a modification6 to the Region 2 Plan, i.e.:

a)	to modify the characteristics of any of its frequency assignments to a space station in the broadcasting-satellite service which are shown in the Region 2 Plan, or for which the procedure in this Article has been successfully applied, whether or not the station has been brought into use; or

b)	to include in the Region 2 Plan a new frequency assignment to a space station in the broadcasting-satellite service; or

c)	to cancel a frequency assignment to a space station in the broadcasting-satellite service,

the following procedure shall be applied before any notification of the frequency assignment is made to the Bureau (see Article 5).

4.2.2	The term “frequency assignment in conformity with the Plan” used in this and the following Articles is defined in Article 1.

MOD	USA/12/168

4.2.3	An administration proposing a modification to the characteristics of a frequency assignment in conformity with the Region 2 Plan, or the inclusion of a new frequency assignment in that Plan, shall seek the agreement of those administrations:

a)	of Regions 1 and 3 having a frequency assignment to a space station in the broadcasting-satellite service which is in conformity with the Regions 1 and 3 Plan with a necessary bandwidth, any portion of which falls within the necessary bandwidth of the proposed assignment; or

b)	of Regions 1 and 3 having a frequency assignment included in the List or for which complete Appendix 4 information has been received by the Bureau in accordance with the provisions of § 4.1.3, and any portion of which falls within the necessary bandwidth of the proposed assignment; or

c)	having no frequency assignment in the broadcasting-satellite service in the channel concerned, but in whose territory the power flux-density value exceeds the prescribed limit as a result of the proposed modification, or having an assignment whose associated service area does not cover the whole of the territory of the administration, and in whose territory outside that service area the power flux-density from the broadcasting-satellite space station subject to this modification exceeds the prescribed limit as a result of the proposed modification; or

d)	having a frequency assignment in the band 12.5-12.7 GHz in Region 1 or 12.2-12.7 GHz in Region 3 to a space station in the fixed-satellite service which is recorded in the Master Register, or for which complete coordination information has been received by the Bureau for coordination under No. 9.7 or under § 7.1 of Article 7; or

e)	having a frequency assignment to a space station in the broadcasting-satellite service in the band 12.5-12.7 GHz in Region 3 with a necessary bandwidth, any portion of which falls within the necessary bandwidth of the proposed assignment, and:

–	which is recorded in the Master Register; or

–	for which complete coordination information has been received by the Bureau for coordination under No. 9.77 or under § 7.1 of Article 7;

f)	whose services are considered to be affected.

ADD	USA/12/169

4.2.3bis	An administration proposing a modification to the characteristics of a frequency assignment in conformity with the Region 2 Plan, or the inclusion of a new frequency assignment in that Plan, shall seek the agreement of those administrations of Region 2 having a frequency assignment in the Region 2 Plan to a space station in the broadcasting-satellite service in the same

or adjacent channel which is in conformity with that Plan, or in respect of which proposed modifications to that Plan have been received by the Bureau in accordance with the provisions of § 4.2.6 whose services are considered to be affected.

4.2.4	Not used.

4.2.5	The services of an administration are considered to be affected when the limits shown in Annex 1 are exceeded.

MOD	USA/12/170

4.2.6	An administration intending to make a modification to the Region 2 Plan shall send to the Bureau, not earlier than eight years but preferably not later than two years before the date on which the assignment is to be brought into use, the relevant information listed in Appendix 4. Modifications to that Plan shall lapse if the assignment is not brought into use by that date.

4.2.7	If the information received by the Bureau under § 4.2.6 is found to be incomplete, the Bureau shall immediately seek from the administration concerned any clarification required and information not provided.

MOD	USA/12/171

4.2.8	The Bureau shall determine, on the basis of Annex 1, the administrations whose frequency assignments are considered to be affected within the meaning of § 4.2.3 and § 4.2.3bis. The Bureau shall publish8, in a Special Section of its BR IFIC, the complete information received under § 4.2.6, together with the names of the affected administrations, the corresponding fixed-satellite service networks, the corresponding broadcasting-satellite service assignments and terrestrial stations, as appropriate. The Bureau shall immediately send the results of its calculations to the administration proposing the modification to the Region 2 Plan.

4.2.9	The Bureau shall send a telegram/fax to the administrations listed in the Special Section of its BR IFIC drawing their attention to the information it contains and shall send them the results of its calculations.

4.2.10	An administration which considers that it should have been included in the list of administrations whose services are considered to be affected may, giving the technical reasons for so doing, request the Bureau to include its name in the list. The Bureau shall study this request on the basis of Annex 1 and shall send a copy of the request, with an appropriate recommendation, to the administration proposing the modification to the Region 2 Plan.

MOD	USA/12/172

4.2.11	Any modification to a frequency assignment which is in conformity with the Region 2 Plan or any inclusion in that Plan of a new frequency assignment which would have the effect of exceeding the limits specified in Section 2 of Annex 1 shall be subject to the agreement of all administrations deemed affected under § 4.2.3bis.

4.2.12	The administration seeking agreement or the administration with which agreement is sought may request any additional technical information it considers necessary. The administrations shall inform the Bureau of such requests.

4.2.13	Comments from administrations on the information published pursuant to § 4.2.8 should be sent either directly to the administration proposing the modification or through the Bureau. In any event, the Bureau shall be informed that comments have been made.

4.2.14	An administration that has not notified its comments either to the administration seeking agreement or to the Bureau within a period of four months following the date of the BR IFIC referred to in § 4.2.8 shall be deemed to have agreed to the proposed assignment. This time-limit may be extended by up to three months for an administration that has requested additional information under § 4.2.12 or for an administration that has requested the assistance of the Bureau under § 4.2.22. In the latter case, the Bureau shall inform the administrations concerned of this request.

4.2.15	If, in seeking agreement, an administration modifies its initial proposal, it shall again apply the provisions of § 4.2 and the consequent procedure with respect to any other administration whose services might be affected as a result of modifications to the initial proposal.

4.2.16	If no comments have been received on the expiry of the periods specified in § 4.2.14, or if agreement has been reached with the administrations which have made comments and with which agreement is necessary, the administration proposing the modification may continue with the appropriate procedure in Article 5, and shall so inform the Bureau, indicating the final characteristics of the frequency assignment together with the names of the administrations with which agreement has been reached.

4.2.17	The agreement of the administrations affected may also be obtained in accordance with this Article, for a specified period.

4.2.18	When the proposed modification to the Region 2 Plan involves developing countries, administrations shall seek all practicable solutions conducive to the economical development of the broadcasting-satellite systems of these countries.

4.2.19	The Bureau shall publish in a Special Section of its BR IFIC the information received under § 4.2.16 together with the names of any administrations with which the provisions of this Article have been successfully applied. The frequency assignment concerned shall enjoy the same status as those appearing in the Region 2 Plan and will be considered as a frequency assignment in conformity with the Plan.

4.2.20	When an administration proposing to modify the characteristics of a frequency assignment or to make a new frequency assignment receives notice of disagreement on the part of an administration whose agreement it has sought, it should first endeavour to solve the problem by exploring all possible means of meeting its requirement. If the problem still cannot be solved by such means, the administration whose agreement has been sought should endeavour to overcome the difficulties as far as possible, and shall state the technical reasons for any disagreement if the administration seeking the agreement requests it to do so.

4.2.21	If no agreement is reached between the administrations concerned, the Bureau shall carry out any study that may be requested by these administrations; the Bureau shall inform them of the result of the study and shall make such recommendations as it may be able to offer for the solution of the problem.

ADD	USA/12/173

4.2.21.A	If, in spite of the application of § 4.2.20 and 4.2.21, there is still continuing disagreement with administrations deemed affected under § 4.2.3 and the notifying administration insists that the proposed new or modified assignment be included in the Plan, the Bureau shall enter the assignment provisionally in the Plan with an indication of those administrations whose assignments were the basis of the disagreement; however, the entry shall be changed from provisional to definitive recording in the Plan only if the Bureau is informed that the new or modified assignment in the Plan has been in use, together with the assignment which was the basis for the disagreement, for at least four months without any complaint of harmful interference or unacceptable interference, as appropriate, being made.

ADD	USA/12/174

4.2.21.B	When an assignment is entered in the Plan provisionally under § 4.2.21.A, the responsible administration is deemed to have undertaken to eliminate any harmful interference or unacceptable interference, as appropriate, immediately after notification of that interference.

ADD	USA/12/175

4.2.21.C	Should the assignments that were the basis of the disagreement not be brought into use within the period specified in No. 11.44 (for non-planned services), or in § 4.1 (for assignments in the Plan, or List or having initiated the procedure under § 4.1), as appropriate, then the status of the assignment in the Plan shall be reviewed accordingly.

ADD	USA/12/176

4.2.21.D	Should harmful interference be caused by an assignment included in the Plan under § 4.2.21.A to any recorded assignment in the Master Register which was the basis of the disagreement and is not subject to a Plan, the administration using the frequency assignment included in the Plan under § 4.2.21.A shall, upon receipt of advice thereof, immediately eliminate this harmful interference. Should unacceptable interference be caused by an assignment included in the Plan under § 4.2.21.A to an assignment in Regions 1 and 3 Plan or List which was the basis of the disagreement and is operated in conformity with this Plan or List, as appropriate, the administration using the frequency assignment included in the Plan under § 4.2.21.A shall, upon receipt of advice thereof, immediately eliminate this unacceptable interference.

4.2.22	An administration may at any stage in the procedure described, or before applying it, request the assistance of the Bureau.

4.2.23	The relevant provisions of Article 5 shall be applied when frequency assignments are notified to the Bureau.

## 4.2.24	Cancellation of frequency assignments

When a frequency assignment in conformity with the Region 2 Plan is no longer required, whether or not as a result of a modification, the administration concerned shall immediately so inform the Bureau. The Bureau shall publish this information in a Special Section of its BR IFIC and delete the assignment from the Region 2 Plan.

## 4.2.25	Master copy of the Region 2 Plan

4.2.25.1	The Bureau shall maintain an up-to-date master copy of the Region 2 Plan, including the overall equivalent protection margins of each assignment, taking account of the application of the procedure set out in this Article. This master copy shall contain the overall equivalent protection margins derived from the Plan as established by the 1983 Conference and those derived from all modifications to the Plan as a result of the successful completion of the modification procedure set out in this Article.

4.2.25.2	An up-to-date version of the Region 2 Plan shall be published by the Secretary-General when justified by the circumstances.

APPENDIX  30A  (WRC-2000)

Provisions and associated Plans and Lists1 for feeder links for the broadcasting-satellite service (11.7-12.5 GHz in Region 1, 12.2-12.7 GHz
in Region 2 and 11.7-12.2 GHz in Region 3) in the frequency
bands 14.5-14.8 GHz2 and 17.3-18.1 GHz in Regions 1 and 3,
           and 17.3-17.8 GHz in Region 2     (WRC-2000)

ARTICLE  4     (WRC-2000)

Procedures for modifications to the Region 2 feeder-link Plan 
or for additional uses in Regions 1 and 3

...

## 4.2	Provisions applicable to Region 2

4.2.1	When an administration intends to make a modification to the Region 2 feeder-link Plan, i.e.:

a)	to modify the characteristics of any of its frequency assignments in the fixed-satellite service which are shown in the Region 2 feeder-link Plan, or for which the procedure in this Article has been successfully applied, whether or not the station has been brought into use; or

b)	to include in the Region 2 feeder-link Plan a new frequency assignment in the fixed-satellite service; or

c)	to cancel a frequency assignment in the fixed-satellite service,

the following procedure shall be applied before any notification of the frequency assignment is made to the Bureau (see Article 5 and Resolution 42 (Rev.Orb-88)).

MOD	USA/12/177

4.2.2	An administration proposing a modification to the characteristics of a frequency assignment in conformity with the Region 2 feeder-link Plan, or the inclusion of a new frequency assignment in that Plan, shall seek the agreement of those administrations8, 9:

a)	having an assignment for feeder-links in the fixed-satellite service (Earth-to-space) which is in conformity with the Regions 1 and 3 feeder-link Plan with the necessary bandwidth, any portion of which falls within the necessary bandwidth of the proposed assignment; or

b)	of Regions 1 and 3 having a feeder-link frequency assignment included in the feeder-link Lists or for which complete Appendix 4 information has been received by the Bureau in accordance with the provisions of § 4.1.3, and any portion of which falls within the necessary bandwidth of the proposed assignment; or

c)	which are considered affected.

ADD	USA/12/178

4.2.2bis	An administration proposing a modification to the characteristics of a frequency assignment in conformity with the Region 2 feeder-link Plan, or the inclusion of a new frequency assignment in that Plan, shall seek the agreement of those administrations8, 9 of Region 2 having a feeder-link frequency assignment in the fixed-satellite service (Earth-to-space) in the same channel or an adjacent channel, which appears in the Region 2 feeder-link Plan or in respect of which proposed modifications to this Plan have been received by the Bureau in accordance with the provisions of § 4.2.6 whose services are considered to be affected.

4.2.3	(Not used.)

4.2.4	The services of an administration are considered to be affected when the limits shown in Annex 1 are exceeded.

MOD	USA/12/179

4.2.5	The agreement referred to in § 4.2.2 and § 4.2.2bis is not required when an administration proposes to bring into use, with characteristics appearing in the Region 2 feeder-link Plan, a fixed feeder-link earth station in the band 17.3-17.8 GHz or a transportable feeder-link earth station in the band 17.3-17.7 GHz. Administrations may communicate to the Bureau the characteristics of such earth stations for inclusion in this Plan.

MOD	USA/12/180

4.2.6	An administration intending to make a modification to the Region 2 feeder-link Plan shall send to the Bureau, not earlier than eight years but preferably not later than two years before the date on which the assignment is to be brought into use, the relevant information listed in Appendix 4. Modifications to that Plan shall lapse if the assignment is not brought into use by that date.

4.2.7	If the information received by the Bureau under § 4.2.6 is found to be incomplete, the Bureau shall immediately seek from the administration concerned any clarification required and information not provided.

MOD	USA/12/181

4.2.8	The Bureau shall determine, on the basis of Annex 1, the administrations whose frequency assignments are considered to be affected within the meaning of § 4.2.2 and § 4.2.2bis. The Bureau shall publish10, in a Special Section of its BR IFIC, the complete information received under § 4.2.6, together with the names of the affected administrations, the corresponding fixed-satellite service networks and the corresponding feeder links to broadcasting-satellite service assignments, as appropriate. The Bureau shall immediately send the results of its calculations to the administration proposing the modification to the Region 2 feeder-link Plan.

4.2.9	The Bureau shall send a telegram/fax to the administrations listed in the Special Section of its BR IFIC drawing their attention to the information it contains and shall send them the results of its calculations.

4.2.10	An administration which considers that it should have been included in the list of administrations whose services are considered to be affected may, giving the technical reasons for so doing, request the Bureau to include its name in the list. The Bureau shall study this request on the basis of Annex 1 and shall send a copy of the request, with an appropriate recommendation, to the administration proposing the modification to the Region 2 feeder-link Plan.

MOD	USA/12/182

4.2.11	Any modification to a frequency assignment which is in conformity with the Region 2 feeder-link Plan or any inclusion in that Plan of a new frequency assignment which would have the effect of exceeding the limits specified in Section 3 of Annex 1 shall be subject to the agreement of all administrations deemed affected under § 4.2.2bis.

4.2.12	The administration seeking agreement or the administration with which agreement is sought may request any additional technical information it considers necessary. The administrations shall inform the Bureau of such requests.

4.2.13	Comments from administrations on the information published pursuant to § 4.2.8 should be sent either directly to the administration proposing the modification or through the Bureau. In any event, the Bureau shall be informed that comments have been made.

4.2.14	An administration which has not notified its comments either to the administration seeking agreement or to the Bureau within a period of four months following the date of the BR IFIC referred to in § 4.2.8 shall be deemed to have agreed to the proposed modification. This time-limit may be extended by up to three months for an administration which has requested additional information under § 4.2.12 or for an administration which has requested the assistance of the Bureau under § 4.2.22. In the latter case, the Bureau shall inform the administrations concerned of this request.

4.2.15	If, in seeking agreement, an administration modifies its initial proposal, it shall again apply the provisions of § 4.2 and the consequent procedure with respect to any other administration whose services might be affected as a result of modifications to the initial proposal.

4.2.16	If no comments have been received on the expiry of the periods specified in § 4.2.14, or if agreement has been reached with the administrations which have made comments and with which agreement is necessary, the administration proposing the modification may continue with the appropriate procedure in Article 5, and shall so inform the Bureau, indicating the final characteristics of the frequency assignment together with the names of the administrations with which agreement has been reached.

4.2.17	The agreement of the administrations affected may also be obtained in accordance with this Article, for a specified period.

4.2.18	When the proposed modification to the Region 2 feeder-link Plan involves developing countries, administrations shall seek all practicable solutions conducive to the economical development of the broadcasting-satellite systems of these countries.

4.2.19	The Bureau shall publish in a Special Section of its BR IFIC the information received under § 4.2.16 together with the names of any administrations with which the provisions of this Article have been successfully applied. The frequency assignment concerned shall enjoy the same status as those appearing in the Region 2 feeder-link Plan and will be considered as a frequency assignment in conformity with that Plan.

4.2.20	When an administration proposing to modify the characteristics of a frequency assignment or to make a new frequency assignment receives notice of disagreement on the part of an administration whose agreement it has sought, it should first endeavour to solve the problem by exploring all possible means of meeting its requirement. If the problem still cannot be solved by such means, the administration whose agreement has been sought should endeavour to overcome the difficulties as far as possible, and shall state the technical reasons for any disagreement if the administration seeking the agreement requests it to do so.

4.2.21	If no agreement is reached between the administrations concerned, the Bureau shall carry out any study that may be requested by these administrations; the Bureau shall inform them of the result of the study and shall make such recommendations as it may be able to offer for the solution of the problem.

ADD	USA/12/183

4.2.21.A	If, in spite of the application of § 4.2.20 and 4.2.21, there is still continuing disagreement with administrations deemed affected under § 4.2.2 and the notifying administration insists that the proposed new or modified assignment be included in the Plan, the Bureau shall enter the assignment provisionally in the Plan with an indication of those administrations whose

assignments were the basis of the disagreement; however, the entry shall be changed from provisional to definitive recording in the Plan only if the Bureau is informed that the new or modified assignment in the Plan has been in use, together with the assignment which was the basis for the disagreement, for at least four months without any complaint of harmful interference or unacceptable interference, as appropriate, being made.

ADD	USA/12/184

4.2.21.B	When an assignment is entered in the Plan provisionally under § 4.2.21.A, the responsible administration is deemed to have undertaken to eliminate any harmful interference or unacceptable interference, as appropriate, immediately after notification of that interference.

ADD	USA/12/185

4.2.21.C	Should the assignments that were the basis of the disagreement not be brought into use within the period specified in No. 11.44 (for non-planned services), or in § 4.1 (for assignments in the Plan, or List or having initiated the procedure under § 4.1), as appropriate, then the status of the assignment in the Plan shall be reviewed accordingly.

ADD	USA/12/186

4.2.21.D	Should harmful interference be caused by an assignment included in the Plan under § 4.2.21.A to any recorded assignment in the Master Register which was the basis of the disagreement and is not subject to a Plan, the administration using the frequency assignment included in the Plan under § 4.2.21.A shall, upon receipt of advice thereof, immediately eliminate this harmful interference. Should unacceptable interference be caused by an assignment included in the Plan under § 4.2.21.A to an assignment in Regions 1 and 3 Plan or List which was the basis of the disagreement and is operated in conformity with this Plan or List, as appropriate, the administration using the frequency assignment included in the Plan under § 4.2.21.A shall, upon receipt of advice thereof, immediately eliminate this unacceptable interference.

4.2.22	An administration may at any stage in the procedure described, or before applying it, request the assistance of the Bureau.

4.2.23	The relevant provisions of Article 5 shall be applied when frequency assignments are notified to the Bureau.

### 4.2.24	Cancellation of frequency assignments

When a frequency assignment in conformity with the Region 2 feeder-link Plan is no longer required, whether or not as a result of a modification, the administration concerned shall immediately so inform the Bureau. The Bureau shall publish this information in a Special Section of its BR IFIC and delete the assignment from the Region 2 feeder-link Plan.

### 4.2.25	Master copy of the Region 2 feeder-link Plan

4.2.25.1	The Bureau shall maintain an up-to-date master copy of the Region 2 feeder-link Plan, including the overall equivalent protection margins of each assignment, taking account of the application of the procedure set out in this Article. This master copy shall contain the overall equivalent protection margins derived from that Plan as established by the 1983 Conference and those derived from all modifications to that Plan as a result of the successful completion of the modification procedure set out in this Article.

4.2.25.2	An up-to-date version of the Region 2 feeder-link Plan shall be published by the Secretary-General when justified by the circumstances.

Revise Sections 4.1 and 4.2 of Appendices 30 and 30A to apply § 4.1.18-4.1.20 provisions with respect to terrestrial services only.

APPENDIX  30*  (WRC-2000)

Provisions for all services and associated Plans and List for
the broadcasting-satellite service in the frequency bands
11.7-12.2 GHz (in Region 3), 11.7-12.5 GHz (in Region 1)
             and 12.2-12.7 GHz (in Region 2)     (WRC-2000)

ARTICLE  4     (WRC-2000)

Procedures for modifications to the Region 2 Plan or 
for additional uses in Regions 1 and 32

## 4.1	Provisions applicable to Regions 1 and 3

MOD	USA/12/187

4.1.1	An administration proposing to include a new or modified assignment in the List shall seek the agreement of those administrations whose services are considered to be affected, i.e. administrations:

a)	of Regions 1 and 3 having a frequency assignment to a space station in the broadcasting-satellite service which is included in the Regions 1 and 3 Plan with a necessary bandwidth, any portion of which falls within the necessary bandwidth of the proposed assignment; or

b)	of Regions 1 and 3 having a frequency assignment included in the List or for which complete Appendix 4 information has been received by the Radiocommunication Bureau in accordance with the provisions of § 4.1.3, and any portion of which falls within the necessary bandwidth of the proposed assignment; or

c)	of Region 2 having a frequency assignment to a space station in the broadcasting-satellite service which is in conformity with the Region 2 Plan, or in respect of which proposed modifications to that Plan have been received by the Bureau in accordance with the provisions of § 4.2.6 with a necessary bandwidth, any portion of which falls within the necessary bandwidth of the proposed assignment; or

d)	having a frequency assignment in the band 11.7-12.2 GHz in Region 2 or 12.2-12.5 GHz in Region 3 to a space station in the fixed-satellite service which is recorded in the Master International Frequency Register (Master Register) or for which complete coordination information has been received by the Bureau for coordination under No. 9.7, or under § 7.1 of Article 7.

ADD	USA/12/188

4.1.1bis	An administration proposing to include a new or modified assignment in the List shall seek the agreement of those administrations whose services are considered to be affected, i.e. administrations having no frequency assignment in the broadcasting-satellite service with a necessary bandwidth, any portion of which falls within the necessary bandwidth of the proposed assignment, but in whose territory the power flux-density value exceeds the prescribed limit as a result of the proposed assignment, or having an assignment whose associated service area does not cover the whole of the territory of the administration, and in whose territory outside that service area the power flux-density from the proposed assignment exceeds the prescribed limit as a result of the proposed assignment.

4.1.2	The services of an administration are considered to be affected when the limits shown in Annex 1 are exceeded.

4.1.3	An administration intending to include a new or modified assignment in the List shall send to the Bureau, not earlier than eight years but preferably not later than two years before the date on which the assignment is to be brought into use, the relevant information listed in Appendix 4. An assignment in the List shall lapse if it is not brought into use by that date.3

4.1.4	If the information received by the Bureau under § 4.1.3 is found to be incomplete, the Bureau shall immediately seek from the administration concerned any clarification required and information not provided.

MOD	USA/12/189

4.1.5	The Bureau shall determine, on the basis of Annex 1, the administrations whose frequency assignments are considered to be affected within the meaning of § 4.1.1 and § 4.1.1bis. The Bureau shall publish4, in a Special Section of its International Frequency Information Circular (BR IFIC), the complete information received under § 4.1.3, together with the names of the affected administrations, the corresponding fixed-satellite service networks, the corresponding broadcasting-satellite service assignments and terrestrial stations, as appropriate. The Bureau shall immediately send the results of its calculations to the administration proposing the assignment.

4.1.6	The Bureau shall send a telegram/fax to the administrations listed in the Special Section of the BR IFIC drawing their attention to the information it contains, and shall send them the results of its calculations.

4.1.7	An administration which considers that it should have been identified in the publication referred to under § 4.1.5 above shall, within four months of the date of publication of its relevant BR IFIC, and giving the technical reasons for so doing, request the Bureau to include its name in the publication. The Bureau shall study this information on the basis of Annex 1 and shall inform both administrations of its conclusions. Should the Bureau agree to the administration’s request, it shall publish an addendum to the publication under § 4.1.5.

ADD	USA/12/190

4.1.7bis	Any new or modified assignment in the List which would have the effect of exceeding the limits specified in Annex 1 shall be subject to the agreement of all administrations deemed affected under § 4.1.1.

4.1.8	The administration seeking agreement or the administration with which agreement is sought may request any additional technical information it considers necessary. The administrations shall inform the Bureau of such requests.

4.1.9	Comments from administrations on the information published pursuant to § 4.1.5 should be sent either directly to the administration proposing the modification or through the Bureau. In any event, the Bureau shall be informed that comments have been made.

4.1.10	An administration that has not notified its comments either to the administration seeking agreement or to the Bureau within a period of four months following the date of its BR IFIC referred to in § 4.1.5 shall be deemed to have agreed to the proposed assignment. This time-limit may be extended:

–	for an administration that has requested additional information under § 4.1.8, by up to three months; or

–	for an administration that has requested the assistance of the Bureau under § 4.1.21, by up to three months following the date at which the Bureau communicated the result of its action.

4.1.11	If, in seeking agreement, an administration modifies its initial proposal, it shall again apply the provisions of § 4.1 and the consequent procedure with respect to any other administration whose services might be affected as a result of modifications to the initial proposal.

4.1.12	If no comments have been received on the expiry of the periods specified in § 4.1.10, or if agreement has been reached with the administrations which have made comments and with which agreement is necessary, the administration proposing the new or modified assignment may continue with the appropriate procedure in Article 5, and shall so inform the Bureau, indicating the final characteristics of the frequency assignment together with the names of the administrations with which agreement has been reached.

4.1.13	The agreement of the administrations affected may also be obtained in accordance with this Article, for a specified period.

4.1.14	Where the proposed assignment involves developing countries, administrations shall seek all practicable solutions conducive to the economical development of the broadcasting-satellite systems of these countries.

4.1.15	The Bureau shall publish in a Special Section of its BR IFIC the information received under § 4.1.12, together with the names of any administrations with which the provisions of this Article have been successfully applied. The frequency assignment concerned shall be included in the List.

4.1.16	In case of disagreement on the part of an administration whose agreement has been sought, the requesting administration should first endeavour to solve the problem by exploring all possible means of meeting its requirement. If the problem still cannot be solved by such means, the administration whose agreement has been sought should endeavour to overcome the difficulties as far as possible, and shall state the technical reasons for any disagreement if the administration seeking the agreement requests it to do so.

4.1.17	If no agreement is reached between the administrations concerned, the Bureau shall carry out any study that may be requested by either one of these administrations; the Bureau shall inform them of the result of the study and shall make such recommendations as it may be able to offer for the solution of the problem.

MOD	USA/12/191

4.1.18	If, in spite of the application of § 4.1.16 and 4.1.17, there is still continuing disagreement with administrations deemed affected under § 4.1.1bis and the notifying administration insists that the proposed assignment be included in the List, the Bureau shall enter the assignment provisionally in the List with an indication of those administrations whose assignments were the basis of the disagreement; however, the entry shall be changed from provisional to definitive recording in the List only if the Bureau is informed that the new assignment in the List has been in use, together with the assignment which was the basis for the disagreement, for at least four months without any complaint of harmful interference being made.

4.1.18bis	When an assignment is entered in the List provisionally, the responsible administration is deemed to have undertaken to eliminate any harmful interference immediately after notification of that interference.

MOD	USA/12/192

4.1.19	Should the assignments that were the basis of the disagreement not be brought into use within the period specified in No. 11.44 (for non-planned services), then the status of the assignment in the List shall be reviewed accordingly.

4.1.20	Should harmful interference be caused by an assignment included in the List under § 4.1.18 to any recorded assignment in the Master Register which was the basis of the disagreement, the administration using the frequency assignment included in the List under § 4.1.18 shall, upon receipt of advice thereof, immediately eliminate this harmful interference.

4.1.21	An administration may, at any stage in the procedure described, or before applying it, request the assistance of the Bureau.

4.1.22	The relevant provisions of Article 5 shall be applied when frequency assignments are notified to the Bureau.

4.1.23	When a frequency assignment included in the List is no longer required, the administration concerned shall immediately so inform the Bureau. The Bureau shall publish this information in a Special Section of its BR IFIC and delete the assignment from the List.

4.1.24	No assignment in the List shall have a period of operation exceeding 15 years, counted from the date of bringing into use, or 2 June 2000, whichever is later. Upon request by the responsible administration received by the Bureau at the latest three years before the expiry of this period, this period may be extended by up to 15 years, on condition that all the characteristics of the assignment remain unchanged.

4.1.25	Where an administration already having included in the List two assignments (not including those systems notified on behalf of a group of named administrations and included in the List by WRC-2000), in the same channel and covering the same service area, proposes to include in the List a new assignment in the same channel over this same service area, it shall apply the following in respect of another administration which has no assignment in the List in the same channel and which proposes to include in the List a new assignment:

a)	if the agreement of the former administration is required following the application of § 4.1 by the latter administration, in order to protect the new assignment proposed by the former administration from interference caused by the assignment proposed by the latter administration, both administrations shall make every possible effort to resolve the difficulties by means of mutually acceptable adjustments to their networks;

b)	in case of continuing disagreement, and if the former administration has not communicated to the Bureau the information specified in Annex 2 to Resolution 49 (Rev.WRC-2000), this administration shall be deemed to have given its agreement to inclusion in the List of the assignment of the latter administration.

4.1.26	This procedure may be applied by the administration of a new ITU Member State in order to include new assignments in the List. Upon completion of the procedure, the next World Radiocommunication Conference may be requested to consider, among the assignments included in the List after the successful completion of this procedure, the inclusion in the Plan of up to 10 channels (for Region 1) and up to 12 channels (for Region 3), over the national territory of the new Member State.

4.1.27	When an administration has successfully applied this procedure and received all the agreements5 required to include in the List assignments over its national territory, at an orbital location and/or in channels different from those appearing in the Plan for its country, it may request the next world radiocommunication conference to consider the inclusion in the Plan of up to 10 (for Region 1) and up to 12 (for Region 3) of these assignments, in replacement of its assignments appearing in the Plan.

4.1.28	The List, as updated, shall be published periodically by the Bureau.

4.1.29	New or modified assignments in the List shall be limited to digital modulation.

## 4.2	Provisions applicable to Region 2

4.2.1	When an administration intends to make a modification6 to the Region 2 Plan, i.e.:

a)	to modify the characteristics of any of its frequency assignments to a space station in the broadcasting-satellite service which are shown in the Region 2 Plan, or for which the procedure in this Article has been successfully applied, whether or not the station has been brought into use; or

b)	to include in the Region 2 Plan a new frequency assignment to a space station in the broadcasting-satellite service; or

c)	to cancel a frequency assignment to a space station in the broadcasting-satellite service,

the following procedure shall be applied before any notification of the frequency assignment is made to the Bureau (see Article 5).

4.2.2	The term “frequency assignment in conformity with the Plan” used in this and the following Articles is defined in Article 1.

MOD	USA/12/193

4.2.3	An administration proposing a modification to the characteristics of a frequency assignment in conformity with the Region 2 Plan, or the inclusion of a new frequency assignment in that Plan, shall seek the agreement of those administrations:

a)	of Regions 1 and 3 having a frequency assignment to a space station in the broadcasting-satellite service which is in conformity with the Regions 1 and 3 Plan with a necessary bandwidth, any portion of which falls within the necessary bandwidth of the proposed assignment; or

b)	of Regions 1 and 3 having a frequency assignment included in the List or for which complete Appendix 4 information has been received by the Bureau in accordance with the provisions of § 4.1.3, and any portion of which falls within the necessary bandwidth of the proposed assignment; or

c)	of Region 2 having a frequency assignment in the Region 2 Plan to a space station in the broadcasting-satellite service in the same or adjacent channel which is in conformity with that Plan, or in respect of which proposed modifications to that Plan have been received by the Bureau in accordance with the provisions of § 4.2.6; or

d)	having a frequency assignment in the band 12.5-12.7 GHz in Region 1 or 12.2-12.7 GHz in Region 3 to a space station in the fixed-satellite service which is recorded in the Master Register, or for which complete coordination information has been received by the Bureau for coordination under No. 9.7 or under § 7.1 of Article 7; or

e)	having a frequency assignment to a space station in the broadcasting-satellite service in the band 12.5-12.7 GHz in Region 3 with a necessary bandwidth, any portion of which falls within the necessary bandwidth of the proposed assignment, and:

–	which is recorded in the Master Register; or

–	for which complete coordination information has been received by the Bureau for coordination under No. 9.77 or under § 7.1 of Article 7;

f)	whose services are considered to be affected.

ADD	USA/12/194

4.2.3bis	An administration proposing a modification to the characteristics of a frequency assignment in conformity with the Region 2 Plan, or the inclusion of a new frequency assignment in that Plan, shall seek the agreement of those administrations having no frequency assignment in the broadcasting-satellite service in the channel concerned, but in whose territory the power flux-density value exceeds the prescribed limit as a result of the proposed modification, or having an assignment whose associated service area does not cover the whole of the territory of the administration, and in whose territory outside that service area the power flux-density from the broadcasting-satellite space station subject to this modification exceeds the prescribed limit as a result of the proposed modification whose services are considered to be affected.

4.2.4	Not used.

4.2.5	The services of an administration are considered to be affected when the limits shown in Annex 1 are exceeded.

MOD	USA/12/195

4.2.6	An administration intending to make a modification to the Region 2 Plan shall send to the Bureau, not earlier than eight years but preferably not later than two years before the date on which the assignment is to be brought into use, the relevant information listed in Appendix 4. Modifications to that Plan shall lapse if the assignment is not brought into use by that date.

4.2.7	If the information received by the Bureau under § 4.2.6 is found to be incomplete, the Bureau shall immediately seek from the administration concerned any clarification required and information not provided.

MOD	USA/12/196

4.2.8	The Bureau shall determine, on the basis of Annex 1, the administrations whose frequency assignments are considered to be affected within the meaning of § 4.2.3 and § 4.2.3bis. The Bureau shall publish8, in a Special Section of its BR IFIC, the complete information received under § 4.2.6, together with the names of the affected administrations, the corresponding fixed-satellite service networks, the corresponding broadcasting-satellite service assignments and terrestrial stations, as appropriate. The Bureau shall immediately send the results of its calculations to the administration proposing the modification to the Region 2 Plan.

4.2.9	The Bureau shall send a telegram/fax to the administrations listed in the Special Section of its BR IFIC drawing their attention to the information it contains and shall send them the results of its calculations.

4.2.10	An administration which considers that it should have been included in the list of administrations whose services are considered to be affected may, giving the technical reasons for so doing, request the Bureau to include its name in the list. The Bureau shall study this request on the basis of Annex 1 and shall send a copy of the request, with an appropriate recommendation, to the administration proposing the modification to the Region 2 Plan.

MOD	USA/12/197

4.2.11	Any modification to a frequency assignment which is in conformity with the Region 2 Plan or any inclusion in that Plan of a new frequency assignment which would have the effect of exceeding the limits specified in Annex 1 shall be subject to the agreement of all administrations deemed affected under § 4.2.3.

4.2.12	The administration seeking agreement or the administration with which agreement is sought may request any additional technical information it considers necessary. The administrations shall inform the Bureau of such requests.

4.2.13	Comments from administrations on the information published pursuant to § 4.2.8 should be sent either directly to the administration proposing the modification or through the Bureau. In any event, the Bureau shall be informed that comments have been made.

4.2.14	An administration that has not notified its comments either to the administration seeking agreement or to the Bureau within a period of four months following the date of the BR IFIC referred to in § 4.2.8 shall be deemed to have agreed to the proposed assignment. This time-limit may be extended by up to three months for an administration that has requested additional information under § 4.2.12 or for an administration that has requested the assistance of the Bureau under § 4.2.22. In the latter case, the Bureau shall inform the administrations concerned of this request.

4.2.15	If, in seeking agreement, an administration modifies its initial proposal, it shall again apply the provisions of § 4.2 and the consequent procedure with respect to any other administration whose services might be affected as a result of modifications to the initial proposal.

4.2.16	If no comments have been received on the expiry of the periods specified in § 4.2.14, or if agreement has been reached with the administrations which have made comments and with which agreement is necessary, the administration proposing the modification may continue with the appropriate procedure in Article 5, and shall so inform the Bureau, indicating the final characteristics of the frequency assignment together with the names of the administrations with which agreement has been reached.

4.2.17	The agreement of the administrations affected may also be obtained in accordance with this Article, for a specified period.

4.2.18	When the proposed modification to the Region 2 Plan involves developing countries, administrations shall seek all practicable solutions conducive to the economical development of the broadcasting-satellite systems of these countries.

4.2.19	The Bureau shall publish in a Special Section of its BR IFIC the information received under § 4.2.16 together with the names of any administrations with which the provisions of this Article have been successfully applied. The frequency assignment concerned shall enjoy the same status as those appearing in the Region 2 Plan and will be considered as a frequency assignment in conformity with the Plan.

4.2.20	When an administration proposing to modify the characteristics of a frequency assignment or to make a new frequency assignment receives notice of disagreement on the part of an administration whose agreement it has sought, it should first endeavour to solve the problem by exploring all possible means of meeting its requirement. If the problem still cannot be solved by such means, the administration whose agreement has been sought should endeavour to overcome the difficulties as far as possible, and shall state the technical reasons for any disagreement if the administration seeking the agreement requests it to do so.

4.2.21	If no agreement is reached between the administrations concerned, the Bureau shall carry out any study that may be requested by these administrations; the Bureau shall inform them of the result of the study and shall make such recommendations as it may be able to offer for the solution of the problem.

ADD	USA/12/198

4.2.21.A	If, in spite of the application of § 4.2.20 and 4.2.21, there is still continuing disagreement with administrations deemed affected under § 4.2.3bis and the notifying administration insists that the proposed new or modified assignment be included in the Plan, the Bureau shall enter the assignment provisionally in the Plan with an indication of those administrations whose assignments were the basis of the disagreement; however, the entry shall be changed from provisional to definitive recording in the Plan only if the Bureau is informed that the new or modified assignment in the Plan has been in use, together with the assignment which was the basis for the disagreement, for at least four months without any complaint of harmful interference being made.

ADD	USA/12/199

4.2.21.B	When an assignment is entered in the Plan provisionally under § 4.2.21.A, the responsible administration is deemed to have undertaken to eliminate any harmful interference immediately after notification of that interference.

ADD	USA/12/200

4.2.21.C	Should the assignments that were the basis of the disagreement not be brought into use within the period specified in No. 11.44 (for non-planned services) then the status of the assignment in the Plan shall be reviewed accordingly.

ADD	USA/12/201

4.2.21.D	Should harmful interference be caused by an assignment included in the Plan under § 4.2.21.A to any recorded assignment in the Master Register which was the basis of the disagreement, the administration using the frequency assignment included in the Plan under § 4.2.21.A shall, upon receipt of advice thereof, immediately eliminate this harmful interference.

4.2.22	An administration may at any stage in the procedure described, or before applying it, request the assistance of the Bureau.

4.2.23	The relevant provisions of Article 5 shall be applied when frequency assignments are notified to the Bureau.

## 4.2.24	Cancellation of frequency assignments

When a frequency assignment in conformity with the Region 2 Plan is no longer required, whether or not as a result of a modification, the administration concerned shall immediately so inform the Bureau. The Bureau shall publish this information in a Special Section of its BR IFIC and delete the assignment from the Region 2 Plan.

## 4.2.25	Master copy of the Region 2 Plan

4.2.25.1	The Bureau shall maintain an up-to-date master copy of the Region 2 Plan, including the overall equivalent protection margins of each assignment, taking account of the application of the procedure set out in this Article. This master copy shall contain the overall equivalent protection margins derived from the Plan as established by the 1983 Conference and those derived from all modifications to the Plan as a result of the successful completion of the modification procedure set out in this Article.

4.2.25.2	An up-to-date version of the Region 2 Plan shall be published by the Secretary-General when justified by the circumstances.

APPENDIX  30A  (WRC-2000)

Provisions and associated Plans and Lists1 for feeder links for the broadcasting-satellite service (11.7-12.5 GHz in Region 1, 12.2-12.7 GHz
in Region 2 and 11.7-12.2 GHz in Region 3) in the frequency
bands 14.5-14.8 GHz2 and 17.3-18.1 GHz in Regions 1 and 3,
           and 17.3-17.8 GHz in Region 2     (WRC-2000)

ARTICLE  4     (WRC-2000)

Procedures for modifications to the Region 2 feeder-link Plan 
or for additional uses in Regions 1 and 3

## 4.1	Provisions applicable to Regions 1 and 3

4.1.1	An administration proposing to include a new or modified assignment in the feeder-link Lists shall seek the agreement of those administrations whose services are considered to be affected, i.e. administrations4:

a)	of Regions 1 and 3 having a feeder-link frequency assignment in the fixed-satellite service (Earth-to-space) to a space station in the broadcasting-satellite service which is included in the Regions 1 and 3 feeder-link Plan with a necessary bandwith, any portion of which falls within the necessary bandwith of the proposed assignment; or

b)	of Regions 1 and 3 having a feeder-link frequency assignment included in the feeder-link Lists or for which complete Appendix 4 information has been received by the Radiocommunication Bureau in accordance with the provisions of § 4.1.3, and any portion of which falls within the necessary bandwidth of the proposed assignment; or

c)	of Region 2 having a feeder-link frequency assignment in the fixed-satellite service (Earth-to-space) to a space station in the broadcasting-satellite service which is in conformity with the Region 2 feeder-link Plan, or in respect of which proposed modifications to that Plan have already been received by the Bureau in accordance with the provisions of § 4.2.6 with a necessary bandwidth, any portion of which falls within the necessary bandwidth of the proposed assignment; or

d)	of Region 2 having a feeder-link frequency assignment in the band 17.8-18.1 GHz in the fixed-satellite service (Earth-to-space) to a space station in the broadcasting-satellite service which is recorded in the Master Register or which has been coordinated or is being coordinated under the provisions of No. 9.7, or under § 7.1 of Article 7, with a necessary bandwidth, any portion of which falls within the necessary bandwidth of the proposed assignment.

4.1.2	The services of an administration are considered to be affected when the limits shown in Annex 1 are exceeded.

4.1.3	An administration intending to include a new or modified assignment in the feeder-link Lists shall send to the Bureau, not earlier than eight years but preferably not later than two years before the date on which the assignment is to be brought into use, the relevant information listed in Appendix 4. An assignment in the feeder-link Lists shall lapse if it is not brought into use by that date.5

4.1.4	If the information received by the Bureau under § 4.1.3 is found to be incomplete, the Bureau shall immediately seek from the administration concerned any clarification required and information not provided.

4.1.5	The Bureau shall determine, on the basis of Annex 1, the administrations whose frequency assignments are considered to be affected. The Bureau shall publish6, in a Special Section of its International Frequency Information Circular (BR IFIC), the complete information received under § 4.1.3, together with the names of the affected administrations, the corresponding fixed-satellite service networks and the corresponding feeder-links to broadcasting-satellite service assignments, as appropriate. The Bureau shall immediately send the results of its calculations to the administration proposing the assignment.

4.1.6	The Bureau shall send a telegram/fax to the administrations listed in the Special Section of its BR IFIC drawing their attention to the information it contains, and shall send them the results of its calculations.

4.1.7	An administration which considers that it should have been identified in the publication referred to under § 4.1.5 above shall, within four months of the date of publication of its relevant BR IFIC, and giving the technical reasons for so doing, request the Bureau to include its name in the publication. The Bureau shall study this information on the basis of Annex 1 and shall inform both administrations of its conclusions. Should the Bureau agree to the administration’s request, it shall publish an addendum to the publication under § 4.1.5.

ADD	USA/12/202

4.1.7bis	Any inclusion in that List of a modified or new frequency assignment which would have the effect of exceeding the limits specified in Annex 1 shall be subject to the agreement of all affected administrations.

4.1.8	The administration seeking agreement or the administration with which agreement is sought may request any additional technical information it considers necessary. The administrations shall inform the Bureau of such requests.

4.1.9	Comments from administrations on the information published pursuant to § 4.1.5 should be sent either directly to the administration proposing the modification or through the Bureau. In any event, the Bureau shall be informed that comments have been made.

4.1.10	An administration that has not notified its comments either to the administration seeking agreement or to the Bureau within a period of four months following the date of its BR IFIC referred to in § 4.1.5 shall be deemed to have agreed to the proposed assignment. This time-limit may be extended:

–	for an administration that has requested additional information under § 4.1.8, by up to three months, or

–	for an administration that has requested the assistance of the Bureau under § 4.1.21, by up to three months following the date at which the Bureau communicated the result of its action.

4.1.11	If, in seeking agreement, an administration modifies its initial proposal, it shall again apply the provisions of § 4.1 and the consequent procedure with respect to any other administration whose services might be affected as a result of modifications to the initial proposal.

4.1.12	If no comments have been received on the expiry of the periods specified in § 4.1.10, or if agreement has been reached with the administrations which have made comments and with which agreement is necessary, the administration proposing the new or modified assignment may continue with the appropriate procedure in Article 5 and shall inform the Bureau, indicating the final characteristics of the frequency assignment together with the names of the administrations with which agreement has been reached.

4.1.13	The agreement of the administrations affected may also be obtained in accordance with this Article, for a specified period.

4.1.14	Where the proposed assignment involves developing countries, administrations shall seek all practicable solutions conducive to the economical development of the broadcasting-satellite systems of these countries.

4.1.15	The Bureau shall publish in a Special Section of its BR IFIC the information received under § 4.1.12, together with the names of any administrations with which the provisions of this Article have been successfully applied. The frequency assignment concerned shall be included in the feeder-link Lists.

4.1.16	In case of disagreement on the part of an administration whose agreement has been sought, the requesting administration should first endeavour to solve the problem by exploring all possible means of meeting its requirement. If the problem still cannot be solved by such means, the administration whose agreement has been sought should endeavour to overcome the difficulties as far as possible, and shall state the technical reasons for any disagreement if the administration seeking the agreement requests it to do so.

4.1.17	If no agreement is reached between the administrations concerned, the Bureau shall carry out any study that may be requested by either one of these administrations; the Bureau shall inform them of the result of the study and shall make such recommendations as it may be able to offer for the solution of the problem.

SUP	USA/12/203

4.1.18

SUP	USA/12/204

4.1.18bis

SUP	USA/12/205

4.1.19

SUP	USA/12/206

4.1.20

4.1.21	An administration may, at any stage in the procedure described, or before applying it, request the assistance of the Bureau.

4.1.22	The relevant provisions of Article 5 shall be applied when frequency assignments are notified to the Bureau.

4.1.23	When a frequency assignment included in the feeder-link Lists is no longer required, the administration concerned shall immediately so inform the Bureau. The Bureau shall publish this information in a Special Section of its BR IFIC and delete the assignment from the feeder-link Lists.

4.1.24	No assignment in the feeder-link Lists shall have a period of operation exceeding 15 years, counted from the date of bringing into use, or 2 June 2000, whichever is later. Upon request by the responsible administration received by the Bureau at the latest three years before the expiry of this period, this period may be extended by up to 15 years, on condition that all the characteristics of the assignment remain unchanged.

4.1.25	Where an administration already having included in the feeder-link Lists two assignments (not including those systems notified on behalf of a group of named administrations and included in the feeder-link Lists by WRC-2000) in the same channel and covering the same service area, proposes to include in the feeder-link Lists a new assignment in the same channel over this same service area, it shall apply the following in respect of another administration which has no assignment in the feeder-link Lists in the same channel and which proposes to include in the feeder-link Lists a new assignment:

a)	if the agreement of the former administration is required following the application of § 4.1 by the latter administration, in order to protect the new assignment proposed by the former administration from interference caused by the assignment proposed by the latter administration, both administrations shall make every possible effort to resolve the difficulties by means of mutually acceptable adjustments to their networks;

b)	in case of continuing disagreement, and if the former administration has not communicated to the Bureau the information specified in Annex 2 to Resolution 49 (Rev.WRC-2000), this administration shall be deemed to have given its agreement to inclusion in the feeder-link Lists of the assignment of the latter administration.

4.1.26	This procedure may be applied by the administration of a new ITU Member State in order to include new assignments in the feeder-link Lists. Upon completion of the procedure, the next world radiocommunication conference may be requested to consider, among the assignments included in the feeder-link Lists after the successful completion of this procedure, the inclusion in the Regions 1 and 3 feeder-link Plan of up to 10 channels (for Region 1) and up to 12 channels (for Region 3), over the national territory of the new Member State.

4.1.27	When an administration has successfully applied this procedure and received all the agreements7 required to include in the feeder-link Lists assignments over its national territory, at an orbital location and/or in channels different from those appearing in the Regions 1 and 3 feeder-link Plan for its country, it may request the next world radiocommunication conference to consider the inclusion in this Plan of up to 10 (for Region 1) and up to 12 (for Region 3) of these assignments, in replacement of its assignments appearing in this Plan.

4.1.28	The feeder-link Lists, as updated, shall be published periodically by the Bureau.

4.1.29	New or modified assignments in the feeder-link Lists shall be limited to digital modulation.

______________

6	For assignments using analogue modulation, the intention not to employ energy dispersal in accordance with § 3.18 of Annex 5 shall be treated as a modification and thus subject to the appropriate provisions of this Article.

7	Or under Resolution 33 (Rev.WRC-97) for assignments for which the API or the request for coordination has been received by the Bureau prior to 1 January 1999.

8	If the payments are not received in accordance with the provisions of Council Decision 482, as amended, on the implementation of cost recovery for satellite network filings, the Bureau shall cancel the publication, after informing the administration concerned. The Bureau shall inform all administrations of such action and that the network specified in the publication in question no longer has to be taken into consideration by the Bureau and other administrations. The Bureau shall send a reminder to the notifying administration, not later than 60 days prior to due date of the payment if payment has not been received by that date. This provision was identified in reply to Resolution 88 (Minneapolis, 1998) of the Plenipotentiary Conference and shall enter into force at a date to be determined by the forthcoming Plenipotentiary Conference.

8 	Agreement with administrations having a frequency assignment in the bands 17.7-17.8 GHz to a terrestrial station or to an earth station in the fixed-satellite service (space-to-Earth) shall be sought under No. 9.17 or No. 9.17A, respectively.

9 	Agreement with administrations having a frequency assignment in the band 17.3-17.8 GHz to an earth station in the broadcasting-satellite service shall be sought under No. 9.19.

10 	If the payments are not received in accordance with the provisions of Council Decision 482, as amended, on the implementation of cost recovery for satellite network filings, the Bureau shall cancel the publication, after informing the administration concerned. The Bureau shall inform all administrations of such action and that the network specified in the publication in question no longer has to be taken into consideration by the Bureau and other administrations. The Bureau shall send a reminder to the notifying administration, not later than 60 days prior to due date of the payment if payment has not been received by that date. This provision was identified in reply to Resolution 88 (Minneapolis, 1998) of the Plenipotentiary Conference and shall enter into force at a date to be determined by the forthcoming Plenipotentiary Conference.

2	The provisions of Resolution 49 (Rev.WRC-2000) apply.

3	The provisions of Resolution 533 (Rev.WRC-2000) apply.

4	If the payments are not received in accordance with the provisions of Council Decision 482, as amended, on the implementation of cost recovery for satellite network filings, the Bureau shall cancel the publication, after informing the administration concerned. The Bureau shall inform all administrations of such action and that the network specified in the publication in question no longer has to be taken into consideration by the Bureau and other administrations. The Bureau shall send a reminder to the notifying administration, not later than 60 days prior to due date of the payment if payment has not been received by that date. This provision was identified in reply to Resolution 88 (Minneapolis, 1998) of the Plenipotentiary Conference and shall enter into force at a date to be determined by the forthcoming Plenipotentiary Conference.

5	In such a case, § 4.1.18 does not apply.

6	For assignments using analogue modulation, the intention not to employ energy dispersal in accordance with § 3.18 of Annex 5 shall be treated as a modification and thus subject to the appropriate provisions of this Article.

7	Or under Resolution 33 (Rev.WRC-97) for assignments for which the API or the request for coordination has been received by the Bureau prior to 1 January 1999.

8	If the payments are not received in accordance with the provisions of Council Decision 482, as amended, on the implementation of cost recovery for satellite network filings, the Bureau shall cancel the publication, after informing the administration concerned. The Bureau shall inform all administrations of such action and that the network specified in the publication in question no longer has to be taken into consideration by the Bureau and other administrations. The Bureau shall send a reminder to the notifying administration, not later than 60 days prior to due date of the payment if payment has not been received by that date. This provision was identified in reply to Resolution 88 (Minneapolis, 1998) of the Plenipotentiary Conference and shall enter into force at a date to be determined by the forthcoming Plenipotentiary Conference.

4 	Agreement with administrations having a frequency assignment in the bands 14.5-14.8 GHz or 17.7-18.1 GHz to a terrestrial station, or having a frequency assignment in the band 17.7-18.1 GHz to an earth station in the fixed-satellite service (space-to-Earth), or having a frequency assignment in the band 17.3-17.8 GHz in the broadcasting-satellite service shall be sought under No. 9.17, No. 9.17A or No. 9.19, respectively.

5 	The provisions of Resolution 533 (Rev.WRC-2000) apply.

6 	If the payments are not received in accordance with the provisions of Council Decision 482, as amended, on the implementation of cost recovery for satellite network filings, the Bureau shall cancel the publication, after informing the administration concerned. The Bureau shall inform all administrations of such action and that the network specified in the publication in question no longer has to be taken into consideration by the Bureau and other administrations. The Bureau shall send a reminder to the notifying administration, not later than 60 days prior to due date of the payment if payment has not been received by that date. This provision was identified in reply to Resolution 88 (Minneapolis, 1998) of the Plenipotentiary Conference and shall enter into force at a date to be determined by the forthcoming Plenipotentiary Conference.

7 	In such a case, § 4.1.18 does not apply.