| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Addendum 36 to Document 25-E |
|  |  |  | 15 April 2003 |
|  |  |  | Original: English |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | PLENARY MEETING |
| --- | --- |
| Asia-Pacific Common Proposals |  |
| PROPOSALS FOR THE WORK OF THE CONFERENCE |  |
| agenda item 1.36 |  |

1.36	to examine the adequacy of the frequency allocations for HF broadcasting from about 4 MHz to 10 MHz, taking into account the seasonal planning procedures adopted by WRC-97

Introduction

This agenda item asks WRC-03 to make an assessment of whether the allocated frequency bands for HF broadcasting (HFBC) services between 4-10 MHz are adequate. The APG has the following proposal for a new Resolution.

Proposal

ADD	ASP/25/256

draft  RESOLUTION  [asp/1.36/hfbc]  (WRC-03)

Identification of additional spectrum for the 
broadcasting service in the HF bands

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that the spectrum allocated to the broadcasting service from 4 MHz to 10 MHz is about 25% of the spectrum allocated to HF broadcasting;

b)	that WARC-79 allocated only an additional 125 kHz of spectrum to the HF broadcasting service below 10 MHz (9 775-9 900 kHz);

c)	that WARC-92 allocated an additional 200 kHz HF to the broadcasting service consisting of 100 kHz at 9 MHz, 50 kHz at 7 MHz and 50 kHz at 6 MHz. This additional spectrum will become available to the broadcasting service from [1 April 2007];

d)	that the agenda for WRC-07 includes the revision of allocations to some services in the 4 MHz to 10 MHz bands;

e)	that the results of coordination under Article 12 demonstrate that the broadcasting bands below 10 MHz are congested, even when there are high levels of sunspot activity, with little more than half of the broadcasting requirements being satisfied;

f)	that recent seasonal planning statistics show that around 250 kHz of additional spectrum is needed to clear the co-channel collisions and up to 800 kHz to clear both the co-channel and adjacent channel collisions in the bands below 10 MHz;

g)	that the introduction and promotion of the new digital technology, that improves spectrum utilization and efficiency, cannot completely solve current congestion problems;

h)	that the broadcasting service, in an era of convergence of services, will play an increased socio-political role;

i)	that many administrations continue to use the HF bands for other services, including for fixed and mobile communications,

recognizing

that the use of the tropical bands is limited in accordance with No. 5.113,

noting

that studies in preparation for WRC-03 identified the following bands for possible consideration for future allocations to the broadcasting service:

4 500-4 650 kHz

5 060-5 250 kHz

5 840-5 900 kHz

7 350-7 650 kHz

9 290-9 400 kHz

9 900-9 940 kHz,

noting further

that further studies are required on the potential allocation of the bands identified in the noting above or any other bands that may be considered for allocation to the broadcasting service,

resolves to invite ITU-R

1	to carry out technical studies on this matter, taking into account technical, operational, economic and other relevant factors, including how introduction of digital emissions will affect the HF broadcasting requirements and how any reallocation will affect existing users;

2	to bring the results of these studies to the attention of WRC-07,

urges administrations

to participate actively in the aforementioned studies by submitting contributions to ITU-R.

Reasons:	Further studies need to be carried out for possible consideration, taking into account the impact on other services in these bands.

_______________

Resolution 801 (WRC-2000) resolves to give the view 2.5.