| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Corrigendum 1 to Document 32-E |
|  |  |  | 26 May 2003 |
|  |  |  | Original: English |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | PLENARY MEETING |
| --- | --- |
| Iran (Islamic Republic of) |  |
| PROPOSALS FOR THE WORK OF THE CONFERENCE |  |

Replace the text of the following two paragraphs in Document 32 pages 18 and 19 under agenda item 1.24, to read:

Agenda item 1.24

Introduction

–	for maritime radar: –105.2 dB(W/(m2  10 MHz)) not to be exceeded for more than 1.0% of the time produced at 36 m above sea level at the [normal baseline, straight baseline, baseline as defined in UN Convention on the Law Of the Sea 1982 or at a point specified by the coastal State].

Proposal

MOD	IRN/32/44

5.502	In the band 13.75-14 GHz, an earth station of a geostationary fixed-satellite service network shall have a minimum antenna diameter of 1.2 m and an earth station of a non-geostationary fixed-satellite service system shall have a minimum antenna diameter of 4.5 m. In addition, the e.i.r.p. averaged over one second, radiated by a station in the radiolocation or radionavigation services shall not exceed 59 dBW above 2º elevations and 65 dBW below.- An administration planning to use FSS earth stations with an antenna diameter less than 4.5 m in a geostationary network in this band shall ensure that the single-entry power flux-density produced by any earth station operating within its territory does not exceed:

–	–105.2 dB(W/(m2  10 MHz)) not to be exceeded for more than 1.0% of the time produced at 36 m above sea level at the [normal baseline, straight baseline, baseline as defined in the UN Convention on the Law Of the Sea 1982 or at a point specified by the coastal State].	     (WRC-03)

__________