| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Document 52-E |
|  |  |  | 9 May 2003 |
|  |  |  | English only |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | PLENARY MEETING |
| --- | --- |
| Note by the Secretary-General |  |
| abu information paper |  |

I have the honour to bring to the attention of the Conference, at the request of the Asia-Pacific Broadcasting Union (ABU), the annexed information paper.

Yoshio UTSUMI
				Secretary-General

Annex: 1

ANNEX

ABU position on WRC-03 agenda items 1.27 and 1.35

Subsequent to the Conference Preparatory Meeting (CPM–02), the Asia-Pacific Broadcasting Union (ABU) has reviewed its position related to WRC-03 agenda items. The ABU position on the BSS related agenda items 1.27 and 1.35 is provided in the following.

Agenda item 1.27: Review of BSS Regulatory Procedures and Sharing Criteria

This agenda item calls for consideration of regulatory procedures and sharing criteria (generally those contained in Annexes 1, 3, 4, and 6 to Appendix 30 and Annexes 1 and 4 to Appendix 30A and in the associated provisions of Articles 9 and 11), vis-à-vis the FSS and terrestrial services in these bands. The regulatory procedures governing the 12 GHz band BSS Plans (and the List) are contained in Appendices 30 and 30A.

The CPM Report includes consideration of technical assumptions for sharing criteria, reference antenna patterns, reference antenna sizes (including 45 cm antenna), protection criteria (delta T/T and C/I approaches) to be used, criteria for various types of intraregional BSS-BSS sharing situations, selection of minimum antenna size for pfd masks, pfd masks for protection of BSS/FSS antennas of 60 and 45 cm, and criteria for protection of BSS earth stations from terrestrial stations or from FSS earth stations. In many cases, multiple solutions were developed.

The CPM Report also addresses issues pertaining to possible amendment of provision Nos. 4.1.18 to 4.1.20 (Appendices 30/30A), focusing on constraints that these provisions would cause to the Regions 1 and 3 Plans. The CPM identified three options; suppressing the provisions, maintaining without any change and maintaining with arrangements to protect the Plan assignments, the latter with appropriate restrictions to inhibit monopolization of orbital through grouping of multiple networks.

ABU Position

1	The ABU strongly supports that the BSS Plan, adopted by WRC-2000, should be sufficiently protected. The related regulatory issues, including procedures for the List and sharing criteria for other services in the BSS bands, should not be detrimental to the use of the Plan.

2	The ABU proposes that in Region 3 significance of an adequate rainfall margin should be taken into account as a substantial part of the region comprises heavy rainfall areas.

3	While reviewing the provisions contained in Nos. 4.1.18 to 4.1.20 of Appendices 30/30A, the expression “harmful interference” should be quantified in order not to seriously constrain the use of the Region 3 Plan. Appropriate studies should be initiate in ITU-R to enable the following WRC to make the decisions with respect to §§ 4.1.18 to 4.1.20 of Appendices 30 and 30A.

4	The sharing criteria should be based on the situations in each country taking into account the geographical conditions of the country/Region.

5	The pfd could exceed the limit contained in Annex 1 to AP30 within the territory of the notifying administration if it does not affect the other administrations more than the Plan assumptions.

6	The minimum antenna size considered to protect the BSS Plan should be:

•	60 cm: in general

•	45 cm: an administration in Region 3, of which BSS Plan has national coverage only and has been brought in service before [DD.MM.YYYY]

7	Annex 3 of Appendix 30 should be maintained with appropriate revisions of the BSS parameters to reflect changes made to the technical parameters in the Plans.

8	Appropriate studies in ITU-R should be initiated to enable a competent WRC to make the decisions with respect to grouping concept.

#########

Agenda item 1.35: Results of compatibility analysis under Articles of Appendices 30/30A.

The CPM has noted partial implementation of Resolution 53 (Rev.WRC-2000), updating of the “Remarks” columns in the Tables of Article 9A of Appendix 30A and Article 11 of Appendix 30 to the Radio Regulations. This information includes compatibility analyses between the uplink and downlink plans of the three Regions.

ABU Position

1	The remarks columns of AP30/30A should be periodically reviewed and updated.

____________