| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Addendum 5 to Document 58-E |
|  |  |  | 13 May 2003 |
|  |  |  | Original: English |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | PLENARY MEETING |
| --- | --- |
| Japan |  |
| PROPOSALS FOR THE WORK OF THE CONFERENCE |  |
| Agenda item 1.29 |  |

1.29	to consider the results of studies related to Resolutions 136 (WRC-2000) and 78 (WRC-2000) dealing with sharing between non-GSO and GSO systems

# 1	Introduction

In Article 22, WRC-2000 adopted single-entry operational limits (see Tables 22-4A through 22-4C) and single-entry additional operational limits (see Table 22-4A1) and made them applicable to non-geostationary (non-GSO) fixed-satellite service (FSS) systems (space-to-Earth) in certain parts of the frequency ranges 10.7-12.75 and 17.8-20.2 GHz.

Resolution 78 (WRC-2000) invites ITU-R, taking into consideration the guidelines in Annex 1 to the Resolution, to conduct, as a matter of urgency, and in time for consideration by WRC-03, the appropriate regulatory studies to develop procedures, not limited to modification of Article 15, for application in cases where the power limits referred to above are exceeded at an operational earth station.

# 2	Proposals

ARTICLE  22

Space services1

Section II  –  Control of interference to geostationary-satellite systems

MOD	J/58A5/1

22.5I		6)	An administration operating a non-geostationary-satellite system in the fixed-satellite service which is in compliance with the limits in Nos. 22.5C, 22.5D and 22.5F shall be considered as having fulfilled its obligations under No. 22.2 with respect to any geostationary-satellite network, irrespective of the dates of receipt by the Bureau of the complete coordination or notification information, as appropriate, for the non-geostationary-satellite system and the geostationary-satellite network, provided that the epfd radiated by the non-geostationary-satellite system in the fixed-satellite service into any operating geostationary fixed-satellite service earth station does not exceed the operational and additional operational limits given in Tables 22-4A,

22-4A1, 22-4B and 22-4C, when the diameter of the earth station antenna is equal to the values given in Table 22-4A, 22-4A1 or 22-4C, or the gain of the earth station is equal to or greater than the values given in Table 22-4B for the corresponding orbital inclination of the geostationary fixed-satellite service satellite. Except as otherwise agreed between concerned administrations, an administration operating a non-geostationary-satellite system in the fixed-satellite service that is subject to the limits in Nos. 22.5C, 22.5D and 22.5F and which radiates epfd into any operating geostationary fixed-satellite service earth station at levels in excess of the operational or additional operational limits given in Tables 22-4A, 22-4A1, 22-4B and 22-4C, when the diameter of the earth station antenna is equal to the values given in Table 22-4A, 22-4A1 or 22-4C, or the gain of the earth station is equal to or greater than the values given in Table 22-4B for the corresponding orbital inclination of the geostationary fixed-satellite service satellite, shall be considered to be in violation of its obligations under No. 22.2. The provisions of Resolution [XXX] (WRC-03) shall apply in the event of non-compliance with the single-entry operational and additional operational limits in Section II of Article 22 by a non-geostationary-satellite system in the fixed-satellite service that is subject to the limits in Nos. 22.5C, 22.5D and 22.5F.      (WRC-03)

ADD	J/58A5/2

DRAFT  RESOLUTION  [XXX]  (WRC-03)

Procedures in case the operational or additional 
operational limits in Article 22 are exceeded

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	 that WRC-2000 adopted in Article 22 single-entry operational limits (see Tables 22-4A through 22-4C) and single-entry additional operational epfd limits (see Table 22-4A1) applicable to non-geostationary (non-GSO) fixed-satellite service (FSS) systems (space-to-Earth) in certain parts of the frequency range 10.7-20.2 GHz to protect geostationary systems operating in the same frequency bands;

b)	that, taking into account Nos. 22.5H and 22.5I, wherever the limits referred to in considering a) are exceeded by a non-GSO FSS system to which the limits apply, this constitutes an infringement of No. 22.2, except where otherwise agreed between concerned administrations;

c)	that WRC-2000 identified the need for specific procedures that correct in the most expeditious manner any cases where the limits in considering a) are exceeded;

d)	that ITU-R has developed Recommendations ITU-R S.1527 and ITU-R [S.4BL23] to assist administrations in identifying the source of interference in excess of the operational epfd limits and measuring levels of epfd levels to verify compliance with the operational limits, respectively;

e)	that no procedures currently exist in the Radio Regulations to expeditiously address the unique regulatory situation of 22.5I,

resolves

that the procedures contained in the Annex shall be applied in the event of non-compliance with the single-entry operational and additional operational limits in Section II of Article 22 by a non-geostationary-satellite system in the FSS that is subject to the limits in Nos. 22.5C, 22.5D and 22.5F.

Annex  TO  DRAFT  RESOLUTION  [XXX]  (WRC-03)

Procedures to be applied in the event of non-compliance with single-entry operational and additional operational limits in Section II of Article 22

1	It is essential that Member States exercise the utmost goodwill and mutual assistance in the application of these procedures for the expeditious elimination of equivalent power flux-density (epfd) interference from non-geostationary-satellite systems in the FSS at levels above the operational epfd limits given in Tables 22-4A, 22-4B and 22-4C and/or the additional operational epfd limits given in Table 22-4A1 (“excess epfd interference”).

2	In securing the expeditious elimination of excess epfd interference, due consideration shall be given to all factors involved, including the relevant technical and operational factors.

3	Administrations shall cooperate in the detection and elimination of excess epfd interference.

4	Where practicable, and subject to agreement between the administrations concerned, the case of excess epfd interference may be dealt with directly between their operating organizations.

5	When a case of excess epfd interference to a frequency assignment in a geostationary-satellite network is detected at an operating earth station associated with the geostationary-satellite network and such excess epfd interference cannot be accepted by the affected administration, the affected administration shall first attempt to identify the source of the excess epfd interference. For purposes of these procedures, the term “affected administration” shall mean the administration on whose territory the receiving earth station associated with the geostationary-satellite network is located or its designee.

6	If an affected administration referred to in No. 5 has difficulty in determining the source or characteristics of the excess epfd interference:

a)	It may send a request for cooperation to any administration which has submitted to the Bureau complete advance publication, coordination or notification information, as appropriate, for non-GSO FSS systems with overlapping frequency assignments that have been brought into use in the frequency bands subject to the limits referred to in No. 1, providing all relevant details in a report of non-compliance with single-entry operational and additional operational equivalent power flux-density (epfd) limits in Section II of Article 22 utilizing the form provided in the Attachment to these procedures. A copy of any such request, including the report of non-compliance with single-entry operational and additional operational equivalent power flux-density (epfd) limits in Section II of Article 22, shall be sent to Bureau.

abis)	It may request the assistance of the Bureau to identify the administrations referred to in No. 6 a). Upon receipt of such a request of assistance, the Bureau shall promptly communicate to the requesting administration the list of administrations which have submitted to the Bureau complete advance publication, coordination or notification information, as appropriate, for a non-geostationary satellite system in the FSS in the frequency bands referred to in No. 1 with overlapping frequency assignments that have been brought into use. Upon receipt from the Bureau of the list of administrations, the affected administration shall then apply No. 6 a).

b)	Upon receipt of such a request for cooperation under No. 6 a), each administration shall, as soon as possible but within 30 days, acknowledge receipt and send to the requesting administration(s), with a copy to the Bureau, information that may be used to identify the source of the excess epfd interference and/or to eliminate one or more non-GSO FSS systems referred to in No. 6 a) as the source of the excess epfd interference.

c)	If an administration fails to respond within 30 days of receipt to a request for cooperation under No. 6 a), an affected administration may request the assistance of the Bureau, in which case the Bureau shall forthwith request the non-responding administration to provide the information referred to in No. 6 b) within 30 days of an affected administration’s request for the assistance of the Bureau.

d)	If an administration fails to respond to the Bureau within the time period established in No. 6 c) above, the Bureau shall:

–	If the procedure of Article 11 has not been completed for the frequency assignments of the non-geostationary-satellite system in the FSS in question, publish a remark in the BR IFIC within one month to the effect that the responsible administration did not respond to a request for cooperation regarding an unresolved complaint of excess epfd interference; or

–	if the procedure of Article 11 has been completed for the frequency assignments of the non-geostationary-satellite system in the FSS in question, enter a remark in the Remarks column of the Master Register against the relevant frequency assignments of the non-GSO FSS system in question to the effect that the responsible administration did not respond to a request for cooperation regarding an unresolved complaint of excess epfd interference.

7	Upon receipt of the information identified in No. 6 a), the Bureau shall promptly communicate to all administrations contacted under No. 6 a) the identity of any non-geostationary fixed-satellite service systems that, on the basis of determining compliance with the epfd validation limits in Tables 22-1A through 22-1D, the Bureau has concluded to have a maximum epfd lower than the limits referred to in No. 1, for all pointing directions towards the geostationary-satellite orbit and therefore would not be responsible for causing epfd interference in excess of the limits referred to in No. 1.

8	Once the source(s) of the excess epfd interference have been identified, an affected administration may send a letter, by fax or other mutually agreed electronic means, to the administration(s) concerned and request immediate corrective action. It shall give all useful information, including a Report of Non-Compliance with Single-Entry Operational and Additional Operational Equivalent Power Flux-Density (epfd) Limits in Section II of Article 22, to enable the responding administration(s) to take such steps as may be necessary to reduce the interference to the epfd levels required in Table 22-4A, 22-4A1, 22-4B or 22-4C, as appropriate, or to higher levels as

may otherwise be or have been agreed between concerned administrations pursuant to No. 22.5I. A copy of any such request for immediate corrective action, including the Report of Excess epfd Interference, shall be sent to Bureau.

9	Upon receipt of such a request for immediate corrective action under No. 8, an administration shall acknowledge receipt to the requesting administration within 30 days, with a copy to the Bureau. Such acknowledgement would not constitute acceptance of responsibility.

10	Within 30 days after receipt of a request for immediate corrective action pursuant to No. 8 above, the administration receiving the request shall either:

a)	Provide the requesting administration and the Bureau with information indicating that no non-geostationary fixed-satellite service system for which it is responsible could have caused the excess epfd interference experienced by the receiving earth station associated with the geostationary-satellite network; or

b)	acknowledge responsibility for causing the excess epfd interference and immediately reduce emissions of the interfering system into the affected receiving earth station associated with the geostationary-satellite network to the epfd levels specified in Table 22-4A, 22-4A1, 22-4B or 22-4C, as appropriate, or to the epfd levels otherwise agreed between concerned administrations pursuant to No. 22.5I, whichever is higher. Full particulars of the action taken by the administration responsible for causing the excess epfd interference shall be provided to the requesting administration.

In either case, the Bureau shall be informed of the action taken.

11	If an administration fails to act in accordance with No. 10 above, an affected administration may request the assistance of the Bureau, in which case the Bureau shall forthwith request the non-responding administration to act in accordance with No. 10 within 30 days of the affected administration’s request for the assistance of the Bureau.

12	If the administration fails to respond to the Bureau within the time period established in No. 11 above, the Bureau shall:

–	if the procedure of Article 11 has not been completed for the frequency assignments of the non-geostationary-satellite system in the fixed-satellite service in question, publish a remark in the BR IFIC within one month to the effect that the responsible administration did not respond to a request for immediate corrective action regarding an unresolved complaint of excess epfd interference; or

–	if the procedure of Article 11 has been completed for the frequency assignments of the non-geostationary-satellite system in the fixed-satellite service in question, enter a remark in the Remarks column of the Master Register against the relevant frequency assignments of the non-GSO FSS system in question to the effect that the responsible administration did not respond to a request for immediate corrective action regarding an unresolved complaint of excess epfd interference.

13	If an administration acknowledges responsibility for causing the excess epfd interference pursuant to No. 10 b) above, but fails to reduce immediately emissions of the interfering system as required:

a)	It shall have an additional 10 days to take the necessary action to correct the excess epfd interference situation pursuant to No. 15.21 of the Radio Regulations.

b)	If, after the 10-day period, the administration responsible for the interference has still not reduced emissions of the interfering system as required, the Bureau shall:

–	if the procedure of Article 11 has not been completed for the frequency assignments of the non-geostationary-satellite system in the FSS in question, publish a remark in the BR IFIC within one month to the effect that the responsible administration is in contravention of its obligations under No. 22.2 and No. 22.5I; or

–	if the procedure of Article 11 has been completed for the frequency assignments of the non-geostationary-satellite system in the FSS in question, enter a remark in the Remarks column of the Master Register against the relevant frequency assignments of the non-GSO FSS system in question to the effect that the use of the affected frequency assignments by the interfering system is in contravention of its obligations under No. 22.2 and No. 22.5I of the Radio Regulations. Notice of the entry of the remark shall be included in the IFIC.

14	The Bureau shall retain any entry in the Remarks column of the Master Register made pursuant to No. 6 d), No. 12 or No. 13 b) above, which shall remain in place until such time as the non-responding administration responds and/or corrects the excess epfd interference, as appropriate.

ATTACHMENT  to  ANNEX  TO  DRAFT  RESOLUTION  [XXX]  (WRC-03)

Report of non-compliance with single-entry operational and additional operational equivalent power flux-density (epfd) limits 
in Section II of Article 22

Part I – Particulars concerning the non-geostationary-satellite system exceeding the operational epfd  limits:

1)	Name of non-geostationary-satellite system, if known or other mean of identification:

2)	Frequency measured:

a)	Date:

b)	Time (UTC):

3)	Class of emission:

4)	Bandwidth (indicate whether measured or estimated):

5)	Measured epfd:

a)	Date:

b)	Time (UTC):

- Duration

- Repeatability (Yes/no)

If yes, associated period

6)	Applicable epfd limit in Article 22 Tables 22-4A through 22-4C:

7)	Observed polarization:

8)	Class of station and nature of service, if known:

Part II – Particulars furnished by the administration responsible for the operating geostationary fixed-satellite service earth station receiving epfd levels exceeding the operational limits in Tables 22-4A through 22-4C:

1)	Name of earth station:

2)	Antenna diameter of the receiving earth station:

3)	Name of the transmitting GSO space station interfered with, if known or other means of identification:

4)	Longitude of the associated geostationary-satellite network:

5)	Orbital inclination of the associated geostationary-satellite network:

6)	Latitude, Longitude Elevation and Azimuth of the earth station at which the epfd levels 
exceeded the operational limits in Tables 22-4A through 22-4C:

7)	Nature of interference:

8)	Characteristics of the wanted emission at the receiving station

a)	Class of emission

b)	Bandwidth (indicate whether measured or estimated, or indicate the necessary bandwidth notified to the Radiocommunication Bureau)

c)	Frequency measured

Date:

Time (UTC):

d)	Field strength or power flux-density

Date:

Time (UTC):

9)	Polarization of the receiving antenna or observed polarization:

10)	Action requested:

Reasons:	It is essential that, in order to protect GSO FSS networks from non-GSO system adequately, new regulatory procedures shall be adopted in the event of non-compliance with the single-entry operational and additional operational limits in Section II of Article 22 in addition to Sections V and VI of Article 15.

Therefore, Japan supports the Method B1 contained in Section 3.3.2.3 of the CPM Report that:

“to make no change to the procedures in Sections V and VI of Article 15.  Instead, add a reference in No. 22.5I to a new WRC Resolution that contains the procedures to be used by affected GSO and non-GSO networks and systems to determine which non-GSO FSS system is responsible for exceedances and to facilitate the expeditious return to the levels required in No. 22.5I. The procedures themselves have been developed within ITU-R, and provide administrations operating non-GSO FSS systems with an incentive to cooperate to expeditiously resolve exceedances of the operational and/or additional operational epfd¯ limits.”

__________