| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Document 94-E |
|  |  |  | 28 May 2003 |
|  |  |  | Original: English |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | PLENARY MEETING |
| --- | --- |
| Bulgaria (Republic of) |  |
| Contribution for the work of the conference |  |
| Agenda item 1.30 (Rule of Procedure on RR Article 9.35) |  |

# 1	Introduction

The Rule of Procedure on RR Article 9.35 provisionally adopted by RRB (Annex 1) is an acceptable proposal for reducing the backlog in the processing of satellite filings and should be adopted. However, a balance is needed to satisfy the requirements of the three parties involved in the process:

a)	ITU-BR is reducing its workload on satellite examinations.

b)	Satellite operators gain the advantage of applying Article 21.17 of the RR.

c)	All administrations with affected terrestrial networks don’t know what to do in this situation and how to perform the pfd examinations themselves.

# 2	The problems with implementing this Rule of Procedure

Resulting from CPM02-2 discussions, several disadvantages are noted (Annex 1).

Analysis of the ITU-BR SRS database shows that many networks already in operation do not send BR the information for notification under No. 11.31 (Annex 2).

The application of Article 21.17 of the RR needs pfd examination and coordination with affected administrations before the notification phase.

Involvement of the administrations in the satellite examination procedures is an appropriate step to eliminate the satellite notification backlog by reducing the workload of ITU-BR and by simplifying the procedures, but proper conditions are needed.

# 3	Proposals

The problems with CPM02-2 disadvantages 1, 3, 4, 7 (Annex 1) could be resolved by preparation of the following preliminary information by ITU-BR and including it in IFIC publications.

–	Identification of the satellite networks with their proper names.

–	Information for satellite networks already in operation, but not sent for notification under No. 11.31.

–	Information extracted from regular questions by administrations.

Disadvantages 2, 5, 6, 8 and 9 (Annex 1) could be resolved if the administrations carry the pfd examinations themselves, but the proper software and data are needed. Further proposals should be included in a respective Resolution for implementation of the Rule of Procedure on RR Article 9.35:

–	ITU-BR should supply proper satellite data for pfd calculations with information on how to use them.

–	BR should provide the administrations (free of charge) with unified and user-friendly software for pfd calculations (using IFIC data).

–	The software should verify and correct for calculation the “maximum power density” errors and “bandwidth of the emission” errors described in (Annex 3).

–	The frequency assignments in narrow bandwidth emissions should be examined in the reference bandwidths (RR Article 21.16).

–	Steerable beams should be examined only on the basis of the published data in the BR IFICs for coordination with affected administrations under RR Article 21.17.

–	BR should speed up the development and the implementation (by BR and by administrations) of the software according to new Recommendation ITU-R SF.[Doc. 4/117] “Outline of a software specification for automating the examination of satellite network filings for compliance with Article 5” (SG 4), including pfd calculation to help the automation of the satellite examination procedure.

Annex 1

New Rule of Procedure on RR Article 9.35

The Rule of Procedure on No. 9.35 provisionally adopted by RRB to reduce the backlog in the processing of satellite filings under Article 9 of the RR (to suspend the pfd examinations on satellite network coordination phase) is:

“The Radio Regulation Board adopted during its 25th meeting (3-7 December 2001) a Rule of Procedure on the suspension, within the No. 9.35 examinations, of examinations. A “qualified favourable” finding, as described in Circular Letter CR/180, is issued, which will need to be confirmed at the notification stage. The “suspended” examinations will be made in the No. 11.31 notification phase. The Rule has been applied to those networks for which complete coordination information has been received by the Bureau on and after 1 June 1999.”

The disadvantages of implementing this Rule of Procedure, as noted on CPM02-2

1)	This could create more workload in BR.

2)	Possibility of work increase for the Bureau pursuant to Nos. 9.41/9.42, as well as due to requests for assistance.

3)	The report of examinations (see the list in footnote 1 above) by BR will not be available to administrations.

4)	The work, instead of being done once by the Bureau on behalf of all administrations, will have to be done by any administration which so wishes and even by the Bureau as a result of requests for assistance.

5)	The number of coordinations is higher because of the surplus generated by assignments that are not in conformity with the provisions of the Radio Regulations.

6)	Administrations are required to coordinate with preceding assignments that are not in conformity with the Radio Regulations and consequently may not be able to carry out in due time the higher number of required coordinations to meet the regulatory time-limit (5 + 2 years) to bring into use frequency assignments.

7)	Possible increase of requests for assistance to the Bureau.

8)	Affording all the assignments in compliance with the Table of Frequency Assignments the status “qualified favourable” at the coordination stage and their potential impact on the Bureau’s records.

9)	By shifting the workload in processing satellite filings from the coordination stage to the notification stage, this method might simply transfer the backlog.

Annex 2

Satellite network notification

Analysis of the ITU-BR SRS database show that many networks already in operation do not send to BR information for notification under No. 11.31:

–	“INTELSAT5 XXX” – 57 notices are under coordination;

–	“INTELSAT7 XXX” – 106 notices are under coordination;

–	“INTELSAT8 XXX” – 126 notices are under coordination;

–	“INTELSAT9 XXX” – all notices are under coordination.

Some examples are given in the Table below:

| ntc_id | sat_name | d_rcv | d_inuse | st_cur | ntf_rsn |
| --- | --- | --- | --- | --- | --- |
| 90500662 | INTELSAT5 PAC3 | 25.9.1984 | 01.1.1985 | 50 | C |
| 90500658 | INTELSAT5 183E | 30.11.1998 | 01.9.1990 | 50 | C |
| 90500574 | INTELSAT5A INDOC2 | 30.11.1998 | 01.1.1985 | 50 | C |
| 90500620 | INTELSAT6 325.5E | 23.11.1987 | 01.11.1990 | 50 | C |
| 90500622 | INTELSAT6 335.5E | 17.7.1984 | 01.10.1987 | 50 | C |
| 90500623 | INTELSAT6 60E | 17.7.1984 | 01.10.1990 | 50 | C |

The question аrises: If the network “INTELSAT5 183E” in the SRS database is the network “INTELSAT5 183E” already being in operation? When should the pfd be examined, if the satellite networks operate without notification? BR shall answer this question.

Annex 3

Incorrect data in the SRS database

The detailed examination of the SRS database and the corresponding IFICs shows more than 20 000 cases (assignments) containing incorrect data:

–	maximum power density in 1 Hz (Pd) is less than average power in Hz of the emission (Pav) with up to 76 dB (Pav_Pd) (Table 1)

Pav = Pa – BWdB

–	maximum power density (Pd) is higher than total power (Pa) (Table 2);

–	bandwidth of the emission (BWemiss) is wider than the bandwidth of the channel (bdwdth) (Table 3), etc.

Examination with such incorrect data results in incorrect findings of the BR. The satellite networks would not work using such values of the parameters, but the coordinations on paper are completed successfully.

Table 1

Errors Pd <Pav in ITU-BR satellite (SRS) database

| ntc_id | adm | sat_name | grp_id | Emiss | BW | BWdB | Pd | Pa | Pav | Pav_Pd | fdg_reg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 94500315 | POR | EUTELSAT 2-10E | 94613616 | 307KG1D-- | 307.00 | 54.87 | -52.70 | 62.50 | 7.63 | 60.33 | A- |
| 96520105 | INS | PALAPA-C5 | 96809197 | 600KG7W-- | 600.00 | 57.78 | -91.10 | 15.00 | -42.78 | 48.32 | A- |
| 95500072 | MOZ | INTELSAT5A 342E | 95601523 | 4K00G1ECF | 4.00 | 36.02 | -46.60 | 37.60 | 1.58 | 48.18 | A- |
| 97520041 | USA | USASAT-14I-2 | 97649063 | 51K0G7W-- | 51.00 | 47.08 | -61.00 | 13.90 | -33.18 | 27.82 | A- |
| 91975176 | G | L-SAT | 91603744 | 40K0G1D-- | 40.00 | 46.02 | -66.00 | 0.00 | -46.02 | 19.98 | A- |
| 91980022 | J | N-STAR-A | 91616233 | 4K00N0N-- | 4.00 | 36.02 | -40.00 | 16.00 | -20.02 | 19.98 | A- |
| 95520172 | INS | PALAPA PACIFIC-4 | 97689737 | 22K0G7W-- | 22.00 | 43.42 | -66.70 | -3.70 | -47.12 | 19.58 | A- |
| 95520173 | INS | PALAPA PACIFIC-5 | 97692292 | 22K0G7W-- | 22.00 | 43.42 | -66.70 | -3.70 | -47.12 | 19.58 | A- |
| 94520204 | USA | INTELSAT8 60E | 97663931 | 45K0G1X-- | 45.00 | 46.53 | -55.10 | 10.10 | -36.43 | 18.67 | A- |
| 94520041 | USA | INTELSAT8 62E | 97662781 | 45K0G1X-- | 45.00 | 46.53 | -55.10 | 10.10 | -36.43 | 18.67 | A- |
| 94520042 | USA | INTELSAT8 64E | 97700225 | 45K0G1X-- | 45.00 | 46.53 | -55.10 | 10.10 | -36.43 | 18.67 | A- |
| 92520101 | USA | INTELSAT8 66E | 97717561 | 45K0G1X-- | 45.00 | 46.53 | -55.10 | 10.10 | -36.43 | 18.67 | A- |
| 90503548 | E | INTELSAT IBS 307E | 92612147 | 82K0G1W-- | 82.00 | 49.14 | -50.90 | 16.10 | -33.04 | 17.86 | A- |
| 90998018 | HOL | INTELSAT7 183E | 90606400 | 36M0G7W-- | 36000.00 | 75.56 | -52.90 | 40.00 | -35.56 | 17.34 | A- |
| 97520079 | B | B-SAT F | 97655833 | 1M50G7W-- | 1500.00 | 61.76 | -78.90 | 0.10 | -61.66 | 17.24 | A- |
| 97520078 | B | B-SAT G | 97692937 | 1M50G7W-- | 1500.00 | 61.76 | -78.90 | 0.10 | -61.66 | 17.24 | A- |

Table 2

Errors Pd >Pa in ITU-BR satellite (SRS) database

| ntc_id | adm | sat_name | grp_id | Emiss | BW | BWdB | Pd | Pa | Pd_Pa |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 96520214 | CHN | ASIASAT-DKX | 96820784 | 2M00G7W-- | 2000.00 | 63.01 | 62.20 | 0.00 | 62.20 |
| 97520091 | G | EAST-2 | 97704647 | 500KG7W-- | 500.00 | 56.99 | 9.20 | 5.00 | 4.20 |
| 97500295 | F | F-SAT ICO WEST | 97987396 | 500KG7W-- | 500.00 | 56.99 | 9.20 | 5.00 | 4.20 |
| 93520111 | J | GMS-140E | 93666277 | 1M00G3D-- | 1000.00 | 60.00 | 42.20 | -1.10 | 43.30 |
| 93500545 | USA | INTELSAT7 174E | 93670536 | 32M2G7W-- | 32200.00 | 75.08 | 63.20 | 11.20 | 52.00 |
| 96520076 | J | MTSAT-135E | 96632976 | 2M00G1D-- | 2000.00 | 63.01 | 51.40 | 6.80 | 44.60 |
| 96520077 | J | MTSAT-140E | 96649668 | 6K00G1D-- | 6.00 | 37.78 | 35.60 | 0.40 | 35.20 |
| 96520078 | J | MTSAT-145E | 96774928 | 260KF3C-- | 260.00 | 54.15 | 46.90 | 6.60 | 40.30 |
| 90504828 | IND | SEO BHASKARA-1 | 90604883 | 30K0PXX-- | 30.00 | 44.77 | -9.00 | -27.00 | 18.00 |
| 97520041 | USA | USASAT-14I-2 | 97648940 | 36M0G7W-- | 36000.00 | 75.56 | 60.70 | 14.90 | 45.80 |
| 97520042 | USA | USASAT-14J-2 | 97657016 | 36M0G7W-- | 36000.00 | 75.56 | 60.70 | 14.90 | 45.80 |
| 98520575 | USA | USASAT-31K | 98096025 | 11M3G7W-- | 11300.00 | 70.53 | 69.00 | 1.50 | 67.50 |
| 98520585 | USA | USASAT-31U | 98098979 | 11M3G7W-- | 11300.00 | 70.53 | 69.00 | 1.50 | 67.50 |
| 98520702 | USA | USASAT-32I | 98099005 | 11M3G7W-- | 11300.00 | 70.53 | 69.00 | 1.50 | 67.50 |
| 98520714 | USA | USASAT-32U | 98099031 | 11M3G7W-- | 11300.00 | 70.53 | 69.00 | 1.50 | 67.50 |
| 98520675 | USA | USASAT-33C | 98099057 | 11M3G7W-- | 11300.00 | 70.53 | 69.00 | 1.50 | 67.50 |

Table 3

Errors for Bw in ITU-BR satellite (SRS) database

| ntc_id | adm | sat_name | grp_id | Emiss | BWemiss | bdwdth |
| --- | --- | --- | --- | --- | --- | --- |
| 90502439 | D | DFS-2 | 85703231 | 45M0F3F-- | 45000,000 | 9000 |
| 90502878 | POR | EUTELSAT 2-10E | 90741229 | 300KG9X-- | 300,000 | 200 |
| 90500116 | IND | INSAT-2C | 87603163 | 37M0F8W-- | 37000,000 | 36000 |
| 90503612 | D | INTELSAT5 ATL2 | 86707887 | 60M0G1X-- | 60000,000 | 34000 |
| 90503706 | I | INTELSAT5 ATL4 | 87716469 | 72M0G1X-- | 72000,000 | 36000 |
| 90503765 | I | INTELSAT5 INDOC1 | 84720438 | 72M0G1X-- | 72000,000 | 36000 |
| 90503954 | AUS | INTELSAT5 PAC1 | 86708154 | 72M0F9W-- | 72000,000 | 71000 |
| 90503967 | J | INTELSAT5 PAC1 | 87716589 | 36M0F9W-- | 36000,000 | 34000 |
| 90504040 | USA | INTELSAT5 PAC3 | 84720682 | 72M0F9W-- | 72000,000 | 36000 |
| 91975080 | E | INTELSAT5A CONT4 | 91601741 | 72M0F8W-- | 72000,000 | 41000 |
| 91975079 | E | INTELSAT5A CONT4 | 91601711 | 72M0F8W-- | 72000,000 | 41000 |
| 91975079 | E | INTELSAT5A CONT4 | 91601719 | 41M0F8W-- | 41000,000 | 36000 |
| 91975079 | E | INTELSAT5A CONT4 | 91601719 | 72M0F8W-- | 72000,000 | 36000 |
| 90504176 | UAE | INTELSAT5A INDOC1 | 90741880 | 13M5F9WWF | 13500,000 | 2500 |
| 90504153 | D | INTELSAT5A INDOC1 | 90741794 | 60M0G1X-- | 60000,000 | 36000 |
| 93520118 | SNG | ST-1A | 93669503 | 45M0F3W-- | 45000,000 | 36000 |
| 93520119 | SNG | ST-1B | 93675815 | 54M0F3W-- | 54000,000 | 36000 |
| 90503081 | RUS | STATSIONAR-7 | 87716169 | 40M0F3F-- | 40000,000 | 36000 |
| 90500381 | USA | USASAT-13I | 86703333 | 54M0G7W-- | 54000,000 | 36000 |
| 90500381 | USA | USASAT-13I | 86703324 | 54M0G7W-- | 54000,000 | 36000 |

_______________

Examinations with respect to:

i)	any power limits referred to in applicable footnotes, Resolutions or Recommendations;

ii)	the power limits for earth stations as specified in Nos. 21.8, 21.10, 21.12 and 21.13;

iii)	the limits of power flux-density from space stations produced at the Earth’s surface as specified in Table 21-4 (21.16), as well as in Tables 22-1A to 22-1D (22.5C);

iv)	the limits of power flux-density from space stations produced at the geostationary orbit as specified in Nos. 22.5 and 22.5A;

v)	the limits of power flux-density from earth stations produced at the GSO as specified in Table 22.2 (22.5D);

vi)	the limits of power flux-density from space stations produced at any point in the geostationary orbit as specified in Table 22.3 (22.5F); and

vii)	the off-axis power limits of earth stations in the fixed-satellite service specified in Nos. 22.26 to 22.39.