| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Document 99-E |
|  |  |  | 2 June 2003 |
|  |  |  | Original: English |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | PLENARY MEETING |
| --- | --- |
| Malaysia |  |
| proposals for the work of the conference |  |
| agenda item 1.32 |  |

1.32	to consider technical and regulatory provisions concerning the band 37.5-43.5 GHz, in accordance with Resolutions 128 (Rev.WRC-2000) and 84 (WRC-2000)

Part A

# Resolution 84 - Power flux-density limits in the bands 37.5-42.5 GHz for the fixed-satellite service, broadcasting-satellite service and mobile-satellite service

Introduction

WRC-2000 established pfd limits for the FSS in the bands 37.5-40.0 GHz and 40.5-42.5 GHz  and the MSS in the band 39.5-40.0 GHz, and also harmonized the allocation to FSS in the band 40.5-42.5 GHz across all Regions. WRC-2000 also established provisional pfd limits for BSS in the range 40.5-42.5 GHz.

Under Resolution 84, WRC-2000 invited the ITU-R to conduct studies on a number of issues, including whether the pfd limits Table 21-4 adopted for the FSS, MSS and BSS adequately protect the FS in the bands 37.5-40.0 GHz and 40.5-42.0 GHz, and studies on appropriate mitigation techniques to improve sharing conditions between the space services under consideration and FS systems.

Results of ITU-R studies have confirmed the suitability of the current pfd limits in the bands 37.5-40.0 GHz and 40.5-42.0 GHz that apply to non-GSO FSS and GSO FSS satellites, in the band 40.5-42.5 GHz that apply to BSS satellites, and in the band 39.5-40.5 GHz that apply to MSS satellites.

Studies pursuant to invites 6 of Resolution 84 are not yet complete and it is proposed that they be continued within the normal activities of the ITU-R.

Proposals

ARTICLE  5

SUP	MLA/99/1

5.551AA

Reasons:	Consequential to the work of the ITU-R.

ADD	MLA/99/2

5.551AAA	While addressing the sharing conditions with the fixed service in the bands 37.5-40 GHz and 40.5-42.5 GHz, the power flux-density at the Earth’s surface from any fixed-satellite service satellite should be at the level(s) required to meet the fixed-satellite service link availability and performance objectives of the subject applications. In any case, the levels shall not exceed the applicable power flux-density limits in Table 21-4.

Reasons:	To address concerns for protection of more sensitive fixed BWA links from FSS emissions.

MOD	MLA/99/3

34.2-40 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 37.5-38 FIXED FIXED-SATELLITE (space-to-Earth) ADD 5.551AAA MOBILE SPACE RESEARCH (space-to-Earth) Earth exploration-satellite (space-to-Earth) 5.547 |  |  |
| 38-39.5 FIXED FIXED-SATELLITE (space-to-Earth) ADD 5.551AAA MOBILE Earth exploration-satellite (space-to-Earth) 5.547 |  |  |
| 39.5-40 FIXED FIXED-SATELLITE (space-to-Earth) ADD 5.551AAA MOBILE MOBILE-SATELLITE (space-to-Earth) Earth exploration-satellite (space-to-Earth) 5.547 |  |  |

MOD	MLA/99/4

40.5-51.4 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 40.5-41 FIXED FIXED-SATELLITE (space-to-Earth) ADD 5.551AAA BROADCASTING BROADCASTING-SATELLITE Mobile 5.547 | 40.5-41 FIXED FIXED-SATELLITE (space-to-Earth) ADD 5.551AAA BROADCASTING BROADCASTING-SATELLITE Mobile Mobile-satellite (space-to-Earth) 5.547 | 40.5-41 FIXED FIXED-SATELLITE (space-to-Earth) ADD 5.551AAA BROADCASTING BROADCASTING-SATELLITE Mobile 5.547 |

41-42.5 FIXED FIXED-SATELLITE (space-to-Earth) ADD 5.551AAA BROADCASTING BROADCASTING-SATELLITE Mobile 5.547 5.551F MOD 5.551G

Reasons:	Consequential to the work of ITU-R. The modification to No. 5.551G is related to results of studies conducted under Resolution 128 (see Part B).

ARTICLE  21

MOD	MLA/99/5

TABLE  21-4 (continued)

| Frequency band | Service* | Limit in dB(W/m2) for angle of arrival () above the horizontal plane |  |  |  | Reference bandwidth |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 0°-5° | 5°-25° |  | 25°-90° |  |
| . . . |  |  |  |  |  |  |
| 37.5-40 GHz | Fixed-satellite (non-geostationary-satellite orbit) Mobile-satellite (non-geostationary-satellite orbit) | –120 10 | –120 + 0.75( – 5) 10 |  | –105 10 | 1 MHz |
| 37.5-40 GHz | Fixed-satellite (geostationary-satellite orbit) Mobile-satellite (geostationary-satellite orbit) | –127 | 5°-20° | 20°-25° | –105 | 1 MHz |
|  |  |  | –127 + (4/3) ( – 5) | –107 + 0.4 ( – 20) |  |  |
| 40-40.5 GHz | Fixed-satellite | –115 | –115 + 0.5( – 5) |  | –105 | 1 MHz |
| 40.5-42 GHz | Fixed-satellite (non-geostationary-satellite orbit) Broadcasting-satellite (non-geostationary-satellite orbit) | –115 10 | –115 + 0.5( – 5) 10 |  | –105 10 | 1 MHz |
| 40.5-42 GHz | Fixed-satellite (geostationary-satellite orbit) Broadcasting-satellite (geostationary-satellite orbit) | –120 | 5°-15° | 15°-25° | –105 | 1 MHz |
|  |  |  | –120 + ( – 5) | –110 + 0.5 ( – 15) |  |  |

MOD	MLA/99/6

TABLE  21-4 (end)

| Reasons: Consequential to the work of the ITU-R. Frequency band | Service* | Limit in dB(W/m2) for angle of arrival () above the horizontal plane |  |  |  | Reference bandwidth |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 0°-5° | 5°-25° |  | 25°-90° |  |
| 42-42.5 GHz | Fixed-satellite (non-geostationary-satellite orbit) Broadcasting-satellite (non-geostationary-satellite orbit) | –120 10, | –120 + 0.75( – 5) 10 |  | –105 10 | 1 MHz |
| 42-42.5 GHz | Fixed-satellite (geostationary-satellite orbit) Broadcasting-satellite (geostationary-satellite orbit) | –127 | 5°-20° | 20°-25° | –105 | 1 MHz |
|  |  |  | –127 + (4/3) ( – 5) | –107 + 0.4 ( – 20) |  |  |

SUP	MLA/99/7

_______________

16	21.16.11

SUP	MLA/99/8

_______________

17	21.16.12

SUP	MLA/99/9

_______________

18	21.16.13

Reasons:	These notes specify interim provisions and are no longer necessary after WRC-03.

SUP	MLA/99/10

RESOLUTION  84  (WRC-2000)

Power flux-density limits in the bands 37.5-42.5 GHz for the fixed-satellite service, broadcasting-satellite service and mobile-satellite service

Reasons:	Consequential to the work of the ITU-R. Studies pursuant to invites 6 to be continued within the normal activities of the ITU-R.

Part B

# Resolution 128 - Protection of the radio astronomy service in the 42.5-43.5 GHz band

Introduction

The band 40.5-42.5 GHz, adjacent to the radioastronomy service in the band 42.5-43.5 GHz, is allocated to the broadcasting-satellite and fixed-satellite services.

WRC-2000 established provisional pfd limits for out-of-band emissions from BSS and FSS stations in accordance with No. 5.551G, and ITU-R was invited to study these provisional limits and to identify technical and operational measures in the band 41.5-42.5 GHz that may be implemented to protect stations in the RAS operating in the band 42.5-43.5 GHz, as well as measures that may be implemented to reduce the susceptibility of stations in the RAS to harmful interference.

The ITU-R has determined that the detrimental interference criteria of RAS stations operating in the band 42.5-43.5 GHz are as follows:

–	Single Dish Telescope (SDT):	–153 dB(W/(m2 · 500 kHz)) for spectral 			line observations, and

–137 dB(W/(m2 · GHz)) for continuum 			observations

–	Very Long Baseline Interferometry (VLBI):	–116 dB(W/(m2 · 500 kHz))

It is noted that the current protection criteria given in No. 5.551G do not take into account the reference bandwidth and the type of observations being conducted at the RAS station, and the revised criteria above would therefore improve on the provisions of No. 5.551G.

Studies on the impact of FSS and BSS unwanted emissions on RAS observations in the band 42.5-43.5 GHz have led to the review of the provisional pfd limits of No. 5.551G as requested by Resolution 128.

Results of studies indicate that unwanted emission levels of the GSO FSS and BSS systems operating in the band 42.0-42.5 GHz, and non-GSO FSS and BSS systems operating in the band 41.5-42.5 GHz meet the detrimental interference threshold for VLBI RAS stations operating in the band 42.5-43.5 GHz.

The detrimental interference thresholds for SDT, for continuum observations and for spectral line observations, may not be met by unwanted emissions from GSO FSS and BSS satellite in the band 42.0-42.5 GHz or by unwanted emissions from non-GSO FSS and BSS systems in the 41.5-42.5 GHz. However, due to the relatively few RAS sites operating with SDTs in the band 42.5-43.5 GHz, it may be feasible to employ other interference mitigation techniques in order to reduce the potential for detrimental interference to the RAS receiver sites operating in this band.

Proposals

Malaysia supports the adoption of permanent unwanted emission limits on GSO and non-GSO networks and systems only in the band 42-42.5 GHz through modification of No. 5.551G, and the suppression of Resolution 128.

ARTICLE  5

MOD	MLA/99/11

5.551G	In order to protect the radio astronomy service in the band 42.5-43.5 GHz, the aggregate power flux-density produced by all the space stations in any non-geostationary-satellite system in the fixed-satellite service (space-to-Earth) or in the broadcasting-satellite service (space-to-Earth) system operating in the 42-42.5 GHz band shall not exceed, for more than 2% of the time, 
–137 dB(W/(m2 · GHz)) for continuum observations and –153 dB(W/(m2 · 500 kHz)) for spectral line observations in the band 42.5-43.5 GHz, at the site of a radio astronomy station registered as a single dish telescope, and shall not exceed, for more than 2% of the time, –116 dB(W/(m2 · 500 kHz)) at the site of a radio astronomy station where very long baseline interferometry observations are being conducted. The power flux-density produced by any geostationary station in the fixed-satellite service (space-to-Earth) or in the broadcasting-satellite service (space-to-Earth) operating in the band 42-42.5 GHz shall not exceed
, for more than 2% of the time, –137 dB(W/(m2 · GHz)) for continuum observations, and 
–153 dB(W/(m2 · 500 kHz)) for spectral line observations in the band 42.5-43.5 GHz, at the site of a radio astronomy station registered as a single dish telescope, and shall not exceed, for more than 2% of the time, –166 dB(W/(m2 · 500 kHz)) at the site of a radio astronomy station where very long baseline interferometry observations are being conducted. These values shall apply at any radio astronomy station that has been notified to the ITU either before [end of WRC-03] or before the date of receipt of the advance publication information of the space station to which the limits are to apply. For other radio astronomy stations, notified after these dates, agreement may be sought with administrations authorizing the space stations.     (WRC-03)

Reasons:	Consequential to the work of the ITU-R studies to protect the radio astronomy service in the band 42.5-43.5 GHz.

SUP	MLA/99/12

RESOLUTION  128  (Rev.WRC-2000)

Protection of the radio astronomy service in the 42.5-43.5 GHz band

Reasons:	Consequential to the work of the ITU-R.

__________