| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Document 102-E |
|  |  |  | 2 June 2003 |
|  |  |  | Original: English |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | PLENARY MEETING |
| --- | --- |
| Malaysia |  |
| PROPOSALS FOR THE WORK OF THE CONFERENCE |  |
| agenda item 1.24 |  |

1.24	to review the usage of the band 13.75-14 GHz, in accordance with Resolution 733 (WRC-2000), with a view to addressing sharing conditions

# 1	Introduction

The CPM has identified Method B in its report to the WRC-03 in order to maintain the delicate balance between the services involved. This method is based on reducing the current limit on the minimum antenna size to 1.2 m and by adding technical conditions, which would adequately manage the interference caused by FSS earth stations into radiolocation and SRS stations.

The advantages offered by this method are:

–	FSS development would no longer be constrained to use only earth station antennas larger than 4.5 m. This would grant relaxation of the current limit to 1.2 m with the addition of appropriate limits to ensure the protection of radiolocation, thus leaving greater flexibility.

–	By allowing a more flexible operation of FSS earth stations in the band, the mismatch between the up- and downlink capacity for the FSS at Ku-band is improved.

–	A quantified protection from each individual FSS earth station in the new diameter range is afforded to radiolocation systems.

–	Relaxation of the constraints on the radiolocation emissions.

–	SRS is assured a managed interference environment in the wider protected bandwidth.

Therefore, the Malaysian Administration would like to propose to the WRC-03 the adoption of Method B as identified in the CPM Report, with the following modifications to satisfy this agenda item:

–	No changes to footnote 5.502 for FSS antennas greater than or equal to 4.5 m.

–	Earth stations employing uplink power control can exceed the pfd limits during rain by up to the level of the corresponding rain fade.

In addition, the single entry pfd limit is proposed to be specified in the Radio Regulations on a mandatory basis but not subject to compliance verification by the BR.

# 2	Proposals

The Malaysian Administration would like to propose that X to be –111 dB(W/(m210 MHz)) and Y to be 0.5%. These values for X and Y are proposed with the aim to reach an appropriate balance of constraints for both radiolocation and FSS.

ARTICLE  5

MOD	MLA/102/1

11.7-14.25 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 13.75-14 FIXED-SATELLITE (Earth-to-space) 5.484A RADIOLOCATION Earth exploration-satellite Standard frequency and time signal-satellite (Earth-to-space) Space research 5.499 5.500 5.501 MOD 5.502 MOD 5.503 |  |  |

Reasons:	Since most of the provisions of No. 5.503A have expired, it is possible to reflect the remaining provisions by introducing in the table of allocation a secondary allocation to EESS and consequently to suppress No. 5.503A.

MOD	MLA/102/2

5.502	In the band 13.75-14 GHz, an earth stationof a geostationary fixed-satellite service network shall have a minimum antenna diameter of 1.2 m and an earth station of a non-geostationary fixed-satellite service system shall have a minimum antenna diameter of 4.5 m. In addition, the e.i.r.p.averaged over one second, radiated by a station in the radiolocation or radionavigation services shall not exceed 59 dBW above 2º elevation and 65 dBW below. -An administration planning to use fixed-satellite service earth stations with an antenna diameter less than 4.5 m in a geostationary network in this band shall ensure that the single entry power flux-density produced by any earth station operating within its territory does not exceed:

–	–111 dB(W/(m210 MHz)) not to be exceeded for more than 0.5% of the time produced at 36 m above sea level at the normal baseline as defined in the UN Convention of the Law Of the Sea 1982;

–	–111 dB(W/(m210 MHz)) not to be exceeded for more than 0.5% of the time produced 3 m above ground at the border of a neighbouring country deploying land mobile radar in this band unless prior agreement has been obtained.     (WRC-03)

Reasons:

•	Relaxation to 1.2 m of the current limitation on the minimum antenna diameter for FSS earth stations will facilitate an efficient utilization of this band for FSS and alleviate the mismatch between up- and downlink capacity for FSS at Ku-band.

•	With the addition of appropriate pfd limits to ensure the protection of maritime and land mobile radiolocation systems, these get a quantified protection from these new terminals.

•	Relaxation of the constraints on the radiolocation emission will facilitate a more efficient operation of the radiolocation service and make the terminals more resistant to interference from, amongst others, the FSS.

•	The values of X being –111 dB(W/(m210 MHz)) and Y being 0.5% are proposed with the aim to reach an appropriate balance of constraints for both radiolocation and FSS.

•	The suppression of the last two sentences of the current footnote No. 5.502 takes into account the fact that the 59/65 dBW limit on radiolocation average e.i.r.p. is intended to protect FSS space stations. Hence no complaint for harmful interference can be made as long as these limits are met by radiolocation stations.

•	The suppression of the provision relating to the FSS earth station e.i.r.p. levels of 85 and 68 dBW results from the fact that this provision has no regulatory implication. These levels are reflected in Recommendation ITU-R S.1068.

MOD	MLA/102/3

5.503	In the band 13.75-14 GHz, geostationary space stations in the space research service for which information for advance publication has been received by the Bureau prior to 31 January 1992 shall operate on an equal basis with stations in the fixed-satellite service; after that date, new geostationary space stations in the space research service will operate on a secondary basis. Until those geostationary space stations in the space research service for which information for advance publication has been received by the Bureau prior to 31 January 1992 cease to operate in this band:

–	the e.i.r.p. density of emissions from any earth station in the fixed-satellite service operating with a space station in geostationary-satellite orbit shall not exceed4.7D + 28 dB(W/40 kHz), where D is the fixed-satellite service earth station antenna diameter (m) from 13.770 to 13.780 GHz for earth station diameters equal to or greater than 1.2 m and less than 4.5 m;

–	49.2 + 20 log(D/4.5) dB(W/40 kHz), where D is the fixed-satellite service earth station antenna diameter (m) from 13.770 to 13.780 GHz for earth station diameters equal to or greater than 4.5 m and less than 31.9 m;

–	66.2 dB(W/40 kHz) for any fixed-satellite service earth station emission in the band 13.770-13.780 GHz for antenna diameters (m) equal to or greater than 31.9 m;

–	for any fixed-satellite service earth station antenna diameter (m) a maximum e.i.r.p. spectral density of 56.2 dB(W/4 kHz) for narrow band (less than 40 kHz of necessary bandwidth) fixed-satellite service earth station emissions in the band 13.770-13.780 GHz;

–	the e.i.r.p. density of emissions from any earth station in the fixed-satellite service operating with a space station in non-geostationary-satellite orbit shall not exceed 51 dBW in the 6 MHz band from 13.772 to 13.778 GHz.

Automatic power control may be used to increase the e.i.r.p. density in these frequency ranges to compensate for rain attenuation, to the extent that the power flux-density at the fixed-satellite service space station does not exceed the value resulting from use by an earth station of an e.i.r.p.meeting the above limits, in clear-sky conditions.     (WRC-03)

Reasons:	To maintain the current protection of SRS operations and to extend the protected bandwidth to 10 MHz.

SUP	MLA/102/4

5.503A

Reasons:	Since most of the provisions of No. 5.503A have expired, it is possible to reflect the remaining provisions by introducing in the table of allocation a secondary allocation to EESS and consequently to suppress No. 5.503A.

ARTICLE  21

Terrestrial and space services sharing frequency bands above 1 GHz

Section III  –  Power limits for earth stations

ADD	MLA/102/5

21.13bis	In the band 13.75-14 GHz, the level of e.i.r.p. emitted by an earth station of a GSO FSS network shall not exceed the following values6bis:

Angle off-axis	Maximum e.i.r.p. in any 1 MHz band

2				7	43 – 25 log  dBW
	7				9.2	22 dBW
	9.2				48	46 – 25 log  dBW
						4 dBW

These limits do not apply to FSS earth stations brought into service prior to WRC-03.

_______________

6bis	21.13bis.1	Earth stations employing uplink power control can exceed these limits during rain by up to the level of the corresponding rain fade. See also No. 5.502.

Reasons:

•	Relaxation to 1.2 m of the current limitation on the minimum antenna diameter for FSS earth stations will facilitate an efficient utilization of this band for FSS and alleviate the mismatch between up- and downlink capacity for FSS at Ku-band.

•	With the addition of appropriate off-axis e.i.r.p. limits to ensure the protection of aeronautical mobile radiolocation systems, these get a quantified protection from these new terminals.

__________