| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Addendum 5 to Document 109-E |
|  |  |  | 3 June 2003 |
|  |  |  | Original: English |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | PLENARY MEETING |
| --- | --- |
| Korea (Republic of) |  |
| proposals for the work of the conference |  |
| agenda item 1.8.2 |  |

1.8.2	consideration of the results of studies, and proposal of any regulatory measures regarding the protection of passive services from unwanted emissions, in particular from space service transmissions, in response to recommends 5 and 6 of Recommendation 66 (Rev.WRC-2000)

Recommendation  66  (Rev.WRC-2000) – Studies of the maximum permitted levels of unwanted emissions

5	study those frequency bands and instances where, for technical or operational reasons, more stringent spurious emission limits than the general limits in Appendix 3 may be required to protect safety services and passive services such as radio astronomy, and the impact on all concerned services of implementing or not implementing such limits;

6	study those frequency bands and instances where, for technical or operational reasons, out-of-band limits may be required to protect safety services and passive services such as radio astronomy, and the impact on all concerned services of implementing or not implementing such limits;

Introduction

Recommendation 66 (Rev.WRC-2000) contains recommends 5 and 6 to study the more stringent spurious or out-of-band emission limit, if necessary to protect safety or passive services such as the radio astronomy service. During its 5th meeting, ITU-R TG 1/7 developed new Recommendation ITU-R SM.[BbB] for WRC-03 agenda item 1.8.2 which contains the co-existence conditions of passive and active services in adjacent frequency bands. These ITU-R studies have been conducted in over 20 band pairs from 1 350 MHz to 52.8 GHz where compatibility concerns have been raised. And it is possible for the out-of-band emissions of space services (space-to-Earth) in some bands adjacent to those allocated to the radio astronomy service on a primary basis to meet protection criteria of the radio astronomy service defined in Recommendation ITU-R RA.769 without impair affection to the active service at the adjacent or nearby frequency bands.

Therefore, the Republic of Korea believes that inclusion of those pfd limits within the Radio Regulations can provide better protection of passive services especially for the radio astronomy service. And APT developed their final common proposals to establish the protection criteria, where mutual agreements were made, as the mandatory pfd limits in the Radio Regulations.

As shown in Table 1, the Republic of Korea has several single-dish and very long baseline interferometry (VLBI) telescopes for radio astronomy and geodesy.

Thus the Republic of Korea proposes to establish the protection criteria within the Radio Regulations where mutual agreements were made by new Recommendation ITU-R SM.[BbB] and supports APT common proposals for WRC-03 agenda item 1.8.2 with some editorial modifications. The Republic of Korea also proposes to suppress Recommendation 66.

Table 1

Operating sites for RA and geodesy in the Republic of Korea

| Site | Longitude (° ' ") | Latitude (° ' ") | Diameter (m) | Freq. for SDT* | Freq. for VLBI |
| --- | --- | --- | --- | --- | --- |
| TRAO (Taejon) | 127 22 18 | 36 23 54 | 13.7 | 43, 86-116, 127-175 GHz | VLBI for 43 GHz |
| Yonsei Univ. (Seoul) | 126 56 35 | 37 33 44 | 20 | 22, 43, 86-150, 127-175 GHz | VLBI for 2, 8, 22, 43, 86-150 GHz |
| Ulsan Univ. (Ulsan) | 129 15 04 | 35 32 33 | 20 | 22, 43, 86-116 GHz | VLBI for 2, 8, 22, 43, 86-150 GHz |
| Tamna Univ. (Cheju) | 126 27 43 | 33 17 18 | 20 | 22, 43, 86-116 GHz | VLBI for 2, 8, 22, 43, 86-150 GHz |
| *SDT: Single-Dish Radio Telescope |  |  |  |  |  |

Proposals

ARTICLE  5

MOD	KOR/109A5/1

1 350-1 525 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 1 452-1 492 FIXED MOBILE except aeronautical mobile BROADCASTING 5.345 5.347 BROADCASTING- SATELLITE 5.345 5.347 ADD 5.AAA | 1 452-1 492 FIXED MOBILE 5.343 BROADCASTING 5.345 5.347 BROADCASTING-SATELLITE 5.345 5.347 ADD 5.AAA |  |
| 5.341 5.342 | 5.341 5.344 |  |

Reasons:	See Reasons for proposal item KOR/109A5/2.

ADD	KOR/109A5/2

5.AAA 	The power flux-density produced by any geostationary space station in the broadcasting-satellite service (space-to-Earth) operating in the band 1 452-1 492 MHz or the mobile-satellite service (space-to-Earth) operating in the band 1 525-1 559 MHz shall not exceed at the radio astronomy station (depending on the type of observations conducted) operating in the band 1 400-1 427 MHz:

–	180 dB(W/m2) in 27 MHz (for single dish continuum observations);

–	–196 dB(W/m2) in any 20 kHz (for single dish spectral-line observations); or

–	–166 dB(W/m2) in any 20 kHz (for VLBI observations).

These values shall apply at any radio astronomy station that has been notified to the ITU either before the end of WRC-03 or before the date of receipt of the advance publication information of the space station to which the limits are to apply. Other radio astronomy stations, notified after these dates, may seek an agreement with administrations authorizing the space stations.

Reasons:	The frequency band 1 400-1 427 MHz is allocated to passive services only, on a primary basis: the radio astronomy service (RAS), the Earth exploration-satellite service (passive), and the space research service (passive). And in footnote No. 5.340, all emissions are prohibited in the band 1 400-1 427 MHz.

ITU-R studies have resulted in that in the band 1 400-1 427 MHz allocated to the RAS on a primary basis, the unwanted emissions produced by space stations in the BSS (space-to-Earth) operating in the band 1 452-1 492 MHz or the MSS (space-to-Earth) operating in the band 1 525-1 559 MHz can meet the protection criteria for radio astronomical observations in the band 1 400-1 427 MHz (for continuum and VLBI observations, and for spectral line observations with appropriate mitigation techniques).

Therefore, RAS stations above notified to BR earlier than the API of the space station need to be protected, and other RAS stations notified later than the API of the space station need to seek an agreement if they hope to be protected.

MOD	KOR/109A5/3

1 525-1 610 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 1 525-1 530 SPACE OPERATION (space-to-Earth) FIXED MOBILE-SATELLITE (space-to-Earth) 5.351A ADD 5.AAA ADD 5.BBB Earth exploration-satellite Mobile except aeronautical mobile 5.349 | 1 525-1 530 SPACE OPERATION (space-to-Earth) MOBILE-SATELLITE (space-to-Earth) 5.351A ADD 5.AAA ADD 5.BBB Earth exploration-satellite Fixed Mobile 5.343 | 1 525-1 530 SPACE OPERATION (space-to-Earth) FIXED MOBILE-SATELLITE (space-to-Earth) 5.351A ADD 5.AAA ADD 5.BBB Earth exploration-satellite Mobile 5.349 |
| 5.341 5.342 5.350 5.351 5.352A 5.354 | 5.341 5.351 5.354 | 5.341 5.351 5.352A 5.354 |

| 1 530-1 535 SPACE OPERATION (space-to-Earth) MOBILE-SATELLITE (space-to-Earth) 5.351A 5.353A ADD 5.AAA ADD 5.BBB Earth exploration-satellite Fixed Mobile except aeronautical mobile | 1 530-1 535 SPACE OPERATION (space-to-Earth) MOBILE-SATELLITE (space-to-Earth) 5.351A 5.353A ADD 5.AAA ADD 5.BBB Earth exploration-satellite Fixed Mobile 5.343 |
| --- | --- |
| 5.341 5.342 5.351 5.354 | 5.341 5.351 5.354 |
| 1 535-1 559 MOBILE-SATELLITE (space-to-Earth) 5.351A ADD 5.AAA ADD 5.BBB 5.341 5.351 5.353A 5.354 5.355 5.356 5.357 5.357A 5.359 5.362A |  |

MOD	KOR/109A5/4

1 559-1 610 AERONAUTICAL RADIONAVIGATION RADIONAVIGATION-SATELLITE (space-to-Earth) (space-to-space) 5.329A ADD 5.BBB ADD 5.CCC 5.341 5.362B 5.362C 5.363

Reasons:	See the Reasons for proposal items KOR/109A5/2, KOR/109A5/5 and KOR/109A5/6.

ADD	KOR/109A5/5

5.BBB	The power flux-density produced by any geostationary space station in the mobile-satellite service (space-to-Earth) operating in the band 1 525-1 559 MHz, radionavigation-satellite service (space-to-Earth) operating in the band 1 559-1 610 MHz or mobile-satellite service (space-to-Earth) operating in the band 1 613.8-1 626.5 MHz shall not exceed at a radio astronomy station (depending on the type of observations conducted) operating in the band 1 610.6-1 613.8 MHz:

–	–194 dB(W/m2) in any 20 kHz (for single dish spectral-line observations); or

–	–166 dB(W/m2) in any 20 kHz (for VLBI observations).

These values shall apply at any radio astronomy station that has been notified to the ITU either before the end of WRC-03 or before the date of receipt of the advance publication information (API) of the space station to which the limits are to apply. Other radio astronomy stations, notified after these dates, may seek an agreement with administrations authorizing the space stations.

Reasons:	ITU-R studies have resulted in that in the band 1 610.6-1 613.8 MHz allocated to the RAS on a primary basis, the unwanted emissions produced by space stations in the MSS (space-to-Earth) operating in the band 1 525-1 559 MHz, RNSS (space-to-Earth) operating in the band 1 559-1 610 MHz or MSS (space-to-Earth) operating in the band 1 613.8-1 626.5 MHz can meet the protection criteria for radio astronomical observations in the band 1 610.6-1 613.8 MHz for VLBI and spectral line observations.

Therefore, RAS stations above notified to the BR earlier than the API of the space station need to be protected, and other RAS stations notified later than the API of the space station need to seek an agreement if they hope to be protected.

ADD	KOR/109A5/6

5.CCC	The equivalent power flux-density produced by all space stations in any non-geostationary-satellite systems in the radionavigation-satellite service (space-to-Earth) operating in the band 1 559-1 610 MHz shall not exceed at a radio astronomy station (depending on the type of observations conducted), operating in the band 1 610.6-1 613.8 MHz for more than 2% of the time:

–	–258 dB(W/m2) in any 20 kHz (for single dish spectral-line observations); or

–	–230 dB(W/m2) in any 20 kHz (for VLBI observations).

These epfd shall be evaluated using the antenna pattern given in Annex 2 to Recommendation ITU-R S.1586 and a maximum RAS antenna gain of 64 dBi and shall be applied for elevation angles higher than the minimum operating angle θmin of the radio telescope (for which a default value of 5° should be adopted in the absence of notified information).

These values shall apply at any radio astronomy station that has been notified to the ITU either before the end of WRC-03 or before the date of receipt of the advance publication information of the space station to which the limits are to apply. Other radio astronomy stations, notified after these dates, may seek an agreement with administrations authorizing the space stations.

Reasons:	ITU-R studies have resulted in that in the band 1 610.6-1 613.8 MHz allocated to the RAS on a primary basis the unwanted emissions produced by all space stations in non-geostationary-satellite systems in the RNSS (space-to-Earth) operating in the 1 559-1 610 MHz can meet the protection criteria for radio astronomical observations in the band 1610.6-1 613.8 MHz (for continuum and VLBI observations, and for spectral line observations with appropriate mitigation techniques).

Therefore, RAS stations above notified to the BR earlier than the API of the space station need to be protected, and other RAS stations notified later than the API of the space station need to seek an agreement if they hope to be protected.

MOD	KOR/109A5/7

1 610-1 660 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 1 613.8-1 626.5 MOBILE-SATELLITE (Earth-to-space) 5.351A AERONAUTICAL RADIONAVIGATION Mobile-satellite (space-to-Earth) ADD 5.BBB | 1 613.8-1 626.5 MOBILE-SATELLITE (Earth-to-space) 5.351A AERONAUTICAL RADIONAVIGATION RADIODETERMINATION- SATELLITE (Earth-to-space) Mobile-satellite (space-to-Earth) ADD 5.BBB | 1 613.8-1 626.5 MOBILE-SATELLITE (Earth-to-space) 5.351A AERONAUTICAL RADIONAVIGATION Mobile-satellite (space-to-Earth) ADD 5.BBB Radiodetermination-satellite (Earth-to-space) |
| 5.341 5.355 5.359 5.363 5.364 5.365 5.366 5.367 5.368 5.369 5.371 5.372 | 5.341 5.364 5.365 5.366 5.367 5.368 5.370 5.372 | 5.341 5.355 5.359 5.364 5.365 5.366 5.367 5.368 5.369 5.372 |

Reasons:	See the Reasons for proposal item KOR/109A5/5.

MOD	KOR/109A5/8

10-11.7 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 10.7-11.7 FIXED FIXED-SATELLITE (space-to-Earth) 5.441 5.484A ADD 5.DDD (Earth-to-space) 5.484 MOBILE except aeronautical mobile | 10.7-11.7 FIXED FIXED-SATELLITE (space-to-Earth) 5.441 5.484A ADD 5.DDD MOBILE except aeronautical mobile | 10.7-11.7 FIXED FIXED-SATELLITE (space-to-Earth) 5.441 5.484A ADD 5.DDD MOBILE except aeronautical mobile |

Reasons:	See the Reasons for proposal item KOR/109A5/9.

ADD	KOR/109A5/9

5.DDD 	The power flux-density produced by any geostationary space station in the fixed-satellite service (space-to-Earth) operating in the band 10.7-10.95 GHz shall not exceed at a radio astronomy station operating only in the very long baseline interferometry mode in the band 10.6-10.7 GHz:

–	–145 dB(W/m2) in any 50 kHz.

These values shall apply at any radio astronomy station that has been notified to the ITU either before the end of WRC-03 or before the date of receipt of the advance publication information of the space station to which the limits are to apply. Other radio astronomy stations, notified after these dates, may seek an agreement with administrations authorizing the space stations.

Reasons:	ITU-R studies have resulted in that in the band 10.6-10.7 GHz allocated to the RAS on a primary basis the unwanted emissions produced by space stations in the FSS (space-to-Earth) operating in the 10.7-10.95 GHz can meet the protection criteria for the VLBI observations.

Therefore, RAS stations above notified to the BR earlier than the API of the space station need to be protected, and other RAS stations notified later than the API of the space station need to seek an agreement if they hope to be protected.

MOD	KOR/109A5/10

18.6-22.21 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 21.4-22 FIXED MOBILE BROADCASTING- SATELLITE 5.530 ADD 5.EEE | 21.4-22 FIXED MOBILE | 21.4-22 FIXED MOBILE BROADCASTING- SATELLITE 5.530 ADD 5.EEE 5.531 |

Reasons:	See the Reasons for the proposal item KOR/109A5/11.

ADD	KOR/109A5/11

5.EEE 	The power flux-density produced by any geostationary space station in the broadcasting-satellite service (space-to-Earth) operating in the band 21.4-22 GHz shall not exceed at a radio astronomy station operating only in the very long baseline interferometry mode in the band 22.21-22.5 GHz:

–	–128 dB(W/m2) in any 250 kHz.

These values shall apply at any radio astronomy station that has been notified to the ITU either before the end of WRC-03 or before the date of receipt of the advance publication information of the space station to which the limits are to apply. Other radio astronomy stations, notified after these dates, may seek an agreement with administrations authorizing the space stations.

Reasons:	ITU-R studies have resulted in that in the band 22.21-22.5 GHz allocated to the RAS on a primary basis the unwanted emissions produced by space stations in the BSS (space-to-Earth) operating in the 21.4-22 GHz can meet the protection criteria for the VLBI observations.

Therefore, RAS stations above notified to the BR earlier than the API of the space station need to be protected, and other RAS stations notified later than the API of the space station need to seek an agreement if they hope to be protected.

APPENDIX  4  (WRC-2000)

Consolidated list and tables of characteristics for use in the 
application of the procedures of Chapter III

ANNEX  2A

Characteristics of satellite networks’ earth stations 
or radio astronomy stations2     (WRC-2000)

...

## A.17	Compliance with aggregate power flux-density limits

...

ADD	KOR/109A5/12

e)	For non-geostationary-satellite systems operating in the radionavigation-satellite service in the band 1 559-1 610 MHz, the calculated equivalent power flux-density produced at the site of a radio astronomy station in any 20 kHz bandwidth for more than 2% of time in the band 1 610.3-1 613.8 MHz, as defined in No. 5.CCC.

Reasons:	Consequential changes of proposals KOR/109A5/3 and KOR/109A5/6.

ADD	KOR/109A5/13

## C.13bis	Type of observations

The type of radio astronomy observations to be taken on the frequency band shown in § C.3 b). The different types of radio astronomy observations are single dish (spectral-line and continuum) or VLBI.

Reasons:	Consequential changes of proposals KOR/109A5/1 through KOR/109A5/11.

ADD	KOR/109A5/14

draft  RESOLUTION  [KOR/XXA5/PSV]  (WRC-03)

Regulatory provisions for the maximum permitted 
levels of unwanted emissions

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that Appendix 3 of the Radio Regulations specifies the maximum permitted levels of spurious and spurious domain emissions, in terms of the mean power level of any spurious and spurious domain components supplied by a transmitter to the antenna transmission line;

b)	that the principal objective of Appendix 3 is to specify the maximum permitted levels of spurious and spurious domain emissions that, while being achievable, provide protection against harmful interference;

c)	that excessive levels of unwanted emissions may give rise to harmful interference;

d)	that Appendix 3 is applied generally to the mean power of a transmitter and its spurious emissions, it also takes into account a variety of emissions where interpretation of the term “mean power”, and thus its measurement, would be difficult, particularly in the cases of digital modulation broadband systems, pulsed modulation and narrow-band high power transmitters;

e)	that unwanted emissions from transmitters operating in space stations may cause harmful interference, particularly emissions from wideband amplifiers which cannot be adjusted after launch;

f)	that unwanted emissions may cause harmful interference to the radio astronomy service and space services using passive sensors;

g)	that, for technical or operational reasons, more stringent spurious limits than the general limits in Appendix 3 may be required to protect specific services, such as the safety services, the radio astronomy service and the space services using passive sensors in specific bands and situations;

h)	that, for the purpose to avoid harmful interference, it is impossible for the radio astronomy service and the space services using passive sensors to shift frequencies to observe/measure since the frequencies are determined by physical laws of the nature;

i)	that broadband digital modulation may cause unwanted emissions at frequencies far from the carrier frequency;

j)	that Recommendation ITU-R SM.[BbB] provides compatibility analysis between a passive service and an active service in adjacent and nearby bands;

k)	that out-of-band emission can be controlled to a certain extent in the design process of satellite through careful design methods but that some narrow-band spurious emissions can be generated through some uncontrollable physical mechanisms,

noting

a)	that ITU-R has decided to recommend not including general out-of-band limits in the Radio Regulations at WRC-2000;

b)	that ITU-R has developed Recommendations ITU-R S.1586 and ITU-R M.1583, both providing a methodology based on the epfd concept for calculations of interference into radio astronomy stations from non-GSO systems of mobile-satellite or radionavigation-satellite services in the first case, and, from the fixed-satellite services in the other case, and containing a model of a radiotelescope antenna pattern;

c)	that ITU-R has developed Recommendation ITU-R RA.1513 providing acceptable levels of data loss to radio astronomy observations, stating in particular that the percentage of data loss caused by any system should be lower than 2%;

d)	that the Bureau is not involved in the consultation procedure defined in this Resolution;

e)	that consultation between administrations may lead to solutions offering the potential for rapid implementation,

recognizing

that it is necessary to ensure an equitable burden sharing for achieving compatibility between the active and passive services,

resolves

that, with regard to the protection of the passive services from unwanted emission of active services, a future competent conference should establish the appropriate regulatory provisions such as the emission limits or operational constraints, and the date of the entry into force of these provisions, taking into account the result of ITU-R studies,

invites ITU-R

1	to continue to study those frequency bands and instances where, for technical or operational reasons, more stringent spurious emission limits than the general limits in Appendix 3 may be required to protect passive services such as the radio astronomy service, and the impact on all concerned services of implementing or not implementing such limits;

2	to continue to study for those frequency bands and instances where ITU-R could not conclude satisfactory results for active and passive services, and where, for technical or operational reasons, out-of-band limits may be required to protect safety services and passive services such as the radio astronomy service, and the impact on all concerned services of implementing or not implementing such limits,

urges administrations

1	that, whenever practicable, space stations be operated under the condition of the unwanted emission levels as given in the Radio Regulations;

2	to actively participate in the studies of ITU-R.

Reasons:	The Republic of Korea considers that the band-by-band studies by ITU-R, which conclude that passive services and active services are compatible with practical measures, provide adequate protection to the passive services without unduly inhibiting the development of new

applications. In this regard, it is necessary for a future competent conference to establish the appropriate regulatory provisions such as the emission limits or operational constraints, and the date of the entry into force of these provisions, taking into account the result of the band-by-studies of ITU-R.

SUP	KOR/109A5/15

RECOMMENDATION  66  (Rev.WRC-2000)

Studies of the maximum permitted levels of unwanted emissions

Reasons:	The Republic of Korea considers that Recommendation 66 should be necessary to suppress. With respect to the issue raised in “recommends 1 and 2” in this Recommendation, it could be performed under normal activities of ITU-R. And the issues of “recommends 3, 5, 6 and 8” in this Recommendation are contained in the new proposed Resolution [KOR/XXA5/PSV] (WRC-03). The content of “recommends 4” has been finished in the study cycle toward WRC-03. Additionally, the issue of “recommends 7” has not been a serious issue, and the out-of-band issue has already been included in Recommendation ITU-R SM.1541.

____________