| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Document 140-E |
|  |  |  | 12 June 2003 |
|  |  |  | Original: English |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | COMMITTEE 8 |
| --- | --- |
| FIRST SERIES OF TEXTS SUBMITTED BY COMMITTEE 6 TO THE EDITORIAL COMMITTEE |  |

ARTICLE  5

Frequency allocations

MOD	5.487	COM6/140/1 (English text only)

5.487	In the band 11.7-12.5 GHz in Regions 1 and 3, the fixed, fixed-satellite, mobile, except aeronautical mobile, and broadcasting services, in accordance with their respective allocations, shall not cause harmful interference to, or claim protection from, broadcasting-satellite stations operating in accordance with the Regions 1 and 3 Plan in Appendix 30.     (WRC-03)

MOD	COM6/140/2

APPENDIX  30*  (WRC-03)

Provisions for all services and associated Plans and List[**] for
the broadcasting-satellite service in the frequency bands
11.7-12.2 GHz (in Region 3), 11.7-12.5 GHz (in Region 1)
             and 12.2-12.7 GHz (in Region 2)     (WRC-03)

(See Articles 9 and 11 of the Radio Regulations)

ADD	COM6/140/3

_______________

[**	The Regions 1 and 3 List of additional uses is annexed to the Master International Frequency Register (see Resolution 542 (WRC-2000)).      (WRC-03)]

ARTICLE  1     (WRC-2000)

General definitions

MOD	COM6/140/4

1.8	Regions 1 and 3 List of additional uses (hereafter called in short the “List”): The List of assignments for additional uses in Regions 1 and 3 as established by WRC-2000 (see Resolution 542 (WRC-2000)), as updated following the successful application of the procedure of § 4.1 of Article 4.       (WRC-03)

ADD 	COM6/140/5

1.9	Frequency assignment in conformity with the List: Any frequency assignment which appears in the List as updated following successful application of § 4.1 of Article 4.      (WRC-03)

ARTICLE  4     (WRC-2000)

Procedures for modifications to the Region 2 Plan or 
for additional uses in Regions 1 and 32

MOD 	COM6/140/6

4.1.26	The procedure of this Article may be applied by the administration of a new ITU Member State in order to include new assignments in the List. Upon completion of the procedure, the next World Radiocommunication Conference may be requested to consider, among the assignments included in the List after the successful completion of this procedure, the inclusion in the Plan of up to 10 channels (for Region 1) and up to 12 channels (for Region 3), over the national territory of the new Member State.      (WRC-03)

ARTICLE  7     (WRC-2000)

Coordination, notification and recording in the Master International Frequency Register of frequency assignments to stations in the fixed-satellite service (space-to-Earth) in the bands 11.7-12.2 GHz (in Region 2),
12.2-12.7 GHz (in Region 3) and 12.5-12.7 GHz (in Region 1), and to stations in the broadcasting-satellite service in the band 12.5-12.7 GHz (in Region 3) 
when frequency assignments to broadcasting-satellite stations in
the bands 11.7-12.5 GHz in Region 1, 12.2-12.7 GHz in Region 2
and 11.7-12.2 GHz in Region 3 are involved11

MOD 	COM6/140/7

_______________

11	These provisions do not replace the procedures prescribed in Articles 9 and 11 when stations other than those in the broadcasting-satellite service subject to this Appendix are involved.       (WRC-03)

MOD 	COM6/140/8

7.2.1	The frequency assignments to be taken into account are:

a)	the assignments in conformity with the appropriate Regional Plan in Appendix 30;

b)	the assignments included in the Regions 1 and 3 List;

c)	the assignments for which the procedure of Article 4 has been initiated, as from the date of receipt of the complete Appendix 4 information under § 4.1.3 or 4.2.6.      (WRC-03)

ARTICLE  9

Power flux-density limits between 12.2 GHz and 12.7 GHz to protect terrestrial services in Regions 1 and 3 from interference
from Region 2 broadcasting-satellite space stations

MOD 	COM6/140/9

9.1	The power flux-density at the Earth’s surface in Regions 1 and 3, produced by emissions from a space station in the broadcasting-satellite service in Region 2 for all conditions and for all methods of modulation shall not exceed the values given in Section 4 of Annex 1 on the territory of any country unless the administration of that country has so agreed.      (WRC-03)

ANNEX  5

Technical data used in establishing the provisions and associated Plans
and the Regions 1 and 3 List, which should be used
               for their application22     (WRC-2000)

MOD 	COM6/140/10

## 3.4	Protection ratio between television signals

For developing the original 1977 broadcasting-satellite service Plan for Regions 1 and 3, the following protection ratios were used27, 28:

–	31 dB for co-channel signals;

–	15 dB for adjacent channel signals.

For revising this Plan at WRC-97, the following aggregate downlink protection ratios were specified in Recommendation ITU-R BO.1297 for the purpose of calculating downlink equivalent protection margins28, 29, 30:

–	24 dB for co-channel signals;

–	16 dB for adjacent channel signals.

In revising the Regions 1 and 3 Plan at WRC-97, the following aggregate overall protection ratio values were used for calculating the overall co-channel and adjacent-channel protection margins as defined in § 1.8 and 1.9:

–	23 dB for co-channel signals;

–	15 dB for adjacent channel signals.

It was also specified that for the revision of the Regions 1 and 3 Plan, no overall co-channel single entry C/I should be lower than 28 dB.

However, for the assignments notified, which are in conformity with this Appendix, brought into use, and for which the date of bringing into use has been confirmed to the Bureau before 27 October 1997, the overall equivalent protection margins were calculated using a co-channel overall protection ratio of 30 dB and lower and upper overall adjacent channel protection ratios of 14 dB31.

WRC-2000 adopted, for the protection of digital assignments from digital emissions, the following protection ratio values to be applied for calculation of downlink equivalent protection margins of the WRC-2000 Regions 1 and 3 Plan:

–	21 dB for co-channel signals;

–	16 dB for adjacent channel signals.

During planning at WRC-2000, these values were used for all assignments of the Regions 1 and 3 Plan and List except those for which WRC-2000 adopted different values used in the planning process32.

Revision of the Regions 1 and 3 Plan at WRC-97 and planning at WRC-2000 were generally based on a set of reference parameters such as the average e.i.r.p., the reference earth station receiving antenna, all test points placed within the –3 dB contour, a bandwidth of 27 MHz and the predetermined value of C/N. The Regions 1 and 3 Plan as established by WRC-2000 is generally based on the use of digital modulation.

Protection masks and associated calculation methods for interference into broadcast satellite systems involving digital emissions inshall be in accordance with Recommendation ITU-R BO.1293-2 (Annexes 1 and 231bis).-

In Region 2, the following protection ratios have been adopted for the purpose of calculating the overall equivalent protection margin32bis:

	28 dB for co-channel signals;

	13.6 dB for adjacent-channel signals;

	–9.9 dB for second adjacent-channel signals.

In Region 2, as a guide for planning, the reduction in the overall C/I ratio due to co-channel interference in the feeder link is taken as equivalent to a degradation in the downlink co-channel C/I ratio of approximately 0.5 dB not exceeded for 99% of the worst month, but the feeder-link and downlink Plans are evaluated on the basis of the overall equivalent protection margin, which includes the combined downlink and feeder-link contributions.

In Region 2, an overall equivalent protection margin of 0 dB, or greater, indicates that the individual protection ratios have been met for the co-channel, the adjacent channels and the second adjacent channels.     (WRC-03)

ADD 	COM6/140/11

_______________

31bis	Annex 3 of this Recommendation may be applied only in compatibility analysis for bilateral coordination between administrations.     (WRC-03)

ADD	COM6/140/12

_______________

32bis	The definition in § 1.7, 1.8, 1.9, 1.10 and 1.11 of the Annex apply to these calculations.     (WRC-03)

## 3.7	Receiving antennas

MOD 	COM6/140/13

### 3.7.2	Receiving antenna reference patterns

Note by the Secretariat: The text of this sub-section appearing on pages 542, 543 and 544 up to Figure 7bis, including Figure 7bis remains unmodified. Modifications indicated shall be introduced in formulas between Figure 7bis and Figure 8 on pages 544 and 545. The text of this sub-section on pages 546 and 548 remains unmodified.

Co-polar pattern:

for  0        m

where:

Gco ()    G1    29  –  25 log r	for m        r

where:

Gco ()    29  –  25 log 	for  r        b

where:

Gco ()    – 5 dBi	for  b	      70

Gco ()    0 dBi	for  70	      180

Cross-polar pattern:

Gcross ()    Gmax  –  25	for  0        0.25 0

where:

0    2λD30.0025    3 dB beamwidth

Gcross ()    Gmax  –  25    8 	for     0.25 0	      0.44 0

Gcross ()    Gmax  –  17	for     0.44 0	      0

Gcross ()    Gmax  –  17  –  C 	for      0	      1

where:

 	is the wavelength corresponding to 12.1 GHz (metres);

C 	= 21 – 25 log 1 – (Gmax – 17);

j1=j0210.1875

Gcross ()    21  –  25 log 	for  1        2

where:

Gcross ()    – 5 dBi	for  2	      70

Gcross ()    0 dBi	for  70	      180

The reference frequency used in calculations for this antenna pattern = 12.1 GHz.

For the 0.60 m antenna pattern, which was used as a reference receiving antenna in replanning the absolute gain of 35.5 dBi was applied.

## 3.9	Guardbands

MOD 	COM6/140/14

3.9.4	The guardbands at both the lower and upper edges may be used to provide space operations functions in accordance with No. 1.23 in support of the operation of geostationary-satellite networks in the broadcasting-satellite service.      (WRC-03)

## 3.13	Transmitting antennas

MOD 	COM6/140/15

### 3.13.3	Transmitting antenna reference patterns

Note by the Secretariat: The text of this sub-section appearing on pages 549-554, 556 and Figure 13 on page 555 remain unmodified. Modifications indicated shall be introduced in formulas on page 555 after Figure 13.

Curve A:	co-polar relative gain (dB relative to main beam gain):

ΔG = minΔG1,ΔG2

where:

ΔG1 = −12ϕϕ02	for 0	  (/0)  0.5

ΔG1 = −12ϕϕ0−xBminϕ02	for 0.5	  (/0)   1.45ϕ0Bmin+x

ΔG1 = −25.3G1 = –25.23	for 1.45ϕ0Bmin+x   (/0)    1.45

MOD 	COM6/140/16

ANNEX  7     (WRC-03)

Orbital position limitations

A	In applying the procedure of Article 4 for proposed modifications to the Region 2 Plan or for proposed new or modified assignments in the Regions 1 and 3 List, administrations should observe the following criteria:

1)	No broadcasting satellite serving an area in Region 1 and using a frequency in the band 11.7-12.2 GHz shall occupy a nominal orbital position further west than 37.2 W or further east than 146° E.

2)	No broadcasting satellite serving an area in Region 2 that involves an orbital position different from that contained in the Region 2 Plan shall occupy a nominal orbital position:

a)	further east than 54 W in the band 12.5-12.7 GHz; or

b)	further east than 44 W in the band 12.2-12.5 GHz; or

c)	further west than 175.2 W in the band 12.2-12.7 GHz.

However, modifications necessary to resolve possible incompatibilities during the incorporation of the Regions 1 and 3 feeder-link Plan into the Radio Regulations shall be permitted.

3)	The purpose of the following orbital position and e.i.r.p. limitations is to preserve access to the geostationary-satellite orbit by the Region 2 fixed-satellite service in the band 11.7-12.2 GHz. Within the orbital arc of the geostationary-satellite orbit between 37.2 W and 10° E, the orbital position associated with any proposed new or modified assignment in the Regions 1 and 3 List of additional uses shall lie within one of the portions of the orbital arc listed in Table 1. The e.i.r.p. of such assignments shall not exceed 56 dBW, except at the positions listed in Table 2.

TABLE  1

Allowable portions of the orbital arc between 37.2°W and 10° E for new or modified 
assignments in the Regions 1 and 3 Plan and List

| TABLE 2 Orbital position | 37.2° W to 36° W | 33.5° W to 32.5° W | 30° W to 29° W | 26° W to 24° W | 20° W to 18° W | 14° W to 12° W | 8° W to 6° W | 4° W 1 | 2° W to 0° | 4° E to 6° E | 9° E 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 Proposed new or modified assignments in the List which involve this orbital position shall not exceed the pfd limit –138 dB(W/(m2  27 MHz)) at any point in Region 2. |  |  |  |  |  |  |  |  |  |  |  |

Nominal positions in the orbital arc between 37.2° W and 10° E at which the 
e.i.r.p. may exceed the limit of 56 dBW

| B The Region 2 Plan is based on the grouping of the space stations in nominal orbital positions of  0.2º from the centre of the cluster of satellites. Administrations may locate those satellites within a cluster at any orbital position within that cluster, provided they obtain the agreement of administrations having assignments to space stations in the same cluster. (See § 4.13.1 of Annex 3 to Appendix 30A.) Orbital position | 37° W  0.2° | 33.5° W | 30° W | 25° W  0.2° | 19° W  0.2° | 13° W  0.2° | 7° W  0.2° | 4° W 1 | 1° W  0.2° | 5° E  0.2° | 9° E 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 Proposed new or modified assignments in the List which involve this orbital position shall not exceed the pfd limit –138 dB(W/(m2  27 MHz)) at any point in Region 2. |  |  |  |  |  |  |  |  |  |  |  |

MOD 	COM6/140/17

APPENDIX  30A*  (WRC-03)

Provisions and associated Plans and Lists[1] for feeder links for the broadcasting-satellite service (11.7-12.5 GHz in Region 1, 12.2-12.7 GHz
in Region 2 and 11.7-12.2 GHz in Region 3) in the frequency
bands 14.5-14.8 GHz2 and 17.3-18.1 GHz in Regions 1 and 3,
           and 17.3-17.8 GHz in Region 2     (WRC-03)

(See Articles 9 and 11 of the Radio Regulations)

ADD 	COM6/140/18

_______________

*	The expression “frequency assignment to a space station”, wherever it appears in this Appendix, shall be understood to refer to a frequency assignment associated with a given orbital position.      (WRC-03)

MOD 	COM6/140/19

_______________

[1	The Regions 1 and 3 feeder-link List of additional uses are annexed to the Master International Frequency Register (see Resolution 542 (WRC-2000)).     (WRC-03)]

ARTICLE  1     (WRC-2000)

General definitions

MOD 	COM6/140/20

1.10	Regions 1 and 3 feeder-link List of additional uses (hereafter called in short the “feeder-link List”): The lists of assignments for additional uses in Regions 1 and 3 as established by WRC-2000 (see Resolution 542 (WRC-2000)), as updated following the successful application of the procedure of § 4.1 of Article 4.     (WRC-03)

ADD 	COM6/140/21

1.11	Frequency assignment in conformity with the feeder-link List: Any frequency assignment which appears in the feeder-link List as updated following successful application of § 4.1 of Article 4.     (WRC-03)

ARTICLE  4     (WRC-2000)

Procedures for modifications to the Region 2 feeder-link Plan 
or for additional uses in Regions 1 and 3

MOD 	COM6/140/22

4.1.15	The Bureau shall publish in a Special Section of its BR IFIC the information received under § 4.1.12, together with the names of any administrations with which the provisions of this Article have been successfully applied. The frequency assignment concerned shall be included in the feeder-link List.     (WRC-03)

MOD 	COM6/140/23

4.1.23	When a frequency assignment included in the feeder-link List is no longer required, the administration concerned shall immediately so inform the Bureau. The Bureau shall publish this information in a Special Section of its BR IFIC and delete the assignment from the feeder-link List.     (WRC-03)

MOD 	COM6/140/24

4.1.24	No assignment in the feeder-link List shall have a period of operation exceeding 15 years, counted from the date of bringing into use, or 2 June 2000, whichever is later. Upon request by the responsible administration received by the Bureau at the latest three years before the expiry of this period, this period may be extended by up to 15 years, on condition that all the characteristics of the assignment remain unchanged.     (WRC-03)

MOD 	COM6/140/25

4.1.25	Where an administration already having included in the feeder-link List two assignments (not including those systems notified on behalf of a group of named administrations and included in the feeder-link List by WRC-2000) in the same channel and covering the same service area, proposes to include in the feeder-link List a new assignment in the same channel over this same service area, it shall apply the following in respect of another administration which has no assignment in the feeder-link List in the same channel and which proposes to include in the feeder-link List a new assignment:

a)	if the agreement of the former administration is required following the application of § 4.1 by the latter administration, in order to protect the new assignment proposed by the former administration from interference caused by the assignment proposed by the latter administration, both administrations shall make every possible effort to resolve the difficulties by means of mutually acceptable adjustments to their networks;

b)	in case of continuing disagreement, and if the former administration has not communicated to the Bureau the information specified in Annex 2 to Resolution 49 (Rev.WRC-03), this administration shall be deemed to have given its agreement to inclusion in the feeder-link List of the assignment of the latter administration.      WRC-03)

MOD 	COM6/140/26

4.1.26	The procedure of this Article may be applied by the administration of a new ITU Member State in order to include new assignments in the feeder-link List. Upon completion of the procedure, the next world radiocommunication conference may be requested to consider, among the assignments included in the feeder-link List after the successful completion of this procedure, the inclusion in the Regions 1 and 3 feeder-link Plan of up to 10 channels (for Region 1) and up to 12 channels (for Region 3), over the national territory of the new Member State.     (WRC-03)

MOD 	COM6/140/27

4.1.27	When an administration has successfully applied this procedure and received all the agreements7 required to include in the feeder-link List assignments over its national territory, at an orbital location and/or in channels different from those appearing in the Regions 1 and 3 feeder-link Plan for its country, it may request the next world radiocommunication conference to consider the inclusion in this Plan of up to 10 (for Region 1) and up to 12 (for Region 3) of these assignments, in replacement of its assignments appearing in this Plan.     (WRC-03)

MOD 	COM6/140/28

4.1.28	The feeder-link List, as updated, shall be published periodically by the Bureau.     (WRC-03)

MOD 	COM6/140/29

4.1.29	New or modified assignments in the feeder-link List shall be limited to digital modulation.     (WRC-03)

ARTICLE  7     (WRC-2000)

Coordination, notification and recording in the Master International Frequency Register of frequency assignments to stations in the fixed-satellite service (space-to-Earth) in Regions 1, 2 and 3 in the band 17.7-18.1 GHz, 
to stations in the fixed-satellite service (Earth-to-space) in Region 2 in 
the band 17.8-18.1 GHz and to stations in the broadcasting-satellite 
service in Region 2 in the band 17.3-17.8 GHz when frequency 
assignments to feeder links for broadcasting-satellite stations 
in the 17.3-18.1 GHz band in Regions 1 and 3 or in the 
band 17.3-17.8 GHz in Region 2 are involved

MOD 	COM6/140/30

7.2.1	The frequency assignments to be taken into account are:

a)	the assignments in conformity with the appropriate Regional feeder-link Plan in Appendix 30A;

b)	the assignments included in the Regions 1 and 3 feeder-link List;

c)	the assignments for which the procedure of Article 4 has been initiated as from the date of receipt of the complete Appendix 4 information under § 4.1.3 or 4.2.6.     (WRC-03)

MOD 	COM6/140/31

7.9	The provisions of No. 9.17A and the associated provisions under Articles 9 and 11 and Appendix 5 are applicable to fixed-satellite service and broadcasting-satellite service receiving earth stations, in respect of frequency assignments to transmitting broadcasting-satellite service feeder-link earth stations, in the fixed-satellite service in the bands 17.3-18.1 GHz in Regions 1 and 3 and 17.3-17.8 GHz in Region 2 which correspond to assignments to receiving broadcasting-satellite service feeder-link space stations already included in the Regions 1 and 3 feeder-link List, or for which the procedure of Article 4 has been initiated, as from the date of receipt of the complete Appendix 4 information.

MOD 	COM6/140/32

ANNEX  3

Technical data used in establishing the provisions and associated
Plans and Regions 1 and 3 feeder-link List, which should
                  be used for their application21     (WRC-03)

# 3	Basic technical characteristics for Regions 1 and 3

MOD 	COM6/140/33

## 3.1	Translation frequency and guardbands

a)	17 GHz feeder-links

The feeder-link Plan generally uses a frequency translation of 5.6 GHz between the 17 GHz feeder-link channels and the 12 GHz downlink channels. Other values of the translation frequency may be used, provided that the corresponding channels have been assigned to the space station of the administration concerned.

With the value of frequency translation between the feeder-link frequency band (17.3-18.1 GHz in Regions 1 and 3) and the downlink frequency band (11.7-12.5 GHz in Region 1 and 11.7-12.2 GHz in Region 3), the guardbands specified in § 3.9 of Annex 5 to Appendix 30 for the downlink Plan result in corresponding guardband bandwidths of 11 MHz at the upper and 14 MHz at the lower feeder-link band edges. These feeder-link guardbands may be used to provide space operation functions in accordance with No. 1.23 in support of the operation of geostationary-satellite networks in the broadcasting-satellite service.     (WRC-03)

NOC

b)	14 GHz feeder-links

NOC

c)	Frequency translation rules

## 3.3	Protection ratios

MOD 	COM6/140/34

For planning in Regions 1 and 3 at the 1988 Conference (WARC Orb-88), the following protection ratios were applied for the purpose of calculating the feeder-link equivalent protection margins26:

–	co-channel protection ratio  40 dB;

–	adjacent channel protection ratio  21 dB.

The method for the calculation of the feeder-link equivalent protection margin is given in § 1.7.

For revising the Regions 1 and 3 feeder-link Plan at WRC-97, the corresponding values of aggregate protection ratio that were used to calculate the feeder-link equivalent protection margins which appear in the alternative formula for overall equivalent protection margin given in § 1.12 are specified in Recommendation ITU-R BO.1297, as follows27, 28:

–	co-channel protection ratio  30 dB;

–	adjacent channel protection ratio  22 dB.     (WRC-2000)

However, it should be noted that the revision of the Regions 1 and 3 feeder-link Plan by WRC-97 was based on “simultaneous planning of feeder links and downlinks with calculation of overall equivalent protection margins” (as defined in § 1.11 of Annex 5 to Appendix 30 and in § 1.12) using the following values of aggregate protection ratio:

–	co-channel  23 dB;

–	adjacent channel  15 dB.     (WRC-03)

It was also specified that for the revision of the Regions 1 and 3 feeder-link Plan no overall co-channel single entry C/I ratio should be lower than 28 dB.     (WRC-20003)

Nevertheless, for assignments notified, which are in conformity with this Appendix, brought into use, and for which the date of bringing into use has been confirmed to the Bureau before 27 October 1997, the overall equivalent protection margins were calculated using a co-channel overall protection ratio of 30 dB and lower and upper overall adjacent channel protection ratios of 14 dB.

Revision of the Regions 1 and 3 feeder-link Plan at WRC-97 and planning at WRC-2000 were generally based on a set of reference parameters such as the average e.i.r.p., the reference earth station transmitting antenna, all test points placed within the –3 dB contour, a bandwidth of 27 MHz and the predetermined value of C/N. The Regions 1 and 3 feeder-link Plan as established by WRC-2000 is generally based on the use of digital modulation.     (WRC-2000)

WRC-2000 adopted for the protection of digital assignments from digital emissions the following protection ratio values to be applied for calculation of feeder-link equivalent protection margins of the WRC-2000 Regions 1 and 3 feeder-link Plan:

–	27 dB for co-channel signals;

–	22 dB for adjacent channel signals.     (WRC-2000)

During planning at WRC-2000, these values were used for all assignments of the Regions 1 and 3 feeder-link Plan and List, except those for which WRC-2000 adopted different values to be used in the planning process29.     (WRC-03)

Protection masks and associated calculation methods for interference into broadcasting-satellite systems involving digital emissions shall be in accordance with Recommendation ITU-R BO.1293-2 (Annexes 1 and 228bis).     (WRC-03)

ADD	COM6/140/35

_______________

28bis	Annex 3 of this Recommendation may be applied only in compatibility analysis for bilateral coordination between administrations.     (WRC-03)

## 3.7	Satellite receiving antenna

MOD 	COM6/140/36

### 3.7.1	Cross-section of receiving antenna beam

Note by the Secretariat: The text of this sub-section on pages 674-676 remains unmodified except description of the definition of  Curves B and B’ (on top of page 675) that should be read as shown below.

Cross-polar component (dBW):     (WRC-03)

MOD 	COM6/140/37

## 3.8	System noise temperature

The satellite system noise temperature values generally used in the Plan at the 1988 Conference (WARC Orb-88) are 1 800 K for 17 GHz and 1 500 K for 14 GHz32. For revising the Regions 1 and 3 Plan at WRC-97 these values are 900 K for 17 GHz and 750 K for 14 GHz. A value of 600 K was used for the 17 GHz band in the revision of the Regions 1 and 3 Plan at WRC-2000. WRC-2000 did not change the value for 14 GHz band.     (WRC-03)

# 4	Basic technical characteristics for Region 2

MOD 	COM6/140/38

## 4.1	Translation frequency and guard bands

The feeder-link Plan is based on the use of a single frequency translation of 5.1 GHz between the 17 GHz feeder-link channels and the 12 GHz downlink channels. Other values of the translation frequency may be used, provided that the corresponding channels have been assigned to the space station of the administration concerned.

With a single value frequency translation between the feeder-link frequency band (17.3-17.8 GHz) and the downlink frequency band (12.2-12.7 GHz), the guard bands present in the downlink Plan result in corresponding bandwidths of 12 MHz at the upper and lower feeder-link band edges. These feeder-link guard bands may be used to provide space operation functions in accordance with No. 1.23 in support of the operation of geostationary-satellite networks in the broadcasting-satellite service.     (WRC-03)

## 4.4	Transmitting antenna

MOD 	COM6/140/39

### 4.4.2	Transmitting antenna reference patterns

Note by the Secretariat: The text of this sub-section on page 684 remains unmodified. The definitions for Curve A (co-polar) and Curve B (cross-polar) appearing on pages 685 (after Figure 6) shall be modified as shown below.

Curve A:	co-polar component (dBi)

Gco = Gmax	for 	0° £  < 0.1°

Gco = 36 – 20 log 	for 	0.1° £  < 0.32°

Gco = 51.3 – 53.2 2	for 	0.32° £  < 0.54°

Gco = Max (29 – 25 log 	for 	0.54° £ £ 180°

If Gco > Gmax: Gco = Gmax

Curve B:	cross-polar component (dBi)

Gcross = Gmax – 30	for	0° £  < (0.6/D)°

Gcross = Max (9 – 20 log 	for 	(0.6/D)° £ £ 180°

If Gcross > Gmax – 30: Gcross = Gmax – 30

## 4.6	Receiving antenna

MOD 	COM6/140/40

### 4.6.3	Receiving antenna reference patterns

Note by the Secretariat: The text of this sub-section appearing on pages 686 and 687 remains unmodified. The definition for Curve A (co-polar) appearing on page 688 (after Figure 8) shall be modified as shown below.

Curve A:	co-polar component (dB relative to main beam gain)

– 12 (/0)2	for	0	  /0    0.5

– 33.33 02 (/0  –  x)2	for	0.5	 /0  

– 25.23		for	          /0    1.5

– (22    20 log (/0))	for			          /0    1.5

after intersection with Curve C, as Curve C.     (WRC-03)

_______________