| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Document 190-E |
|  |  |  | 17 June 2003 |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

| B2 | PLENARY MEETING |
| --- | --- |
| second SERIES OF TExts submitted by the editorial committee to the plenary meeting |  |
| The following texts are submitted to the Plenary Meeting for first reading: |  |

| Source | Document | Title |
| --- | --- | --- |
| COM 5 | 152 | ARTICLE 5 – band 17.3-17.7 GHz – Number 5.514 – band 148-149.9 MHz – Number 5.221 RESOLUTION 214 (Rev.WRC-2000) |
| COM 5 | 164 | ARTICLE 5 – band 14-14.25 GHz – band 14.25-14.3 GHz – band 14.3-14.4 GHz – band 14.4-14.47 GHz – band 14.47-14.5 GHz – Number 5.BB01 – Number 5.BB02 – Number 5.BB03 – Number 5.BB04 – band 31.8-32 GHz – band 32-32.3 GHz – band 32.3-33 GHz – Number 5.547C – Number 5.548 |
|  |  | ARTICLE 21 – Table 21-4 RESOLUTION 216 (Rev.WRC-2000) |
| COM 4 | 174 | ARTICLE 19 – Number 19.49 c) – Number 19.50.1 – Number 19.68 § 30 1) – Number 19.68AC01 1A) – Number 19.72 § 32 – Number 19.82AC02 § 35A |
|  |  | ARTICLE 23 – Number 23.11 C – Number 23.12 § 3 RESOLUTION 136 (Rev.WRC-03) RESOLUTION 348 (WRC-97) RESOLUTION 537 (WRC-97) RECOMMENDATION 515 (Rev.WRC-97) RECOMMENDATION 519 (WARC-92) |
| COM 6 | 183 | ARTICLE 5 – Number 5.491 – band 12.2-12.5 GHz |
| COM 6 | 189 | APPENDIX 30B (Rev.WRC-03) – Article 6 – Annex 1 – Annex 2 |

Annex: 18 pages

article  5

MOD	COM5/152/1	(B2/190/1)

15.63-18.6 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 17.3-17.7 FIXED-SATELLITE (Earth-to-space) 5.516 Radiolocation | 17.3-17.7 FIXED-SATELLITE (Earth-to-space) 5.516 BROADCASTING-SATELLITE Radiolocation | 17.3-17.7 FIXED-SATELLITE (Earth-to-space) 5.516 Radiolocation |
| MOD 5.514 | MOD 5.514 5.515 5.517 | MOD 5.514 |

MOD	COM5/152/2	(B2/190/2)

5.514	Additional allocation:  in Algeria, Germany, Angola, Saudi Arabia, Austria, Bahrain, Bangladesh, Bosnia and Herzegovina, Cameroon, Costa Rica, El Salvador, the United Arab Emirates, Finland, Guatemala, Honduras, India, Iran (Islamic Republic of), Iraq, Israel, Japan, Jordan, Kuwait, Libyan Arab Jamahiriya, Nepal, Nicaragua, Oman, Uzbekistan, Pakistan, Qatar, Serbia and Montenegro, Slovenia and Sudan, the band 17.3-17.7 GHz is also allocated to the fixed and mobile services on a secondary basis. The power limits given in Nos. 21.3 and 21.5 shall apply.     (WRC-03)

MOD	COM5/152/3	(B2/190/3)

148-223 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 148-149.9 FIXED MOBILE except aeronautical mobile (R) MOBILE-SATELLITE (Earth-to-space) 5.209 | 148-149.9 FIXED MOBILE MOBILE-SATELLITE (Earth-to-space) 5.209 |  |
| 5.218 5.219 MOD 5.221 | 5.218 5.219 MOD 5.221 |  |

MOD	COM5/152/4	(B2/190/4)

5.221	Stations of the mobile-satellite service in the band 148-149.9 MHz shall not cause harmful interference to, or claim protection from, stations of the fixed or mobile services operating in accordance with the Table of Frequency Allocations in the following countries: Albania, Algeria, Germany, Saudi Arabia, Australia, Austria, Bahrain, Bangladesh, Barbados, Belarus, Belgium, Benin, Bosnia and Herzegovina, Botswana, Brunei Darussalam, Bulgaria, Cameroon, China, Cyprus, Congo, Korea (Rep. of), Côte d’Ivoire, Croatia, Cuba, Denmark, Egypt, the United Arab Emirates, Eritrea, Spain, Estonia, Ethiopia, Finland, France, Gabon, Ghana, Greece, Guinea, Guinea Bissau, Hungary, India, Iran (Islamic Republic of), Ireland, Iceland, Israel, Italy, Jamaica, Japan, Jordan, Kazakstan, Kenya, Kuwait, Latvia, The Former Yugoslav Republic of Macedonia, Lebanon, Libyan Arab Jamahiriya, Liechtenstein, Lithuania, Luxembourg, Malaysia, Mali, Malta, Mauritania, Moldova, Mongolia, Mozambique, Namibia, Norway, New Zealand, Oman, Uganda, Uzbekistan, Pakistan, Panama, Papua New Guinea, Paraguay, the Netherlands, the Philippines, Poland, Portugal, Qatar, Syrian Arab Republic, Kyrgyzstan, Slovakia, Romania, the United Kingdom, the Russian Federation, Senegal, Serbia and Montenegro, Sierra Leone, Singapore, Slovenia, Sri Lanka, South Africa, Sweden, Switzerland, Swaziland, Tanzania, Chad, Thailand, Togo, Tonga, Trinidad and Tobago, Tunisia, Turkey, Ukraine, Viet Nam, Yemen, Zambia, and Zimbabwe.     (WRC-03)

SUP	COM5/152/5	(B2/190/5)

RESOLUTION  214  (Rev.WRC-2000)

Sharing studies relating to consideration of the allocation of bands
below 1 GHz to the non-geostationary mobile-satellite service

article 5

MOD	COM5/164/1	(B2/190/6)

11.7-14.25 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 14-14.25 FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 RADIONAVIGATION 5.504 Mobile-satellite (Earth-to-space) ADD 5.BB02 Space research 5.505 |  |  |

MOD	COM5/164/2	(B2/190/7)

14.25-15.63 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 14.25-14.3 FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 RADIONAVIGATION 5.504 Mobile-satellite (Earth-to-space) ADD 5.BB03 Space research 5.505 5.508 5.509 |  |  |

| 14.3-14.4 FIXED FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) ADD 5.BB04 Radionavigation-satellite | 14.3-14.4 FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 Mobile-satellite (Earth-to-space) Radionavigation-satellite | 14.3-14.4 FIXED FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) ADD 5.BB04 Radionavigation-satellite |
| --- | --- | --- |
| 14.4-14.47 FIXED FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) ADD 5.BB04 Space research (space-to-Earth) |  |  |
| 14.47-14.5 FIXED FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) ADD 5.BB01 ADD 5.BB04 Radio astronomy 5.149 |  |  |

ADD	COM5/164/3	(B2/190/8)

5.BB01	Aircraft earth stations operating in the aeronautical mobile-satellite service in the band 14-14.5 GHz shall comply with the provisions of Annex 1, Part C of Recommendation ITU-R M.1643, with respect to any radio astronomy station performing observations in the 14.47-14.5 GHz band located on the territory of Spain, France, India, Italy, the United Kingdom and South Africa.

5.BB02	In the band 14-14.25 GHz, the power flux-density produced on the territory of the countries of Saudi Arabia, Botswana, Côte d’Ivoire, Egypt, Guinea, India, Iran, Lesotho, Nigeria, Oman, Syrian Arab Republic and Tunisia by any aircraft earth station in the aeronautical mobile-satellite service shall not exceed the limits given in Annex 1, Part B of Recommendation ITU-R M.1643, unless otherwise specifically agreed by the affected administration(s). The provisions of this footnote in no way derogate the obligations of the aeronautical mobile-satellite service to operate as a secondary service in accordance with No. 5.29.

5.BB03	In the band 14.25-14.3 GHz, the power flux-density produced on the territory of the countries of Saudi Arabia, Botswana, China, Côte d’Ivoire, Egypt, France, Guinea, India, Iran, Italy, Lesotho, Nigeria, Oman, Syrian Arab Republic, the United Kingdom and Tunisia by any aircraft earth station in the aeronautical mobile-satellite service shall not exceed the limits given in Annex 1, Part B of Recommendation ITU-R M.1643, unless otherwise specifically agreed by the affected administration(s). The provisions of this footnote in no way derogate the obligations of the aeronautical mobile-satellite service to operate as a secondary service in accordance with No. 5.29.

5.BB04	In the band 14.3-14.5 GHz, the power flux-density produced on the territory of the countries of Saudi Arabia, Botswana, Cameroon, China, Côte d’Ivoire, Egypt, France, Gabon, Guinea, India, Iran, Italy, Lesotho, Nigeria, Oman, Syrian Arab Republic, the United Kingdom, Sri Lanka, Tunisia and Viet Nam by any aircraft earth station in the aeronautical mobile-satellite service shall not exceed the limits given in Annex 1, Part B of Recommendation ITU-R M.1643, unless otherwise specifically agreed by the affected administration(s). The provisions of this footnote in no way derogate the obligations of the aeronautical mobile-satellite service to operate as a secondary service in accordance with No. 5.29.

SUP	COM5/164/4	(B2/190/9)

RESOLUTION  216  (Rev.WRC-2000)

Possible broadening of the secondary allocation to the mobile-satellite
service (Earth-to-space) in the band 14-14.5 GHz to
cover aeronautical applications

ARTICLE  5

MOD	COM5/164/5	(B2/190/10)

29.9-34.2 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 31.8-32 FIXED 5.547A RADIONAVIGATION SPACE RESEARCH (deep space) (space-to-Earth) 5.547 5.547B MOD 5.548 |  |  |

MOD	COM5/164/6	(B2/190/11)

32-32.3 FIXED 5.547A RADIONAVIGATION SPACE RESEARCH (deep space) (space-to-Earth) 5.547 MOD 5.547C MOD 5.548

MOD	COM5/164/7	(B2/190/12)

32.3-33 FIXED 5.547A INTER-SATELLITE RADIONAVIGATION 5.547 5.547D MOD 5.548

MOD	COM5/164/8	(B2/190/13)

5.547C	Alternative allocation:  in the United States, the band 32-32.3 GHz is allocated to the radionavigation and space research (deep space) (space-to-Earth) services on a primary basis.     (WRC-03)

MOD	COM5/164/9	(B2/190/14)

5.548	In designing systems for the inter-satellite service in the band 32.3-33 GHz, for the radionavigation service in the band 32-33 GHz, and for the space research service (deep space) in the band 31.8-32.3 GHz, administrations shall take all necessary measures to prevent harmful interference between these services, bearing in mind the safety aspects of the radionavigation service (see Recommendation 707).     (WRC-03)

ARTICLE  21

MOD	COM5/164/10	(B2/190/15)

TABLE  21-4 (continued)

| Frequency band | Service* | Limit in dB(W/m2) for angle of arrival () above the horizontal plane |  |  | Reference bandwidth |
| --- | --- | --- | --- | --- | --- |
|  |  | 0°-5° | 5°-25° | 25°-90° |  |
| ... |  |  |  |  |  |
| 32.3-33 GHz | Inter-satellite | –135 | –135 + ( – 5) | –115 | 1 MHz |
| ... |  |  |  |  |  |

ARTICLE  19

Identification of stations

Section III  –  Formation of call signs

SUP	COM4/174/1	(B2/190/16)

19.49	c)

MOD	COM4/174/2	(B2/190/17)

_______________

4 	19.50.1 	For call sign series beginning with B, F, G, I, K, M, N, R, W and 2, only the first character is required for nationality identification. In the cases of half series (i.e. when the first two characters are allocated to more than one Member State), the first three characters are required for nationality identification.

MOD	COM4/174/3	(B2/190/18)

19.68	§ 30	1)

–	one character (provided that it is the letter B, F, G, I, K, M, N, R or W) and a single digit (other than 0 or 1), followed by a group of not more than four characters, the last of which shall be a letter, or

–	two characters and a single digit (other than 0 or 1), followed by a group of not more than four characters, the last of which shall be a letter.

ADD	COM4/174/4	(B2/190/19)

19.68AC01		1A)	On special occasions, for temporary use, administrations may authorize use of call signs with more than the four characters referred to in No. 19.68.

Section IV  –  Identification of stations using radiotelephony

MOD	COM4/174/5	(B2/190/20)

19.72	§ 32	Stations using radiotelephony shall be identified as indicated in Nos. 19.73 to 19.82AC02.

ADD	COM4/174/6	(B2/190/21)

19.82AC02	§ 35A	Amateur stations and experimental stations

–	a call sign (see No. 19.68).

ARTICLE  23

MOD	COM4/174/7	(B2/190/22)

23.11	C  –  HF bands allocated to the broadcasting service except the bands 
	as referred to in No. 23.6.

MOD	COM4/174/8	(B2/190/23)

23.12	§ 3	Transmitting stations of the broadcasting service operating in the HF bands allocated to the broadcasting service shall meet the system specifications contained in Appendix 11.

MOD	COM4/174/9	(B2/190/24)

RESOLUTION  136  (Rev.WRC-03)

Frequency sharing in the range 37.5-50.2 GHz between geostationary
 fixed-satellite service networks and non-geostationary 
fixed-satellite service systems

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that WRC-2000 made provisions for the operation of geostationary fixed-satellite service (GSO FSS) networks and non-GSO FSS systems in the 10-30 GHz frequency range;

b)	that there is an emerging interest in operating GSO FSS networks and non-GSO FSS systems in the 37.5-50.2 GHz frequency range;

c)	that there is a need to provide for the orderly development and implementation of new satellite technologies in the 37.5-50.2 GHz frequency range;

d)	that systems based on the use of new technologies associated with both GSO FSS networks and non-GSO FSS systems are capable of providing the most isolated regions of the world with high-capacity and low-cost means of communication;

e)	that there should be equitable access to the radio-frequency spectrum and orbital resources in a mutually acceptable manner that allows for new entrants in the provision of services;

f)	that the Radio Regulations should be sufficiently flexible to accommodate the introduction and implementation of innovative technologies as they evolve;

g)	that -in the bands 37.5-50.2 GHz where there has been little or no deployment of satellite systems to date, both GSO FSS and non-GSO FSS operators should be expected to exhibit flexibility in achieving the appropriate balance in the sharing environment;

h)	that this Conference, having considered the outcome of the ITU-R studies on this subject, as summarized in the CPM Report to this Conference, has decided that further studies are needed before the conditions for non-GSO FSS systems to share these bands with GSO FSS networks can reliably be determined,

resolves to urge administrations

to seek balanced sharing arrangements between GSO FSS networks and non-GSO FSS systems in the application of Article 22 to such systems in the 37.5-50.2 GHz frequency range, prior to the review by [WRC-07/a future competent conference] of the results of the studies called for by this Resolution,

invites ITU-R

1	to undertake, as a matter of urgency, further technical, operational and regulatory studies on sharing arrangements which achieve an appropriate balance between GSO FSS networks and non-GSO FSS systems in the frequency range 37.5-50.2 GHz. Such further studies should include, but not necessarily be limited to:

a)	techniques which individually or in combination avoid, or otherwise adequately mitigate, main beam-to-main beam coupling of interference in both directions between non-GSO FSS systems and GSO FSS networks at “in-line” instants. The studies should be based on the key parameters of systems firmly planned to operate in the bands concerned, and should be pursued sufficiently far to establish appropriate long-term and short-term interference criteria and to compute the time statistics of interference from

non-GSO systems to GSO networks, and from GSO networks to non-GSO systems, to determine whether those criteria would be met. The computations and comparisons should be made firstly assuming no mitigation, and subsequently with each of the various mitigation techniques or combinations of mitigation techniques envisaged. The mitigation techniques thus investigated should include:

–	satellite diversity or arc avoidance;

–	geographical isolation between earth stations;

–	site diversity;

–	adaptive coding;

–	link balancing;

–	other appropriate techniques, if any;

b)	the development of technical, operational and regulatory guidance which would enable [WRC-07/a future competent conference] to decide whether or not to include, in the Radio Regulations, epfd limits on non-GSO FSS systems for the protection of GSO FSS networks, and off-axis e.i.r.p. density limits on earth stations in GSO FSS networks for the protection of non-GSO FSS systems, in the frequency range 37.5-50.2 GHz. Such guidance should include quantitative values for suitable epfd, epfd and off-axis e.i.r.p. density limits;

2	to report the results of these studies to [WRC-07/a future competent conference].

SUP	COM4/174/10	(B2/190/25)

RESOLUTION  348  (WRC-97)

Studies required to provide priority to distress communications originated
by shore-based search and rescue authorities

SUP	COM4/174/11	(B2/190/26)

RESOLUTION  537  (WRC-97)

Survey of HF broadcasting transmitter and receiver statistics
as called for in Resolution 517 (Rev.WRC-97)

SUP	COM4/174/12	(B2/190/27)

RECOMMENDATION  515  (Rev.WRC-97)

Introduction of high-frequency broadcasting transmitters and receivers
capable of operation with spectrum-efficient modulation techniques

SUP	COM4/174/13	(B2/190/28)

RECOMMENDATION  519  (WARC-92)

Introduction of single-sideband (SSB) emissions and possible advancement
of the date for cessation of the use of double-sideband (DSB) emissions in
the HF bands allocated to the broadcasting service

ARTICLE  5

Frequency allocations

SUP	COM6/183/1	(B2/190/29)

5.491

MOD	COM6/183/2	(B2/190/30)

11.7-14.25 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 11.7-12.5 FIXED BROADCASTING BROADCASTING-SATELLITE MOBILE except aeronautical mobile | 11.7-12.1 FIXED 5.486 FIXED-SATELLITE (space-to-Earth) 5.484A Mobile except aeronautical mobile 5.485 5.488 | 11.7-12.2 FIXED MOBILE except aeronautical mobile BROADCASTING BROADCASTING-SATELLITE |
|  | 12.1-12.2 FIXED-SATELLITE (space-to-Earth) 5.484A |  |
|  | 5.485 5.488 5.489 | 5.487 5.487A 5.492 |
|  | 12.2-12.7 FIXED MOBILE except aeronautical mobile BROADCASTING BROADCASTING-SATELLITE | 12.2-12.5 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile BROADCASTING |
| 5.487 5.487A 5.492 |  | 5.484A 5.487 SUP |
| 12.5-12.75 | 5.487A 5.488 5.490 5.492 | 12.5-12.75 |
| FIXED-SATELLITE (space-to-Earth) 5.484A (Earth-to-space) 5.494 5.495 5.496 | 12.7-12.75 FIXED FIXED-SATELLITE (Earth-to-space) MOBILE except aeronautical mobile | FIXED FIXED-SATELLITE (space-to-Earth) 5.484A MOBILE except aeronautical mobile BROADCASTING- SATELLITE 5.493 |

APPENDIX  30B  (Rev.WRC-03)

Provisions and associated Plan for the fixed-satellite service
in the frequency bands 4 500-4 800 MHz, 6 725-7 025 MHz,
10.70-10.95 GHz, 11.20-11.45 GHz and 12.75-13.25 GHz

ARTICLE  6

Procedures for implementation of the Plan and regulation of 
the fixed-satellite service in the planned bands1     (WRC-2000)

ADD	COM6/189/1	(B2/190/31)

6.43bis	In the case where multiple consecutive complete notices belonging to the same notifying administration have to be examined under § 6.43 without any notice from other administrations having been received in the meantime, the Bureau shall implement, whenever feasible, the following measures, where applicable, in order to accelerate, to the maximum extent possible, the processing of the notices:

–	simultaneous processing of the information relating to the 6/4 and 13/11-10 GHz frequency bands in the same network having the same or different date of receipt;

–	sequential examination of networks having the same or different date of receipt. The Bureau’s finding for all these networks shall be given at the same time and the publication of all related special sections under § 6.49 shall be included in a single set of publications with one single deadline for comments and published in the same BR IFIC.    (WRC-03)

ADD	COM6/189/2	(B2/190/32)

6.56bis	In the case where multiple consecutive complete notices belonging to the same notifying administration have to be examined under § 6.56 without any notice from other administrations having been received in the meantime, the Bureau shall implement, whenever feasible, the following measures, where applicable, in order to accelerate, to the maximum extent possible, the processing of the notices:

–	simultaneous processing of the information relating to the 6/4 and 13/11-10 GHz frequency bands in the same network having the same or different date of receipt;

–	sequential examination of networks having the same or different date of receipt. The Bureau’s finding for all these networks shall be given at the same time.    (WRC-03)

MOD	COM6/189/3	(B2/190/33)

ANNEX  1    (WRC-03)

Parameters used in characterizing the fixed-satellite service Plan

SUP	COM6/189/4	(B2/190/34)

Section A  –  Technical data used in establishing the Allotment Plan 
and the associated provisions

# 1	Basic technical characteristics

MOD	COM6/189/5	(B2/190/35)

## 1.7	Space station characteristics

Note by the Secretariat: The text of this sub-section appearing on page 722 remains unmodified. The text on page 723 including Figure 2 (on this page) shall be modified as shown below.

Curve A:	dB relative to main beam gain

– 12 (/0)2		for  0    (/0)    0.5

– 12 (ϕϕ0)–xBminϕ02	for  0.5    (/0)    1.45 Bminϕ0 + x

– 25.23		for  1.45 Bminϕ0 + x < (ϕϕ0) ≤ 1.45

– (22    20 log (/0))	for  (0)    1.45

after intersection with Curve B: Curve B.

Curve B:	Minus the on-axis gain (Curve B represents examples of four antennas having different values of 0 as labelled in Fig. 2. The on-axis gains of these antennas are approximately 28.3, 34.3, 40.4 and 46.4 dBi, respectively)

where:

 :	off-axis angle (degrees)

0 :	cross-sectional half-power beamwidth in the direction of interest (degrees)

x = 0.51 − Bminϕ0

where:

Bmin=0.8° for 1310−11 GHz1.6° for 64 GHz

SUP	COM6/189/6	(B2/190/36)

ANNEX  2

Basic data to be furnished in notices relating to stations
in the fixed-satellite service entering the design stage
using frequency bands of the Plan

ADD	COM6/189/7	(B2/190/37)

ANNEX  2

Basic data to be furnished in notices relating to stations
in the fixed-satellite service entering the design stage
using frequency bands of the Plan    (WRC-03)

These data are listed in Appendix 4.    (WRC-03)

_______________