| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Document 225-E |
|  |  |  | 20 June 2003 |
|  |  |  | Original: English |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | COMMITTEE 8 |
| --- | --- |
| FIFTH SERIES OF TEXTS SUBMITTED BY COMMITTEE 5 TO THE EDITORIAL COMMITTEE |  |

Committee 5 considered agenda items 1.5, 1.12, 1.17 and 1.32.

The attached text is submitted for your consideration with a view to its subsequent submission to the Plenary.

Note that Committee 5 may further consider proposed modifications to Article 5, in the 5 250-5 350 MHz band.

A. JAMIESON
	Chairman, Committee 5, Box 233

ARTICLE  5

MOD	COM5/225/1

2 700-4 800 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 2 900-3 100 RADIONAVIGATION 5.426 RADIOLOCATION ADD 5.BA01 5.425 5.427 |  |  |

ADD	COM5/225/2

5.BA01	In the band 2 900-3 100 MHz, stations in the radiolocation service shall not cause harmful interference to, nor claim protection from, radar systems in the radionavigation service.

MOD	COM5/225/3

4 800-5 830 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 5 250-5 255 EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION SPACE RESEARCH 5.447D 5.448 5.448A ADD 5.BD01 |  |  |
| 5 255- 5 350 EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION SPACE RESEARCH (active) 5.448 5.448A ADD 5.BD01 |  |  |

ADD	COM5/225/4

5.BD01	Additional allocation: The band 5 250-5 350 MHz is also allocated to the fixed service on a primary basis in the following countries in Region 3: Australia, India, Indonesia, Iran (Islamic Republic of), Japan, Malaysia, Papua New Guinea, Philippines, Thailand and Viet Nam. The use of this band by the fixed service is intended for the implementation of fixed wireless access (FWA) systems and shall comply with Recommendation ITU-R F.1613. In addition, the fixed service shall not claim protection from the radiodetermination, Earth exploration-satellite (active) and space research (active) services, although the provisions of No. 5.43A do not apply to the fixed service with respect to the Earth exploration-satellite (active) and space research (active) services. After implementation of FWA systems in the fixed service with protection for the existing radiodetermination systems, no more stringent constraints should be imposed on the FWA systems by future radiodetermination implementations.

NOC	COM5/225/5

14.25-15.63 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 14.8-15.35 FIXED MOBILE Space research 5.339 |  |  |

SUP	COM5/225/6

5.551AA

MOD	COM5/225/7

34.2-40 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 37.5-38 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE SPACE RESEARCH (space-to-Earth) Earth exploration-satellite (space-to-Earth) 5.547 |  |  |
| 38-39.5 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE Earth exploration-satellite (space-to-Earth) 5.547 |  |  |
| 39.5-40 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE MOBILE-SATELLITE (space-to-Earth) Earth exploration-satellite (space-to-Earth) 5.547 |  |  |

MOD	COM5/225/8

40.5-51.4 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 41-42.5 FIXED FIXED-SATELLITE (space-to-Earth) BROADCASTING BROADCASTING-SATELLITE Mobile 5.547 5.551F 5.551G |  |  |

ARTICLE  21

Terrestrial and space services sharing frequency bands above 1 GHz

MOD	COM5/225/9

TABLE  21-4 (continued)

| Frequency band | Service* | Limit in dB(W/m2) for angle of arrival () above the horizontal plane |  |  |  | Reference bandwidth |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 0°-5° | 5°-25° |  | 25°-90° |  |
| ... |  |  |  |  |  |  |
| 37.5-40 GHz | Fixed-satellite (non-geostationary-satellite orbit) Mobile-satellite (non-geostationary-satellite orbit) | –120 10, 19 | –120 + 0.75( – 5) 10, 19 |  | –105 10, 19 | 1 MHz |
| 37.5-40 GHz | Fixed-satellite (geostationary-satellite orbit) Mobile-satellite (geostationary-satellite orbit) | –127 19 | 5°-20° | 20°-25° | –105 19 | 1 MHz |
|  |  |  | –127 + (4/3) ( – 5) 19 | –107 + 0.4 ( – 20) 19 |  |  |
| 40-40.5 GHz | Fixed-satellite | –115 | –115 + 0.5( – 5) |  | –105 | 1 MHz |
| 40.5-42 GHz | Fixed-satellite (non-geostationary-satellite orbit) Broadcasting-satellite (non-geostationary-satellite orbit) | –115 10, 19 | –115 + 0.5( – 5) 10, 19 |  | –105 10, 19 | 1 MHz |
| 40.5-42 GHz | Fixed-satellite (geostationary-satellite orbit) Broadcasting-satellite (geostationary-satellite orbit) | –120 19 | 5°-15° | 15°-25° | –105 19 | 1 MHz |
|  |  |  | –120 + ( – 5) 19 | –110 + 0.5 ( – 15) 19 |  |  |

MOD

TABLE  21-4 (end)

| SUP COM5/225/10 Frequency band | Service* | Limit in dB(W/m2) for angle of arrival () above the horizontal plane |  |  |  | Reference bandwidth |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 0°-5° | 5°-25° |  | 25°-90° |  |
| 42-42.5 GHz | Fixed-satellite (non-geostationary-satellite orbit) Broadcasting-satellite (non-geostationary-satellite orbit) | –120 10, 19 | –120 + 0.75( – 5) 10, 19 |  | –105 10, 19 | 1 MHz |
| 42-42.5 GHz | Fixed-satellite (geostationary-satellite orbit) Broadcasting-satellite (geostationary-satellite orbit) | –127 19 | 5°-20° | 20°-25° | –105 19 | 1 MHz |
|  |  |  | –127 + (4/3) ( – 5) 19 | –107 + 0.4 ( – 20) 19 |  |  |

_______________

16	21.16.11

SUP	COM5/225/11

_______________

17	21.16.12

SUP	COM5/225/12

_______________

18	21.16.13

ADD	COM5/225/13

_______________

19	21.16.14	When addressing the sharing conditions between the fixed service and the fixed-satellite service in the bands 37.5-40 GHz and 40.5-42.5 GHz, the power flux-density at the Earth’s surface from any FSS satellite should be no greater than the level(s) required to meet the FSS link availability and performance objectives of the subject applications, taking into account the technical and operational requirements of the overall satellite network design. In any case, the levels shall not exceed the applicable power flux-density limits in Table 21-4.

SUP	COM5/225/14

RESOLUTION  84  (WRC-2000)

Power flux-density limits in the bands 37.5-42.5 GHz for the fixed-satellite service, broadcasting-satellite service and mobile-satellite service

____________