| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Document 228-E |
|  |  |  | 20 June 2003 |
|  |  |  | Original: English |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

| Source: Documents 202, 219 and 220 | COMMITTEE 8 |
| --- | --- |
| FIFTH SERIES OF TEXTS SUBMITTED BY COMMITTEE 4 TO THE EDITORIAL COMMITTEE |  |

1.	The following texts are submitted to Committee 8:

–	Article 15

–	Resolution 78 (WRC-2000)

–	Article 22

–	Article 52, No. 52.221A

–	Draft Resolution [COM4/3] (WRC-03)

–	Appendix 17

–	Draft Resolution [COM4/2] (WRC-03)

–	Resolution 207 (Rev.WRC-03)

–	Appendix 11

–	Recommendation 517 (Rev.WRC-03)

–	Resolution 29 (WRC-97)

–	Resolution 21 (Rev.WRC-03)

2.	Syria indicated that it will make a statement in Plenary with regard to recommends in MOD Recommendation 517 (Rev.WRC-03).

E. GEORGE
	Chairman, Committee 4, Box 123

NOC	COM4/228/1

ARTICLE  15

Interferences

SUP	COM4/228/2

RESOLUTION  78  (WRC-2000)

Development of procedures in case the operational or additional
operational limits in Article 22 are exceeded

ARTICLE  22

Space services1

NOC	COM4/228/3

22.1 to 22.5H

NOC	COM4/228/4

TABLE  22-4A     (WRC-2000)

Operational limits to the epfd¯ radiated by non-geostationary-satellite
systems in the fixed-satellite service in certain frequency bands20, 21, 22

NOC	COM4/228/5

TABLE  22-4A1     (WRC-2000)

Additional operational limits to the epfd radiated by non-geostationary-satellite systems in the fixed-satellite
service into 3 m and 10 m geostationary fixed-satellite service earth station antennas

NOC	COM4/228/6

TABLE  22-4B     (WRC-2000)

Operational limits to the epfd¯ radiated by non-geostationary-satellite
systems in the fixed-satellite service in certain frequency bands20, 24

NOC	COM4/228/7

TABLE  22-4C     (WRC-2000)

Operational limits to the epfd¯ radiated by non-geostationary-satellite systems
in the fixed-satellite service in certain frequency bands25

NOC	COM4/228/8

22.5J to 22.39

ARTICLE  52

Special rules relating to the use of frequencies

MOD	COM4/228/9

52.221A		2)	Calling on the carrier frequencies 12 290 kHz and 16 420 kHz shall be permitted only to and from rescue coordination centres (see No. 30.6.1), subject to the safeguards of Resolution [COM4/3] (WRC-03). The alternative carrier frequencies 12 359 kHz and 16 537 kHz may be used by ship stations and coast stations for calling on a simplex basis, provided that the peak envelope power does not exceed 1 kW.     (WRC-03)

ADD	COM4/228/10

DRAFT  Resolution  [COM4/3]  (WRC-03)

Use of the carrier frequencies 12 290 kHz and 16 420 kHz for limited 
safety-related calling to and from rescue coordination centres

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that this Conference modified No. 52.221A to allow limited safety-related calling on the carrier frequencies 12 290 kHz and 16 420 kHz;

b)	that the limited safety-related calling function on these carrier frequencies will enhance the capability of those search and rescue organizations which maintain watch on these distress and safety frequencies to call ships not fitted for the GMDSS,

noting

a)	that regulation IV/4.8 of the International Convention for the Safety of Life at Sea (SOLAS), 1974, as amended, requires that SOLAS ships, while at sea, be capable of transmitting and receiving general radiocommunications to and from shore-based radio systems or networks;

b)	that general communications may consist of safety-related communications necessary for the safe operation of shipping,

further noting

that safety-related communications require adequate, effective and immediate access and protection,

recognizing

a)	that the International Maritime Organization notes that distress, urgency and safety radiocommunications include, but are not limited to:

•	transmissions of maritime safety information;

•	distress calls and traffic;

•	acknowledgment and relaying of distress calls;

•	search and rescue coordination communications;

•	ship movement service communications;

•	communications related to the safe operation of ships;

•	communications related to navigation;

•	meteorological warnings;

•	meteorological observations;

•	ship position reports; and

•	medical emergencies (e.g. MEDICO/MEDIVAC);

b)	that distress, urgency and safety communications are defined in Articles 32 and 33 of the Radio Regulations,

resolves

1	that the carrier frequencies 12 290 kHz and 16 420 kHz be used only for distress, urgency and safety communications, and limited safety-related calling;

2	that safety-related calling be initiated only after determination that other communications are not present on these frequencies;

3	that safety-related calling be minimized and not cause interference to distress, urgency and safety communications,

urges administrations

to encourage the coast and ship stations under their jurisdiction to use digital selective calling techniques,

instructs the Secretary-General

to bring this Resolution to the attention of the International Maritime Organization.

MOD	COM4/228/11

APPENDIX  17  (Rev.WRC-03)

Frequencies and channelling arrangements in the
high-frequency bands for the maritime mobile service

(See Article 52)

PART  A  –  Table of subdivided bands

In the Table, where appropriate1, the assignable frequencies in a given band for each usage are:

–	indicated by the lowest and highest frequency, in heavy type, assigned in that band;

–	regularly spaced, the number of assignable frequencies (f.) and the spacing in kHz being indicated in italics.

Table of frequencies (kHz) to be used in the band between 4 000 kHz and 27 500 kHz
allocated exclusively to the maritime mobile service

| Band (MHz) | 4 | 6 | 8 | 12 | 16 | 18/19 | 22 | 25/26 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Limits (kHz) | 4 063 | 6 200 | 8 195 | 12 230 | 16 360 | 18 780 | 22 000 | 25 070 |
| Frequencies assignable to ship stations for oceanographic data transmission c) | 4 063.3 to 4 064.8 6 f. 0.3 kHz |  |  |  |  |  |  |  |
| Limits (kHz) | 4 065 | 6 200 | 8 195 | 12 230 | 16 360 | 18 780 | 22 000 | 25 070 |
| Frequencies assignable to ship stations for telephony, duplex operation a) i) | 4 066.4 to 4 144.4 27 f. 3 kHz | 6 201.4 to 6 222.4 8 f. 3 kHz | 8 196.4 to 8 292.4 33 f. 3 kHz | 12 231.4 to 12 351.4 41 f. 3 kHz | 16 361.4 to 16 526.4 56 f. 3 kHz | 18 781.4 to 18 823.4 15 f. 3 kHz | 22 001.4 to 22 157.4 53 f. 3 kHz | 25 071.4 to 25 098.4 10 f. 3 kHz |
| Limits (kHz) | 4 146 | 6 224 | 8 294 | 12 353 | 16 528 | 18 825 | 22 159 | 25 100 |

Table of frequencies (kHz) to be used in the band between 4 000 kHz and 27 500 kHz
allocated exclusively to the maritime mobile service (continued)

| Band (MHz) | 4 | 6 | 8 | 12 | 16 | 18/19 | 22 | 25/26 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Limits (kHz) | 4 146 | 6 224 | 8 294 | 12 353 | 16 528 | 18 825 | 22 159 | 25 100 |
| Frequencies assignable to ship stations and coast stations for telephony, simplex operation a) | 4 147.4 to 4 150.4 2 f. 3 kHz | 6 225.4 to 6 231.4 3 f. 3 kHz | 8 295.4 to 8 298.4 2 f. 3 kHz | 12 354.4 to 12 366.4 5 f. 3 kHz | 16 529.4 to 16 547.4 7 f. 3 kHz | 18 826.4 to 18 844.4 7 f. 3 kHz | 22 160.4 to 22 178.4 7 f. 3 kHz | 25 101.4 to 25 119.4 7 f. 3 kHz |
| Limits (kHz) | 4 152 | 6 233 | 8 300 | 12 368 | 16 549 | 18 846 | 22 180 | 25 121 |
| Frequencies assignable to ship stations for wide-band telegraphy, facsimile and special transmission systems | 4 154 to 4 170 5 f. 4 kHz | 6 235 to 6 259 7 f. 4 kHz | 8 302 to 8 338 10 f. 4 kHz | 12 370 to 12 418 13 f. 4 kHz | 16 551 to 16 615 17 f. 4 kHz | 18 848 to 18 868 6 f. 4 kHz | 22 182 to 22 238 15 f. 4 kHz | 25 123 to 25 159 10 f. 4 kHz |
| Limits (kHz) | 4 172 | 6 261 | 8 340 | 12 420 | 16 617 | 18 870 | 22 240 | 25 161.25 |
| Frequencies assignable to ship stations for oceanographic data transmission c) |  | 6 261.3 to 6 262.5 5 f. 0.3 kHz | 8 340.3 to 8 341.5 5 f. 0.3 kHz | 12 420.3 to 12 421.5 5 f. 0.3 kHz | 16 617.3 to 16 618.5 5 f. 0.3 kHz |  | 22 240.3 to 22 241.5 5 f. 0.3 kHz |  |
| Limits (kHz) | 4 172 | 6 262.75 | 8 341.75 | 12 421.75 | 16 618.75 | 18 870 | 22 241.75 | 25 161.25 |
| Frequencies (paired) assignable to ship stations for narrow-band direct-printing (NBDP) telegraphy and data transmission systems at speeds not exceeding 100 Bd for FSK and 200 Bd for PSK d) j) m) p) | 4 172.5 to 4 181.5 18 f. 0.5 kHz | 6 263 to 6 275.5 25 f. 0.5 kHz |  |  |  |  |  |  |
| Limits (kHz) | 4 181.75 | 6 275.75 | 8 341.75 | 12 421.75 | 16 618.75 | 18 870 | 22 241.75 | 25 161.25 |
| Calling frequencies assignable to ship stations for A1A or A1B Morse telegraphy g) p) |  |  |  |  |  |  |  |  |
| Limits (kHz) | 4 186.75 | 6 280.75 | 8 341.75 | 12 421.75 | 16 618.75 | 18 870 | 22 241.75 | 25 161.25 |
| Frequencies (paired) assignable to ship stations for NBDP telegraphy and data transmission systems at speeds not exceeding 100 Bd for FSK and 200 Bd for PSK d) m) p) |  | 6 281 to 6 284.5 8 f. 0.5 kHz |  |  |  |  |  |  |
| Limits (kHz) | 4 186.75 | 6 284.75 | 8 341.75 | 12 421.75 | 16 618.75 | 18 870 | 22 241.75 | 25 161.25 |

Table of frequencies (kHz) to be used in the band between 4 000 kHz and 27 500 kHz
allocated exclusively to the maritime mobile service (continued)

| Band (MHz) | 4 | 6 | 8 | 12 | 16 | 18/19 | 22 | 25/26 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Limits (kHz) | 4 186.75 | 6 284.75 | 8 341.75 | 12 421.75 | 16 618.75 | 18 870 | 22 241.75 | 25 161.25 |
| Working frequencies assignable to ship stations for A1A or A1B Morse telegraphy e) f) h) p) | 4 187 to 4 202 31 f. 0.5 kHz | 6 285 to 6 300 31 f. 0.5 kHz | 8 342 to 8 365.5 48 f. 0.5 kHz | 12 422 to 12 476.5 110 f. 0.5 kHz | 16 619 to 16 683 129 f. 0.5 kHz |  | 22 242 to 22 279 75 f. 0.5 kHz | 25 161.5 to 25 171 20 f. 0.5 kHz |
| Limits (kHz) | 4 202.25 | 6 300.25 | 8 365.75 | 12 476.75 | 16 683.25 | 18 870 | 22 279.25 | 25 171.25 |
| Calling frequencies assignable to ship stations for A1A or A1B Morse telegraphy g) p) |  |  |  |  |  |  |  |  |
| Limits (kHz) | 4 202.25 | 6 300.25 | 8 370.75 | 12 476.75 | 16 683.25 | 18 870 | 22 284.25 | 25 172.75 |
| Working frequencies assignable to ship stations for A1A or A1B Morse telegraphy e) f) p) |  |  | 8 371 to 8 376 11 f. 0.5 kHz |  |  |  |  |  |
| Limits (kHz) | 4 202.25 | 6 300.25 | 8 376.25 | 12 476.75 | 16 683.25 | 18 870 | 22 284.25 | 25 172.75 |
| Frequencies (paired) assignable to ship stations for NBDP telegraphy and data transmission systems at speeds not exceeding 100 bauds for FSK and 200 bauds for PSK d) j) m) p) |  |  | 8 376.5 to 8 396 40 f. 0.5 kHz | 12 477 to 12 549.5 146 f. 0.5 kHz | 16 683.5 to 16 733.5 101 f. 0.5 kHz | 18 870.5 to 18 892.5 45 f. 0.5 kHz | 22 284.5 to 22 351.5 135 f. 0.5 kHz | 25 173 to 25 192.5 40 f. 0.5 kHz |
| Limits (kHz) | 4 202.25 | 6 300.25 | 8 396.25 | 12 549.75 | 16 733.75 | 18 892.75 | 22 351.75 | 25 192.75 |
| Calling frequencies assignable to ship stations for A1A or A1B Morse telegraphy g) p) |  |  |  |  |  |  |  |  |
| Limits (kHz) | 4 202.25 | 6 300.25 | 8 396.25 | 12 554.75 | 16 738.75 | 18 892.75 | 22 351.75 | 25 192.75 |
| Frequencies (paired) assignable to ship stations for NBDP telegraphy and data transmission systems at speeds not exceeding 100 bauds for FSK and 200 bauds for PSK d) m) p) |  |  |  | 12 555 to 12 559.5 10 f. 0.5 kHz | 16 739 to 16 784.5 92 f. 0.5 kHz |  |  |  |
| Limits (kHz) | 4 202.25 | 6 300.25 | 8 396.25 | 12 559.75 | 16 784.75 | 18 892.75 | 22 351.75 | 25 192.75 |

Table of frequencies (kHz) to be used in the band between 4 000 kHz and 27 500 kHz
allocated exclusively to the maritime mobile service (continued)

| Band (MHz) | 4 | 6 | 8 | 12 | 16 | 18/19 | 22 | 25/26 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Limits (kHz) | 4 202.25 | 6 300.25 | 8 396.25 | 12 559.75 | 16 784.75 | 18 892.75 | 22 351.75 | 25 192.75 |
| Frequencies (non paired) assignable to ship stations for NBDP telegraphy and data transmission systems at speeds not exceeding 100 Bd for FSK and 200 Bd for PSK and for A1A or A1B Morse telegraphy (working) b) p) | 4 202.5 to 4 207 10 f. 0.5 kHz | 6 300.5 to 6 311.5 23 f. 0.5 kHz | 8 396.5 to 8 414 36 f. 0.5 kHz | 12 560 to 12 576.5 34 f. 0.5 kHz | 16 785 to 16 804 39 f. 0.5 kHz | 18 893 to 18 898 11 f. 0.5 kHz | 22 352 to 22 374 45 f. 0.5 kHz | 25 193 to 25 208 31 f. 0.5 kHz |
| Limits (kHz) | 4 207.25 | 6 311.75 | 8 414.25 | 12 576.75 | 16 804.25 | 18 898.25 | 22 374.25 | 25 208.25 |
| Frequencies assignable to ship stations for digital selective calling k) l) | 4 207.5 to 4 209 4 f. 0.5 kHz | 6 312 to 6 313.5 4 f. 0.5 kHz | 8 414.5 to 8 416 4 f. 0.5 kHz | 12 577 to 12 578.5 4 f. 0.5 kHz | 16 804.5 to 16 806 4 f. 0.5 kHz | 18 898.5 to 18 899.5 3 f. 0.5 kHz | 22 374.5 to 22 375.5 3 f. 0.5 kHz | 25 208.5 to 25 209.5 3 f. 0.5 kHz |
| Limits (kHz) | 4 209.25 | 6 313.75 | 8 416.25 | 12 578.75 | 16 806.25 | 18 899.75 | 22 375.75 | 25 210 |
| Limits (kHz) | 4 209.25 | 6 313.75 | 8 416.25 | 12 578.75 | 16 806.25 | 19 680.25 | 22 375.75 | 26 100.25 |
| Frequencies (paired) assignable to coast stations for NBDP and data transmission systems, at speeds not exceeding 100 Bd for FSK and 200 Bd for PSK d) n) o) p) | 4 209.5 to 4 219 20 f. 0.5 kHz | 6 314 to 6 330.5 34 f. 0.5 kHz | 8 416.5 to 8 436 40 f. 0.5 kHz | 12 579 to 12 656.5 156 f. 0.5 kHz | 16 806.5 to 16 902.5 193 f. 0.5 kHz | 19 680.5 to 19 703 46 f. 0.5 kHz | 22 376 to 22 443.5 136 f. 0.5 kHz | 26 100.5 to 26 120.5 41 f. 0.5 kHz |
| Limits (kHz) | 4 219.25 | 6 330.75 | 8 436.25 | 12 656.75 | 16 902.75 | 19 703.25 | 22 443.75 | 26 120.75 |
| Frequencies assignable to coast stations for digital selective calling l) | 4 219.5 to 4 220.5 3 f. 0.5 kHz | 6 331 to 6 332 3 f. 0.5 kHz | 8 436.5 to 8 437.5 3 f. 0.5 kHz | 12 657 to 12 658 3 f. 0.5 kHz | 16 903 to 16 904 3 f. 0.5 kHz | 19 703.5 to 19 704.5 3 f. 0.5 kHz | 22 444 to 22 445 3 f. 0.5 kHz | 26 121 to 26 122 3 f. 0.5 kHz |
| Limits (kHz) | 4 221 | 6 332.5 | 8 438 | 12 658.5 | 16 904.5 | 19 705 | 22 445.5 | 26 122.5 |
| Frequencies assignable to coast stations for wide-band and A1A or A1B Morse telegraphy, facsimile, special and data transmission systems and direct-printing telegraphy systems |  |  |  |  |  |  |  |  |
| Limits (kHz) | 4 351 | 6 501 | 8 707 | 13 077 | 17 242 | 19 755 | 22 696 | 26 145 |

Table of frequencies (kHz) to be used in the band between 4 000 kHz and 27 500 kHz
allocated exclusively to the maritime mobile service (end)

| Band (MHz) | 4 | 6 | 8 | 12 | 16 | 18/19 | 22 | 25/26 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Limits (kHz) | 4 351 | 6 501 | 8 707 | 13 077 | 17 242 | 19 755 | 22 696 | 26 145 |
| Frequencies assignable to coast stations for telephony, duplex operation a) | 4 352.4 to 4 436.4 29 f. 3 kHz | 6 502.4 to 6 523.4 8 f. 3 kHz | 8 708.4 to 8 813.4 36 f. 3 kHz | 13 078.4 to 13 198.4 41 f. 3 kHz | 17 243.4 to 17 408.4 56 f. 3 kHz | 19 756.4 to 19 798.4 15 f. 3 kHz | 22 697.4 to 22 853.4 53 f. 3 kHz | 26 146.4 to 26 173.4 10 f. 3 kHz |
| Limits (kHz) | 4 438 | 6 525 | 8 815 | 13 200 | 17 410 | 19 800 | 22 855 | 26 175 |

a)	See Part B, Section I.

b)	See Part B, Section III.

c)	The frequency bands may also be used by buoy stations for oceanographic data transmission and by stations interrogating these buoys.

d)	See Part B, Section II.

e)	In the frequency bands to be used by ship stations for A1A Morse telegraphy working at speeds not exceeding 40 Bd, administrations may assign additional frequencies interleaved between the assignable frequencies. Any frequencies so assigned shall be multiples of 100 Hz. Administrations shall ensure a uniform distribution of such assignments within the bands.

f)	See Part B, Section V.

g)	See Part B, Section IV.

h)	For the conditions of use of the frequency 8 364 kHz, see Appendix 13.

i)	For the use of the carrier frequencies 4 125 kHz, 6 215 kHz, 8 291 kHz, 12 290 kHz and 16 420 kHz in these sub-bands by ship and coast stations for distress and safety purposes, by single-sideband radiotelephony, see Article 31 and Appendix 13.

j)	For the use of the frequencies 4 177.5 kHz, 6 268 kHz, 8 376.5 kHz, 12 520 kHz and 16 695 kHz in these sub-bands by ship and coast stations for distress and safety purposes, by NBDP telegraphy, see Article 31.

k)	For the use of the frequencies 4 207.5 kHz, 6 312 kHz, 8 414.5 kHz, 12 577 kHz and 16 804.5 kHz in these sub-bands by ship and coast stations for distress and safety purposes, by digital selective calling, see Article 31.

l)	The following paired frequencies (for ship/coast stations) 4 208/4 219.5 kHz, 6 312.5/6 331 kHz, 8 415/ 8 436.5 kHz, 12 577.5/12 657 kHz, 16 805/16 903 kHz, 18 898.5/19 703.5 kHz, 22 374.5/22 444 kHz and 25 208.5/26 121 kHz are the first choice international frequencies for digital selective calling (see Article 54).

m)	Frequencies from these frequency bands may also be used for A1A or A1B Morse telegraphy (working) (see Part B, Section II).

n)	The frequencies 4 210 kHz, 6 314 kHz, 8 416.5 kHz, 12 579 kHz, 16 806.5 kHz, 19 680.5 kHz, 22 376 kHz and 26 100.5 kHz are the exclusive international frequencies for the transmission of Maritime Safety Information (MSI) (see Articles 31 and 33).

o)	The frequency 4 209.5 kHz is an exclusive international frequency for the transmission of NAVTEX type information (see Articles 31 and 33).

p)	These sub-bands, except the frequencies referred to in Notes j), n) and o), may be used for the initial testing and the possible future introduction within the maritime mobile service of new digital technologies. Stations using these sub-bands for this purpose shall not cause harmful interference to, and shall not claim protection from, other stations operating in accordance with Article 5.

PART  B  –  Channelling arrangements

Section I  –  Radiotelephony

SUP	COM4/228/12

ADD	COM4/228/13

DRAFT  RESOLUTION  [COM4/2]  (WRC-03)

Review of the frequency and channel arrangements in the maritime MF and 
HF bands with a view to improving efficiency by considering the use 
of new digital technology by the maritime mobile service

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that the agenda of this Conference included consideration of the use of new digital technology in the MF and HF bands in the maritime mobile service;

b)	that the introduction of new digital technology in the maritime mobile service shall not disrupt the distress and safety communications in the MF and HF bands including those established by the International Convention for the Safety of Life at Sea, 1974, as amended;

c)	that changes made in Appendix 17 should not prejudice the future use of these frequencies or the capabilities of systems or new applications required for use by the maritime mobile service;

d)	that the requirement to use new digital technologies in the maritime mobile service is growing rapidly;

e)	that the use of new digital technology on maritime HF and MF frequencies will make it possible to better respond to the emerging demand for new services;

f)	that the maritime HF A1A Morse telegraphy and narrow-band direct-printing frequency bands are significantly under-utilized at present;

g)	that the ITU Radiocommunication Sector is conducting ongoing studies to improve the efficiency in these bands,

noting

[a)	Resolution 347 (WRC-97);]

b)	that different digital technologies have already been developed and used in the MF and HF bands in several radio services,

noting also

that this conference has modified Appendix 17 to permit, on a voluntary basis, the use of various channels or bands identified in the MF and HF bands for initial testing and future introduction of new digital technology,

resolves

1	that, in order to provide full worldwide interoperability of equipment on ships, there should be one technology, or more than one interoperable worldwide technology, implemented under Appendix 17;

2	that, as soon as the ITU-R studies are completed, a future competent conference should consider necessary changes to Appendix 17 to enable the use of new technology by the maritime mobile service,

invites ITU-R

to finalize studies currently ongoing:

–	to identify future requirements of the maritime mobile service;

–	to identify the technical characteristics necessary to facilitate use of digital systems in the MF and HF maritime mobile bands taking into account any relevant ITU-R Recommendations;

–	to identify the digital system(s) to be used by the MF/HF maritime service;

–	to identify any necessary modifications to the frequency plan contained within Appendix 17;

–	to propose a timetable for the introduction of new digital technology and any consequential changes to Appendix 17;

–	to recommend how digital technology can be introduced while ensuring compliance with the distress and safety requirements,

instructs the Secretary-General

to bring this Resolution to the attention of the International Maritime Organization, the International Civil Aviation Organization, the International Association of Marine Aids to Navigation and Lighthouse Authorities and the Comité International Radio-Maritime.

MOD	COM4/228/14

RESOLUTION  207 (Rev.WRC-03)

Measures to address unauthorized use of and interference to frequencies in the bands allocated to the maritime mobile service and 
to the aeronautical mobile (R) service

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that the HF frequencies currently used by the aeronautical and maritime mobile services for distress, safety and other communications, including allotted operational frequencies, suffer from harmful interference and are often subject to difficult propagation conditions;

b)	that WRC-97 considered some aspects of the use of the HF bands for distress and safety communications in the context of the Global Maritime Distress and Safety System (GMDSS), especially with regard to regulatory measures;

c)	that unauthorized operations using maritime and aeronautical frequencies in the HF bands are continuing to increase and are already a serious risk to HF distress, safety and other communications;

d)	that some administrations have resorted to, for example, transmitting warning messages on operational HF channels as a means of deterring unauthorized users;

e)	that provisions of the Radio Regulations prohibit the unauthorized use of certain safety frequencies for communications other than those related to safety;

f)	that enforcing compliance with these regulatory provisions is becoming increasingly difficult with the availability of low-cost HF single side-band (SSB) transceivers;

g)	that monitoring observations of the use of frequencies in the band 2 170-2 194 kHz and in the bands allocated exclusively to the maritime mobile service between 4 063 kHz and 27 500 kHz and to the aeronautical mobile (R) service between 2 850 kHz and 22 000 kHz show that a number of frequencies in these bands are still being used by stations of other services, many of which are operating in contravention of No. 23.2;

h)	that, in certain situations, HF radio is the sole means of communication for the maritime mobile service and that certain frequencies in the bands mentioned in considering g) are reserved for distress and safety purposes;

i)	that, in certain situations, HF radio is the sole means of communication for the aeronautical mobile (R) service and that this is a safety service;

j)	that WRC-2000 and this Conference have reviewed the use of the HF bands by the aeronautical mobile (R) and maritime mobile services with a view to protecting operational, distress and safety communications;

k)	that this Resolution identifies several interference mitigation techniques that can be employed by administrations on a non-mandatory basis,

considering in particular

a)	that it is of paramount importance that the distress and safety channels of the maritime mobile service be kept free from harmful interference, since they are essential for the protection of the safety of life and property;

b)	that it is also of paramount importance that channels directly concerned with the safe and regular conduct of aircraft operations be kept free from harmful interference, since they are essential for the safety of life and property,

resolves to invite ITU-R and ITU-D, as appropriate

to increase regional awareness of appropriate practices in order to help mitigate interference in the HF bands, especially on distress and safety channels,

invites administrations

1	to ensure that stations of services other than the maritime mobile service abstain from using frequencies in distress and safety channels and their guardbands and in the bands allocated exclusively to that service, except under the conditions expressly specified in Nos. 4.4, 5.128, 5.129, 5.137 and 4.13 to 4.15; and to ensure that stations of services other than the aeronautical mobile (R) service abstain from using frequencies allocated to that service except under the conditions expressly specified in Nos. 4.4 and 4.13;

2	to make every effort to identify and locate the source of any unauthorized emission capable of endangering human life or property and the safe and regular conduct of aircraft operations, and to communicate their findings to the Radiocommunication Bureau;

3	to participate, in accordance with item 4 in the Annex, in any monitoring programmes organized by the Radiocommunication Bureau or administrations, if so agreed among those administrations, without adversely affecting the rights of other administrations or conflicting with any provisions of the Radio Regulations;

4	to make every effort to prevent unauthorized transmissions in bands allocated to the maritime mobile service and the aeronautical mobile (R) service;

5	to request their competent authorities to take, within their respective jurisdiction, such legislative or regulatory measures which they consider necessary or appropriate in order to prevent stations from unauthorized use of distress and safety channels or from operating in contravention of No. 23.2;

6	to take all necessary steps in such cases of contravention of No. 23.2 to ensure the cessation of any transmissions contravening the provisions of the Radio Regulations on the frequencies or in the bands referred to in this Resolution;

7	to employ as many of the interference mitigation techniques in the Annex as are appropriate for the maritime mobile and aeronautical mobile (R) services,

instructs the Radiocommunication Bureau

1	to seek the cooperation of administrations in identifying the sources of those emissions by all available means and in securing the cessation of those emissions;

2	when the station of another service transmitting in a band allocated to the maritime mobile service or to the aeronautical mobile (R) service has been identified, to inform the administration concerned;

3	to include the problem of interference to maritime and aeronautical distress and safety channels on the agenda of relevant regional radiocommunication seminars,

instructs the Secretary-General

to bring this Resolution to the attention of the International Maritime Organization and the International Civil Aviation Organization for such actions as they may consider appropriate.

ADD	COM4/228/15

Annex  to  Resolution  207  (Rev.WRC-03)

Interference mitigation techniques

This Annex lists several possible HF interference mitigation techniques that may be used to protect receivers either in combination or singly, depending on the resources of administrations. Use of any or all of these techniques is not mandatory.

# 1	Alternative modulation methods

The use of digitally modulated emissions, such as QPSK, to replace or supplement analogue SSB voice (J3E) and data (J2B) emissions. This initiative would need to be adopted internationally to allow the interoperability of equipment. For example, ICAO has adopted the HF data-link standard to provide packet data communications using automated link establishment and adaptive frequency control techniques as a supplement to analogue SSB voice communications (see ICAO Convention Annex 10).

# 2	Passive and dynamic/adaptive antenna systems

Use of passive and active/adaptive antenna systems to reject unwanted signals.

# 3	Channel barring

Administrations should ensure through their licensing, equipment standardization and inspection arrangements that, in compliance with No. 43.1, HF radio equipment cannot transmit on frequencies exclusively assigned to the aeronautical mobile (R) service, as detailed in Appendix 27, except for frequencies allocated for worldwide use and shared with the aeronautical mobile (OR) service (see Appendix 26/3.4).

# 4	Regional HF monitoring and direction-finding facilities

Collaboration and cooperation between regional administrations to coordinate the use of monitoring and direction-finding facilities.

# 5	Transmission of warning messages

Transmission of multilanguage warning messages on specific channels affected by strong or persistent interference. Such transmissions should be conducted after coordination with the users of the affected services and the administration(s) or competent authorities concerned.

# 6	Education and publicity initiatives

Administrations provide education and publicity initiatives on the proper use of the radio-frequency spectrum in these bands.

MOD	COM4/228/16

APPENDIX  11

System specifications for double-sideband (DSB), single-sideband (SSB)
and digitally modulated emissions in the HF broadcasting service

PART  B  –  Single-sideband (SSB) system

MOD	COM4/228/17

## 1.1	Channel spacing

In a mixed DSB, SSB and digital environment (see Resolution 517 (Rev.WRC-03)), the channel spacing shall be 10 kHz. In the interest of spectrum conservation, it is also permissible to interleave SSB emissions midway between two adjacent DSB channels, i.e., with 5 kHz separation between carrier frequencies, provided that the interleaved emission is not to the same geographical area as either of the emissions between which it is interleaved.

In an all inclusive SSB environment, the channel spacing and carrier frequency separation shall be 5 kHz.

MOD	COM4/228/18

## 2.6	Carrier reduction (relative to peak envelope power)

In a mixed DSB, SSB and digital environment the carrier reduction shall be 6 dB to allow SSB emissions to be received by conventional DSB receivers with envelope detection without significant deterioration of the reception quality.

MOD	COM4/228/19

## 3.2	Demodulator and carrier acquisition

The reference receiver is equipped with a synchronous demodulator, using for the carrier acquisition a device which regenerates a carrier by means of a suitable control loop which locks the receiver to the incoming carrier. The reference receiver should work as well with DSB emissions as with SSB emissions having a carrier reduced to 6 dB below peak envelope power.

ADD	COM4/228/20

PART C    Digital system

# 1	System parameters

## 1.1	Channel spacing

The initial spacing for digitally modulated emissions use shall be 10 kHz. However, interleaved channels with a separation of 5 kHz may be used in accordance with the appropriate protection criteria appearing in Resolution [COM4/1] (WRC-03), provided that the interleaved emission is not to the same geographical area as either of the emissions between which it is interleaved.

## 1.2	Channel utilization

Channels using digitally modulated emissions may share the same spectrum or be interleaved with analogue emissions in the same HFBC band provided the protection to the analogue emissions is at least as great as that which is currently in force with analogue-to-analogue protection. To accomplish this may require that the digital spectral power density (and total power) be lower by several dB than is currently used for the same emission circuit using either DSB or SSB emissions.

# 2	Emission characteristics

## 2.1	Bandwidth and centre frequency

A full digitally modulated emission will have a 10 kHz bandwidth with its centre frequency at any of the 5 kHz centre frequency locations in the same channel raster within the HFBC bands.

Among several possible “simulcast” modes are those having a combination of analogue and digital emissions of the same programme in the same channel, that may use a digital emission of 5 kHz or 10 kHz bandwidth, next to either a 5 kHz or 10 kHz analogue emission. In all cases of this type, the 5 kHz interleaved raster used in HFBC shall be adhered to in placing the emission within the HFBC bands.

## 2.2	Frequency tolerance

The frequency tolerance shall be 10 Hz.1

## 2.3	Audio-frequency band

Digital source coding within a 10 kHz bandwidth, taking account of the need for various levels of error avoidance, detection and correction coding emission mitigation, can range from the equivalent of monophonic FM (approximately 15 kHz) to low-level speech codec performance of the order of 3 kHz. The choice of audio quality is connected to the needs of the broadcaster/listener, and includes such characteristics to consider as the propagation channel conditions expected. There is no single specification, only the upper and lower bounds noted in this paragraph.

## 2.4	Modulation

Quadrature amplitude modulation (QAM) with Orthogonal frequency division multiplexing (OFDM) shall be used. 64-QAM is feasible under many propagation conditions; others such as 32-, 16- and 8-QAM are specified for use when needed.

## 2.5	RF protection ratio values

The protection ratio values for analogue and digital emissions for co-channel and adjacent channel conditions shall be in accordance with Resolution [COM4/1] (WRC-03) as a provisional RF protection ratio value subject to revisions or confirmation by WRC-07.

MOD	COM4/228/21

RECOMMENDATION  517  (Rev.wrc-03)

Relative RF protection ratio values for single-sideband (SSB) 
emissions in the HF bands allocated
to the broadcasting service

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that WRC-97 adopted Article 12 as the seasonal planning procedure for the HF bands allocated to the broadcasting service;

b)	that this procedure is based principally on the use of double-sideband (DSB) emissions;

c)	that the RF co-channel protection ratio is one of the fundamental planning parameters;

d)	that this Conference has adopted Resolution 517 (Rev.WRC-03) relating to the introduction of digitally modulated and SSB emissions in the HF bands allocated to the broadcasting service;

e)	that the SSB system characteristics for HF broadcasting are contained in Appendix 11;

f)	that studies have shown that SSB emissions may require a lower RF co-channel protection ratio for the same reception quality,

recommends

that, in the preparation of the relevant Rules of Procedure for the application of Article 12, the Bureau should use the values of relative RF protection ratio given in the Annex to this Recommendation relating to SSB and DSB emissions in the HF bands allocated to the broadcasting service.

ANNEX  TO  RECOMMENDATION  517  (Rev.wrc-03)

Relative RF protection ratio values

1	The values of relative RF protection ratio given in the table should be used whenever SSB emissions in conformity with the specification in Appendix 11 are involved in the use of the HF bands allocated to the broadcasting service.

2	For the reception of DSB and SSB (6 dB carrier reduction relative to peak envelope power) wanted signals, a conventional DSB receiver with envelope detection designed for a channel spacing of 10 kHz is assumed.

3	SSB signals with 6 dB carrier reduction relative to peak envelope power assume equivalent sideband power as specified in Appendix 11, Part B, § 1.2.

4	The figures for case 2 in the following Table relate to a situation where the centre frequency of the intermediate frequency pass-band of the DSB receiver is tuned to the carrier frequency of the wanted SSB signal. If this is not the case, the value for a difference of +5 kHz may increase to –1 dB.

Relative RF protection ratio values with reference to the co-channel RF protection ratio for DSB wanted and 
unwanted signals (dB)1 for use in the HF bands allocated to the broadcasting service

|  | Wanted signal | Unwanted signal | Carrier frequency separation f unwanted – f wanted, f (kHz) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | –20 | –15 | –10 | –5 | 0 | +5 | +10 | +15 | +20 |
| 1 | DSB | SSB (6 dB carrier reduction relative to p.e.p.) | –51 | –46 | –32 | +1 | 3 | –2 | –32 | –46 | –51 |
| 2 | SSB (6 dB carrier reduction relative to p.e.p.) | DSB | –54 | –49 | –35 | –3 | 0 | –3 | –35 | –49 | –54 |
| 3 | SSB (6 dB carrier reduction relative to p.e.p.) | SSB (6 dB carrier reduction relative to p.e.p.) | –51 | –46 | –32 | +1 | 0 | –2 | –32 | –46 | –51 |

1	Frequency separation f less than –20 kHz, as well as f greater than 20 kHz, need not be considered.

SUP	COM4/228/22

RESOLUTION  29  (WRC-97)

Information on the occupancy by fixed and mobile services in the additional
HF bands allocated by WARC-92 to the broadcasting service

MOD	COM4/228/23

RESOLUTION  21  (Rev.WRC-03)

Implementation of changes in frequency allocations
between 5 900 kHz and 19 020 kHz

The World Radiocommunication Conference (Geneva, 1995),

considering

a)	that parts of the frequency bands between 5 900 kHz and 19 020 kHz which were previously allocated on an exclusive or shared basis to the fixed and mobile services have been reallocated to the broadcasting service;

b)	that some existing fixed and mobile assignments may need to be removed progressively from those reallocated bands to make way for the broadcasting service;

c)	that the assignments to be removed, termed “displaced assignments”, must be reaccommodated in other appropriate frequency bands;

d)	that developing countries may require special assistance from the Radiocommunication Bureau in replacing their displaced assignments with appropriate protection;

e)	that procedures already exist in Article 11 that may be used to this effect,

recognizing

the difficulties that administrations and the Bureau might encounter during the period of transition from the previous allocations to those made by WARC-92,

resolves

1	that the duration of the transition period shall be from 1 April 1992 to 1 April 2007;

2	that administrations should no longer notify any frequency assignments to stations of the fixed and mobile services in the reallocated bands. Assignments notified in these bands after 1 April 1992 shall bear a symbol to indicate that the finding will be examined by the Bureau as of 1 April 2007 in accordance with the provisions of No. 11.31;

3	that the Bureau shall undertake a continuing action to review the Master International Frequency Register with the help of administrations. In this respect, the Bureau shall periodically consult the administrations concerning the frequency assignments to links for which another satisfactory means of telecommunication exists, with a view to either downgrading assignments of class of operation A or deleting such assignments;

4	that administrations shall, for assignments of class of operation A in the reallocated bands, either notify the replacement frequencies to the Bureau or request the Bureau’s assistance in selecting the replacement frequencies in application of Articles 7 and 13;

5	that the Bureau shall develop in due time a draft procedure to be used for the replacement of remaining frequency assignments and shall consult administrations in accordance with Article 14;

6	that the Bureau should modify the draft procedures taking into account, to the extent practicable, comments received from administrations, and propose replacement assignments at the latest three years before 1 April 2007. In so doing, the Bureau shall request administrations to take appropriate action to bring their assignments in conformity with the Table of Frequency Allocations by the due date;

7	that a replacement frequency assignment whose basic characteristics, with the exception of the assigned frequency, have not been modified in the above process, shall keep its original date. However, if these basic characteristics of a replacement frequency assignment are different from those of the displaced assignment, the replacement assignment shall be treated in accordance with the relevant provisions of Section II of Article 11,

invites administrations

when seeking reaccommodation of the displaced assignments for their fixed and mobile services in the bands between 5 900 kHz and 19 020 kHz which have been reallocated to the broadcasting service, to make every effort to find replacement assignments in the bands allocated to the fixed and mobile services concerned.

_________________

1 	Within the non-shaded boxes.

1 	See Note 21 of Appendix 2.