| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Document 231-E |
|  |  |  | 20 June 2003 |
|  |  |  | Original: English |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | COMMITTEE 4 |
| --- | --- |
| Working Group 4A |  |
| third report of working group 4a to committee 4 |  |

Working Group 4A considered Addendum 3 of Document 4, the Report of the Director, BR, and decided to propose modifications to the Radio Regulations based on 4.2, 4.3, 4.7, 4.18 and 4.19 of Table 4 and § 3.3.1, 3.4 and 4.2 of the BR Report. The proposed modifications are submitted to Committee 4 for consideration.

ARTICLE  24

Fixed service

Section II  –  Frequencies for the international exchange of police information

SUP

24.3

SUP

24.4

Section III  –  Frequencies for the international exchange of synoptic
meteorological information

SUP

24.5

SUP

24.6

APPENDIX  2

Table of transmitter frequency tolerances

Notes in the table of transmitter frequency tolerances

SUP

6	In countries covered by the North American Regional Broadcasting Agreement (NARBA) the tolerance of 20 Hz may continue to be applied.

SUP

RECOMMENDATION  709

Relating to sharing frequency bands between the aeronautical
mobile service and the inter-satellite service1

SUP

RECOMMENDATION  710

Relating to the use of airborne radars in the frequency bands shared
between the inter-satellite service and the radiolocation service1

ARTICLE  11

Notification and recording of frequency assignments1, 2, 3, 4

Section I  –  Notification

ADD

11.21B	f)	any terrestrial station in the bands referred to in a footnote having reference to No. 9.21, if it pertains to a service which is subject to the application of the procedure for seeking agreement under No. 9.21;

MOD

11.22	g)	earth stations whose coordination area includes the territory of another administration, or where the earth station is located within the coordination area of an earth station operating in the opposite direction of transmission;6, 7

MOD

11.23	h)	earth stations whose interference potential is greater than that of a coordinated typical earth station.6

Section II  –  Examination of notices and recording of frequency assignments
in the Master Register

ADD

11.50		The Bureau shall review periodically each section of the Master Register with the aim of improving and maintaining its accuracy, with a particular emphasis on the review of the findings so as to adjust them to the changing allocation situation after each conference.

ARTICLE  4

Assignment and use of frequencies

Section I  –  General rules

MOD

4.8		Where, in adjacent Regions or sub-Regions, a band of frequencies is allocated to different services of the same category (see Sections I and II of Article 5), the basic principle is the equality of right to operate. Accordingly, the stations of each service in one Region or sub-Region must operate so as not to cause harmful interference to any service of the same category in the other Regions or sub-Regions.

ARTICLE  21

Terrestrial and space services sharing frequency bands above 1 GHz

Section II  –  Power limits for terrestrial stations

MOD

TABLE  21-2     (WRC-2000)

| Frequency band | Service | Limit as specified in Nos. |
| --- | --- | --- |
| 1 610-1 645.5 MHz (No. 5.359) 1 646.5-1 660 MHz (No. 5.359) 1 675-1 690 MHz (Region 2) 1 690-1 700 MHz (Region 2 countries listed in No. 5.381) 1 700-1 710 MHz (Region 2) 1 980-2 010 MHz 2 010-2 025 MHz (Region 2) 2 025-2 110 MHz 2 200-2 290 MHz 2 655-2 670 MHz5 (Regions 2 and 3) 2 670-2 690 MHz 5 670-5 725 MHz (Nos. 5.453 and 5.455) 5 725-5 755 MHz5 (Region 1 countries listed in Nos. 5.451, 5.453 and 5.455) | Fixed-satellite Meteorological-satellite Space research Space operation Earth exploration-satellite Mobile-satellite | 21.2, 21.3, 21.4 and 21.5 |
| 5 755-5 850 MHz5 (Region 1 countries listed in Nos. 5.451, 5.453, 5.455 and 5.456) 5 850-7 075 MHz 7 900-8 400 MHz 8 400-8 500 MHz |  |  |

MOD

TABLE  21-2 (end)

| Frequency band | Service | Limit as specified in Nos. |
| --- | --- | --- |
| 10.7-11.7 GHz5 (Region 1) 12.5-12.75 GHz5 (Nos. 5.494 and 5.496) 12.7-12.75 GHz5 (Region 2) 12.75-13.25 GHz 13.75-14 GHz (Nos. 5.499 and 5.500) 14.0-14.25 GHz (No. 5.505) 14.25-14.3 GHz (Nos. 5.505, 5.508 and 5.509) 14.3-14.4 GHz5 (Regions 1 and 3) 14.4-14.5 GHz 14.5-14.8 GHz | Fixed-satellite | 21.2, 21.3 and 21.5 |
| 17.3-17.7 GHz (No. 5.514) 17.7-18.4 GHz 18.6-18.8 GHz 19.3-19.7 GHz 22.55-23.55 GHz 24.45-24.75 GHz (Regions 1 and 3) 24.75-25.25 GHz (Region 3) 25.25-29.5 GHz 29.5-31 GHz (No. 5.542) | Fixed-satellite Earth exploration-satellite Space research Inter-satellite | 21.2, 21.3, 21.5 and 21.5A |

____________

1	WRC-97 made editorial amendments to this Recommendation.