| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Document 234-E |
|  |  |  | 20 June 2003 |
|  |  |  | Original: English |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | COMMITTEE 5 |
| --- | --- |
| Sixth Report from Working Group 5E to Committee 5 on agenda item 1.38 |  |

The following report on agenda item 1.38 is presented to Committee 5 for consideration.

J. ZUZEK
				Chairman, Working Group 5E, Box 507

Annex: 1

annex

For the Report of the Chairman of Committee 5

Working Group 5E accepted an offer from the Space Frequency Coordination Group (SFCG) to provide and maintain a freely available website containing up-to-date advanced operational schedule information concerning each and every EESS observation campaign in the band 432-438 MHz.

The Administration of India brought to the attention of WRC-03 the need to protect the primary radio astronomy service in the band 406.1-410 MHz from potential spurious/out-of-band emissions from active sensors operating in the Earth exploration-satellite service (active) in a proposed secondary allocation in the band 432-438 MHz. After wide-ranging discussions, the Administration of India has agreed to pursue resolution of this issue through the medium of SFCG and the Scientific Committee on Frequency Allocations for Radio Astronomy and Space Science (IUCAF). It is hoped that the SFCG will develop and publish a Recommendation on this issue in the near future.

ARTICLE  5

MOD	COM5/234/1

410-470 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 430-432 AMATEUR RADIOLOCATION | 430-432 RADIOLOCATION Amateur |  |
| 5.271 5.272 5.273 5.274 5.275 5.276 5.277 | 5.271 5.276 5.277 5.278 5.279 |  |

MOD	COM5/234/2

| 432-438 AMATEUR RADIOLOCATION Earth exploration-satellite (active) ADD 5.5E03 | 432-438 RADIOLOCATION Amateur Earth exploration-satellite (active) ADD 5.5E03 |
| --- | --- |
| 5.138 5.271 5.272 5.276 5.277 5.280 5.281 5.282 | 5.271 5.276 5.277 5.278 5.279 5.281 5.282 |

MOD	COM5/234/3

| 438-440 AMATEUR RADIOLOCATION | 438-440 RADIOLOCATION Amateur |
| --- | --- |
| 5.271 5.273 5.274 5.275 5.276 5.277 5.283 | 5.271 5.276 5.277 5.278 5.279 |

ADD	COM5/234/4

5.5E03	The use of this band by sensors in the Earth exploration-satellite service (active) shall be in accordance with Recommendation ITU-R SA.1260-1. Additionally, the EESS (active) in the band 432-438 MHz shall not cause harmful interference to the aeronautical radionavigation service in China.

The provisions of this footnote in no way derogate the obligation of the Earth exploration-satellite (active) to operate as a secondary service in accordance with Nos. 5.29 and 5.30.

SUP	COM5/234/5

RESOLUTION  727  (Rev.WRC-2000)

Use of the frequency band 420-470 MHz by the 
Earth exploration-satellite service (active)

______________