| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Document 272-E |
|  |  |  | 24 June 2003 |
|  |  |  | Original: English |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | COMMITTEE 8 |
| --- | --- |
| sixth series of texts submitted by committee 4 to the editorial committee |  |

The following texts are submitted to Committee 8:

–	Article 4

–	Article 5

–	Article 11

–	Article 21 – Table 21-2

–	Article 22

–	Article 24

–	Appendix 2

–	Resolution 26

–	Resolution 76

–	Resolution 137

–	Recommendation 705

–	Recommendation 709

–	Recommendation 710

E. GEORGE
	Chairman, Committee 4, Box 123

ARTICLE  4

Assignment and use of frequencies

Section I  –  General rules

MOD	COM4/272/1

4.8		Where, in adjacent Regions or sub-Regions, a band of frequencies is allocated to different services of the same category (see Sections I and II of Article 5), the basic principle is the equality of right to operate. Accordingly, the stations of each service in one Region or sub-Region must operate so as not to cause harmful interference to any service of the same or higher category in the other Regions or sub-Regions.

article  5

Frequency allocations

MOD	COM4/272/2

5.56	The stations of services to which the bands 14-19.95 kHz and 20.05-70 kHz and in Region 1 also the bands 72-84 kHz and 86-90 kHz are allocated may transmit standard frequency and time signals. Such stations shall be afforded protection from harmful interference. In Armenia, Azerbaijan, Belarus, Bulgaria, Georgia, Kazakstan, Mongolia, Kyrgyzstan, Slovakia, the Czech Rep., Russian Federation, Tajikistan and Turkmenistan, the frequencies 25 kHz and 50 kHz will be used for this purpose under the same conditions.     (WRC-03)

MOD	COM4/272/3

5.68	Alternative allocation:  in Angola, Burundi, the Congo, Malawi, Dem. Rep. of the Congo, Rwanda and South Africa, the band 160-200 kHz is allocated to the fixed service on a primary basis.

MOD	COM4/272/4

5.70	Alternative allocation:  in Angola, Botswana, Burundi, Cameroon, the Central African Rep., the Congo, Ethiopia, Lesotho, Madagascar, Malawi, Mozambique, Namibia, Nigeria, Oman, Dem. Rep. of the Congo, Rwanda, South Africa, Swaziland, Tanzania, Chad, Zambia and Zimbabwe, the band 200-283.5 kHz is allocated to the aeronautical radionavigation service on a primary basis.

MOD	COM4/272/5

5.87	Additional allocation:  in Angola, Botswana, Lesotho, Malawi, Mozambique, Namibia, South Africa, Swaziland and Zimbabwe, the band 526.5-535 kHz is also allocated to the mobile service on a secondary basis.

MOD	COM4/272/6

5.96	In Germany, Armenia, Austria, Azerbaijan, Belarus, Denmark, Estonia, Finland, Georgia, Hungary, Iceland, Ireland, Israel, Kazakstan, Latvia, Liechtenstein, Lithuania, Malta, Moldova, Norway, Uzbekistan, Poland, Kyrgyzstan, Slovakia, the Czech Rep., the United Kingdom, the Russian Federation, Sweden, Switzerland, Tajikistan, Turkmenistan and Ukraine,

administrations may allocate up to 200 kHz to their amateur service in the bands 1 715-1 800 kHz and 1 850-2 000 kHz. However, when allocating the bands within this range to their amateur service, administrations shall, after prior consultation with administrations of neighbouring countries, take such steps as may be necessary to prevent harmful interference from their amateur service to the fixed and mobile services of other countries. The mean power of any amateur station shall not exceed 10 W.     (WRC-03)

MOD	COM4/272/7

5.98	Alternative allocation:  in Angola, Armenia, Azerbaijan, Belarus, Belgium, Bulgaria, Cameroon, the Congo, Denmark, Egypt, Eritrea, Spain, Ethiopia, Georgia, Greece, Italy, Kazakstan, Lebanon, Lithuania, Moldova, Syria, Kyrgyzstan, the Russian Federation, Somalia, Tajikistan, Tunisia, Turkmenistan, Turkey and Ukraine, the band 1 810-1 830 kHz is allocated to the fixed and mobile, except aeronautical mobile, services on a primary basis.     (WRC-03)

MOD	COM4/272/8

5.99	Additional allocation:  in Saudi Arabia, Austria, Bosnia and Herzegovina, Iraq, Libya, Uzbekistan, Slovakia, Romania, Slovenia, Chad, Togo and Yugoslavia, the band 1 810-1 830 kHz is also allocated to the fixed and mobile, except aeronautical mobile, services on a primary basis.     (WRC-03)

MOD	COM4/272/9

5.107	Additional allocation:  in Saudi Arabia, Eritrea, Ethiopia, Iraq, Lesotho, Libya, Somalia and Swaziland, the band 2 160-2 170 kHz is also allocated to the fixed and mobile, except aeronautical mobile (R), services on a primary basis. The mean power of stations in these services shall not exceed 50 W.     (WRC-03)

MOD	COM4/272/10

5.112	Alternative allocation:  in Bosnia and Herzegovina, Denmark, Malta, Sri Lanka and Yugoslavia, the band 2 194-2 300 kHz is allocated to the fixed and mobile, except aeronautical mobile, services on a primary basis.     (WRC-03)

MOD	COM4/272/11

5.114	Alternative allocation:  in Bosnia and Herzegovina, Denmark, Iraq, Malta, and Yugoslavia, the band 2 502-2 625 kHz is allocated to the fixed and mobile, except aeronautical mobile, services on a primary basis.     (WRC-03)

MOD	COM4/272/12

5.117	Alternative allocation:  in Bosnia and Herzegovina, Côte d’Ivoire, Denmark, Egypt, Liberia, Malta, Sri Lanka, Togo and Yugoslavia, the band 3 155-3 200 kHz is allocated to the fixed and mobile, except aeronautical mobile, services on a primary basis.     (WRC-03)

MOD	COM4/272/13

5.118	Additional allocation:  in the United States, Mexico, Peru and Uruguay, the band 3 230-3 400 kHz is also allocated to the radiolocation service on a secondary basis.

MOD	COM4/272/14

5.140	Additional allocation:  in Angola, Iraq, Kenya, Rwanda, Somalia and Togo, the band 7 000-7 050 kHz is also allocated to the fixed service on a primary basis.

MOD	COM4/272/15

5.152	Additional allocation:  in Armenia, Azerbaijan, China, Côte d’Ivoire, Georgia, Iran (Islamic Republic of), Kazakstan, Uzbekistan, Kyrgyzstan, the Russian Federation, Tajikistan, Turkmenistan and Ukraine, the band 14 250-14 350 kHz is also allocated to the fixed service on a primary basis. Stations of the fixed service shall not use a radiated power exceeding 24 dBW.     (WRC-03)

MOD	COM4/272/16

5.154	Additional allocation:  in Armenia, Azerbaijan, Georgia, Kazakstan, Kyrgyzstan, the Russian Federation, Tajikistan, Turkmenistan and Ukraine, the band 18 068-18 168 kHz is also allocated to the fixed service on a primary basis for use within their boundaries, with a peak envelope power not exceeding 1 kW.     (WRC-03)

MOD	COM4/272/17

5.155	Additional allocation:  in Armenia, Azerbaijan, Belarus, Bulgaria, Georgia, Kazakstan, Moldova, Mongolia, Uzbekistan, Kyrgyzstan, Slovakia, the Czech Rep., Russian Federation, Tajikistan, Turkmenistan and Ukraine, the band 21 850-21 870 kHz is also allocated to the aeronautical mobile (R) services on a primary basis.

MOD	COM4/272/18

5.163	Additional allocation:  in Armenia, Azerbaijan, Belarus, Georgia, Hungary, Kazakstan, Latvia, Lithuania, Moldova, Mongolia, Uzbekistan, Kyrgyzstan, Slovakia, the Czech Rep., Russian Federation, Tajikistan, Turkmenistan and Ukraine, the bands 47-48.5 MHz and 56.5-58 MHz are also allocated to the fixed and land mobile services on a secondary basis.

MOD	COM4/272/19

5.174	Alternative allocation:  in Bulgaria, Hungary and Romania, the band 68-73 MHz is allocated to the broadcasting service on a primary basis and used in accordance with the decisions in the Final Acts of the Special Regional Conference (Geneva, 1960).     (WRC-03)

MOD	COM4/272/20

5.177	Additional allocation:  in Armenia, Azerbaijan, Belarus, Bulgaria, Georgia, Kazakstan, Latvia, Moldova, Uzbekistan, Kyrgyzstan, the Russian Federation, Tajikistan, Turkmenistan and Ukraine, the band 73-74 MHz is also allocated to the broadcasting service on a primary basis, subject to agreement obtained under No. 9.21.     (WRC-03)

MOD	COM4/272/21

5.179	Additional allocation:  in Armenia, Azerbaijan, Belarus, Bulgaria, China, Georgia, Kazakstan, Lithuania, Moldova, Mongolia, Kyrgyzstan, Slovakia, Russian Federation, Tajikistan, Turkmenistan and Ukraine, the bands 74.6-74.8 MHz and 75.2-75.4 MHz are also allocated to the aeronautical radionavigation service, on a primary basis, for ground-based transmitters only.

MOD	COM4/272/22

5.181	Additional allocation:  in Egypt, Israel and Syria, the band 74.8-75.2 MHz is also allocated to the mobile service on a secondary basis, subject to agreement obtained under No. 9.21. In order to ensure that harmful interference is not caused to stations of the aeronautical radionavigation service, stations of the mobile service shall not be introduced in the band until it is no longer required for the aeronautical radionavigation service by any administration which may be identified in the application of the procedure invoked under No. 9.21.     (WRC-03)

MOD	COM4/272/23

5.203B	Additional allocation:  in Saudi Arabia, United Arab Emirates, Oman and Syria, the band 136-137 MHz is also allocated to the fixed and mobile, except aeronautical mobile, services on a secondary basis until 1 January 2005.     (WRC-03)

MOD	COM4/272/24

5.204	Different category of service:  in Afghanistan, Saudi Arabia, Bahrain, Bangladesh, Bosnia and Herzegovina, Brunei Darussalam, China, Cuba, the United Arab Emirates, India, Indonesia, Iran (Islamic Republic of), Iraq, Malaysia, Oman, Pakistan, Philippines, Qatar, Singapore, Thailand, Yemen and Yugoslavia, the band 137-138 MHz is allocated to the fixed and mobile, except aeronautical mobile (R), services on a primary basis (see No. 5.33).

MOD	COM4/272/25

5.210	Additional allocation:  in France, Italy, the Czech Rep. and the United Kingdom, the bands 138-143.6 MHz and 143.65-144 MHz are also allocated to the space research service (space-to-Earth) on a secondary basis.     (WRC-03)

MOD	COM4/272/26

5.212	Alternative allocation:  in Angola, Botswana, Burundi, Cameroon, the Central African Rep., the Congo, Gabon, Gambia, Ghana, Guinea, Iraq, Jordan, Lesotho, Liberia, Libya, Malawi, Mozambique, Namibia, Oman, Uganda, Dem. Rep. of the Congo, Rwanda, Sierra Leone, South Africa, Swaziland, Chad, Togo, Zambia and Zimbabwe, the band 138-144 MHz is allocated to the fixed and mobile services on a primary basis.     (WRC-03)

MOD	COM4/272/27

5.221	Stations of the mobile-satellite service in the band 148-149.9 MHz shall not cause harmful interference to, or claim protection from, stations of the fixed or mobile services operating in accordance with the Table of Frequency Allocations in the following countries: Albania, Algeria, Germany, Saudi Arabia, Australia, Austria, Bahrain, Bangladesh, Barbados, Belarus, Belgium, Benin, Bosnia and Herzegovina, Botswana, Brunei Darussalam, Bulgaria, Cameroon, China, Cyprus, Congo, Korea (Rep. of), Croatia, Cuba, Denmark, Egypt, the United Arab Emirates, Eritrea, Spain, Estonia, Ethiopia, Finland, France, Gabon, Ghana, Greece, Guinea, Guinea Bissau, Hungary, India, Iran (Islamic Republic of), Ireland, Iceland, Israel, Italy, Jamaica, Japan, Jordan, Kazakstan, Kenya, Kuwait, Latvia, The Former Yugoslav Republic of Macedonia, Lebanon, Libya, Liechtenstein, Lithuania, Luxembourg, Malaysia, Mali, Malta, Mauritania, Moldova, Mongolia, Mozambique, Namibia, Norway, New Zealand, Oman, Uganda, Uzbekistan, Pakistan, Panama, Papua New Guinea, Paraguay, the Netherlands, the Philippines, Poland, Portugal, Qatar, Syria, Kyrgyzstan, Slovakia, Romania, the United Kingdom, the Russian Federation, Senegal, Sierra Leone, Singapore, Slovenia, Sri Lanka, South Africa, Sweden, Switzerland, Swaziland,

Tanzania, Chad, Thailand, Togo, Tonga, Trinidad and Tobago, Tunisia, Turkey, Ukraine, Viet Nam, Yemen, Yugoslavia, Zambia, and Zimbabwe.     (WRC-03)

MOD	COM4/272/28

5.237	Additional allocation:  in the Congo, Eritrea, Ethiopia, Gambia, Guinea, Libya, Malawi, Mali, Sierra Leone, Somali, Chad and Zimbabwe, the band 174-223 MHz is also allocated to the fixed and mobile services on a secondary basis.     (WRC-03)

MOD	COM4/272/29

5.262	Additional allocation:  in Saudi Arabia, Armenia, Azerbaijan, Bahrain, Belarus, Bosnia and Herzegovina, Botswana, Bulgaria, Colombia, Costa Rica, Cuba, Egypt, the United Arab Emirates, Ecuador, Georgia, Hungary, Iran (Islamic Republic of), Iraq, Israel, Jordan, Kazakstan, Kuwait, Liberia, Malaysia, Moldova, Uzbekistan, Pakistan, the Philippines, Qatar, Syria, Kyrgyzstan, Romania, the Russian Federation, Singapore, Somalia, Tajikistan, Turkmenistan, Ukraine and Yugoslavia, the band 400.05-401 MHz is also allocated to the fixed and mobile services on a primary basis.     (WRC-03)

MOD	COM4/272/30

5.271	Additional allocation:  in Azerbaijan, Belarus, China, India, Latvia, Lithuania, Kyrgyzstan and Turkmenistan, the band 420-460 MHz is also allocated to the aeronautical radionavigation service (radio altimeters) on a secondary basis.     (WRC-03)

MOD	COM4/272/31

5.273	Different category of service:  in Libya, the allocation of the bands 430-432 MHz and 438-440 MHz to the radiolocation service is on a secondary basis (see No. 5.32).

MOD	COM4/272/32

5.277	Additional allocation:  in Angola, Armenia, Azerbaijan, Belarus, Cameroon, Congo, Djibouti, Georgia, Hungary, Israel, Kazakstan, Mali, Moldova, Mongolia, Uzbekistan, Poland, Kyrgyzstan, Slovakia, the Czech Rep., Romania, the Russian Federation, Rwanda, Tajikistan, Chad, Turkmenistan and Ukraine, the band 430-440 MHz is also allocated to the fixed service on a primary basis.     (WRC-03)

MOD	COM4/272/33

5.294	Additional allocation:  in Burundi, Cameroon, the Congo, Côte d’Ivoire, Ethiopia, Israel, Kenya, Lebanon, Libya, Malawi, Sudan, Syria, Chad and Yemen, the band 470-582 MHz is also allocated to the fixed service on a secondary basis.

MOD	COM4/272/34

5.296	Additional allocation:  in Germany, Austria, Belgium, Côte d’Ivoire, Denmark, Spain, Finland, France, Ireland, Israel, Italy, Libya, Lithuania, Malta, Morocco, Monaco, Norway, the Netherlands, Portugal, Syria, the United Kingdom, Sweden, Switzerland, Swaziland and Tunisia, the band 470-790 MHz is also allocated on a secondary basis to the land mobile service, intended for applications ancillary to broadcasting. Stations of the land mobile service in the countries listed in this footnote shall not cause harmful interference to existing or planned stations operating in accordance with the Table in countries other than those listed in this footnote.     (WRC-03)

MOD	COM4/272/35

5.312	Additional allocation:  in Armenia, Azerbaijan, Belarus, Bulgaria, Georgia, Hungary, Kazakstan, Moldova, Mongolia, Uzbekistan, Poland, Kyrgyzstan, Slovakia, the Czech Rep., Romania, Russian Federation, Tajikistan, Turkmenistan and Ukraine, the band 645-862 MHz is also allocated to the aeronautical radionavigation service on a primary basis.     (WRC-03)

MOD	COM4/272/36

5.316	Additional allocation:  in Germany, Saudi Arabia, Bosnia and Herzegovina, Burkina Faso, Cameroon, Côte d’Ivoire, Croatia, Denmark, Egypt, Finland, Greece, Israel, Jordan, Kenya, The Former Yugoslav Republic of Macedonia, Libya, Liechtenstein, Mali, Monaco, Norway, the Netherlands, Portugal, the United Kingdom, Syria, Sweden, Switzerland and Yugoslavia, the band 790-830 MHz, and in these same countries and in Spain, France, Gabon and Malta, the band 830-862 MHz, are also allocated to the mobile, except aeronautical mobile, service on a primary basis. However, stations of the mobile service in the countries mentioned in connection with each band referred to in this footnote shall not cause harmful interference to, or claim protection from, stations of services operating in accordance with the Table in countries other than those mentioned in connection with the band.     (WRC-03)

MOD	COM4/272/37

5.323	Additional allocation:  in Armenia, Azerbaijan, Belarus, Bulgaria, Hungary, Kazakstan, Moldova, Mongolia, Uzbekistan, Poland, Kyrgyzstan, Slovakia, the Czech Rep., Romania, Russian Federation, Tajikistan, Turkmenistan and Ukraine, the band 862-960 MHz is also allocated to the aeronautical radionavigation service on a primary basis. Such use is subject to agreement obtained under No. 9.21 with administrations concerned and limited to ground-based radiobeacons in operation on 27 October 1997 until the end of their lifetime.     (WRC-03)

MOD	COM4/272/38

5.330	Additional allocation:  in Angola, Saudi Arabia, Bahrain, Bangladesh, Cameroon, China, the United Arab Emirates, Eritrea, Ethiopia, Guyana, India, Indonesia, Iran (Islamic Republic of), Iraq, Israel, Japan, Jordan, Kuwait, Lebanon, Libya, Mozambique, Nepal, Pakistan, the Philippines, Qatar, Syria, Somalia, Sudan, Chad, Togo and Yemen, the band 1 215-1 300 MHz is also allocated to the fixed and mobile services on a primary basis.     (WRC-03)

MOD	COM4/272/39

5.331	Additional allocation:  in Algeria, Germany, Austria, Bahrain, Belgium, Benin, Bosnia and Herzegovina, Burkina Faso, Burundi, Cameroon, China, Croatia, Denmark, the United Arab Emirates, France, Greece, India, Iran (Islamic Republic of), Iraq, Kenya, The Former Yugoslav Republic of Macedonia, Liechtenstein, Luxembourg, Mali, Mauritania, Norway, Oman, the Netherlands, Portugal, Qatar, Slovenia, Somalia, Sudan, Sri Lanka, Sweden, Switzerland, Turkey and Yugoslavia, the band 1 215-1 300 MHz is also allocated to the radionavigation service on a primary basis.     (WRC-03)

MOD	COM4/272/40

5.338	In Azerbaijan, Mongolia, Kyrgyzstan, Slovakia, the Czech Rep., Romania and Turkmenistan, existing installations of the radionavigation service may continue to operate in the band 1 350-1 400 MHz.     (WRC-03)

MOD	COM4/272/41

5.340	All emissions are prohibited in the following bands:

1 400-1 427 MHz,

2 690-2 700 MHz,	except those provided for by No. 5.422,

10.68-10.7 GHz,		except those provided for by No. 5.483,

15.35-15.4 GHz,		except those provided for by No. 5.511,

23.6-24 GHz,

31.3-31.5 GHz,

31.5-31.8 GHz,		in Region 2,

48.94-49.04 GHz,		from airborne stations,

50.2-50.4 GHz2,		except those provided for by No. 5.555A,

52.6-54.25 GHz,

86-92 GHz,

100-102 GHz,

109.5-111.8 GHz,

114.25-116 GHz,

148.5-151.5 GHz,

164-167 GHz,

182-185 GHz,

190-191.8 GHz,

200-209 GHz,

226-231.5 GHz,

250-252 GHz.			(WRC-03)

MOD	COM4/272/42

5.347	Different category of service:  in Bangladesh, Bosnia and Herzegovina, Botswana, Bulgaria, Burkina Faso, Cuba, Denmark, Egypt, Greece, Ireland, Italy, Mozambique, Portugal, Sri Lanka, Swaziland, Yemen, Yugoslavia and Zimbabwe, the allocation of the band 1 452-1 492 MHz to the broadcasting-satellite service and the broadcasting service is on a secondary basis until 1 April 2007.     (WRC-03)

MOD	COM4/272/43

5.355	Additional allocation:  in Bahrain, Bangladesh, Congo, Egypt, Eritrea, Iraq, Israel, Kuwait, Lebanon, Malta, Qatar, Syria, Somalia, Sudan, Chad, Togo and Yemen, the bands 1 540-1 559 MHz, 1 610-1 645.5 MHz and 1 646.5-1 660 MHz are also allocated to the fixed service on a secondary basis.     (WRC-03)

MOD	COM4/272/44

5.359	Additional allocation:  in Germany, Saudi Arabia, Armenia, Austria, Azerbaijan, Belarus, Benin, Bosnia and Herzegovina, Bulgaria, Cameroon, Spain, France, Gabon, Georgia, Greece, Guinea, Guinea-Bissau, Hungary, Jordan, Kazakstan, Kuwait, Lebanon, Libya, Lithuania, Mauritania, Moldova, Mongolia, Uganda, Uzbekistan, Pakistan, Poland, Syria, Kyrgyzstan, the Dem. People’s Rep. of Korea, Romania, the Russian Federation, Swaziland, Tajikistan, Tanzania, Tunisia, Turkmenistan and Ukraine, the bands 1 550-1 559 MHz, 1 610-1 645.5 MHz and 1 646.5-1 660 MHz are also allocated to the fixed service on a primary basis. Administrations are urged to make all practicable efforts to avoid the implementation of new fixed-service stations in these bands.     (WRC-03)

MOD	COM4/272/45

5.362B	Additional allocation:  The band 1 559-1 610 MHz is also allocated to the fixed service on a primary basis until 1 January 2005 in Germany, Armenia, Azerbaijan, Belarus, Benin, Bosnia and Herzegovina, Bulgaria, Spain, France, Gabon, Georgia, Greece, Guinea, Guinea-Bissau, Hungary, Kazakstan, Lithuania, Moldova, Mongolia, Nigeria, Uganda, Uzbekistan, Pakistan, Poland, Kyrgyzstan, the Dem. People’s Rep. of Korea, Romania, the Russian Federation, Senegal, Swaziland, Tajikistan, Tanzania, Turkmenistan and Ukraine, and until 1 January 2010 in Saudi Arabia, Cameroon, Jordan, Kuwait, Lebanon, Libya, Mali, Mauritania, Syria and Tunisia. After these dates, the fixed service may continue to operate on a secondary basis until 1 January 2015, at which time this allocation shall no longer be valid. Administrations are urged to take all practicable steps to protect the radionavigation-satellite service and the aeronautical radionavigation service and not authorize new frequency assignments to fixed-service systems in this band.     (WRC-03)

MOD	COM4/272/46

5.369	Different category of service:  in Angola, Australia, Burundi, China, Eritrea, Ethiopia, India, Iran (Islamic Republic of), Israel, Lebanon, Liberia, Libya, Madagascar, Mali, Pakistan, Papua New Guinea, Dem. Rep. of the Congo, Syria, Sudan, Swaziland, Togo and Zambia, the allocation of the band 1 610-1 626.5 MHz to the radiodetermination-satellite service (Earth-to-space) is on a primary basis (see No. 5.33), subject to agreement obtained under No. 9.21 from countries not listed in this provision.     (WRC-03)

MOD	COM4/272/47

5.381	Additional allocation:  in Afghanistan, Costa Rica, Cuba, India, Iran (Islamic Republic of) and Pakistan, the band 1 690-1 700 MHz is also allocated to the fixed and mobile, except aeronautical mobile, services on a primary basis.     (WRC-03)

MOD	COM4/272/48

5.382	Different category of service:  in Saudi Arabia, Armenia, Azerbaijan, Bahrain, Belarus, Bosnia and Herzegovina, Bulgaria, the Congo, Egypt, the United Arab Emirates, Eritrea, Ethiopia, Guinea, Hungary, Iraq, Israel, Jordan, Kazakstan, Kuwait, the Former Yugoslav Republic of Macedonia, Lebanon, Mauritania, Moldova, Mongolia, Oman, Uzbekistan, Poland, Qatar, Syria, Kyrgyzstan, Romania, Russian Federation, Somalia, Tajikistan, Tanzania, Turkmenistan, Ukraine, Yemen and Yugoslavia, the allocation of the band 1 690-1 700 MHz to the fixed and mobile, except aeronautical mobile, services is on a primary basis (see No. 5.33), and in the Dem. People’s Rep. of Korea, the allocation of the band 1 690-1 700 MHz to the fixed service is on a primary basis (see No. 5.33) and to the mobile, except aeronautical mobile, service on a secondary basis.     (WRC-03)

MOD	COM4/272/49

5.386	Additional allocation:  the band 1 750-1 850 MHz is also allocated to the space operation (Earth-to-space) and space research (Earth-to-space) services in Region 2, in Australia, Guam, India, Indonesia and Japan on a primary basis, subject to agreement obtained under No. 9.21, having particular regard to troposcatter systems.

MOD	COM4/272/50

5.387	Additional allocation:  in Azerbaijan, Belarus, Georgia, Kazakstan, Mongolia, Kyrgyzstan, Slovakia, Romania, Tajikistan and Turkmenistan, the band 1 770-1 790 MHz is also allocated to the meteorological-satellite service on a primary basis, subject to agreement obtained under No. 9.21.      (WRC-03)

SUP	COM4/272/51

5.389D

MOD	COM4/272/52

5.400	Different category of service:  in Angola, Australia, Bangladesh, Burundi, China, Eritrea, Ethiopia, India, Iran (Islamic Republic of), Lebanon, Liberia, Libya, Madagascar, Mali, Pakistan, Papua New Guinea, Dem. Rep. of the Congo, Syria, Sudan, Swaziland, Togo and Zambia, the allocation of the band 2 483.5-2 500 MHz to the radiodetermination-satellite service (space-to-Earth) is on a primary basis (see No. 5.33), subject to agreement obtained under No. 9.21 from countries not listed in this provision.     (WRC-03)

MOD	COM4/272/53

5.418	Additional allocation:  in Korea (Rep. of), India, Japan, Pakistan and Thailand, the band 2 535-2 655 MHz is also allocated to the broadcasting-satellite service (sound) and complementary terrestrial broadcasting service on a primary basis. Such use is limited to digital audio broadcasting and is subject to the provisions of Resolution 528 (WARC-92). The provisions of No. 5.416 and Table 21-4 of Article 21, do not apply to this additional allocation. Use of non-geostationary-satellite systems in the broadcasting-satellite service (sound) is subject to Resolution 539 (WRC-2000).     (WRC-03)

SUP	COM4/272/54

5.421

MOD	COM4/272/55

5.422	Additional allocation:  in Saudi Arabia, Armenia, Azerbaijan, Bahrain, Belarus, Bosnia and Herzegovina, Brunei Darussalam, Congo, Côte d’Ivoire, Cuba, Egypt, the United Arab Emirates, Eritrea, Ethiopia, Gabon, Georgia, Guinea, Guinea-Bissau, Iran (Islamic Republic of), Iraq, Israel, Jordan, Lebanon, Mauritania, Moldova, Mongolia, Nigeria, Oman, Uzbekistan, Pakistan, the Philippines, Qatar, Syria, Kyrgyzstan, the Dem. Rep. of the Congo, Romania, the Russian Federation, Somalia, Tajikistan, Tunisia, Turkmenistan, Ukraine, Yemen and Yugoslavia, the band 2 690-2 700 MHz is also allocated to the fixed and mobile, except aeronautical mobile, services on a primary basis. Such use is limited to equipment in operation by 1 January 1985.     (WRC-03)

MOD	COM4/272/56

5.428	Additional allocation:  in Azerbaijan, Cuba, Mongolia, Kyrgyzstan, Romania and Turkmenistan, the band 3 100-3 300 MHz is also allocated to the radionavigation service on a primary basis.     (WRC-03)

MOD	COM4/272/57

5.429	Additional allocation:  in Saudi Arabia, Bahrain, Bangladesh, Brunei Darussalam, China, the Congo, Korea (Rep. of), the United Arab Emirates, India, Indonesia, Iran (Islamic Republic of), Iraq, Israel, Japan, Jordan, Kenya, Kuwait, Lebanon, Libya, Malaysia, Oman, Pakistan, Qatar, Syria, Dem. People’s Rep. of Korea and Yemen, the band 3 300-3 400 MHz is also allocated to the fixed and mobile services on a primary basis. The countries bordering the Mediterranean shall not claim protection for their fixed and mobile services from the radiolocation service.     (WRC-03)

MOD	COM4/272/58

5.430	Additional allocation:  in Azerbaijan, Cuba, Mongolia, Kyrgyzstan, Romania and Turkmenistan, the band 3 300-3 400 MHz is also allocated to the radionavigation service on a primary basis.     (WRC-03)

MOD	COM4/272/59

5.431	Additional allocation:  in Germany, Israel and the United Kingdom, the band 3 400-3 475 MHz is also allocated to the amateur service on a secondary basis.

MOD	COM4/272/60

5.447	Additional allocation:  in Germany, Austria, Belgium, Denmark, Spain, Estonia, Finland, France, Greece, Israel, Italy, Japan, Lebanon, Liechtenstein, Lithuania, Luxembourg, Malta, Norway, Pakistan, the Netherlands, Portugal, Syria, the United Kingdom, Sweden, Switzerland and Tunisia, the band 5 150-5 250 MHz is also allocated to the mobile service, on a primary basis, subject to agreement obtained under No. 9.21.     (WRC-03)

MOD	COM4/272/61

5.448	Additional allocation:  in Azerbaijan, Libya, Mongolia, Kyrgyzstan, Slovakia, Romania and Turkmenistan, the band 5 250-5 350 MHz is also allocated to the radionavigation service on a primary basis.     (WRC-03)

MOD	COM4/272/62

5.450	Additional allocation:  in Austria, Azerbaijan, Iran (Islamic Republic of), Mongolia, Kyrgyzstan, Romania, Turkmenistan and Ukraine, the band 5 470-5 650 MHz is also allocated to the aeronautical radionavigation service on a primary basis.     (WRC-03)

MOD	COM4/272/63

5.453	Additional allocation:  in Saudi Arabia, Bahrain, Bangladesh, Brunei Darussalam, Cameroon, China, Congo, Korea (Rep. of), Côte d’Ivoire, Egypt, the United Arab Emirates, Gabon, Guinea, India, Indonesia, Iran (Islamic Republic of), Iraq, Israel, Japan, Jordan, Kenya, Kuwait, Lebanon, Libya, Madagascar, Malaysia, Nigeria, Oman, Pakistan, the Philippines, Qatar, Syria, the Dem. People’s Rep. of Korea, Singapore, Swaziland, Tanzania, Chad and Yemen, the band 5 650-5 850 MHz is also allocated to the fixed and mobile services on a primary basis.     (WRC-03)

MOD	COM4/272/64

5.454	Different category of service:  in Azerbaijan, Georgia, Mongolia, Uzbekistan, Kyrgyzstan, the Russian Federation, Tajikistan and Turkmenistan, the allocation of the band 5 670-5 725 MHz to the space research service is on a primary basis (see No. 5.33).     (WRC-03)

MOD	COM4/272/65

5.455	Additional allocation:  in Armenia, Azerbaijan, Belarus, Cuba, Georgia, Hungary, Kazakstan, Latvia, Moldova, Mongolia, Uzbekistan, Kyrgyzstan, Russian Federation, Tajikistan, Turkmenistan and Ukraine, the band 5 670-5 850 MHz is also allocated to the fixed service on a primary basis.

MOD	COM4/272/66

5.456	Additional allocation:  in Cameroon, the band 5 755-5 850 MHz is also allocated to the fixed service on a primary basis.

MOD	COM4/272/67

5.466	Different category of service:  in Israel, Singapore and Sri Lanka, the allocation of the band 8 400-8 500 MHz to the space research service is on a secondary basis (see No. 5.32).     (WRC-03)

SUP	COM4/272/68

5.467

MOD	COM4/272/69

5.468	Additional allocation:  in Saudi Arabia, Bahrain, Bangladesh, Brunei Darussalam, Burundi, Cameroon, China, the Congo, Costa Rica, Egypt, the United Arab Emirates, Gabon, Guyana, Indonesia, Iran (Islamic Republic of), Iraq, Jamaica, Jordan, Kenya, Kuwait, Lebanon, Libya, Malaysia, Mali, Morocco, Mauritania, Nepal, Nigeria, Oman, Pakistan, Qatar, Syria, Dem. People’s Rep. of Korea, Senegal, Singapore, Somalia, Swaziland, Tanzania, Chad, Togo, Tunisia and Yemen, the band 8 500-8 750 MHz is also allocated to the fixed and mobile services on a primary basis.     (WRC-03)

MOD	COM4/272/70

5.469	Additional allocation:  in Armenia, Azerbaijan, Belarus, Georgia, Hungary, Lithuania, Moldova, Mongolia, Uzbekistan, Poland, Kyrgyzstan, the Czech Rep., Romania, the Russian Federation, Tajikistan, Turkmenistan and Ukraine, the band 8 500-8 750 MHz is also allocated to the land mobile and radionavigation services on a primary basis.     (WRC-03)

MOD	COM4/272/71

5.473	Additional allocation:  in Armenia, Austria, Azerbaijan, Belarus, Bulgaria, Cuba, Georgia, Hungary, Moldova, Mongolia, Uzbekistan, Poland, Kyrgyzstan, Romania, the Russian Federation, Tajikistan, Turkmenistan and Ukraine, the bands 8 850-9 000 MHz and 9 200-9 300 MHz are also allocated to the radionavigation service on a primary basis.     (WRC-03)

MOD	COM4/272/72

5.477	Different category of service:  in Algeria, Saudi Arabia, Bahrain, Bangladesh, Brunei Darussalam, Cameroon, Egypt, the United Arab Emirates, Eritrea, Ethiopia, Guyana, India, Indonesia, Iran (Islamic Republic of), Iraq, Jamaica, Japan, Jordan, Kuwait, Lebanon, Liberia, Malaysia, Nigeria, Oman, Pakistan, Qatar, the Dem. People's Rep. of Korea, Singapore, Somalia, Sudan, Trinidad and Tobago, and Yemen, the allocation of the band 9 800-10 000 MHz to the fixed service is on a primary basis (see No. 5.33).     (WRC-03)

MOD	COM4/272/73

5.478	Additional allocation:  in Azerbaijan, Bulgaria, Mongolia, Kyrgyzstan, Romania, Turkmenistan and Ukraine, the band 9 800-10 000 MHz is also allocated to the radionavigation service on a primary basis.     (WRC-03)

MOD	COM4/272/74

5.481	Additional allocation:  in Germany, Angola, Brazil, China, Costa Rica, Côte d’Ivoire, El Salvador, Ecuador, Spain, Guatemala, Hungary, Japan, Kenya, Morocco, Nigeria, Oman, Uzbekistan, Paraguay, Peru, the Dem. People’s Rep. of Korea, Tanzania, Thailand and Uruguay, the band 10.45-10.5 GHz is also allocated to the fixed and mobile services on a primary basis.     (WRC-03)

MOD	COM4/272/75

5.482	In the band 10.6-10.68 GHz, stations of the fixed and mobile, except aeronautical mobile, services shall be limited to a maximum equivalent isotropically radiated power of 40 dBW and the power delivered to the antenna shall not exceed –3 dBW. These limits may be exceeded subject to agreement obtained under No. 9.21. However, in Saudi Arabia, Armenia, Azerbaijan, Bahrain, Bangladesh, Belarus, China, the United Arab Emirates, Georgia, India, Indonesia, Iran (Islamic Republic of), Iraq, Japan, Kazakstan, Kuwait, Latvia, Lebanon, Moldova, Nigeria, Pakistan, the Philippines, Qatar, Syria, Tajikistan and Turkmenistan, the restrictions on the fixed and mobile, except aeronautical mobile, services are not applicable.

MOD	COM4/272/76

5.483	Additional allocation:  in Saudi Arabia, Armenia, Azerbaijan, Bahrain, Belarus, Bosnia and Herzegovina, China, Colombia, Korea (Rep. of), Costa Rica, Egypt, the United Arab Emirates, Georgia, Iran (Islamic Republic of), Iraq, Israel, Jordan, Kazakstan, Kuwait, Lebanon, Mongolia, Uzbekistan, Qatar, Kyrgyzstan, the Dem. People’s Rep. of Korea, Romania, Tajikistan, Turkmenistan, Yemen and Yugoslavia, the band 10.68-10.7 GHz is also allocated to the fixed and mobile, except aeronautical mobile, services on a primary basis. Such use is limited to equipment in operation by 1 January 1985.     (WRC-03)

MOD	COM4/272/77

5.494	Additional allocation:  in Algeria, Angola, Saudi Arabia, Bahrain, Cameroon, the Central African Rep., the Congo, Côte d'Ivoire, Egypt, the United Arab Emirates, Eritrea, Ethiopia, Gabon, Ghana, Guinea, Iraq, Israel, Jordan, Kuwait, Lebanon, Libya, Madagascar, Mali, Morocco, Mongolia, Nigeria, Qatar, Dem. Rep. of the Congo, Syria, Somalia, Sudan, Chad, Togo and Yemen, the band 12.5-12.75 GHz is also allocated to the fixed and mobile, except aeronautical mobile, services on a primary basis.     (WRC-03)

MOD	COM4/272/78

5.495	Additional allocation:  in Bosnia and Herzegovina, Croatia, France, Greece, Liechtenstein, Monaco, Uganda, Portugal, Romania, Slovenia, Switzerland, Tanzania, Tunisia and Yugoslavia, the band 12.5-12.75 GHz is also allocated to the fixed and mobile, except aeronautical mobile, services on a secondary basis.     (WRC-03)

MOD	COM4/272/79

5.500	Additional allocation:  in Algeria, Angola, Saudi Arabia, Bahrain, Brunei Darussalam, Cameroon, Egypt, the United Arab Emirates, Gabon, Indonesia, Iran (Islamic Republic of), Iraq, Israel, Jordan, Kuwait, Lebanon, Madagascar, Malaysia, Mali, Malta, Morocco, Mauritania, Nigeria, Pakistan, Qatar, Syria, Singapore, Sudan, Chad and Tunisia, the band 13.4-14 GHz is also allocated to the fixed and mobile services on a primary basis.     (WRC-03)

MOD	COM4/272/80

5.501	Additional allocation:  in Azerbaijan, Hungary, Japan, Mongolia, Kyrgyzstan, Romania, the United Kingdom and Turkmenistan, the band 13.4-14 GHz is also allocated to the radionavigation service on a primary basis.     (WRC-03)

MOD	COM4/272/81

5.505	Additional allocation:  in Algeria, Angola, Saudi Arabia, Bahrain, Bangladesh, Botswana, Brunei Darussalam, Cameroon, China, Congo, Korea (Rep. of), Egypt, the United Arab Emirates, Gabon, Guatemala, Guinea, India, Indonesia, Iran (Islamic Republic of), Iraq, Israel, Japan, Jordan, Kuwait, Lesotho, Lebanon, Malaysia, Mali, Morocco, Mauritania, Oman, Pakistan, the Philippines, Qatar, Syria, the Dem. People's Rep. of Korea, Singapore, Somalia, Sudan, Swaziland, Tanzania, Chad and Yemen, the band 14-14.3 GHz is also allocated to the fixed service on a primary basis.     (WRC-03)

MOD	COM4/272/82

5.508	Additional allocation:  in Germany, Bosnia and Herzegovina, France, Italy, The Former Yugoslav Republic of Macedonia, Libya, the United Kingdom, Slovenia and Yugoslavia, the band 14.25-14.3 GHz is also allocated to the fixed service on a primary basis.     (WRC-03)

MOD	COM4/272/83

5.512	Additional allocation:  in Algeria, Angola, Saudi Arabia, Austria, Bahrain, Bangladesh, Bosnia and Herzegovina, Brunei Darussalam, Cameroon, the Congo, Costa Rica, Egypt, El Salvador, the United Arab Emirates, Finland, Guatemala, India, Indonesia, Iran (Islamic Republic of), Jordan, Kenya, Kuwait, Libya, Malaysia, Morocco, Mozambique, Nepal, Nicaragua, Oman, Pakistan, Qatar, Singapore, Slovenia, Somalia, Sudan, Swaziland, Tanzania, Chad, Yemen and Yugoslavia, the band 15.7-17.3 GHz is also allocated to the fixed and mobile services on a primary basis.     (WRC-03)

MOD	COM4/272/84

5.514	Additional allocation:  in Algeria, Angola, Saudi Arabia, Austria, Bahrain, Bangladesh, Bosnia and Herzegovina, Cameroon, Costa Rica, El Salvador, the United Arab Emirates, Finland, Guatemala, Honduras, India, Iran (Islamic Republic of), Iraq, Israel, Japan, Jordan, Kuwait, Libya, Nepal, Nicaragua, Oman, Pakistan, Qatar, Slovenia, Sudan and Yugoslavia, the band 17.3-17.7 GHz is also allocated to the fixed and mobile services on a secondary basis. The power limits given in Nos. 21.3 and 21.5 shall apply.     (WRC-03)

MOD	COM4/272/85

5.521	Alternative allocation:  in Germany, Denmark, the United Arab Emirates, Greece and the band 18.1-18.4 GHz is allocated to the fixed, fixed-satellite (space-to-Earth) and

mobile services on a primary basis (see No. 5.33). The provisions of No. 5.519 also apply.     (WRC-03)

SUP	COM4/272/86

5.534

MOD	COM4/272/87

5.545	Different category of service:  in Armenia, Azerbaijan, Georgia, Mongolia, Kyrgyzstan, Tajikistan and Turkmenistan, the allocation of the band 31-31.3 GHz to the space research service is on a primary basis (see No. 5.33).     (WRC-03)

MOD	COM4/272/88

5.546	Different category of service:  in Saudi Arabia, Armenia, Azerbaijan, Belarus, Egypt, the United Arab Emirates, Spain, Estonia, Finland, Georgia, Hungary, Iran (Islamic Republic of), Israel, Jordan, Latvia, Lebanon, Moldova, Mongolia, Uzbekistan, Poland, Syria, Kyrgyzstan, Romania, the United Kingdom, the Russian Federation, South Africa, Tajikistan, Turkmenistan and Turkey, the allocation of the band 31.5-31.8 GHz to the fixed and mobile, except aeronautical mobile, services is on a primary basis (see No. 5.33).     (WRC-03)

MOD	COM4/272/89

5.549	Additional allocation:  in Saudi Arabia, Bahrain, Bangladesh, Egypt, the United Arab Emirates, Gabon, Indonesia, Iran (Islamic Republic of), Iraq, Israel, Jordan, Kuwait, Lebanon, Libya, Malaysia, Mali, Malta, Morocco, Mauritania, Nepal, Nigeria, Oman, Pakistan, the Philippines, Qatar, Dem. Rep. of the Congo, Syria, Singapore, Somalia, Sudan, Sri Lanka, Togo, Tunisia and Yemen, the band 33.4-36 GHz is also allocated to the fixed and mobile services on a primary basis.     (WRC-03)

MOD	COM4/272/90

5.550	Different category of service:  in Armenia, Azerbaijan, Belarus, Georgia, Mongolia, Uzbekistan, Kyrgyzstan, the Russian Federation, Tajikistan and Turkmenistan, the allocation of the band 34.7-35.2 GHz to the space research service is on a primary basis (see No. 5.33).     (WRC-03)

SUP	COM4/272/91

5.555A

SUP	COM4/272/92

5.563

ARTICLE  11

Notification and recording of frequency assignments1, 2, 3, 4

Section I  –  Notification

ADD	COM4/272/93

11.21B	f)	any terrestrial station in the bands referred to in a footnote having reference to No. 9.21, if it pertains to a service which is subject to the application of the procedure for seeking agreement under No. 9.21;

(MOD)	COM4/272/94

11.22	g)	earth stations whose coordination area includes the territory of another administration, or where the earth station is located within the coordination area of an earth station operating in the opposite direction of transmission;6, 7

(MOD)	COM4/272/95

11.23	h)	earth stations whose interference potential is greater than that of a coordinated typical earth station.6

Section II  –  Examination of notices and recording of frequency assignments
in the Master Register

ADD	COM4/272/96

11.50		The Bureau shall review periodically each section of the Master Register with the aim of improving and maintaining its accuracy, with a particular emphasis on the review of the findings so as to adjust them to the changing allocation situation after each conference.

ARTICLE  21

Terrestrial and space services sharing frequency bands above 1 GHz

Section II  –  Power limits for terrestrial stations

MOD	COM4/272/97

TABLE  21-2     (WRC-2000)

| Frequency band | Service | Limit as specified in Nos. |
| --- | --- | --- |
| 1 610-1 645.5 MHz (No. 5.359) 1 646.5-1 660 MHz (No. 5.359) 1 675-1 690 MHz (Region 2) 1 690-1 700 MHz (Region 2 countries listed in No. 5.381) 1 700-1 710 MHz (Region 2) 1 980-2 010 MHz 2 010-2 025 MHz (Region 2) 2 025-2 110 MHz 2 200-2 290 MHz 2 655-2 670 MHz5 (Regions 2 and 3) 2 670-2 690 MHz 5 670-5 725 MHz (Nos. 5.453 and 5.455) 5 725-5 755 MHz5 (Region 1 countries listed in Nos. 5.451, 5.453 and 5.455) | Fixed-satellite Meteorological-satellite Space research Space operation Earth exploration-satellite Mobile-satellite | 21.2, 21.3, 21.4 and 21.5 |
| 5 755-5 850 MHz5 (Region 1 countries listed in Nos. 5.451, 5.453, 5.455 and 5.456) 5 850-7 075 MHz 7 900-8 400 MHz 8 400-8 500 MHz |  |  |

MOD	COM4/272/98

TABLE  21-2 (end)

| Frequency band | Service | Limit as specified in Nos. |
| --- | --- | --- |
| 10.7-11.7 GHz5 (Region 1) 12.5-12.75 GHz5 (Nos. 5.494 and 5.496) 12.7-12.75 GHz5 (Region 2) 12.75-13.25 GHz 13.75-14 GHz (Nos. 5.499 and 5.500) 14.0-14.25 GHz (No. 5.505) 14.25-14.3 GHz (Nos. 5.505, 5.508 and 5.509) 14.3-14.4 GHz5 (Regions 1 and 3) 14.4-14.5 GHz 14.5-14.8 GHz | Fixed-satellite | 21.2, 21.3 and 21.5 |
| 17.3-17.7 GHz (No. 5.514) 17.7-18.4 GHz 18.6-18.8 GHz 19.3-19.7 GHz 22.55-23.55 GHz 24.45-24.75 GHz (Regions 1 and 3) 24.75-25.25 GHz (Region 3) 25.25-29.5 GHz 29.5-31 GHz (No. 5.542) | Fixed-satellite Earth exploration-satellite Space research Inter-satellite | 21.2, 21.3, 21.5 and 21.5A |

ARTICLE  22

Space services1

Section II  –  Control of interference to geostationary-satellite systems

MOD	COM4/272/99

22.5I		6)	An administration operating a non-geostationary-satellite system in the fixed-satellite service which is in compliance with the limits in Nos. 22.5C, 22.5D and 22.5F shall be considered as having fulfilled its obligations under No. 22.2 with respect to any geostationary-satellite network, irrespective of the dates of receipt by the Bureau of the complete coordination or notification information, as appropriate, for the non-geostationary-satellite system and the geostationary-satellite network, provided that the epfd radiated by the non-geostationary-satellite system in the fixed-satellite service into any operating geostationary fixed-satellite service earth station does not exceed the operational and additional operational limits given in Tables 22-4A, 22-4A1, 22-4B and 22-4C, when the diameter of the earth station antenna is equal to the values given in Table 22-4A, 22-4A1 or 22-4C, or the gain of the earth station is equal to or greater than the values given in Table 22-4B for the corresponding orbital inclination of the geostationary fixed-satellite service satellite. Except as otherwise agreed between concerned administrations, an administration operating a non-geostationary-satellite system in the fixed-satellite service that is subject to the limits in Nos. 22.5C, 22.5D and 22.5F and which radiates epfd into any operating geostationary fixed-satellite service earth station at levels in excess of the operational or additional operational limits given in Tables 22-4A, 22-4A1, 22-4B and 22-4C, when the diameter of the earth station antenna is equal to the values given in Table 22-4A, 22-4A1 or 22-4C, or the gain of the earth station is equal to or greater than the values given in Table 22-4B for the corresponding orbital inclination of the geostationary fixed-satellite service satellite, shall be considered to be in violation of its obligations under No. 22.2, and the provisions of Article 15 (Section V) apply. In addition, administrations are encouraged to use the relevant ITU-R Recommendations to determine whether such a violation has occurred.     (WRC-03)

ARTICLE  24

Fixed service

Section II  –  Frequencies for the international exchange of police information

SUP	COM4/272/100

24.3

SUP	COM4/272/101

24.4

Section III  –  Frequencies for the international exchange of synoptic
meteorological information

SUP	COM4/272/102

24.5

SUP	COM4/272/103

24.6

APPENDIX  2

Table of transmitter frequency tolerances

Notes in the table of transmitter frequency tolerances

SUP	COM4/272/104

6	In countries covered by the North American Regional Broadcasting Agreement (NARBA) the tolerance of 20 Hz may continue to be applied.

NOC	COM4/272/105

RESOLUTION  26  (Rev.WRC-97)

Footnotes to the Table of Frequency Allocations in
Article 5 of the Radio Regulations

NOC	COM4/272/106

RESOLUTION  76  (WRC-2000)

Protection of geostationary fixed-satellite service and geostationary broadcasting-satellite service networks from the maximum
aggregate equivalent power flux-density produced by 
multiple non-geostationary fixed-satellite service 
systems in frequency bands where equivalent 
power flux-density limits have been adopted

SUP	COM4/272/107

RESOLUTION  137  (WRC-2000)

Further studies on the sharing conditions between geostationary fixed-satellite service networks and non-geostationary fixed-satellite service systems 
and between non-geostationary fixed-satellite service systems

NOC	COM4/272/108

RECOMMENDATION  705

Criteria to be applied for frequency sharing between the
broadcasting-satellite service and the terrestrial broadcasting
service in the band 620-790 MHz1

SUP	COM4/272/109

RECOMMENDATION  709

Relating to sharing frequency bands between the aeronautical
mobile service and the inter-satellite service1

SUP	COM4/272/110

RECOMMENDATION  710

Relating to the use of airborne radars in the frequency bands shared
between the inter-satellite service and the radiolocation service1

____________

1 	A.22.1	In applying the provisions of this Article, the level of accepted interference (see No. 1.168) shall be fixed by agreement between the administrations concerned, using the relevant ITU-R Recommendations as a guide.

1	WRC-97 made editorial amendments to this Recommendation.