| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Corrigendum 1 to Document 280-E |
|  |  |  | 26 June 2003 |
|  |  |  | Original: English |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | COMMITTEE 4 |
| --- | --- |
| Chairman, Working Group 4B |  |
| SixTH REPORT FROM THE CHAIRMAN OF WORKING GROUP 4B TO COMMITTEE 4 ON Agenda Item 1.37 (EPFD Issues at 6/4 GHz Band) |  |

Replace Table 22-1E, page 2, with the following table:

ADD

TABLE  22-1E

Limits to the epfd radiated by non-geostationary-satellite systems in
the fixed-satellite service in certain frequency bands12bis

| Frequency band (MHz) | epfd (dB(W/m2)) | Percentage of time during which epfd may not be exceeded | Reference bandwidth (kHz) | Reference antenna diameter and reference radiation pattern |
| --- | --- | --- | --- | --- |
| 3 700-4 200 | –195.4 | 100 | 4 | 1.8 m13ter |
|  | –197.9 | 100 | 4 | 2.4 m13ter |
|  | –201.6 | 100 | 4 | 3.7 m13ter |
|  | –203.3 | 100 | 4 | 4.5 m13ter |
|  | –204.5 | 100 | 4 | 5.5 m13ter |
|  | –207.5 | 100 | 4 | 8 m13ter |
|  | –208.5 | 100 | 4 | 10 m13ter |
|  | –212.0 | 100 | 4 | 15 m13ter |

______________