| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Document 332-E |
|  |  |  | 30 June 2003 |
|  |  |  | Original: English |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | COMMITTEE 8 |
| --- | --- |
| NINth series of texts submitted by committee 6 to the editorial committee |  |

APPENDIX  30*  (WRC-2000)

MOD	COM6/332/1

ANNEX  3

Method for determining the limiting interfering power flux-density at
the edge of a broadcasting-satellite service area in the frequency
bands 11.7-12.2 GHz (in Region 3), 11.7-12.5 GHz (in Region 1)
and 12.2-12.7 GHz (in Region 2), and for calculating the power
flux-density produced in these bands by a terrestrial station or a 
transmitting earth station in the fixed-satellite service 
in the band 12.5-12.7 GHz

# 1	General

1.1	This Annex describes a method of calculating the interference potential from terrestrial transmitters or transmitting earth stations in the fixed-satellite service to receiving earth stations in the broadcasting-satellite service.

1.2	The method is in two parts:

a)	the calculation of the maximum permissible interfering power flux-density at the edge of the broadcasting-satellite service area concerned;

b)	the calculation of the likely power flux-density produced at any point on the edge of the service area by the terrestrial transmitter or transmitting earth stations in the fixed-satellite service of another administration.

1.3	The interference potential of the terrestrial transmitters or the transmitting earth stations in the fixed-satellite service must be considered case by case; the power flux-density produced by each terrestrial transmitter or transmitting earth station Fp is compared to the limiting power flux-density F at any point on the edge of the service area of a broadcasting-satellite station of another administration. If, for a given transmitter, the value of the power flux-density produced Fp is lower than the value of the limiting power flux-density F at any point on the edge of the service area, the interference caused to the broadcasting-satellite service by this transmitter is considered to be lower than the permissible value and no coordination is required between administrations before the terrestrial service or the transmitting earth station is brought into use. Where this is not the case, coordination and more precise calculations derived from a mutually agreed basis are necessary.

Section 2 calculates the limit of power flux-density F at the edge of the service area.

Section 3 calculates the power flux-density produced Fp by a terrestrial station or a transmitting earth station.

1.4	It is emphasized that, should the calculation described in this Annex indicate that the maximum permissible power flux-density is exceeded, it does not necessarily preclude the introduction of the terrestrial or the fixed-satellite service since the calculations are necessarily based on worst-case assumptions for:

a)	the nature of the terrain of the interference path;

b)	the off-beam discrimination on the broadcasting-satellite receiving installations;

c)	the necessary protection ratios for the broadcasting-satellite service;

d)	the type of reception in the broadcasting-satellite service, i.e., assuming individual reception, this being more critical than community reception for the angles of elevation concerned;

e)	the value of power flux-density to be protected in the broadcasting-satellite service;

f)	the propagation conditions between the terrestrial station and the broadcasting-satellite service area.

# 2	Limit of power flux-density

## 2.1	General

The limiting power flux-density not to be exceeded at the edge of the service area in order to protect the broadcasting-satellite service of an administration is given by the formula:

F    F0  –  R    D    P	(1)

where:

F 	the maximum permissible interfering power flux-density (dB(W/m2)) within the necessary bandwidth of the broadcasting-satellite;

F0 	the wanted power flux-density (dB(W/m2)) at the edge of the service area;

R 	the protection ratio (dB) between the wanted and interfering signals;

D 	angular antenna discrimination (dB) provided by the radiation pattern of the broadcasting-satellite receiver antenna;

P 	polarization discrimination (dB) between the wanted and interfering signals.

## 2.2	Wanted power flux-density (F0)

The value of F0 is equal to:

For the Regions 1 and 3 Plan and List, Region 2 Plan and Article 4 submissions under § 4.1.3 and 4.2.6:

a)	108 dB(W/m2) in 27 MHz for service areas in Regions 1 and 3, and

b)	115 dB(W/m2) in 24 MHz, as well as in 27 MHz with respect to the cases mentioned in the footnote to Section 3.8 of Annex 5 concerning necessary bandwidths in Region 2.

For the analogue BSS assignments in the Region 2 Plan:

–	– 107 dB(W/m2) for 24 MHz, as well as for 27 MHz with respect to the cases mentioned in the footnote to Section 3.8 of Annex 5 concerning necessary bandwidths in Region 2.

## 2.3	Protection ratio (R)

2.3.1	For digital BSS assignments, the single entry protection ratio is equal to 30 dB.

2.3.2	For the analogue BSS assignments in the Region 2 Plan and for notified BSS assignments in Regions 1 and 3 Plan and List which are in conformity with the Plans and List of Appendix 30 and which have been brought into use and for which the date of bringing into use has been confirmed to the Bureau before 9 June 2003, the single entry protection ratio against all types of terrestrial transmissions, with the exception of amplitude-modulation multichannel television systems, is 35 dB for carrier frequency differences between the wanted and interfering signals of up to  10 MHz, decreasing linearly from 35 dB to 0 dB for carrier frequency differences between 10 MHz and 35 MHz, and is 0 dB for frequency differences in excess of 35 MHz (see Fig. 1). For amplitude-modulation multichannel television systems which produce high peaks of power flux-density spread over a wide range of their necessary bandwidth, the protection ratio R is 35 dB and is independent of the carrier frequency difference.

2.3.3	The carrier frequency difference should be determined by reference to the frequency assignments in the broadcasting-satellite Plan or, in the case of assignments not contained within a plan, by reference to the characteristics of the proposed or operational system.

2.3.4	A signal from a terrestrial station or a transmitting earth station in the fixed-satellite service should be considered only if its necessary bandwidth overlaps the necessary bandwidth of the broadcasting-satellite assignment.

## 2.4	Angular antenna discrimination (D)

2.4.1	For all Regions (digital):

NOTE 2 – For the graphical determination of

The value of D to be assumed in equation (1) is derived from the following equations, which are based on Recommendation ITU-R BO.1213 (also found in Annex 5 to this Appendix):

D = 0.0025((d/)*)2	dB	for	0	≤		<	m

D = Gmax – (29 – 25 log())	dB	for	m	≤		<	r

D = Gmax – (29 – 25 log())	dB	for	r	≤		≤	14.45

D = Gmax			dB	for				>	14.45

where:

:	elevation angle in degrees for the proposed or operational broadcasting-satellite system for the broadcasting-satellite service area concerned

m:	(/d)((Gmax – G1)/(0.0025))0.5 in degrees

G1:	29 – 25 log(r) in dB

r:	95(/d) in degrees

Gmax:	maximum gain of the antenna in dBi

d:	diameter of the antenna in m

:	the wavelength (m).

NOTE 1 – If more than one value of  is specified for a particular service area, the appropriate value of  should be used for each section of the edge of the service area under consideration.

For Regions 1 and 3, Gmax = 35.5 dBi corresponding to a 60-cm antenna at 11.7 GHz and 65% efficiency. For Region 2, Gmax = 33.3 dBi corresponding to a 45-cm antenna at 12.2 GHz and 65% efficiency. For a graphical depiction of this antenna discrimination see Fig. 2.

2.4.2	For the analogue BSS assignments in the Region 2 Plan:

The Discrimination D should be derived from the expression (3) below where  is the elevation angle for the proposed or operational broadcasting-satellite system for the broadcasting-satellite service area concerned.

NOTE 1 – If more than one value of  is specified for a particular service area, the appropriate value of  should be used for each section of the edge of the service area under consideration.

| D  0 dB | for 0°    0.43° | (3) |
| --- | --- | --- |
| D  4.15 2 dB | for 0.43°    1.92° |  |
| D  8.24  25 log  dB | for 1.92°    25° |  |
| D  43.2 dB | for   25° |  |

NOTE 2 – For the graphical determination of D see Fig. 3. The unit for  is degrees.

## 2.5	Polarization discrimination (P)

The value of P is equal to:

a)	3 dB when the interfering service uses linear polarization and the broadcasting-satellite service uses circular polarization or vice versa;

b)	0 dB when the interfering service and the broadcasting-satellite service both use circular or both use linear polarization.

# 3	Power flux-density produced by a terrestrial station or a transmitting earth station (Fp)

The power flux-density Fp (dB(W/m2)) produced at any point on the edge of the service area by the terrestrial station or the transmitting earth station is determined from the following formula:

Fp    E  –  A    10 log (4/2)	(4)

where:

E 	the equivalent isotropically radiated power (dBW) of the terrestrial station or the transmitting earth station in the direction of the point concerned on the edge of the service area;

A 	the total path loss (dB);

: 	wavelength (m).

## 3.1	Evaluation of path loss A for a terrestrial station or a transmitting earth station at the edge of the service area of the broadcasting satellite

The following propagation model is to be used for determining the minimum path loss between the interfering terrestrial transmitter or transmitting earth station and the edge of the BSS service area.

## 3.2	Propagation model

### 3.2.1	Distance limits

#### 3.2.1.1	Minimum distance limit

The minimum coordination distance is given as:

dmin(f)=100+(βp−f)2	(5)

where:

f:	frequency (GHz)

βp:	radiometeorological parameter, which reflects the relative incidence of clear-air anomalous propagation conditions.

The value of βp is latitude dependent. The latitude to be used in determining the correct value for βp is given by:

ζr = 	(6)

where  is earth station latitude (degrees). βp is then determined using:

βp = 	(7)

#### 3.2.1.2	Maximum distance limit

The maximum distance, dmax, for paths comprising a single climatic zone must not exceed the value for that climatic zone given in the Table below. For mixed paths comprising multiple zones the overall maximum distance must not exceed the value in the Table below corresponding to the climatic zone in the mixed path having the largest value (e.g. for a mixed path comprising zones A1 and A2, dmax is 500 km).

| Climatic zone* | Maximum distance (dmax)** |
| --- | --- |
| A1 | 500 |
| A2 | 375 |
| B | 900 |
| C | 1 200 |
| * For the definition see Appendix 7, Sections 1.5.1 and 1.5.3.2. ** As computed in Section 2 of Appendix 7. |  |

### 3.2.2	Ducting model

#### 3.2.2.1	Distance-independent part of the losses (dB) for ducting

For BSS earth stations, no additional protection can be assumed to be available from the earth station horizon elevation angle, i.e. Ah, the total terrain shielding attenuation, is 0 dB. However, if the detailed information for the transmitting station is known, including any site shielding mitigation techniques that are used, and as such all these factors need to be included in the determination of the coordination distance.

Reduction in attenuation arising from direct coupling into oversea ducts (dB):

(8)

where dc (km) is the distance from a land-based transmitting station to the coast in the direction being considered. dc is zero in other circumstances.

Distance-independent part of the losses (dB) for ducting:

(9)

#### 3.2.2.2	Distance-dependent part of the losses (dB) for ducting

3.2.2.2a	The specific attenuation (dB/km) due to dry air is given as:

γ0=7.19×10–3+6.09f2+0.227+4.81(f−57)2+1.50f2×10–3	(10)

3.2.2.2b	The specific attenuation due to water vapour is given as a function of , the water vapour density in units of g/m3, by the following equation:

γw(ρ) = 0.050 + 0.0021ρ + 3.6(f − 22.2)2 + 8.5 f2ρ × 10−4	(11)

3.2.2.2c	The specific attenuation (dB/km) due to water vapour for the ducting propagation model using a water vapour density of 7.5 g/m3 for paths over land, Zones A1 and A2, is given as:

wdl    w (7.5)	(12)

3.2.2.2d	The specific attenuation (dB/km) due to water vapour for the ducting propagation model using a water vapour density of 10.0 g/m3 for paths over sea, Zones B and C, is given as:

wds    w (10.0)	(13)

Note that the value of 10 g/m3 is used for both Zones B and C in view of the lack of data on the variability of water vapour density on a global basis, particularly the minimum values.

3.2.2.2e	Specific attenuation due to gaseous absorption (dB/km):

γg=γ0+γwdldtdi+γwds1−dtdi	(14)

where:

dt (km):	aggregate land distance, Zone A1 + Zone A2, along the path

di (km):	path length considered, it lies within the range between a minimum calculation distance and a maximum calculation distance.

3.2.2.2f	Values for zone-dependent parameters:

τ=1−exp−4.12×10–4(dlm)2.41	(15)

where:

dlm (km):	longest continuous inland distance, Zone A2, along the path considered

μ1=–dtm1016–6.6τ+10−(0.496+0.354τ50.2	(16)

where:

dtm (km):	longest continuous land (i.e. inland + coastal) distance, Zone A1 + Zone A2 along the path considered.

1 limited to 1  1.

σ=–0.6–8.5×10–9di3.1τ	(17)

 limited to   –3.4.

μ2=2.48×10–4di2σ	(18)

2 limited to 2  1.

μ4=10(−0.935+0.0176ζr) log μ1           for      ζ    ≤   70°      	(19)

3.2.2.2g	Path-dependent incidence of ducting, β, and the related parameter, Γ1 that are used to calculate time dependency of the path loss are given as:

β=βe⋅μ1⋅μ2⋅μ4	(20)

Γ1=1.076(2.0058−logβ)1.012exp−9.51−4.8logβ+ 0.198(log β)2×10–6di1.13	(21)

NOTE – For coordination of terrestrial mobile transmitting stations, fixed stations and transmitting earth stations, the mitigation factor C2i was set equal to zero.

3.2.2.2h	Distance-dependent part of the losses (dB) for ducting:

L5(p)=(γd+γg)di+(1.2+ 3.7 × 10–3di) log pβ+12pβΓ1+C2i	(22)

where:

p:	the maximum percentage of time for which the permissible interference power may be exceeded; p = 0.3%

γd:	the frequency-dependent ducting specific attenuation (dB/km).

γd = 0.05f13	(23)

3.2.2.2i	Attenuation due to ducting:

(24)

### 3.2.3	For the tropospheric scatter model

#### 3.2.3.1	Distance-independent part of the losses (dB) for tropospheric scatter

A2=187.36+10εh+Lf−0.15No−10.1−logp500.7	(25)

where:

εh:	earth station horizon elevation angle (degrees)

No:	path centre sea level surface given as:

N0 = 330 + 62.6 e−ζ−232.72	(26)

Lf: 	the frequency-dependent part of the losses (dB), given as:

Lf = 25 log(f) − 2.5logf22	(27)

#### 3.2.3.2	Distance-dependent part of the losses (dB) for tropospheric scatter

L6p=20 logdi+5.73  ×10–4112−15cos2ζdi+γ0+γwtdi+C2i	(28)

Total attenuation due to tropospheric scatter:

(29)

#### 3.2.3.3	Minimum path loss

The minimum path loss, Amin, between interfering transmit site and edge of BSS service area is given by:

Amin = Min (Aduct, Atrop)	(30)

__________