| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Document 338-E |
|  |  |  | 29 June 2003 |
|  |  |  | Original: English |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | COMMITTEE 4 |
| --- | --- |
| Chairman, Working Group 4B |  |
| tenth report from the chairman of working group 4B to committee 4 ON |  |
| agenda items 1.30 and 1.37 |  |

Working Group 4B has considered the proposals submitted by the Administrations of the Republic of Korea and Japan in Addendum 11 to Document 109 and Addendum 12 to Document 58 under agenda items 1.30 and 1.37, as indicated in Document DT/93. After detailed consideration, this has led to a proposal to allow the possibility to use the broadcasting-satellite service (sound) in these two countries to be extended to include the additional band 2 605-2 630 MHz. This change is introduced by introducing provision No. 5.418bis. The relevant sharing situations with respect to terrestrial services in the band 2 605-2 630 MHz have been addressed by introducing a pfd mask for GSO BSS (sound) and for non-GSO BSS (sound) by extending the application of Resolution 539 (as revised by this Conference) to include the 2 605-2 630 MHz band. The sharing situation with respect to other space services has been addressed by introducing provisions Nos. 5.418Abis, 5.418Bbis and 5.418Cbis to cover the band 2 605-2 630 MHz. Annex 1 provides the detailed texts.

The modifications to the Radio Regulations contained in Annex 1 should come into force as from 5 July 2003.

D. JANSKY
	Chairman, Working Group 4B, Box 46

Annex: 1

annex 1

article  5

Frequency allocations

ADD

5.418bis	In applying provision No. 5.418, in Korea (Rep. of) and Japan, resolves 3 of Resolution 528 (Rev.WRC-03) is relaxed to allow the broadcasting-satellite service (sound) and the complementary terrestrial broadcasting service to additionally operate on a primary basis in the band 2 605-2 630 MHz. This use is limited to national systems. The provisions of No. 5.416 and Table 21-4 of Article 21 do not apply. Use of non-geostationary-satellite systems in the broadcasting-satellite service (sound) in the band 2 605-2 630 MHz is subject to the provisions of Resolution 539 (Rev.WRC-03). The power flux-density at the Earth’s surface produced by emissions from a geostationary broadcasting-satellite service (sound) space station operating in the band 2 605-2 630 MHz, for which complete Appendix 4 advance publication information has been received after 4 July 2003, for all conditions and for all methods of modulation, shall not exceed the following limits:

130	dB(W/(m2 · MHz))	for	0	  	5

130  0.4 (  5)	dB(W/(m2 · MHz))	for	5	  	25

122	dB(W/(m2 · MHz))	for	25	  	35

where  is the angle of arrival of the incident wave above the horizontal plane, in degrees. These limits may be exceeded on the territory of any country whose administration has so agreed.

ADD

5.418Abis	In Korea (Rep. of) and Japan, use of the band 2 605-2 630 MHz by non-geostationary-satellite systems in the broadcasting-satellite service (sound) for which complete Appendix 4 coordination information, or notification information, has been received after 4 July 2003, is subject to the application of the provisions of No. 9.12A, in respect of geostationary-satellite networks for which complete Appendix 4 coordination information, or notification information, is considered to have been received after 4 July 2003, and No. 22.2 does not apply. No. 22.2 shall continue to apply with respect to geostationary-satellite networks for which complete Appendix 4 coordination information, or notification information, is considered to have been received before 5 July 2003. Use of the band by non-geostationary-satellite systems in the broadcasting-satellite service (sound) is subject to the provisions of Resolution 539 (Rev.WRC-03).

ADD

5.418Bbis	Use of the band 2 605-2 630 MHz by non-geostationary-satellite systems for which complete Appendix 4 coordination information, or notification information, has been received after 4 July 2003, is subject to the application of the provisions of No. 9.12. Resolution 539 (Rev.WRC-03) applies.

ADD

5.418Cbis	Use of the band 2 605-2 630 MHz by geostationary-satellite networks for which complete Appendix 4 coordination information, or notification information, has been received after 4 July 2003 is subject to the application of the provisions of No. 9.13 with respect to non-geostationary-satellite systems in the broadcasting-satellite service (sound), and No. 22.2 does not apply. Resolution 539 (Rev.WRC-03) applies.

MOD

2 520-2 700 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 2 520-2 655 FIXED 5.409 5.410 5.411 MOBILE except aeronautical mobile 5.384A BROADCASTING-SATELLITE 5.413 5.416 | 2 520-2 655 FIXED 5.409 5.411 FIXED-SATELLITE (space-to-Earth) 5.415 MOBILE except aeronautical mobile 5.384A BROADCASTING-SATELLITE 5.413 5.416 | 2 520-2 535 FIXED 5.409 5.411 FIXED-SATELLITE (space-to-Earth) 5.415 MOBILE except aeronautical mobile 5.384A BROADCASTING-SATELLITE 5.413 5.416 |
|  |  | 5.403 5.415A |
|  |  | 2 535-2 655 FIXED 5.409 5.411 MOBILE except aeronautical mobile 5.384A BROADCASTING-SATELLITE 5.413 5.416 |
| 5.339 5.403 5.405 5.412 5.418B ADD 5.418Bbis 5.418C ADD 5.418Cbis | 5.339 5.403 5.418B ADD 5.418Bbis 5.418C ADD 5.418Cbis | 5.339 5.418 ADD 5.418bis 5.418A ADD 5.418Abis 5.418B ADD 5.418Bbis 5.418C ADD 5.418Cbis |

Note to the Editorial Committee – The Editorial Committee is requested to update the text of Resolution 539, as revised by this Conference, to cover the band 2 605-2 655 MHz.

____________