| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Document 339-E |
|  |  |  | 30 June 2003 |
|  |  |  | Original: English |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | COMMITTEE 8 |
| --- | --- |
| eighth series of texts submitted by committee 4 to the editorial committee |  |

The following texts are submitted to Committee 8:

–	Article 1

–	Article 5

–	Article 9

–	Article 11

–	Article 13

–	Article 25

–	Article 29

–	Resolution 56

–	Resolution 77

–	Resolution 221

–	Resolution [COM4/10]

–	Resolution [COM4/24]

E. GEORGE

Chairman, Committee 4, Box 123

ARTICLE  1

Terms and definitions

MOD	COM4/339/1

1.189		geostationary satellite:  A geosynchronous satellite whose circular and direct orbit lies in the plane of the Earth's equator and which thus remains fixed relative to the Earth; by extension, a geosynchronous satellite which remains approximately fixed relative to the Earth.

ARTICLE  5

Frequency allocations

MOD	COM4/339/2

5.164	Additional allocation:  in Albania, Germany, Austria, Belgium, Bosnia and Herzegovina, Botswana, Bulgaria, Côte d’Ivoire, Denmark, Spain, Estonia, Finland, France, Gabon, Greece, Ireland, Israel, Italy, Jordan, Lebanon, Libya, Liechtenstein, Luxembourg, Madagascar, Mali, Malta, Morocco, Mauritania, Monaco, Nigeria, Norway, the Netherlands, Poland, Syria, the United Kingdom, Slovenia, Sweden, Switzerland, Swaziland, Chad, Togo, Tunisia, Turkey and Yugoslavia the band 47-68 MHz, in Romania the band 47-58 MHz, in South Africa the band 47-50 MHz, in the Czech Rep. the band 66-68 MHz, are also allocated to the land mobile service on a primary basis. However, stations of the land mobile service in the countries mentioned in connection with each band referred to in this footnote shall not cause harmful interference to, or claim protection from, existing or planned broadcasting stations of countries other than those mentioned in connection with the band.     (WRC-03)

MOD	COM4/339/3

5.388A	In Regions 1 and 3, the bands 1 885-1 980 MHz, 2 010-2 025 MHz and 2 110-2 170 MHz and, in Region 2, the bands 1 885-1 980 MHz and 2 110-2 160 MHz may be used by high altitude platform stations as base stations to provide International Mobile Telecommunications-2000 (IMT-2000), in accordance with Resolution 221 (Rev.WRC-03). The use by IMT-2000 applications using high altitude platform stations as base stations does not preclude the use of these bands by any station in the services to which they are allocated and does not establish priority in the Radio Regulations.     (WRC-03)

ADD	COM4/339/4

5.388B	In Algeria, Saudi Arabia, Bahrain, Cameroon, Comoros, Côte d’Ivoire, Cuba, Djibouti, Egypt, United Arab Emirates, Iran (Islamic Republic of), Israel, Jordan, Kenya, Kuwait, Libyan Arab Jamahiriya, Mali, Morocco, Mauritania, Nigeria, Oman, Uganda, Qatar, Syrian Arab Republic, Sudan, Tanzania, Chad, Tunisia, Yemen, Zambia and Zimbabwe, for the purpose of protecting fixed and mobile services including IMT-2000 mobile stations in their territories from co-channel interference, a HAPS operating as an IMT-2000 base station in neighbouring countries shall not exceed a co-channel power flux-density (pfd) of –127 dB (W/(m2 · MHz)) at the Earth’s surface outside a country’s borders unless explicit agreement of the affected administration is provided at the time of the notification of HAPS.

MOD	COM4/339/5

5.395	In France and Turkey, the use of the band 2 310-2 360 MHz by the aeronautical mobile service for telemetry has priority over other uses by the mobile service.

MOD	COM4/339/6

5.488	The use of the band 11.7-12.2 GHz by geostationary-satellite networks in the fixed-satellite service in Region 2 is subject to application of the provisions of No. 9.14 for coordination with stations of terrestrial services in Regions 1, 2 and 3-. See also Resolution [COM4/24] (WRC-03). For the use of the band 12.2-12.7 GHz by the broadcasting-satellite service in Region 2, see Appendix 30.     (WRC-03)

ARTICLE  9

Procedure for effecting coordination with or 
obtaining agreement of other administrations1, 2, 3, 4, 5, 6, 6bis

ADD	COM4/339/7

6bis	For the purpose of this Article, a satellite in the geostationary orbit is a satellite with an orbit the inclination of which is less than or equal to 15 degrees.

MOD	COM4/339/8

9.2		Amendments to the information sent in accordance with the provisions of No. 9.1 shall also be sent to the Bureau as soon as they become available. The use of an additional frequency band or modification of the orbital location by more than  6º for a space station using the geostationary-satellite orbit will require the application of the advance publication procedure for this band or orbital location, as appropriate.     (WRC-03)

MOD	COM4/339/9

9.14	i)	for a space station of a satellite network for which the requirement to coordinate is included in a footnote to the Table of Frequency Allocations referring to this provision or to No. 9.11A in respect of stations of terrestrial services where the threshold value is exceeded;     (WRC-03)

ARTICLE  11

Notification and recording of frequency assignments1, 2, 3, 4, 4bis

ADD	COM4/339/10

4bis	For the purpose of this Article, a satellite in the geostationary orbit is a satellite with an orbit the inclination of which is less than or equal to 15 degrees.

ADD	COM4/339/11

11.8bis	g)	if that assignment is subject to Resolution 221 (Rev.WRC-03).

MOD	COM4/339/12

11.24		Notices relating to assignments to stations of terrestrial services, except for those referred to in Nos. 11.25, 11.26 or 11.26bis, shall reach the Bureau not earlier than three months before the assignments are brought into use.

ADD	COM4/339/13

11.26bis		Notices relating to assignments for high altitude platform stations operating as base stations to provide IMT-2000 in the bands identified in 5.388A shall reach the Bureau not earlier than three years before the assignments are brought into use.

ARTICLE  13

Instructions to the Bureau

ADD	COM4/339/14

Section 0  –  Development of the Rules of Procedure and proposals to resolve inconsistencies encountered in the application of the Radio Regulations

ADD	COM4/339/15

13.0.1		The Board shall develop a new Rule of Procedure only when there is a clear need with proper justification for such a Rule. For all such Rules, the Board shall submit to the coming world radiocommunication conference the necessary modifications to the Radio Regulations, to alleviate such difficulties or inconsistencies and include their suggestions in the Report of the Director to the next world radiocommunication conference.

ADD	COM4/339/16

13.0.2		If such a need is not identified by No. 13.0.1, the Board shall submit also to the coming world radiocommunication conference the necessary modifications, to the Radio Regulations to alleviate such difficulties or inconsistencies.

ADD	COM4/339/17

13.12bis		In the preparation and development of the Rules of Procedure, the RRB, the Bureau and administrations shall apply the following steps:

a)	the Bureau shall also publish under No. 13.17 on the ITU website a list of future proposed Rules and the time-frame for their consideration by the Board and for comments by administrations on the list of future proposed Rules;

b)	any practice used by the Bureau in the application of the provisions of the Radio Regulations shall be identified and proposed for inclusion in the Rules of Procedure in accordance with the procedures of this section;

c)	all draft Rules prepared by the Bureau shall be available to administrations on the ITU website and by Circular Letter at least ten weeks prior to the start of the RRB meeting;

d)	any comments on these draft Rules of Procedure from administrations shall be submitted to the Bureau at least four weeks before the start of the RRB meeting;

e)	in submitting comments administrations should, if possible, suggest the actual text of their proposed Rules;

f)	all comments from administrations shall be posted on the ITU website. However, those comments that do not meet the above time-limits shall not be considered by the Board;

g)	any Rules of Procedure are to be in conformity with the spirit and principle of the Constitution, Convention and the Radio Regulations and shall avoid any relaxation to the application of the corresponding provisions of the Radio Regulation to which the Rules make reference.

MOD	COM4/339/18

13.18		Within one week after a meeting of the Board, a summary of all decisions including the reasons for each decision, taken in that meeting shall be made available on the ITU website. After each Board meeting the approved minutes of that meeting, shall normally be circulated at least one month before the start of the following meeting to administrations by means of a circular letter and these approved minutes shall also be made available on the ITU website.     (WRC-03)

ARTICLE  25

Amateur services

MOD	COM4/339/19

25.3		2)	Amateur stations may be used for transmitting international communications on behalf of third parties only in case of emergencies or disaster relief. An administration may determine the applicability of this provision to amateur stations under its jurisdication.

SUP	COM4/339/20

25.4

ARTICLE  29

Radio astronomy service

MOD	COM4/339/21

29.12	§ 9	In applying the measures outlined in this Section, administrations should bear in mind that the radio astronomy service is extremely susceptible to interference from space and airborne transmitters (for further information, see the most recent version of Recommendation ITU-R RA.769).

MOD	COM4/339/22

RESOLUTION  56  (rev.WRC-03)

Modification of the procedures and requirements
for advance publication

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	Resolution 86 (Marrakesh, 2002) of the Plenipotentiary Conference;

b)	that there is concern among a number of administrations that some of the current procedures and requirements for advance publication may give rise to inequalities in the satellite filing and coordination process,

resolves

1	that, as of 5 July 2003, the Radiocommunication Bureau and administrations shall apply the provisions of No. 9.2, as revised by this Conference;

2	that any request for coordination or modifications to a previously submitted API received by the Bureau after 5 July 2003 shall be examined in accordance with the provisions of No. 9.2 as revised by this Conference.

SUP	COM4/339/23

RESOLUTION  77  (WRC-2000)

Protection of terrestrial services in all Regions from geostationary-satellite networks in the fixed-satellite service in Region 2 
using the frequency band 11.7-12.2 GHz

MOD	COM4/339/24

RESOLUTION  221  (Rev.WRC-03)

Use of high altitude platform stations providing IMT-2000 in the bands 
1 885-1 980 MHz, 2 010-2 025 MHz and 2 110-2 170 MHz in Regions 1 
and 3 and 1 885-1 980 MHz and 2 110-2 160 MHz in Region 2

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that the bands 1 885-2 025 MHz and 2 110-2 200 MHz are identified in No. 5.388 as intended for use on a worldwide basis for IMT-2000, including the bands -1 980-2 010 MHz and 2 170-2 200 MHz for the terrestrial and satellite components of IMT-2000;

b)	that a high altitude platform station (HAPS) is defined in No. 1.66A as “a station located on an object at an altitude of 20 to 50 km and at a specified, nominal, fixed point relative to the Earth”;

c)	that HAPS may offer a new means of providing IMT-2000 services with minimal network infrastructure as they are capable of providing service to a large footprint together with a dense coverage;

d)	that the use of HAPS as base stations within the terrestrial component of IMT-2000 is optional for administrations, and that such use should not have any priority over other terrestrial IMT-2000 use;

e)	that, in accordance with No. 5.388 and Resolution 212 (Rev.WRC-97), administrations may use the bands identified for IMT-2000, including the bands referred to in this Resolution, for stations of other primary services to which they are allocated;

f)	that these bands are allocated to the fixed and mobile services on a co-primary basis;

g)	that, in accordance with No. 5.388A, HAPS may be used as base stations within the terrestrial component of IMT-2000 in the bands 1 885-1 980 MHz, 2 010-2 025 MHz and 2 110-2 170 MHz in Regions 1 and 3 and 1 885-1 980 MHz and 2 110-2 160 MHz in Region 2; the use by IMT-2000 applications using HAPS as base stations does not preclude the use of these bands by any station in the services to which they are allocated and does not establish priority in the Radio Regulations;

h)	that ITU-R has studied sharing and coordination between HAPS and other stations within IMT-2000, has considered compatibility of HAPS within IMT-2000 with some services having allocations in the adjacent bands, and has approved Recommendation ITU-R M.1456;

i)	that radio interfaces of IMT-2000 HAPS are compliant with Recommendation ITU-R M.1457;

j)	that ITU-R has addressed sharing between systems using HAPS and some existing systems, particularly PCS (personal communications system), MMDS (multichannel multipoint distribution system) and systems in the fixed service, which are currently operating in some countries in the bands 1 885-2 025 MHz and 2 110-2 200 MHz;

-

k)	that HAPS stations are intended to transmit in the band 2 110-2 170 MHz in Regions 1 and 3 and in the band 2 110-2 160 MHz in Region 2;

l)	that administrations planning to implement a HAPS as an IMT-2000 base station may need to exchange information, on a bilateral basis, with other concerned administrations, including data items describing the HAPS characteristics in a more detailed manner than the data items currently included in Annexes 1A and 1B of Appendix 4, as indicated in the Annex to this Resolution,

resolves

1	that:

1.1	for the purpose of protecting IMT-2000 mobile stations in neighbouring countries from co-channel interference, a HAPS operating as an IMT-2000 base station shall not exceed a co-channel power flux-density (pfd) of 
 –117 dB (W/(m2 · MHz)) at the Earth’s surface outside a country's borders unless explicit agreement of the affected administration is provided at the time of the notification of HAPS;

1.2	a HAPS operating as an IMT-2000 base station shall not --transmit outside the frequency bands 2 110-2 170 MHz in Regions 1 and 3 and 2 110-2 160 MHz in Region 2;

1.3	in Region 2, for the purpose of protecting MMDS stations in some neighbouring countries in the band 2 150-2 160 MHz from co-channel interference, a HAPS operating as an IMT-2000 base station shall not exceed the following co-channel power flux-density (pfd) at the Earth’s surface outside a country’s borders unless explicit agreement of the affected administration is provided at the time of the notification of the HAPS;

–	–127 dB(W/(m2 · MHz)) for angles of arrival () less than 7° above the horizontal plane;

	–127 + 0.666 ( – 7) dB(W/(m2 · MHz)) for angles of arrival between 7° and 22° above the horizontal plane; and

	–117 dB(W/(m2 · MHz)) for angles of arrival between 22° and 90° above the horizontal plane;

1.3bis	in some countries (see No. 5.388B), for the purpose of protecting fixed and mobile services including IMT-2000 mobile stations in their territories from co-channel interference caused by a HAPS operating as an IMT-2000 base station in accordance with No. 5.388A in neighbouring countries;

2	that the limits referred to in this Resolution shall apply to all HAPS operating in accordance with No. 5.388A;

3	that administrations wishing to implement HAPS within a terrestrial IMT-2000 system shall comply with the following:

3.1	for the purpose of protecting IMT-2000 stations operating- in neighbouring countries from co-channel interference, a HAPS operating as a base station within IMT-2000 shall use antennas that comply with the following antenna pattern:

G(y)  Gm – 3(y/yb)2 	dBi	for	 0°	£ y £ y1

G(y)  Gm  LN	dBi	for	y1	< y £ y2

G(y)  X – 60 log (y)	dBi	for	y2	< y £ y3

G(y)  LF	dBi	for	y3	< y £ 90°

where:

G(y)  :	gain at the angle y from the main beam direction (dBi)

Gm  :	maximum gain in the main lobe (dBi)

yb  : 	one-half of the 3 dB beamwidth in the plane considered (3 dB below Gm) (degrees)

LN  : 	near side-lobe level in dB relative to the peak gain required by the system design, and has a maximum value of –25 dB

LF  :	far side-lobe level, Gm – 73 dBi

y1  yb −LN3	degrees

y2  3.745 yb    	degrees

X  Gm  LN + 60 log (y2)	dBi

y3 = 10(X−LF)60	degrees

The 3 dB beamwidth (2yb) is again estimated by:

(yb)2  7 442/(100.1Gm)	degrees2

where Gm is the peak aperture gain (dBi);

3.2	for the purpose of protecting mobile earth stations within the satellite component of IMT-2000 from interference, a HAPS operating as an IMT-2000 base station, shall not exceed an out-of-band pfd of –165 dB(W/(m2 ·  4 kHz)) at the Earth’s surface in the bands 2 160-2 200 MHz in Region 2 and 2 170-2 200 MHz in Regions 1 and 3;

3.3	a HAPS operating as a base station to provide IMT-2000, in order to protect fixed stations from interference, shall not exceed the following limits of out-of-band power flux-density (pfd) at the Earth’s surface in the bands 2 025-2 110 MHz:

	–165 dB(W/(m2 · MHz)) for angles of arrival () less than 5 above the horizontal plane;

	–165 + 1.75 ( – 5) dB(W/(m2 · MHz)) for angles of arrival between 5 and 25 above the horizontal plane; and

	–130 dB(W/(m2 · MHz)) for angles of arrival between 25 and 90 above the horizontal plane;

4	that, for facilitating consultations between administrations, administrations planning to implement a HAPS as an IMT-2000 base station shall furnish to the concerned administrations the additional data elements listed in the Annex to this Resolution, if so requested;

5	that administrations planning to implement a HAPS as an IMT-2000 base station shall notify the radio-frequency spectrum assignment(s) by submitting all mandatory elements of Appendix 4 and the additional characteristics listed in Annex to this Resolution to the Bureau for the examination;

6	that, as of 5 July 2003, the Bureau and administrations shall apply provisionally the provisions of Nos. 5.388A and 5.388B as revised by this Conference for the frequency assignments to HAPS referred to in this Resolution, including those received before this date but not yet processed by the Bureau,

invites ITU-R

to develop, as a matter of urgency, an ITU-R Recommendation providing technical guidance to facilitate consultations with neighbouring administrations.

-

ADD	COM4/339/25

ANNEX to resolution 221 (Rev.WRC-03)

List of characteristics of a HAPS operating as a base station to provide IMT-2000 in the frequency bands in accordance 
with Resolution 221 (Rev.WRC-03)

# A	General characteristics to be provided for the station

## A.1	Identity of the station

a)	Identity of the station

b)	Country

## A.2	Date of bringing into use

The date (actual or foreseen, as appropriate) of bringing the frequency assignment (new or modified) into use.

## A.3	Operating administration or agency

Symbols for the operating administration or agency and for the address of the administration to which communication should be sent on urgent matters regarding interference, quality of emissions and questions referring to the technical operation of the station (see Article 15).

## A.4	Position information of HAPS stations

a)	The nominal geographical longitude for the HAPS

b)	The nominal geographical latitude for the HAPS

c)	The nominal altitude for the HAPS

d)	The planned longitudinal and latitudinal tolerance for the HAPS

e)	The planned tolerance of altitude for the HAPS

## A.5	Agreements

If appropriate, the country symbol of any administration or administration representing a group of administrations with which agreement has been reached, including where the agreement is to exceed the limits prescribed in Resolution 221 (Rev.WRC-03).

# B	Characteristics to be provided for each antenna beam

## B.1	HAPS antenna characteristics

a)	The maximum isotropic gain (dBi).

b)	HAPS antenna gain contours plotted on a map of the Earth’s surface.

# C	Characteristics to be provided for each frequency assignments for HAPS antenna beam

## C.1	Frequency range

## C.2	Power density characteristics of the transmission

The maximum value of the maximum power density (dB(W/MHz)), averaged over the worst 1 MHz supplied to the input of the antenna.

# D	Calculated pfd limit produced over any country in visibility of HAPS

The maximum power flux-density (pfd) calculated at the Earth’s surface within each administration’s territory over which the HAPS may be visible and over which these calculated pfd levels exceed the limits indicated in resolves 1.1 and 1.3 of Resolution 221 (Rev.WRC-03).

# E	Table of characteristics to be submitted for a HAPS operating as a base station to provide IMT-2000 in the frequency bands in accordance with Resolution 221 (Rev.WRC-03)

| Item | Notification for HAPS station |
| --- | --- |
| A.1.a | X |
| A.1.b | X |
| A.2 | X |
| A.3 | O |
| A.4.a | X |
| A.4.b | X |
| A.4.c | X |
| A.4.d | X |
| A.4.e | X |
| A.5 | + |
| B.1.a | X |
| B.1.b | X |
| C.1 | X |
| C.2 | X |
| D | X |
| X: Mandatory O: Optional +: Required in special cases |  |

ADD	COM4/339/26

draft  Resolution  [COM4/10]  (WRC-03)

Scope and criteria to be used for the implementation of 
Resolution 86 (Rev.Marrakesh, 2002)

The World Radiocommunication Conference (Geneva, 2003),

considering

that the Plenipotentiary Conference of Marrakesh, 2002, discussed the application of Resolution 86 and decided to request WRC-03 to determine the scope and criteria to be used by future world radiocommunication conferences (WRCs) in the application of Resolution 86,

resolves

that the scope and criteria of Resolution 86 (Rev.Marrakesh, 2002) to be considered by future WRCs be as follows:

1	to consider any proposals which deal with deficiencies in the advance publication, coordination and notification procedures of the Radio Regulations for space services which have either been identified by the RRB and included in the Rules of Procedure or which have been identified by administrations or by the Bureau, as appropriate;

2	to consider any proposals which are intended to transform the content of the Rules of Procedure into a regulatory text;

3	to ensure that these procedures, characteristics and appendices reflect the latest technologies, as far as possible;

4	to consider any proposals intended to facilitate, in accordance with Article 44 of the Constitution, the rational, efficient and economical use of radio frequencies and the associated orbits including the geostationary orbit in accordance with resolves 2 of Resolution 80 (Rev.WRC-2000) and resolves to request the 2003 and subsequent world radiocommunication conferences of Resolution 86 (Rev.Marrakesh, 2002);

5	to consider any changes to provisions of the Radio Regulations for space services that would result in the simplification of the procedures and the work of BR and/or administrations;

6	to consider any changes to the Radio Regulations that follow from decisions of a plenipotentiary conference on space matters.

ADD	COM4/339/27

DRAFT  RESOLUTION  [COM4/24]  (WRC-03)

Transitional arrangements relating to use of the frequency 
band 11.7-12.2 GHz by geostationary-satellite networks
in the fixed-satellite service in Region 2

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that, in Region 2, the band 11.7-12.1 GHz is allocated on a co-primary basis to terrestrial services (except in the countries listed in No. 5.486) and to the fixed-satellite service (FSS);

b)	that, in Region 2, the band 12.1-12.2 GHz is allocated on a co-primary basis to terrestrial services in Peru (see No. 5.489) and to the FSS;

c)	that, in Regions 1 and 3, the band 11.7-12.2 GHz is allocated on a co-primary basis to terrestrial services and to the broadcasting-satellite service (BSS);

d)	that WRC-2000 adopted Resolution 77 to protect terrestrial services in Regions 1, 2 and 3 from GSO networks in the FSS in Region 2, but did not clarify which procedures should be applied;

e)	that the Rule of Procedure concerning No. 5.488 extended the applicability of Resolution 77 to coordination requests received from 1 January 1999 and to coordination requests received prior to 1 January 1999 for which Special Sections under former Article 14 had not been published;

f)	that WRC-03 suppressed Resolution 77 and, through revision of No. 5.488, replaced it with application of No. 9.14 for FSS in Region 2 to coordinate with stations in terrestrial services in all three Regions,

recognizing

that transitional measures are needed to address implementation of No. 9.14 for GSO FSS in Region 2 in the band 11.7-12.2 GHz,

resolves

1	that for requests for coordination under Article 9 for GSO networks in the FSS in Region 2 in the band 11.7-12.2 GHz for which complete Appendix 4 information was received after 1 May 2002 by the Bureau, the Bureau shall apply No. 9.14 as adopted at this Conference;

2	that for requests for coordination processed previously under Resolution 77, the Bureau shall apply No. 9.14 as adopted at this Conference which may involve publication of a list of such networks to initiate the No. 9.14 process;

3	that for requests for notification under Article 11 involving networks processed under resolves 1 and 2 above, the provisions in Article 11 associated with No. 9.14 shall be applied;

4	that the provisions of Nos. 5.488, 9.14 and the part of Table 5-1 of Appendix 5 (Rev.WRC-03) referring to No. 9.14 as amended by this Conference shall be provisionally applied as from 5 July 2003.

________________