| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Document 369-E |
|  |  |  | 1 July 2003 |
|  |  |  | Original: English |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | PLENARY MEETING |
| --- | --- |
| Chairman, ad hoc Group 5 of the Plenary |  |
| second report from the chairman of ad hoc group 5 of the plenary |  |
| agenda items 1.30 and 1.37 |  |

Ad hoc Group 5 of the Plenary has considered the proposals submitted by the Administrations of the Republic of Korea and Japan in Addendum 11 to Document 109 and Addendum 12 to Document 58 under agenda items 1.30 and 1.37, as indicated in Document DT/93. After detailed consideration, this has led to a proposal to allow the possibility to use the broadcasting-satellite service (sound) in these two countries to be extended to include the additional band 2 605-2 630 MHz. This change is introduced by introducing provision No. 5.418bis. The relevant sharing situations with respect to terrestrial services in the band 2 605-2 630 MHz have been addressed by introducing a pfd mask together with a coordination distance for GSO BSS (sound), and for non-GSO BSS (sound) by extending the application of Resolution 539 (as revised by this Conference) to include the band 2 605-2 630 MHz. The sharing situation with respect to other space services has been addressed by introducing provisions Nos. 5.418Abis, 5.418Bbis and 5.418Cbis to cover the band 2 605-2 630 MHz.

Annex 1 provides the detailed texts.

When reviewing these sharing situations it was also possible to address similar issues concerning the use of BSS (sound) in the band 2 630-2 655 MHz as provided for in provision No. 5.418. This has led to the detailed texts provided in Annex 2. These modifications also address and respond to a matter raised by the Director, BR in Section 3.1.2 in Addendum 3 of Document 4 concerning the application of No. 5.418C. The text for the modification to provision No. 5.418 is under discussion between the concerned administrations.

The modifications to the Radio Regulations contained in Annexes 1 and 2 should come into force as from 5 July 2003.

J. LEWIS
				Chairman, ad hoc Group 5 of the Plenary, Box 145

Annexes: 2

annex 1

article  5

Frequency allocations

ADD

5.418bis	In applying provision No. 5.418, in Korea (Rep. of) and Japan, resolves 3 of Resolution 528 (Rev.WRC-03) is relaxed to allow the broadcasting-satellite service (sound) and the complementary terrestrial broadcasting service to additionally operate on a primary basis in the band 2 605-2 630 MHz. This use is limited to national systems. An administration listed in No. 5.418 shall not have simultaneously two overlapping frequency assignments, one under the provisions of No. 5.418, and the other one under the provisions of No. 5.416. The provisions of No. 5.416 and Table 21-4 of Article 21 do not apply. Use of non-geostationary-satellite systems in the broadcasting-satellite service (sound) in the band 2 605-2 630 MHz is subject to the provisions of Resolution 539 (Rev.WRC-03). The power flux-density at the Earth’s surface produced by emissions from a geostationary broadcasting-satellite service (sound) space station operating in the band 2 605-2 630 MHz, for which complete Appendix 4 coordination information, or notification information, has been received after 4 July 2003, for all conditions and for all methods of modulation, shall not exceed the following limits:

130     dB(W/(m2 · MHz))	for	0				5

130  0.4 (  5)     dB(W/(m2 · MHz))	for	5				25

122     dB(W/(m2 · MHz))	for	25				90

where  is the angle of arrival of the incident wave above the horizontal plane, in degrees. These limits may be exceeded on the territory of any country whose administration has so agreed. As an exception to the limits above, the pfd value of –122 dB(W/(m2 · MHz)) shall be used as a threshold for coordination under No. 9.11 in an area of 1 000 km around the territory of the BSS notifying administration, for angles of arrival greater than 35 degrees.

ADD

5.418Abis	In Korea (Rep. of) and Japan, use of the band 2 605-2 630 MHz by non-geostationary-satellite systems in the broadcasting-satellite service (sound), pursuant to No. 5.418bis, for which complete Appendix 4 coordination information, or notification information, has been received after 4 July 2003, is subject to the application of the provisions of No. 9.12A, in respect of geostationary-satellite networks for which complete Appendix 4 coordination information, or notification information, is considered to have been received after 4 July 2003, and No. 22.2 does not apply. No. 22.2 shall continue to apply with respect to geostationary-satellite networks for which complete Appendix 4 coordination information, or notification information, is considered to have been received before 5 July 2003.

ADD

5.418Bbis	Use of the band 2 605-2 630 MHz by non-geostationary-satellite systems in the broadcasting-satellite service (sound), pursuant to No. 5.418bis, for which complete Appendix 4 coordination information, or notification information, has been received after 4 July 2003, is subject to the application of the provisions of No. 9.12.

ADD

5.418Cbis	Use of the band 2 605-2 630 MHz by geostationary-satellite networks for which complete Appendix 4 coordination information, or notification information, has been received after 4 July 2003 is subject to the application of the provisions of No. 9.13 with respect to non-geostationary-satellite systems in the broadcasting-satellite service (sound), pursuant to No. 5.418bis, and No. 22.2 does not apply.

MOD

2 520-2 700 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 2 520-2 655 FIXED 5.409 5.410 5.411 MOBILE except aeronautical mobile 5.384A BROADCASTING-SATELLITE 5.413 5.416 | 2 520-2 655 FIXED 5.409 5.411 FIXED-SATELLITE (space-to-Earth) 5.415 MOBILE except aeronautical mobile 5.384A BROADCASTING-SATELLITE 5.413 5.416 | 2 520-2 535 FIXED 5.409 5.411 FIXED-SATELLITE (space-to-Earth) 5.415 MOBILE except aeronautical mobile 5.384A BROADCASTING-SATELLITE 5.413 5.416 |
|  |  | 5.403 5.415A |
|  |  | 2 535-2 655 FIXED 5.409 5.411 MOBILE except aeronautical mobile 5.384A BROADCASTING-SATELLITE 5.413 5.416 |
| 5.339 5.403 5.405 5.412 5.418B ADD 5.418Bbis 5.418C ADD 5.418Cbis | 5.339 5.403 5.418B ADD 5.418Bbis 5.418C ADD 5.418Cbis | 5.339 5.418 ADD 5.418bis 5.418A ADD 5.418Abis 5.418B ADD 5.418Bbis 5.418C ADD 5.418Cbis |

Note to the Editorial Committee – The Editorial Committee is requested to update the text of Resolution 539, as revised by this Conference, to cover the band 2 605-2 655 MHz.

ANNEX 2

[MOD

5.418	Additional allocation:  in Bangladesh, Belarus, Korea (Rep. of), India, Japan, Pakistan, Singapore, Sri Lanka and Thailand, the band 2 535-2 655 MHz is also allocated to the broadcasting-satellite service (sound) and complementary terrestrial broadcasting service on a primary basis. Such use is limited to digital audio broadcasting and is subject to the provisions of Resolution 528 (WARC-92). The provisions of No. 5.416 and Table 21-4 of Article 21, do not apply to this additional allocation. Use of non-geostationary-satellite systems in the broadcasting-satellite service (sound) is subject to Resolution 539 (Rev.WRC-03). Geostationary BSS (sound) systems for which complete Appendix 4 coordination information has been received after 1 June 2005 are limited to national systems. The power flux-density at the Earth’s surface produced by emissions from a geostationary broadcasting-satellite service (sound) space station operating in the band 2 630-2 655 MHz, and for which Appendix 4 coordination information has been received after 1 June 2005, shall not exceed the following limits, for all conditions and for all methods of modulation:

130     dB(W/(m2 · MHz))	for	0				5

130  0.4 (  5)     dB(W/(m2 · MHz))	for	5				25

–122     dB(W/(m2 · MHz)) 	for					25

where  is the angle of arrival of the incident wave above the horizontal plane, in degrees. These limits may be exceeded on the territory of any country whose administration has so agreed. As an exception to the limits above, the pfd value of –122 dB(W/(m2 · MHz)) shall be used as a threshold for coordination under No. 9.11 in an area of 1 000 km around the territory of the BSS notifying administration, for angles of arrival greater than 35 degrees.

In addition, an administration listed in this provision shall not have simultaneously two overlapping frequency assignments, one under this provision and the other one under the provision No. 5.416 for systems for which Appendix 4 coordination information has been received after 1 June 2005.     (WRC-03)]

MOD

5.418A	In certain Region 3 countries listed in No. 5.418, use of the band 2 630-2 655 MHz by non-geostationary-satellite systems in the broadcasting-satellite service (sound) for which complete Appendix 4 coordination information, or notification information, has been received after 2 June 2000, is subject to the application of the provisions of No. 9.12A, in respect of geostationary-satellite networks for which complete Appendix 4 coordination information, or notification information, is considered to have been received after 2 June 2000, and No. 22.2 does not apply. No. 22.2 shall continue to apply with respect to geostationary-satellite networks for which complete Appendix 4 coordination information, or notification information, is considered to have been received before 3 June 2000.-     (WRC-03)

MOD

5.418B	Use of the band 2 630-2 655 MHz by non-geostationary-satellite systems in the broadcasting-satellite service (sound), pursuant to No. 5.418bis, for which complete Appendix 4 coordination information, or notification information, has been received after 2 June 2000, is subject to the application of the provisions of No. 9.12.-     (WRC-03)

MOD

5.418C	Use of the band 2 630-2 655 MHz by geostationary-satellite networks for which complete Appendix 4 coordination information, or notification information, has been received after 2 June 2000 is subject to the application of the provisions of No. 9.13 with respect to non-geostationary-satellite systems in the broadcasting-satellite service (sound), pursuant to No. 5.418bis and No. 22.2 does not apply.     (WRC-03)

____________