| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Document 391-E |
|  |  |  | 2 July 2003 |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

| B23 | PLENARY MEETING |
| --- | --- |
| twenty-third SERIES OF TExts submitted by the editorial committee to the plenary meeting |  |
| The following texts are submitted to the Plenary Meeting for first reading: |  |

| Source | Document | Title |
| --- | --- | --- |
| PLEN | 368 | RESOLUTION 539 (Rev.WRC-03) |
|  |  | APPENDIX 5 (Rev.WRC-03) |
|  |  | – Table 5-1 |

Annex: 5 pages

MOD	(B23/391/1)

RESOLUTION  539  (Rev.WRC-03)

Use of the band 2 630-2 655 MHz in certain Region 3 countries 
by non-geostationary satellite systems in the 
broadcasting-satellite service (sound)

The World Radiocommunication Conference (Geneva, 03),

considering

a)	that the band 2 535-2 655 MHz is allocated under No. 5.418 to the broadcasting-satellite service (BSS) (sound) in certain Region 3 countries;

b)	that the provisions of Resolution 528 (WARC-92) currently limit the use of this band by systems in the BSS (sound) to the upper 25 MHz of the band;

c)	that, prior to WRC-2000, there were no coordination procedures applicable to non-geostationary (non-GSO) BSS (sound) systems in this band in relation to other non-GSO or GSO satellite networks;

d)	that satellite technology has now advanced to the stage where non-GSO systems in the BSS (sound) are technically and economically feasible when operated with high elevation angles and that there are practical designs available to ensure that the radiation of the non-GSO satellite in the BSS (sound) outside the main beam is kept at low levels;

e)	that satellite systems in the BSS as described in considering d) can be used for the delivery of high-quality, spectrally efficient  BSS (sound) to portable and mobile terminals;

f)	that non-GSO systems in the BSS (sound) in the band 2 630-2 655 MHz in Region 3 have been notified to ITU and are expected to be brought into use in the near future;

g)	that, prior to WRC-2000, the protection of existing terrestrial services was addressed through the coordination procedures of No. 9.11;

h)	that the provision cited in considering g) may be inadequate to ensure the future deployment of terrestrial services in this band;

i)	that a regulatory procedure is required in order to meet the dual objectives of providing adequate long-term protection to existing and planned terrestrial services while not placing undue constraints on the development and implementation of the non-GSO BSS (sound) system;

j)	that there are non-GSO systems being planned for operation in the BSS (sound) in the band 2 630-2 655 MHz in Region 3 that have highly elliptical orbits;

k)	that ITU-R has undertaken studies of the likely aggregate interference from a number of co-frequency broadcasting-satellite systems sharing with the terrestrial services on a co-primary basis;

l)	that ITU-R has undertaken studies that assumed there is only one satellite active at any time in a non-GSO system operating in a highly elliptical orbit,

invites

a)	administrations planning to operate non-GSO BSS (sound) systems in accordance with this Resolution, to take measures to design the system to minimize interference to terrestrial services outside the non-GSO BSS (sound) service area, for example as in considering d) above;

b)	administrations, whose territory is geographically close to the territory of an administration planning to operate a non-GSO BSS (sound) system in accordance with this Resolution, and for which there is a correspondingly high elevation angle to the active satellite, to take measures to facilitate the operation of non-GSO BSS (sound) systems,

resolves

1	that any BSS (sound) system using non-geostationary orbits brought into operation in the band 2 630-2 655 MHz in Region 3 shall be operated such that the minimum elevation angle over the service area is not less than 55, for the purposes of sharing with terrestrial services;

2	that, before an administration notifies to the Radiocommunication Bureau or brings into use a frequency assignment for a BSS (sound) system using non-GSO satellites in the band 2 630-2 655 MHz, for which complete Appendix 4 coordination information or notification information has been received after 2 June 2000, the following regulatory arrangements shall apply.-

The following mask of power flux-density values at the Earth’s surface produced by emissions from a space station for all conditions and for all methods of modulation shall be used as the basis of the regulatory procedures of this Resolution:

130	dB(W/(m2 · MHz))	for 	0	   	5

–130 + 0.4 ( – 5)	dB(W/(m2 · MHz))	for 	5°	<   	25°

–122	dB(W/(m2 · MHz))	for 	25°	<   	45°

–122 + [0.2] ( – 45)	dB(W/(m2 · MHz))	for 	45°	<   	65°

[–118	dB(W/(m2 · MHz))	for 	76°	<   	90°]

[–118 + 0.18 ( – 65)	dB(W/(m2 · MHz))	for 	65°	<   	82°]

[–115			dB(W/(m2 · MHz))	for	82°	<   90°]

where  is the angle of arrival of the incident wave above the horizontal plane, in degrees.

These values relate to the power flux-density and angles of arrival which would be obtained under free-space propagation conditions.

Furthermore:

–	for angles of arrival less than 76° in the power flux-density mask above, if the limits are exceeded, the notifying administration shall obtain the explicit agreement from any administration identified by the Bureau in its examination below;

–	for angles of arrival from 76° to 90° in the power flux-density mask above, the coordination procedure with respect to those administrations identified by the Bureau in its examination below will be that of No. 9.11;

-

3	that systems in the broadcasting-satellite service (sound) using non-GSO satellites shall be limited to national services unless agreement has been reached to include the territories of other administrations in the service area;

4	that, within the context of this Resolution, an administration listed in No. 5.418 shall not have simultaneously two overlapping frequency assignments, one under that provision, and the other one under provision No. 5.416;

5	that, as of 5 July 2003, the Bureau and administrations shall apply the provisions of Articles 9 and 11 taking into account Nos. 5.418, 5.418A, 5.418B, 5.418C, and this Resolution, as revised by this Conference,

-

instructs the Radiocommunication Bureau

1	when applying resolves 2, to use the power flux-density mask in resolves 2; and

–	for angles of arrival less than 76°, identify the affected administrations which have a primary allocation to terrestrial services in the same frequency band and on whose territory the power flux-density is exceeded and inform both the notifying and the affected administrations. At the notification stage the lack of any necessary agreement is considered as non-conformity with No. 11.31;

–	for angles of arrival from 76° to 90°, identify the affected administrations which have a primary allocation to terrestrial services in the same frequency band and on whose territory the power flux-density is exceeded and inform both the notifying and the affected administrations. At the notification stage each notice shall be examined in the application of No. 11.32 and, if appropriate, under No. 11.32A with respect to the probability of harmful interference that may be caused to assignments for which coordination could not be successfully completed;

2	as from 5 July 2003, to apply resolves 5 in its examination of requests for coordination and notifications for any BSS (sound) systems using non-GSO satellites in the 2 630-2 655 MHz band for which complete Appendix 4 coordination information or notification information has been received after 2 June 2000.

APPENDIX  5  (Rev.WRC-03)

MOD	(B23/391/2)

TABLE  5-1 (continued)

| Reference of Article 9 | Case | Frequency bands (and Region) of the service for which coordination is sought | Threshold/condition | Calculation method | Remarks |
| --- | --- | --- | --- | --- | --- |
| No. 9.11 GSO, non-GSO/ terrestrial | A space station in the broadcasting-satellite service (BSS) in any band shared on an equal primary basis with terrestrial services and where the BSS is not subject to a Plan, in respect of terrestrial services | 620-790 MHz 1 452-1 492 MHz 2 310-2 360 MHz 2 535-2 655 MHz (Nos. 5.418 and 5.418bis) 12.5-12.75 GHz (Region 3) 17.3-17.8 GHz (Region 2) 21.4-22 GHz (Region 1 and 3) 74-76 GHz | Bandwidths overlap: -The detailed conditions for the application of No. 9.11 in the bands 2 630-2 655 MHz and 2 605-2 630 MHz are provided in Resolution 539 (Rev.WRC-03) for non-GSO BSS (sound) systems pursuant to Nos. 5.418 and 5.418bis, and in Nos. 5.418 and 5.418bis for GSO BSS (sound) networks pursuant to those provisions. Resolution [COM4/5] (WRC-03) applies in the 620-790 MHz band. | Check by using the assigned frequencies and bandwidths |  |

Note to the Plenary – The modifications to Table 5-1 take account of the decisions of ad hoc Group 5 of the Plenary and the proposal on page 3 of Document 320 and the proposal on page 3 of Document 259.