|  | Radiocommunication Study Groups |  |  |
| --- | --- | --- | --- |
| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| Source: Document 5A/TEMP/124(Rev.1) |  | Annex 16 to Document 5A/298-E |  |
|  |  | 18 November 2016 |  |
|  |  | English only |  |
| Annex 16 to Working Party 5A Chairman’s Report |  |  |  |
| Working document towards a PRELIMINARY DRAFT NEW REPORT OF TECHNICAL AND OPERATIONAL CHARACTERISTICS, IMPLEMENTATION AND SPECTRUM NEEDS OF RSTT |  |  |  |

Table of contents

1	Introduction		5

2	Scope		5

3	Overview of RSTT		5

3.1	Description		5

3.2	 Generic Architecture of RSTT		5

3.3	Main applications		6

3.3.1	Train radio		6

3.3.2	Train positioning information		7

3.3.3	Train remote		8

3.3.4	Train surveillance		8

3.4	Generic deployment scenarios		8

3.4.1	Railway lines		8

3.4.2	Railway stations		9

3.4.3	Shunting yards		10

3.4.4	Maintenance Bases		10

3.4.5	Railway hub		11

4	Current technical and operational parameters/characteristics		12

4.1	Radiocommunication systems used for Train radio		12

4.1.1	Analogue Radio		13

4.1.2	Digital Radio		16

4.1.3	Trunked Radio		19

4.1.4	GSM-R		20

4.1.5	LTE based RSTT		21

4.1.6	Emergencey System		22

[4.1.X Possibe other train radios]		24

4.2	Radiocommunication systems used for Train positioning information		26

4.2.1	Position System		27

4.3	Radiocommunication systems used for Train remote systems		29

4.3.X	Shunting System		30

4.4	Radiocommunication systems used for Train surveillance		31

4.4.1	Survilliance System		32

5	Technologies of RSTT		34

5.1	GSM-R based RSTT		34

5.2	TETRA based RSTT		34

5.3	LTE based RSTT		35

5.4	New Radio based RSTT		36

5.5	Millimetric wave based RSTT		36

6	Spectrum		38

6.1	 Current spectrum usage of RSTT		38

6.1.1	Radiocommunication systems used for Train radio		39

6.1.2	Radiocommunication systems used for Train positioning information		40

6.1.3	Radiocommunication systems used for Train remote systems		40

6.1.4	Radiocommunication systems used for Train surveillance		40

6.2	Planning spectrum usage of RSTT		40

6.3	spectrum requirements of RSTT		40

7	Summary		40

Annex 1 – Vocabulary of Terms		41

Annex 2 – Summary of Response to the Questionnaire on the Usage of Railway Radiocommunication Systems		42

A2.1	Introduction		42

A2.2	Summary of the situation of feedback of the questionnaire		42

A2.3	Summary table of answers to each question		43

A2.4	Original questionnaire answers		43

Annex 3 – Consideration of the Doppler Effect in railway radiocommunication 
systems between high-speed trains and tracksides		44

A3.1	Introduction		44

A3.2	Evaluation of the Doppler shift for different frequency bands and different 
train speeds		44

A3.3	Conclusions		48

Annex 4 – RSTT in Japan		49

A4.1	Overview		49

A4.1.2 Frequency usage		50

A4.1.3 Operational environment		51

A4.2	150 MHz band RSTT		53

A4.2.1 Train Radio System		53

A4.2.2 Radiocommunication system for Emergency Cut-Off System		55

A4.2.3 Yard Radio		56

A4.3	300 MHz band RSTT		57

A4.3.1 Train Radio System		57

A4.3.2 Emergency Alarm Radio System		58

A4.3.3 Radiocommunication system for Emergency Cut-Off System		59

A4.3.4 Radiocommunication system for Electronic Blocking System		60

A4.3.5 JRTC Radio		61

A4.3.6 Yard Radio		62

A4.4	400 MHz band RSTT		63

A4.4.1 Train Radio System		63

A4.4.2 Radiocommunication system for High Speed Trains		64

A4.4.3 Yard Radio		66

A4.5	RSTT in the 40 GHz band		66

A4.5.1  40-GHz band video transmission system (MVT)		66

A4.5.2 Train Radio System in the 40 GHz band (TRS-40GHz)		68

A4.6	W-band RSTT		70

A4.6.1 Network architecture of W-band RSTT		70

A4.6.2 Deployment scenario		71

A4.6.3 Coexistence of 90-GHz band RSTT with the passive services		72

A4.6.4 W-band transceivers characteristics		73

A4.6.5 W-band propagation characteristics		76

A4.6.6 Table of frequency allocation of W-band		78

A4.7	60 GHz RSTT		79

A4.7.1 60-GHz band train platform monitoring system		79

A4.7.2 60-GHz band RSTT		79

Annex 5 – RSTT in China		81

A5.1	Overview		81

A5.2	450 MHz -band RSTT		81

A5.3	900 MHz -band RSTT		81

Annex 6 – RSTT in Russia		83

A6.1	Overview		83

A6.2	Train radio systems for train operation and railway traffic control in 
the HF band		83

A6.2.1 Architecture of train radio system in HF-band		83

A6.2.2 Typical technical characteristics of HF train radio system radiostations		83

A6.3	Train radio systems for train operation and railway traffic control in 
the VHF band		83

A6.3.1 Architecture of train radio system in VHF-band		83

A6.3.2 Typical technical characteristics of VHF train radio system radiostations		83

A6.4	Train radio systems for train operation and railway traffic control in 
the UHF band		84

A6.4.1 Architecture of train radio system in UHF-band		84

A6.4.2 Typical technical characteristics of UHF train radio system radiostations		84

Annex 7 – RSTT in Korea		86

A7.1	Overview		86

A7.2	700 MHz band RSTT		86

A7.2.1 Architecture of LTE-R		86

A7.2.2 Technical characteristics		87

A7.2.3 Operational characteristics		88

A7.3	150 MHz band RSTT		88

A7.3.1 Technical characteristics		88

A7.3.2 Operational characteristics		88

A7.4	800 MHz band RSTT		89

A7.4.1 Technical characteristics		89

A7.4.2 Operational characteristics		90

A7.5	400 MHz band RSTT		90

A7.5.1 Technical characteristics		91

A7.6	18 GHz band RSTT		91

A7.6.1 Technical characteristics		91

# 1	Introduction

Railway Radiocommunication Systems between Train and Trackside (RSTT) provide improved railway traffic control, passenger safety and improved security for train operations. WRC-19 agenda item 1.11 calls upon the World Radiocommunication Conference 2019 (WRC-19) to take necessary actions, as appropriate, to facilitate global or regional harmonized frequency bands, to the extent possible, for the implementation of RSTT, within existing mobile service allocations. Resolution 236 (WRC-15) recognized that timely studies are required on technologies providing for railway radiocommunication and that international standards and harmonized spectrum would facilitate worldwide deployment of radiocommunication systems between train and trackside. Further, Resolution 236 (WRC-15) invited ITU-R to study the spectrum needs, technical and operational characteristics and implementation of railway radiocommunication systems between train and trackside.

# 2	Scope

This report introduces the definitions, architecture, applications, working scenarios, and also studies technical and operational characteristics, spectrum needs of RSTT. This report summarizes the current implementation of various systems of RSTT by administrations and also describes current and evolving technologies for RSTT. This Report covers train to track communications for all types of trains (e.g. high speed trains, passenger trains, freight trains, and metro trains).

Editor’s note: The content in this section need to be further improved by next WP 5A meeting.

# 3	Overview of RSTT

## 3.1	Description

[Various radiocommunication systems/technologies have been used for many years for railway operational applications.] RSTT provide improved railway traffic control, passenger safety and improved security for train operations. RSTT carry train control, command, operational information as well as monitoring data between on-board radio equipment and related radio infrastructure located along trackside.

## 3.2	 Generic Architecture of RSTT

The main elements of the RSTT may consist of on board radio equipment, radio access units and other trackside radio infrastructure. Other systems, such as the core network, etc., are supporting systems for the RSTT.

–	Radio Access Unit: including antenna and base station, aiming to provide radio access to the terminals (especially cab radio).

–	On board radio equipment: Radio equipment installed on train as well as handsets. For example, mobile terminals of automatic train control (ATC).

–	Other trackside radio infrastructure: Radio infrastructure operating along trackside. For example: shunting radio devices.

A diagram of the architecture of RSTT is illustrated in Figure 3.2-1.

Figure 3.2-1

Diagram of the Architecture of RSTT

Editor’s note: The figure below need to be improved and discussed by next meeting, and short explanatory text is expected.

[]

## 3.3	Main applications

Editor’s note: This section need to be further improved by adding proper descriptions for each of the following applications.

### 3.3.1	Train radio

#### 3.3.1.1	Voice/Dispatch

Editor’s note: System for voice communication (Point-to-Point Call or Group-Call) between dispatcher and train driver

Editor’s note: There is a need to have an Annex to address all the related ITU-R reports.
As requested, this is the placeholder for the mentioning the Report ITU-R M.2014.

One of the main functions of RSTT is to provide Dispatching Communication, which is to provide specific voice communication features for railway shown in Table 1.

Table 3.3.1.1-1

Dispatching Communication Functionalities

| Service Type | Feature Description |
| --- | --- |
| REC/enhanced REC | Railway Emergency Call / enhanced Railway Emergency Call |
| eMLPP | enhanced Multi-Level Precedence and Pre-emption |
| FA | Functional Addressing |
| LDA | Location Dependent Addressing |
| VGCS | Voice Group Call Service |
| VBS | Voice Broadcast Service |
| PTT | Push-To-Talk |
| … |  |

For further information, please refer to UIC Project EIRENE functional requirements specification.

#### 3.3.1.2	Maintenance

Editor’s note: System for voice communication (Point-to-Point, point to many point call, or Group-Call) for maintenance services in railway infrastructure.

#### 3.3.1.3	Train Control (Interlock/movement authorization)

Editor’s note: System for data services consisting of “permission to proceed” and “corresponding route information” which is transmitted via Train Radio system to the train.

#### 3.3.1.4	Emergency

Editor’s note: System for a Railway Emergency Call dedicated to urgency.

#### 3.3.1.5	Train information

Generally, railway information transmitted by RSTT could be classified into two kinds:

–	to provide the railway transportation information for the train operators, such as train operating status, mobile ticketing and check-in services.

–	to provide relevant railway transportation information for passengers, such as travel information.

### 3.3.2	Train positioning information

The knowledge of the positions of all trains and other vehicles on the tracks in normal and high-speed operations is one of the essential information to provide for railways traffic control, passenger safety, and security of train operations and therefore systems and applications providing information on the intermittent train positioning or constant train tracking are integral part of RSTT.

#### 3.3.2.1	Balises

Editor’s note: Specific fixed transponders placed between the rails. The transponders serve as "beacons" giving the exact location of a train as well as transmitting signalling information in a digital telegram to the train.

#### 3.3.2.2	Loops/Leaky cable

Editor’s note: Leaky feeder cable transmitting signalling information.

#### 3.3.2.3	Annunciators

Editor’s note: System to control level crossings when a train route has been set and the indication point is passed by an approaching train.

#### 3.3.2.4	Radar [and/or optical] [Scanners]

Editor’s note: System to control e.g. level crossings when train is approaching.

#### 3.3.2.5	Axle counters

Editor’s note: System to control the integrity of trains in all operations, by counting the number of axles at a given position.

### 3.3.3	Train remote

#### 3.3.3.1	Shunting

Editor’s note: System for remote operation between train driver and shunting locomotive.

### 3.3.4	Train surveillance

Editor’s note: Systems to provide for video surveillance (in different qualities) in various use cases of train operations, the quality is varying due to the different requirements of the scenarios.

#### 3.3.4.1	CCTV: platform view/crossings

#### 3.3.4.2	CCTV: coach/compartments

#### 3.3.4.3	CCTV: device monitoring

## 3.4	Generic deployment scenarios

This section provides a brief overview of RSTT working scenarios. These scenarios are Railway line, Railway station, Shunting yard, Maintenance Base and Railway Hub. The general service characteristics of RSTT in different working scenarios are listed in Table 3.4-1.

Table 3.4-1

General Service Characteristics of RSTT in different working scenarios

|  | Priority | Latency | Reliable | Density | Moving speed |
| --- | --- | --- | --- | --- | --- |
| Railway line | High | Low | High | Low | High |
| Railway station | High | Low | High | High | High/Stop |
| Shunting yard | High | Low | High | High | Low/Stop |
| Maintenance Base | Low | Medium | High | High | Stop |
| Railway hub | High | Low | High | High | High/Low/Stop |

### 3.4.1	Railway lines

The train communication between the tracksides and moving trains, in this working scenario, requires reliable wireless radio-links. It needs to satisfy all train to track communication application, including voice and data services, for example, the data transmission for the control-command of trains, provided by railway operators.

Whenever needed, the interoperability requirements of the RSTT should be taken into account during cross-border railway transportation. Compatible RSTT system can support international roaming and international data exchange, and also helpful to improve the efficiency of cross-border transportation and to reduce the relevant cost.

Figure 3.4.1-1

Railway lines

In addition, there are several specific operating scenarios of railway lines, e.g. parallel railway lines, viaducts and tunnels etc.

Figure 3.4.1-2

Several specific operating scenarios

(a) Parallel railway lines	(b) Viaducts	(c) Tunnel

### 3.4.2	Railway stations

Compared to the railway lines, the typical services and applications in railway stations may include train control, monitoring, railway information etc., e.g., device monitoring system (DMS), information transmission system of end-of-train safety equipment.

Figure 3.4.2-1

Railway station

### 3.4.3	Shunting yards

In shunting mode, the typical applications may include voice and alerting data mixed transmission, monitoring. (Source: FRS 8.0.pdf)

Figure 3.4.3-1

Shunting mixed with railway lines

### 3.4.4	Maintenance Bases

The working scenario of RSTT in the maintenance bases is similar to that of in railway stations. In this scenario, RSTT need to support the following applications: monitoring, maintenance information. (Sources: FRS 8.0.pdf)

Figure 3.4.4-1

Maintenance Base

### 3.4.5	Railway hub

The RSTT in hub scenario is the combination of other typical railway scenarios. Figure 3.4.5-1 is a diagrammatic sketch in a big city, in which railway stations (including Maintenance base and shunting yard etc.) are connected by different railway lines. Due to the complex operations in the hub, the moving speed of the trains in the hub is quite different, ranging from 0 to high speed level.

Figure 3.4.5-1

Railway hub

# 4	Current technical and operational parameters/characteristics

Editor’s note: Administrations are encouraged to check the existing data in the table of this Chapter and to fill their other RSTT’s technical and operational parameters into proper table in this Chapter.

## 4.1	Radiocommunication systems used for Train radio

Editor’s note: Brief introduction is needed to describe each Radiocommunication system used for train radio.

### 4.1.1	Analogue Radio

Editor’s note: Administrations are encourage to provide a shore paragraph here to introduce their Analogue Radio systems in this sub-section.

Table 4.1.1-1

Parameter and usage of Analogue Radio

| # |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use by which Administration(s) |  | J | J | J | J | J | J | RUS |
| Specific name (if have) |  | Analogue TRS | Yard Radio | Analogue TRS | REBS | Yard Radio | Yard Radio |  |
| Frequency Range (MHz) |  | 140-144, 146-149.9 | 140-144,146-149.9, 150.05-156.4875, 156.8375 - 160 | 335.4-360 | 335.4-340 | 335.4-399.9 | 450-470 | 2.124-2.136 2.144-2.156 |
| Radiocommunication Standards |  |  |  |  |  |  |  |  |
| Channel separation (kHz) |  | 20 | 20 | 12.5 | 12.5 | 12.5 | 12.5 | 20 |
| Antenna gain (dBi) |  | BS: +15 MS: +4.2 | [TBD] | BS: +11 MS: +1 | +1 | [TBD] | [TBD] | 0 |
| Polarization |  | vertical | Vertical | vertical | vertical | vertical | vertical | dual |
| Transmitting radiation power (dBm) |  | BS: +47 MS: +40 | [TBD] | BS: +36 MS: +30 | +30 | +37 | +37 | 11.5 |
| e.i.r.p. (dBm) |  | BS:+62 MS: +44.2 | [TBD] | BS: +47 MS: +31 | +31 | [TBD] | [TBD] | 11.5 |
| Receiving noise figure (dB) Fixed Station/Mobile/Portable |  | <10 | <10 | <10 | <10 | <10 | <10 | <12 |
| Transmission data rate (kb/s) |  | NA | [TBD] | NA | NA | [TBD] | [TBD] | NA |
| Transmission distance (km) |  | 3-40 | [TBD] | 1.5 - 3 | max 5 | [TBD] | [TBD] | max 20 |
| Modulation |  | FM | FM | FM | FM | FM | FM | FM |
| Multiplexing method |  | FDD | None | FDD | [TBD] | [TBD] | None | FDMA |
| Reception quality |  | SNR > 45 | SNR > [30] | SNR > 30 | NA | SNR > 30 | SNR > 30 | NA |
| Applications | Voice/Dispatch | X | X | X | X | X |  |  |
|  | Maintenance |  |  |  |  |  |  |  |
|  | Train Control |  |  |  |  |  |  |  |
|  | Emergency |  |  |  |  |  |  |  |
| Scenarios | Railway line | X |  |  |  |  |  |  |
|  | Railway station, | X | X |  | X | X | X |  |
|  | Shunting yard |  | X |  |  | X | X |  |
|  | Maintenance Base |  | X |  |  | X | X |  |

Table 4.1.1-1 (continued)

Parameter and usage of Analogue Radio

| # |  | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use by which Administration(s) |  | J | J |  |  |  |  |  |
| Specific name (if have) |  | Analogue TRS | Analogue RHST |  |  |  |  |  |
| Frequency Range (MHz) |  | 410 - 420 | 410 - 420 450 - 455 |  |  |  |  |  |
| Radiocommunication Standards |  |  |  |  |  |  |  |  |
| Channel separation (kHz) |  | 12.5 | UL: 12.5, 25, 230, 288 DL: 12.5, 25, 64, 230, 288, 640 |  |  |  |  |  |
| Antenna gain (dBi) |  | BS: +11 MS: +1 | BS: (LCX) MS: +5 |  |  |  |  |  |
| Polarization |  | vertical | vertical |  |  |  |  |  |
| Transmitting radiation power (dBm) |  | +30 | BS: +33 MS: +36 |  |  |  |  |  |
| e.i.r.p. (dBm) |  | BS: +41 MS: +31 | BS: (LCX) MS: +41 |  |  |  |  |  |
| Receiving noise figure (dB) Fixed Station/Mobile/Portable |  | <10 | < 10 |  |  |  |  |  |
| Transmission data rate (kb/s) |  | NA | NA |  |  |  |  |  |
| Transmission distance (km) |  | 1.3 -3 | LCX-MS: 0.001-0.002 (BS interval: 30) |  |  |  |  |  |
| Modulation |  | FM | DL: PM UL: FM |  |  |  |  |  |
| Multiplexing method |  | none | DL: FDM UL: FDMA/FDD |  |  |  |  |  |
| Reception quality |  | SNR > 20 | SNR > 30 |  |  |  |  |  |
| Applications | Voice/Dispatch | X | X |  |  |  |  |  |
|  | Maintenance | X | X |  |  |  |  |  |
|  | Train Control |  |  |  |  |  |  |  |
|  | Emergency |  |  |  |  |  |  |  |
| Scenarios | Railway line | X | X |  |  |  |  |  |
|  | Railway station, | X | X |  |  |  |  |  |
|  | Shunting yard | X |  |  |  |  |  |  |
|  | Maintenance Base |  |  |  |  |  |  |  |

### 4.1.2	Digital Radio

Editor’s note: Administrations are encourage to provide a shore paragraph here to introduce their Digital Radio systems in this sub-section.

Table 4.1.2-1

Parameter and usage of Digital Radio

| # |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use by which Administration(s) |  | J | J | J | J | J | CHN | KOR |
| Specific name (if have) |  | Digital TRS (Type 1) | Digital TRS (Type 2) | Digital TRS (Type 3) | Digital TRS | JRTC Radio | 450 MHz Band Wireless Train Dispatching System | VHF system |
| Frequency Range (MHz) |  | 140 - 144, | 146 - 149.9 | 146 - 149.9 | 335.4 - 360 | 335.4 - 360 | 457.200-458.650 467.200-468.650 | 153 |
| Radiocommunication Standards |  |  |  |  |  |  |  |  |
| Channel separation (kHz) |  | 6.25 | 25 | 6.25 | 6.25 | 6.25 | 25 | 25 |
| Antenna gain (dBi) |  | BS: +11 MS: +1 | BS: +11 MS: +1 | BS: +11 MS: +1 | BS: +11 MS: +1 | BS: +11 MS: +1 | Fixed radio station: 9(Omni-directional), 12(Directional) Locomotive station: 0 | 3 |
| Polarization |  | vertical | vertical | Vertical | Vertical | vertical | NA | NA |
| Transmitting radiation power (dBm) |  | BS: +40 MS: +30 | BS: +37 MS: +30 | BS: +30 MS: +30 | BS: +36 MS: +24.8 | BS: +34.8 MS: +30 | Fixed Radio Station: 34.7-37(simplex), 37-40 (duplex), Locomotive Station: 37(simplex), 40 (duplex), Handset: 34.7 | BS: 25W Cab: 25W UE: 3-3.8W |
| e.i.r.p. (dBm) |  | BS:+51 MS: +31 | BS:+48 MS: +31 | BS:+41 MS: +31 | BS: +47 MS: +24.8 | BS: +45.8 MS: +31 | NA | NA |
| Receiving noise figure (dB) Fixed Station/Mobile/Portable |  | <10 | <10 | <10 | <10 | <10 | NA | -113 |
| Transmission data rate (kb/s) |  | 9.6 | 32 | 4.8 | 9.6 | 9.6 | NA | 9.6 |
| Transmission distance (km) |  | 1-3 | 1-2 | 1-3 | 1.5-2 | 2-3 | NA | NA |
| Modulation |  | π/4QPSK | π/4QPSK | 4FSK | π/4QPSK | π/4QPSK | FM | FM |
| Multiplexing method |  | FDMA or SCPC | TDMA | FDMA or SCPC | FDMA | FDD, TDM-TDMA | NA | NA |
| Reception quality |  | BER < 10-4 | BER < 10-4 | BER < 10-4 | BER < 10-4 | BER < 10-4 | Adjacent-channel Selectivity (receiver) ≥65dB ,Co-channel Rejection≥-8dB | NA |
| Applications | Voice/Dispatch | X | X | X | X |  | X |  |
|  | Maintenance |  |  |  |  |  | X |  |
|  | Train Control |  |  |  |  | X |  |  |
|  | Emergency | X |  |  |  |  |  |  |
| Scenarios | Railway line | X | X | X | X | X | X |  |
|  | Railway station, | X | X | X | X | X | X |  |
|  | Shunting yard |  |  |  |  |  | X |  |
|  | Maintenance Base |  |  |  |  |  | X |  |

Table 4.1.2-2 (continued)

Parameter and usage of Digital Radio

| # |  | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use by which Administration(s) |  | RUS | RUS | RUS | J | J |  |  |
| Specific name (if have) |  |  | TETRA System | GSM-R System | digital RHST (type1) | digital RHST (type 2) |  |  |
| Frequency Range (MHz) |  | 146-174 | 450-470 | 900 | 410 - 420 450 - 455 | 410 - 420 450 - 455 |  |  |
| Radiocommunication Standards |  |  |  |  |  |  |  |  |
| Channel separation (kHz) |  | 12.5 / 25 | 25 | 200 | UL: 12.5, 25, 230, 288 DL: 12.5, 25, 64, 230, 288, 640 | UL: 12.5, 25, 230, 288 DL: 12.5, 25, 64, 230, 288, 640 |  |  |
| Antenna gain (dBi) |  | 2-5 | 3-7 | 2-18 | BS: (LCX) MS: +5 | BS: (LCX) MS: +5 |  |  |
| Polarization |  | vertical | vertical | vertical, oblique | NA | NA |  |  |
| Transmitting radiation power (dBm) |  | 16 | 15 | 17 | BS: +33 MS: +36 | BS: +27 MS: +36 |  |  |
| e.i.r.p. (dBm) |  | 18-21 | 18-22 | 19 - 35 | BS: NA MS: +41 | BS: NA MS: +41 |  |  |
| Receiving noise figure (dB) Fixed Station/Mobile/Portable |  | <10 | <12 | <8 | < 10 | < 10 |  |  |
| Transmission data rate (kb/s) |  | NA | NA | 8x22.8 | DL: 384+307.2 UL: 384+307.2 | DL: 384+384 UL: 384+307.2 |  |  |
| Transmission distance (km) |  | max 20 | max 30 | max 20 | LCX 0.001-0.002 (BS interval: 30) | LCX 0.001-0.002 (BS interval: 30) |  |  |
| Modulation |  | π/4QPSK | π/4DQPSK | GMSK | π/4QPSK | DL: π/4QPSK UL: π/4QPSK and QPSK |  |  |
| Multiplexing method |  | FDMA | TDMA | TDMA | DL: TDM UL: TDMA FDD | DL: TDM UL: TDMA FDD |  |  |
| Reception quality |  | NA | NA | NA | BER < 10-4 | BER < 10-4 |  |  |
| Applications | Voice/Dispatch |  |  |  | X | X |  |  |
|  | Maintenance |  |  |  | X | X |  |  |
|  | Train Control |  |  |  | X |  |  |  |
|  | Emergency |  |  |  |  |  |  |  |
| Scenarios | Railway line |  |  |  | X | X |  |  |
|  | Railway station, |  |  |  | X | X |  |  |
|  | Shunting yard |  |  |  | X | X |  |  |
|  | Maintenance Base |  |  |  |  |  |  |  |

### 4.1.3	Trunked Radio

Editor’s note: Administrations are encourage to provide a shore paragraph here to introduce their Trunked Radio in this sub-section.

Table 4.1.3-1

Parameter and usage of Trunked Radio

| # |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use by which Administration(s) |  | KOR | KOR |  |  |  |  |  |
| Specific name (if have) |  | ASTRO | TETRA |  |  |  |  |  |
| Frequency Range (MHz) |  | 806-811 851-865 | 806-811 851-865 |  |  |  |  |  |
| Radiocommunication Standards |  |  |  |  |  |  |  |  |
| Channel separation (kHz) |  | 12.5 | 4x25 |  |  |  |  |  |
| Antenna gain (dBi) |  | 3 | 3 |  |  |  |  |  |
| Polarization |  | NA | NA |  |  |  |  |  |
| Transmitting radiation power (dBm) |  | BS: 70W Cab: 30W UE: 3W | BS: 25W Cab: 3W UE: 1W |  |  |  |  |  |
| e.i.r.p. (dBm) |  | NA | NA |  |  |  |  |  |
| Receiving noise figure (dB) Fixed Station/Mobile/Portable |  | -118 | BS: -125 UE: -112 |  |  |  |  |  |
| Transmission data rate (kb/s) |  | 9.6 | 36 |  |  |  |  |  |
| Transmission distance (km) |  | NA | NA |  |  |  |  |  |
| Modulation |  | C4FM | π/4 DQPSK |  |  |  |  |  |
| Multiplexing method |  | FDMA | TDMA |  |  |  |  |  |
| Reception quality |  | NA | NA |  |  |  |  |  |
| Applications | Voice/Dispatch |  |  |  |  |  |  |  |
|  | Maintenance |  |  |  |  |  |  |  |
|  | Train Control |  |  |  |  |  |  |  |
|  | Emergency |  |  |  |  |  |  |  |
| Scenarios | Railway line |  |  |  |  |  |  |  |
|  | Railway station, |  |  |  |  |  |  |  |
|  | Shunting yard |  |  |  |  |  |  |  |
|  | Maintenance Base |  |  |  |  |  |  |  |

### 4.1.4	GSM-R

Editor’s note: Administrations are encourage to provide a shore paragraph here to introduce their GSM-R systems in this sub-section.

Table 4.1.4-1

Parameter and usage of GSM-R

| # |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use by which Administration(s) |  | CHN | CEPT |  |  |  |  |  |
| Specific name (if have) |  | GSM-R | GSM-R |  |  |  |  |  |
| Frequency Range (MHz) |  | 885 - 889 930 - 934 | 876-880.1 921-925.1 |  |  |  |  |  |
| Radiocommunication Standards |  |  |  |  |  |  |  |  |
| Channel separation (kHz) |  | 200 | 12,5 kHz |  |  |  |  |  |
| Antenna gain (dBi) |  | MS: ≥0 BS: 17(21) |  |  |  |  |  |  |
| Polarization |  | dual |  |  |  |  |  |  |
| Transmitting radiation power (dBm) |  | UE: 33 Cab: 39 BS: 46 |  |  |  |  |  |  |
| e.i.r.p. (dBm) |  | NA |  |  |  |  |  |  |
| Receiving noise figure (dB) Fixed Station/Mobile/Portable |  | NA |  |  |  |  |  |  |
| Transmission data rate (kb/s) |  | NA | 4.8 |  |  |  |  |  |
| Transmission distance (km) |  | NA | min 2 |  |  |  |  |  |
| Modulation |  | GMSK |  |  |  |  |  |  |
| Multiplexing method |  | TDMA |  |  |  |  |  |  |
| Reception quality |  | SNR MS: ≤-104 SNR BS: ≤ -110 | SNR: –107 |  |  |  |  |  |
| Applications | Voice/Dispatch | X |  |  |  |  |  |  |
|  | Maintenance | X |  |  |  |  |  |  |
|  | Train Control | X |  |  |  |  |  |  |
|  | Emergency | X |  |  |  |  |  |  |
| Scenarios | Railway line | X |  |  |  |  |  |  |
|  | Railway station, | X |  |  |  |  |  |  |
|  | Shunting yard |  |  |  |  |  |  |  |
|  | Maintenance Base | X |  |  |  |  |  |  |

### 4.1.5	LTE based RSTT

Editor’s note: Administrations are encourage to provide a shore paragraph here to introduce their LTE based RSTT systems in this sub-section.

Table 4.1.5-1

Parameter and usage of LTE based RSTT

| # |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use by which Administration(s) |  | KOR |  |  |  |  |  |  |
| Specific name (if have) |  | LTE-R |  |  |  |  |  |  |
| Frequency Range (MHz) |  | 718 - 728 773 - 783 |  |  |  |  |  |  |
| Radiocommunication Standards |  |  |  |  |  |  |  |  |
| Channel separation (kHz) |  | 10 MHz |  |  |  |  |  |  |
| Antenna gain (dBi) |  | NA |  |  |  |  |  |  |
| Polarization |  | NA |  |  |  |  |  |  |
| Transmitting radiation power (dBm) |  | UE: 2 BS: 80 |  |  |  |  |  |  |
| e.i.r.p. (dBm) |  | NA |  |  |  |  |  |  |
| Receiving noise figure (dB) Fixed Station/Mobile/Portable |  | NA |  |  |  |  |  |  |
| Transmission data rate (kb/s) |  | DL: 75000 UL: 37000 |  |  |  |  |  |  |
| Transmission distance (km) |  | NA |  |  |  |  |  |  |
| Modulation |  | DL: OFDMA UL: SC-FDMA |  |  |  |  |  |  |
| Multiplexing method |  | TFD FDD |  |  |  |  |  |  |
| Reception quality |  |  |  |  |  |  |  |  |
| Applications | Voice/Dispatch |  |  |  |  |  |  |  |
|  | Maintenance |  |  |  |  |  |  |  |
|  | Train Control |  |  |  |  |  |  |  |
|  | Emergency |  |  |  |  |  |  |  |
| Scenarios | Railway line |  |  |  |  |  |  |  |
|  | Railway station, |  |  |  |  |  |  |  |
|  | Shunting yard |  |  |  |  |  |  |  |
|  | Maintenance Base |  |  |  |  |  |  |  |

### 4.1.6	Emergency System

Editor’s note: Administrations are encourage to provide a shore paragraph here to introduce their Emergency System in this sub-section.

Table 4.1.6-1

Parameter and usage of Emergency Systems

| # |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use by which Administration(s) |  | J | J | KOR |  |  |  |  |
| Specific name (if have) |  | Japan REMCOS | Japan EARS | Korea TRPD |  |  |  |  |
| Frequency Range (MHz) |  | 140-144, 146-149.9, 150.05-156.4875, 156.8375-160 | 370-380 | 433.3125 |  |  |  |  |
| Radiocommunication Standards |  |  |  |  |  |  |  |  |
| Channel separation (kHz) |  | 20 | 6.25 | 1x12.5 |  |  |  |  |
| Antenna gain (dBi) |  | [TBD] | +1 | 3 |  |  |  |  |
| Polarization |  | vertical | vertical | vertical |  |  |  |  |
| Transmitting radiation power (dBm) |  | [TBD] | [TBD] | 36 |  |  |  |  |
| e.i.r.p. (dBm) |  | [TBD] | [TBD] | 39 |  |  |  |  |
| Receiving noise figure (dB) Fixed Station/Mobile/Portable |  | <10 | <10 | <2 |  |  |  |  |
| Transmission data rate (kb/s) |  | [TBD] | [TBD] | 8 |  |  |  |  |
| Transmission distance (km) |  | [TBD] | [TBD] | 4 |  |  |  |  |
| Modulation |  | [TBD] | [TBD] | GMSK |  |  |  |  |
| Multiplexing method |  | [TBD] | [TBD] | Single |  |  |  |  |
| Reception quality |  | [TBD] | [TBD] |  |  |  |  |  |
| Applications | Voice/Dispatch |  |  |  |  |  |  |  |
|  | Maintenance |  |  |  |  |  |  |  |
|  | Train Control |  |  |  |  |  |  |  |
|  | Emergency | X | X |  |  |  |  |  |
| Scenarios | Railway line | X | X |  |  |  |  |  |
|  | Railway station, | X | X |  |  |  |  |  |
|  | Shunting yard |  |  |  |  |  |  |  |
|  | Maintenance Base |  |  |  |  |  |  |  |

### [4.1.X	Possible other train radios]

Editor’s note: Administrations are encourage to provide a shore paragraph here to introduce their other type of Train radio system in this sub-section.

Table 4.1.X-1

Parameter and usage of [XXX Systems]

| # |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use by which Administration(s) |  |  |  |  |  |  |  |  |
| Specific name (if have) |  |  |  |  |  |  |  |  |
| Frequency Range (MHz) |  |  |  |  |  |  |  |  |
| Radiocommunication Standards |  |  |  |  |  |  |  |  |
| Channel separation (kHz) |  |  |  |  |  |  |  |  |
| Antenna gain (dBi) |  |  |  |  |  |  |  |  |
| Polarization |  |  |  |  |  |  |  |  |
| Transmitting radiation power (dBm) |  |  |  |  |  |  |  |  |
| e.i.r.p. (dBm) |  |  |  |  |  |  |  |  |
| Receiving noise figure (dB) Fixed Station/Mobile/Portable |  |  |  |  |  |  |  |  |
| Transmission data rate (kb/s) |  |  |  |  |  |  |  |  |
| Transmission distance (km) |  |  |  |  |  |  |  |  |
| Modulation |  |  |  |  |  |  |  |  |
| Multiplexing method |  |  |  |  |  |  |  |  |
| Reception quality |  |  |  |  |  |  |  |  |
| Applications | Voice/Dispatch |  |  |  |  |  |  |  |
|  | Maintenance |  |  |  |  |  |  |  |
|  | Train Control |  |  |  |  |  |  |  |
|  | Emergency |  |  |  |  |  |  |  |
| Scenarios | Railway line |  |  |  |  |  |  |  |
|  | Railway station, |  |  |  |  |  |  |  |
|  | Shunting yard |  |  |  |  |  |  |  |
|  | Maintenance Base |  |  |  |  |  |  |  |

## 4.2	Radiocommunication systems used for Train positioning information

Editor’s note: Brief introduction is needed here to describe each Radiocommunication system used for Train positioning information systems.

### 4.2.1	Position System

Editor’s note: Administrations are encourage to provide a brief introduction for their position systems in this sub-section.

Table 4.2.1-1

Parameter and usage of Position Systems

| # |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use by which Administration(s) |  |  |  |  |  |  |  |  |
| Specific name (if have) |  |  |  |  |  |  |  |  |
| Frequency Range (MHz) |  |  |  |  |  |  |  |  |
| Channel separation (kHz) |  |  |  |  |  |  |  |  |
| Antenna gain (dBi) |  |  |  |  |  |  |  |  |
| Polarization |  |  |  |  |  |  |  |  |
| Transmitting radiation power (dBm) |  |  |  |  |  |  |  |  |
| e.i.r.p. (dBm) |  |  |  |  |  |  |  |  |
| Receiving noise figure (dB) |  |  |  |  |  |  |  |  |
| Transmission data rate (kb/s) |  |  |  |  |  |  |  |  |
| Transmission distance (km) |  |  |  |  |  |  |  |  |
| Modulation |  |  |  |  |  |  |  |  |
| Multiplexing method |  |  |  |  |  |  |  |  |
| Reception quality |  |  |  |  |  |  |  |  |
| Applications | Balises |  |  |  |  |  |  |  |
|  | Loops/Leaky cable |  |  |  |  |  |  |  |
|  | Annunciators |  |  |  |  |  |  |  |
|  | Radar Scanners |  |  |  |  |  |  |  |
|  | Axle counters |  |  |  |  |  |  |  |
| Scenarios Applications | Railway line |  |  |  |  |  |  |  |
|  | Railway station, |  |  |  |  |  |  |  |
|  | Shunting yard |  |  |  |  |  |  |  |
|  | Maintenance Base |  |  |  |  |  |  |  |

## 4.3	Radiocommunication systems used for Train remote systems

Editor’s note: Brief introduction is needed to describe each Radiocommunication system used for train Remote systems.

### 4.3.X	Shunting System

Table 4.2.1-1

Parameter and usage of Shunting Systems

| # |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use by which Administration(s) |  |  |  |  |  |  |  |  |
| Specific name (if have) |  |  |  |  |  |  |  |  |
| Frequency Range (MHz) |  |  |  |  |  |  |  |  |
| Radiocommunication Standards |  |  |  |  |  |  |  |  |
| Channel separation (kHz) |  |  |  |  |  |  |  |  |
| Antenna gain (dBi) |  |  |  |  |  |  |  |  |
| Polarization |  |  |  |  |  |  |  |  |
| Transmitting radiation power (dBm) |  |  |  |  |  |  |  |  |
| e.i.r.p. (dBm) |  |  |  |  |  |  |  |  |
| Receiving noise figure (dB) Fixed Station/Mobile/Portable |  |  |  |  |  |  |  |  |
| Transmission data rate (kb/s) |  |  |  |  |  |  |  |  |
| Transmission distance (km) |  |  |  |  |  |  |  |  |
| Modulation |  |  |  |  |  |  |  |  |
| Multiplexing method |  |  |  |  |  |  |  |  |
| Reception quality |  |  |  |  |  |  |  |  |
| Applications | Shunting |  |  |  |  |  |  |  |
| Scenarios | Railway line |  |  |  |  |  |  |  |
|  | Railway station, |  |  |  |  |  |  |  |
|  | Shunting yard |  |  |  |  |  |  |  |
|  | Maintenance Base |  |  |  |  |  |  |  |

## 4.4	Radiocommunication systems used for Train surveillance

Editor’s Note: Brief introduction is needed to describe each Radiocommunication system used for Train surveillance systems.

### 4.4.1	Surveillance System

Table 4.4.1-1

Parameter and usage of Surveillance systems

| # |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use by which Administration(s) |  | [J] | [J] | KOR | J | J |  |  |
| Specific name (if have) |  | [W-Band] | [mmBH] |  | MVT | PMS |  |  |
| Frequency Range (GHz) |  | [43.5 – 47] | [92-94. 94.1-100, 102-109.5] | 18.86-18.92 19.20-19.26 | 43.5 - 43.7 | 57 - 66 |  |  |
| Radiocommunication Standards |  | [NA] | [NA] | 6x10000 | 40000 | 125000 |  |  |
| Channel separation (kHz) |  | [1.0 - 1.5] | [1] | NA | [TBD] | BS: 3.5 MS: 7 |  |  |
| Antenna beamwidth (degree) |  | [32] | [44] | NA | 33 | BS: 31 MS: 26 |  |  |
| Antenna gain (dBi) |  | [circular] | [linear] | NA | Linear | Linear |  |  |
| Polarization |  | [10] | [10] | 100 | +1 | +10 |  |  |
| Transmitting radiation power (dBm) |  | [42] | [54] | NA | +34 | BS: +41 MS: +36 |  |  |
| e.i.r.p. (dBm) |  | [<10] | [<10] | NA | <20 | <8 |  |  |
| Receiving noise figure (dB) |  | [100] | [5-1010*1000 (Stationary), 1*1000 (Running)] | NA | NA | 100 |  |  |
| Transmission data rate (Mb/s) |  | [0-3.5] | [0.5-11 (Open), 3 (Tunnel)] | 1.5-2.5 | 0.6 | 0.1 |  |  |
| Transmission distance (km) |  | [64QAM, OFDM] | [PSK, QPSK 16QAM, 64QAM] | OFDM | FM | ASK |  |  |
| Modulation |  | [FDD] | [FDD/TDD] | NA | FDM | FDD |  |  |
| Multiplexing method |  | [NA] | [NA] | NA | NA | NA |  |  |
| Reception quality |  |  |  |  |  |  |  |  |
| Applications | CCTV:platform view/crossings | [X] | [X] |  | X | X |  |  |
|  | CCTV:coach/compartments | [X] | [X] |  |  |  |  |  |
|  | CCTV:device monitoring | [X] | [X] |  |  |  |  |  |
| Scenarios | Railway line | [X] | [X] |  |  |  |  |  |
|  | Railway station, | [X] | [X] |  | X | X |  |  |
|  | Shunting yard |  |  |  |  |  |  |  |
|  | Maintenance Base | [X] | [X] |  |  |  |  |  |

# 5	Technologies of RSTT

## 5.1	GSM-R based RSTT

GSM-R supports mobile radio connectivity between train and track and serves terminals mounted on or integrated in trains from base stations along the trackside. A description of GSM-R features and specifications can be found in UIC-GSM-R:

GSM-R, Global System for Mobile Communications – Railway or GSM-Railway is a wireless communications standard for railway communication and applications. A sub-system of European Rail Traffic Management System (ERTMS), it is used for communication between train and the track. GSM-R is built on GSM technology, and benefits from the economies of scale of its GSM technology.

The specifications were finalized in 2000, based on the European Union-funded MORANE (Mobile Radio for Railways Networks in Europe) project. The specification is being maintained by the International Union of Railways (UIC) project ERTMS. GSM-R is a secure platform for voice and data communication between railway operational staff, including drivers, dispatchers, shunting team members, train engineers, and station controllers. It delivers features such as group calls (VGCS); voice broadcast (VBS), location-based connections, and call pre-emption in case of an emergency. This will support applications such as cargo tracking, and passenger information services.

According to the GSM-R industry, GSM-R will be supported until 2025-2030.

## 5.2	TETRA based RSTT

Terrestrial Trunked Radio (TETRA) is a professional land mobile radio standard  specifically designed for use by government agencies, emergency services, public safety networks, rail transport, transport services and the military. TETRA is a European Telecommunications Standards Institute (ETSI) standard, first version published 1995. TETRA uses Time Division Multiple Access (TDMA) with PI/4 QPSK modulation with four user channels on one radio carrier and 25 kHz channel raster. Both point-to-point and point-to-multipoint transfer can be used. Digital data transmission is also defined in the standard.

TETRA Mobile Stations can communicate direct-mode operation (DMO) or using trunked-mode operation (TMO), using switching and management infrastructure (SwMI) made of TETRA base stations (TBS). As well as allowing direct communications in situations where network coverage is not available, DMO also includes the possibility of using a sequence of one or more TETRA terminals as relays. This functionality is called DMO gateway (from DMO to TMO) or DMO repeater (from DMO to DMO). In emergencies, this feature allows direct communications underground or in areas of bad coverage.

In addition to voice and dispatch services, the TETRA system supports several types of data communication. Status messages and short data services (SDS) are provided over the system's main control channel, while packet-switched data or circuit-switched data communication uses specifically assigned channels. TETRA provides for authentication of terminals towards infrastructure and vice versa. For protection against eavesdropping, air interface encryption and end-to-end, encryption is available. The common mode of operation is in a group-calling mode in which a single button push will connect the user to the users in a selected call group and/or a dispatcher.

TETRA has been successfully deployed in a number of high-speed and a large number of METRO projects around the world and is being considered in many European countries as well. A list of TETRA Rail projects is enclosed as Annex 1.

Studies conducted on TETRA train communication systems at speeds of up to 500 km/h show that the performance of the channels at higher speeds is not significantly different from that at lower speeds. This is due to the forward error correction applied, which has better performance at higher speeds. Fading causes bursts of errors for the duration of a fade, and TETRA compensates for this by interleaving bits over a timeslot so that the error bits during a fade are spread out in between ‘good’ bits before the error correction mechanism operates on the decoded information. As speed increases, whereas the fades become closer together, the duration of each fade becomes shorter, affecting fewer bits. An example of the TETRA system used for High speed Train communications is the Taiwan High Speed Rail (THSR) system that connects Taipei city in the  north to Kaohsiung city in the south, a distance of 
345 km. THSR’s service  operation speed is 300 km/h, but was designed and tested at
 315 km/h.  THSR has been in operation since January 2007. TETRA was also tested during the French TGV (Train à Grande Vitesse) with train speed at 574.8 km/h.

## 5.3	LTE based RSTT

LTE supports mobile broadband radio connectivity between base stations (eNBs) and terminals (UEs). Hence LTE is able to serve terminals being mounted on or being integrated in trains from base stations along the trackside. In addition, relaying and direct device-to-device (D2D) communications are also supported.

A description of LTE features up to and including Release 12 can be found in [M.2012]. In addition 3GPP has been working on the following LTE enhancements in Release 13 and 14, which might be relevant also for RSTT:

–	UE performance enhancements for high speed scenario, where the target moving speed is at least 350 km/h and at most 750 km/h, depending on candidate solution, which can be found in TR36.878.

–	Coverage enhancements with up to 2048 repetitions leading to ~20 dB coverage extension.

–	Narrowband operation with a minimum channel spacing of 200 kHz.

–	Multi-antenna transmissions with up to 32 steerable antenna ports, which can be used for beamforming to reach far away receivers.

–	Vehicle-to-vehicle (V2V) sidelink designed for direct communication with up to 500 km/h velocity.

–	Optimizations for vehicle-to-network/infrastructure/pedestrian (V2N/V2I/V2P) communication.

–	Latency reduction reducing both signalling and data transmission delays.

A summary of main technical characteristics of LTE, including eMTC and NB-IOT can be found in PCG37_11 - Reply letter to ITU-R WP 5D in reply to ITU-R WP 1A/TEMP/100 = RP-151138 on Report ITU-R SM.[SMART_GRID] on the Smart Grid project, which is submitted as a parallel contribution to this ITU-R WP 5A meeting.

Examples Technical characteristics

| Parameter | LTE |
| --- | --- |
| Frequency Range | From ~400 MHz up to ~6 GHz, see 3GPP TS36.101 |
| Channel separation | 1.4, 3, 5, 10, 15, 20 MHz carrier bandwidth |
| Antenna gain (dBi) | Not limited by 3GPP standards |
| Polarization | - |
| Transmitting radiation power (dBm) | UE power classes defined for 20, 23, 26 and 31 dBm BS power classes defined from below 11 dBm up to 38 dBm, no upper limit for Wide Area BSs |
| e.i.r.p. (dBm) | Not limited by 3GPP standards |
| Receiving noise figure (dB) | As per 3GPP 36.101 & 36.104 |
| Transmission data rate (kb/s) | DL: between 21.3 kbps (NB-IOT), 800 kbps (eMTC) and 21.2 Gbps (LTE) UL: between 62.5 kbps (NB-IOT), 1Mbps (eMTC) and 11.6Gbps (LTE) |
| Transmission distance (km) | max 100 km (eMTC, LTE) and 40 km (NB-IOT) |
| Modulation | DL: OFDM UL:SC-FDMA single-tone FDMA possible with NB-IOT, |
| Multiplexing method | FDD, TDD |
| … |  |

## 5.4	New Radio based RSTT

New Radio (NR) is being designed to provide enhanced and flexible connectivity between base stations and terminals, including direct device-to-device as well as backhaul links. In fact, NR is envisaged to expand and support diverse families of usage scenarios and applications including:

–	eMBB (enhanced Mobile Broadband)

–	mMTC (massive Machine Type Communications)

–	URLLC (Ultra-Reliable and Low Latency Communications)

Among the target deployment scenarios and requirements, NR design includes also enhanced support and performance for High speed trains and eV2X services.

Hence NR is expected to provide very efficient support of RSTT type of communications, i.e. improved serving of terminals being mounted on or being integrated in trains from base stations along the trackside.

The NR Study Item will complete in Release 14, followed by normative specification in Release 15 and 16.

The NR technical characteristics are not know yet; aspects related to 3GPP RAN requirements can be found in in TR 38.913 “Study on Scenarios and Requirements for Next Generation Access Technologies”, currently under development in 3GPP.

## 5.5	Millimetric wave based RSTT

Millimeter-wave radio technologies can provide broadband transmission capabilities which can transmit high-speed data for train control, command, operational information, monitoring data as well as video to the train crews to realize more secure and comfortable railway transport services. The platform monitoring systems using millimeter-waves have been widely used for safety operation of railway systems in many stations. There is urgent demand to monitor the inside of railway coaches in real time to avoid unexpected accident. Millimeter-wave technologies can provide multiplexed uncompressed high-definition video transmission from train to trackside and vice versa. The millimeter-wave antenna can be easily equipped in the train driver's and the conductor’s rooms because the size of antenna and transceiver can be made compact. The pencil beam of millimeter-wave antenna can reduce the frequency interference between on-board and trackside stations.

Editor’s note: This is the end of section 5.5.

Editor’s note: The texts below could be used to provide further materials for developing Chapter 5.

To reach the necessary safety level, today’s trains need on-board real time video surveillance to monitor and assess any critical or abnormal situation inside the Engine and the coaches, alongside the track or on platforms. On board real time information is also becoming mandatory to train operators. Further IP based RAN will replace the existing circuit Radio based GSM-R network for train to track communications as shown in figure below.

IP based systems needed for Rail communications to provide signalling and control systems will need to be dedicated systems to ensure safety and security.

A typical architecture of IP based RSTT is shown below:

Similarly TETRA operates in 400 MHz (380-470 MHz) and 800 MHz 
(806-824/851-869 MHz) and a number of trunking channels of 25 KHz each are allocated depending on the need for each train system.

Following IMT bands could be considered for LTE system for train to track communications:

a)	450-470 MHz, traditionally a NB LMR band, but also harmonized for IMT in many countries under the ITU Radio Regulations.

b)	Parts of 900 MHz IMT band (880-915/925-960 MHz).

c)	Parts of 700 MHz IMT band (718-728/773-783 MHz).

Other IMT bands such as 1 800 MHz could also be considered in countries where 450-
470 MHz or 900 MHz band is not available.

450-470 MHz

450-470 MHz is currently the lowest spectrum band currently identified for IMT in the Radio Regulations and can provide one 5+5 MHz or multiple 1.4+1.4 MHz FDD LTE channels, which are capable of supporting up to 13 Mbps in the downlink and 3.8 Mbps in the uplink supporting both data and voice. In addition, this band is used and very suitable for TETRA communications.

The band provides excellent coverage and bandwidth suitable for train control and communications between train and trackside. However, when introducing LTE, there are Interference challenges (low separation between the uplink and downlink and a short guard band to the TV channels operating above 470 MHz) and a limited ecosystem for LTE.

873-915/918-960 MHz

Many countries currently use part of 873-890/918-935 MHz for train to track communications using GSM-R technology. When considering systems for train to track communications  based on broadband channels , that enable 5+5 MHz, 3+3 MHz or 1.4+1.4 MHz that are supported by technologies such as LTE, measures should be developed to insure no significant interference occurs to existing services or systems operating in same or adjacent spectrum. An example of the above is when a CDMA/WCDMA/LTE network operating within 824-879 or 824-889 MHz.

Editor’s note: This is the end of the above mentioned texts.

# 6	Spectrum

## 6.1	Current spectrum usage of RSTT

Editor’s note: The structure of section 6.1 has been agreed in November 2016 meeting. Summary tables of Spectrum usage of RSTT need to be further improved base on Chapter 4.

### 6.1.1	Radiocommunication systems used for Train radio

Table 6.1.1-1

Current spectrum usage of Radiocommunication systems used for Train radio

| Systems’ name | Spectrum usage In Region 1 (Member States amount: XX ) |  |  |  | Spectrum usage In Region 2 (Member States amount: YY ) |  |  |  | Spectrum usage In Region 3 (Member States amount: ZZ ) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [Sys1] |  | Freq From (MHz) | Freq To (MHz) | Member State amount Subtotal |  | Freq From (MHz) | Freq To (MHz) | Member State amount Subtotal |  | Freq From (MHz) | Freq To (MHz) | Member State amount Subtotal |
|  | Subtotal | 140 | 470 | 50 | Subtotal | 140 | 470 | 70 | Subtotal | 140 | 470 | 45 |
|  | Group1 | 140 | 150 | 50 | Group1 | 140 | 150 | 66 | Group1 | 140 | 150 | 44 |
|  | Group2 | 407 | 430 | 9 | Group2 | 407 | 430 | 54 | Group2 | 407 | 430 | 20 |
|  | Group3 | 450 | 470 | 48 | Group3 | 450 | 470 | 69 | Group3 | 450 | 470 | 39 |
|  | Group4 |  |  |  | Group4 |  |  |  | Group4 |  |  |  |
| [Sys2] | Subtotal |  |  |  | Subtotal |  |  |  | Subtotal |  |  |  |
|  | Group1 |  |  |  | Group1 |  |  |  | Group1 |  |  |  |
|  | Group2 |  |  |  | Group2 |  |  |  | Group2 |  |  |  |
|  | Group3 |  |  |  | Group3 |  |  |  | Group3 |  |  |  |
|  | Group4 |  |  |  | Group4 |  |  |  | Group4 |  |  |  |
| [Sys3] | Subtotal |  |  |  | Subtotal |  |  |  | Subtotal |  |  |  |
|  | Group1 |  |  |  | Group1 |  |  |  | Group1 |  |  |  |
|  | Group2 |  |  |  | Group2 |  |  |  | Group2 |  |  |  |
|  | Group3 |  |  |  | Group3 |  |  |  | Group3 |  |  |  |
|  | Group4 |  |  |  | Group4 |  |  |  | Group4 |  |  |  |

### 6.1.2	Radiocommunication systems used for Train positioning information

### 6.1.3	Radiocommunication systems used for Train remote systems

### 6.1.4	Radiocommunication systems used for Train surveillance

## 6.2	Planning spectrum usage of RSTT

Editor’s note: This section need to be further improved.

## 6.3	spectrum requirements of RSTT

Editor’s note: Administrations are encouraged to provide their spectrum requirements for each of the RSTT systems through their contributions. And this section need to be further improved according to those contributions.

# 7	Summary

ANNEX 1

Vocabulary of Terms

Editor’s note: This section need to be further improved by SWG 5A2-1 meeting in May 2017.

3G		Third Generation Mobile Cellular

CBTC		Communications Based Train Control

DMO		Direct Mode Operation

ERTMS		European Railway Traffic Management System

ETSI		European Telecommunications Standards Institute

ETCS		European Train Control System

GSM-R		GSM for Railways

LMR		Land Mobile Radio

LTE		Long Term Evolution (4G cellular technology based on OFDM)

NB		Narrow Band (typically 25 KHz)

OFDM		Orthogonal Frequency Division Multiplexing

OFDMA		Orthogonal Frequency Division Multiple Access

QPSK		Quadrature Phase Shift Keying

RAN		Radio Access Network

RSTT		Railway Radiocommunication systems between train and trackside

S&C		Signalling and Control in Train and track management

SwMI		Switching and Management Infrastructure in a TETRA system

TBS		TETRA base Station

TDMA		Time Division Multiple Access

TETRA		Terrestrial trunk Radio based on ETSI standard

TGV		Train à Grande Vitesse – Very High speed Train

THSR		Taiwan High Speed Rail

TMO		Trunk Mode Operation (in TETRA)

UIC		Union Internationale des Chemins de fer-(International Union of Railways)

UE		User Equipment

VBS		Voice Broadcast (in GSM-R)

VGCS		Voice Group Call (in GSM-R)

WiFi		Wireless Fidelity – a broadband wireless Local area network

ANNEX 2

Summary of Response to the Questionnaire on the Usage of Railway Radiocommunication Systems

Editor’s note: This section is the outcome of DG-Questionnaire of SWG-5A-2-1 in November 2016.

# A2.1	Introduction

WRC-15 adopted Resolution 236 (WRC-15), which invites ITU-R to study the spectrum needs, technical operational characteristic and implementation of railway radiocommunication systems between train and trackside. Resolution 809 (WRC-15) decided that the agenda item 1.11 of WRC-19 is ‘to take necessary actions, as appropriate, to facilitate global or regional harmonized frequency bands to support railway radiocommunication systems between train and trackside within existing mobile service allocations, in accordance with Resolution 236 (WRC-15)’.

The technical and operational characteristics used by railway radiocommunication systems vary by country. In order to get a clear overview on these systems, it is necessary to summarize the technical and operational characteristics for railway radiocommunication systems between train and trackside for traffic control, passenger safety, and security for train operation. ITU-R Working Party 5A is currently studying the usage of railway radiocommunication systems. Gathering the information on the usage of such systems would facilitate the preparation for WRC-19 agenda item 1.11.

On 3rd June 2016, The Radiocommunication Bureau invited by Administrations of Member States of the ITU to provide answers to questions by Administrative Circular 5/LCCE/60.  Response to the Questionnaire from each Administration of Member States is compiled in the document 5A/273R2.

# A2.2	Summary of the situation of feedback of the questionnaire

Radiocommunication Bureau has received responses from 25 Administrations and one ITU regional organization by 31 October 2016, as follows.

Armenia,

Australia

Bosnia-Herzegovina

Canada

China

Czech Republic

Finland

France

Germany

Hungary

Italy

Japan

Republic of Korea

Malta

Netherlands

Norway

Qatar

Spain

Sweden

Switzerland

Thailand

United Arab Emirates

United Kingdom

Uzbekistan

Viet Nam

CEPT

# A2.3	Summary table of answers to each question

The answers are compiled and summarized to the excel format table is attached below.

# A2.4	Original questionnaire answers

The original detailed information in 5A/273(Rev.2) is compiled as follows.

ANNEX 3

Consideration of the Doppler Effect in railway radiocommunication systems between high-speed trains and tracksides

Editor’s note: This Annex is based on one Contribution Document (5A/48) of WP 5A in May 2016, and need to be further considered in WP 5A’s November meeting in 2016.

# A3.1	Introduction

WRC-15 (November 2015) adopted Resolution ITU-R СОМ6/12 (WRC-15) which included new item 1.11 in the agenda of the next WRC-19: "To take appropriate measures, as the case may be, facilitating harmonization of frequency bands on global and regional levels, in order to ensure operation of railway radiocommunication systems between train and tracksides within existing allocations to mobile service".

This Resolution "invites ITU-R to undertake a study of spectrum requirements, technical and operational characteristics and implementation issues for railway radiocommunication systems between train and tracksides". In addition, during the 2012-2015 study period, Chinese Administration submitted to the ITU-R WP 3K the Document R12-WP3K-C-0059 "Draft new ITU-R Question for SG 3. Methods for propagation prediction for high-speed train radio services". The document noted the need to study possible impact of the Doppler shift on the operation of railway radiocommunication systems between high-speed trains and tracksides. In the previous study period, the Chinese Administration submitted to ITU-R WP 3K the Document R12-WP3K-C-0093 "Preliminary draft new Recommendation ITU-R P.XXXX-VERSION. Method for propagation prediction for wireless radio systems on high-speed trains. This document also proposes to take into account possible impact of the Doppler shift on signal fading in railway radiocommunication systems between high-speed trains and tracksides.

The Documents R12-WP3K-C-0059 and R12-WP3K-C-0093 of the Chinese Administration did not get further consideration most likely due to the fact that there were no appropriate items in the WRC-15 agenda concerning the development of wireless radiocommunication systems for high-speed trains. Currently, agenda item 1.11 which is directly related to such systems is included in the Agenda of the forthcoming WRC-19.

The purpose of the contribution is the initial analysis of the need to take into account possible impact of the Doppler shift on the propagation prediction process for wireless communication links between high-speed trains and tracksides. This approach is based on approximate evaluation of the Doppler shift for different carrier frequencies and different train speeds. The evaluation of the Doppler shift is based on the known mathematical equation describing the Doppler Effect.

# A3.2	Evaluation of the Doppler shift for different frequency bands and different train speeds

The Doppler shift (Fd) for the case of moving objects (transmitter or receiver) relative each other with a certain speed (V), is calculated using the well-known equation:

Fd = ﴾│V×cos(θ)│×f﴿ /c,   Hz.	(1)

where:

V – 	speed of an object, m/s;

θ – 	angle between direction of an object motion (velocity vector) and the direction towards a stationary object, degrees (see Figure A3.2-1);

f – 	carrier frequency, Hz;

с – 	light velocity in free space, 3×108 m/s.

FIGURE A3.2-1

The Doppler shift measurement layout.

It can be seen from the equation (1) and Figure A3.2-1, that the Doppler shift has a maximum value when the angle θ has a minimum value, i.e. when the distance between two objects is sufficiently large. In general, this condition is mostly met as applied to the geometry of high-speed train motion, and the angle θ could be taken zero.

The Doppler shift was calculated using the equation (1) for train speeds in the range from 300 to 560 km/h and for carrier frequencies corresponding to the central frequency of the frequency bands allocated to the land mobile service in Region 1, i.e. 800 MHz, 1 800 MHz, 1 950 MHz, 2 100 MHz, 2 350 MHz, 2 595 MHz and 3 475 MHz. Calculated Doppler shifts for wireless radio systems between high-speed trains and tracksides are summarized in Table A3.2-1 and shown in Figure A3.2-2. It should be noted that Doppler shifts in Table A3.2-1 are absolute values. That is total Doppler shift in wireless radio systems between high-speed trains and tracksides will be twice the value of Table A3.2-1.

ТАBLE A3.2-1

Calculated Doppler shift in wireless radio systems between high-speed trains and tracksides

| Train speed V, km/h | Carrier frequency f, GHz |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0.8 | 1.8 | 2.1 | 2.235 | 2.595 | 3.475 | 45 | 97 |
|  | Doppler shift Fd , kHz |  |  |  |  |  |  |  |
| 300 | 0.222 | 0.5 | 0.583 | 0.653 | 0.721 | 0.965 | 12.5 | 26.9 |
| 320 | 0.237 | 0.533 | 0.622 | 0.696 | 0.769 | 1.03 | 13.3 | 28.7 |
| 340 | 0.252 | 0.567 | 0.661 | 0.74 | 0.817 | 1.094 | 14.2 | 30.5 |
| 360 | 0.267 | 0.6 | 0.7 | 0.783 | 0.865 | 1.158 | 15.0 | 32,3 |
| 380 | 0.281 | 0.633 | 0.739 | 0.827 | 0.913 | 1.223 | 15.8 | 34.1 |
| 400 | 0.296 | 0.667 | 0.778 | 0.87 | 0.961 | 1.287 | 16.7 | 35.9 |
| 420 | 0.311 | 0.7 | 0.817 | 0.914 | 1.009 | 1.351 | 17.5 | 37.7 |
| 440 | 0.326 | 0.733 | 0.856 | 0.957 | 1.057 | 1.416 | 18.3 | 39.5 |
| 460 | 0.341 | 0.767 | 0.894 | 1.001 | 1.105 | 1.48 | 19.2 | 41.3 |
| 480 | 0.356 | 0.8 | 0.933 | 1.044 | 1.153 | 1.544 | 20.0 | 43.4 |
| 500 | 0.37 | 0.833 | 0.972 | 1.088 | 1.201 | 1.609 | 20.8 | 44.9 |
| 520 | 0.385 | 0.867 | 1.011 | 1.131 | 1.249 | 1.673 | 21.7 | 46.7 |
| 540 | 0.4 | 0.9 | 1.05 | 1.175 | 1.298 | 1.738 | 22.5 | 48.5 |
| 560 | 0.415 | 0.933 | 1.089 | 1.219 | 1.346 | 1.802 | 23.3 | 50.3 |
| 580 |  |  |  |  |  |  |  | 52.1 |
| 600 |  |  |  |  |  |  |  | 53.9 |

FIGURE A3.2-2

The Doppler shift in a wireless radio system between high-speed trains and tracksides 
vs train speed for different carrier frequencies

Let us see Figure A3.2-3 to analyze the impact of the Doppler shift on the operation of land mobile radio systems in a high-speed train.

FIGURE A3.2-3

Analysis of impact of the Doppler shift on the operation of wireless radio systems 
between high-speed trains and tracksides

Let us assume that a user terminal (UT) is located in a high-speed train, and a base station (BS) is fixed. Let us also assume that for the certain moment of time radio communication is established using radio frequency f0n between UT and BS. Due to the Doppler effect, fixed BS will receive UT carrier frequency f0n subject to the Doppler shift Fd, i.e. f0n + Fd. Accordingly, the bandwidth of the wanted UT signal will be also shifted to the right on the frequency scale by the same value Fd, as it is shown in Figure A3.2-3. Since the channel carrier frequencies are located in such a way that their spectra are side-by-side to each other (as it is shown in the upper part of Figure A3.2-3), then with such Doppler shift of n-th channel to the right, a part of the wanted spectrum of UT channel could fall into the bandwidth of the adjacent (n+1)-th BS channel receiver (as it is shown in the lower part of Figure A3.2-3). This share of the wanted signal power of n-th channel in the bandwidth of the adjacent (n+1)-th channel is highlighted by shadowing in the lower part of Figure A3.2-3. It is obvious, that the share of this power is proportional to the shift of the wanted signal spectrum, that is, to the Fd. Therefore, this share of the power (percentage of the total power of the wanted signal spectrum) could be evaluated by the ratio of the Doppler shift Fd to the spectrum of wanted signal Δfs (assuming that the bandwidth of the BS receiver is equal to this value). Table A3.2-2 shows such evaluations for different standards of land mobile communications (and, accordingly, for different central frequencies and parameters of signal) with maximum train speed (500 km/h, or 139 m/s).

TАBLE A3.2-2

Summary of calculated impact of the Doppler shift on wireless radio systems 
between high-speed trains and tracksides

| Land mobile system (center frequency, МHz) | GSM-R (800) | GSM-1800 (1 800) | UMTS- 2100 (2 100) | WiMAX (2 350) | WiMAX (2 595) | WiMAX (3 475) | LTE (1 950) | LTE (2 350) | LTE (2 595) | mmw (45) | mmw (97) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| max(Fd), (kHz) | 0.37 | 0.833 | 0.972 | 1.088 | 1.201 | 1.609 | 0.904 | 1.088 | 1.201 | 20.8 | 44.9 |
| Δfs (MHz) | 0.2 | 0.2 | 3.84 | 9.2 | 9.2 | 9.2 | 9.0 | 9.0 | 9.0 | 35 | 250 |
| max(Fd)/Δfs, (%) | 0.185 | 0.417 | 0.025 | 0.012 | 0.013 | 0.017 | 0.01 | 0.012 | 0.013 | 0.059 | 0.00018 |

# A3.3	Conclusions

Based on data from Table A3.2-1, Table A3.2-2, and Figure A3.2-2 we can make the following conclusions:

–	carrier frequency shift due to the Doppler effect becomes noticeable only for train speeds about 500 km/h and for frequency 2 GHz and higher. Carrier frequency is shifted by ±1.609 kHz from its nominal value (for the WiMAX land mobile system) and by ±20.8 kHz for mmw;

–	although carrier shift at 97-GHz band is larger than that of lower frequency, the Doppler shift is compensated by the carrier tracking method due to larger symbol rate than
100 Mb/s.

–	taking into account the Doppler shift of the carrier frequency and bandwidth of one radio channel in the up-to-date land mobile systems, the share of the UT signal power falling into the bandwidth of receiver of adjacent BS channel does not exceed 0.18% for all considered standards of land mobile systems;

–	when developing propagation models for radio communication scenarios between 
high-speed train and trackside, the effect of the Doppler shift is negligible to be taken into account.

ANNEX 4

RSTT in Japan

# A4.1	Overview

The frequency 100 kHz-band, 150 MHz-band, 300 MHz-band and 400 MHz-band have been allocated to the RSTT for safety and stable train operation since 1960s. Regarding the requirement to the conventional RSTT, the broadband transmission capability has become one of important functions to provide high-speed data to the train crews and passengers to realize more secure and comfortable railway transport services. To realize these requests, digital train radio systems have been introduced to the high speed railway. And now many digital radio systems are used for RSTT. Furthermore, Japan has begun to study millimetre-wave. Microwave as well as millimetre-wave frequencies are well known as the spectrum resources supporting the broadband data signal transmission.

## A4.1.1	List of the RSTT

Table A4.1.1-1 is the list of major RSTT used in Japan.  This table shows name of system, frequency band, applications, and users of each RSTT.

Table A4.1.1-1

| Name of System | Frequency | Applications and Users of the system |
| --- | --- | --- |
| Train Radio System (TRS) | 150 MHz band 300 MHz band 400 MHz band | [Application] ･Traffic control information for drivers ･Automatic train control ･Vehicle status monitoring for maintenance crews ･Passenger guidance for conductors [Users] ･Train traffic controllers ･Train drivers and conductors ･Automatic train control equipment ･Station managers ･Maintenance crews |
| Radiocommunication system for High Speed Train (RHST) | 400 MHz band | [Application] ･Traffic control ･Automatic train control ･Vehicle status monitoring, Passenger guidance, Cabin monitoring [Users] ･Train traffic controllers ･Train drivers and conductors ･Automatic train control equipment ･Maintenance crews |
| Emergency Alarm Radio System (EARS) | 300 MHz band | [Application] ･Emergency signals from train or ground to trains to alert some dangers situations to surrounding drivers by buzzer [Users] ･Train drivers and conductors ･Train traffic controllers ･Station attendants |
| Radiocommunication system for Emergency Cut Off System (REMCOS) | 150 MHz band | [Application] ･Emergency signal from train to ground to stop trains by powering Cut Off [Users] ･Train drivers and conductors ･Train traffic controllers ･Ground maintenance crews |
| Radiocommunication system for Electronic Blocking System (REBS) | 300 MHz band | [Application] ･Trigger signal transmission from train to ground to control block section [Users] ･Train drivers ･Ground Interlocking equipment |
| Radiocommunication system for Japan Radio Train Control system (JRTC Radio) | 300 MHz band | [Application] ･Automatic train control [Users] ･Ground Train controller equipment ･On-board train controller equipment |
| Yard Radio (YR) | 150 MHz band 300 MHz band 400 MHz band | [Application] ･Vehicle maintenance ･Shunting [Users] ･Train drivers ･Ground maintenance crews |
| Millimetre wave Video Transmission system (MVT) | 43 GHz band | [Application] ･Platform Monitoring [Users] ･Train drivers and conductors |

## A4.1.2	Frequency usage

Table A4.1.2-1 shows details of frequency usage of RSTT used in Japan. This table also shows frequency usage of future RSTT under study.

Table A4.1.2-1

Frequency usage of RSTT in Japan

| Frequency bands | Systems |
| --- | --- |
| 140 - 144 MHz, 146 - 148 MHz, 148 - 149.9 MHz | Train Radio System, Radiocommunication system for Emergency Cut-Off system, Yard Radio |
| 150.05 - 156.4875 MHz | Radiocommunication system for Emergency Cut-Off system, Yard Radio |
| 156.8375 - 160 MHz | Radiocommunication system for Emergency Cut-Off system, Yard Radio |
| 335.4 - 340 MHz | Train Radio System, JRTC radio, Radiocommunication system for Electronic Blocking System, Yard Radio |
| 340 - 360 MHz | Train Radio System, JRTC radio, Radiocommunication system for Emergency Cut-Off system, Yard Radio |
| 360 - 370 MHz | Radiocommunication system for Emergency Cut-Off system, Yard Radio |
| 370 - 380 MHz | Emergency Alarm Radio System, Yard Radio |
| 380 - 387 MHz, 387 – 390 MHz, 390 - 399.9 MHz | Yard Radio |
| 410 - 420 MHz | Radiocommunication system for High Speed Train, |
| 414.4 - 415.5 MHz | Train Radio System |
| 450 - 455 MHz | Radiocommunication system for High Speed Train |
| 455 - 456 MHz, 456 - 459 MHz, 459 - 460 MHz, 460 - 470 MHz | Yard Radio |
| 43.5 - 47 GHz | Millimetre wave Video Transmission system, Future radio communication system for high speed trains (under study) |
| 92 - 94 GHz, 94.1 - 100 GHz, 102 - 109.5 GHz | Future radio communication system for high speed trains (under study) |

### A4.1.3	Operational environment

Radio propagation characteristics in railway environment depend on the type of track surfaces (e.g. ballast track, concrete slab track, etc.).

Table A4.1.3-1 and Table A4.1.3-2 shows typical railway alignment, size of propagation area and location of antennae for RSTTs. Figure A4.1.3-1 shows an example of cross section view of railway track.

Table A4.1.3-1

Operational environment (deployment scenarios) in inter-city railway lines or urban transport systems

| Parameters | Values |
| --- | --- |
| Antenna height | Base station : [5 m - 15 m] from rail surface level Mobile station : [3.5 m - 4 m] from rail surface level |
| Maximum train speed | 160 km/h |
| Roadbed width | Typ. [10] m |
| Vehicle width | Max. 3.0 m |
| Vehicle height | Max. 4.1 m |
| Minimum Radius of curve | Typ. 2200 m (Min. 400 m in railway station area) |
| Minimum Vertical radius of curve | Typ. 2000 m |
| Gradient of track | 35‰ |
| Maximum Superelevation of track | 105mm @ rail gauge = 1067mm |

Table A4.1.3-2

Operational environment (deployment scenarios) in high speed railway line

| Parameters | Values |
| --- | --- |
| Antenna height | Base station(LCX) : [0 m - 3 m] from rail surface level Mobile station : [0.6 m - 1 m] from rail surface level |
| Maximum train speed | 320 km/h |
| Roadbed width | Typ. [12] m |
| Vehicle width | Max. 3.4 m |
| Vehicle height | Max. 4.5 m |
| Minimum Radius of curve | Typ. 4000 m |
| Minimum Vertical radius of curve | Typ. 10000 m |
| Maximum Gradient of track | 35‰ |
| Maximum Superelevation of track | 200 mm @ rail gauge = 1435 mm |

FIGURE A4.1.3-1

An example of cross section view of railway track (viaduct section)

# A4.2	150 MHz band RSTT

## A4.2.1	Train Radio System

### A4.2.1.1	System architecture

Train Radio System (TRS) is used for inter-city and inner-city train, but not for high speed train.  TRS carries traffic control information, train control command, passenger information and vehicle status monitoring data between trains and control centres.  In general, a control centre covers several railway lines and TRS accommodate some radio zones, which correspond to each railway line.

Figure A4.2.1.1-1 shows the architecture of TRS.   A set of radio frequencies is allocated to each line. There are some base stations in a zone, about 1 km - 40 km each according for propagation scenarios. The Central System connects commanders in the Control Centre and crews on-board. The commanders are able to inform drivers about train control issues. The controllers are also able to inform conductors about passenger guidance. Furthermore, data transmissions for vehicle status monitoring are available.

On-board antennas are on the top of each side of diver’s room. Base station antennas are on the top of poles beside the track and directing the rail along. In some train lines, the system is applied not only for voice and data communications but also for the train control and emergency cut-off system.

FIGURE A4.2.1.1-1

Architecture of Train radio system

## A4.2.1.2	Technical parameters

Table A4.2.1.2-1 summarizes technical characteristics of TRS operating in 150 MHz band.

Table A4.2.1.2-1

Technical characteristics of Digital Train Radio System (TRS) operating in 150 MHz band

| Parameters | Analog TRS | Digital TRS (Type 1) | Digital TRS (Type 2) | Digital TRS (Type 3) |
| --- | --- | --- | --- | --- |
| Frequency Range | 140 MHz - 144 MHz, 146 MHz - 149.9 MHz | 140 MHz - 144 MHz | 146 MHz - 149.9 MHz | 146 MHz - 149.9 MHz |
| Channel separation | 20 kHz | 6.25 kHz | 25 kHz | 6.25 kHz |
| Maximum Antenna gain | Base station : +15 dBi Mobile station : +4.2 dBi | Base station : + 11dBi Mobile station : +1 dBi | Base station: + 11dBi Mobile station : +1 dBi | Base station: + 11dBi Mobile station : +1 dBi |
| Polarization | Vertical | Vertical | Vertical | Vertical |
| Maximum Transmission power | Base station : +47 dBm Mobile station : +40 dBm | Base station : +40 dBm Mobile station : +30 dBm | Base station : +37dBm Mobile station : +30 dBm | Base station: +30 dBm Mobile station: +30 dBm |
| E.I.R.P. | Base station :+62 dBm Mobile station: +44.2 dBm | Base station : +51 dBm Mobile station: +31 dBm | Base station : +48 dBm Mobile station : +31 dBm | Base station : +41 dBm Mobile station: +31 dBm |
| Receiving noise figure | < 10 dB | < 10 dB | < 10 dB | < 10 dB |
| Data rate | (n/a) | 9.6 kbps | 32 kbps | 4.8 kbps |
| Reception quality | SNR > 45 dB | BER < 10-4 | BER < 10-4 | BER < 10-4 |
| Transmission distance(km) | 3 - 40 km | 1 - 3 km | 1 - 2 km | 1 - 3 km |
| Modulation | FM | π/4QPSK | π/4QPSK | 4FSK |
| Multiplexing method | FDD | FDMA or SCPC | TDMA | FDMA or SCPC |

## A4.2.2	Radiocommunication system for Emergency Cut-Off System

### A4.2.2.1	System architecture

Radiocommunication system for Emergency Cut-Off System (REMCOS) is used to avoid accidents.  The system is used for sending signal to a railway electrification system on ground and electric power for trains in some emergency aria is cut-off.

Figure A4.2.2.1-1 shows the system architecture of REMCOS. When a train driver confirms some emergency circumstances, the driver operates REMCOS on-board and emergency radio signal is transmitted to Central System in Control Centre via Base Stations. In Control Centre, the operational commander manually cuts off the power for trains near the emergency area or REMCOS automatically sends signal to a railway electrification system to cut off the power.

FIGURE A4.2.2.1-1

Architecture of Radiocommunication system for Emergency Cut Off System

Control

Centre

Central System

Base Station

Sub
Station

Emergency radio signal

Remote cut-off of supply current from a Substation

### A4.2.2.2	Technical parameters

Table A4.2.2.2-1 summarizes technical characteristics of REMCOS operating in 150 MHz band.

Table A4.2.2.2-1

Technical characteristics of Radiocommunication system for Emergency Cut-Off System
operating in 150 MHz band

| Parameters | Radiocommunication system for Emergency Cut-Off |
| --- | --- |
| Frequency Range | 140 MHz - 144 MHz, 146 MHz - 149.9 MHz, 150.05 MHz - 156.4875 MHz, 156.8375 MHz - 160 MHz |
| Channel separation | 20 kHz |
| Antenna gain | [to be filled out in future] |
| Polarization | Vertical |
| Maximum Transmission power | [to be filled out in future] |
| E.I.R.P. | [to be filled out in future] |
| Receiving noise figure | < 10 dB |
| Transmission distance (km) | [to be filled out in future] |
| Modulation | [to be filled out in future] |
| Multiplexing method | none |

### A4.2.3	Yard Radio

#### A4.2.3.1	System architecture

Yard radio (YR) is used for voice communication between operator in operation room and drivers on board to switch trains in yards or stations. Figure A4.2.3.1-1 shows the system architecture of YR.

FIGURE A4.2.3.1-1

Architecture of Yard radio

### A4.2.3.2	Technical parameters

Table A4.2.3.2-1 summarizes technical characteristics of YR operating in 150 MHz band.

Table A4.2.3.2-1

Technical characteristics of Yard radio operating in 150 MHz band

| Parameters | Yard radio |
| --- | --- |
| Frequency Range (MHz) | 140 MHz - 144 MHz 146 MHz - 149.9 MHz 150.05 MHz - 156.4875 MHz 156.8375 MHz - 160 MHz |
| Channel separation | 20 kHz |
| Antenna gain | [to be filled out in future] |
| Polarization | Vertical |
| Maximum Transmission power | [to be filled out in future] |
| E.I.R.P. | [to be filled out in future] |
| Receiving noise figure | < 10 dB |
| Reception quality | SNR > [30] dB |
| Transmission distance (km) | [to be filled out in future] |
| Modulation | FM |
| Multiplexing method | none |

# A4.3	300 MHz band RSTT

## A4.3.1	Train Radio System

### A4.3.1.1	System architecture

System architecture of the Train radio system (TRS) operating in 300 MHz band is similar to one of 150 MHz band described in A4.2.1.1.   But transmission distance from a base station is about
 1.5 km - 3 km in this system.

### A4.3.1.2	Technical parameters

Table A4.3.1.2-1 summarizes technical characteristics of TRS operating in 300 MHz band.

Table A4.3.1.2-1

Technical characteristics of Train radio system operating in 300 MHz band

| Parameters | Analog TRS | Digital TRS |
| --- | --- | --- |
| Frequency Range | 335.4 MHz - 360 MHz | 335.4 MHz - 360 MHz |
| Channel separation | 12.5 kHz | 6.25 kHz |
| Maximum Antenna gain | Base station : +11 dBi Mobile station : +1 dBi | Base station: + 11dBi Mobile station : +1 dBi |
| Polarization | Vertical | Vertical |
| Maximum Transmission power | Base station : +36 dBm Mobile station : +30 dBm | Base station : +36 dBm Mobile station : +24.8 dBm |
| E.I.R.P. | Base station : +47 dBm Mobile station : +31 dBm | Base station : +47 dBm Mobile station: +25.8 dBm |
| Receiving noise figure | < 10 dB | < 10 dB |
| Data rate | (n/a) | 9.6 kbps |
| Reception quality | SNR > 30 dB | BER < 10-4 |
| Transmission distance (km) | 1.5-3 km | 1.5 - 2 km |
| Modulation | FM | π/4QPSK |
| Multiplexing method | FDD | FDMA |

## A4.3.2	Emergency Alarm Radio System

### A4.3.2.1	System architecture

Emergency Alarm Radio System (EARS) is used to avoid accidents. When a train driver confirms some emergency circumstances on track such as line blocked objects, a train derailment, a fire, etc. the driver is expected to send alarm to approaching train’s drivers by EARS in order to avoid a secondary accident. When EARS is operated, emergency radio signal is directly transmitted to approaching trains as shown in Figure A4.3.2.1-1.

EARS is a very simple system. It consists of only mobile-stations on-board. The mobile-station consists of a radio equipment, a transmission button, and an antenna. When the transmission button is pressed, emergency radio signal is transmitted to approaching trains. When the approaching train’s mobile-station receives the signal, it sounds a warning tone and the driver should take necessary actions such as stopping the train. The emergency radio signal reaches nominally within 1 km radius. If it is difficult to reach the emergency to approaching train according to geographical scenario, such as in tunnels, repeaters are installed on trackside in order to expand the coverage of radio propagation.

EARS is used not only for a train to trains but also for ground to trains. In some stations, “Emergency train stop buttons” are prepared at platforms and anyone can push the button to stop trains around the station in emergency, such as someone falling down from platform. If the button is pushed, emergency radio signal as described above is transmitted from the station to approaching trains. And in some railway lines, EARS is also used to send emergency alarm to stop trains when earthquake occurs.

FIGURE A4.3.2.1-1

Architecture of Emergency Alarm Radio System

Emergency radio signal

### A4.3.2.2	Technical parameters

Table A4.3.2.2-1 summarizes technical characteristics of EARS operating in 300 MHz band.

Table A4.3.2.2-1

Technical characteristics of Emergency Alarm Radio System operating in 300 MHz band

| Parameters | Emergency Alarm Radio System |
| --- | --- |
| Frequency Range (MHz) | 370 MHz - 380 MHz |
| Channel separation | 6.25 kHz |
| Antenna gain | + 1 dBi |
| Polarization | Vertical |
| Maximum Transmission power | [to be filled out in future] |
| E.I.R.P. | [to be filled out in future] |
| Receiving noise figure | < 10 dB |
| Transmission distance (km) | Min. 1 km |
| Modulation | [to be filled out in future] |
| Multiplexing method | none |

## A4.3.3	Radiocommunication system for Emergency Cut-Off System

### A4.3.3.1	System architecture

System architecture of the Radiocommunication system for Emergency Cut-Off System (REMCOS) operating in 300 MHz band is similar to one of 150 MHz band REMCOS (see A4.2.2.1).

### A4.3.3.2	Technical parameters

Table A4.3.3.2-1 summarizes technical characteristics of REMCOS operating in 300 MHz band.

Table A4.3.3.2-1

Technical characteristics of Radiocommunication system for Emergency Cut-Off System
operating in 300 MHz band

| Parameters | Radiocommunication system for Emergency Cut-Off |
| --- | --- |
| Frequency Range (MHz) | 340 MHz - 370 MHz |
| Channel separation | 12.5 kHz |
| Antenna gain | [to be filled out in future] |
| Polarization | Vertical |
| Maximum Transmission power | [to be filled out in future] |
| E.I.R.P. | [to be filled out in future] |
| Receiving noise figure | < 10 dB |
| Transmission distance (km) | [to be filled out in future] |
| Modulation | [to be filled out in future] |
| Multiplexing method | none |

## A4.3.4	Radiocommunication system for Electronic Blocking System

### A4.3.4.1	System architecture

Radiocommunication system for Electronic Blocking System (REBS) is a radio communication system for Electric Blocking System. The Electric Blocking System is used at single-track railroads in rural areas. Figure A4.3.4.1-1 shows the system architecture. When a train stops at a station and is ready for departure, the diver pushes a button of a radio transmitter on-board. The radio transmitter sends radio signal “departure request” to Station Equipment through Radiative Pair Cable (RPC) antenna and Radio Equipment set up at the machine room of the station. The Station Equipment controls electric switch machines, leaving signals, and home signals then the driver can start the train in safety.

FIGURE A4.3.4.1-1

Architecture of Electronic Blocking System

RPC

Antenna

Radio

Equipment

Station

Equipment

Leaving

signal

Home

signal

#### A4.3.4.2	Technical parameters

Table A4.3.4.2-1 summarizes technical characteristics of REBS operating in 300 MHz band.

Table A4.3.4.2-1

Technical characteristics of Radiocommunication system for Electronic Blocking System
operating in 300 MHz band

| Parameters | Radiocommunication system for Electronic Blocking System |
| --- | --- |
| Frequency Range (MHz) | 335.4 MHz-340 MHz |
| Channel separation | 12.5 kHz |
| Antenna gain | + 1 dBi |
| Polarization | Vertical |
| Maximum Transmission power | +30 dBm |
| E.I.R.P. | +31 dBm |
| Receiving noise figure | < 10 dB |
| Transmission distance (km) | Max. 5 m |
| Modulation | FM |
| Multiplexing method | none |

## A4.3.5	JRTC Radio

### A4.3.5.1	System architecture

JRTC Radio is a sub-system of Japan Radio Train Control system (JRTC). JRTC is automatic train control system that is based on telecommunications between trains and base stations for train traffic management and railway infrastructure control.

Figure A4.3.5.1-1 shows the system architecture of JRTC. On train, the on-board controller detects its own location information that is consists of its location and speed. The mobile station sends the location information to the Ground Controller though Base Stations. With location information of trains, condition of electric switch machines, and condition of level crossing, the Ground Controller calculates the limit in which the train could run safely and sends the stopping limit to the train.
The Ground Controller controls the ground equipment as well, such as electric switch machines, level crossings, etc. On the train, the on-board controller calculates a brake pattern and an upper limit speed curve, by using its own brake performance to stop at the running limit directed by the Ground Controller. The on-board controller directs adequate train-speed to the train-driver and if train-speed exceeds the brake pattern, the on-board controller makes the train slow-down or stop by controlling the brake automatically. Requirements of basic function and system construction have been defined in Japanese Industrial Standards as JIS E 3801. JRTC corresponds to the train control system of ERTMS/ETCS Level 3 in Europe.

FIGURE A4.5.3.1-1

System Architecture of JRTC

Electric switch machine

Figure A4.3.5.1-2 shows the frequency usage of JRTC Radio. Four pairs of frequencies are used repeatedly along railways. Cover area of radio base station is about 3 km.

FIGURE A4.5.3.1-2

Frequency usage of JRTC Radio

### A4.3.5.2	Technical parameters

Table A4.3.5.2-1 summarizes technical characteristics of JRTC Radio.

Table A4.3.5.1-1

Technical characteristics of JRTC Radio

| Parameters | JRTC Radio |
| --- | --- |
| Frequency Range | 335.4 MHz - 360 MHz |
| Channel separation | 6.25 kHz |
| Maximum Antenna gain | Base station : +11 dBi Mobile station : +1 dBi |
| Polarization | Vertical |
| Maximum Transmission power | Base station : +34.8 dBm Mobile station : +30 dBm |
| E.I.R.P. | Base station : +45.8 dBi Mobile station : +31 dBi |
| Receiving noise figure | < 10 dB |
| Data rate | 9.6 kbps |
| Reception quality | BER < 1x10-4 |
| Transmission distance (km) | 2 - 3 km |
| Modulation | π/4 QPSK |
| Multiplexing method | FDD, TDM-TDMA |

## A4.3.6	Yard Radio

### A4.3.6.1	System architecture

System architecture of the yard radio (YR) operating in 300 MHz band is similar to one of
150 MHz band yard radio system (see A4.2.3.1).

### A4.3.6.2	Technical parameters

Table A4.3.6.2-1 summarizes technical characteristics of YR operating in 300 MHz band.

Table A4.3.6.2-1

Technical characteristics of Yard radio operating in 300 MHz band

| Parameters | Yard radio |
| --- | --- |
| Frequency Range (MHz) | 335.4 MHz - 399.9 MHz |
| Channel separation | 12.5 kHz |
| Antenna gain | [to be filled out in future] |
| Polarization | Vertical |
| Maximum Transmission power | +37 dBm |
| E.I.R.P. | [to be filled out in future] |
| Receiving noise figure | < 10 dB |
| Reception quality | SNR > [30] dB |
| Transmission distance (km) | [to be filled out in future] |
| Modulation | FM |
| Multiplexing method | none |

# A4.4	400 MHz band RSTT

## A4.4.1	Train Radio System

### A4.4.1.1	System architecture

System architecture of the Train radio system (TRS) operating in 400 MHz band is similar to one of 150 MHz band described in A4.2.1.1.  But, base stations are located at railway stations, and transmission distance from a base station is about 1.5 km - 3 km in this 400 MHz band system.

### A4.4.1.2	Technical parameters

Table A4.4.1.2-1 summarizes technical characteristics of TRS operating in 400 MHz band.

Table A4.4.1.2-1

Technical characteristics of Train Radio System operating in 400 MHz band

| Parameters | Train radio system |
| --- | --- |
| Frequency Range | 410 - 420 MHz |
| Channel separation | 12.5 kHz |
| Maximum Antenna gain | Base station : +11 dBi Mobile station : +1 dBi |
| Polarization | Vertical |
| Maximum Transmission power | +30 dBm |
| E.I.R.P. | Base station : +41 dBm Mobile station : +31 dBm |
| Receiving noise figure | < 10 dB |
| Reception quality | SNR > 20 dB |
| Transmission distance (km) | 1.5 -3 km |
| Modulation | FM |
| Multiplexing method | none |

## A4.4.2	Radiocommunication system for High Speed Trains

### A4.4.2.1	System architecture

The radiocommunication system for high speed trains (RHST) is developed and deployed to support safe and stable operation of Shinkansen trains.   The most distinctive feature of this system is to use leaky coaxial cables (LCX) all along the line even at no-tunnel area.

Figure A4.4.2.1-1 shows the system architecture of RHST. LCX as shown at right above is a type of coaxial cable that has holes called “slot”. Through these slots, radio wave gradually leaks outside of the cable. The radio wave is propagated to antennas installed at the “skirt” of the vehicle. LCX method allows the distance between LCX and antennas on board to be so close constantly that the affection of interference or noise can be so smaller and it is possible to maintain stable communication regardless of the location of train, open-site or inside of tunnels. Applying the whole LCX method to train radio systems makes it possible to achieve more than 99.99% connections throughout the entire line even when trains are running at high speed (above 300 km/h).

A Central Unit in Control Centre accommodates Ground Communication Controllers, which are located in the key stations. The Ground Communication Controllers take handover through accommodated Base Stations. Base Stations are located in almost every station and repeaters that compensate for LCX propagation loss, are sided at every 1.3 km intervals along track between Base Stations. Four antennas that are installed at body side of the front vehicle, receive radio waves from LCX.

Because of this stable feature of radio communication, some channels are assigned for automatic train control and the radio based train control system, as described in 8.2.1.4, is in practical use in some high-speed train lines.

FIGURE A4.4.2.1-1

Architecture of Radiocommunication system for High Speed Train

LCX

### A4.4.2.2	Technical parameters

Table A4.4.2.2-1 summarizes technical characteristics of Radiocommunication system for High Speed Train (RHST) operating in 400 MHz band.

Table A4.4.2.2-1

Technical characteristics of Radiocommunication system for High Speed Train (RHST)

| System | Analog RHST | Digital RHST (Type 1) | Digital RHST (Type 2) |
| --- | --- | --- | --- |
| Frequency Range | 410 MHz - 420 MHz, 450 MHz - 455 MHz |  |  |
| Channel separation | up link: 12.5 kHz, 25 kHz, 230 kHz, 288 kHz down link: 12.5 kHz, 25 kHz, 64 kHz, 230 kHz, 288 kHz, 640 kHz |  |  |
| Maximum Antenna gain | Base station : Leaky Coaxial Cable (Coupling loss = 55 dB, 60 dB, 70 dB, 80 dB) Mobile station : Slot array antenna (Gain = [+5] dBi) |  |  |
| Maximum Transmission power | Base station: +33 dBm Mobile station: +36 dBm |  | Base station: +27 dBm Mobile station: +36 dBm |
| Receiving noise figure | < 10 dB |  |  |
| Reception quality | SNR > 30 dB BER < 10-4 | BER < 10-4 |  |
| Transmission distance | 30 km (installation interval of base stations) Radio wave propagation distance between LCX and on-board antenna is about 1 - 2 m. |  |  |
| Modulation | down link : PM up link : FM | π/4 QPSK | down link : π/4 QPSK up link : π/4 QPSK and QPSK |
| Multiplexing method | down link : FDM up link : FDMA FDD | down link : TDM up link : TDMA FDD |  |

## A4.4.3	Yard Radio

### A4.4.3.1	System architecture

System architecture of the yard radio (YR) operating in 400 MHz band is similar to one of 
150 MHz band yard radio system (see A4.2.3.1).

### A4.4.3.2	Technical parameters

Table A4.4.3.2-1 summarizes technical characteristics of YR operating in 400 MHz band.

Table A4.4.3.2-1

Technical characteristics of Yard radio operating in 400 MHz band

| Parameters | Yard radio |
| --- | --- |
| Frequency Range (MHz) | 450 MHz - 470 MHz |
| Channel separation | 12.5 kHz |
| Antenna gain | [to be filled out in future] |
| Polarization | Vertical |
| Maximum Transmission power | +37 dBm |
| E.I.R.P. | [to be filled out in future] |
| Receiving noise figure | < 10 dB |
| Reception quality | SNR > 30 dB |
| Transmission distance (km) | [to be filled out in future] |
| Modulation | FM |
| Multiplexing method | none |

# A4.5	RSTT in the 40 GHz band

## A4.5.1	40-GHz band video transmission system (MVT)

### A4.5.1.1	System architecture

In order to keep passengers’ safety, MVT has already been deployed for many railways, in which trains are driving without any conductor. In this case drivers must confirm platform situations by themselves at each station before departure. MVT enables drivers to confirm platform situations by showing these in the driver’s room.

Figure A4.5.1.1-1 shows the architecture of MVT. CCTV cameras are located at several points in every platform. Millimetre waves transmit these cameras’ video streams to the diver’s room through transmitter and receiver. Monitors in the driver’s room show the conditions of the platform from several cameras simultaneously without latency.  Therefore, the driver can confirm the situations of platform then start the train safely.

Figure A4.5.1.1-1

Architecture of MVT

### A4.5.1.2	Technical parameters

Table A4.5.1.2-1 shows technical characteristics of the video transmission system (MVT). Now the modulation of the system is FM because of its low latency feature. But in future, digital modulations will be adopted for digital ITV with low latency video codecs.

Table A4.5.1.2-1

Technical characteristics of MVT

| Parameter | MVT |
| --- | --- |
| Frequency Range (GHz) | 43.5-43.7 |
| Channel separation | 40 MHz |
| Antenna gain (dBi) | 33 typ. |
| Polarization | Vertical |
| Transmitting radiation power (dBm) | 1 |
| e.i.r.p. (dBm) | 33 typ. |
| Receiving noise figure (dB) | <20 |
| Transmission distance (m) | < 60 |
| Modulation | FM |
| Multiplexing method | FDM |

## A4.5.2	Train Radio System in the 40 GHz band (TRS-40GHz)

### A4.5.2.1	System architecture

Recently there is a growing demand for video transmission from running trains to the control centre. It will be used for real time cabin watching to keep the passenger’s safety, or for immediate outside watching from driver’s room to confirm some emergency circumstances on track.

For video streaming, the broadband transmission capabilities are required and 40 GHz band is the most expected candidate for these applications as the band is already used for video transmission system of platform.

As for TRS-40GHz, some field trials have been continued towards the next generation of TRS.
The architecture of the system is the same as the traditional TRS except for the radio communication between train and track side where LCX is used. The image of communication between train and track side is shown in Figure A4.4.1-1. Millimetre-waves are transmitted to the train by narrow-beam width antennas set at the trackside poles.

Figure A4.5.2.1-1

Image of RSTT in the 40 GHz band

### A4.5.2.2	Deployment scenario

The deployment scenario so called “Linear cell concept” is shown in Figure A4.4.2-1. Track-side antennas are linearly distributed along the track and millimetre-waves from these antennas, with the same signal and the same frequency, would compose a linear cell. Optical feeders are used to connect between the trackside antennas and the base stations. The linear cells with frequency 1 and 2 are alternately repeated itself. By using this concept, frequent handovers that cause throughput reducing, are able to be avoidable especially for high-speed trains. Furthermore, the spectral utilization is efficient because if the length of the linear cell is long enough, only two frequencies are needed for inter-cell interference prevention.

Figure A4.5.2.2-1

Linear cell concept

The upper view of track-side antennas setting at track-side poles are shown in Figure A4.4.2-2. These antennas are aimed at the on-board antennas which were located at the point of maximum transmission distance. The on-board antennas are equipped around driver’s room and aimed straight forward. The maximum transmission distance is calculated in the situation of heavy rain.

As the antennas are deployed as explained above, the millimetre wave beams are not obstructed by track-side poles or oncoming trains and even in heavy rain. The deployment method promises to achieve a reliable radio transmission that can be used to keep the railway safe.

Figure A4.5.2.2-2

Upper view of track-side antennas

### A4.5.2.3	Technical parameters

Table A4.4.3-1 summarizes technical and operational characteristics of RSTT stations operating in 43.5-47 GHz band. The bandwidth of 3.5 GHz can be used for data transmission between on-board radio equipment and related radio infrastructure located along trackside. The transmission distance of these equipment varies according to the railroad line condition.

Table A4.5.2.3-1

Preliminary system parameters of RSTT stations operating in 43.5-47 GHz band

| Frequency Range (GHz) | 43.5-47.0 |
| --- | --- |
| Antenna gain (dBi) | 32 |
| Antenna beamwidth (degree) | 1.0-1.5 |
| Polarization | Circular |
| Transmitting radiation power (dBm) | 10 |
| e.i.r.p. (dBm) | 42 |
| Receiving noise figure (dB) | <10 |
| Transmission data rate (Mb/s) | 100 |
| Transmission distance (km) | 0-3.5 |
| Modulation | 64QAM, OFDM |
| Multiplexing method | FDD |

### A4.5.2.4	Propagation characteristics in the 40 GHz band

Japan’s note: The detailed explanation will be added at the next meeting.

### A4.5.2.5	Coexistence between RSTT and the radiocommunication services

Japan’s note: The detailed explanation will be added at the next meeting.

# A4.6	W-band RSTT

## A4.6.1	Network architecture of W-band RSTT

A network topology of a typical broadband TCN with a radio access system is shown in Fig. A4.6.1-1. A central unit (CU) located in an operation direction center (ODC) transmits, receives and manages all the communication signals. The signals are transmitted to an optical carrier station (OCS) via an optical fiber link; the OCS is worked as a repeater. Typical distance between the OCSs will be much longer than 10 km, whose length will be limited by an optical transceiver output power. A node base station (NBS), which is controlled by a control station (CS) directly connected to the OCS, is worked as a radio base station to transport the signal to track-side radio access units (TS-RAUs) located along a railway trackside. The network for connection between the NBS and TS-RAUs is a bus-type wireline network, a passive-single-star-type network, or point-to-point links to each TS-RAU. The TS-RAU is a frontend to transmit and receive radio signals from/to radio transceivers on the train. In general, distance between the TS-RAUs depends on a coverage size of the TS-RAU; for instance, the distance will be much longer than 1 km if microwave-band radio systems are implemented. In this configuration, an optical fiber network will be suitable for configuring network for between CS and NBSs, and between NBS and TS-RAUs for high-speed railway systems.

Figure A4.6.1-1

Conceptual diagram of a broadband TCN using W-band RSTT

## A4.6.2	Deployment scenario

Figure A4.6.2-1 shows the schematic concept of seamless wireless connection between on-board equipment and trackside radio access unit. The concept shows that 10 trackside radio access units with two antennas are equipped along the railway line. Two on-board transceivers are equipped with the driver’s room and the conductor’s room which are usually placed at the end of train. Both on-board transceivers are complementally connected to the trackside radio access units to seamlessly maintain link connection through 90-GHz carrier.

Figure A4.6.2-1

Concept of seamless wireless connection between on-board and trackside equipment

## A4.6.3	Coexistence of 90-GHz band RSTT with the passive services

Table A4.6.3-1 shows the frequency band which are already allocated for use of mobile services in the frequency range 92-109.5 GHz. In accordance with Article 5 to Chapter II to Radio Regulations (see Annex), in the adjacent bands of those frequencies all emissions are prohibited in the following bands; 86-92 GHz, 100-102 GHz and 109.5-111.8 GHz. In order to coexist with passive services, the same schemes developed by Report ITU-R F.2239, Coexistence between fixed service operating in 71-76 GHz, 81-86 GHz and 92-94 GHz bands and passive services, could be used for sharing and compatibility studies of railway radiocommunication systems. The following sharing and compatibility cases should be addressed, as shown in Figure A4.6.3-1:

1)	mobile service stations such as on-board radio equipment and related radio infrastructure located along trackside operating in the band 92-94 GHz with respect to the protection of Earth exploration-satellite service (EESS) stations operating in the adjacent band 86-92 GHz;

2)	mobile service stations such as on-board radio equipment and related radio infrastructure located along trackside operating in the band 94.1-100 GHz and 102-109.5 GHz with respect to the protection of Earth exploration-satellite service (EESS) stations operating in the adjacent band 100-102 GHz;

3)	mobile service stations such as on-board radio equipment and related radio infrastructure located along trackside operating in the band 102-109.5 GHz with respect to the protection of Earth exploration-satellite service (EESS) stations operating in the adjacent band 109.5-111.8 GHz;

4)	mobile service stations such as on-board radio equipment and related radio infrastructure located along trackside operating in the band 92-94 GHz, 94.1-100 GHz and 102-109.5 GHz with respect to the protection of radio astronomy service (RAS) stations operating in the band 86-111.8 GHz.

Table A4.6.3-1

Frequency bands already allocated for mobile servicers

|  | 92-94 |  | 94.1-100 |  | 102-109.5 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | MS |  | MS |  | MS |  |
|  | BW1=2 GHz |  | BW2=5.9 GHz |  | BW3=7.5 GHz |  |

FIGURE A4.6.3-1

Sharing and compatibility schemes for coexistence between mobile services and passive services

The bands 92-94 GHz, 94.1-100 GHz and 102-109.5 GHz are allocated on an equal primary basis to the mobile service and radio astronomy service in all three Regions. The protection criterion used is derived from Recommendation ITU-R RA.769-2. The protection threshold level is calculated by the following equation:

Pinterference＝Pt+G-Loss-J(ν)

＝Pt+G-(92.5+20*log(f)+20*log(d)+Ag)-J(ν)

where

Pt: 	transmission power of on-board equipment;

G: 	Antenna gain;

d: 	Separation distance;

L0: 	Propagation loss given by Recommendation ITU-R P.452-16;

J(ν): 	Knife-edge diffraction loss given by Recommendation ITU-R P.452-16.

The separation distance which satisfies with the requirement of protection level is calculated from the above equation. The line-of-sight scenario from the on-board equipment to the radio astronomy antenna gives the worst case.

Japan’s note: The separation distance will be calculated under this assumption at the next meeting. Administration are encouraged to submit contribution with respect to protection of the passive services.

## A4.6.4	W-band transceivers characteristics

Table A4.6.4-1 summarizes technical and operational characteristics of RSTT stations operating in 92-94 GHz, 94.1-100 GHz and 102-109.5 GHz bands. The total bandwidth of 15.4 GHz can be used for data transmission between on-board radio equipment and trackside radio access units. The transmission distance of these equipment is designed by the railroad line environment.

Table A4.6.4-1

Preliminary system parameters

| Frequency Range (GHz) | 92-94. 94.1-100, 102-109.5 |
| --- | --- |
| Seamless connection mechanism | Backward and forward switching method |
| Channel bandwidth (MHz) | 250 |
| Channelization (MHz) | TBD |
| Channel aggregation pattern | TBD |
| Antenna gain (dBi) | 44 |
| Antenna beamwidth (degree) | 1 |
| Antenna height from rail surface (m) | 4 (Maximum) |
| Polarization | Linear |
| Average transmitting power (dBm) | 10 |
| Average e.i.r.p. (dBm) | 54 |
| Receiving noise figure (dB) | <10 |
| Maximum transmission data rate (Gb/s) | 5-1010 (Stationary), 1 (Running) |
| Maximum transmission distance (km) | 0.5-11 (Open), 3 (Tunnel) |
| Modulation | PSK, QPSK, 16QAM, 64QAM |
| Multiplexing method | FDD/TDD |
| Space diversity | TBD |
| Maximum running speed (km/h) | 600 |
| Switching time of trackside radio access unit (s) | TBD |
| Average distance between on-board equipment and trackside radio access unit | TBD |
| Rainfall attenuation margin (dB) | TBD |
| Wired interface of trackside radio access unit | TBD |
| Propagation model between train and trackside | TBD |

Figure A4.6.4-1 shows a block diagram of W-band track-side radio access unit. The transmitter (Tx unit) consists of a multiplier (MP), amplifiers and an up-converter. The IF signals are up-converted by a broadband up-converter to W-band signals. The LO signal of the receiver is also supplied from the node base station through an optical fiber. It is also multiplied by a multiplier and then supplied to a mixer to down-convert the received W-band signals to IF signals (Rx unit). Figure A4.6.4-2 shows the external view of W-band track-side radio access unit.

Figure A4.6.4-1

Block diagram of W-band transceiver for track-side radio access unit

Figure A4.6.4-2

External view of W-band track-side radio access unit

## A4.6.5	W-band propagation characteristics

Japan’s note: The example of propagation measurement for the specific railway environment is given in this section. Administrations are encouraged to make comments on 90-GHz band propagation measurement for the specific railway environment.

The viaduct is most commonly used construction for the railway systems. Figure A4.6.5-1 shows the typical structure of viaduct used for high speed railway systems. Figure A4.6.5-2 shows the attenuation characteristics of 90 GHz frequencies guided by viaduct. Table A4.6.5-1 shows the system parameters for propagation measurement. There are attenuation loss differences between wave propagation in the free space and in the guided viaduct. In addition to that, there are also attenuation loss differences between the lower and higher height of the receiver antenna than the height of side walls. This results show that the side wall affects the propagation characteristics of 90 GHz frequencies and decreases the propagation loss.

Figure A.1.6.5-1

Cross sectional view of viaduct.

Figure A4.6.5-2

Attenuation characteristics 90-GHz frequencies guided by viaduct.

TABLE A4.6.5-1

System parameters for measurement.

| Centre Frequency | 93.2 GHz |
| --- | --- |
| Transmitter output power | -5 dBm |
| Transmitter antenna type | Horn Antenna |
| Transmitter antenna gain | 25 dBi |
| Transmitter antenna half-value angle | 10 degrees |
| Transmitter antenna height | 0,92 m or 1.92 m |
| Receiver antenna Type | Horn Antenna |
| Receiver antenna gain | 25 dBi |
| Receiver antenna half-value angle | 10 degrees |
| Receiver antenna height | 0.92 m or 1.92 m |

## A4.6.6	Table of frequency allocation of W-band

86-111.8 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 86-92 EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) 5.340 |  |  |
| 92-94 FIXED 5.338A MOBILE RADIO ASTRONOMY RADIOLOCATION 5.149 |  |  |
| 94-94.1 EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION SPACE RESEARCH (active) Radio astronomy 5.562 5.562A |  |  |
| 94.1-95 FIXED MOBILE RADIO ASTRONOMY RADIOLOCATION 5.149 |  |  |
| 95-100 FIXED MOBILE RADIO ASTRONOMY RADIOLOCATION RADIONAVIGATION RADIONAVIGATION-SATELLITE 5.149 5.554 |  |  |
| 100-102 EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) 5.340 5.341 |  |  |
| 102-105 FIXED MOBILE RADIO ASTRONOMY 5.149 5.341 |  |  |
| 105-109.5 FIXED MOBILE RADIO ASTRONOMY SPACE RESEARCH (passive) 5.562B 5.149 5.341 |  |  |
| 109.5-111.8 EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) 5.340 5.341 |  |  |

# A4.7	60 GHz RSTT

## A4.7.1	 60-GHz band train platform monitoring system

Since passenger safety at the station is a primary concern of railway system, the train platform monitoring system is introduced to monitor passengers on the track line of the station. The video monitors are equipped at the control room in the station room, the train driver’s room and the conductor’s room. Several video cameras are placed to monitor almost entire train platform. The 60-GHz transceivers are connected to those video cameras and monitors to transmit/receive video signals. Due to surveillance capabilities of the monitoring system, serious accident of passengers at the station platform can be prevented. The system configuration is shown in Figure 4.7.1-1.

FIGURE 4.7.1-1

System configuration of 60-GHz train platform monitoring system

60-GHz band train platform monitoring system

## A4.7.2	60-GHz band RSTT

60-GHz band train platform monitoring system is widely introduced for safety operation of railway systems. The train platform monitoring system assists train crews to open/close train doors safely to prevent passengers’ accidents by looking at the monitor equipped in the train driver’s room, as shown in Figure 4.7.1-1. This section provides technical and operational characteristics of 60-GHz band train platform monitoring system which was already developed and is deployed in many stations.

### A4.7.2.1	Technical characteristics

Table 4.7.2-1 shows technical characteristics of 60-GHz band train platform monitoring system.

Table 4.7.2-1

Technical characteristics of 60-GHz train platform monitoring system

| Parameters | Fixed station | On-board station |
| --- | --- | --- |
| Frequency Range (GHz) | 57-66 | 57-66 |
| Channel separation (MHz) | 125 | 125 |
| Antenna gain (dBi) | 31 | 26 |
| Antenna beam width (degree) | 3.5 | 7 |
| Polarization | Linear | Linear |
| Transmitting radiation power (mW) | 10 | 10 |
| e.i.r.p. (dBm) | 41 | 31 |
| Receiving noise figure (dB) | 8 | 8 |
| Transmission data rate (Mb/s) | 100 | 100 |
| Transmission distance (m) | 100 | 100 |
| Modulation | ASK | ASK |
| Multiplexing method | FDD | FDD |
| Network interface | 100 Base-TX | 100 Base-TX |

### A4.7.2.2	Operational characteristics

Japan’s note: This information will be provided at the next meeting.

ANNEX 5

RSTT in China

# A5.1	Overview

Chinese railway radiocommunication technology has witnessed rapid development since 1950s, especially in 450 MHz wireless train dispatching system and 900 MHz GSM-R system. These systems have been implemented into the whole Chinese railway network, which is of great importance for the safety of railway transportation.

# A5.2	450 MHz -band RSTT

Wireless train dispatching system has been implemented in China since 1950s, which used for voice communication and dispatching order transmission. Up to 2015, 450 MHz system has been deployed over 84,000 kilometers lines in China.

With the development of railway radiocommunication technology, the existing 450 MHz wireless train dispatching system might be gradually replaced by advanced technologies in China, for instance the GSM-R or other next generation railway radiocommunication technologies. Wireless train dispatching system has been implemented in China since 1950s, which used for voice communication and dispatching order transmission. Up to 2015, 450 MHz system has been deployed over 84,000 kilometers lines in China.

Table A5.2-1

450 MHz band RF characteristics

| Parameter | 450 MHz Band Wireless Train Dispatching System |
| --- | --- |
| Frequency Range (MHz) | 457.200-458.650 467.200-468.650 |
| Channel Spacing | 25 kHz |
| Frequency Tolerance | ≤5×10-6 |
| Antenna Gain (dBi) | Fixed radio station: 9(Omni-directional), 12(Directional) Locomotive station: 0 |
| Carrier Power (dBm) | Fixed Radio Station: 34.7-37(simplex), 37-40 (duplex) Locomotive Station: 37(simplex), 40 (duplex) Handset: 34.7 |
| Modulation type | FM |
| Adjacent-channel Selectivity (receiver) (dB) | ≥65 |
| Co-channel Rejection(dB) | ≥-8 |

# A5.3	900 MHz -band RSTT

Since the first deployment of GSM-R system in the Qinghai-Tibet Railway, 900 MHz GSM-R system has been implemented in all new lines from 2006. Up to 2015, GSM-R has been implemented over 37,000 kilometers lines in China, including part of existing regular lines and all high-speed lines. In China, GSM-R system provides voice service and data service for railway transportation.

Voice Service: On-train outgoing voice communication from the driver towards the controller(s) of the train, on-train incoming voice communication from the controller towards a driver, railway emergency communication, trackside maintenance communication and public emergency call, etc.

Data service: Automatic train control communication, Monitoring and control of critical infrastructure, Shunting data communication, etc.

China has set up series of technical specifications for GSM-R. The RF characteristics of GSM-R in China are listed in the following Table.

Table A5.3-1

900 MHz band RF characteristics

| Parameter | 900 MHz Band GSM-R System |
| --- | --- |
| Frequency Range (MHz) | 885~889 (uplink) 930~934 (downlink) |
| Channel separation (kHz) | 200 |
| Antenna gain (dBi) | Mobile station: ≥0 Base station : 65°(Half-power Beam width):17 Or 33°(Half-power Beam width):21 |
| Polarization | Dual-polarized |
| Transmitting radiation power (dBm) | Mobile station (Handset: 33, Locomotive station: 39) Base station: 46 |
| Modulation | GMSK |
| Multiplexing method | TDMA |
| Receiver sensitivity(dBm) | Mobile station: ≤-104 Base station: ≤ -110 |

ANNEX 6

RSTT in Russia

# A6.1	Overview

Radiocommunication systems are widely used in railway transportation in Russia. Those systems provide for fulfilling important tasks including railway traffic management and control, as well as provision of railway transportation safety. The systems also provide train crews, conductors and passengers with services associated with different information exchange and entertainment functions. Relevant State Radiofrequency Commission Decisions allotted different HF, VHF and UHF bands for operation of those systems.

# A6.2	Train radio systems for train operation and railway traffic control in the HF band

## A6.2.1	Architecture of train radio system in HF-band

[TBD]

## A6.2.2	Typical technical characteristics of HF train radio system radiostations

Typical technical characteristics for radiostations operating in train radio systems in 2 MHz band are shown in Table A6.2-1.

TABLE A6.2-1

Sample radiostation parameters

| Frequency band (MHz) | 2.124-2.136/ 2.144-2.156 |
| --- | --- |
| Channel passband (kHz) | 20 |
| Antenna gain (dBi) | 0 |
| Polarization | Vertical/Horizontal |
| Transmitter output power (dBW) | 11,5 |
| e.i.r.p. (dBW) | 11,5 |
| Receiver noise figure (dB) | <12 |
| Data rate (kbps) | - |
| Radiocommunication distance (km) | 20 (max) |
| Modulation | FM |
| Multiplexing mode | FDMA |

# A6.3	Train radio systems for train operation and railway traffic control in the VHF band

## A6.3.1	Architecture of train radio system in VHF-band

## A6.3.2	Typical technical characteristics of VHF train radio system radiostations

Typical technical characteristics for radiostations operating in train radio systems in 150 MHz band are shown in Table A6.3-1.

TABLE A6.3-1

Sample radiostation parameters

| Frequency band (MHz) | 146-174 |
| --- | --- |
| Channel passband (kHz) | 12.5, 25 |
| Antenna gain (dBi) | 2 - 5 |
| Polarization | Vertical |
| Transmitter output power (dBW) | 16 |
| e.i.r.p. (dBW) | 18 - 21 |
| Receiver noise figure (dB) | <10 |
| Data rate (kbps) | - |
| Radiocommunication distance (km) | 20 (max) |
| Modulation | FM, /4QPSK |
| Multiplexing mode | FDMA |

# A6.4	Train radio systems for train operation and railway traffic control in the UHF band

## A6.4.1	Architecture of train radio system in UHF-band

## A6.4.2	Typical technical characteristics of UHF train radio system radiostations

Typical technical characteristics for radiostations operating in train radio systems TETRA standard in 450 MHz band are shown in Table A6.4-1.

table A6.4-1

Sample radiostation parameters

| Frequency band (MHz) | 450-470 |
| --- | --- |
| Channel passband (kHz) | 25 |
| Antenna gain (dBi) | 3 - 7 |
| Polarization | Vertical |
| Transmitter output power (dBW) | 15 |
| e.i.r.p. (dBW) | 18 - 22 |
| Receiver noise figure (dB) | <12 |
| Data rate (kbps) | - |
| Radiocommunication distance (km) | 30 (max) |
| Modulation | /4-DQPSK |
| Multiplexing mode | TDMA |

Typical technical characteristics for radiostations operating in GSM-R train radio systems in
900 MHz band are shown in Table A6.4-2.

TaBLE A6.4-2

Sample radiostation characteristics

| Frequency band (MHz) | 900 |
| --- | --- |
| Channel passband (kHz) | 200 |
| Antenna gain (dBi) | 2-18 |
| Polarization | Vertical, oblique |
| Transmitter output power (dBW) | 17 |
| e.i.r.p. (dBW) | 19 - 35 |
| Receiver noise figure (dB) | <8 |
| Data rate (kbps) | 8 × 22.8 |
| Radiocommunication distance (km) | 20 (max) |
| Modulation | GMSK |
| Multiplexing mode | TDMA |

ANNEX 7

RSTT in Korea

# A7.1	Overview

Korean railway uses 150 MHz, 400 MHz, 700 MHz, 800 MHz and 18 GHz band for RSTT that carries train control, command, operational information as well as monitoring data between on-board radio equipment and related radio infrastructure located along trackside. Very High Frequency (VHF) system using 150 MHz band supports only voice calls between control centre/base station and a train crew or inter-mobile station radiocommunication. Train Radio Protection Device (TRPD) system is used to alert some dangerous situations to trains within about
4 km range through emergency signals at the 400 MHz band. LTE-R system provides various RSTT functions using LTE based wireless communication operated at the 700 MHz band. Korea is using two TRS (Trunked Radio System) schemes, i.e., TRS-ASTRO and TRS-TETRA. These systems are operated at the 800 MHz band for transferring operational information between train and control centre. The government has a plan to replace these TRS systems to LTE-R systems. In addition, CCTV video systems are used to monitor the platform situations. The system transmits these cameras’ video stream to driver’s room using a wireless communication operated at the 18 GHz band. Table A7.1 is the list of RSTTs used in Korea.

Table A7.1

List of RSTTs used in Korea

| # | Name of the Systems | Frequency bands in use |
| --- | --- | --- |
| 1 | LTE-R | 718-728 MHz (uplink), 773-783 MHz (downlink) |
| 2 | VHF | 153 MHz |
| 3 | TRS | ASTRO: 806-811 MHz (uplink), 851-856 MHz (downlink) TETRA: 806-811 MHz (uplink), 851-856 MHz (downlink) |
| 4 | TRPD | 433.3125 MHz |
| 5 | Video for Platform | 18.86-18.92 GHz, 19.20-19.26 GHz |

# A7.2	700 MHz band RSTT

LTE based 700 MHz band system, LTE based Railway communication (LTE-R), provides voice, data, video and control data radiocommunication services among railway entities including control centre, base station, train crews, drivers, and workers in high-speed train. LTE-R system uses single channel for uplink and downlink.

## A7.2.1	Architecture of LTE-R

Railway communication system is composed of railway train control, LTE Core for railway, LTE Access Network for railway, LTE On-Board Infra for railway, LTE trackside equipment, and other networks to be shared or interoperable. Railway train control consists of Centralized Train Control Centre (CTC) and Radio Block Control Centre (RBC). There is no need that CTC and RBC reside in the same place.

LTE-R Core consists of Evolved Packet Core (EPC), IP Multimedia Subsystem (IMS), Backbone network and Switch network. Gateways to interface with legacy wireless networks are not depicted in Figure A7.2-1.

Figure A7.2-1

LTE based Railway Communication System Architectural Concept

## A7.2.2	Technical characteristics

This system is based on the 3GPP Release 12 basically and Release 13 optionally including D2D function.

Table A7.2

Technical characteristics of LTE-R system

| Parameter | System 1: (LTE-R) |
| --- | --- |
| Frequency Range (MHz) | Uplink: 718-728 MHz, Downlink: 773-783 MHz |
| Number of Channels | 1 |
| Channel separation (kHz) | 10 MHz |
| Antenna gain (dBi) | - |
| Polarization | - |
| Transmitting radiation power (dBm) | Terminal: Max 2 W, Base station: Max 80 W |
| e.i.r.p. (dBm) | - |
| Receiving noise figure (dB) | - |
| Transmission data rate (kb/s) | Downlink: Max 75 Mbps, Uplink: Max 37 Mbps |
| Parameter | System 1: (LTE-R) |
| Transmission distance (km) | - |
| Modulation | Downlink: OFDMA, Uplink: SC-FDMA |
| Multiplexing method | Full Duplex FDD |

## A7.2.3	Operational characteristics

LTE system provides voice and data services for control and operation of conventional and high speed railway.

# A7.3	150 MHz band RSTT

150 MHz system provides point-to-point radiocommunication scheme between control centre/base station and a train crew or inter-mobile station radiocommunications in conventional train. VHF system uses 4 channels for exchanging data at the 153 MHz frequency band. Since the radiocommunication is established by voice call depending on propagation range, users must be careful to be in radiocommunication range. Due to point-to-point scheme, various radiocommunication functions such as group radiocommunication, priority radiocommunication are not supported. Furthermore, the main requirement for railway wireless networks, i.e., safety, reliability, and security, are not guaranteed. Table A7.3-1 represents frequency band allocation for VHF system.

## A7.3.1	Technical characteristics

Table A7.3-1 shows technical characteristics of VHF system.

Table A7.3-1

Technical characteristics of VHF system

| Technical Parameters | Technical characteristics |
| --- | --- |
| Frequency Range | 153 MHz |
| Number of channels | 4 |
| Channel separation | 25 kHz |
| Antenna gain | 3 dBi |
| Polarization |  |
| Transmitting radiation power | Base statin: 25 W, Train: 25 W, Portable terminal: 3-4.8 W |
| e.i.r.p. |  |
| Receiving noise figure | -113 dB |
| Transmission data rate | 9.6 kbps |
| Transmission distance | - |
| Modulation | FM |
| Multiplexing method |  |

## A7.3.2	Operational characteristics

Table A7.3-2 presents channel arrangement for VHF system.

Table A7.3-2

Channel arrangement for VHF system

| Item | CH | Tx | Rx |
| --- | --- | --- | --- |
| Portable terminal (worker, driver) | 1(Normal) | 153.440 MHz | Same as Tx |
|  | 2(Emergency) | 153.250 MHz |  |
|  | 3(Work) | 153.280 MHz |  |
|  | 4(Work) | 153.660 MHz |  |
| Portable terminal (shunter) | 1(Normal) | 153.440 MHz | Same as Tx |
|  | 2(Emergency) | 153.340 MHz |  |
|  | 3(Work) | 153.740 MHz |  |
|  | 4(Work) | 153.660 MHz |  |
| Mobile terminal (crew) | 1(Normal) | 153.440 MHz | Same as Tx |
|  | 2(Emergency) | 153.520 MHz |  |
|  | 3(Work) | 153.590 MHz | 153.110 MHz |
|  | 4(Work) | 153.620 MHz | 153.200 MHz |
| Base station | 1(Normal) | 153.440 MHz | Same as Tx |
|  | 2(Emergency) | 153.520 MHz |  |
|  | 3(Work) | 153.110 MHz | 153.590 MHz |
|  | 4(Work) | 153.200 MHz | 153.620 MHz |

# A7.4	800 MHz band RSTT

Korea is using two TRS (Trunked Radio System) schemes, i.e., TRS-ASTRO and TRS-TETRA. TRS-ASTRO uses FDMA (Frequency Division Multiple Access) which uses one channel per 
12.5 kHz. TRS-TETRA uses TDMA (Time Division Multiple Access) and provides 4 channels with 25 kHz bandwidth.

## A7.4.1	Technical characteristics

Table A7.4-1 shows technical characteristics of TRS system.

Table A7.4-1

Technical characteristics of TRS-ASTRO and TRS-TETRA system

| Technical Parameters | Technical characteristics |  |
| --- | --- | --- |
|  | ASTRO | TETRA |
| Frequency Range | Uplink: 806-811 MHz, Downlink: 851-856 MHz | Uplink: 806-811 MHz, Downlink: 851-856 MHz |
| Channel separation | 12.5 kHz, 25 kHz | 25 kHz |
| Antenna gain | 3 dBi | 3 dBi |
| Polarization | - | - |
| Transmitting radiation power | Base station: 70 W, Train: 30W, Portable terminal: 3W | Base station: 25 W, Train: 3W, Portable terminal: 1W |
| e.i.r.p. | - | - |
| Receiving noise figure | -118 dB | Terminal: -112 dB, Base station: -125 dB |
| Transmission data rare | 9.6 kbps | 36 kbps |
| Transmission distance | - | - |
| Modulation | C4FM (Continuous 4 level FM) | π/4 DQPSK |
| Multiplexing method | FDMA | TDMA |

## A7.4.2	Operational characteristics

TRS provides voice service such as one-to-one call, one-to-many call, group call, emergency call, and direct call as well as data service such as message and packet transmission. TRS-TETRA has versatile availability for railway wireless network compared with TRS-ASTRO and VHF.

# A7.5	400 MHz band RSTT

TRPD (Train Radio Protection Device) in 400 MHz band provides accident information to adjacent trains to avoid additional accidents. This system has a wireless train protection function which is installed on the train for the railway vehicle the event of emergencies such as accidents and dangerous situations. It promptly informs and makes stop or decrease speed of the nearby to prevent from subsequent accidents.

## A7.5.1	Technical characteristics

Table A7.5-1

Technical characteristics of TRPD system

| Technical Parameters | Technical characteristics |
| --- | --- |
| Frequency Range | 433.3125 MHz |
| Number of channels | 1 |
| Channel separation | 12.5 kHz |
| Antenna gain | 3 dBi |
| Polarization | Vertical |
| Transmitting radiation power | 36 dBm |
| e.i.r.p. | 39 dBm |
| Technical Parameters | Technical characteristics |
| Receiving noise figure | Under 2 |
| Transmission data rare | 8 kbps |
| Transmission distance | 4 km |
| Modulation | GMSK (Gaussian Minimum Shift Keying) |
| Multiplexing method | Single |

# A7.6	18 GHz band RSTT

Platform Video System provides video streams to driver from the camera when the train enters to the platform of a station to monitor the clearance of the trackside.

## A7.6.1	Technical characteristics

Table A7.6-1

Technical characteristics of platform video system

| Technical Parameters | Technical characteristics |
| --- | --- |
| Frequency Range | 18.86-18.92 GHz, 19.20-19.26 GHz |
| Number of channels | 6 |
| Channel separation | 10 MHz |
| Antenna gain | - |
| Polarization | - |
| Transmitting radiation power | 100 mW |
| e.i.r.p. | - |
| Receiving noise figure | - |
| Transmission data rate | - |
| Transmission distance | 1.5-2.5 km |
| Modulation | OFDM |
| Multiplexing method | - |

____________________

Shunting mode is the term used to describe the application that will regulate and control user access to facilities and features in the mobile while it is being used for shunting communications.

From the GSM-R Industry Group’s strategic key messages: http://www.gsm-rail.com/drupal/messages.

See list of TETRA projects https://en.wikipedia.org/wiki/Terrestrial_Trunked_Radio.

From TETRA Rail group http://www.tandcca.com/Library/Documents/TETRA_Resources/Library/Presentations/MiddleEasti2011Davis.pdf.

http://en.wikipedia.org/wiki/Taiwan_High_Speed_700T_train.