QUESTION ITU-R 217/1

Compatibility between short range devices operating within the band 59-64 GHz and industrial, scientific and medical (ISM) applications
operating in the band 61-61.5 GHz*

(1998)

The ITU Radiocommunication Assembly,

considering

a)	that it is expected that there will be increasing use of short range devices operating within the band 59-64 GHz, and of industrial, scientific, and medical (ISM) applications designated for use in the band 61-61.5 GHz;

b)	that short range devices within the band 59-64 GHz are subject in some countries to emission limits for the purpose of preventing radio interference;

c)	that short range devices in or around the 61-61.5 GHz ISM-designated band may receive interference from ISM applications;

d)	that such interference may increase as short range device or ISM use increases,

further considering

a)	that, pursuant to No. 5.138 of the Radio Regulations (RR), use of the band 61-61.5 GHz for ISM applications is subject to special authorization by the administrations concerned and must be in accordance with agreement of administrations whose radiocommunication services might be affected by such applications;

b)	that WARC-79 Resolution 63 considering e) stated that Atlantic City, 1947, and Geneva, 1959, designated some frequency bands within which radiocommunication services must accept harmful interference produced by ISM equipment, and that WARC-79 accepted an increase in the number of bands to be designated for ISM equipment, but only on the condition that limits of radiation from such equipment be specified within the bands newly designated for worldwide use and outside all the bands designated for ISM equipment;

c)	that after study, the ITU-R, in collaboration with the CISPR, has not recommended emission limits for ISM applications in the “newly designated” bands in part because no basis (radiocommunication use, inter alia) was identified for their establishment; and

d)	that the band 59.3-64 GHz is allocated worldwide on a primary basis to the fixed, mobile, inter-satellite, and radiolocation services,

decides that the following Question should be studied

1	What are the characteristics of short range devices currently operating or planned to operate within the band 59-64 GHz (see also Question ITU-R 213/1)?

2	What are the characteristics of ISM applications currently operating or planned to operate in the designated band 61-61.5 GHz?

3	What methods can be applied to promote compatibility between these short range devices and these ISM applications?

further decides

1	that the results of the above studies should be included in (a) Recommendation(s);

2	that the above studies should be completed by 2005.

_______________

*	This Question should be brought to the attention of Radiocommunication Study Groups 4, 8 and 9. Similarly, as ISM matters fall to some extent within the responsibility of the International Electrotechnical Commission (IEC), this Question should also be sent to the International Special Committee on Radio Interference (CISPR) with a request for appropriate collaboration.