Report ITU-R BT.1088-2 (11/2009)

Interfaces for digital video signals in 525-line and 625-line television systems

BT Series Broadcasting service (television)

Foreword

The role of the Radiocommunication Sector is to ensure the rational, equitable, efficient and economical use of the radio-frequency spectrum by all radiocommunication services, including satellite services, and carry out studies without limit of frequency range on the basis of which Recommendations are adopted.

The regulatory and policy functions of the Radiocommunication Sector are performed by World and Regional Radiocommunication Conferences and Radiocommunication Assemblies supported by Study Groups.

# Policy on Intellectual Property Right (IPR)

ITU-R policy on IPR is described in the Common Patent Policy for ITU-T/ITU-R/ISO/IEC referenced in Annex 1 of Resolution ITU-R 1. Forms to be used for the submission of patent statements and licensing declarations by patent holders are available from http://www.itu.int/ITU-R/go/patents/en where the Guidelines for Implementation of the Common Patent Policy for ITU-T/ITU-R/ISO/IEC and the ITU-R patent information database can also be found.

| Series of ITU-R Reports (Also available online at http://www.itu.int/publ/R-REP/en) |  |
| --- | --- |
| Series | Title |
| BO | Satellite delivery |
| BR | Recording for production, archival and play-out; film for television |
| BS | Broadcasting service (sound) |
| BT | Broadcasting service (television) |
| F | Fixed service |
| M | Mobile, radiodetermination, amateur and related satellite services |
| P | Radiowave propagation |
| RA | Radio astronomy |
| RS | Remote sensing systems |
| S | Fixed-satellite service |
| SA | Space applications and meteorology |
| SF | Frequency sharing and coordination between fixed-satellite and fixed service systems |
| SM | Spectrum management |

Note: This ITU-R Report was approved in English by the Study Group under the procedure detailed in Resolution ITU-R 1.

Electronic Publication

Geneva, 2010

ITU 2010

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without written permission of ITU.

REPORT  ITU-R  BT.1088-2

Interfaces for digital video signals in 525-line and 625-line television systems

(1986-1990-2009)

Preface

Introduction/purpose

Readers of this Report should be aware that the text in this Report is the original text generated during the development of the SDTV digital interfaces. It is has been retained as an historic record of the digital interface development. Many of the concepts and technical parameters developed during the 1982 to 1986 time-frame have changed, readers are advised to consult the latest ITU-R Recommendations before implementing digital interfaces.

Reference is made to the International Radio Consultative Committee (CCIR), the year 1920 saw the beginning of sound broadcasting at the improvised studios of the Marconi Company, and in 1927, the CCIR was established at a conference held in Washington D.C. The International Telephone Consultative Committee (CCIF, set up in 1924), the International Telegraph Consultative Committee (CCIT, set up in 1925) and the CCIR were made responsible for coordinating the technical studies, tests and measurements being carried out in the various fields of telecommunications, as well as for drawing up international standards. In 1989 the CCIR was reorganized into the International Telecommunication Union (ITU), over time, the Union was streamlined into three Sectors, corresponding to its three main areas of activity: Telecommunication Standardization (ITU-T), Radiocommunication (ITU-R) and Telecommunication Development (ITU-D).

Today the ITU-R has the responsibility for broadcasting services.

The broadcasting and related industries depend 100% on the interfaces and their derivatives that were proposed back pre-1986. In 1986 in Houston Texas the first digital serial interface as we know it today was demonstrated at a trade show, since that time the serial interface which originally ran at 270 Mbit/s (an interim interface ran at 243 Mbit/s, this interface had only very limited use) has been extended to 3 Gbit/s. The serial interface has also been adapted to carry almost any data signal in addition to the uncompressed video signals that was the original intent. Further treatment of the characteristics of the serial interfaces as implement can be found in EBU Tech. 3283.

The following text is presented as an historic record of the digital interface development, along with a copy of the original Recommendations CCIR 601 and 656. The original Recommendation CCIR 656 was never implemented on a widespread basis as it was constrained to 8-bit operation. A later version, Recommendation ITU-R BT.656-2 replaced the 8/9 code mapping with scrambled NRZ coding. It is the scrambled NRZ version of the serial digital interface that found very widespread implementation. The original Recommendation CCIR 601 was originally called CCIR-AA/11 and was approved in 1982. The attached Recommendation CCIR 601-1 has been restored using the original text along with any errors that may have been in this text.

# 1	Introduction

Recommendation CCIR 656 specifies interfaces for digital studio equipment, in conformity with the basic parameter values contained in Recommendation CCIR 601.

This Report summarizes the contributions received on digital video interfaces which provided the basis for Recommendation CCIR 656, it includes supplementary information on the subject, and indicates areas in which further studies are required.

# 2	Definitions

Interface is a concept involving the specification of the interconnection between two items of equipment or systems. The specification includes the type, quantity and function of the interconnection circuits and the type and form of the signals to be interchanged by these circuits:

–	a parallel interface is an interface in which the bits of a data word are sent simultaneously via separate channels;

–	a serial interface is an interface in which the bits of a data word, and successive data words, are sent consecutively via a single channel. A parallel-serial (hybrid) interface is an interface in which portions of a data word are sent consecutively via separate channels.

# 3 	Primary encoding format

There are features of the basic data organization which are common to the three types of interface defined above and which are the subject of Part I of Recommendation CCIR 656. They comprise: the organization of the video data into words and blocks; the timing reference codes providing video synchronization; ancillary data signal structure; data signals during blanking intervals; details of the multiplexing.

## 3.1 	Blanking and synchronization considerations

[CCIR, 1982-86a, b, and c] agreed on the form and use of timing reference signals. Each timing reference signal consists of a four-word sequence. The first three words are a fixed preamble. The fourth word contains information defining:

–	first or second field identification;

–	state of field blanking;

–	state of line blanking;

–	error protection data.

[CCIR, 1982·86d] proposed that only one timing reference signal should be used, located at the end of each line-blanking period. This identification signal includes a clock burst (for a bit-serial interface), indication of the initial point of the data frame, field-blanking period and first and second field periods. End-of-line information will be obtained by counting clock pulses.

The EBU [CCIR, 1982-86e] suggested that additional codes be included in the data stream and that the beginning and end of the digital active line will be identified in the de-multiplexed Y, CR, CB data streams. It further proposed that these codes be included at the 4:4:4 level for both Y, CR, CB and R, G, B signals in digital form.

[CCIR, 1982-86f] stated that the timing reference (digital synchronization) codes inserted into the parallel code should be easily usable in a serial code.

Proposals for 525-line and 625-line differ in their definition of digital field-blanking intervals. [CCIR, 1982-86b, c, and d] stated that only 9 lines in both fields 1 and 2 belong to the field-blanking interval. [CCIR, 1982-86a and f] specify the digital field-blanking interval of 24 lines (field 1) and 25 lines (field 2). It may be advisable to shorten the digital field-blanking interval so as to allow for complex vertical filtering, though this problem needs further study.

Amongst other considerations [CCIR, 1982-86g] drew attention to the fact that “timing reference signals” should be referred to as “timing reference codes”.

In those data words occurring during digital blanking intervals that are otherwise unspecified, the OIRT [CCIR, 1982-86e] proposed that the digital codes equivalent to blanking level for Y, CR, CB be included in the appropriate locations in the multiplex.

## 3.2 	Ancillary signals

Provision is made for ancillary data signals to be inserted synchronously into the video multiplex during both horizontal and vertical blanking intervals. It is noted that digital video tape recorders (see Recommendation CCIR 657) do not record any of the horizontal blanking intervals or some lines in the vertical blanking intervals. For that reason the EBU has allocated only four vertical blanking lines for ancillary signals. The unrecordable blanking periods can be used to transfer data between other studio equipment if required.

[CCIR, 1982-86 d and f] contain some details of the ancillary signals. [CCIR, 1982-86a, band c] propose the ancillary data signal format.

Time-code is an essential ancillary signal for control of post-production processes and the synchronization of video and audio. Four formats are currently recognised, IEC format [IEC Pub. 461] in the vertical interval and longitudinal forms, audio time code in accordance with Recommendation CCIR 647 and time code associated with the R-DAT audio recording format.

Ancillary data formats to include this information in the vertical interval are a current study in a number of Administrations and offer possibilities to maintain the synchronism of video and audio through various processes [CCIR, 1986-1990a).

Recommendation CCIR 656 specifies only a timing reference code ANC; the data field following the ANC is left unspecified. There have been discussions about various packet formats for the ancillary data.

Some information with higher priority and predetermined format might have a fixed data packet length and probably also a fixed time slot in the data stream. Less important ancillary data not having a predetermined format might have variable packet length.

(CCIR 1986-1990b] mentions digital line numbers as possibly useful information which should be considered as an ancillary signal. The document contains a proposal for two modes of digital line numbering. In addition one method for introduction of respective code words into the video data is proposed.

The study of the requirements for sound signals is included in Decision 60 to ensure that any possible effects upon the associated sound signals caused by the video interface parameters will be duly considered. Except for the need to control the relative delay between the video and the sound, no such effects have been identified.

# 4 	Parallel interfaces

A number of proposals [CCIR, 1982-86a, b, c, d and f] suggested using eight conductor pairs, where each should carry, in NRZ format, a multiplex stream of bits (of the same significance) of each of the component signals, namely, Y, CR, CB. The eight' pairs should also carry timing reference information and may carry ancillary signals that are time-multiplexed into the data stream during video blanking intervals. A ninth pair would provide a synchronous clock at 27 MHz. These proposals, with [CCIR, 1982-86e], contributed to the preparation of Recommendation CCIR 656 (see also [EBU, 1983]).

The signals on the interface may be transmitted using balanced conductor pairs for a distance of up to 50 m without equalization and up to 200 m with appropriate equalization [CCIR, 1982-86a].

Appropriate coding of the clock signal, such as the use of an alternating parity (AP) coding, has been shown to extend this distance by reducing the effects of cable attenuation [CCIR, 1982-86h].

# 5	Serial interfaces

[CCIR, 1982-86d] gives an example of a data sequence using 216 Mbit/s multiplexing. Particular attention is paid to ease of clock extraction and word synchronization by the inclusion of words within the data stream which generate clock bursts.

[CCIR, 1982-86f] refers to channel coding and states that transmission should be effected via 75 Q coaxial cables for distances up to 1 km.

[CCIR, 1982-86e] contains a detailed consideration of the special requirements for a serial interface and proposes in Annex I a draft Recommendation for a bit-serial interface for the 4:2:2 level of Recommendation CCIR 601. This contributed to the preparation of Recommendation CCIR 656 (see also [EBU, 1985]).

In [CCIR, 1982-86e] the transmission of signals is considered in both electrical form, using coaxial cable, and in optical form using an optical fibre. The special requirements for bit-serial signal transmission between studios, or between equipments in a studio are given as:

–	low cost and low complexity coupled with high reliability;

–	very low intrinsic error rate in the transmission due to the very short distances;

–	multiple outputs for monitoring and distribution;

–	rapid recovery from errors introduced by switching of the transmission path, the video source or signal interruptions;

–	full compatibility with the format of the bit-parallel interface and signal code commonality of both electrical and optical implementations of the bit-serial interface;

–	usable over a range of distances from zero to at least 500 m, with a minimum of adjustments and extremely low error rates;

–	applicable to a range of cable types.

These requirements are confirmed in [CCIR, 1982-86i], which also points out that in the implementation of a digital video installation, preference would normally be given to the parallel interface for short connection lengths and that recourse would be made to the serial interface mainly in the case of long or complex connection paths, where the cost of the interface terminal equipment would not override the saving in the physical support of the connection itself. Coaxial cables would probably be preferred for connections of medium length, while preference would go to optical fibres for very long connection lengths.

This contribution also suggests that the code used should be structured so as to permit the redundant bits to be employed to implement a system for measuring the BER at the receiving end of the connection and thus automatically monitoring its performance.

It further suggests that in a fully integrated digital installation or system it may be useful for all interconnections to be transparent to any appropriate digital stream, irrespective of the message content. Thus, although the interface will be used to transmit a video signal, it should be “transparent” to the message content, i.e. it should not base its operation on the known structure of the message itself.

(CCIR, 1982-86e) reviews the characteristics of transmission media, including interference susceptibility and describes the proposals received for source encoding, channel encoding and error management.

Two methods of source encoding have been proposed. (CCIR, 1982-86j) suggests the use of a parallel scrambler with the addition of a parity bit for synchronization and limited error detection purposes. According to a preliminary investigation, it appears that the sending end, at least, of such an interface could be integrated in a single gate-array chip.

A second method (CCIR, 1982-86e) providing spectrum control, clock and word synchronization by an 8-bit to 9-bit adaptive mapped code, is adopted in Recommendation CCIR 656.

In relation to these methods of source encoding, two different approaches to channel encoding have been proposed. In the scrambled system the channel coding is the AMI (Alternate mark inversion) for coaxial cable, and NRZ for optical fibre. The AMI code restricts the required bandwidth. In the bit-mapped system the encoded bit-stream, in NRZ format, is suitable for feeding both transmission media.

The bit-parallel interface defined in Recommendation CCIR 656 includes the possible addition of two bits to each word, thus enhancing the accuracy of the sample from 8 bits to 10 bits. In some applications, such as computer graphics, this improvement has been found advantageous. In the case of the serial interface of Recommendation CCIR 656, this extension is not feasible, thus limiting the application of the serial interface in both its electrical and optical forms. Certain Administrations are studying methods to convey a 10-bit word length in the serial interface, based on scrambled NRZ coding techniques.

# 6	Parallel-serial (hybrid) interfaces

[CCIR, 1982-86d and f] also discuss an alternative solution in which signals are divided into multiple channels of 108 Mbit/s each in order to reduce the bit rate per channel. This method also enables various members of the extensible family of compatible coding standards to be accommodated within a multi-channel arrangement. However, as stated in (CCIR, 1982-86k], the main advantage advocated for the hybrid interface is that it reduces the bit rate sent on each of the parallel cables, but if 2 parallel cables are used, which is the most frequent proposal, then the bit rate is halved but the new bit rate is still too high to be implemented by means of much cheaper technologies.

On the other hand, the use of a hybrid interface involves complications at the sending and receiving ends, where circuits are needed to multiplex and demultiplex the bit stream, and also to phase the bit streams received on the cables.

These complications, and the cost of the additional cable (or cables) in the hybrid interface, appear overwhelmingly to militate in favour of a fully serial interface, rather than a hybrid interface, in those cases when the parallel interface cannot be used.

# 7	Optical interfaces

Work has been reported concerning the optimum characteristics of an optical fibre interface for use in the studio, [CCIR, 1986c, d, e]. The use of a single-mode fibre driven by a laser or LED at a wavelength of approximately 1300 nm is suggested. Appendix 1 contains a draft text, as yet incomplete, to form the content of § 7 of Recommendation CCIR 656. Administrations are invited to make studies and contributions to complete this section in the current study period.

[CCIR, 1986-90fJ describes a new approach to the switching and routing of digital signals by optical means within a large studio centre. An arrangement is suggested in which the central routing switcher is eliminated by conveying all of the signals to every destination along a single optical fibre. The signals are assembled by a combination of time-division multiplexing (TDM) to a bit rate of the order of 2 Gbit/s, and optical wavelength-division multiplexing. The use of TDM means that the system is applicable to a wide range of bit rates including those required for digital HDTV. If this approach proves successful, appropriate interface specifications will be required. Document [CCIR, 1986-90g] described a method, applicable also to HDTV systems, for the transmission of three analogue wideband (up to 60 MHz) signals (R, G, B) through three optical fibres. The method used consists of the linearization of the characteristics of the optical device. The same document details the advantages of serial digital optical transmission at 1.15 Gbit/s for HDTV on a single fibre.

# 8	Practical implementation of interfaces

[Grimaldi et al, 1986] describes the all-digital studio in final implementation in France. Although some functions are still analogue (e.g. cameras) the system uses a large number of pieces of digital equipment, in particular a mixer/switcher, video tape recorders and miscellaneous other functions. This equipment is connected by coaxial cables using the serial interface of Recommendation CCIR 656, with some minor differences due to the early implementation. An optical link using the same signal format is operative over 6 km. A discussion of the solutions adopted is included.

[Baraclough et al, 1987] provides information on practical experience in the design, installation and operation of an experimental digital television production centre in the United Kingdom employing the parallel interface of Recommendation CCIR 656. The solutions adopted for problems encountered are given, including, for example, those associated with multiple equipment interconnections, synchronization and timing.

Further contributions on this subject are invited.

# 9	Interference with other services

Processing and transmission of digital data, such as digital video signals, at high data rates produces a wide spectrum of energy that has the potential to cause cross-talk or interference. In particular, attention is drawn in Recommendation CCIR 656 to the fact that the ninth and eighteenth harmonics of the 13.5 MHz sampling frequency (nominal value) specified in Recommendation CCIR 601 fall at the 121.5 and 243 MHz aeronautical emergency channels. Appropriate precautions must therefore be taken in the design and operation of interfaces to ensure that no interference is caused at these frequencies. Permitted maximum levels of radiated signals from digital data processing equipment are the subject of various national and international standards, and it should be noted that emission levels for such related equipment are given in CISPR Recommendation: “Information technology equipment -Limits of interference and measuring methods” Document CISPR/B (Central Office) 16.

In the case of the bit- parallel interface [CCIR, 1982-861] states that according to studies and experiments effected at the Canadian Broadcasting Corporation (CBC), with a correct shielding of the cables, no interference problem with other services is to be expected. This contribution recommends that radiation levels should comply with the limits given in Table I [CSA, 1983]. These limits are equivalent to those of the FCC in the United States of America.

TABLE 1

Limits of spurious emissions (CSA Class A)

| Frequency (MHz) | Maximum field strength (dB(µV/m)) at 30 m |
| --- | --- |
| 30-88 88-216 216-1 000 | 30 50 70 |

In relation to the bit serial interface [CCIR, 1982-86e] states that transmission by optical fibres eliminates radiation generated by the cable and also prevents conducted common-mode radiation, but the performance of coaxial cable can also be made near-perfect. It is believed that the major portion of any radiation would be from the processing logic and high-power drivers common to both methods. It adds that due to the wideband, random nature of the digital signal, little is gained by frequency optimization.

NOTE 1 – See Report ITU-R BT.1209.

# 10	Further studies

Further studies are required:

–	on interfaces for the 4:4:4 level, and for lower members of the family of digital coding standards;

–	to establish the types of ancillary signals to be carried, including their characterization and location in the data stream, and to propose international standards as necessary;

–	to determine what special provisions may be necessary in relation to the associated sound channels, for example, to avoid excessive relative time delays;

–	on the practical methods required to ensure acceptably low levels of radiated interference from the digital signals;

–	on optical interfaces for bit-serial signals.

References

BARACLOUGH, J.N., DALTON, C.J. and GREEN, N.W. [1987] Experience with an experimental digital production centre, IEE colloquium Digest 1987/11, p. 13/1-13/6.

CSA [1983] Electromagnetic emissions from data processing equipment and electronic office machines: limits and methods C-I08.8-MI983 - ISSN 0317-5669, Canadian Standards Association.

EBU [1983] EBU parallel interface for 625-line digital video signals. Tech. Document 3246.

EBU [1985] EBU serial interface for 625-line digital video signal. Tech. Document 3247.

GRIMALDI, J.L., NASSE, D. and CAYET, A. [1986] An experimental all-digital television centre, J.S.M.P.T.E., Vol. 95 No. I, Pt. 1, p. 13-19.

CCIR Documents

[1982.86]: a. 11/126 (EBU); b. 11/61 (United States of America); c. 11/94 (Canada); d. 11/24 (Japan); e. 11/291 (IWP11/7); f. 11/136 (OIRT); g. It1336 (Italy); h. 11/347 (Italy); i. 11/335 (Italy); j. 11/356 (Italy); k. 11/354 (Italy); I. 11/385 (Canada).

[1986-90]: a. IWP 11/7-257 (Australia); b. IWP 11/7-186 (OIRT); c. IWP 11/7-115 (United Kingdom); d. 11/112 (Canada); e. 11/124 (Canada); f. IWP 11/7-141 (United Kingdom); g. 11/28 (Thomson-CSF).

Bibliography

STICKLER, M.J., NASSE, D. and BRADSHAW, D. [August, 1984] The EBU bit-serial interface for 625-line digital video signals. EBU Rev. Tech., 212, 181-187.

Appendix 1 

Proposed draft additions to Recommendation CCIR 656
concerning an optical interface

# 7	Characteristics of the optical interface

## 7.1	Source characteristics

### 7.1.1 	Output wavelength

1300 nm nominal

Maximum spectral line width 150 nm between half power points.

### 7.1.2 	Output power

Maximum  0 dBm

Minimum  −25 dBm

### 7.1.3	Logic convention

Maximum power output corresponds to the signalling of a logical 1.

### 7.1.4	Rise and fall times

To be decided.

### 7.1.5 	Jitter

To be decided.

### 7.1.6	Isolation

Transmitter must withstand 10% of its output power returned by reflection.

## 7.2	Optical fibre link

FIBRE (compatible with optical fibre specified in CCITT Recommendation G.652)

Fibre type		–	single mode

Dimensions:	mode field dia. 	–	9-10 um +1-10%

cladding 	–	125 um

Operating window	–	around 1 300 nm

Mode field concentricity 	–	< 3 um

Cladding non circularity	–	< 2%

Cut-off wave length 	–	1 100-1 280 nm

Attenuation at 1 300 nm	–	< 1 dB/km

Max. dispersion (1 270-1 340 nm)	–	6 ps/nm.km

CONNECTOR

Type		–	biconical

## 7.3	Destination characteristics

### 7.3.1	Sensitivity

Input power for a mean bit error rate of 1 in 109 –35 dBm.

Maximum input power –20 dBm.

### 7.3.2	Maximum input power

Receiver shall operate with a mean bit-error rate better than 1 in 109 up to a power level of

Attachment 1

Attachment 2

Rec. 601 -1

SECTION 11F : DIGITAL METHODS OF TRANSMITTING TELEVISION INFORMATION Recommendations and Reports

RECOMMENDATION 601-1

ENCODING PARAMETERS OF DIGITAL TELEVISION FOR STUDIOS*
(Question 25/11, Study Programmes 25G/11, 25H/1 1)

(1982-1986)

The CCIR,

CONSIDERING

- that there are clear advantages for television broadcasters and programme producers in digital studio standards which have the greatest number of significant parameter values common to 525-line and 625-line systems,

- that a world-wide compatible digital approach will permit the development of equipment with many common features, permit operating economies and facilitate the international exchange of programmes;

- that an extensible family of compatible digital coding standards is desirable. Members of such a family could correspond to different quality levels, facilitate additional processing required by present production techniques, and cater for future needs;

- that a system based on the coding of components is able to meet some, and perhaps all, of these desirable objectives,

- that the co-siting of samples representing luminance and colour-difference signals (or, if used, the red, green and blue signals) facilitates the processing of digital component signals, required by present production techniques,

UNANIMOUSLY RECOMMENDS

that the following be used as a basis for digital coding standards for television studios in countries using the 525-line system as well as in those using the 625-line system:

- Component coding

The digital coding should be based on the use of one luminance and two colour-difference signals (or, if used, the red, green and blue signals).

The spectral characteristics of the signals must be restricted to eliminate aliasing. When using one luminance and two colour-difference signals as defined in Table I, this can be achieved by using filters as defined in Annex 111, Figs. I and 2. When using E'R, E',G, E'B  signals or luminance and colour-difference signals as defined in Table II of Annex I, the characteristics defined by Fig. 1 of Annex III will apply.

Note. — The values shown for the luminance filter, used when sampling at 13.5 MHz, given in Fig. 1 of Annex III should be considered as provisional. Administrations are requested to perform urgent studies to confirm these values.

- Extensible family of compatible digital coding standards

The digital coding should allow the establishment and evolution of an extensible family of compatible digital coding standards.

It should be possible to interface simply between any two members of the family.

The member of the family to be used for the standard digital interface between main digital studio equipment, and for international programme exchange (i.e. for the interface with video recording equipment and for the interface with the transmission system) should be that in which the luminance and colour-difference sampling frequencies are related in the ratio 4:2:2.

In a possible higher member of the family the sampling frequencies of the luminance and colour-difference signals (or, if used, the red, green and blue signals) could be related by the ratio 4 : 4 : 4. Tentative specifications for the 4:4:4 member are included in Annex I (see Note).

Note. — Administrations are urgently requested to conduct further studies in order to specify parameters of the digital standards for other members of the family. Priority should be accorded to the members of the family below 4 :2 :2. The number of additional standards specified should be kept to a minimum.

- Specifications applicable to any member of the family

3.1	Sampling structures should be spatially static. This is the case, for example, for the orthogonal sampling structure specified in § 4 of the present Recommendation for the 4:2:2 member of the family.

3.2     If the samples represent luminance and two simultaneous colour-difference signals, each pair of colour-difference samples should be spatially co-sited. If samples representing red, green and blue signals are used they should be co-sited.

3.3    The digital standard adopted for each member of the family should permit world-wide acceptance and application in operation; one condition to achieve this goal is that, for each member of the family, the number of samples per line specified for 525-line and 625-line systems shall be compatible (preferably the same number of samples per line).

- Encoding parameter values for the 4 : 2 : 2 member of the family

The following specification (Table I) applies to the 4 :2 :2 member of the family, to be used for the standard digital interface between main digital studio equipment and for international programme exchange.

TABLE I — Encoding parameter values.for the 4:2:2 member of the family

| Parameters | 525-line, 60 Field/s () systems | 625-line, 50 field/s(2) systems |
| --- | --- | --- |
| 1. Coded signals: Y, CR, CB | These signals are obtained from gamma pre-corrected signals, namely: E'Y, E'R — E'Y, E'B — E'Y (Annex I1, § 2 refers) |  |
| 2. Number of samples per total line: -luminance signal (Y) -each colour-difference signal(CR, CB) | 858 429 | 864 432 |
| 3. Sampling structure | Orthogonal, line, field and frame repetitive. CR and CB samples co-sited with odd (1st, 3rd, 5th, etc.) Y samples in each line. |  |
| 4. Sampling frequency: - luminance signal - each colour-difference signal | 13.5 MHz () 6.75 MHz(3) The-tolerance for the sampling frequencies should coincide with the tolerance for the line frequency of the relevant colour television standard |  |
| 5. Form of coding | Uniformly quantized PCM, 8 bits per sample, for the luminance signal and each colour-difference signal |  |
| 6. Number of samples per digital active line: - luminance signal - each colour-difference signal | 720 360 |  |
| 7. Analogue-to-digital horizontal timing relationship: - from end of digital active line to OH | 16 luminance clock periods 12 luminance clock periods | 12 luminance clock periods |
| 8. Correspondence between video signal levels and quantization levels: - scale - luminance signal - each colour-difference signal | - 0 to 255 - - 220 quantization levels with the black level corresponding to level 16 and the peak white level corresponding to level 235. The signal level may occasionally excurse beyond level 235 -225 quantization levels in the centre part of the quantization scale with zero signal corresponding to level 128 |  |
| 9. Code-word usage | Code-words corresponding to quantization levels 0 and 255 are used exclusively for synchronization. Levels 1 to 254 are available for video |  |

ANNEX I

TENTATIVE SPECIFICATION OF THE 4:4:4 MEMBER OF THE FAMILY

This Annex provides for information purposes a tentative specification for the 4:4:4 member of the family of digital coding standards.

The following specification could apply to the 4:4:4 member of the family suitable for television source equipment and high quality video signal processing applications.

TABLE 11 — A tentative specification for the 4:4:4 member of the family

| Parameters | 525-line, 60 field/s systems | 625-line, 50 field/s systems |
| --- | --- | --- |
| - Coded signals: Y, CR, CB or R, G, B | These signals are obtained from gamma pre-corrected signals, namely: E'Y, ER— E'Y, E'B — E'Y, or E’R, E’G, E’B |  |
| - Number of samples per total line for each signal | 858 | 864 |
| - Sampling structure | Orthogonal, line, field and frame repetitive. The three sampling structures to be coincident and coincident also with the luminance sampling structure of the 4 :2 :2 member |  |
| 4 Sampling frequency for each signal | - MHz |  |
| 5 Form of coding | Uniformly quantized PCM. At least 8 bits per sample |  |
| 6 Duration of the digital active line expressed in number of samples | At least 720 |  |
| 7 Correspondence between video signal levels and the 8 most significant bits (MBS) of the quantization level for each sample: - scale - R, G. B or luminance signal (') - each colour-difference signal () | - 0 to 255 - 220 quantization levels with the black level corresponding to level 16 and the peak white level corresponding to level 235. The signal level may occasionally excurse beyond level 235 225 quantization levels in the centre part of the quantization scale with zero signal corresponding to level 128 |  |

ANNEX II

DEFINITION OF SIGNALS USED IN THE DIGITAL CODING STANDARDS

Relationship of digital active line to analogue sync. reference

The relationship between 720 digital active line luminance samples and the analogue synchronizing references for 625-line and 525-line systems is shown below.

TABLE III

| 525-line, 60 field/s systems | 122 T | 720 T | 16 T |  |
| --- | --- | --- | --- | --- |
| 0H (leading edge of line syncs., period half-amplitude reference) |  | Digital active-line period | 0H | Next line |
| 625-line, 50 field/s systems | 132T | 720 T | 12 T |  |

T: one luminance sampling clock period (74 ns nominal).

The respective numbers of colour-difference samples can be obtained by dividing the number of luminance samples by two. The (12, 132) and (16, 122) were chosen symmetrically to dispose the digital active line about the permitted variations. They do not form part of the digital line specification and relate only to the analogue interface.

2.	Definition of the digital signals Y, CR, CB, from the primary (analogue) signals E'R, E'G and E'B

This section describes, with a view to defining the signals Y, CR, CB, the rules for construction of these signals from the primary analogue signals E'R, E'G and E'B. The signals are constructed by following the three stages described in § 2.1, 2.2 and 2.3 below. The method is given as an example, and in practice other methods of construction from these primary signals or other analogue or digital signals may produce identical results. An example is given in § 2.4.

2.1      Construction of luminance (E'Y) and colour-difference (E'R — E'Y) and (E'B — E'Y) signals

The construction of luminance and colour-difference signals is as follows:

ElY = 0.299E'R + 0.587E'G + 0.114E'B     (See Note)

whence:

(E'R — E'Y)= E'R — 0.299E'R — 0.587 E'G — 0.114E'B

= 0.701 E'R — 0.587E'G — 0.114E'B

and:

(E'B — E'Y)= E'B — 0.299E'R — 0.587E'G — 0114EIB

= —0.299 E'R — 0.587 E'G + 0.886 E'B

Note. — Report 624 Table II refers.

Taking the signal values as normalized to unity (e.g., 1.0 V maximum levels), the values obtained for white, black and the saturated primary and complementary colours are as follows:

TABLE IV

| Condition | E'R | E'G | E'B | E'Y | E'R— E'Y | E'B— E'Y |
| --- | --- | --- | --- | --- | --- | --- |
| White Black | 1.0 0 | 1.0 0 | 1.0 0 | 1.0 0 | 0 0 | 0 0 |
| Red Green Blue | 1.0 0 0 | 0 1.0 0 | 0 0 1.0 | 0.299 0.587 0.114 | 0.701 -0.587 -0.114 | -0.299 -0.587 0.886 |
| Yellow Cyan Magenta | 1.0 0 1.0 | 1.0 1.0 0 | 0 1.0 1.0 | 0.886 0.701 0.413 | 0.114 -0.701 0.587 | -0.886 0.299 0.587 |

2.2      Construction of re-normalized colour-difference signals (E'CR and ECB)

Whilst the values for Ely have a range of 1.0 to 0, those for (E'R –ElY) have a range of + 0.701 to –0.701 and for (E'B – ElY) a range of +0.886 to –0.886. To restore the signal excursion of the colour-difference signals to unity (i.e. +0.5 to –0.5), coefficients can be calculated as follows:

KR = 0.50.701 = 0.713;   KB = 0.50.886 = 0.564

Then:

E'CR = 0.713 (E’R – E'	Y ) =   0.500 ER – 0.419 E'G – 0.081 E'B

and:

E’CB = 0.564 (E'B - 	E'Y )   = –0.169E'R – 0.331 EG + 0.500E'B

where E'CR  and E'CB, are the re-normalized red and blue colour-difference signals respectively (see Notes 1 and 2).

Note 1. – The symbols E'CR and E'CB will be used only to designate re-normalized colour-difference signals, i.e. having the same nominal peak-to-peak amplitude as the luminance signal ElY, thus selected as the reference amplitude.

Note 2. — In the circumstances when the component signals are not normalized to a range of 1 to 0, for example, when converting from analogue component signals with unequal luminance and colour-difference amplitudes, an additional gain factor will be necessary and the gain factors KR, KB should be modified accordingly.

2.3	Quantization

In the case of a uniformly-quantized 8-bit binary encoding, 28, i.e. 256, equally spaced quantization levels are specified, so that the range of the binary numbers available is from 0000 0000 to 1111 1111 (00 to FF in hexadecimal notation), the equivalent decimal numbers being 0 to 255, inclusive.

In the case of the 4 : 2 : 2 system described in this Recommendation, levels 0 and 255 are reserved for synchronization data, while levels 1 to 254 are available for video.

Given that the luminance signal is to occupy only 220 levels, to provide working .margins, and that black is to be at level 16, the decimal value of the luminance signal, Y, prior to quantization, is:

Y= 219 (E'Y) + 16,

and the corresponding level number after quantization is the nearest integer value.

Similarly, given that the colour-difference signals are to occupy 225 levels and that the zero level is to be level 128, the decimal values of the colour-difference signals, CR and CB, prior to quantization are:

CR = 224 [0.713 (E'R — E'Y)] + 128

and:

CB  = 224 [0.564 (E'B — E'Y)] + 128

which simplify to the following:

CR  = 160 (E'R — E'Y)] + 128

and:

CB = 126 (E'S — E'Y)] + 128

and  the corresponding level number, after quantization, is the nearest integer value.

The digital equivalents are termed Y, CR and CB.

2.4	Construction of Y, CR, CB via quantization of E'R, EG, E'B

In the case where the components are derived directly from the gamma pre-corrected component signals E'R, EG, E'B, or directly generated in digital form, then the quantization and encoding shall be equivalent to:

Then:

taking the nearest integer coefficients, base 256. To obtain the 4:2:2 components Y, CR, CB, low-pass filtering and sub-sampling must be performed on the 4:4:4 CR, CB signals described above. Note should be taken that slight differences could exist between CR, CB components derived in this way and those derived by analogue filtering prior to sampling.

ANNEX III

Filtering Characteristics

5

0

4

0

3

0

2

0

1

0

0

0

1

2

3

4

5

6

7

8

9

1

0

1

1

1

2

1

3

1

4

1

5

12dB

40dB

0

1

2

3

4

5

6

0

1

2

3

4

5

6

0

0

5

–

5

dB

dB

ns

F

r

e

q

u

e

n

c

y

(

M

H

z

)

a

)

T

e

m

p

l

a

t

e

f

o

r

i

n

s

e

r

t

i

o

n

l

o

s

s

/

f

r

e

q

u

e

n

c

y

c

h

a

r

a

c

t

e

r

i

s

t

i

c

F

r

e

q

u

e

n

c

y

(

M

H

z

)

F

r

e

q

u

e

n

c

y

(

M

H

z

)

2ns

4

n

s

6

n

s

b

)

P

a

s

s

b

a

n

d

r

i

p

p

l

e

t

o

l

e

r

a

n

c

e

5

.

7

5

1

3

.

5

6

.

7

5

5

.

7

5

5

.

7

5

5

.

5

0

.

0

1

0

.

0

5

0

.

1

d

B

0

.

0

5

–

0

.

0

5

c

)

P

a

s

s

b

a

n

d

g

r

o

u

p

-

d

e

l

a

y

t

o

l

e

r

a

n

c

e

N

o

t

e

1

–

T

h

e

l

o

w

e

s

t

i

n

d

i

c

a

t

e

d

v

a

l

u

e

s

i

n

b

)

a

n

d

c

)

a

r

e

f

o

r

1

k

H

z

(

i

n

s

t

e

a

d

o

f

0

M

H

z

)

.

F

I

G

U

R

E

1

S

p

e

c

i

f

i

c

a

t

i

o

n

f

o

r

a

l

u

m

i

n

a

n

c

e

o

r

R

G

B

s

i

g

n

a

l

f

i

l

t

e

r

u

s

e

d

w

h

e

n

s

a

m

p

l

i

n

g

a

t

1

3

.

5

M

H

z

D

1

0

1

2

3

4

5

6

7

0

5

5

d

B

6

d

B

0

1

2

3

5

0

4

0

3

0

2

0

1

0

0

6

0

2

.

7

5

0

.

1

0

.

5

–

0

.

5

–

0

.

1

dB

F

r

e

q

u

e

n

c

y

(

M

H

z

)

a

)

T

e

m

p

l

a

t

e

f

o

r

i

n

s

e

r

t

i

o

n

l

o

s

s

/

f

r

e

q

u

e

n

c

y

c

h

a

r

a

c

t

e

r

i

s

t

i

c

F

r

e

q

u

e

n

c

y

(

M

H

z

)

b

)

P

a

s

s

b

a

n

d

r

i

p

p

l

e

t

o

l

e

r

a

n

c

e

F

I

G

U

R

E

3

S

p

e

c

i

f

i

c

a

t

i

o

n

f

o

r

a

d

i

g

i

t

a

l

f

i

l

t

e

r

f

o

r

s

a

m

p

l

i

n

g

-

r

a

t

e

c

o

n

v

e

r

s

i

o

n

f

r

o

m

4

:

4

:

4

t

o

4

:

2

:

2

c

o

l

o

u

r

-

d

i

f

f

e

r

e

n

c

e

s

i

g

n

a

l

s

(

dB

0

.

1

d

B

2

.

7

5

3

.

3

7

5

6

.

2

5

6

.

7

5

S

e

e

N

o

t

e

3

N

o

t

e

s

t

o

F

i

g

s

.

1

,

2

a

n

d

3

:

N

o

t

e

1

–

R

i

p

p

l

e

a

n

d

g

r

o

u

p

d

e

l

a

y

a

r

e

s

p

e

c

i

f

i

e

d

r

e

l

a

t

i

v

e

t

o

t

h

e

i

r

v

a

l

u

e

s

a

t

1

k

H

z

.

T

h

e

f

u

l

l

l

i

n

e

s

a

r

e

p

r

a

c

t

i

c

a

l

l

i

m

i

t

s

a

n

d

t

h

e

d

a

s

h

e

d

l

i

n

e

s

g

i

v

e

s

u

g

g

e

s

t

e

d

l

i

m

i

t

s

f

o

r

t

h

e

t

h

e

o

r

e

t

i

c

a

l

d

e

s

i

g

n

.

N

o

t

e

2

–

I

n

t

h

e

d

i

g

i

t

a

l

f

i

l

t

e

r

,

t

h

e

p

r

a

c

t

i

c

a

l

a

n

d

d

e

s

i

g

n

l

i

m

i

t

s

a

r

e

t

h

e

s

a

m

e

.

T

h

e

d

e

l

a

y

d

i

s

t

o

r

t

i

o

n

i

s

z

e

r

o

,

b

y

d

e

s

i

g

n

.

N

o

t

e

3

–

I

n

t

h

e

d

i

g

i

t

a

l

f

i

l

t

e

r

(

F

i

g

.

3

)

,

t

h

e

a

m

p

l

i

t

u

d

e

/

f

r

e

q

u

e

n

c

y

c

h

a

r

a

c

t

e

r

i

s

t

i

c

(

o

n

l

i

n

e

a

r

s

c

a

l

e

s

)

s

h

o

u

l

d

b

e

s

k

e

w

-

s

y

m

m

e

t

r

i

c

a

l

a

b

o

u

t

t

h

e

h

a

l

f

-

a

m

p

l

i

t

u

d

e

p

o

i

n

t

,

w

h

i

c

h

i

s

i

n

d

i

c

a

t

e

d

o

n

t

h

e

f

i

g

u

r

e

.

N

o

t

e

4

–

I

n

t

h

e

p

r

o

p

o

s

a

l

s

f

o

r

t

h

e

f

i

l

t

e

r

s

u

s

e

d

i

n

t

h

e

e

n

c

o

d

i

n

g

a

n

d

d

e

c

o

d

i

n

g

p

r

o

c

e

s

s

e

s

,

i

t

h

a

s

b

e

e

n

a

s

s

u

m

e

d

t

h

a

t

,

i

n

t

h

e

p

o

s

t

-

f

i

l

t

e

r

s

w

h

i

c

h

f

o

l

l

o

w

d

i

g

i

t

a

l

-

t

o

-

a

n

a

l

o

g

u

e

c

o

n

v

e

r

s

i

o

n

,

c

o

r

r

e

c

t

i

o

n

f

o

r

t

h

e

(

s

i

n

x

/

x

)

c

h

a

r

a

c

t

e

r

i

s

t

i

c

o

f

t

h

e

s

a

m

p

l

e

-

a

n

d

-

h

o

l

d

c

i

r

c

u

i

t

s

i

s

p

r

o

v

i

d

e

d

.

----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Main digital television terms used in the Recommendation are defined in Report 629

See Report 624, Table I.

The sampling frequencies of 13.5 MHz (luminance) and 6.75 MHz (colour-difference) are integer multiples of 2.25 MHz, the lowest common multiple of the line frequencies in 525/60 and 625/50 systems, resulting in a static orthogonal sampling pattern for both.

If used