REPORT  ITU-R  M.2039

Characteristics of terrestrial IMT-2000 systems 
for frequency sharing/interference analyses

(2004)

# 1	Introduction

IMT-2000 is an advanced mobile communication application concept intended to provide telecommunication services on a worldwide scale regardless of location, network or terminal used. At WARC-92, spectrum was identified for IMT-2000 in the bands 1 885-2 025 MHz and 2 110-2 200 MHz, including the bands 1 980-2 010 MHz and 2 170-2 200 MHz for the satellite component of IMT-2000. At WRC-2000, additional spectrum was identified for IMT-2000 in the bands 806-960 MHz, 1 710-1 885 MHz and 2 500-2 690 MHz.

Frequency sharing studies and interference analyses involving IMT-2000 systems and other systems and services operating in bands identified for IMT-2000 may need to be undertaken within ITU-R. To perform the necessary sharing studies between IMT-2000 systems and systems in other services, characteristics of the terrestrial component of IMT-2000 systems are needed.

This Report provides the baseline characteristics of terrestrial IMT-2000 systems for use in frequency sharing and interference analysis studies involving IMT-2000 systems and between IMT-2000 systems and other systems.

# 2	Characteristics

Table 1 provides an explanation of the terminology used for the IMT-2000 terrestrial technologies. Tables 2 and 3 contain typical technical and operational characteristics of IMT-2000 mobile and base stations systems, respectively. The values of some characteristics, such as data rates, etc. may vary when considering technology enhancements (such as high-rate packet data (HRPD) or high-speed downlink packet access (HSDPA)).

Additional information is contained in the references following these Tables.

TABLE  1

IMT-2000 terrestrial radio interfaces

| Full name | Common names |
| --- | --- |
| IMT-2000 CDMA Direct Spread | UTRA FDD WCDMA UMTS |
| IMT-2000 CDMA Multi-Carrier | CDMA2000 1X and 3X CDMA2000 1xEV-DO CDMA2000 1xEV-DV |
| IMT-2000 CDMA TDD (time-code) | UTRA TDD 3.84 Mchip/s high chip rate UTRA TDD 1.28 Mchip/s low chip rate (TD-SCDMA) UMTS |
| IMT-2000 TDMA Single-Carrier | UWC-136 EDGE |
| IMT-2000 FDMA/TDMA (frequency-time) | DECT |
| FDD: frequency division duplex TDD: time division duplex UTRA: universal terrestrial radio access |  |

TABLE  2

Characteristics of IMT-2000 mobile stations

| Parameter | IMT-2000 CDMA Direct Spread [1] | IMT-2000 CDMA Multi-Carrier(1) |  | IMT-2000 CDMA TDD (time-code) |  | IMT-2000 TDMA Single-Carrier |  | IMT-2000 FDMA/TDMA (frequency-time) [5] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1.28 Mchip/s low chip rate [2] | 3.84 Mchip/s high chip rate [2] |  |  |  |
| Carrier spacing | 5 MHz ± n × 0.2 MHz | 1.25 MHz (1X) | 3.75 MHz (3X) | 1.6 MHz  n × 0.2 MHz | 5 MHz  n × 0.2 MHz | 30 kHz [14] | 200 kHz [7] | 1.728 MHz |
| Duplex method | FDD | FDD | FDD | TDD | TDD | FDD | FDD | TDD |
| Transmitter power, dBm (typical)(2) | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 10 |
| Transmitter power, dBm (maximum) | 24 or 21 | 24 | 24 | 24 or 21 | 24 or 21 | 30 [15] | 30 [8] | 24 |
| Antenna gain (dBi) | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Antenna height (m) | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 |
| Access techniques | CDMA(3) | CDMA | CDMA | TDMA/CDMA | TDMA/CDMA | TDMA [15] | TDMA(4) | MC/TDMA(5) |
| Data rates supported | Pedestrian: 384 kbit/s, Vehicular: 144 kbit/s, Indoors: 2 Mbit/s Higher data rates up to 10 Mbit/s are supported by technology enhancements (HSDPA) [23] | Up to 625.35 kbit/s on forward link and up to 433.35 kbit/s on reverse link Higher data rates up to 2 457 kbit/s are supported by technology enhancements (HRPD) [22] | Up to 2 084.55 kbit/s on forward link and up to 1 354.95 kbit/s on reverse link | Pedestrian: 384 kbit/s, Vehicular: 144 kbit/s, Indoors: 2 Mbit/s Higher data rates up to 2.8 Mbit/s are supported by technology enhancements (HSDPA) [23] | Pedestrian: 384 kbit/s, Vehicular: 144 kbit/s, Indoors: 2 Mbit/s Higher data rates up to 10.2 Mbit/s are supported by technology enhancements (HSDPA) [23] | 13.0 kbit/s (/4 DQPSK) 19.95 kbit/s (8-PSK downlink) 18.6 kbit/s (8-PSK uplink) | 144 kbit/s [9] 384 kbit/s | 1.152 Mbit/s 32 kbit/s/ timeslot ( 2 Mbit/s with aggregated time slots and 8 level modulation) |

TABLE  2 (continued)

| Parameter | IMT-2000 CDMA Direct Spread [1] | IMT-2000 CDMA Multi-Carrier(1) |  | IMT-2000 CDMA TDD (time-code) |  | IMT-2000 TDMA Single-Carrier |  | IMT-2000 FDMA/TDMA (frequency-time) [5] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1.28 Mchip/s low chip rate [2] | 3.84 Mchip/s high chip rate [2] |  |  |  |
| Modulation type | HPSK(6) | QPSK/BPSK | QPSK/BPSK | QPSK/8-PSK | QPSK | /4-DQPSK 8-PSK | GMSK 8-PSK | GMSK (BT  0.5) ( multi-level modulation options) |
| Emission bandwidth | [1] | [20] | [20] | [2] | [2] | [16] |  | [5] |
| –3 dB |  |  |  |  |  |  | 0.12 MHz [10], 0.12 MHz [11] |  |
| –20 dB |  |  |  |  |  |  | 0.18 MHz [10], 0.18 MHz [11] |  |
| –60 dB |  |  |  |  |  |  | 0.40 MHz [10], 0.60 MHz [11] |  |
| Receiver NF, (worst case) | 9 dB | 9 dB | 9 dB | 9 dB | 9 dB | 9 dB | 9 dB | 10 dB |
| Thermal noise in specified bandwidth(7) | 108 dBm in 3.84 MHz | –113 dBm | –108 dBm | –113 dBm in 1.28 MHz | 108 dBm in 3.84 MHz | 128 dBm(8) | 121 dBm(9) | 113 dBm in 1.152 MHz |
| Receiver thermal noise level | –99 dBm in 3.84 MHz | 125 dBm(10) 113 dBm 104 dBm(12) | 125 dBm(11) 113 dBm 99 dBm(13) | –104 dBm in 1.28 MHz | 99 dBm in 3.84 MHz | 119 dBm | 112 dBm | 102 dBm in 1.728 MHz |

TABLE  2 (continued)

| Parameter | IMT-2000 CDMA Direct Spread [1] | IMT-2000 CDMA Multi-Carrier(1) |  | IMT-2000 CDMA TDD (time-code) |  | IMT-2000 TDMA Single-Carrier |  | IMT-2000 FDMA/TDMA (frequency-time) [5] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1.28 Mchip/s low chip rate [2] | 3.84 Mchip/s high chip rate [2] |  |  |  |
| Receiver bandwidth | [1] | [20] | [20] | [2] | [2] | [17] | [12] | [5] |
| –3 dB |  |  |  |  |  |  |  |  |
| –20 dB |  |  |  |  |  |  |  |  |
| –60 dB |  |  |  |  |  |  |  |  |
| Eb/N0 for Pe  103 |  | [20] | Performance not available |  |  | 7.8 dB | 8.4 dB | 11 dB (non-coherent detection) |
| Receiver reference sensitivity(14), Îor | 117 dBm in 3.84 MHz | 104 dBm total received power in fully loaded system. Single 9 600 bit/s traffic channel is at –119.6 dBm in AWGN for 0.5% FER | 99 dBm total received power in fully loaded system. Single 9 600 bit/s traffic channel is at –119.6 dBm in AWGN for 0.5% FER | –108 dBm in 1.28 MHz | 105 dBm in 3.84 MHz | 113 dBm [18] | 102 dBm [9] | –94 dBm typical (spec.: –86 dBm for speech and generally 83 dBm) |
| Interference threshold(15) | 105 dBm in 3.84 MHz | 110 dBm in 1.25 MHz | 105 dBm in 3.75 MHz | –110 dBm in 1.28 MHz | 105 dBm in 3.84 MHz | No equivalent | [13] | 105 dBm typical (–97 dBm for specification speech) |
| Transmitter ACLR | [1] | [20](16) | [20](17) | [2] | [2] |  |  | [5] |

TABLE  2 (end)

| Parameter | IMT-2000 CDMA Direct Spread [1] |  | IMT-2000 CDMA Multi-Carrier(1) |  | IMT-2000 CDMA TDD (time-code) |  | IMT-2000 TDMA Single-Carrier |  | IMT-2000 FDMA/TDMA (frequency-time) [5] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 1.28 Mchip/s low chip rate [2] | 3.84 Mchip/s high chip rate [2] |  |  |  |  |
| 1st adjacent channel | 33 dB @  5 MHz | 31.6 dB @  3.75 MHz |  | –33 dBc in 3.84 MHz @  3.08 MHz | 33 dB @  1.6 MHz | 33 dB @  5 MHz |  |  |  |  |
| 2nd adjacent channel | 43 dB @  10 MHz | 48.2 dB @  8.75 MHz |  | –43 dBc in 3.84 MHz @  8.08 MHz | 43 dB @  3.2 MHz | 43 dB @  10 MHz |  |  |  |  |
| Transmitter spurious emissions | [1] | [20] |  | [20] | [2] | [2] |  |  |  | [5] |
| Receiver ACS | 33 dB | 64 dB(18) |  | 50 dB | 33 dB | 33 dB |  |  |  |  |
| Receiver blocking levels | [1] | [20] |  | [20] | [2] | [2] |  |  |  | [5] |
| ACS: adjacent channel selectivity ACLR: adjacent channel leakage power ratio AWGN: additive White Gaussian noise FER: frame error rate GMSK: Gaussian filtered minimum shift keying. (1) The IMT-2000 minimum performance requirements recorded here for IMT-2000 CDMA multicarrier are defined in the band class 6 (i.e. 2 GHz band) requirements in [20]. This is also relevant to the technology enhancements (HRPD) requirements contained in [22]. (2) May not be appropriate for all scenarios, for example when calculating aggregate interference from all users in a cell. (3) Desired signal at sensitivity, I/N  6 dB for a 10% loss in range applicable to cases where interference effects a limited number of cells. In other cases, e.g. international coordination with BSS sound in the 2.5 GHz band a trigger value of I/N  10 dB is appropriate. (4) TDMA, comprising 8 timeslots (577 s) per single TDMA frame (4.615 ms). For user packet data service, 1-4 timeslots per frame may be used by mobile stations having multi-slot classes that do not require simultaneous transmission and reception, i.e. classes for which a duplexer is not required. (5) Ten frequency channels with 24 time slots (32 kbit/s) per frame. The frame length is 10 ms. |  |  |  |  |  |  |  |  |  |  |

Notes relative to Table 3 (end): (6) Hybrid phase shift keying: a method peculiar to IMT-2000 CDMA Direct Spread in which the peak to average ratio is reduced in comparison to a QPSK signal by mixing the orthogonal variable spreading factor (OSVF) with both information sources as real signals, i.e. those destined for I and Q modulation components, and then shifting one component by 90 to produce an equivalent imaginary signal and then utilizing gain control on the Q channel to preserve orthogonality. (7) 10 log (k T b) + 30 (dBm) where: k Boltzman’s constant = 1.38  10–23, T reference temperature = average Earth temperature = 277 K, b noise equivalent bandwidth (Hz). (8) In the receiver bandwidth. (9) In the receiver bandwidth. (10) In bandwidth equal to data rate: for IMT-2000 CDMA multicarrier, values are given for 9 600 bit/s speech services and nominal supported rate (153.6 kbit/s) for data services. (11) In bandwidth equal to data rate: for IMT-2000 CDMA multicarrier, values are given for 9 600 bit/s speech services and nominal supported rate (153.6 kbit/s) for data services. (12) In the receiver bandwidth. (13) In the receiver bandwidth. (14) For a 103 raw bit error rate, Îor, the received power spectral density (integrated in a bandwidth of (1 + α) times the chip rate and normalized to the chip rate) of the downlink signal as measured at the UE antenna connector. (15) I/N  6 dB for a 10% loss in range applicable to cases where interference effects a limited number of cells. In other cases, e.g. sharing with BSS (sound) in the 2 630-2 655 MHz band, a value of I/N = –10 dB is appropriate. (16) Currently [20], [21] and [22] do not contain explicit 1X mobile station or base station ACLR requirements. Nevertheless, the 1X spectrum emission limits described in [20] already provide protection of adjacent channels. A lower bound for the effective ACLR can be calculated by integrating the maximum allowed 1X emissions over a 3.84 MHz integration bandwidth centred at the specified frequency offset are considered. Results summarized in this Table are calculated by assuming a 24 dBm mobile station output power, and a one 43 dBm output power base station. The actual 1X ACLR value in practical implementations will be considerably better since the emission limits (i.e. flat mask, no slope) in the region of the second adjacent channel do not realistically model a power amplifier emissions roll-off. (17) The requirements at offsets of 3.08 and 8.08 MHz are equivalent to ACLR requirements of 33 and 43 dB from a 3X mobile station transmitter into a 3X or IMT-DS mobile station receiver offset by 5 and 10 MHz respectively. With regard to base stations, [19] currently does not contain an explicit ACLR requirement for base stations. Nevertheless, the 1X spectrum emission limits described in [19] already provide protection of adjacent channels. A lower bound for the effective ACLR can be calculated by integrating the maximum allowed emissions of three neighbouring IMT-MC 1X channels over a 3.84 MHz integration bandwidth centred at the specified frequency offset. Results summarized in this Table are produced assuming three adjacent 38 dBm output power 1X base stations; the aggregate output power over the 5 MHz of assigned channels is 43 dBm. (18) The test equipment ACLR (i.e. in-band emissions contributions) effectively limits the mobile station ACS that can be tested.

TABLE  3

Characteristics of IMT-2000 base stations

| Parameter | IMT-2000 CDMA Direct Spread [3], [6] |  |  | IMT-2000 CDMA Multi-Carrier(1) |  |  |  |  |  | IMT-2000 CDMA TDD (time-code) |  |  |  |  |  | IMT-2000 TDMA Single-Carrier(2) |  |  |  |  | IMT-2000 FDMA/TDMA (frequency-time) [5] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | 1.28 Mchip/s low chip rate [4] |  |  | 3.84 Mchip/s high chip rate [4] |  |  |  |  |  |  |  |  |
| Carrier spacing | 5 MHz  n × 0.2 MHz |  |  | 1.25 MHz (1X) |  |  | 3.75 MHz (3X) |  |  | 1.6 MHz  n  0.2 MHz |  |  | 5 MHz  n × 0.2 MHz |  |  | 30 kHz |  | 200 kHz |  |  | 1.728 MHz |
| Duplex method | FDD |  |  | FDD |  |  | FDD |  |  | TDD |  |  | TDD |  |  | FDD |  | FDD |  |  | TDD |
| Cell type | Macro | Micro | Pico | Macro | Micro | Pico | Macro | Micro | Pico | Macro | Micro | Pico | Macro | Micro | Pico | Macro | Micro |  |  | Pico | Omni |
| Transmitter power dBm(3) | 43 | 38 | 24 | 40 | tbd | tbd | 40 | tbd | tbd | 43 | tbd | tbd | 43 | tbd | tbd | 40 | tbd |  |  | tbd | 24 |
| Antenna gain(4), (5) (dBi/120 sector) | 17 | 5 | 0 | 17 | tbd | tbd | 17 | tbd | tbd | 17 | 5 | 0 | 17 | 5 | 0 | 17 | tbd |  |  | tbd | Maximum 12 Normal 0 |
| Antenna height (m) | 30 | 5 | 1.5 | 30 | tbd | tbd | 30 | tbd | tbd | 30 | 5 | 1.5 | 30 | 5 | 1.5 | 30 | tbd |  |  | tbd | 1.5-10 (typical 2.5) |
| Tilt of antenna (degrees down) | 2.5 | 0 | 0 | 2.5 | tbd | tbd | 2.5 | tbd | tbd | 2.5 | 0 | 0 | 2.5 | 0 | 0 | 2.5 | tbd |  |  | tbd | tbd |
| Access techniques | CDMA |  |  | CDMA |  |  | CDMA |  |  | TDMA/CDMA |  |  | TDMA/CDMA |  |  | TDMA |  |  | TDMA |  | MC/TDMA |
| Data rates supported | Pedestrian: 384 kbit/s, Vehicular: 144 kbit/s, Indoors: 2 Mbit/s Higher data rates up to 10 Mbit/s are supported by technology enhancements (HSDPA), [23] |  |  | Up to 625.35 kbit/s on forward link and up to 433.35 kbit/s on reverse link Higher data rates up to 2 457 kbit/s are supported by technology enhancements (HRPD), [21] |  |  | Up to 2 084.55. kbit/s on forward link and up to 1 354.95 kbit/s on reverse link |  |  | Pedestrian: 384 kbit/s, Vehicular: 144 kbit/s, Indoors: 2 Mbit/s Higher data rates up to 2.8 Mbit/s are supported by technology enhancements (HSDPA), [23] |  |  | Pedestrian: 384 kbit/s, Vehicular: 144 kbit/s, Indoors: 2 Mbit/s Higher data rates up to 10.2 Mbit/s are supported by technology enhancements (HSDPA), [23] |  |  | 30 kbit/s 44 kbit/s |  |  | 384 kbit/s |  | 1.152 Mbit/s 32 kbit/s/timeslot ( 2 Mbit/s with aggregated time slots and 8 level modulation) |

TABLE  3 (continued)

| Parameter | IMT-2000 CDMA Direct Spread [3], [6] | IMT-2000 CDMA Multi-Carrier(1) |  | IMT-2000 CDMA TDD (time-code) |  | IMT-2000 TDMA Single-Carrier(2) |  | IMT-2000 FDMA/TDMA (frequency-time) [5] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1.28 Mchip/s low chip rate [4] | 3.84 Mchip/s high chip rate [4] |  |  |  |
| Modulation type | QPSK | QPSK/BPSK 8-PSK/16-QAM(6) | QPSK/BPSK | QPSK/8-PSK | QPSK | /4-DQPSK 8-PSK | GMSK 8-PSK | GMSK (BT  0.5) ( multi-level modulation options) |
| Emission bandwidth | [3] | [19] | [19] | [4] | [4] |  |  | [5] |
| 3 dB |  |  |  |  |  | 0.03 MHz | 0.18 MHz |  |
| 20 dB |  |  |  |  |  | 0.03 MHz | 0.22 MHz |  |
| 60 dB |  |  |  |  |  | 0.04 MHz | 0.24 MHz |  |
| Receiver noise figure (worst case) | 5 dB for macro BS | 5 dB | 5 dB | 7 dB for macro BS | 5 dB for macro BS | 5 dB | 5 dB | 10 dB |
| Receiver thermal noise level | 103 dBm in 3.84 MHz for macro BS | 129 dBm 117 dBm(7) 108 dBm(8) | 129 dBm 117 dBm(9) 103 dBm(10) | 106 dBm in 1.28 MHz for macro BS | 103 dBm in 3.84 MHz for macro BS | 125 dBm(11) | 117 dBm(12) | 103 dBm in 1.152 MHz |
| Receiver bandwidth | < 5 MHz [3] | [19] | [19] |  1.6 MHz [4] | < 5 MHz [4] |  |  | [5] |
| 3 dB |  |  |  |  |  | 0.03 MHz | 0.18 MHz |  |
| 20 dB |  |  |  |  |  | 0.04 MHz | 0.25 MHz |  |
| 60 dB |  |  |  |  |  | 0.09 MHz | 0.58 MHz |  |
| Eb/N0 for Pe = 103 | [3] | [19] | Performance not available |  |  | 7.8 dB | 8.4 dB | 11 dB (non-coherent detection) |

TABLE  3 (continued)

| Parameter | IMT-2000 CDMA Direct Spread [3], [6] | IMT-2000 CDMA Multi-Carrier(1) |  | IMT-2000 CDMA TDD (time-code) |  | IMT-2000 TDMA Single-Carrier(2) |  | IMT-2000 FDMA/TDMA (frequency-time) [5] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1.28 Mchip/s low chip rate [4] | 3.84 Mchip/s high chip rate [4] |  |  |  |
| Receiver reference sensitivity(13) | 121 dBm(14) for macro BS –111 dBm for micro BS –107 dBm for pico BS | 119 dBm for fundamental channel in AWGN | 119 dBm for fundamental channel in AWGN | –110 dBm for macro and micro BS –96 dBm for pico BS | 109 dBm for macro and micro BS –95 dBm for pico BS | 117 dBm | 108 dBm | 94 typical (specification: –86 dBm for speech and generally 83 dBm) |
| Interference threshold for macro BS 1(15) | 109 dBm in 3.84 MHz(16) | 114 dBm in 1.25 MHz | 109 dBm in 3.75 MHz | –112 dBm in 1.28 MHz | 109 dBm in 3.84 MHz | 131 dBm | 123 dBm | 105 dBm typical (97 dBm for speech specification) |
| Transmitter ACLR for macro/micro/ pico BS | [3], [6] | [19](17) | [19](18) | [4] | [4] |  |  |  |
| 1st adjacent | 45 dB @  5 MHz | 50.8 dB @  3.75 MHz | 49.3 dB @  5 MHz | 40 dB @  1.6 MHz | 45 dB @  5 MHz |  |  |  |
| 2nd adjacent | 50 dB @  10 MHz | 67.2 dB @  8.75 MHz | 62.2 dB @  10 MHz | 45 dB @  3.2 MHz | 55 dB @  10 MHz |  |  |  |
| Transmitter spurious emissions | [3], [6] | [19] | [19] | [4] | [4] |  |  |  |
| Macro BS receiver ACS (relative ACS) | 52 dBm (46 dB)(16) | –53 dBm | –49 dBm | 55 dBm (46 dB)(16) | 52 dBm (46 dB)(16) |  |  |  |

TABLE  3 (end)

| Parameter | IMT-2000 CDMA Direct Spread [3], [6] | IMT-2000 CDMA Multi-Carrier(1) |  | IMT-2000 CDMA TDD (time-code) |  | IMT-2000 TDMA Single-Carrier(2) |  | IMT-2000 FDMA/TDMA (frequency-time) [5] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1.28 Mchip/s low chip rate [4] | 3.84 Mchip/s high chip rate [4] |  |  |  |
| Micro BS receiver ACS (relative ACS) | 42 dBm (46 dB)(16) | tbd | tbd | 41 dBm (46 dB)(16) | 38 dBm (46 dB)(19) |  |  |  |
| Pico BS receiver ACS (relative ACS) | –38 dBm (46 dB)(16) | tbd | tbd | –41 dBm (46 dB)(16) | 38 dBm (46 dB)(16) |  |  |  |
| Receiver blocking levels | [3], [6] | [19] | [19] | [4] | [4] |  |  |  |
| tbd: to be determined (1) The IMT-2000 minimum performance requirements recorded here for IMT-2000 CDMA multicarrier are defined in the band class 6 (i.e. 2 GHz band) requirements in [19]. This is also relevant to the technology enhancements (HRPD) requirements contained in [21]. (2) IMT-2000 TDMA single carrier consists of three components: enhancements to the 30 kHz channels (designated as 136) for advanced voice and data capabilities, a 200 kHz carrier component for high speed data (384 kbit/s) accommodating high mobility (designated as 136HS outdoor), and a 1.6 MHz carrier component for very high speed data (2 Mbit/s) in low mobility applications (designated as 136HS indoor). The combined result constitutes the IMT-2000 radio interface referred to as IMT-2000 TDMA single carrier. (3) May not be appropriate for all scenarios. (4) Feeder losses are not included in the values and should be considered in the sharing/compatibility issues. (5) The reference pattern is specified in Recommendation ITU-R F.1336 with (k  0.2). (6) Both HRPD and IMT-2000 CDMA multicarrier revision C support 8-PSK and 16-QAM on the forward packet channel. (7) In bandwidth equal to data rate: for IMT-2000 CDMA multicarrier, values are given for 9 600 bit/s speech services and nominal supported rate for data services. (8) In the receiver bandwidth. |  |  |  |  |  |  |  |  |

Notes relative to Table 3 (fin): (9) In bandwidth equal to data rate: for IMT-2000 CDMA multicarrier, values are given for 9 600 bit/s speech services and nominal supported rate for data services. (10) In the receiver bandwidth. (11) In bandwidth equal to data rate: for IMT-2000 CDMA multicarrier, values are given for 9 600 bit/s speech services and nominal supported rate for data services. (12) In bandwidth equal to data rate: for IMT-2000 CDMA multicarrier, values are given for 9 600 bit/s speech services and nominal supported rate for data services. (13) For a 103 raw bit error rate, theoretical Eb /N0. (14) The thermal noise figure for a WCDMA receiver is –108 dBm based on k T f where k is Boltzmann’s constant (1.38  10–23), T is the temperature (K), and f is the bandwidth (Hz). For a noise figure of 4 dB (typical value for a base station receiver), the thermal noise becomes –104 dBm. However, receiver sensitivity depends on the service (i.e. voice, packet, etc.). For example, the voice (DTCH 32) sensitivity for the base station receiver is –121 dBm for BER  0.001. (15) I/N  6 dB for a 10% loss in range applicable to cases where interference effects a limited number of cells. In other cases, e.g. sharing with BSS (sound) in the 2 630-2 655 MHz band a value of I/N  –10 dB is appropriate. (16) The tolerable I/N thresholds are as follows: coordinated use (–6 dB), agreement trigger (–10 dB), licence exempt (–20 dB). (17) Currently [20], [21] and [22] do not contain explicit 1X mobile station or base station ACLR requirements. Nevertheless, the 1X spectrum emission limits described in [20] already provide protection of adjacent channels. A lower bound for the effective ACLR can be calculated by integrating the maximum allowed 1X emissions over a 3.84 MHz integration bandwidth centred at the specified frequency offset are considered. Results summarized in this Table are calculated by assuming a 24 dBm mobile station output power, and a one 43 dBm output power base station. The actual 1X ACLR value in practical implementations will be considerably better since the emission limits (i.e. flat mask, no slope) in the region of the second adjacent channel do not realistically model a power amplifier emissions roll-off. (18) The requirements at offsets of 3.08 and 8.08 MHz are equivalent to ACLR requirements of 33 and 43 dB from a 3X mobile station transmitter into a 3X or IMT-DS mobile station receiver offset by 5 and 10 MHz respectively. With regard to base stations, [19] currently does not contain an explicit ACLR requirement for base stations. Nevertheless, the 1X spectrum emission limits described in [19] already provide protection of adjacent channels. A lower bound for the effective ACLR can be calculated by integrating the maximum allowed emissions of three neighbouring IMT-MC 1X channels over a 3.84 MHz integration bandwidth centred at the specified frequency offset. Results summarized in this Table are produced assuming three adjacent 38 dBm output power 1X base stations; the aggregate output power over the 5 MHz of assigned channels is 43 dBm. (19) The absolute ACS values are the test values as specified in 3GPP TS25.104 and TS 25.105. The following conversion formula: ACS_relative  ACS_test – Noise_floor – 10*log10(10M/10–1) can be used to derive relative ACS values, where M is the margin (dB) used in the ACS test, which is the useful signal level above the reference sensitivity level. For both IMT-2000 CDMA direct spread and IMT-2000 CDMA TDD (time code), M  6 dB. ACS relative values are often used in sharing studies.

TABLE  4

An example of IMT-2000 assumed traffic model characteristics for a mature network(1)

| Parameter | Value |
| --- | --- |
| Traffic environments | Rural Vehicular Pedestrian In-building (central business district) |
| Maximum data rates | Rural – 9.6 kbit/s Vehicular – 144 kbit/s Pedestrian – 384 kbit/s In-building – 2 Mbit/s |
| Cell size | Rural – 10 km radius Vehicular – 1 000 m radius Pedestrian – 315 m radius In-building – 40 m radius |
| Users per cell during busy hour | Rural – not significant Vehicular – 4 700 Pedestrian – 42 300 In-building – 1 275 |
| Percent of total uplink traffic  64 kbit/s during busy hour | Rural – not significant Vehicular – 34% Pedestrian – 30% In-building – 28% |
| Percent of total downlink traffic  64 kbit/s during busy hour | Rural – not significant Vehicular – 78% Pedestrian – 74% In-building – 73% |
| Average number of users per cell per MHz during busy hour assuming frequency duplex operation | Rural – not significant Vehicular  64 kbit/s – 16  64 kbit/s – 4 Pedestrian  64 kbit/s – 150  64 kbit/s – 64 In-building  64 kbit/s – 4  64 kbit/s – 2 |
| (1) Values in Table 4 were derived from Report ITU-R M.2023. |  |

References

[1]	3GPP TS 25.101 v5.5.0 (2002-12) 3rd Generation Partnership Project; Technical Specification Group Radio Access Networks; UE Radio Transmission and Reception (FDD) (Release 5).

[2]	3GPP TS 25.102 v5.3.0 (2002-12): 3rd Generation Partnership Project; Technical Specification Group Radio Access Networks; UE Radio Transmission and Reception (TDD) (Release 5).

[3]	3GPP TS 25.104 v6.0.0 (2002-12): 3rd Generation Partnership Project; Technical Specification Group Radio Access Networks; BS Radio Transmission and Reception (FDD) (Release 6).

[4]	3GPP TS 25.105 v5.3.0 (2002-12): 3rd Generation Partnership Project; Technical Specification Group Radio Access Networks; BS Radio Transmission and Reception (TDD) (Release 5).

[5]	Final Draft ETSI EN 300 175-2 v1.6.0 (2001-04): Digital Enhanced Telecommunications (DECT) Common Interface (CI) part 2: Physical Layer.

[6]	3GPP TR 25.951 v1.5.0 (2003-02): 3rd Generation Partnership Project; Technical Specification Group Radio Access Networks, FDD Base Station Classification (Release 6).

[7]	TR45 technical specification, TIA/EIA-136-290); RF Minimum performance requirements 136HS Outdoor and 136HS Indoor Bearers, clause 2.

[8]	TR45 technical specification, TIA/EIA-136-290; RF Minimum performance requirements 136HS Outdoor and 136HS Indoor Bearers, clause 4.1.1.2 refers to Power Class II mobile station.

[9]	TR45 technical specification, TIA/EIA-136-290; RF Minimum performance requirements 136HS Outdoor and 136HS Indoor Bearers, clause 6.2 specifies data rates and reference sensitivity. Reference sensitivity listed for 144 kbit/s at a 10% block erasure rate (BLER).

[10]	TR45 technical specification, TIA/EIA-136-290; RF Minimum performance requirements 136HS Outdoor and 136HS Indoor Bearers, Table A3a: Modulation and noise spectrum mask due to GMSK modulation. Measurement bandwidth is 30 kHz.

[11]	TR45 technical specification, TIA/EIA-136-290; RF Minimum performance requirements 136HS Outdoor and 136HS Indoor Bearers, Table A3b: Modulation and noise spectrum mask due to 8-PSK modulation. Measurement bandwidth is 30 kHz.

[12]	TR45 technical specification, TIA/EIA-136-290; RF Minimum performance requirements 136HS Outdoor and 136HS Indoor Bearers, clause 5.1:

The mobile station shall meet the requirements set forth in clause 6.2 in the presence of an unmodulated carrier at the following frequencies and amplitudes:

TABLE  5

Requirements of clause 6.2

| Frequency of blocking signal | Amplitude of blocking signal (dBm) |
| --- | --- |
| 600 kHz <  ¦f – f0¦  800 kHz | 43 |
| 800 kHz <  ¦f – f0¦  1.6 MHz | 43 |
| 1.6 MHz < = ¦f – f0¦  3 MHz | 33 |
| 3 MHz = ¦f – f0¦ | 26 |

[13]	TR45 technical specification, TIA/EIA-136-290; RF Minimum performance requirements 136HS Outdoor and 136HS Indoor Bearers, clause 6.3:

In Table 6 the reference co-channel interference (C/Ic), block error rate (BLER) performance is defined for each of the channel conditions. The actual interference ratio is defined as the interference ratio for which this performance is met. The actual interference ratio shall be less than a specified limit, called the reference interference ratio. For 200 kHz bearers the reference interference ratio shall be, for BTS and all types of MS:

TABLE  6

Input signal level and interference ratio for outdoor 
BTS at reference performance

| Bearer | Environment | Speed (km/h) | Coding scheme | Error rate | C/I (dB) |
| --- | --- | --- | --- | --- | --- |
| 136HS Outdoor | Pedestrian A | 3 | GCS-1 | 10% BLER | 7 |
| 136HS Outdoor | Pedestrian A | 3 | GCS-2 | 10% BLER | 8.5 |
| 136HS Outdoor | Pedestrian A | 3 | GCS-3 | 10% BLER | 9.5 |
| 136HS Outdoor | Pedestrian A | 3 | GCS-4 | 10% BLER | 13.5 |
| 136HS Outdoor | Pedestrian A | 3 | PCS-1 | 10% BLER | 13 |
| 136HS Outdoor | Pedestrian A | 3 | PCS-2 | 10% BLER | 16 |
| 136HS Outdoor | Pedestrian A | 3 | PCS-3 | 10% BLER | 18 |
| 136HS Outdoor | Pedestrian A | 3 | PCS-4 | 10% BLER | 19.5 |
| 136HS Outdoor | Pedestrian A | 3 | PCS-5 | 10% BLER | 21 |
| 136HS Outdoor | Pedestrian A | 3 | PCS-6 | 10% BLER | 24.5 |
| 136HS Outdoor | Vehicular A | 50 | GCS-1 | 10% BLER | 3.5 |
| 136HS Outdoor | Vehicular A | 50 | GCS-2 | 10% BLER | 7 |
| 136HS Outdoor | Vehicular A | 50 | GCS-3 | 10% BLER | 8.5 |
| 136HS Outdoor | Vehicular A | 50 | GCS-4 | 10% BLER | 17 |
| 136HS Outdoor | Vehicular A | 50 | PCS-1 | 10% BLER | 9 |
| 136HS Outdoor | Vehicular A | 50 | PCS-2 | 10% BLER | 13 |
| 136HS Outdoor | Vehicular A | 50 | PCS-3 | 10% BLER | 14.5 |
| 136HS Outdoor | Vehicular A | 50 | PCS-4 | 10% BLER | 18 |
| 136HS Outdoor | Vehicular A | 50 | PCS-5 | 10% BLER | 21 |
| 136HS Outdoor | Vehicular A | 50 | PCS-6 | 10% BLER | (see Note 1) |
| 136HS Outdoor | Vehicular A | 120 | GCS-1 | 10% BLER | 7 |
| 136HS Outdoor | Vehicular A | 120 | GCS-2 | 10% BLER | 8.5 |
| 136HS Outdoor | Vehicular A | 120 | GCS-3 | 10% BLER | 9.5 |
| 136HS Outdoor | Vehicular A | 120 | GCS-4 | 10% BLER | 13.5 |
| 136HS Outdoor | Vehicular A | 120 | PCS-1 | 10% BLER | 13 |
| 136HS Outdoor | Vehicular A | 120 | PCS-2 | 10% BLER | 16 |
| 136HS Outdoor | Vehicular A | 120 | PCS-3 | 10% BLER | 18 |
| 136HS Outdoor | Vehicular A | 120 | PCS-4 | 10% BLER | 19.5 |
| 136HS Outdoor | Vehicular A | 120 | PCS-5 | 10% BLER | 21 |
| 136HS Outdoor | Vehicular A | 120 | PCS-6 | 10% BLER | 24.5 |
| NOTE 1 – This is the GMSK interfering channel. The channel models in the above Table are taken directly from Recommendation ITU-R M.1225. |  |  |  |  |  |

[14]	TR45 technical specification, SP-4027-270b); Mobile Station Minimum Performance, clause 2.3.1.3.1.

[15]	TR45 technical specification, SP-4027-270b); Mobile Station Minimum Performance, clause 1.4 and clause 3.2.2. Refers To Power Class Ii Mobile Station.

[16]	TR45 technical specification, SP-4027-270b); Mobile Station Minimum Performance, clause 3.4.1.1.3.

[17]	TR45 technical specification, SP-4027-270b); Mobile Station Minimum Performance, clause 2.3.2.4.3:

TABLE  7

Blocking and spurious response rejection

| Frequency band | Desired signal (frequency, fc) | Blocking signal (frequency, f0) | Spurious response limit (frequency, f0) | Error rate (%) |
| --- | --- | --- | --- | --- |
| ¦fc-f0¦  3 MHz /4 DQPSK) | 102 | 30 | 45 | 3 |
| 3 MHz  ¦fc-f0¦  90 kHz /4 DQPSK) | 102 | 45 | 45 | 3 |
| ¦fc-f0¦  3 MHz (8-PSK) | 99 | 30 | 45 | 3 |
| 3 MHz  ¦fc-f0¦ > 90 kHz (8-PSK) | 99 | 45 | 45 | 3 |

[18]	TR45 technical specification, SP-4027-270b); Mobile Station Minimum Performance, clause 2.3.1.1.3.

[19]	TR45 technical specification, TIA-97-E; Recommended minimum performance Standards for cdma2000 spread spectrum base stations.

[20]	TR45 technical specification, TIA-98-E; Recommended minimum performance Standards for cdma2000 spread spectrum mobile stations.

[21]	TR45 technical specification, TIA-864-E; Recommended minimum performance Standards for cdma2000 High Rate Packet Data Access Network.

[22]	TR45 technical specification, TIA-866-E; Recommended minimum performance Standards for cdma2000 High Rate Packet Data Access Terminal.

[23]	3GPP TS 25.308 v5.4.0 (2003-03); 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; High Speed Downlink Packet Access (HSDPA); Overall description; Stage 2 (Release 5).

Bibliography

TR45 technical specification, TIA/EIA 136-131; Digital Traffic Channel Layer 1, clause 1.3.

3GPP TS 25.942; 3rd Generation Partnership Project; Technical Specification Group Radio Access Networks; RF System Scenarios, clause 4.1.1.2. Body loss expectation is that values are similar for all technologies. Footnote retained for information purposes.