RESOLUTION 741 [COM5/1] (WRC-03)

Protection of the radio astronomy service in the band 4 990-5 000 MHz from 
unwanted emissions of the radionavigation-satellite service (space-to-Earth) 
operating in the frequency band 5 010-5 030 MHz

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that unwanted emissions from space stations of the radionavigation-satellite service (RNSS) operating in the frequency band 5 010-5 030 MHz may cause interference to the radio astronomy service (RAS) in the band 4 990-5 000 MHz;

b)	that WRC-2000 decided to introduce a provisional power flux-density (pfd) limit in the band 4 990-5 000 MHz to protect the RAS, and invited ITU-R to conduct studies to review this limit;

c)	that protection requirements for the RAS are given in Recommendations ITU-R RA.769 and ITU-R RA.1513, and are different for geostationary (GSO) and non-geostationary (non-GSO) satellite systems,

noting

a)	that Recommendation ITU-R M.1583 provides a methodology based on the equivalent pfd (epfd) concept for calculation of interference resulting from unwanted emissions from non-GSO systems of the mobile-satellite or radionavigation-satellite services into radio astronomy stations;

b)	that Recommendation ITU-R RA.1631 provides antenna patterns and maximum antenna gain to be used for compatibility analyses between non-GSO systems and RAS stations based on the epfd concept;

c)	that Recommendation ITU-R RA.1513 recommends acceptable levels of data loss to radio astronomy observations, stating in particular that the percentage of data loss caused by any system should be lower than 2%,

resolves

1	that in order not to cause harmful interference to the radio astronomy service in the band 4 990-5 000 MHz, the pfd produced in this band by any GSO RNSS network operating in the 5 010-5 030 MHz band shall not exceed –171 dB(W/m2) in a 10 MHz band at any radio astronomy station;

2	that in order not to cause harmful interference to the radio astronomy service in the band 4 990-5 000 MHz, over the whole sky, for elevations higher than the minimum operating elevation angle min1 specified for the radio telescope, the epfd produced in this band by all space stations within any non-GSO RNSS system operating in the 5 010-5 030 MHz band shall not exceed 245 dB(W/m2) in a 10 MHz band at any radio astronomy station for more than 2% of the time, using the methodology in Recommendation ITU-R M.1583 and a reference antenna with a radiation pattern and maximum antenna gain given in Recommendation ITU-R RA.1631;

3	that the limits referred to in resolves 1 and 2 shall apply to RNSS systems as from 3 June 2000;

4	that administrations planning to operate a GSO or a non-GSO RNSS system in the band 5 010-5 030 MHz, for which complete coordination or notification information, as appropriate, has been received by the Bureau after 2 June 2000, shall send to the Radiocommunication Bureau the value of the maximum level of pfd as referred to in resolves 1 or the value of the maximum level of epfd as referred to in resolves 2, as appropriate,

instructs the Radiocommunication Bureau

as from the end of WRC-03, to review all RNSS systems for which complete coordination or notification information, as appropriate, has been received by the Bureau before the end of WRC-03 for the band 5 010-5 030 MHz, and, if appropriate, to revise its findings regarding compliance with No. 5.443B, taking into account additional information received under resolves 4.

1	 	Until adoption of a definition of min by ITU-R, and publication of notified radio astronomy observatory data, a value of 5° should be assumed in appropriate calculations.