# Chapter 4

# Space Science Services & Radioastronomy

The meeting regarding Chapter: 4 “Space Science Services & Radioastronomy took place Tuesday, April 13, 1999, starting at 3.45 hs. and chaired by the group coordinator Mr. Eduardo Gabelloni (Argentina).

Documents included in OD-2 and  documents 1210, 1211 Corr.1, and 1220 attributed to the group meeting were examined.

Agenda Item 1.16:  to consider allocation of frequency bands above 71 GHz to the earth exploration-satellite (passive) and radio astronomy services, taking into account Resolution 723 (WRC-97) (COM5-1);

ISSUE:  Passive Allocations above 71 GHz

BACKGROUND:  The passive services, (e.g. for remote sensing, radio astronomy, and meteorological observations) have substantial interests in the above 71 GHz frequency range, up to the 275 GHz limit of the allocation table, and beyond.  The active services are interested in using this spectrum in the future, e.g. for applications involving high data rate transmission and because propagation characteristics allow extensive frequency reuse.

There are a number of space science missions employing passive sensing instruments at frequencies above 71 GHz and many more are planned. Two Preliminary View documents were submitted on this Agenda item by the administrations along with one preliminary draft proposal for realignment of the 71 – 275 GHz band.

Considerable discussion took place among Working Group 4 members in regard to the content of the preliminary draft proposal and the possible resulting consequences to active service allocations within the band.  Comments were provided by one administration on the preliminary draft proposal from the perspective of the Fixed Satellite Service.  The Working Group decided that due to the extensive nature of the proposed allocations changes throughout the 71 – 275 GHz band, it was premature to put the proposal forward for consideration as a Joint CITEL Proposal.  Instead, the Working Group has included the preliminary draft proposal in an appendix to this chapter (see Annex 1).   Also included in the appendix are additional clarifying comments taken from draft CPM text developed by WP 7D and the comments pertaining to the FSS generated at this meeting.

#### Views of Member states

One administration noted that it is following the work of WP7D and is in the process of identifying requirements for radio astronomy and earth exploration-satellite (passive) services above 71 GHz.  The administration also stated in its preliminary view that they agree in general with the principles put forth in the CPM text pertaining to this agenda item as proposed by WP 7D at its March 1999 meeting.  They also noted that other radio services should retain, and have access to, spectrum equivalent to their current allocations in the Radio Regulations.

Another administration indicated that WRC-2000 offers an opportunity to provide additional allocations to satisfy passive service needs, to protect existing allocations for future use, and to re-organize the allocations above 71 GHz as needed so that spectrum can be successfully shared between passive and active services in that range.  The interests of other services allocated in this portion of the spectrum must also be safeguarded and satisfied as well.  The administration submitted a proposal for consideration by the Working Group that follows the principles outlined in the WP 7D CPM text.  This proposal has been extensively coordinated with those responsible for preparation of the CEPT proposal on this agenda item.  Significant agreement has been reached between this proposal and the one under development within CEPT.  The band re-allocation will meet the needs of all the radio services involved.

Agenda Item 1.17: to consider possible worldwide allocation for the earth exploration-satellite (passive) and space research (passive) services in the band 18.6 - 18.8 GHz, taking into account the results of the ITU-R studies;

ISSUE: Earth exploration-satellite (passive) service allocation in the band 18.6 – 18.8 GHz

BACKGROUND:  Passive spaceborne sensors provide environmental data on soil moisture, water salinity, ocean temperature, rain, snow, ice, sea state, and oil spills.  A set of frequency bands is essential for these measurements because simultaneous measurements at several frequencies are needed to isolate any single effect since the energy emitted at any one frequency is determined by several overlapping effects.  The band between 18.6 and 18.8 GHz is one component in this set, without which these environmental data could not be obtained.  At present, the allocation for the Earth exploration-satellite (passive) service is on a primary basis in Region 2, but on a secondary basis in Regions 1 and 3.

At WRC-97, a joint CITEL proposal was presented to upgrade the allocations to the Earth exploration-satellite (passive) service to primary.  This proposal also included a pfd limit of -101 dBW/m2 in a reference bandwidth of 200 MHz on the fixed-satellite service.  A limit of -3 dBW on the power in the band that is delivered to an antenna of a fixed service station with a -10 dBi backlobe gain was also included in the proposal.  At the conference, agreement could not be reached on the appropriate limits on the fixed and fixed-satellite services to enable passive sensors to perform their mission while permitting acceptable operation of fixed and fixed-satellite service systems.

Draft CPM text developed by WP 7D (WRC-2000) proposes a revised pfd limit of –95 dBW/m2/200MHz for FSS systems and a revised power limit of 0 dBW in 200 MHz at the input of FS antenna as compromise values to enable the co-existence of EESS (passive) with the other services co-allocated in the band.

#### preliminary common views

The preliminary view of CITEL administrations on this agenda item is that all incumbent services including the FS, FSS and EESS (passive) should be able to co-exist in the frequency band 18.6-18.8 GHz. The CPM report to WRC-97 discussed the difference in the pfd levels required for the protection to EESS (passive) services and those required for the operation of FSS services. At WRC-97 various options were discussed to reduce the current pfd values for FSS in the Radio Regulations. While there was some progress made towards arriving at a compromise pfd value in order to accommodate the global requirements for EESS (passive) services, this matter was not concluded at WRC-97. The current draft CPM text developed by WP 7D (WRC-2000) proposes a pfd limit of –95 dBW/m2/200MHz for FSS systems and a power limit of 0 dBW in 200 MHz at the input of FS antenna. The CITEL administrations supported the view that these values can form the technical basis for sharing between passive sensors and FSS/FS systems.

Additionally, one administration submitted a draft proposal to upgrade EESS (passive) to primary in Regions 1&3, using the limits on FSS power flux density and FS power delivered to the antenna as developed in WP 7D (WRC-2000) and noted above. This proposal was agreed to as a Draft CITEL Proposal and appears as Annex 2 to this document.  CITEL members are encouraged to review this proposal prior to the XIII the meeting of PCC.III with a view to make it a CITEL Common Proposal.

ANNEX 1

PRELIMINARY DRAFT PROPOSAL

AGENDA ITEM 1.16

To consider allocations of frequency bands above 71 GHz to the earth exploration-satellite (passive) and radio astronomy services, taking into account Resolution 723;

Background Information: The following tables depict an outline of work toward modifications to the allocation tables above 71 GHz.  These modifications will accommodate the requirements of the passive sensing systems in the radio astronomy and earth exploration-satellite (passive) services and are consistent with the principles set forth in the CPM text developed by WP 7D at the March 1999 meeting.  The following information, which has been excerpted from the WP 7D CPM text, is provided to aid CITEL Administrations in their understanding of the motivation for the extensive modifications to the allocations in the 71 – 275 GHz band.  It should be recognized that passive allocation requirements are driven in response to observational requirements determined by natural (e.g., astrophysical or atmospheric) processes.

#### Services allocated in the 71 - 275 GHz frequency range

The frequency range under consideration includes exclusive allocations to the passive services, as well as some shared, co-primary allocations with the fixed, mobile, inter-satellite (ISS) and fixed-satellite (FSS) services. The passive services are also allocated by footnotes RR S5.149 and RR S5.340 to a number of bands and/or subbands that are shared with other services. The amateur, amateur-satellite, radiolocation, radionavigation, radionavigation-satellite, broadcasting, broadcasting-satellite (BSS), mobile-satellite (MSS), ISS and FSS have primary allocations, and the space research (space-to-Earth), amateur, amateur-satellite and radiolocation have secondary allocations in the frequency range 71 - 275 GHz.

#### Astronomical importance of the millimetre and submillimetre bands

Radio astronomy at millimetre and submillimetre wavelengths developed greatly during the last two decades, and is possibly the fastest growing area within radio astronomy. Growth is driven by the scientific opportunities available only in this frequency range, (e.g. in astrochemistry, physics of stellar and circumstellar envelopes, formation of stars, planets and protoplanets, chemical composition of the interstellar medium, physical conditions in the early Universe and Galaxy formation, etc.) which has been made possible by technological developments, such as, the availability of low noise receivers and the development of millimetre wave instrumentation in general. Ground-based radio telescopes in the mm-range are currently operated by administrations in all three ITU Regions, up to frequencies of 900 GHz.

Millimetre-wave observatories represent a major investment and are, therefore, required to maximize their scientific return. Consequently, great care is being taken to locate them at the best possible sites. To cover the entire sky, there is a need to have access to good observing sites in the both northern and southern hemispheres. Extremely dry locations at high elevations are preferred. Several large telescopes (interferometers and single dish telescopes), that will cover the millimetre and submillimetre wavelength ranges are either under construction or in advanced stages of planning. The US MMA, the European Large Southern Array (LSA), and the Japanese Large Millimeter and Submillimeter Array (LMSA) are to be located in Chile, and will likely be combined into one international megascience project. Other large telescopes under construction are the US Smithsonian Astrophysical Observatory's Submillimeter Array (SMA) and the Large Millimeter Telescope (LMT), a US-Mexico collaborative project, being built in Mexico. In addition to these large international facilities there are some others scattered around the world, that have a more restricted range of applications.

As an example of the new telescopes using the most up-to-date technology, the MMA-LSA-LMSA is planned to consist of 40 - 60 parabolic antennas of 12-metre diameter. From just above the oxygen absorption band near 60 GHz to near 850 GHz in the submillimetre range, there will be a series of receivers at each antenna which will cover most of the spectrum. The remoteness of the site will allow access to most of the millimetre wavelength spectrum, except that which contains emissions from satellites or aircraft.

Observations at millimetre-submillimetre wavelengths from space also contribute significantly to several areas of radio astronomy. Sharing criteria applicable to ground-based radio astronomy are usually not applicable to radio astronomy from space.

#### Access to spectral lines of importance to radio astronomy

Over 2 100 spectral lines of 80 chemical compounds have been identified to date, many detected lines are yet to be identified, and more species and lines are predicted. Many of the lines are astrophysically very important (see e.g., Recommendation ITU-R RA.314-8), and interest in others may be triggered by new discoveries. To facilitate sharing of the spectrum with other users, the International Astronomical Union (IAU) periodically revises the list of lines of most astrophysical interest. Radio astronomers are interested in preserving access to as many of these spectral lines as possible, because they may yield unique information about physical processes in certain celestial objects.

Astronomical discoveries of new molecular species have stimulated other scientific fields such as molecular spectroscopy and quantum chemistry, with potential practical applications. One example of a discovery directly stimulated by millimeter-wave radio astronomy is the discovery of fullerenes, first detected in the laboratory experiments designed to understand the chemistry in atmospheres of giant stars, observed to produce abundant long carbon-chain molecules, which brought about the development of an entire new branch of chemistry.

There are also many astrophysically important spectral lines at frequencies above 275 GHz (see e.g., Recommendation ITU-R RA.314-8). Many millimetre/submillimetre observatories now operate up to 900 GHz and space-borne telescopes will soon operate above 1 000 GHz. Table 1 lists the frequency bands in the 71 - 275 GHz, containing most spectral lines of interest to radio astronomy that are accessible from the ground.

Table 1

| Frequency range (GHz) | Number of detected molecular lines | Lines listed in ITU-R Rec. RA 314-8 |
| --- | --- | --- |
| 71.0 - 86.0 | 326 | 1 |
| 86.0 - 92.0 | 224 | 7 |
| 92.0 - 94.0 | 21 | 1 |
| 94.1 - 105.0 | 322 | 1 |
| 105.0 - 116.0 | 194 | 4 |
| 136.0 - 170.0 | 208 | 3 |
| 200.0 - 209.0 | 9 | 0 |
| 215.0 - 217.0 | 49 | 0 |
| 217.0 - 231.0 | 321 | 3 |
| 231.0 - 265.0 | 450 | 1 |
| 265.0 - 275.0 | 15 | 3 |

#### Scientific requirements for spaceborne passive sensing in the 71 - 275 GHz frequency range

Almost all of the current spaceborne passive sensing allocations in the range 71 - 275 GHz were decided by WARC-79. Many technological and scientific advances and discoveries have occurred since that time and these allocations need to be revised to reflect present and foreseeable future requirements for spaceborne passive sensing for a myriad of Earth observation applications including agriculture, climatology, meteorology and study of global change of the Earth and its environment. The frequency range 71 - 275 GHz is of fundamental importance for atmospheric sounding of temperature and water vapour and retrieval of ozone and other radiatively and chemically active atmospheric trace gases. Additionally, increased awareness of the stress being placed on the Earth system has led the global user community, including the World Meteorological Organization, Global Climate Observing System, and World Climate Research Programme, to define satellite data requirements for atmospheric parameters including temperature and water vapour profiles, ozone concentration, and other radiatively and chemically active trace gases, which can only be met by satellite passive sensors.

Information on the spaceborne passive sensing spectrum requirements in the range 71 - 275 GHz are given in Table 2. General comments on the scientific applications of each of these bands is also given. It should be noted that many atmospheric constituent lines (e.g., nitrous oxide, carbon monoxide, chlorine, ozone, etc.) exist throughout this frequency range and only those lines deemed most critical have been included in the table. Furthermore, multiple lines attributed to the same atmospheric constituent are often required in order to map out the atmospheric concentrations at different altitudes.

TABLE 2

| Bands required for spaceborne passive sensing | Scientific applications |
| --- | --- |
| 86 - 92 GHz | Reference window used in conjunction with the oxygen absorption band around 118.75 GHz. Band also used for clouds and precipitation measurements. |
| 100 - 102 GHz | Band contains a nitrous oxide line near 101 GHz for atmospheric limb sounding. |
| 109.8 - 111.8 GHz | Band contains ozone line at 110.8 GHz for microwave limb sounding. |
| 114.25 - 122.25 GHz | Band is a unique natural resource for sounding the temperature of the lower troposphere from geostationary orbiting passive sensors using the oxygen absorption line at 118.75 GHz. Also, contains carbon monoxide line at 115.27 GHz for microwave limb sounding. |
| 148.5 - 151.5 GHz and 164 - 167 GHz | These bands are necessary to collect meteorological information at the Earth's surface as well as near the surface of the Earth and is used in conjunction with the water vapour line near the 183.31 GHz for atmospheric sounding. Additionally, these bands contain a nitrous oxide line at 150.74 GHz and a ClO line at 164.38 GHz for microwave limb sounding. |
| 174.8 - 191.8 GHz | This band is a unique natural resource to provide a complete tropospheric water vapour vertical profile from LEO passive sensors and from geostationary orbiting passive sensors using the water vapour absorption line whose peak is at 183.31 GHz. |
| 200 - 209 GHz | This band is the optimal band for microwave limb sounding of water vapour and other atmospheric constituents in the low troposphere. |
| 226 - 231.5 GHz | This band is required for microwave limb sounding of various atmospheric constituents and also to provide 4 GHz reference window for water vapour sounding near 325 GHz. |
| 235 - 238 GHz | This band contains two critical ozone lines for microwave limb sounding of the atmosphere at 235.71 GHz and 237.15 GHz. |
| 250 - 252 GHz | This band contains a nitrous oxide line near 251 GHz for use for microwave limb sounding applications. |

#### Methods to satisfy the agenda item 1.16

In proposing to satisfy the agenda item, the following considerations should be kept in mind:

–	Passive allocations should be made in response to observational requirements determined by natural (e.g., astrophysical or atmospheric) processes.

–	Passive allocations in the range 71 - 275 GHz need to be revised to reflect current scientific requirements. There is a continued requirement for allocations for some exclusive passive bands in the millimetre-wave region, because: a) Passive sensors of the EES (passive) service are operated on a global basis and coordination with other services is generally not practical; b) space-based radio telescopes can observe in bands of high atmospheric opacity that may not be observed from the ground; c) space-based radio telescopes are also used in conjunction with ground-based radio telescopes for VLBI, and some allocations are needed which are accessible to both terrestrial and space-based radio telescopes; d) passive sensor atmospheric measurements are also taken from Earth-based platforms in many passive bands.

–	The 71 - 275 GHz part of the Table of Frequency Allocations was adopted at WARC-79, as what was then thought to be a reasonable balance among the various services that are involved. While the technology for the active services has not been yet developed to a level that would permit the use of these services, it is foreseen that the technology is going to be available and that active service systems will be implemented in the future. Based on the rapidly increasing use of the frequency bands below 71 GHz by the active services, it is clear that future systems will be required to operate in the frequency bands above 71 GHz. Consequently, any changes to the frequency allocations in the 71 - 275 GHz range must continue to accommodate active service requirements. In particular, as much as practicable:

–	changes in the amount of spectrum allocated to other services above 71 GHz should be minimized;

–	additional allocations should not affect existing sharing arrangements between other services;

–	splintering of allocated bands should be avoided.

#### Comments on the Preliminary Draft Proposal for the Frequency Band 71 – 275 GHz

To further aid in the review of the allocations modifications in the tables below, the following comments are provided which bare mainly upon the Fixed Satellite Service (FSS) allocations.  In addition to the comments below, it was noted by this administration that it would be desirable for the FSS and the Fixed Service (FS) not to share spectrum allocations in these bands due to sharing difficulties resulting from deployment of ubiquitous terminals by both services.

- 71 – 74 GHz

The interchange of FSS uplinks and downlinks in the 70/80 GHz band segments has resulted in the space-to-Earth links having somewhat increased atmospheric absorption.  Higher absorption on the space-to-Earth links is less desirable than on the Earth-to-space links because of the difficulty of generating the increased power in the spacecraft.  The increased absorption as a result of this change, however, may be a relatively small factor when compared against the margin that may be needed in the space-to-Earth link to overcome rain attenuation.

- 74 – 76 GHz

The co-primary allocation of FSS and BSS may be problematic because both services are likely to require bi-directional operation to ubiquitous terminals.  The trend for consolidation of FSS and BSS at these high frequencies may obviate this problem, however, it is not certain whether such consolidation will in fact occur.

- 76 – 77.5 GHz

The radio astronomy allocation in this and several other frequency bands are subject to Res ZZZ because it is foreseen that the basic requirements for locating observatories for these bands will lead to a very small number of observatories.  Such observatories will be located in remote radio-quite zones that are arid and at very high altitude, above the majority of the atmosphere and water vapor.  A possible approach for increased sharing with FSS systems in some frequency bands allocated to radio astronomy may be to restrict to a very small value the number of radio astronomy observatory locations that would be allowed to observe at a given frequency range.  This could allow the remaining global area to be available for allocation to the FSS in those frequency bands.  Studies could be carried out by the ITU-R to determine the optimum locations for such observatories.  Use of this concept may lead to increased flexibility in the allocation of the spectrum.  It should be noted, however, that other than in exclusive passive bands, the radio astronomy allocations are shared with the Fixed Service on a co-primary basis.  Consideration of the usefulness of the approach discussed here must also take into account the difficulties in sharing between the FS and FSS as discussed above.

4.  95 – 105 GHz

The removal of FSS and MSS from this band in order to accommodate radio astronomy spectral lines and wideband continuum observations at a relatively few remote locations seems to be a measure that might otherwise have been achieved by using the approach in comment 3.  Alternately, portions of the band could be used for FSS (Earth-to-space) which may be compatible with the radio astronomy service.

- 116 – 119.8 GHz

The restriction of several Inter-satellite Service (ISS) allocations to GSO satellites could be limiting.  The proposal, however, does provide 28 GHz of ISS allocation on a non-restricted basis which may be adequate provision for future NGSO systems.  An additional 18 GHz of ISS allocation are subject to the GSO restriction.  The current allocations provide a total of 35 GHz for ISS use.  Current speculation that NGSO systems will be more likely to use optical inter-satellite methods rather than to employ RF methods in the ISS allocations above 71 GHz, is premature at this time.  The choice between optical and RF methods will be dictated only by future technology development.

- 123 – 130 GHz

The FSS and MSS allocations have no directionality assigned.  It is probably now appropriate to assign a direction.  (Such decision should be left to the FSS and MSS communities).  The co-primary allocation to FSS and MSS is likely prudent since the distinctions between these services at this frequency band will likely be very small.

- 151 – 158.5 GHz

It may be appropriate to consider application of the concept described in comment 3 in this band segment.

- General Comment

The EESS (passive) and radio astronomy services should be encouraged to identify mitigation techniques which may permit them to achieve their desired measurements but occupy a smaller portion of the radio spectrum.

| GHz 71 - 126 |  |  |
| --- | --- | --- |
| Current Allocations | Proposed Allocations | Reason for Change/Remarks |
| Regions 1, 2, 3 | Regions 1, 2, 3 |  |
| 71- 74 FIXED FIXED-SATELLITE (Earth-to-space) MOBILE MOBILE-SATELLITE (Earth-To-Space) S5.149 S5.556 | 71 - 74 FIXED FIXED-SATELLITE (Earth-to-space) FIXED-SATELLITE (space-to-Earth) MOBILE MOBILE-SATELLITE (Earth-To-Space) MOBILE-SATELLITE (space-to-Earth) S5.149 S5.556 | Uplinks and downlinks in 71-74 GHz and 81-84 GHz bands interchanged. Atmospheric absorption slightly higher in 71-74 GHz band than in 81-84 GHz band. RAS Footnotes S5.149 and S5.556 deleted, in favor of allocations above 76 GHz. Reference to 72.77-72.91 GHz band in S5.149 and S5.556 deleted. |
| 74-75.5 FIXED FIXED-SATELLITE (Earth-to-space) MOBILE Space Research (space-to-Earth) | 74 - 75.5 76 AMATEUR AMATEUR-SATELLITE BROADCASTING-SATELLITE FIXED FIXED-SATELLITE (Earth-to-space) FIXED-SATELLITE (space-to Earth) MOBILE MOBILE-SATELLITE (Earth-to-space) Space Research (space-to-Earth) S5.556 MOD S5.561 S5.EEE | BSS currently allocated to the 84-86 GHz band relocated to this band, to protect RAS above 76 GHz. Atmospheric absorption slightly higher in 74-76 GHz band than in 84-86 GHz band Amateur and Amateur-Satellite allocations shifted to 80.5-81 GHz . Footnote S5.EEE protects existing Amateur and Amateur-Satellite operations in the 75.5-76 GHz band until the year 200[X]. FSS (Earth-to-space) allocation moved to 84-86 GHz band. The proposed allocations in the 74-84 GHz range preserve a contiguous 10 GHz space research downlink (secondary), required for space VLBI purposes. Footnote S5.561 modified to recognize change in BSS allocation. |
| 75.5-76 AMATEUR AMATEUR-SATELLITE Space Research (space-to-Earth) |  |  |

| GHz 71 -126 (continued) |  |  |
| --- | --- | --- |
| Current Allocations | Proposed Allocations | Reason for Change/ Remarks |
| Regions 1, 2, 3 | Regions 1, 2, 3 |  |
| 76-81 RADIOLOCATION Amateur Amateur-Satellite Space Research (space-to-Earth) S5.560 | 76 - 77.5 RADIO ASTRONOMY S5.ZZZ RADIOLOCATION Amateur Amateur-Satellite Space Research (space-to-Earth) S5.560 MOD S5.149 | Radio astronomy allocation and new footnote S5.ZZZ, along with RES ZZZ satisfy requirement for radio astronomy spectral line and wide band continuum observations from a few remote locations, world-wide. Amateur and amateur-satellite services shifted by 0.5 GHz, to accommodate BS, FSS and MSS downlinks at lower portion of atmospheric window. No change in sharing between services, except for introduction of RAS allocation in band. Band added to those listed under S5.149 S5.560 doesn't apply to this band. |
|  | 77.5 - 78 Amateur Amateur-Satellite RADIOLOCATION Amateur Amateur-Satellite Radio Astronomy Space Research (space-to-Earth) MOD S5.149 | Band added to those listed under S5.149 RAS added on a secondary basis. |
|  | 78 - 81 RADIO ASTRONOMY S5.ZZZ RADIOLOCATION Amateur Amateur-Satellite Space Research (space-to-Earth) S5.560 MOD S5.149 | Radio astronomy allocation and new footnote S5.ZZZ, along with RES ZZZ, satisfy requirement for radio astronomy spectral line and wide band continuum observations from a few remote locations world-wide. No change in sharing between services, except for introduction of RAS allocation in band. Band added to footnote S5.149. |

| GHz 71 -126 (continued) |  |  |
| --- | --- | --- |
| Current Allocations | Proposed Allocations | Reason for Change/ Remarks |
| Regions 1, 2, 3 | Regions 1, 2, 3 |  |
| 81-84 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE MOBILE-SATELLITE (space-to-Earth) Space Research (space-to-Earth) | 81 - 84 FIXED FIXED SATELLITE (space-to-Earth) FIXED-SATELLITE (Earth-to-space) MOBILE MOBILE-SATELLITE (Earth-to-space) RADIO ASTRONOMY S5.ZZZ Space Research (space-to-Earth) S5.561 MOD S5.149 S5.DDD | Direction of satellite downlinks reversed, to allow radio astronomy observations. Uplink paired with 71-74 GHz downlink. Radio astronomy allocation and new footnote S5.ZZZ, along with RES ZZZ, satisfy requirement for radio astronomy spectral line and wide band continuum observations from a few remote locations, world-wide. Footnote S5.DDD added to maintain amount of secondary amateur and amateur-satellite allocation. Band added to footnote S5.149. |
| 84-86 FIXED MOBILE BROADCASTING BROADCASTING-SATELLITE S5.561 | 84 - 86 BROADCASTING BROADCASTING-SATELLITE FIXED FIXED-SATELLITE (Earth-to-space) MOBILE RADIO ASTRONOMY S5.ZZZ MOD S5.149 S5.561 | Broadcasting Satellite allocation relocated to 74-76 GHz band. Direction of satellite downlinks reversed, to allow radio astronomy observations. Uplink paired with 74-76 GHz downlink. Radio astronomy allocation and new footnote S5.ZZZ, along with RES ZZZ, satisfy requirement for radio astronomy spectral line and wide band continuum observations from a few remote locations, world-wide. Band added to footnote S5.149. Footnote no longer needed |
| 86-92 EARTH EXPLORATION SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) S5.340 | 86-92 EARTH EXPLORATION SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) S5.340 | NOC This band is of crucial importance to the RAS, SR(passive) and EES (passive) services; it is the window for the band around 118.75 GHz. No active services are acceptable in this band. |

| GHz 71 -126 (continued) |  |  |  |
| --- | --- | --- | --- |
| Current Allocations | Proposed Allocations |  | Reason for Change/ Remarks |
| Regions 1, 2, 3 | Regions 1, 2, 3 |  |  |
| 92-94 FIXED FIXED-SATELLITE (Earth-to-space) MOBILE RADIOLOCATION S5.149 S5.556 | 92-94 FIXED FIXED-SATELLITE (Earth-to-space) MOBILE RADIO ASTRONOMY S5.ZZZ RADIOLOCATION MOD S5.149 S5.556 |  | Radio astronomy allocation and new footnote S5.ZZZ, along with RES ZZZ satisfy requirement for radio astronomy spectral line and wide band continuum observations from a few remote locations world-wide. RAS interest up to now recognised via footnote S5.556. FSS (Earth-to-space) allocation no longer needed to balance 102-105 GHz, relocated to 71-76 GHz band. Band added to those listed under S5.149, footnote S5.556 no longer necessary. |
| 94-94.1 RADIOLOCATION EARTH EXPLORATION-SATELLITE (active) SPACE RESEARCH (active) S 5.562 | 94-94 EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION SPACE RESEARCH (active) Radio Astronomy S 5.562 |  | Radio astronomy secondary to active services. No change in sharing between services, except for introduction of secondary RAS allocation in band. |
| 94.1-95 FIXED FIXED-SATELLITE (Earth-to-space) MOBILE RADIOLOCATION S5. 149 S5.556 | 94.1-95 FIXED FIXED-SATELLITE (Earth-to-space) MOBILE RADIO ASTRONOMY S.5.ZZZ RADIOLOCATION MOD S5. 149 S5.556 |  | Radio astronomy allocation and new footnote S5.ZZZ, along with RES ZZZ satisfy requirement for radio astronomy spectral line and wide band continuum observations from a few remote locations world-wide. No change in sharing between services, except for introduction of RAS allocation in band. FSS (Earth-to-space) allocation no longer needed to balance 102-105 GHz, relocated to 71-76 GHz band. Footnote S5.556 is not relevant to this band and should have been suppressed as a consequence of WRC-97 actions. Band added to those listed under S5.149. Footnote S5.556 no longer necessary. |

| GHz 71 -126 (continued) |  |  |
| --- | --- | --- |
| Current Allocations | Proposed Allocations | Reason for Change/ Remarks |
| Regions 1, 2, 3 | Regions 1, 2, 3 |  |
| 95-100 MOBILE S5.553 MOBILE-SATELLITE RADIONAVIGATION RADIONAVIGATION-SATELLITE Radiolocation S5.149 S5.554 S5.555 | 95-100 FIXED MOD S5.553 MOBILE MOD S5.553 MOBILE-SATELLITE RADIO ASTRONOMY S5.ZZZ RADIOLOCATION RADIONAVIGATION RADIONAVIGATION-SATELLITE Radiolocation MOD S5.149 S5.554 S5.555 | Radio astronomy allocation and new footnote S5.ZZZ, along with RES ZZZ satisfy requirement for radio astronomy spectral line and wide band continuum observations from a few remote locations, world-wide. Radiolocation upgraded to primary, consequential to the RAS upgrade. Mobile Satellite Service cannot share with the Radiolocation service. Band added to those listed under S5.149. Band deleted from footnote S5.555, which allocates the 97.88-98.08 subband to the RAS on a primary basis. S5.553 modified to include stations in the fixed service |
| 100 – 102 EARTH EXPLORATION-SATELLITE (passive) FIXED MOBILE SPACE RESEARCH (passive) S5.341 | 100 – 102 EARTH EXPLORATION-SATELLITE (passive) FIXED MOBILE RADIO ASTRONOMY S5.ZZZ SPACE RESEARCH (passive) MOD S5.149 S5.341 | Radio astronomy allocation and new footnote S5.ZZZ, along with RES ZZZ satisfy requirement for radio astronomy spectral line and wide band continuum observations from a few remote locations, world-wide. No change in sharing between services, except for introduction of RAS allocation in band. Used by EES (passive) for limb sounding of atmospheric constituents (NO line at 100.49 GHz). Band added to those listed under S5.149 |
| 102-105 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE S5.341 | 102-105 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE RADIO ASTRONOMY S5.ZZZ MOD S5.149 S5.341 | FSS allocation moved to 74-76 GHz band, to eliminate downlinks in the middle of the atmospheric window. Atmospheric absorption in these two windows is close. Radio astronomy allocation and new footnote S5.ZZZ, along with RES ZZZ satisfy requirement for radio astronomy spectral line and wide band continuum observations from a few remote locations, world-wide. Band added to those listed under S5.149 |

| GHz 71 -126 (continued) |  |  |
| --- | --- | --- |
| Current Allocations | Proposed Allocations | Reason for Change/ Remarks |
| Regions 1, 2, 3 | Regions 1, 2, 3 |  |
| 105-116 EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) S5.340 S5.341 | 105 – 116 109.5 EARTH EXPLORATION-SATELLITE (passive) FIXED MOBILE RADIO ASTRONOMY SPACE RESEARCH (passive) S5.CCC S5.340 MOD S5.149 S5.341 | Passive sensors have no known use for, and do not need the band 105-109.5 GHz. Addition of the fixed and mobile services,relocated from 116 - 122.5 GHz band, where deletion of these services is needed to protect essential passive sensor operations Band no longer passive, consequently S5.340 should be deleted. Band added to those included under S5.149, to reflect need to protect radio astronomy in a band no longer passive. Footnote S5.CCC limits Space Research (passive) allocation to space-based radio astronomy in this band. |
|  | 109.5 - 111.8 EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) S5.340 S5.341 | NOC Ozone line at 110.8 GHz used for microwave limb sounding. The 109.5-111.8 GHz band is of vital importance to radio astronomy for observations of the CO lines at 109.8 and 110.2 GHz, and for continuum observations. |
|  | 111.8 - 114.25 EARTH EXPLORATION-SATELLITE (passive) FIXED MOBILE RADIO ASTRONOMY SPACE RESEARCH (passive) S5.CCC S54.340 S5.341 MOD S5.149 | Passive sensors do not need the band 111.8--114.25 GHz. Addition of the fixed and mobile services, relocated from 116 - 122.5 GHz band, where deletion of these services is needed to protect essential passive sensor operations. Band added to those included under S5.149, to reflect need to protect radio astronomy in a band no longer passive. New footnote S5.CCC limits Space Research (passive) allocation to space-based radio astronomy in this band. |
|  | 114.25-116 EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) S5.340 S5.341 | NOC The band 114.25-116 GHz is of vital importance to radio astronomy for observations of the 115.3 GHz CO line. |

| GHz 71 -126 (continued) |  |  |
| --- | --- | --- |
| Current Allocations | Proposed Allocations | Reason for Change/ Remarks |
| Regions 1, 2, 3 | Regions 1, 2, 3 |  |
| 116 – 119.98 EARTH EXPLORATION-SATELLITE (passive) FIXED INTER-SATELLITE MOBILE S5.558 SPACE RESEARCH (passive) S5.138 S5.341 | 116 -119.98 EARTH EXPLORATION-SATELLITE (passive) FIXED INTER-SATELLITE S5.XXX MOBILE S5.558 SPACE RESEARCH (passive) S5.138 S5.341 | This band is of crucial importance for passive sensing as it is the lower flank of the oxygen absorption band, with peak at 118.75 GHz. The associated reference window is the 86-92 GHz band.Fixed and mobile services moved down to 105 - 109.5 GHz, as sharing with passive sensors would severely restrict their operation in this band. ISS needs to be limited by footnote S5.XXX to links between GSO satellites only, with pfd limits as specified in sharing studies. S5.138 deleted, it is considered to be a misprint in the RR. |
| 119.98 – 120.02 EARTH EXPLORATION-SATELLITE (passive) FIXED INTER-SATELLITE MOBILE S5.558 SPACE RESEARCH (passive) Amateur S5.138 S5.341 | 119.98 – 120.02 EARTH EXPLORATION-SATELLITE (passive) FIXED INTER-SATELLITE S5.XXX MOBILE S5.558 SPACE RESEARCH (passive) Amateur S5.138 S5.341 | This band is of crucial importance for passive sensing, as it is part of the upper flank of the oxygen absorption band, with peak at 118.75 GHz. The associated reference window is the 86-92 GHz band.Fixed and mobile services moved down to 105 - 109.5 GHz, as sharing with passive sensors would severely restrict them in this band. ISS needs to be limited by footnote S5.XXX to links between GSO satellites only, with pfd limits as specified in sharing studies. Secondary allocation to amateur service is also moved to 122.5-123 GHz band, to avoid interference to passive sensors. |

| GHz 71 -126 (continued) |  |  |
| --- | --- | --- |
| Current Allocations | Proposed Allocations | Reason for Change / Remarks |
| Regions 1, 2, 3 | Regions 1, 2, 3 |  |
| 120.02 – 126 EARTH EXPLORATION-SATELLITE (passive) FIXED INTER-SATELLITE MOBILE S5.558 SPACE RESEARCH (passive) S5.138 S5.341 | 120.02 – 122.25 EARTH EXPLORATION-SATELLITE (passive) FIXED INTER-SATELLITE S5.XXX MOBILE S5.558 SPACE RESEARCH (passive) S5.138 S5.341 | This band is of crucial importance for passive sensing, as it is the lower flank of the oxygen absorption band, with peak at 118.75 GHz. The associated reference window is the 86-92 GHz band. Fixed and mobile services moved down to 105 - 109.5 GHz, as sharing with passive sensors would severely restrict them in this band. ISS needs to be limited by footnote S5.XXX to links between GSO satellites only, with pfd limits as specified in sharing studies. |
|  | 122.25 - 123 EARTH EXPLORATION-SATELLITE (passive) FIXED INTER-SATELLITE MOBILE MOD S5.558 SPACE RESEARCH (passive) Amateur S5.138 S5.341 | Band not needed by passive sensors. Secondary amateur service allocation added to compensate loss at 119.98-120.02 GHz |

| GHz 126-200 |  |  |  |
| --- | --- | --- | --- |
| Current Allocations | Proposed Allocations | Reason for Change / Remarks |  |
| Regions 1,2,3 | Regions 1,2,3 |  |  |
| 126 - 134 FIXED INTER-SATELLITE MOBILE S5. 558 RADIOLOCATION S5. 559 | 123 - 130 EARTH EXPLORATION SATELLITE (passive) FIXED FIXED-SATELLITE INTER-SATELLITE MOBILE S5.558 MOBILE-SATELLITE RADIOLOCATION S5. 559 RADIONAVIGATION RADIONAVIGATION-SATELLITE SPACE RESEARCH (passive) Radio Astronomy S5.138 S5.341 MOD S5.554 |  | Satellite downlinks from 141-153 GHz band moved here to avoid interference to the Radio Astronomy Service. Fixed, mobile and radiolocation allocations moved to 141-153 GHz band. Sharing conditions between the ISS and the FSS, MSS, RNS and RNSS services need to be developed, but no imminent use of the band by these services is contemplated. MSS directional indicator left undefined. Footnote S5.554 suitably modified. Band deleted from S5.559 |
|  | 130 - 134 FIXED INTER-SATELLITE MOBILE MOD S5.558 RADIO ASTRONOMY S5.ZZZ RADIOLOCATION S5.559 |  | Radio astronomy allocation and new footnote S5.ZZZ, along with RES ZZZ satisfy requirement for radio astronomy spectral line and wide band continuum observations from a few remote locations world-wide. Sharing conditions between the RAS and the ISS need to be developed. Footnote S5.558 modified to reflect new mobile service band limit. |

| GHz 126-200 (continued) |  |  |  |
| --- | --- | --- | --- |
| Current Allocations | Proposed Allocations | Reason for Change / Remarks |  |
| Regions 1,2,3 | Regions 1,2,3 |  |  |
| 134-142 MOBILE S5.553 MOBILE-SATELLITE RADIONAVIGATION RADIONAVIGATION-SATELLITE Radiolocation S5.149 S5.340 S5.554 S5.555 | 134-136 AMATEUR AMATEUR-SATELLITE MOBILE S5.553 MOBILE-SATELLITE RADIONAVIGATION RADIONAVIGATION-SATELLITE Radio Astronomy Radiolocation S5.149 S5.340 S5.554 S5.555 |  | Amateur and amateur-satellite services moved here from 142-144 GHz band to avoid interference to radio astronomy at higher frequencies. Radio astronomy added as secondary service. |
|  | 136 - 141 MOBILE S5.553 MOBILE-SATELLITE RADIO ASTRONOMY S5.ZZZ RADIOLOCATION RADIONAVIGATION RADIONAVIGATION-SATELLITE Amateur Amateur-Satellite Radiolocation MOD S5.149 S5.340 S5.554 S5.555 |  | Services allocated to 144 –149 GHz band moved here. Radio astronomy allocation and new footnote S5.ZZZ, along with RES ZZZ satisfy requirement for radio astronomy spectral line and wide band continuum observations from a few remote locations world-wide. Band added to those listed under S5.149. Band no longer passive and removed from S5.340 S5.554 is no longer relevant to this band. S5.555 modified to reflect primary status of the RAS in the full 136-141 GHz band. |

| GHz 126-200 (continued) |  |  |  |
| --- | --- | --- | --- |
| Current Allocations | Proposed Allocations | Reason for Change / Remarks |  |
| Regions 1,2,3 | Regions 1,2,3 |  |  |
| 142 - 144 AMATEUR AMATEUR-SATELLITE | 141 - 148.5 AMATEUR AMATEUR-SATELLITE FIXED MOBILE S5.553 MOBILE-SATELLITE RADIO ASTRONOMY S5.ZZZ RADIOLOCATION RADIONAVIGATION RADIONAVIGATION-SATELLITE Amateur Amateur-Satellite Radiolocation MOD S5.149 S5.340 S5.554 S5.555 |  | Allocations transferred to this band from 126-134 GHz band. Bandwidth reduced to 7.5 GHz to accommodate EES (passive) and SR (passive) requirements in 148.5-151.5 GHz band. Radio astronomy allocation and new footnote S5.ZZZ, along with RES ZZZ satisfy requirement for radio astronomy spectral line and wide band continuum observations from a few remote locations world-wide. Band no longer passive; deleted from those listed under S5.340. Band added to those listed under S5.149 S5.554 no longer applies S5.555 no longer refers to any portion of this band. |
| 144 - 149 RADIOLOCATION Amateur Amateur-Satellite S 5.149 S5.555 |  |  |  |
| 149-150 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE |  |  |  |

| GHz 126-200 (continued) |  |  |
| --- | --- | --- |
| Current Allocations | Proposed Allocations | Reason for Change / Remarks |
| Regions 1,2,3 | Regions 1,2,3 |  |
| 150-151 EARTH EXPLORATION-SATELLITE (passive) FIXED FIXED-SATELLITE (space-to-Earth) MOBILE SPACE RESEARCH (passive) S5.149 S5.385 | 148.5 - 151.5 EARTH EXPLORATION-SATELLITE (passive) FIXED FIXED-SATELLITE (space-to-Earth) MOBILE RADIO ASTRONOMY RADIOLOCATION SPACE RESEARCH (passive) Amateur Amateur-Satellite S 5.149 S 5.385 S5.555 MOD S5.340 | Current passive allocation of 150-151 GHz has insufficient bandwidth and is not adequately protected from interference. Band purely passive, added to those listed under S5.340, no longer listed under S5.149. S5.385 no longer needed S5.555 no longer refers to any portion of this band. |
| 151-156 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE | 151.5 - 155.5 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE RADIO ASTRONOMY S5.ZZZ RADIOLOCATION MOD S5.149 | FSS downlink allocation incompatible with primary radio astronomy requirement. Radio astronomy allocation and new footnote S5.ZZZ, along with RES ZZZ satisfy requirement for radio astronomy spectral line and wide band continuum observations from a few remote locations world-wide. Band added to those listed under S5.149 Radiolocation allocation compensates for removal from 126-134 GHz band. |

| GHz 126-200 (continued) |  |  |  |
| --- | --- | --- | --- |
| Current Allocations | Proposed Allocations | Reason for Change / Remarks |  |
| Regions 1,2,3 | Regions 1,2,3 |  |  |
| 156-158 EARTH EXPLORATION-SATELLITE (passive) FIXED FIXED-SATELLITE (space-to-Earth) MOBILE | 155.5-158.5 EARTH EXPLORATION-SATELLITE (passive) S5.AAA FIXED S5.BBB FIXED-SATELLITE (space-to-Earth) MOBILE S5.BBB RADIO ASTRONOMY S5.ZZZ SPACE RESEARCH (passive) S5.CCC MOD S55.149 | FSS downlink allocation incompatible with primary radio astronomy requirement. Radio astronomy allocation and new footnote S5.ZZZ, along with RES ZZZ satisfy requirement for radio astronomy spectral line and wide band continuum observations from a few remote locations world-wide. Band added to those listed under S5.149 EES operations in the band 155.5-158.5 GHz need to be protected until 1/1/2018. After this date the fixed and mobile services need to co-ordinate with radio astronomy sites only. Space Research (passive) allocation limited to space-based radio astronomy in this band |  |
| 158 -164 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE | 158.5 - 164 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE MOBILE-SATELLITE (space-to Earth) | Mobile-satellite allocation added to compensate for loss of 134-142 GHz band |  |

| GHz 126-200 (continued) |  |  |  |
| --- | --- | --- | --- |
| Current Allocations |  | Proposed Allocations | Reason for Change / Remarks |
| Regions 1,2,3 |  | Regions 1,2,3 |  |
| 164 - 168 EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) |  | 164 - 168 167 EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) MOD S5.340 | NOC Passive sensors require only a 3 GHz band from 164-167 GHz. This band, along with the band 148.5-151.5 GHz will become the harmonised reference window for passive sensor observations of the 183.31 GHz water vapor line. The band is also used for microwave limb sounding of the 164.38 GHz Cl O line. Band added to those listed under S5.340. Band 164-168 GHz previously not listed under S5.340 by mistake. |
| 168 - 170 FIXED MOBILE | 167 - 174 EARTH EXPLORATION-SATELLITE (passive) FIXED FIXED SATELLITE (Space-to-Earth) INTER-SATELLITE MOBILE MOD S5.558 RADIO ASTRONOMY SPACE RESEARCH (passive) |  | Passive services do not need the 167-168 GHz band Fixed, mobile, inter-satellite and fixed-satellite downlinks added. S5.558 added next to mobile allocation. S5.558 modified to include 167-174 GHz band . |
| 170-174.5 FIXED INTER-SATELLITE MOBILE S5.558 S5.149 S5.385 |  |  |  |

| GHz 126-200 (continued) |  |  |  |
| --- | --- | --- | --- |
| Current Allocations |  | Proposed Allocations | Reason for Change / Remarks |
| Regions 1,2,3 |  | Regions 1,2,3 |  |
| 174.5-176.5 EARTH EXPLORATION-SATELLITE (passive) FIXED INTER-SATELLITE MOBILE S5.558 SPACE RESEARCH (passive) S5.149 S5.385 | 174 - 174.8 EARTH EXPLORATION-SATELLITE (passive) FIXED INTER-SATELLITE MOBILE MOD S5.558 SPACE RESEARCH (passive) MOD S5.149 MOD S5.385 |  | Band not needed for passive uses. FSS (space-to-Earth) added, to compensate for loss of 151-158 GHz band in middle of atmospheric window. Band deleted from those listed in S5.149 and S5.385. |
|  | 174.8-176.5 EARTH EXPLORATION-SATELLITE (passive) FIXED INTER-SATELLITE S5.YYY MOBILE S5.558 SPACE RESEARCH (passive) S5.149 S5.385 |  | The band 174.8-191.8 GHz is of crucial importance for passive sensing of the water vapour absorption line whose peak is at 183.31 GHz.. Sharing with fixed and mobile services is not possible, so they need to be reallocated. The ISS service needs to be limited to links between GSO satellites and to a pfd limit as specified in sharing studies. Footnote S5.YYY added to reflect this requirement Band deleted from those listed under S5.149, S5.385 (secondary radio astronomy allocation) . Footnotes appropriately modified. |

| GHz 126-200 (continued) |  |  |
| --- | --- | --- |
| Current Allocations | Proposed Allocations | Reason for Change / Remarks |
| Regions 1,2,3 | Regions 1,2,3 |  |
| 176.5-182 FIXED INTER-SATELLITE MOBILE S5.558 S5.149 S5.385 | 176.5-182 EARTH EXPLORATION-SATELLITE (passive) FIXED INTER-SATELLITE S5.YYY MOBILE S5.558 SPACE RESEARCH (passive) MOD S5.149 S5.385 | This band is of crucial importance for passive sensing of the water vapor absorption line whose peak is at 183.31 GHz. Sharing with fixed and mobile services is not possible, so they need to be reallocated. The ISS service needs to be limited to links between GSO satellites and to a pfd limit as specified in sharing studies. |
| 182-185 EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) S5.340 S5.563 | 182-185 EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) MOD S5.340 S5.563 MOD S5.149 | No terrestrial radio astronomy use of the band is possible due to high atmospheric absorption. Radio astronomy observations of the water vapor line from space are conducted in this band under the Space Research (passive) allocation. The band 174.8-191.8 GHz is required for sensing of the water vapour absorption line whose peak is at 183.31 GHz. |

| GHz 126-200 (continued) |  |  |  |
| --- | --- | --- | --- |
| Current Allocations |  | Proposed Allocations | Reason for Change / Remarks |
| Regions 1,2,3 |  | Regions 1,2,3 |  |
| 185-190 FIXED INTER-SATELLITE MOBILE S5.558 S5.149 S5.385 | 185-190 EARTH EXPLORATION SATELLITE (passive) FIXED INTER-SATELLITE S5.YYY MOBILE S5.558 SPACE RESEARCH (passive) S5.149 S5.385 |  | This band is of crucial importance for passive sensing of the water vapor absorption line whose peak is at 183.31 GHz. Sharing with fixed and mobile services is not possible, so they need to be reallocated. The ISS service needs to be limited to links between GSO satellites and to a pfd limit as specified in sharing studies. Band deleted from those listed in S5.149. |
| 190 - 200 MOBILE S5.553 MOBILE-SATELLITE RADIONAVIGATION RADIONAVIGATION-SATELLITE S5.341 S5.554 | 190 - 191.8 EARTH EXPLORATION-SATELLITE (passive) MOBILE S5.553 MOBILE-SATELLITE (space-to-Earth) RADIONAVIGATION RADIONAVIGATION-SATELLITE SPACE RESEARCH (passive) S5.341 S5.554 MOD S5.340 |  | This band is of crucial importance for passive sensing of the water vapor absorption line whose peak is at 183.31 GHz. Sharing with the mobile, mobile-satellite, RN and RNS services is not possible, and they need to be reallocated. S5.554 deleted from this band, to reflect deletion of active services, and modified to reflect change. S5.341 does not apply to this band. S5.340 modified to include this band. |
|  | 191.8-200 FIXED MOD S5.553 INTER-SATELLITE MOBILE MOD S5.553 MOBILE-SATELLITE RADIONAVIGATION RADIONAVIGATION-SATELLITE S5.341 MOD S5.554 |  | ISS and FS allocations added. S5.553 and S5.554 modified, to reflect deletion of terrestrial services from 190.0 -191.8 GHz band. S5.553 modified to include stations in the fixed service. |

| GHz 200-275 |  |  |
| --- | --- | --- |
| Current Allocations | Proposed Allocations | Reason for Change / Remarks |
| Regions 1,2,3 | Regions 1,2,3 |  |
| 200-202 EARTH EXPLORATION-SATELLITE (passive) FIXED MOBILE SPACE RESEARCH (passive) S5.341 | 200-202 EARTH EXPLORATION-SATELLITE (passive) FIXED MOBILE RADIO ASTRONOMY SPACE RESEARCH (passive) S5.341 MOD S5.340 | Band needed by EESS for atmospheric chemistry (limb sounding ) and atmospheric remote sensing of nitrous oxide at 201 GHz. Fixed and mobile services needs to be relocated from this band. Radio astronomy allocation satisfies requirement for radio astronomy spectral line and wide band continuum observations from a few remote locations world-wide. Band added to those listed under S5.340 |
| 202-217 FIXED FIXED-SATELLITE (Earth-to-space) MOBILE S5.341 | 202 – 217 209 EARTH EXPLORATION-SATELLITE (passive) FIXED FIXED-SATELLITE (Earth-to-space) MOBILE RADIO ASTRONOMY SPACE RESEARCH (passive) S5.341 MOD S5.340 | Band needed by EES service for atmospheric chemistry (limb sounding ) and atmospheric remote sensing of water vapor at 203.4 GHz and ozone at 208.5 GHz.. FSS( Earth-to-space) service relocated from this band to 217-226 GHz band. Fixed and mobile services may need to be relocated from this band after results of sharing studies with EESS are known. Radio astronomy allocation added. Band added to those listed under S5.340 |
|  | 209 – 217 FIXED FIXED-SATELLITE (Earth-to-space) MOBILE RADIO ASTRONOMY S5.ZZZ S5.341 MOD S5.149 | Radio astronomy allocation and new footnote S5.ZZZ, along with RES ZZZ satisfy requirement for radio astronomy spectral line and wide band continuum observations from a few remote locations world-wide. Footnote S5.149 added Band added to those listed under S5.149 |

| GHz 200-275 (continued) |  |  |  |
| --- | --- | --- | --- |
| Current Allocations |  | Proposed Allocations | Reason for Change / Remarks |
| Regions 1,2,3 |  | Regions 1,2,3 |  |
| 217-231 EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) S5.340 S5.541 | 217 – 231 226 EARTH EXPLORATION-SATELLITE (passive) FIXED FIXED-SATELLITE (Earth-to-space) MOBILE RADIO ASTRONOMY SPACE RESEARCH (passive) S5.CCC S5.341 MOD S5.149 S5.340 |  | Passive sensor allocation can be deleted from this band. Band no longer passive, added to those listed under S5.149, removed from list of S5.340 Band no longer passive, S5.340 deleted. MOD S5.149 added. |
|  | 226 - 231 231.5 EARTH EXPLORATION-SATELLITE (passive) FIXED FIXED-SATELLITE (space-to-Earth) MOBILE RADIO ASTRONOMY SPACE RESEARCH (passive) Radiolocation MOD S5.340 S5.341 |  | Passive sensors require exclusive use of only the 226-231.5 GHz portion of the 217 -231 GHz band for limb sounding of atmospheric constituents. In addition, the band provides a 4 GHz reference window for higher frequency water vapor measurements. This band is of vital importance to the radio astronomy service for observations of the 230.5 GHz CO line. S5.340 modified to take into account that 217-226 GHz band is no longer passive. The FS, FSS (s-E), and MS services have been deleted from the 231-231.5 Ghz portion of the band. |
| 231 - 235 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE Radiolocation |  | 231 231.5 - 235 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE Radiolocation | See previous band adjustment. |

| GHz 200-275 (continued) |  |  |
| --- | --- | --- |
| Current Allocations | Proposed Allocations | Reason for Change / Remarks |
| Regions 1,2,3 | Regions 1,2,3 |  |
| 235-238 EARTH EXPLORATION-SATELLITE (passive) FIXED FIXED-SATELLITE (space-to-Earth) MOBILE SPACE RESEARCH (passive) | 235-238 EARTH EXPLORATION-SATELLITE (passive) [FIXED] FIXED-SATELLITE (space-to-Earth) [MOBILE] RADIO ASTRONOMY [S5.ZZZ] SPACE RESEARCH (passive) [S5.340] | Passive sensors are limited to microwave limb sounding in the band 235-238 GHz. FSS (space-to-Earth) reallocated elsewhere. Fixed and mobile services may need to be relocated from this band after results of sharing studies are known. S5.340 to be added if all active services need to be relocated. Radio astronomy allocation satisfies requirement for radio astronomy spectral line and wide band continuum observations. Footnote S5.ZZZ to be added if active services do not require to be reallocated. |
| 238-241 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE Radiolocation | 238-241 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE RADIOLOCATION RADIONAVIGATION RADIONAVIGATION-SATELLITE Radiolocation | Additional allocations to the radiolocation, radionavigation and radionavigation-satellite services, to compensate for losses in the 150-160 GHz frequency range. |

| GHz 200-275 (continued) |  |  |
| --- | --- | --- |
| Current Allocations | Proposed Allocations | Reason for Change / Remarks |
| Regions 1,2,3 | Regions 1,2,3 |  |
| 241-248 RADIOLOCATION Amateur Amateur-Satellite S5.138 | 241-248 RADIO ASTRONOMY S5.ZZZ RADIOLOCATION Amateur Amateur-Satellite S5.138 MOD S5.149 | Radio astronomy allocation and new footnote S5.ZZZ, along with RES ZZZ satisfy requirement for radio astronomy spectral line and wide band continuum observations from a few remote locations world-wide. Band added to those listed under S5.149 No change in sharing between services, except for introduction of RAS allocation in band. |
| 248-250 AMATEUR AMATEUR-SATELLITE | 248 -250 AMATEUR AMATEUR-SATELLITE Radio Astronomy | RAS added on a secondary basis. No change in sharing between active services. |
| 250-252 EARTH EXPLORATION-SATELLITE (passive) SPACE RESEARCH (passive) S5.149 S5.555 | 250 - 252 EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) S5.149 S5.555 MOD S5.340 | Limb sounding of nitrous oxide near 251 GHz. Radio astronomy added to the other passive services. Addition of another passive service does not alter sharing scenario. S5.149 and S5.555 deleted. S5.340 modified, to reflect passive nature of band. |

| GHz 200-275 (continued) |  |  |
| --- | --- | --- |
| Current Allocations | Proposed Allocations | Reason for Change / Remarks |
| Regions 1,2,3 | Regions 1,2,3 |  |
| 252-265 MOBILE S5.553 MOBILE-SATELLITE RADIONAVIGATION RADIONAVIGATION-SATELLITE S5.149 S5.385 S5.554 S5.555 S5.564 | 252-265 FIXED MOD S5.553 MOBILE MOD S5.553 MOBILE-SATELLITE (Earth-to-space) RADIO ASTRONOMY S5.ZZZ RADIONAVIGATION RADIONAVIGATION-SATELLITE MOD S5.149 S5.385 MOD S5.554 S5.555 S5.564 | Directional indicator added to MSS allocation, paired with 190-200 GHz band. Atmospheric absorption in this band is relatively constant and higher than in the paired downlink band. Radio astronomy allocation and new footnote S5.ZZZ, along with RES ZZZ satisfy requirement for radio astronomy spectral line (secondary allocation to radio astronomy at 257.5 - 258 GHz deleted) and wide band continuum observations from a few remote locations world-wide. Band added to those listed under MOD S5.149. Band deleted from S5.385. Footnote S5.564 no longer needed. |
| 265-275 FIXED FIXED-SATELLITE (Earth-to-space) MOBILE RADIO ASTRONOMY S5.149 | 265-275 FIXED FIXED-SATELLITE (Earth-to-space) MOBILE RADIO ASTRONOMY S 5.ZZZ MOD S5.149 | Footnote S5.ZZZ added to Radio Astronomy allocation in this band. |
| 275-400 (Not allocated) S5.565 | 275 - 400 1 000 (Not allocated) MOD S5.565 | Change upper limit for applicability of footnote MOD S5.565, to account for various passive service needs above 275 GHz. Many lines and windows for radio astronomy and EES applications above 275 GHz. |

Footnotes Applicable in the 71 - 1000 GHz Frequency Range

| S5.138 | The following bands: 6 765 - 6 795 kHz (center frequency 6 780 kHz), 433.05 - 434.79 MHz (center frequency 433.92 MHz) in Region 1 except in the countries mentioned in No S5.280, 61 - 61.5 GHz (center frequency 61.25 GHz), 122 - 123 GHz (center frequency 122.5 GHz), and 244 - 246 GHz (center frequency 245 GHz) are designated for industrial, scientific and medical (ISM) applications. The use of these frequency bands for ISM applications shall be subject to special authorization by the administration concerned, in agreement with other administrations whose radiocommunication services might be affected. In applying this provision, administrations shall have due regard to the latest relevant ITU-R Recommendations. |  |  |
| --- | --- | --- | --- |
| MOD S5.149 | In making assignments to stations of other services to which the bands: |  |  |
|  | 13 360-13 410 kHz, 25 550-25 670 kHz, 37.5-38.25 MHz, 73-74.6 MHz in Regions 1 and 3, 150.05-153 MHz in Region 1, 322-328.6 MHz*, 406.1-410 MHz, 608-614 MHz in Regions 1 and 3, 1 330-1 400 MHz*, 1 610.6-1 613.8 MHz*, 1 660-1 670 MHz, 1 718.8-1 722.2 MHz*, 2 655-2 690 MHz, 3 260-3 267 MHz*, 3 332-3 339 MHz*, 3 345.8-3 352.5 MHz*, 4 825-4 835 MHz*, 4 950-4 990 MHz, 4 990-5 000 MHz, 6 650-6 675.2 MHz*, 10.6-10.68 GHz, 14.47-14.5 GHz*, | 22.01-22.21 GHz*, 22.21-22.5 GHz, 22.81-22.86 GHz*, 23.07-23.12 GHz*, 31.2-31.3 GHz, 31.5-31.8 GHz in Regions 1 and 3, 36.43-36.5 GHz*, 42.5-43.5 GHz, 42.77-42.87 GHz*, 43.07-43.17 GHz*, 43.37-43.47 GHz*, 48.94-49.04 GHz*, 72.77-72.91 GHz*, 76.5-81.5 GHz, 81.5-84.5 GHz, 84.5-86 GHz, 93.07-93.27 GHz*, 92-94 GHz, 94.1-95 GHz, 95-100 GHz, 100-102 GHz, 97.88-98.08 GHz*, 102-105 GHz, 105-109.5 GHz, | 111.8-114.25 GHz, 140.69-140.98 GHz*, 141-148.5 GHz, 148.5-151.5 GHz, 144.68-144.98 GHz*, 145.45-145.75 GHz*, 146.82-147.12 GHz*, 150-151 GHz*, 174.42-175.02 GHz*, 177-177.4 GHz*, 178.2-178.6 GHz*, 181-181.46 GHz*, 186.2-186.6 GHz*, 209-226 GHz, 250-251 GHz*, 257.5-258 GHz*, 261-265 GHz, 262.24-262.76 GHz*, 252-265 GHz, 265-275 GHz, 265.64-266.16 GHz*, 267.34-267.86 GHz*, 271.74-272.26 GHz* |
|  | are allocated (* indicates radio astronomy use for spectral line observations), administrations are urged to take all practicable steps to protect the radio astronomy service from harmful interference. Emissions from spaceborne or airborne stations can be particularly serious sources of interference to the radio astronomy service (see Nos. S4.5 and S4.6 and Article S29). |  |  |

| MOD S5.340 | All emissions are prohibited in the following bands: 1 400-1 427 MHz, 2 690-2 700 MHz except those provided for by Nos. S5.421 and S5.422, 10.68-10.7 GHz except those provided for by No. S5.483, 15.35-15.4 GHz except those provided for by No. S5.511, 23.6-24 GHz, 31.3-31.5 GHz, 31.5-31.8 GHz in Region 2, 48.94-49.04 GHz from airborne stations, 50.2-50.4 GHz1 except those provided for by No. S5.555A, 52.6-54.25 GHz, 86-92 GHz, 105-116 GHz, 109.5-111.8 GHz, 114.25-116 GHz, 140.69-140.98 GHz from airborne stations and from space stations in the space-to-Earth direction, 148.5-151.5 GHz, 164-167 GHz, 182-185 GHz except those provided for by No. S5.563, 190-191.8 200-202 GHz, [202-209 GHz,] 217-228 GHz 226-231.5 GHz, [235-238] 250-252 GHz |
| --- | --- |
| S5.341 | In the bands 1 400 - 1 727 MHz, 101 - 120 GHz and 197 - 220 GHz, passive research is being conducted by some countries in a program for the search for intentional emissions of extraterrestrial origin. |
| MOD S5.385 | Additional allocation: the bands 1 718.8 - 1 722.2 MHz, 150 - 151 GHz, 174.42 - 175.02 GHz, 177 - 177.4 GHz, 178.2 - 178.6 GHz, 181 - 181.46 GHz, and 186.2 - 186.6 GHz and 257.5 - 258 GHz are also allocated to the radio astronomy service on a secondary basis for spectral line observations. |
| MOD S5.553 | In the bands 43.5 - 47 GHz, 66 - 71 GHz, 95 - 100 GHz, 134 - 142 GHz, 190191.8 - 200 GHz and 252 - 265 GHz, stations in the fixed and land mobile services may be operated subject to not causing harmful interference to the space radiocommunication services to which these bands are allocated (see No. S5.43). |
| MOD S5.554 | In the bands 43.5 - 47 GHz, 66 - 71 GHz, 95 -100 GHz, 134 - 142 126 – 134 GHz, 190191.8 - 200 GHz and 252 - 265 GHz, satellite links connecting land stations at specified fixed points are also authorized when used in conjunction with the mobile-satellite service or the radionavigation-satellite service. |
| MOD S5.555 | Additional allocation: the bands 48.94 - 49.04 GHz, 97.88 - 98.08 GHz, 140.69 - 140.98 GHz, 144.68 - 144.98 GHz, 145.45 - 145.75 GHz, 146.82 - 147.12 GHz, 250 - 251 GHz and 262.24 - 262.76 GHz are is also allocated to the radio astronomy service on a primary basis. |
| MOD S5.556 | In the bands 51.4 - 54.25 GHz, 58.2 - 59 GHz, and 64 - 65 GHz, 72.77 - 72.91 GHz and 93.07 - 93.27 GHz, radio astronomy observations may be carried out under national arrangements. |
| MOD S5.558 | In the bands 55.78-58.2 GHz, 59-64 GHz, 66-71 GHz, 123-134 122.5-126 GHz, 170-182 GHz and 185-190 GHz, stations in the aeronautical mobile service may be operated subject to not causing harmful interference to the inter-satellite service (see No. S5.43). |
| MOD S5.559 | In the bands 59 -64 GHz and 126 - 134 GHz, airborne radars in the radiolocation service may be operated subject to not causing harmful interference to the inter-satellite service (see No. S5.43) |
| S5.560 | In the band 78 - 79 GHz radars located on space stations may be operated on a primary basis in the earth exploration-satellite service and in the space research service. |
| MOD S5.561 | In the band 84 - 86 74-76 GHz, stations in the fixed, and mobile and broadcasting services shall not cause harmful interference to broadcasting-satellite stations operating in accordance with the decisions of the appropriate frequency assignment planning conference for the broadcasting-satellite service. |
| S5.562 | The use of the band 94-94.1 GHz by the earth exploration-satellite (active) and space research (active) services is limited to spaceborne cloud radars. |
| SUP S5.564 | Additional allocation: in Germany, Argentina, Spain, Finland, France, India, Italy, the Netherlands and Sweden, the band 261 - 265 GHz is also allocated to the radio astronomy service on a primary basis. |
| MOD S5.565 | The frequency band 275 - 400 1000 GHz may be used by administrations for experimentation with, and development of, various active and passive services. In this band a need has been identified for the following spectral line measurements for passive services: — radio astronomy service: 278 - 280 GHz and 343 - 348 GHz; 275 -323 GHz, 327-371 GHz, 388 - 434 GHz, 426 - 442 GHz, 453 -510 GHz, 623 - 711 GHz and 795 - 909 GHz — space research service (passive) and earth exploration-satellite service (passive): 275 - 277 GHz, 300299 - 3026 GHz, 32416 - 32634 GHz, 34539 - 3478 GHz, 363 - 365 GHz, and 3791.5 - 38178.5 GHz, 409 - 411 GHz, 419 - 430 GHz, 443 - 444 GHz, 485 - 487 GHz, 498 - 505 GHz, 523 - 527 GHz, 538 - 584 GHz, 624 - 629 GHz, 640 - 653 GHz and 951 - 956 GHz. |
| ADD S5.XXX | Use of the bands 116-123 GHz by the inter-satellite service is limited to satellites in the geostationary-satellite orbit. The single-entry power flux-density, at all altitudes from 0 km to 1 000 km above the Earth's surface and in the vicinity of all geostationary orbital positions occupied by passive sensors, produced by a station in the inter-satellite service, for all conditions and for all methods of modulation, shall not exceed -148 dBW/m2/MHz for all angles of arrival. |
| ADD S5.AAA | In the band 155.5-158.5 GHz, the allocation to the Earth exploration-satellite (passive) and space research (passive) services shall terminate on 1 January 2018. |
| ADD S5.BBB | The date of entry for the allocation to the fixed and mobile services in the band 155.5 – 158.5 GHz shall be 1 January 2018. |
| ADD S5.CCC | Use of this allocation is limited to space-based radio astronomy only. |
| ADD S5.DDD | The 81- 81.5 GHz band is also allocated to the amateur and amateur-satellite services on a secondary basis. |
| ADD S5.EEE | The band 75.5-76 GHz is also allocated to the amateur and amateur-satellite services on a primary basis until the year 200[6]. |
| ADD S5.YYY | Use of the bands 174.5-182 GHz by the inter-satellite service is limited to satellites in the geostationary-satellite orbit. The single-entry power flux-density, at all altitudes from 0 km to 1 000 km above the Earth's surface and in the vicinity of all geostationary orbital positions occupied by passive sensors, produced by a station in the inter-satellite service, for all conditions and for all methods of modulation, shall not exceed -144 dBW/m2/MHz for all angles of arrival. |
| ADD S5.ZZZ | Use of this band by the radio astronomy service shall be in accordance with the terms of Resolution ZZZ |

RESOLUTION ZZZ

USE OF THE BANDS [      ] BY THE RADIO ASTRONOMY SERVICE

The World Radiocommunication Conference (Istanbul, 2000),

considering

a) 	that a large number of spectral lines of astrophysical interest above 71 GHz provide unique information about cosmic processes, such as the chemistry of the interstellar medium and the formation of stars and planets, and that this information cannot be obtained from any other source;

b) 	that Doppler shifted lines, which are also of great interest for astronomical studies, are found far removed from the rest frequency of some spectral lines and that highly Doppler shifted lines may offer the only means to obtain information about the very early Universe and the formation of galaxies;

c)	that mm-wave radio astronomy receivers are designed to cover substantial portions of the atmospheric windows above 70 GHz to take advantage of the information contained in spectral lines, as well as in continuum radiation;

d)	that several Administrations operate mm-wave radio astronomy observatories and that some are building or are planning to build a limited number of large new facilities to exploit the most advanced technologies; and that these facilities are intended to serve the needs of the worldwide scientific community;

f)	that mm-wave observatories must be located on high mountain tops or plateaus to take advantage of the driest possible atmospheric conditions necessary  to obtain high quality observations; and require substantial investments on behalf of the scientific communities concerned, and that therefore their number will remain low,

noting

that sharing between the radio astronomy service and other terrestrial services operating in bands above 71 GHz is facilitated by the natural attenuation provided by atmospheric gases, and that it can be further facilitated by adequate geographic separation,

urges:

Administrations to establish coordination zones around mm-wave radio astronomy sites operating in bands above 71 GHz.  Coordination zone radii should be determined following the procedure outlined in Rec. ITU-R  RA.1031-1, separately for ground based transmitters, airborne transmitters and transmitters that may be located on High Altitude Platforms (HAPS).

resolves:

1.	that in the frequency bands referred to in this Resolution, co-primary status of the radio astronomy service shall be recognized within coordination zones established by Administrations. No coordination requirements should be imposed upon terrestrial services outside established coordination zones.

2.	that in the bands referred to in this Resolution, co-primary services operating stations within a coordination zone should coordinate their operations with affected radio astronomy stations within five years of the date of notification of the radio astronomy site to the Radiocommunication Bureau

Annex 1 lists the radio astronomy sites that operate, or plan to operate in the bands referred to in this Resolution as of [June 8, 2000]. Observatories that operate only up to 92 GHz are identified with *** under the SITE column.

[  Annex 1  ]*

List of Radio Astronomical Observatories Operating

in Bands Above 71 GHz

REGION 1

| COUNTRY | ### SITE | ### LONG o ' " | LAT o ' " | ALT (m) | DIAM (m) | REMARKS |
| --- | --- | --- | --- | --- | --- | --- |
| Finland | Metsahovi | 24 23 17 | 60 13 04 | 61 | 13.7 |  |
| France | Bordeaux Plateau de Bure | -00 31 37 05 54 26 | 44 50 10 44 38 01 | 73 2552 | 2.5 15 |  |
| Germany | Effelsberg | 06 53 00 | 50 31 32 | 369 | 100 |  |
| Italy | Medicina*** Noto*** | 11 38 43 15 03 00 | 44 31 14 36 31 48 | 44 | 32 | EVLBI EVLBI |
| Russia | Zelenchukskaya | 41 26 30 | 43 39 12 | 2100 |  |  |
| Spain | Pico Veleta Robledo Yepes | -03 23 34 -04 14 57 -03 06 00 | 37 03 58 40 25 38 40 31 30 | 2870 761 931 | 30 |  |
| Turkey | Gebse-Kocaeli | 29 26 52 | 40 47 06 | 200 |  |  |

* ALL OF ANNEX 1 SHOULD BE CONSIDERED IN [   ]

REGION 2

| COUNTRY | SITE | LONG. o ' " | LAT. o ' " | ALT (m) | DIAM (m) | REMARKS |
| --- | --- | --- | --- | --- | --- | --- |
| Chile | San Pedro de Atacama La Silla Las Campanas Pampa La Bola | 67 44 00 70 44 04 70 41 10 67 42 00 | -23 02 -29 15 34 -29 01 43 -22 58 00 | 5000 2300 2440 4800 | 15 4 | MMA (planned) SEST LMSA (planned) |
| Mexico | Sierra Negra | 97 18 00 | 18 59 00 | 4500 | 50 | Large Millimeter Telescope (LMT-under construction) |
| USA | Green Bank,WVA*** Socorro, NM *** ......... St. Croix, VI *** Hancock, NH *** North Liberty, IO*** Ft. Davis, TX *** Los Alamos, NM *** Pie Town, NM *** Kitt Peak, AZ *** Owens Valley, CA*** Brewster, WA *** Mauna Kea, HI *** ................. Kitt Peak, AZ Amherst, MA Owens Valley, CA Hat Creek, CA Westford, MA Mauna Kea, HI Mauna Kea, HI | 79 50 24 107 37 06 ........... 64 35 01 71 59 12 91 34 27 103 56 41 106 14 44 108 07 09 111 36 45 118 16 37 119 41 00 155 27 19 ................ 111 36 50 72 20 40 118 17 36 121 28 24 71 29 19 155 28 20 | 38 25 59 34 04 44 .............. 17 45 24 42 56 01 41 46 17 30 38 06 35 46 31 34 18 04 31 57 23 37 13 54 48 07 52 19 48 05 ............... 31 57 10 42 23 33 37 13 54 40 49 04 42 37 23 19 49 33 | 946 2155 ........... 46 340 272 1646 1997 2402 1946 1237 286 3751 ........... 1930 314 1236 1042 122 4000 | 100 25 .......... 25 25 25 25 25 25 25 25 25 25 ......... 12 13.7 10.4 6.1 36 10.4 | NRAO-GBT NRAO-VLA .................... NRAO VLBA NRAO VLBA NRAO VLBA NRAO VLBA NRAO VLBA NRAO VLBA NRAO VLBA NRAO VLBA NRAO VLBA NRAO VLBA ....................... NRAO 12 m FCRAO (Five Colleges Obs.) Caltech BIMA Haystack Obs. J.C. Maxwell Tel. CSO |

REGION  3

| COUNTRY | SITE | LONG. o ' " | LAT. o ' " | ALT. m | DIAM m | REMARKS |
| --- | --- | --- | --- | --- | --- | --- |
| AUSTRALIA | Parkes Mopra Narrabri, NSW | 148 15 44 149 05 58 149 32 56 | -33 00 00 -31 16 04 -30 59 52 | 60 | 64 | Austr. Tel. Compact Array |
| CHINA | Delingha | 97 43 75 | 37 22 43 | 3200 | 13.7 |  |
| JAPAN | Nobeyama Kashima Mizusawa Nagoya Mt. Fuji Kagoshima | 138 28 32 140 39 46 141 08 09 136 58 24 138 45 06 130 26 32 | 35 56 29 35 57 15 39 08 00 35 08 55 35 21 30 31 44 52 | 1350 50 87 70 3776 520 | 45 34 10 4 1.2 20 | Comm. Res. Lab. Only >300GHz VERA (planned) |
| KOREA | Taejon | 127 22 18 | 36 23 54 | 120 | 13.7 |  |

# OTHER

| COUNTRY | SITE | LONG. o ' " | LAT. o ' " | ALT. m | DIAM m | REMARKS |
| --- | --- | --- | --- | --- | --- | --- |
|  | ANTARCTICA |  | -90 00 00 |  |  |  |

# ANNEX 2

# DRAFT COMMON CITEL Proposal

# for Agenda Item 1.17

## to consider possible worldwide allocation for the earth exploration-satellite (passive) and space research (passive) service in the band 18.6 - 18.8 GHz, taking into account the results of the ITU-R studies

Background Information: At present, the allocations for the Earth exploration-satellite (passive) and the space research (passive) services in the band 18.6 - 18.8 GHz are on a primary basis in Region 2, but on a secondary basis in Regions 1 and 3.  This allocation must be upgraded to primary status if the long-term ability to obtain environmental data with passive spaceborne sensors is to be preserved.  Compatibility between the passive sensors and the fixed and fixed-satellite services requires adoption of constraints on the parameters of the fixed and fixed-satellite systems that use the band.  A pfd limit of -95 dBW/m2 in a reference bandwidth of 200 MHz on geostationary systems in the fixed-satellite service will enable passive sensors to perform their mission if measurements are restricted to portions of the sensor orbit where the sensor is moving away from the equator while taking sensor data over land masses.  Similarly, limiting the power delivered to any antenna of a station in the fixed service measured across the band 18.6 - 18.8 GHz to not exceed 0 dBW in 200 MHz along with an antenna pattern complying with Recommendation ITU-R F.699-4 will enable sharing with the fixed service.

Proposal:

IAP/1.17/01

MOD

| GHz 18.6 – 18.8 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Allocation to Services |  |  |  |  |
| Region 1 | Region 2 | Region 3 |  |
| 18.6 – 18.8 EARTH EXPLORATION- SATELLITE (passive) FIXED FIXED-SATELLITE (space-to-Earth) MOD S5.523 MOBILE except aeronautical mobile Earth Exploration-Satellite (passive) Space Research (passive) MOD S5.522 | 18.6 – 18.8 EARTH EXPLORATION- SATELLITE (passive) FIXED FIXED-SATELLITE (space-to-Earth) MOD S5.523 MOBILE except aeronautical mobile SPACE RESEARCH (passive) MOD S5.522 | 18.6 – 18.8 EARTH EXPLORATION- SATELLITE (passive) FIXED FIXED-SATELLITE (space-to-Earth) MOD S5.523 MOBILE except aeronautical mobile Earth Exploration-Satellite (passive) Space Research (passive) MOD S5.522 |  |

Reasons:  To establish a common worldwide primary allocation to the Earth exploration-satellite (passive) services to be used for environmental measurements.

IAP/1.17/02

MOD

S5.522  In making assignments to stations in the fixed and mobile services, administrations are invited to take account of passive sensors in the Earth exploration-satellite and space research services operating in the band 18.6 - 18.8 GHz.  In this band, administrations should endeavour to limit as far as possible both the power delivered by the transmitter to the antenna and the e.i.r.p. in order to reduce the risk of interference to passive sensors to the minimum. In the band 18.6 - 18.8 GHz, fixed and mobile service stations shall be limited to a total power delivered to each antenna of 0 dBW across the entire 200 MHz band.

Reasons:  To enable passive sensors and the fixed service to operate in the band without excessive interference to the sensors.

S5.523  In assigning frequencies to stations in the fixed-satellite service in the direction space-to-Earth, administrations are requested to limit as far as practicable the power flux-density at the Earth’s surface in the band 18.6-18.8 GHz, in order to reduce the risk of interference to passive sensors in the earth exploration-satellite and space research services.In the band 18.6 - 18.8 GHz, the fixed-satellite service shall be limited to a power flux density at the Earth’s surface of -95 dBW/m2 in a reference bandwidth of 200 MHz for all angles of arrival.

IAP/1.17/03

MOD

Reasons:  To enable passive sensors and the fixed-satellite service to operate in the band without excessive interference to the sensors.

WP 7D (WRC-2000) proposed an alternative limit of –3 dBW on the output power from each FS channel within the 18.6-18.8 GHz band.

The Observatoire de Plateau de Bure interferometer consists of 3 antennas of 15 m diameter.

The  USA MMA (MilliMeter Array) will consist of 40 antennas of 8-m diameter, on a ring configuration.  
   The diameter of the ring will be capable of variation, ranging from 80 m to 10 km across.

The Japanese LMSA (Large Southern Millimeter Array) will consist of 50 antennas of 10-m diameter.

The VLA consists of 27 antennas of 25-m diameter, arranged in a Y pattern up to 36 km across.

The VLBA consists of 10 antennas of 25 m diameter, distributed across the continental US, Hawaii and 
   the US Virgin Islands

The Caltech Interferometer consists of 3 antennas of 10.4 m diameter

The BIMA (Berkeley-Illinois-Maryland Array) currently consists of 9 antennas of 6.1-m diameter. The 
   final configuration will consist of 11 antennas.

7 The Nobeyama site includes a 45 m diamater telescope, an interferometer that consists of 6 antennas of 
  10 m diameter, and a 60 cm diamter submillimeter telescoope.