| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0110 |  |
|  |  |  | English only |  |
| WG(s):1, 2 |  | 2nd FG IPTV meeting: Busan, 16 - 20 October 2006 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | Cisco Systems, Inc. |  |  |  |
| Title: | Guidelines for General IPTV Functional Architecture |  |  |  |

Abstract

This contribution proposes some key considerations for the development of a general IPTV framework architecture, which should be used as a basis for the IPTV FG work to specify an IPTV functional architecture.

Introduction

At the previous IPTV FG meeting there was extensive discussion on an IPTV functional architecture to enable the work in the various WGs to progress within a coherent overall architectural context.  However, it must be admitted there was a lack of sufficient progress, since there appeared to be some confusion as to the relationship between requirements for the IPTV architecture and those of the IMS architecture being developed as part of the NGN framework.

This contribution clarifies the relationship between these differing but related requirements, and suggests key criteria that should be considered in developing a general IPTV functional architecture capable of accommodating all aspects of this complex technology and its potential evolution.  While there may be common elements and potential relationships with an IMS architecture, at this stage it is more important to focus on the basic elements of the IPTV framework and view the relationship with IMS in the context of the overall NGN framework.

Discussion

In the previous FG IPTV meeting we proposed a general IPTV functional architecture, which was included as a further study item in the WG 1 living list together with other proposals based on differing partitioning of the functional elements.  This architecture is reproduced here in Fig.1 for reference.

Figure 1	A General Functional Architecture for IPTV

Considering that there are many ways to approach building of a functionally equivalent IPTV architecture, it is important to define the key criteria that need to be taken into account in order to arrive at a design that enables all potential solutions to be described in a reasonably inclusive way.  In other words, a restrictive architectural approach that attempts to base IPTV on a specific model or existing solution should not be followed at this stage.

It is important to note that the general architecture shown in Fig. 1 has significant benefits in this respect.  Not only is it consistent with the NGN framework architecture, but it also broadly represents typical IPTV network architectures that are already deployed, better enabling a graceful transition to any future more standard-based approach.

In addition, it defines the main functional domains required for IPTV service delivery in accordance with the work partitioning being undertaken in the IPTV FG, so that the Middleware, IPTV Network Control, QoS, Home Network/End System, etc. work can be seamlessly integrated by means of this model.

However, based on previous discussions there appears to be broad support for the view that a generic IPTV functional architecture should be consistent with the overall NGN framework architecture developed by the ITU-T over the past few years.  For example the parent Study Group for the IPTV Focus Group was decided to be SG13 where NGN architecture is being developed.  In any case, the NGN framework is sufficiently general to support a range of multimedia services, including those that are both session and non-session based, and therefore does not constrain any specific IPTV functional architecture. Consequently, IPTV may be easily incorporated as part of the NGN, in order to take advantage of important NGN attributes and capabilities such as QoS, security and survivability, etc., which are also desirable for delivering IPTV services.

In addition, the NGN framework already includes the IMS functional architecture as part of the control plane for session based services.  In the previous FG IPTV meeting it was proposed by some experts that IPTV should be “based” on IMS, although what this means in practice is unclear, particularly since at this stage it is not decided that all IPTV services are classified as session based only.

Since IMS functions are generally considered part of an overall NGN framework architecture, the question arises as to what could be the relationship of IPTV to IMS from an architectural perspective.  Considering that IPTV services can be supported independent of IMS as well as take advantage of them if available, a better approach might be to consider them both parts of an overall NGN framework and enable a logical relationship to exist between IPTV and IMS as represented below in Fig. 2.

Figure 2	IPTV Relationship to IMS

Fig 2 shows that both IPTV and IMS functional architecture can be considered as a part of the overall NGN framework, This architectural approach enables the potential sharing of common elements where appropriate, but may also utilize independent components as necessary to accommodate existing solutions or differing requirements.  Interaction between IPTV and IMS services may be undertaken via suitable interworking “Gateway” functions as shown, or by having applications act as both IPTV and IMS applications leveraging services from both domains.

The gateway functions can address needed co-ordination and management of shared resources or both IPTV and IMS functions can leverage common functions in the Transport Stratum and network infrastructure.  The details of this interworking gateway function will depend on the specific services being required in either domain and shared resources being managed, and this should be considered as a key item of further study by the IPTV FG.

A major advantage of this approach is that IPTV and IMS can be developed independently as the two technologies evolve and develop, while maintaining the possibility of component sharing and convergence as the services mature.

Another major benefit of such an architecture is that IPTV solutions that are independent of IMS, which is the case for most existing approaches, can be accommodated into the IPTV framework, while offering the possibility of convergence within the NGN framework as the respective services evolve.

It should be noted that there is essentially no significant difference between the approach suggested in Fig. 2 and previous discussions proposing that the IMS framework be studied in support of IPTV services.  Interworking between such services and IPTV to provide a basis for triple or quadruple- play service profiles is a logical and obvious first step towards full convergence.

Proposal

It is therefore proposed that the relationship between IPTV and IMS services should be defined as shown in Fig.2 above, and should be developed as part of an overall NGN framework architecture.

It is proposed that the interworking gateway functional requirements between IPTV and IMS services should be studied and clearly identified first in order to provide a basis for further work on the detailed functional entities, interfaces and protocols that may need to be specified.

____________________________