| Introduction INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0119 |  |
|  |  |  | Original: English |  |
| WG(s): 1 |  | 2nd FG IPTV meeting: Busan, 16-20 October 2006 |  |  |
| INPUT DOCUMENT |  |  |  |  |
| Source: | Huawei Technologies Co., Ltd. |  |  |  |
| Title: | IPTV Architecture Requirement of fixed and mobile convergence |  |  |  |

During the first meeting of IPTV focus group, the output document of WG1 [1] has given several high-level requirements of fixed and mobile convergence, which indexed with REQ_Arch_03 and REQ_Arch_04.

This contribution proposes more specific requirements as a supplement to the requirements mentioned above.

# Discussion

REQ_Arch_03: IPTV service should support operation over heterogeneous delivery networks.

We propose additional sub requirements as follows：

REQ_Arch_03_01：Support of unified business management for IPTV service delivered over fixed and mobile network.

This includes product management, subscriber management, partner settlement, customer care, rating and billing functions, etc.

REQ_Arch_03_02：Support of unified operation and maintenance  management for the IPTV service operator, including device and network management functions.

REQ_Arch_03_03：Support of unified media delivery mechanism to both fixed and mobile terminals, including unified delivery network topology , media transport protocol and session control protocol.

REQ_Arch_03_04：Open IPTV capability interface to other services in order to support  convergence service.

REQ_Arch_03_05：Support of unified QoS  control policies regardless of different access networks.

REQ_Arch_03_06：Support of unified service security mechanism regardless of different access networks.

REQ_Arch_03_07：Support of unified user identity management, i.e. unified user identifier, unified user authentication and authorization  mechanism.

REQ_Arch_03_08：Support of sso(single sign on) for user to use individual IPTV services with only one authentication procedure.

REQ_Arch_03_09：Support of unified content protection mechanism in mobile and fixed network environment, including encrypting technologies and security protocols.

REQ_Arch_03_10：Support of mobility management to enable networks to locate roaming and nomadic terminals  for media delivery and maintain connections with terminals that change their points of attachment.

REQ_Arch_04: The IPTV should support usage by different end devices.

We propose additional sub requirements as follows：

REQ_Arch_04_01：Support of service discovery and presentation by different end devices. Should provide the same type of presentation style and experience to fixed as well as mobile terminals.

REQ_Arch_04_02：Support of usage by  different end devices with different media decoding format.

Because of historic reason, there have been  a lot of media coding formats such as WMV, MPEG4, REAL , deployed in existing systems and terminals. It’s necessary for the IPTV media delivery system to have the ability to recognize and adapt to the terminals’ ability.

REQ_Arch_04_03：Support of usage by  terminals with different access link bandwidth and can dynamically adapt to the changes of bandwidth and other network characteristics.

REQ_Arch_04_04：Support of usage by  terminals with  different  display size and resolution.

REQ_Arch_04_05：Should have error resilience capability while delivering service over wireless networks which may have poor transport quality.

REQ_Arch_04_06：Should keep the service continuity when user switches from mobile terminal to fixed terminal or vice versa.

# Proposal

It is proposed to adopt the  requirements above to ensure that IPTV service in mobile network environment is consistent with that in a fixed network environment.

References

[1] ITU-T  FG IPTV-OD-0024: Working Document on Requirements for IPTV

______________