| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0127 |  |
|  |  |  | Original: English |  |
| WG(s): 2, 1 |  | 2nd FG IPTV meeting: Busan, 16-20 October 2006 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | ETRI |  |  |  |
| Title: | Classification of IPTV services based on network QoS requirements |  |  |  |

- 1. Introduction

- This document contributes to classification of IPTV services based on network QoS requirements. Network providers, one of main players in IPTV services, have to know what level of QoS they shall provide for each one of IPTV services.  Thus, service classification based on network QoS perspective is required, and this can be an initial working document for the work.

- 2. References

- The following ITU-T recommendation and IPTV document are referenced in this document.

- ITU-T Recommendation Y.1541 (2006), Network performance objectives for IP-based services.

- FG IPTV-OD-0026 (2006), Working Document on Service Scenario for IPTV

- 3. Abbreviations

- This document uses the following abbreviations:

- IPTD	IP Packet Transfer Delay

- IPDV	IP Packet Delay Variation

- IPLR	IP Packet Loss Rate

- IPER	IP Error Rate

- IPRR	IP Reordering Ratio

- VoIP	Voice over IP

- VTC	Video Teleconference

- U	Unspecified

- BE 	Best Effort

- LL	Low Loss

- I	Interactive

- RTI	Real-Time Interactive

- RTMU	Real-Time Multicast & Unicast

- 4. Network QoS classes

- IPU-T Recommendation Y.1541[1] defines classes of network Quality of Service (QoS) with objectives for Internet Protocol performance parameters. Each QoS class defines a specific combination of limits on the performance values. The following Table 1 shows IP network QoS definitions, network performance objectives and guidance for applicability. Class 6 and 7 are intended to support the performance requirements of high bit rate user applications that have more stringent loss/error requirements than those supported by classless 0 through 4.

- Table 1/Y.1541 – IP network QoS class definitions and network performance objectives/Applications

| - QoS class | - IPTD | - IPDV | - IPLR | - IPER | - IPRR | - Applications (examples) |
| --- | --- | --- | --- | --- | --- | --- |
| - 0 | - 100 ms | - 50 ms | - 1 x 10-3 | - 1 x 10-4 | - - | - Real-time, jitter sensitive, - high interaction (VoIP, VTC) |
| - 1 | - 400 ms | - 50 ms | - 1 x 10-3 | - 1 x 10-4 | - - | - Real-time, jitter sensitive, - Interactive |
| - 2 | - 100 ms | - U | - 1 x 10-3 | - 1 x 10-4 | - - | - Transaction data, - highly interactive (Signalling) |
| - 3 | - 400 ms | - U | - 1 x 10-3 | - 1 x 10-4 | - - | - Transaction data, interactive |
| - 4 | - 1 s | - U | - 1 x 10-3 | - 1 x 10-4 | - - | - Low loss only (short transaction, bulk data, video streaming) |
| - 5 | - U | - U | - U | - U | - - | - Traditional applications of - default IP network |
| - 6 | - 100ms | - 50 ms | - 1 x 10-6 | - 1 x 10-5 | - 1 x 10-6 | - High bit rate, strictly low loss/error (TV broadcast on IP) |
| - 7 | - 400ms | - 50 ms | - 1 x 10-6 | - 1 x 10-5 | - 1 x 10-6 | - High bit rate, strictly low loss/error |

- 5. Association of IPTV Services with Y.1541 QoS classes

- This section presents classified sets of IPTV services by network QoS requirements and associates them with IP QoS classes defined in Y.1541. We classify IPTV services into 5 categories. The first set is named Best Effort (BE) service. In this set, services are regarded as traditional default IP network applications. They do not need to be real-time, and errors can be recovered using transmission control protocol on the best effort network. So this equals to the QoS class 5 of Y.1541. The second set is Low Loss (LL) service set. This set is equal to the QoS class 4, because of its features of short transaction and bulk data. The next sets are Interactive (I) service, Real-Time Interactive (RTI) service, and Real-Time Multicast & Unicast (RTMU) service. And they are corresponding to QoS class 2/3, QoS class 0/1 and QoS class 6/7, respectively. And Table 2 also shows the IPTV service examples of each category.

- Table 2 - Association of IPTV services with Y.1541 QoS classes

| - IPTV service category | - IP QoS class | - IPTV service examples |
| --- | --- | --- |
| - Best Effort (BE) service | - QoS class 5 | - content download service |
|  |  | - regulatory information service |
|  |  | - T-information |
|  |  | - e-mail |
| - Low Loss (LL) service | - QoS class 4 | - VOD, MOD |
|  |  | - T-commerce |
|  |  | - T-learning based on VOD |
| - Interactive (I) service | - QoS class 2/3 | - messenger |
|  |  | - T-learning based on interactive |
| - Real-Time Interactive (RTI) service | - QoS class 0/1 | - VOIP, video phone |
|  |  | - multi player game |
| - Real-Time Multicast & Unicast (RTMU) service | - QoS class 6/7 | - linear/broadcast |
|  |  | - multi angle |
|  |  | - pay per view (PPV) |
|  |  | - networked personal video record (PVR, time-shift) |

- 6. Mapping of IPTV services with QoS classes

- In this section, we tried to mark required QoS class or category on each IPTV services known so far. The list and definition of IPTV services still needs to be clarified further, according to the WG1 work progress on defining IPTV services[2]. Thus, this table should be updated after the service list and each IPTV service scenarios are clearly defined.

- Table 3 - Mapping of IPTV services with QoS classes

| IPTV services | IPTV service category/QoS class |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BE | LL | I |  | RTI |  | RTMU |  |
|  | 5 | 4 | 3 | 2 | 1 | 0 | 7 | 6 |
| Linear/broadcast TV (audio, video and data) |  |  |  |  |  |  |  |  |
| Multi-angle service |  |  |  |  |  |  |  |  |
| Time-shift TV (network PVR) |  |  |  |  |  |  |  |  |
| Pay Per View (PPV) (Near Vod) |  |  |  |  |  |  |  |  |
| Real VoD |  |  |  |  |  |  |  |  |
| Downloaded based video content distribution (Push VoD) |  |  |  |  |  |  |  |  |
| Content download service |  |  |  |  |  |  |  |  |
| Consumer originated content (video, etc. and applications) |  |  |  |  |  |  |  |  |
| Consumer originated broadcast |  |  |  |  |  |  |  |  |
| Linear broadcast audio |  |  |  |  |  |  |  |  |
| Music on demand including audio book |  |  |  |  |  |  |  |  |
| Pictures |  |  |  |  |  |  |  |  |
| T-learning based on VoD |  |  |  |  |  |  |  |  |
| T-learning based on interactive |  |  |  |  |  |  |  |  |
| Single player games (game on demand) |  |  |  |  |  |  |  |  |
| Multiplayer games |  |  |  |  |  |  |  |  |
| T-information (news, weather, transportation, etc.) |  |  |  |  |  |  |  |  |
| T-commerce (banking, stock, shopping, ticketing, auction, delivery, event etc.) |  |  |  |  |  |  |  |  |
| T-communication data (e-mail, SMS, web surfing, etc.) |  |  |  |  |  |  |  |  |
| T-communication interactive (messenger, channel chatting) |  |  |  |  |  |  |  |  |
| T-communication voice (VoIP, multiple video conference, video phone, etc.) |  |  |  |  |  |  |  |  |
| T-entertainment data (photo album, lottery, blog, etc.) |  |  |  |  |  |  |  |  |
| T-entertainment VOD (karaoke) |  |  |  |  |  |  |  |  |
| Presence service |  |  |  |  |  |  |  |  |
| Communication messaging |  |  |  |  |  |  |  |  |

__________